3GPP TSG RAN WG1 #107bis-e																		R1-2200381

e-Meeting, January 17th – 25th, 2022

Source: 	Intel Corporation

Title:	Remaining details on joint channel estimation for PUSCH

Agenda item:	8.8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN1#107-e meeting, the following conclusions, agreements and working assumptions were made regarding joint channel estimation for PUSCH [1]:

Agreement

Support Option 1’-a

Option 1’-a:

- If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions)

Agreement

- For non-back-to-back PUSCH/PUCCH transmissions across consecutive slots, the other uplink transmission in the middle of two PUSCH/PUCCH transmissions constitutes an event that violates power consistency and phase continuity.

Conclusion

- Dynamic indication of the window length L of the configured TDW by DCI or indicated by TDRA table with one additional entry is not supported.

Agreement

This working Assumption is confirmed.

Working assumption:

- The start of the first actual TDW is the first symbol (at least determined by TDRA table) for the first PUSCH transmission in an available slot within the configured TDW.

- The end of the actual TDW is

      - the last symbol (at least determined by TDRA table) for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

      - the last symbol (at least determined by TDRA table) of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

- For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first symbol (at least determined by TDRA table) for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement

- The action of gNB indicated TA commands constitutes an event that violates power consistency and phase continuity.

Agreement

- UE should not perform UE autonomous TA adjustment during the actual time domain window.

Agreement

- The candidate values of the window length L of the configured TDW can be any integer value that is larger than 1 and no larger than the maximum duration.

Agreement

The following agreement is clarified as follows.

- For PUSCH repetition type A counting based on available slots,

  - “The configured TDWs are determined based on available slots” in the agreement means “The start of the configured TDWs is determined based on available slots”

Agreement(RAN1#106b-e)

- For PUSCH repetition type A counting based on physical slots

  - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW

- For PUSCH repetition type A counting based on available slots

  - The configured TDWs are determined based on available slots, where start of a configured TDWs is the first available slot after the last available slot of a previous configured TDW.

Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Agreement

- The TDW determination procedure agreed for PUSCH repetition type A is reused, when applicable, for PUSCH repetition type B and TBoMS with or without repetition.

- No additional specification enhancements for PUSCH repetition type B and TBoMS

Agreement

The following working assumption is confirmed

Working assumption:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs.

- Each configured TDW consists of one or multiple consecutive physical slots.

- The window length L of the configured TDW(s) can be explicitly configured with a single value and L is no longer than the maximum duration.

  - FFS: The maximum value of L is the duration of all repetitions

  - FFS: Solutions to error propagation issue if  L is longer than the maximum duration is to be discussed further.

  - FFS: The window length L is configured per UL BWP

- The start of the first configured TDW is the first PUSCH transmission

  - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

- The start of other configured TDWs can be implicitly determined prior to first repetition.

  - FFS: The configured TDWs are consecutive for paired spectrum/SUL band

  - FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration.

- The end of the last configured TDW is the end of the last PUSCH transmission.

  - FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

- Within one configured TDW, one or multiple actual TDWs can be implicitly determined:

  - The start of the first actual TDW is the first PUSCH transmission within the configured TDW.

    - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

  - After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended.

    - The actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

      - FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

    - An event occurs that violates power consistency and phase continuity

      - FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling.

      - FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated.

  - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling.

    - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event,

      - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event.

    - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW.

    - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not

Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above.

Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration.

Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

Agreement

- If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

  - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event.

  - Note: At least frequency hopping event is considered as semi-static event.

Working assumption:

- The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity.

  - If UE is configured to accumulate TPC commands,

    - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW.

  - If UE is not configured to accumulate TPC commands

    - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW.

      - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Agreement

- If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation constitutes an event that violates power consistency and phase continuity.

  - FFS: UL beam switching for multi-TRP operation is regarded as a semi-static event.

In the contribution, we discuss remaining issues for joint channel estimation for PUSCH coverage enhancement. Our views on enhancement on PUSCH repetition type A, TB processing over multi-slot PUSCH, PUCCH enhancements, and Msg3 PUSCH repetition are presented in our companion contributions [2], [3], [4] and [5], respectively.

# Discussion on time domain window

At the RAN1#107-e meeting, it was agreed that for UL beam switching for multi-TRP operation constitutes an event that violates power consistency and phase continuity. It is FFS whether UL beam switching for multi-TRP operation is regarded as a semi-static event in case of DMRS bundling [1]. In our view, given that UL beam switching is triggered in the scheduling DCI, our view is that the design can be similar to frequency hopping, i.e., to treat UL beam switching as semi-static event. In this case, UE should restart the DMRS bundling within a nominal TDW in case of UL beam switching.

Proposal 1

- UL beam switching for multi-TRP operation is regarded as a semi-static event.

Further, it was agreed that dropping or cancellation of a PUSCH and PUCCH repetition based on Rel-15/16 is regarded as an event that violates power consistency and phase continuity [6]. In this case, when PUCCH repetition with low priority is dropped when overlapping with PUSCH transmission with high priority, which is defined in Clause 9 in TS38.213 [7], this should be treated as an event. The draft text proposal can be updated as follows [8]:

=== Start of text proposal for 38.214 Subclause 6.1.7 === Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’ or ‘noncodebook’, a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations, different spatial relations are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Frequency hopping. === end of text proposal for 38.214 Subclause 6.1.7 ===

Proposal 2

- Adopt the following text in 38.214 Subclause 6.1.7.

  - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213].

# Discussion on TPC command

At the RAN1#107-e meeting, it was agreed as working assumption that the action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. Further, if UE is configured to accumulate TPC commands, and if UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW [1].

In our view, when DMRS bundling is configured and employed for PUSCH repetitions, for UEs in coverage enhancement mode, it is expected that UE would apply maximum transmit power for the transmission of PUSCH repetitions. In this case, power control accumulation mechanism may not be necessary for DMRS bundling. Further, the working assumption is only applied for FDD system. For TDD scenario, when UE receives the group command TPC command in a PDCCH, this indicates that UE cannot maintain power consistency and phase continuity.

Note that for DMRS bundling, if gNB realizes that a transmit power adjustment is essential for coverage enhancement of uplink transmission, it may be more desirable to treat this as an event so that UE can react quickly and dynamically adjust the transmit power as early as possible. Hence, in our view, the working assumption for action of group common TPC commands is not confirmed. Further, action of group common TPC commands is considered as an event for DMRS bundling should be considered as an event for DMRS bundling.

The draft text proposal can be updated as follows [8]:

=== Start of text proposal for 38.214 Subclause 6.1.7 === Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’ or ‘noncodebook’, a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations, different spatial relations are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Transmit power adjustment in response to a TPC command in DCI format 2_2 according to Clause 7.1 and 7.2 of [6, TS 38.213]. - Frequency hopping. === end of text proposal for 38.214 Subclause 6.1.7 ===

Proposal 3

- Action of group common TPC commands is considered as an event for DMRS bundling.

  - The working assumption for action of group common TPC commands is not confirmed.

- Adopt the following text in 38.214 Subclause 6.1.7.

  - Transmit power adjustment in response to a TPC command in DCI format 2_2 according to Clause 7.1 and 7.2 of [6, TS 38.213].

# Conclusions

In this contribution, we discussed remaining issues for joint channel estimation for PUSCH coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- UL beam switching for multi-TRP operation is regarded as a semi-static event.

Proposal 2

- Adopt the following text in 38.214 Subclause 6.1.7.

  - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213].

Proposal 3

- Action of group common TPC commands is considered as an event for DMRS bundling.

  - The working assumption for action of group common TPC commands is not confirmed.

- Adopt the following text in 38.214 Subclause 6.1.7.

  - Transmit power adjustment in response to a TPC command in DCI format 2_2 according to Clause 7.1 and 7.2 of [6, TS 38.213].

# References

- Chairman’s notes, RAN1 #107 e-Meeting, November 2021

- R1-2200379, “Remaining details of enhancements on PUSCH repetition type A”, Intel Corporation, e-Meeting, January 2022

- R1-2200380, “Remaining details on TB processing over multi-slot PUSCH”, Intel Corporation, e-Meeting, January 2022

- R1-2200382, “Remaining details on PUCCH enhancements”, Intel Corporation, e-Meeting, January 2022

- R1-2200383, “Remaining details on Msg3 PUSCH repetition”, Intel Corporation, e-Meeting, January 2022

- Chairman’s notes, RAN1 #106 e-Meeting, August 2021

- TS38.213, v16.7.0, “NR: Physical layer procedures for control”.

- R1-2112946, “Introduction of NR coverage enhancements”, November 2021