3GPP TSG RAN WG1 #107bis-e	  														             R1-2200393

e-Meeting, January 17th – 25th, 2022

Source: 	Intel Corporation

Title:	Discussion on UE features for NR coverage enhancement

Agenda item:	8.15.8

Document for:	Discussion and Decision

# Introduction

At the RAN1#107-e meeting, the following agreements were made regarding UE features for NR coverage enhancement [1].

Agreement

- Add an FG for the repetition of TB processing over multi-slot PUSCH as follows

| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support Repetition of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether to merge with FG 30-3 | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Keep FGs 30-1 and 30-1a as follows

| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configured grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 12 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2 configured grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Keep FGs 30-2 and 30-2a as follows

| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-2a | Configured grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Keep FG 30-3 as follows

| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Keep FGs 30-4 to 30-4d as follows

| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions FFS whether to merge with FGs 30-4a/4b/4c FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Keep FGs 30-4e and 30-4f as follows

| 30. NR_cov_enh | 30-4e | Enhanced Inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH FFS whether to merge with FG 30-4f | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling FFS whether to merge with FG 30-4e | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- FG 30-4g is kept as “Restart DM-RS bundling after the events that violate power consistency and phase continuity” as follows

| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Revised the component in FG 30-5 as “Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4”

- Add a component in FG 30-5 for support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4

- FG 4-23 and/or FG 25-2 are the prerequisite feature groups for FG 30-5

- Add FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4

Agreement

- FG 30-6 is kept as “Msg3 repetition Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI” as follows

| 30. NR_cov_enh | 30-6 | Msg3 repetition Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of Msg3 repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

In the contribution, we discuss UE features for NR coverage enhancement. The updated UE features for NR coverage enhancement after RAN1#107-e meeting were captured in [2] and are listed in the Appendix.

# Discussion on UE features for NR coverage enhancement

## UE features for PUSCH repetition type A enhancement

At the RAN1#107-e meeting, it was agreed to keep 30-1 and 30-1a, FGs 30-2 and 30-2a for PUSCH reception type A enhancement [1]. In our view, it is more appropriate to divide the UE feature group into DG-PUSCH and CG-PUSCH for both increased maximum number of repetitions and PUSCH repetition type A with counting based on available slots. In this regard, current structure needs to be kept and merging FG 30-1a with FG 30-1, and FG 30-2a with FG 30-2 is not necessary.

At the RAN1#107-e meeting, it was agreed that increased maximum number of repetitions is supported for Type 1 and Type 2 CG-PUSCH. In particular, repK in ConfiguredGrantConfig supports up to 32 repetitions [1]. Based on this agreement, Type 1 CG-PUSCH needs to be included as part of FG30-1a.

Based on the discussions above, Table 1 illustrates suggested update for UE feature groups for PUSCH repetition type A enhancement.

Table 1. UE feature groups for PUSCH repetition type A enhancement

| Index | Feature group | Components |
| --- | --- | --- |
| 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. |
| 30-1a | Increased maximum number of Type 2 configured grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV indicated in a TDRA list and by repK-r17. A row index of the TDRA list is indicated by a Type 1 and Type 12 configured grant configuration. |
| 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. |
| 30-2a | Configured grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for Type 1 and Type 2 configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |

Proposal 1

- Merging FG 30-1a with FG 30-1, and FG 30-2a with FG 30-2 is not needed.

- Consider Table 1 for UE feature groups of PUSCH repetition type A enhancement.

## UE features for TBoMS

At the RAN1#107-e meeting, it was agreed to keep FG30-3 and it is FFS whether to split FG 30-3 into at least 2 separate FGs: one for DG and another for CG [1]. Note that as agreed in the RAN1#106-e meeting, TB processing over multiple slots (TBoMS) is supported for both configured grant and dynamic grant [4]. Similar to PUSCH repetition type A enhancement, it is more appropriate to differentiate DG-PUSCH and CG-PUSCH for the UE feature group. Table 2 illustrates suggested update for UE feature groups for TBoMS.

Table 2. UE feature groups for TBoMS

| Index | Feature group | Components |
| --- | --- | --- |
| 30-3 | TB processing over multi-slot PUSCH for DG-PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode for DG-PUSCH. |
| 30-3a | TB processing over multi-slot PUSCH for CG-PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode for CG-PUSCH. |

Proposal 2

- For TBoMS, split FG30-3 into DG-PUSCH and CG-PUSCH.

- Consider Table 2 for UE feature groups of TBoMS.

## UE features for DMRS bundling for PUSCH and PUCCH

In the previous RAN1 meetings [3][4][5], the following use cases were agreed to be supported for joint channel estimation of PUSCH:

- back-to-back PUSCH transmissions (of the same TB) for repetition type A and B scheduled by dynamic grant or configured grant

- back-to-back PUSCH transmissions for TB processing over multiple slots

- non-back-to-back PUSCH transmissions across consecutive slots (of the same TB) for repetition type A and type B scheduled by dynamic grant or configured grant.

Considering different use cases as mentioned above, it is more appropriate to divide the UE feature groups for back to back PUSCH transmission and non-back to back PUSCH transmission. The same principle can also apply for DMRS bundling for PUCCH repetitions, i.e., UE feature groups can be divided into back-to-back PUCCH transmission and non-back to back PUCCH transmission.

Further, it was agreed that the time domain window determination procedure agreed for PUSCH repetition type A is reused, when applicable, for PUSCH repetition type B and TBoMS with or without repetition [1]. This indicates that a single FG for DMRS bundling can be generally applied for PUSCH transmission, including PUSCH repetition type A and type B, and TBoMS. In addition, it is more desirable to differentiate DMRS bundling for PUSCH and PUCCH repetitions in two separate FGs. Table 3 illustrates suggested update for UE feature groups for DMRS bundling.

Table 3. UE feature groups for DMRS bundling

| 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |
| --- | --- | --- |
| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A, type B and TB processing over multi-slot PUSCH |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH |
| 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions |

Proposal 3

- For UE features of DMRS bundling, split back-to-back and non-back-to-back PUSCH/PUCCH transmission.

- Merge FGs 30-4b, 30-4c into FG 30-4a.

- Consider Table 3 for UE feature groups of DMRS bundling.

For inter-slot frequency hopping with inter-slot bundling, current structure of UE feature groups can be kept for both PUSCH and PUCCH repetitions. Further update on the detailed description for UE feature groups for inter-slot frequency hopping with inter-slot bundling can be illustrated in Table 4.

Table 4. UE feature groups for inter-slot frequency hopping with inter-slot bundling

| 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH |
| --- | --- | --- |
| 30-4f | Enhanced inter-slot frequency hopping with inter-slot bundling for PUCCH repetitions with DMRS bundling | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUCCH repetitions with DMRS bundling |

Proposal 4

- Keep FGs 30-4e and 30-4f.

- Consider Table 4 for UE feature groups of inter-slot frequency hopping with inter-slot bundling.

# Conclusions

In this contribution, we discussed UE features for NR coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- Merging FG 30-1a with FG 30-1, and FG 30-2a with FG 30-2 is not needed.

- Consider Table 1 for UE feature groups of PUSCH repetition type A enhancement.

Proposal 2

- For TBoMS, split FG30-3 into DG-PUSCH and CG-PUSCH.

- Consider Table 2 for UE feature groups of TBoMS.

Proposal 3

- For UE features of DMRS bundling, split back-to-back and non-back-to-back PUSCH/PUCCH transmission.

- Merge FGs 30-4b, 30-4c into FG 30-4a.

- Consider Table 3 for UE feature groups of DMRS bundling.

Proposal 4

- Keep FGs 30-4e and 30-4f.

- Consider Table 4 for UE feature groups of inter-slot frequency hopping with inter-slot bundling.

# References

- Chairman’s notes, RAN1 #107 e-Meeting, November 2021

- R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, November 2021

- Chairman’s notes, RAN1 #104 e-Meeting, January 2021

- Chairman’s notes, RAN1 #106 e-Meeting, August 2021

- Chairman’s notes, RAN1 #105 e-Meeting, May 2021

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether to merge with FG 30-3 | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions FFS whether to merge with FGs 30-4a/4b/4c FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH FFS whether to merge with FG 30-4f | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling FFS whether to merge with FG 30-4e | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | 4-23 and/or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |