3GPP TSG RAN WG1 #107b-e		                          		    R1-2200395

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.3.1

Source:	InterDigital, Inc.

Title:	Remaining issues on HARQ enhancements for URLLC

Document for:	Discussion and Decision

# Introduction

In the RAN#94 plenary meeting, RAN1 was tasked to complete the remaining normative work for Rel-17 enhanced IIoT and URLLC by Q1 of 2022 and to finalize decision that impact other WGs in RAN1#107bis-e. In this contribution we discuss the remaining issues of one-shot HARQ retransmission.

# Discussion

## One-shot HARQ-ACK retransmission

In RAN1#107e meeting, it was agreed to support a 1-bit triggering DCI field for triggering one-shot HARQ retransmission with some details left for further study:

Agreement Apply a 1-bit triggering DCI field for triggering indication of one-shot HARQ re-transmission on PUCCH. - The triggering DCI with the triggering bit set to ‘1’ is not able to schedule PDSCH. - Some unused bit field in the DCI is used to indicate the HARQ slot offset. - FFS: if the ‘one-shot HARQ-ACK request’ field can be reused - FFS: which unsed DCI field in the DCI is used for HARQ slot offset indication - FFS: The indication of whether the PDSCH is not scheduled will reuse Rel-16 type-3 HARQ ACK CB UE behavior

Since the triggering DCI of HARQ-ACK retransmission will not be able to schedule a PDSCH, one of scheduling information bitfield in the DCI can be re-used to indicate the HARQ slot offset. The MCS bitfield with its 5 bits can indicate the HARQ slot offset. The MCS field for transport block 1 can point to one of the possible values.

Proposal 1:  MCS field in the DCI is re-used to indicate the HARQ slot offset for the one-shot HARQ-ACK re-transmission.

For the SPS deferral feature, up to 32 for deferral value was supported. Similarly, up to 32 for HARQ-ACK retransmission slot offset can be supported. For the negative range, up to -16 can be supported:

Proposal 2:  The HARQ slot offset value range of the one-shot HARQ-ACK re-transmission is {-16,32}.

Since the simultaneous configuration of one-shot HARQ retransmission and Rel-16 type 3/enhanced type 3 HARQ ACK CB is not supported in Rel-17, the DCI will either trigger one-shot HARQ retransmission or Rel-16 type 3/enhanced type 3 HARQ ACK CB transmission. In case one-shot HARQ retransmission is configured, there will be no one-shot HARQ request field for type 3 HARQ ACK CB in the DCI to re-use for one-shot HARQ retransmission. Thus, it is not possible to re-use the filed when one-shot HARQ retransmission is configured. Furthermore, it was agreed to introduce a new 1-bit triggering for one-shot HARQ retransmission. When the triggering bit set to ‘1’, it is not possible to schedule PDSCH. Re-using the indication of whether the PDSCH is not scheduled in Rel-16 type -3 HARQ-ACK CB UE behavior is not needed.

Observation 1:  The one-shot HARQ-ACK request cannot be re-used for triggering indication of one-shot HARQ re-transmission.

# Conclusion

In this contribution we propose the following:

Proposal 1:  MCS field in the DCI is re-used to indicate the HARQ slot offset for the one-shot HARQ-ACK re-transmission.

Proposal 2:  The HARQ slot offset value range of the one-shot HARQ-ACK re-transmission is {-16,32}.

Observation 1:  The one-shot HARQ-ACK request cannot be re-used for triggering indication of one-shot HARQ re-transmission.

# References

- R1-2112758, “Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT”, Moderator (Nokia)

- “Chair’s Notes RAN1#106b-e”, 3GPP TSG RAN WG1 Meeting #107-e e-Meeting.