3GPP TSG-RAN WG1 #107bis-e 	R1-2200399

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.7.2

Source:	Panasonic

Title:	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time

Document for:	Discussion/Decision

# Introduction

- In RAN1#107-e and #106bis-e meeting, the agreements [1][2] in the appendix were achieved regarding extensions to Rel-16 DCI-based power saving adaptation.

In this contribution, we share our views on more details on the common design of search space set group (SSSG) switching and PDCCH skipping.

# Discussion

On bit mapping of DCI indication PDCCH monitoring adaptation

In the different cases agreed for PDCCH monitoring adaptation, Case 3 and 5 share most of the bit mapping except the state if ‘11’. Current for Case 3, whether ‘11’ is reserved is in the bracket for further confirmation. In addition, it is still FFS that whether Case 5 is supported, which defines ‘11’ as Beh 1A for PDCCH skipping. In our view, having 3 SSSGs configured does not necessarily mean the PDCCH skipping can not be used for the PDCCH adaptation. The specification effort is straight forward such that the Case 3 and Case 5 should both be supported as a single case, where the state ‘11’ behaviour is RRC configurable, which can be NULL or a skipping duration for Beh 1A.

Proposal 1: On bit mapping design of DCI indicated PDCCH monitoring adaptation, Case 5 and Case 3 should be supported as a single case, where the state ‘11’ UE behaviour is RRC configurable.

On Exceptional PDCCH monitoring

It has been agreed that the PDCCH based monitoring adaptation is applied to USS and type-3 CSS. When PDCCH skipping is indicated, it is still FFS on whether to continue monitoring PDCCH scrambled by C-RNTI for Type 0/0A/1/2 CSS or not.

For UE power saving aspect, allowing UE to continue monitoring PDCCH will lower the power saving gain. From the network side, some opportunity to schedule UE even in the skipping period has benefit to stability of system operation. Thus this can be up to gNB implementation and the level of monitoring activity should be sufficiently low.

When SSSGs are configured, for a search space not associated with any SSSGs, the default UE behaviour should be always monitor the search space regardless of the SSSG switching indication. A search space not associated with any SSSGs is possible because the SSSG ID is optionally configured by RRC. Thus, gNB may make use of this type of search space to make UE continue monitor PDCCH no matter which kind of PDCCH adaptation is indicated. This has low specification impact, network control flexibility and follows the principles of legacy UE behaviour. If network wants UE to strictly skip the PDCCH monitoring, it will not configure a search space without SSSG ID.

Proposal 2: PDCCH scrambled by C-RNTI is monitored in Type 0/0A/1/2 CSS.

Proposal 3: When a search space is not configured with any SSSG ID, UE continues monitoring PDCCH in this search space without being impact by the DCI-based PDCCH monitoring adaptation.

- On the timer for SSSG switching

- Regarding the condition of resetting the timer configured for SSSG switching, below agreement lists three alternatives:

Agreement If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot o Alt 2a: for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 o Alt 2b: for the Type3-PDCCH CSS set or the USS set o Alt 2c: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI - otherwise, decrease the timer value by one after each slot. - FFS: When the timer expires in a slot

- In line with the proposal 2 and 3, in our view, UE should monitor PDCCH in Type 0/0A/1/2/3 CSS, and also the USS set not configured with any SSSG ID. Once UE detects a DCI format in any of them, timer should be reset. Between Alt 2b and 2c, considering we support the PDCCH monitoring in also other CSSs besides Type 3, our preference is Alt 2c, which is able to cover all the possible cases. In this sense, Alt 2b is our second preference.

- Proposal 4: UE resets the timer configured for SSSG switching after a slot when UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI. Our second preference is UE resets the timer after detecting a DCI format in a slot for the Type3-PDCCH CSS set or the USS set.

- Regarding FFS on the UE behaviour when the timer expires, it is related to the last FFS bullet in the below agreement:

Agreement - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , one of the following is supported - Alt a: 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - If M = 1, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A (stay in current SSSG) - [‘11’ is reserved] - If M = 2, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A for duration T1 (stay in current SSSG) - ‘11’ is Beh 1A for duration T2 (stay in current SSSG) - Note: for ‘00’ and ‘01’, UE performs Beh 1(i.e., does not perform PDCCH skipping ) - Note: By a single DCI indication, a UE is not expected to simultaneously perform two behaviors of SSSG switching and PDCCH skipping. - FFS SSSG switching timer behavior, and timer value related to the value of T1/T2

- Our view is UE should switch to SSSG#0 after the timer expires, no matter whether the slot of expiration is within an indicated PDCCH skipping duration or not. If it is the case, UE just needs to continue skipping PDCCH for finish the duration.

- Proposal 5: When SSSG timer expires, UE switches to SSSG#0. If this happens within PDCCH skipping duration, UE performs SSSG fallback switching firstly. Then PDCCH skipping continues until the end of PDCCH skipping duration.

- Regarding how the timer values are configured:

Agreement Select one of the alternatives from the following: - Alt 1: Separate RRC configuration for timer value(s) is supported for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 respectively. - Alt 2: the timer value(s) for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per cell. - Alt 3: the timer value(s) for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per BWP.

- Different SSSG may be configured with different periodicities for monitoring occasions, which is the purpose of traffic adaptation. Using different fallback timers can literally enable gNB to flexibly control after how long “time” UE should fallback to default SSSG, in terms of the number of valid monitoring occasions. The specification impact is low to support this. Therefore,

- Proposal 6: Separate RRC configuration for timer value(s) is supported for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 respectively, which are configured per BWP.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: On bit mapping design of DCI indicated PDCCH monitoring adaptation, Case 5 and Case 3 should be supported as a single case, where the state ‘11’ UE behaviour is RRC configurable.

Proposal 2: PDCCH scrambled by C-RNTI is monitored in Type 0/0A/1/2 CSS.

Proposal 3: When a search space is not configured with any SSSG ID, UE continues monitoring PDCCH in this search space without being impact by the DCI-based PDCCH monitoring adaptation.

- Proposal 4: UE resets the timer configured for SSSG switching after a slot when UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI. Our second preference is UE resets the timer after detecting a DCI format in a slot for the Type3-PDCCH CSS set or the USS set.

- Proposal 5: When SSSG timer expires, UE switches to SSSG#0. If this happens within PDCCH skipping duration, UE performs SSSG fallback switching firstly. Then PDCCH skipping continues until the end of PDCCH skipping duration.

- Proposal 6: Separate RRC configuration for timer value(s) is supported for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 respectively, which are configured per BWP.

# Reference

[1] RAN1 Chair’s Notes for 3GPP RAN1#106bis-e

[2] RAN1 Chair’s Notes for 3GPP RAN1#107-e

# Appendix:

Agreements achieved in RAN1#107-e meeting:

Agreement

The following agreement in RAN1#106-E is updated as follows,

Agreement(extracted from RAN1#106-E)

- At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell

  - FFS: the bit size of the indication is configurable

  - FFS: bit mapping to the PDCCH monitoring behaviour

  - FFS: details of indication of multiple cells case

  - A new indication field in scheduling DCI is used for indicating PDCCH monitoring adaptation

Agreement

Confirm the working assumptions with the following updates (extract from RAN1#106-bis agreements)

- The value of the timer in slots for monitoring PDCCH in the active DL BWP of the serving cell before moving to the default search space group is

- {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

- {1,2,3,…,40, 60, 80, 100, 100,160,200} for 30 kHz SCS,

- {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

- {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

Agreement

- For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , one of the following is supported

  - Alt a: 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors

    - If M = 1, then

      - ‘00’ is Beh 2

      - ‘01’ is Beh 2A

      - ‘10’ is Beh 1A (stay in current SSSG)

      - [‘11’ is reserved]

    - If M = 2, then

      - ‘00’ is Beh 2

      - ‘01’ is Beh 2A

      - ‘10’ is Beh 1A for duration T1  (stay in current SSSG)

      - ‘11’ is Beh 1A for duration T2  (stay in current SSSG)

- Note: for  ‘00’ and ‘01’, UE performs Beh 1(i.e., does not perform PDCCH skipping )

- Note: By a single DCI indication, a UE is not expected to simultaneously perform two behaviors of SSSG switching and PDCCH skipping.

- FFS SSSG switching timer behavior, and timer value related to the value of T1/T2

Agreement

If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

-            resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot

o    Alt 2a: for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2

o    Alt 2b: for the Type3-PDCCH CSS set or the USS set

o    Alt 2c: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI

-            otherwise, decrease the timer value by one after each slot.

-            FFS: When the timer expires in a slot

Agreement

-            For value X in Beh 1A, candidate skipping values are

o    Up to [100ms] length is supported,

o    The X is configured and indicated in the unit of slot.

o    Working assumption for candidate values for X

o    {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

o    {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS,

o    {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

o    {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

o    FFS: Equal to or longer than the applicable minimum scheduling offset

o    FFS: additional  symbol level / PDCCH monitoring period level skipping duration

Agreements achieved in RAN1#106bis-e meeting:

Agreement

Confirm the four working assumptions(extracted from package 1)

Working assumption(extracted from package 1):

Beh 1: PDCCH skipping is not activated

Working assumption(extracted from package 1):

Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported

Working assumption(extracted from package 1):

At most 3 SSSGs is supported to be configured for PDCCH monitoring adaptation.

Working assumption(extracted from package 1):

Beh 2B: stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2.

Agreement

Scheduling DCIs indicating timer value for a SSSG is not supported.

Agreement

For Beh 1A,

  - The UE can be configured to be indicated by DCI a value of X slots (i.e., skipping duration) among up to M = {1, 2, 3}multiple RRC configured values by scheduling DCIs indicating PDCCH schedules data

    - The bits for indicating PDCCH monitoring adaptation also indicating skipping duration. Details FFS

    - The maximum value of M = [2 or 3]

    - Note: M = 1 is not precluded.

Agreement

The bit mapping of DCI indication PDCCH monitoring adaptation is as follows,

- For Case 1 (i.e., PDCCH skipping), the following is supported

  - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=1

    - ‘0’ is Beh 1 and ‘1’ is Beh 1A

  - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=2 or 3

    - ‘00’ is Beh 1

    - ‘01’ is Beh 1A with skipping duration 1

    - ‘10’ is Beh 1A with skipping duration 2

    - ‘11’ is Beh 1A with skipping duration 3 if M=3, reserved if M=2

- For Case 2  (i.e., 2 SSSG switching) , the following is supported

  - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors

    - ‘0’ is Beh 2 and ‘1’ is Beh 2A

- For Case 3 (i.e., 3 SSSG switching) , the following is supported

  - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors

    - ‘00’ is Beh 2

    - ‘01’ is Beh 2A

    - ‘10’ is Beh 2B

    - [‘11’ is reserved]

      - FFS ‘11’ is Beh 1A

- For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , the following is supported

  - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors,

    - Indicated UE behaviors are Beh 1A, 2, 2A and FFS whether indicate Beh 1

    - FFS details bit mapping

    - FFS Timer behavior (e.g., reset timer value) for PDCCH monitoring adaptation indication when Beh 1A is indicated

- FFS: For Case 5 (i.e., 3 SSSG switching and skipping)

  - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors

    - ‘00’ is Beh 2

    - ‘01’ is Beh 2A

    - ‘10’ is Beh 2B

    - ‘11’ is Beh 1A

  - FFS Timer behavior (e.g., reset timer value) for PDCCH monitoring adaptation indication when Beh 1A is indicated

- Note: The UE can be configured to be indicated by DCI a value of X slots (i.e., skipping duration) among M RRC configured values by scheduling DCIs indicating PDCCH schedules data

- FFS whether to restrict Skipping duration to be shorter than SSSG initial timer value

- FFS whether the configuration is same or different for DCI format x_1 and DCI format x_2

Agreement

-         The value of the SSSG switching timer in slots for monitoring PDCCH in the active DL BWP of the serving cell SSSG#1 and/or SSSG#2 before moving to the default search space group is can be configured as

o    {[1...20,40,60,80,100]} for 15 kHz SCS,

o    {[1...40, 80,100,160,200]} for 30 kHz SCS,

o    {[1...80, 160,200,320,400]} for 60kHz SCS,

o    {[1...160,320,400,640,800]} for 120kHz SCS

Agreement

-         If the UE monitors PDCCH according to SSSG#1 and the timer expires, the UE starts monitoring PDCCH according to Beh 2.

-         If the UE monitors PDCCH according to SSSG#2 and the timer expires,

o    Alt 1: the UE monitoring PDCCH according to Beh 2

o    Other alternatives are not precluded

-         Timer can be optionally configured.

Agreement

Select one of the alternatives from the following:

-         Alt 1: Separate RRC configuration for timer value(s) is supported for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 respectively.

-         Alt 2: the timer value(s) for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per cell.

-         Alt 3: the timer value(s) for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per BWP.

Agreement

The following application delay for a scheduling DCI based PDCCH monitoring adaptation indication can be considered is as follows,

-         For PDCCH skipping,

o    Option b

o    Option f

o    Option d for downlink grant and Option c for uplink grant

o    Option i

o    Option j

o    Note: down-select between based on the options in RAN1#107-E

-         For SSSG switching,

o    Option a

o    Option d for downlink grant  and Option c for uplink grant

o    Option h

o    Option b

o    Option d for downlink grant and Option g for uplink grant

o    Note: down-select between based on the options in RAN1#107-E

-         The Options a – j is defined as follows,

o    Option a: the application timelines provided in Table 10.4-1 in TS38.213 for search-space group switching for unlicensed band form is reused.

u       for SCS configuration , FFS X = 25 or 39

u      FFS:

o    Option b:  the application delay needed for PDCCH processing for Rel-16 minimum application delay for K0min/K2min indication is reused/extended.

o    Option c: PDCCH monitoring adaptation command applies after PUSCH transmission if triggered by UL DCI

o    Option d: PDCCH monitoring adaptation command applies after HARQ-ACK transmission (or plus some margin for HARQ-ACK decoding).

o    Option e: after successfully decoding TB.

o    Option f:  Application delay should be “ZERO”  for PDCCH monitoring adaptation. PDCCH monitoring adaptation would be applied after UE receive the additional PDCCH monitoring adaptation control signaling bit(s) in DCI

o    Option g:  Application delay(s) are configured via RRC signaling

o    Option h:  Application delay applies after drx-RetransmissionTimerUL expires

o    Option i: Leave up to implementation

o    Option j: UE applies the skipping immediately (e.g. next symbol/slot) after the UE receives the indication in DL assignment. If the UE fails to decode the associated PDSCH and transmits a NACK, skipping is canceled in the slots after the NACK transmission. Option g ( application delay configured via RRC signaling) is used for uplink grant. If RRC signaling is not provided, UE applies the skipping immediately (e.g. next symbol/slot) after the UE receives the indication in UL grant.

o    Other options not precluded.

-         FFS reference points for the application delay,

o        e.g., the PDCCH monitoring indication applies at a first slot that is at least  symbols after the last symbol of the PDCCH, where T is defined as application delay

-         FFS whether the same or different and how application delay for PDCCH monitoring adaptation indicated by DCI and timer expiration

-         FFS non-scheduling DCI if supported