3GPP TSG RAN WG1 #107bis-e		R1-2200432

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.15.7

Source:	Apple Inc.

Title:	Views on UE features for Rel-17 UE power saving

Document for:	Discussion/Decision

# Introduction

The latest UE feature list is provided in R1-2112902 after RAN1#107-e discussion. We provide our views on R17 UE power saving features in this contribution.

# R17 UE Power Saving Features

The main open issue is whether to define 29-3c and 29-3d as separate FGs. There are still some design details to be decided for these two features. But on the high level,

- For 29-3c, the switching among 3 SSSGs is more complicated compared to the switching between 2 SSSGs, in terms of e.g. how the timer-based fallback works between the SSSGs.

- For 29-3d, the behavior is also more complicated e.g. in terms of the interaction between SSSG switching and PDCCH skipping.

Therefore, we think 29-3c and 29-3d should be defined as separate FGs.

The table below summarizes our proposal for the UE feature list, which also includes some minor changes for other FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability in paging PDCCH and paging early indication PDCCH |  | N |  |  | Per UE | N | N | N | L1 indication for TRS availability in paging early indication PDCCH is supported only if UE reports the support of FG 29-1. | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |