3GPP TSG RAN WG1#107-bis-e	R1-2200494

e-Meeting, January 17th – 25th, 2022

Source:	Sharp

Title:	Initial access aspects

Agenda Item:	8.2.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide discussions on initial access aspects of supporting NR from 52.6GHz to 71 GHz.

- DRS related aspects

The following agreement was achieved during RAN1 107-e [1]:

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used Agreement - Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - Supported values of NSSBQCL: {16, 32, 64} - Note: - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled - UE is expected to be configured with NSSBQCL=64 in licensed operations - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

An LS for confirming the use of the spare bit in MIB has been sent to RAN2. In a case that RAN2 does not agree to use the spare bit for Q indication, a comprised solution at RAN1 could be indicating Q = {32, 64} or {16, 32} with 1 MIB payload bit (i.e., subCarrierSpacingCommon), but we believe that is too restrictive. Partial Q indication by MIB could be a solution to alleviate the problem. More specifically, assuming 1 MIB payload bit is used for indication of Q, rather than indicating only Q = {32, 64} by codepoints ‘1’ and ‘0’ respectively, the codepoint ‘0’ indicates Q = 64 and the codepoint ‘1’ indicates QMIB = 16 (covering both Q = 16 and Q = 32). That is, only a part of Q information is indicated by MIB and the complete Q information can be indicated by SIB1 or can be obtained by combining QMIB and the remained 1 bit that is indicated by SIB1. Before decoding SIB1, the UE makes QCL assumption based on the indicated QMIB value of 16 for the codepoint ‘1’. For the case that QMIB = 16 but Q = 32 is used by gNB for SSB transmission, the only difference is that the UE might need to perform an extra PDCCH blind decoding, compared to that full Q information is indicated. In this way, values of {16, 32, 64} can be indicated by 1 MIB payload bit, which is obviously better than {32, 64}.

Proposal 1: Use 1 MIB payload bit to partially indicate Q values of {16, 32, 64} and the full indication for Q and DBTW enabled/disabled can be complemented by SIB1. Adopt the following text proposal.

Text proposal for TS38.213 V17.0.0:

4 Synchronization procedures 4.1 Cell search [omitted text] For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. In a case that only subCarrierSpacingCommon is used for NSSBQCL indication, ‘scs15or60’ indicates NSSBQCL={16, 32} and ‘scs30or120’ indicates NSSBQCL=64. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and spare to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; spare; NSSBQCL. scs15or60; 0; 16. scs15or60; 1; 32. scs30or120; 0; 64. scs30or120; 1; reserved. [omitted text]

- Conclusion

In this contribution, we provide the following observations and proposals:

Proposal 1: Use 1 MIB payload bit to partially indicate Q values of {16, 32, 64} and the full indication for Q and DBTW enabled/disabled can be complemented by SIB1. Adopt the following text proposal.

Text proposal for TS38.213 V17.0.0

- References

  - 3GPP RAN1#107-e Chairman’s note, November 2021