3GPP TSG RAN WG1 #107bis-e	R1- 2200502

e-Meeting, January 17th – 25th, 2022

Source:	Sharp

Title:	PUCCH coverage enhancement

Agenda Item:	8.8.2

Document for:	Discussion and Decision

Background

RAN1 specification for Rel-17 CovEnh has been published. We discuss issues for finalization of the work.

List of issues in the current specification

List of issues in the current specification is as the following:

- Inter-slot frequency hopping with DMRS bundling for PUSCH and PUCCH

  - Inter-slot frequency hopping with DMRS bundling

For inter-slot frequency hopping with DMRS bundling, there are remaining issues as the following:

- Enabling/disabling of inter-slot frequency hopping with DMRS bundling

- Configuration of hopping interval

- Hopping pattern determination by physical slot indices

- Both hopping interval and window length L are not configured

38.214 Clause 6.3.1 … In case of inter-slot frequency hopping, and when PUSCH-DMRS-Bundling is not enabled, the starting RB during slot is given by: , Agreement For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured

Hopping pattern determination by physical slot indices

First, we consider FFS point in the above agreement [1], a hopping pattern if a hopping interval H is configured, and UEs with the hopping pattern should be multiplexed independently from starting slots of PUSCH transmissions of the UEs.

Proposal 1: If a hopping interval H is configured, UEs with the hopping pattern should be multiplexed independently from starting slots of PUSCH transmissions of the UEs.

Currently, the hopping pattern is not defined and not specified. Using the hopping interval H, the hopping pattern should be semi-statically determined based on physical slot indices ns,fμ and SFN nf as:

RBstartnf,ns,fμ=RBstart,  &(Nslotframe,μnf+ns,fμ)/H mod 2=0RBstart+RBoffset mod NBWPsize,  &(Nslotframe,μnf+ns,fμ)/H mod 2=1.

If only physical slot index ns,fμ is used to determine the hopping pattern, the frequency is hopped at boundary between two frames and both frequency diversity and DMRS bundling gain are degraded. For example, when H=4 and μ=0, the first hop is used in slots corresponding to ns,f0=0,1,2,3,8,9 and the second hop is used in slots corresponding to ns,f0=4,5,6,7.

Proposal 2: If a hopping interval H is configured, the hopping pattern should be determined as:

RBstartnf,ns,fμ=RBstart,  &(Nslotframe,μnf+ns,fμ)/H mod 2=0RBstart+RBoffset mod NBWPsize,  &(Nslotframe,μnf+ns,fμ)/H mod 2=1.

Configuration of hopping interval

Configuration of hopping interval is not currently specified. In our view, it is reasonable that hopping interval is configured per UL BWP and dynamic adaptation is unnecessary.

Proposal 3: Hopping interval for PUCCH should be configured per UL BWP.

Enabling/disabling of inter-slot frequency hopping with DMRS bundling

In the current specification, the legacy inter-slot frequency hopping for PUSCH is performed when PUSCH-DMRS-Bundling is not enabled. However, how to enable the new inter-slot frequency hopping with DMRS bundling is not specified.

Considering RRC configuration impacts, we suggest to reuse frequencyHopping, frequencyHoppingDCI-0-2, and frequencyHoppingDCI-0-1 in PUSCH-Config. When frequency hopping scheme is indicated as interSlot and PUSCH-DMRS-bundling is enabled, the UE should determine to perform the new inter-slot frequency hopping with DMRS bundling.

Proposal 4: When frequency hopping scheme is indicated as interSlot and PUSCH-DMRS-Bundling is enabled, the UE should determine to perform the new inter-slot frequency hopping with DMRS bundling.

When the frequency hopping scheme is indicated as intraSlot and PUSCH-DMRS-bundling is enabled, the UE should determine to perform the intra-slot frequency hopping to not reduce flexibility of Rel-15/16.

Proposal 5: When the frequency hopping scheme is indicated as intraSlot and PUSCH-DMRS-Bundling is enabled, the UE should determine to perform the intra-slot frequency hopping.

Both hopping interval and window length L are not configured

Although almost hopping interval determinations were agreed and specified as shown in Table 1 [1], a case that both hopping interval and window length L are not configured is FFS.

Table 1: Hopping interval determination

| Enabling of DMRS bundling | Window length L | Hopping interval configuration (H) | Hopping interval (Slot) |  |
| --- | --- | --- | --- | --- |
| Y | Y | Y | H | Agreement Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured |
| Y | Y | N | L | Agreement if hopping interval is not configured, the default hopping interval is the same as the configured TDW length |
| Y | N | Y | H | Agreement A Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured |
| Y | N | N | TBD | FFS: if both hopping interval and TDW length are not configured |
| N | - | - | 1 | Current spec |

When DMRS bundling is enabled, the frequency hopping scheme is indicated as interSlot. and both hopping interval and window length L are not configured, the configured TDW length L’ is equal to min(maximum duration, all repetition duration). However, if the default hopping interval is also equal to the configured TDW, frequency hopping is potentially not performed during the all repetition duration. For this reason, we propose that when hopping interval and window length L are not configured, a half of the configured TDW (i.e., L'/2) should be equal to the default hopping interval to achieve both frequency hopping gain and DMRS bundling gain.

Proposal 6: When hopping interval and window length L are not configured, a half of the configured TDW should be equal to the default hopping interval to achieve both frequency hopping gain and DMRS bundling gain.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: If a hopping interval H is configured, UEs with the hopping pattern should be multiplexed independently from starting slots of PUSCH transmissions of the UEs.

Proposal 2: If a hopping interval H is configured, the hopping pattern should be determined as:

RBstartnf,ns,fμ=RBstart,  &(Nslotframe,μnf+ns,fμ)/H mod 2=0RBstart+RBoffset mod NBWPsize,  &(Nslotframe,μnf+ns,fμ)/H mod 2=1.

Proposal 3: Hopping interval for PUCCH should be configured per UL BWP.

Proposal 4: When frequency hopping scheme is indicated as interSlot and PUSCH-DMRS-Bundling is enabled, the UE should determine to perform the new inter-slot frequency hopping with DMRS bundling.

Proposal 5: When the frequency hopping scheme is indicated as intraSlot and PUSCH-DMRS-Bundling is enabled, the UE should determine to perform the intra-slot frequency hopping.

Proposal 6: When hopping interval and window length L are not configured, a half of the configured TDW should be equal to the default hopping interval to achieve both frequency hopping gain and DMRS bundling gain.

References

- [1]	3GPP, RAN1#107-e, RAN1 Chairman’s Notes, Nov. 2021.