3GPP TSG RAN WG1 Meeting#107bis-e		R1-2200526

e-Meeting, January 17th – January 25th, 2022

Agenda item:	8.12.2

Source:	TD Tech, Chengdu TD Tech

Title:	PUCCH formats for enhancement to NACK-ONLY based HARQ-ACK feedback

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. At present, the L1 related CR on HARQ-ACK feedback for NR MBS was approved with PUCCH format 0/1 for the NACK-ONLY based HARQ-ACK feedback. In order to improve the PUCCH reception performance, PUCCH formats 2-4 are suggested in the contribution for the enhancement to the NACK-ONLY based HARQ-ACK feedback.

- PUCCH formats for enhancement to NACK-only based HARQ-ACK feedback

There are 5 PUCCH formats: PUCCH formats 0 through 4. PUCCH format 0/1 can carry 1 or 2 bit while PUCCH format 2/3/4 can carry more than 2 bits. PUCCH format 2/3/4 has better decoding performance than PUCCH format 0 and PUCCH format 1. In other words, PUCCH format 2/3/4 has better uplink coverage than PUCCH format 0 and PUCCH format 1.

Because the NACK information for NACK-only based HARQ-ACK feedback consists of only 1 bit, therefore PUCCH format 0 and PUCCH format 1 are used to carry NACK information. But in order to provide better uplink coverage for PUCCH carrying the NACK information，PUCCH format 2/3/4 can be used to carry the NACK information according to the following two steps:

- Step 1: Map the NACK information of 1 bit long onto a bit sequence of N>2 bits long, where the sequence can be an N-bits long random sequence.

- Step 2: UE sends the bit sequence of N bits long with PUCCH format 2/3/4.

For each MBS session with NACK-only based HARQ-ACK feedback, N can be configured as a part of the configuration information of the MBS session. According to the configuration information of the MBS session, UE can map the NACK information of 1 bit long onto a bit sequence of N bits long. Then UE can send the sequence with PUCCH format 2/3/4 to extend uplink coverage of NACK-only based HARQ-ACK feedback.

Based on the above discussion, the following proposals are suggested.

Proposal 1: Support PUCCH format 2/3/4 for NACK-only based HARQ-ACK feedback.

Proposal 2: If PUCCH format 2/3/4 is used to feed back the NACK-ONLY information for an MBS session, N is configured as a part of the configuration information of the MBS session. UE maps the NACK information of 1 bit long onto a sequence of N>2 bits long. Then send the NACK information with PUCCH format 2/3/4.

- Conclusion

Based on the discussion in the above section, the following proposals are suggested.

Proposal 1: Support PUCCH format 2/3/4 for NACK-only based HARQ-ACK feedback.

Proposal 2: If PUCCH format 2/3/4 is used to feed back the NACK-ONLY information for an MBS session, N is configured as a part of the configuration information of the MBS session. UE maps the NACK information of 1 bit long onto a sequence of N>2 bits long. Then send the NACK information with PUCCH format 2/3/4.

- Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e

- R1-2101637, Study on the reliability for RRC_CONNECTED UEs, Chengdu TD Tech, TD Tech, RAN1 Meeting #104-e

- R1-2107951, Reliability related discussion for RRC_CONNECTED UEs, Chengdu TD Tech, TD Tech, RAN1 Meeting #106-e