3GPP TSG RAN WG1 Meeting #107bis-e		R1-2200557

e-Meeting, January 17th – 25th, 2022

Agenda Item: 8.15.3

Source: MediaTek Inc.

Title: Views on UE features for enhanced IIoT and URLLC

Document for: Discussion and Decision

# Introduction

This paper provides our views on UE features for enhanced IIoT/URLLC [1]. Detailed proposals are described in the following section.

# Discussion

Regarding FG25-18, the support of parallel PUCCH and PUSCH transmission across CCs in inter-band CA should be reported per band combination.

| Index | Feature group | Components | Prerequisite FG | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Per BC |  | Optional |

- Change the Type of FG25-18 from “Per UE” to “Per BC”.

PUCCH cell switching (FG25-9 & FG25-10): PUCCH cell switching feature should have UL CA as Prerequisite FG. Also, the type for FG25-9 & FG25-10 should be per band combination. Given the different required UE implementation complexities, the PUCCH cell switching should be reported separately for the case where the carriers have different numerologies.

| Index | Feature group | Components | Prerequisite FG | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 25-9 | Semi-static PUCCH cell switching |  | FG6-6 | Per BC |  | Optional |
| 25-10 | PUCCH cell switching based on dynamic indication |  | FG6-6 | Per BC |  | Optional |

- Change the Type of FG25-9 & FG25-10 from “Per UE” to “Per BC”, and add FG6-6 as “Prerequisite FG” for these two FGs.

- FG25-9 & FG25-10 are reported separately for the case where the carriers have different numerologies.

# Conclusion

In this contribution, we provided our views on UE features for enhanced IIoT/URLLC and we have the following proposals:

- Change the Type of FG25-18 from “Per UE” to “Per BC”.

- Change the Type of FG25-9 & FG25-10 from “Per UE” to “Per BC”, and add FG6-6 as “Prerequisite FG” for these two FGs.

- FG25-9 & FG25-10 are reported separately for the case where the carriers have different numerologies.

# References

- R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, Moderators (AT&T, DOCOMO), RAN1#107-e, November 2021.