3GPP TSG RAN WG1 #107bis	R1-2200622

e-Meeting, 17 – 25 January, 2022

Agenda item:		8.16.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for further enhancements on NR-MIMO

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined [1, 2]. In this contribution we present our views on the UE features for further enhancements on NR-MIMO.

# Discussion on unified TCI framework

One major controversial point on FeMIMO UE features is the arrangement of FGs related to unified TCI framework. While RAN1#107-e managed to agree on a few components that are supposed to be mapped to one or more FGs, there is no agreement on the baseline structure of such FGs. The following agreement has been reached on the components of unified TCI states:

Agreement:

- The following are the components of the feature “joint TCI for intra-cell operation”

- Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI]  activation assignment)

- Common cross multi-CC TCI update and activation (involving RRC common TCI state pool)

- Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS]

- Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS

- [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]

- [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC]

- [Maximum number of CCs configured with BFR]

- [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH]

- [Configuration of both R17 TCI and R15/16 TCI and spatial relation]

- The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band

- [The maximum number of configured joint TCI states across all BWPs and all CCs in a band]

- [The maximum number of configured joint TCI states per CC [in a band]]

- [Support The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]]

- [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)]

- [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]

- [The minimum time gap between the beam indication PDCCH and first slot where beam is applied]

- This list gives no guidance as to how to group these components into FGs. They may be mapped to one or more FGs and one or more of these FGs may be agreed as basic. Specifically, different FGs can be grouped together through notes “a UE supporting this FG must also support FG 23-x-y”. This allows to create “basic” FGs while allowing, e.g., different types for different components/candidate values.

- The purpose of this list is to find the individual components without having to worry about the signalling structure

Before getting into the details of the components listed above, it is useful to have a short review on what are the main elements of unified TCI framework, so that one can ensure that the FGs are meaningful and correspond to implementable features in the end, both in UE and network sides.

Basic unified TCI state operation is described below, and illustrated in Figure 1:

- RRC configures set of joint and/or separate TCI states

  - Different needs, e.g. to address MPE issue, beam resource allocation flexibility at gNB

- MAC activation of up to 8 joint or separate TCI states

- DCI indicates one of the activated TCI states/codepoints to be the indicated TCI state ( = common TCI state)

  - Before first indication, the first activated TCI state is the current indicated TCI state

  - DCI format 1_1/1_2 with and without DL assignment

  - Indication confirmed by HARQ-ACK by UE

  - Application time of the beam indication

  - One beam application time (BAT) for a given SCS

Figure 1: Basic operation of unified TCI states

Another essential functionality of unified TCI states is the support for common cross-CC TCI update, described briefly below and exemplified in Figure 2:

- Two cases

  - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP/CC as in Rel-15/16

    - Source RS in each CC to provide a reference signal for the quasi-colocation for DM-RS of PDSCH, DM-RS of PDCCH and CSI-RS in the CC, and a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and SRS in the CC is associated to source RS of the indicated TCI-State

  - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC

    - Source RS of the indicated TCI-State provides a reference signal for the quasi-colocation for DM-RS of PDSCH, DM-RS of PDCCH and CSI-RS in the set of configured CCs, and a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and SRS in the set of configured CCs

(a)

(b)

Figure 2: Cross-CC TCI state update: (a) RRC-configured TCI state pool(s) for each BWP/CC and (b) RRC-configured TCI state pool(s) in a reference BWP/CC.

Finally, UL power control is another key aspect of unified TCI state operation, briefly described below:

- A PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state of (if applicable) joint TCI state

- For each of PUSCH and PUCCH and SRS, the UL control parameters can be associated with UL TCI state or (if applicable) joint TCI state per BWP

In addition, while it is true that joint TCI state update provides optimized signaling and room for exta TCI states, all use cases for joint TCI state updates can be provided by separate TCI state updates directly. Hence, separate TCI states should be considered as a baseline.

Based on the discussion above, the following mapping can be considered for the essential unified TCI state FGs for joint DL/UL TCI update:

| Index | Feature group | Components | Pre-requisites | Notes |
| --- | --- | --- | --- | --- |
| 23-1-1 | Unified TCI for intra-cell beam management with joint DL/UL TCI update | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE -based TCI activation - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH - The maximum number of configured joint TCI states per CC [in a band] - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) - The maximum number of MAC-CE activated joint TCI states [per BWP per CC | 23-1-1c | Separate TCI state update needs to be supported by a UE supporting FG 23-1-1. Note: Components 2, 3, 5, 6, 7 are common with 23-1-1c (for separate TCI states), and they do not need to be repeated here if 23-1-1c is a pre-requisite to 23-1-1 |
| 23-1-1a | Multi-CC unified TCI state operation with joint DL/UL TCI update and separate TCI state for each BWP/CC | - Common cross multi-CC TCI update and activation - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band | 23-1-1 | Note: Must be supported if UE supports 23-1-1 and more than 1 CC/BWP |
| 23-1-1b | Multi-CC unified TCI state operation with joint DL/UL TCI update and reference BWP/CC | - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - The maximum number of configured joint TCI state pools for configured reference BWP/CC shared by a set of BWP/CC across all BWPs and all CCs in a band | 23-1-1a |  |

For separate UL/DL TCI state update, the following principles should be followed:

- Maximise utilization of FGs defined for joint TCI update case as pre-requisites where applicable

- Support at least one DL and one UL TCI state needs to be ensured if UE supports separate TCI state updates

| Index | Feature group | Components | Pre-requisites | Notes |
| --- | --- | --- | --- | --- |
| 23-1-1c | Unified TCI for intra-cell beam management with separate DL/UL TCI update | - 1. Support of separate DL/UL TCI update (configuration mechanism, applicable source and target signals, including non-serving SSB as direct/indirect QCL source) Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - The maximum number of configured separate TCI states per CC [in a band]. - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) - The maximum number of MAC-CE activated TCI states [per BWP per CC], where at least 1 DL and 1 UL TCI activated states need to be supported [per BWP per CC] |  | Separate TCI state update needs to be supported by a UE supporting FG 23-1-1. |
| 23-1-1d | Multi-CC unified TCI state operation with separate DL/UL TCI update and separate TCI state for each BWP/CC | - Common cross multi-CC TCI update and activation - The maximum number of configured separate TCI state pools across all BWPs and all CCs in a band | 23-1-1c | Note: Must be supported if UE supports 23-1-1c and more than 1 CC/BWP |
| 23-1-1e | Multi-CC unified TCI state operation with separate DL/UL TCI update and reference BWP/CC | - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - The maximum number of configured separate TCI state pools for configured reference BWP/CC shared by a set of BWP/CC across all BWPs and all CCs in a band | 23-1-1d |  |

Finally, for inter-cell TCI state update, the structures above can be used as baseline one they are stable. In addition the following principles need to be followed for better organization of the FGs and to avoid complexity on network and UEs alike:

- Intra-cell operation be pre-requisite to inter-cell, as it is not sensible to imagine UEs that can support Rel-17 TCI state updates only for inter-cell scenarios.

- Counting to be done consistently for both intra- and inter-cell cases, e.g. N TCI states type of features

One specific aspect of inter-cell TCI state updates are the related measurements. Based on discussions in RAN1#107-e, the following candidate components can be considered further:

| Potential components based on RAN1#107-e discussions | Comments |
| --- | --- |
| - [The maximum number of PCI(s) different from serving cell PCI that can be configured for measurement] | Needed, but there is little value in too fine granularity of values. Potential range {1, 3, 7}. |
| - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR1] | No clear need for this measurement |
| - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR2] | No clear need for this measurement |
| - [ [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI] | Should be part of a basic FG. Can be merged with 15. |
| - [ [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance]] | Should be part of a basic FG. Can be merged with 14. |
| - The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states | Needed |

# Discussion on features for multi-TRP operation

UE FGs for multi-TRP have not been discussed in RAN1#107-e, and for those there is no baseline available currently either. The following structure can be further considered for Multi-TRP PUSCH repetition (type A) FGs, where the original component numbers from [2] for easier reference:

| Index | Feature group | Components | Pre-requisites | Notes |
| --- | --- | --- | --- | --- |
| 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 6. Support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission |  |  |
| 23-3-1a | PHR reporting related to M-TRP PUSCH repetition | 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) | 23-3-1 |  |
| 23-3-1b | CG PUSCH transmission for M-TRP | 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) | 23-3-1 |  |
| 23-3-1c | A-CSI report for PUSCH repetitions | 8. Support of A-CSI report on two PUSCH repetitions | 23-3-1 |  |
| 23-3-1d | SP-CSI report for PUSCH repetitions | 9. Support of SP-CSI report on two PUSCH repetitions | 23-3-1 |  |

Note that component “Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources]” might need its own FG as well, but current formulation is too vague and imprecise. More elaboration is needed before it can be further considered.

The following structure can be further considered for Multi-TRP PUCCH repetition FGs, where the original component numbers from [2] for easier reference:

| Index | Feature group | Components | Pre-requisites | Notes |
| --- | --- | --- | --- | --- |
| 23-3-2 | Multi-TRP inter-slot PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |  |  |
| 23-3-2a | Multi-TRP intra-slot PUCCH repetition | 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |  |  |

# Discussion on other features

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107-e are available in the Annex.

  - 23-1-3:

    - Component 2 candidate values: It was agreed that UE can report up to 4 pairs of P-MPR and SSBRI/CRI values

    - Component 3 candidate values: In order to enable the UE to find an new candidate beam, the pool of configured RS should be as large as possible to enable UE to find new candidate beams, i.e. 64 RS.

  - 23-1-4:

    - Confirm the FG. RAN1 already agreed that UE reports a list of UE capability value sets, the correspondence between each reported CSI-RS and/or SSB resource index and one of the UE capability value sets in the reported list is determined by the UE (analogous to Rel-15/16) and is informed to NW in a beam reporting instance.

- 23-2-1:

    - No independent signaling for component 1, as it is not allowed to individually disable components of an FG. No need to separate the FG though, support to component 1 is given by support to the FG itself.

    - Component 3 is not needed, these relations are expected to be described in specifications instead

- 23-2-3:

    - Confirm the FG

  - 23-3-1-1:

    - Confirm the FG. Details to be further discussed based on structure of 23-3-1.

  - 23-4:

  - 23-5-1:

    - Component 1 and 5 essential components.

    - Component 2 is redundant or equivalent to component 1+5.

    - Components 3, 4, and 6 are OK.

  - 23-5-2:

    - Component 8 is a key feature for multi-DCI case. Single-DCI can be captured as a separate FG.

  - 23-6-1:

    - Confirm the FG. Details to be further discussed.

  - 23-6-2:

    - Confirm the FG. Details to be further discussed.

  - 23-6-3:

    - Confirm the FG. Details to be further discussed.

  - 23-6-4:

    - Confirm the FG.

    - Components 4-6 can be confirmed.

- 23-7-1:

    - Component 1: A basic configuration for a MTRP CSI resource set always requires N=1 Resource Pair, so the reference to Nmax can be dropped. The component can be described as “Support of N=1 Resource Pair in an NZP CSI-RS Resource Set for channel measurement” (and the note on NCJT can be removed as it becomes obsolete).

    - Component 2: it was agreed to discuss the default value of Ks, max with UE capability. In our view the minimum value should be Ks,max=4 because, if sharedCMR is absent and N = 1, M1 = K1 - 1 and M2 = K2 – 1 (see R1-2112472, Sec. 6.3.1.1.2), hence K1≥2, K2≥2 and the minimum value of Ks=K1+K2=4

Agreement(RAN1#105-e)

For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks,max,

- Alt 1: Ks,max = 4

- Alt 2: Ks,max = 2

- Alt 3: Ks,max = 4 for FR2, and Ks,max = 2 for FR1

- Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature.

Conclusion: (RAN1#106-e)

Default value of Ks, max can be discussed later with Rel-17 MIMO UE capability.

- New Component 3: ”Support of two Resource Groups in an NZP CSI-RS Resource Set for channel measurement, with K1≥1 resources in Group 1 and K2≥1 resources in Group 2”. Candidate values for K1 and K2: {1,2,3,4,5,6,7}

- New Component 4: ”Support of csi-ReportMode 1 and 2”

- New Component 5: ”Support of numberOfSingleTRP-CSI-Mode1 X=0”

- 23-7-2:

    - Confirm the FG. Details to be further discussed.

    - Propose to rename to “Support of more than 16 Tx ports in one resource”. Based on the following agreement, up to 16 Tx ports per resource (i.e. 32 across a Resource Pair) should be supported as part of basic capability

Agreement(RAN1#103-e)

- The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

  - 23-7-3:

    - Confirm the FG. Details to be further discussed.

    - Propose to rename to: “Maximum number of resources Ks,max” with description: “Support of up to Ks,max resources in an NZP CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs”. Candidate values should be {4,5,6,7,8}.

  - 23-9-2:

    - It is ok to have a separate list of triplets for M=2.

    - Component 3 regarding the window size N is not needed.

    - Typo in the FG name where Mν should be replaced by M

  - 23-9-3:

    - Separate capabilities to support rank 3,4 with M=1 and M=2 are not needed

  - 23-9-4:

    - No need for a separate list of triplets as those indicated for M=2 can be supported for both R=1 and R=2.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for further enhancements on NR-MIMO, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107-e, Nov. 2021

- R1-2112902, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, Moderators (AT&T, NTT DOCOMO, INC.), Nov. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] |  |  |  |  | [per band] |  |  |  | [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values FFS 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] | [2-24, 2-29] |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  |  |  |  |  |  |  |  | 2. Candidate value of {1,2,3, 4} 3. Candidate valueFFS | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  |  |  |  | FR2 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP] |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement] |  |  |  |  |  |  |  |  | [Candidate values: {1, 2,…}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. [Support of scheme B for PDCCH] 2. Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  |  |  | [FR1 only] |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 |  |  |  | Per UE |  | Yes |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for HST] | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) | FFS |  |  |  | [Per band / per BC] |  |  |  | [Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 |  |  |  | Per band |  | FR2 only |  | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | Per BC |  |  |  | Component 1 candidate values: FFS | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  |  |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] | 23-8-6 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=1 and R=1 - Support rank 1,2 - Support parameter combinations with M=1 | 2-35 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1] |  |  |  | FFS |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N] | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] | 23-9-2 |  |  |  | [per band and per BC] |  |  |  |  | Optional with capability signalling |