3GPP TSG RAN WG1 #107bis-e                                             R1-2200713

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.8.3

Title: 	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3

Source: 	Moderator (ZTE Corporation)

Document for:	Discussion and Decision

- Introduction

This contribution provides a summary of remaining issues on Type A PUSCH repetitions for Msg3 proposed in contributions submitted under AI 8.8.3 and AI 8.8.4.

- Summary of Tdocs

  - Indication of MCS index for Msg3 re-transmission with repetitions

    - [Open] Issue #1: Indication of MCS index for Msg3 re-transmission

In RAN1#107-e, it was agreed to use MCS information filed for repetition indication of Msg3 repetition. For Msg3 re-transmission, the repetition factor is indicated the same way as initial transmission, i.e., using 2 MSB bits of the MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI for selecting one value from the four candidate values configured by SIB1. However, how to indicate the MCS index for re-transmission is not concluded yet.

Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Agreement - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission

Companies’ views on indication of MCS index for Msg3 re-transmission are summarized below.

- Option 1: The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission.

  - Option 1-A: The 8 candidate MCS indexes are MCS 0~7.

    - Support: [1, Huawei, HiSilicon], [3, ZTE], [5, Nokia, Nokia Shanghai Bell], [7, NTT DOCOMO], [13, xiaomi], [14, 	China Telecom], [15, Sharp], [19, WILUS]

  - Option 1-B: The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.

    - Support: [6, Samsung], [10, OPPO], [11, Intel], [17, CMCC], [18, LG]

  - Option 1-C: The first four candidate MCS indexes are the ones configured by SIB for initial transmission. If this configuration is absent, MCS 0~3 are used; Additional four candidate MCS indexes configured by SIB1 are used for retransmission. If this configuration is absent, MCS 4~7 are used.

    - [9, Panasonic ], [16, 	InterDigital]

- Option 2: The same set of MCS indexes for Msg3 initial transmission is adopted for Msg3 retransmission. LSB 2 bits of MCS information field in DCI format 0_0 scrambled with TC-RNTI are used to indicate MCS configuration.

  - Support: [2, vivo], [4, CATT]

  - [4, CATT]: For the remaining 4 states of 3 LSB bits of MCS bit field, X=3 states are corresponding to the last 3 lines in Table 5.1.3.1-1 when transform precoding is disabled. X=4 states are corresponding to the last 4 lines in Table 6.1.4.1-1 when transform precoding is enabled.

In addition, [22, Nokia] provides some evaluation results and have the following observation:

- Restricting the MCS index configuration to indices from MCS#0 and MCS#7 does not yield deterministic and significant performance loss/increase as compared to higher MCS indices counterparts (it depends on the considered configuration).

First round

For above options, FL understanding is provided below.

- Option 1-A is simpler and has no additional RRC impact.

- Theoretically, Option 1-B/1-C can provide better performance. However, it seems MCS index larger than 7 would not be typically used based on the evaluation in RAN1#107-e. In addition, Option 1-B/1-C has additional RRC impacts, which should be avoided if possible in maintenance phase.

  - The main difference between Option 1-B and 1-C is whether to use one RRC parameter or two RRC parameters for configuring the candidate MCS indexes.

  - If no consensus can be reached for the additional RRC impact of Option 1-B/1-C, Option 1-A would be the default option as MCS 0~7 are also proposed to use if no RRC configuration is present for Option 1-B/1-C.

- For Option 2, the restriction of using the same set of MCS indexes between Msg3 initial and re-transmission seems not necessary. Note that, the previous agreements only states to use the same mechanism as supported for Msg3 initial transmission for repetition indication of Msg3 re-transmission (i.e., no restriction on MCS indication).

With above analysis, it seems more reasonable to adopt Option 1-A. However, for the first round of discussion, FL would like to collect companies’ views first.

| Company | Comments |
| --- | --- |
| Samsung | If we start from zero, we would agree FL’s comments that it is reasonable to adopt MCS 0~7, as we originally think MCS 0~3 should be enough, however, it’s However, we are not. Given the fact that initial msg3 transmission scheduled by RAR UL grant can use SIB1 configured 4 values; we did not see why we need a different option for msg3 retransmission, which purely use default 0~7 MCS values. |
| Intel | We prefer Option 1-B. We share similar view as Samsung, and Option 1-B can provide certain flexibility on the MCS for Msg3 retransmission and also reduce the signaling overhead. |
| Nokia/NSB | We prefer Option 1-A and have similar opinion as the one expressed by FL on the impact of this decision on RRC parameters. We do not see a strong use case for the introduction of a new parameter. On the other hand, we have no technical objection to Option 1-B, if this is what the group wishes to agree on. |
| Sharp | We prefer Option 1-A. In maintenance phase, whether a proposed change is essential or not should be discussed. In the current specification, it clearly states that no RRC configured MCS sets are applied for msg3 retransmission. We don’t think proposed changes in options than Option 1-A is not essential. Further, technically we have the following observation. Option 1-B/C causes increased SIB1 overhead. Option 2 has less flexibility. 8 code points should be utilized. |
| Panasonic | We support the above FL analysis. We are also fine with Option 1-A. |
| OPPO | We share similar views as Samsung and Intel. SIB1 configured 4 values can be used for Msg3 initial transmission for flexibility. We wonder why the similar mechanism can not be used for Msg3 re-transmission. For option 1-B, there is still only one RRC parameter, which has no additional RRC impacts. For the SIB1 overhead, the overhead of RRC parameter indicating 4 and 8 MCS values depends on how to indicate the MCS values from the MCS table. It is not certainly that option 1-B has increased SIB1 overhead. |
| Qualcomm | We prefer Option 1-A. |
| Apple | Tend to support 1-A (similar view as FL) |
| vivo1 | We prefer option 2, there’s no need to use a different set of MCS values for initial and retransmission of Msg3. |
| LG | We prefer to configure candidate values of MCS for re-transmission of msg3 PUSCH repetition via by SIB1 because this is aligned with previous agreement for initial transmission of msg3 PUSCH repetition. But, if majority companies are supporting to use only default set of MCS for re-transmission (i.e., option 1-A), we can be flexible to support option 1-A for progress. |
| Ericsson | 1-B is preferred. The SIB1 parameter of the candidate MCS indexes for initial transmission and retransmission of Msg3 can be configured as a sequence of 4 or 8 values as follows. mcs-Msg3repetition CHOICE { mcs-4values SEQUENCE (SIZE (4)) OF MCS mcs-8values SEQUENCE (SIZE (8)) OF MCS } The CHOICE type means there is no need to configure both mcs-4values and mcs-8values. If only a sequence of 4 MCS indexes is configured, the default MCS indexes are used for Msg3 retransmission, which is similar to Option 1-A. But 1-B allows the possible configuration of 8 MCS indexes, where the first four values can be used for Msg3 initial transmission, and all indexes are for Msg3 retransmission. |
| WILUS | We prefer Option 1-A and support the FL’s analysis. Additional flexibility other than MCS 0~7 is not necessary for CE UE. |
| CATT | To clarify, we think the current logic of initial transmission can be followed: (1) If SIB1 explicitly configures MCS indices, use them; (2) If not configured, use predefined default MCS indices values. As for re-transmission, it seems natural that (1) SIB1 can explicitly configure (4 or) 8 MCS indices for retransmission. (2) If not configured, predefined default MCS indices are used. Hence, Option 1-B, 1-C and Option 2 are fine to us. For default MCS indices, it is important to include last 3 or 4 MCS indices (depends on waveform and the corresponding MCS table) in current MCS table. In this case, TBS is the same with initial transmission, and only modulation order is indicated. Code rate is automatically adjusted. This is already supported since Rel-15, which can provide more flexible time-frequency resource allocation for gNB to handle. Otherwise, the gNB can only use very limited time-frequency resource combinations to guarantee the same TBS. FL: For the reserved rows, FL’s understanding is it can only be used in case of the latest PDCCH for the same transport block using . It means it can only be used for the re-transmission of Msg3 re-transmission. Even if more than 1 re-transmission happens, gNB can still choose to use low MCS index. So, it seems not typical to use the reserved rows for Msg3. |
| China Telecom | We prefer Option 1-A. |
| NTT DOCOMO | Prefer Option 1-A. Unless any benefit using MCS index larger than 7 is observed in terms of coverage performance, we prefer the way to avoid any impacts on RRC parameters. |
| CMCC | Both Option 1-A and 1-B are acceptable for us. The Option 1-A makes the procedure much easier and does not add more bits in SIB1 which means better performance in coverage. But the Option 1-B is similar with what we have agreed for initial transmission, and the flexibility may take effort when a larger payload transmit on Msg3. |
| Spreadtrum | We prefer Option 1-A. |
| Xiaomi | We share the similar view as FL and Nokia. The default MCS 0~7 is enough according to simulation results given by Nokia and we can’t see the need to configure 8 candidate values which will increase the overhead of SIB1. For Option 1-B, the MCS configuration of initial transmission and re-transmission is bound. And, if 8 MCS candidate values for re-transmission are not configured, then a higher MCS level than MCS 3 can’t be obtained for initial transmission, which is against the original intention of our design. FL: Agree that Option 1-B will bundle initial and re-transmission together and have the issue you pointed out. For Option 1-C, an example is given below: Assume the first four MCS candidate indexes are {1, 4, 7, 10} configured by SIB1, and no additional four candidate values are given by SIB1. Then, available 8 MCS indexes for re-transmission are {1, 4, 7, 10, 4, 5, 6, 7} {1, 4, 5, 6, 7, 10}, i.e., the value range of initial transmission and re-transmission may be the same and 3 bits seem too many for the indication of the MCS field for re-transmission… FL: Even your example, 3 bits are still needed. Thus, we propose using only 2 bits to indicate the MCS field for re-transmission can also be considered, just the same way as initial transmission. Besides, we are also fine with Option 1-A. |
| ZTE | Option 1-A is preferred while we are also ok with Option 1-B. |
| Huawei, HiSilicon | We prefer Option 1-A. |
| FL | @CATT, Xiaomi, Please find my reply inline above. The current status is summarized below. Options; Support; Can live with; Pros&Cons. Option 1-A; Nokia/NSB, Sharp, Panasonic, Qualcomm, Apple, WILUS, China Telecom, NTT DOCOMO, CMCC, Spreadtrum, ZTE, Huawei/HiSilicon; LG; Simpler without additional RRC impact; A bit less flexible.. Option 1-B; Samsung, Intel, OPPO, LG, Ericsson, CATT, CMCC; Nokia/NSB, ZTE; Better flexibility while require additional RRC impact and cause more SIB1 overhead.. Option 2; vivo, CATT, Xiaomi. For the upcoming GTW, FL suggests to go with Option 1-A, with also providing Option 1-B as one alternative. Proposal for Issue#1: The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission. - Option 1-A: The 8 candidate MCS indexes are MCS 0~7. - Support/Can live with: Nokia/NSB, Sharp, Panasonic, Qualcomm, Apple, WILUS, China Telecom, NTT DOCOMO, CMCC, Spreadtrum, ZTE, Huawei/HiSilicon, LG - [Option 1-B: The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.] - Support/Can live with: Samsung, Intel, OPPO, LG, Ericsson, CATT, CMCC, Nokia/NSB, ZTE |

Second round

Based on the input in the first round, it seems no consensus to introduce a solution with additional RRC impact. In such case, FL thinks the only way we can go is Option 1-A.

- If no consensus can be reached for the additional RRC impact of Option 1-B/1-C, Option 1-A would be the default option as MCS 0~7 are also proposed to use if the RRC configuration is not present for Option 1-B/1-C.

So, FL would like to check whether companies could live with the following proposal. Please comment only if you have strong concerns.

Proposal for Issue#1: The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission.

- The 8 candidate MCS indexes are MCS 0~7.

| Company | Comments |
| --- | --- |
| Intel | We do not support the proposal. Although we understand the intention, it is not clear to us why same mechanism cannot be defined for Msg3 initial transmission. The same level of flexibility should be introduced for Msg3 retransmission. We still support Option 1-B. |
| vivo2 | According to the discussions in the first round, we do not see reasonable technical justification of using a higher modulation order or target coding rate for the retransmission compared to initial transmission of Msg3 when Msg3 is already requested to be repeated, in which case same or lower MCS should be used by retransmission of Msg3 in our understanding. With that, we still prefer option 2 to use same MCS value set for initial and retransmission of Msg3 when repeated. FL: If Option 1-A is chosen, it doesn’t mean re-transmission will use higher modulation order larger than initial transmission, for which the MCS is configurable and could be large. |
| Samsung | This discussion situation is amusing. In the last RAN1 meeting, there is strong preference (many simulations even are presented) intending to keep 4bits MCS and not repurpose half of it to be 2bits. It leads to 2LSBs are indicating 4 MCS index configured by gNB to provide a way for using higher MCS index. Now it comes to msg3 repetition, suddenly there is strong preference to keep using default 0~7. Of course, one can say, 0~7 is enough. But who has really approved that 0~3 is not enough at the beginning. FL: I think you have already pointed out why the situation is ‘amusing’ ^_^. The situation is different for Msg3 re-transission as it has 3 bits left for MCS indication. If you check previous simulation assumption (e.g., summarized in R1-2112023), you can find the typical MCS indexes used for simulation is not larger than 7. That’s the majority think MCS0~7 is sufficient for re-transmission. Besides, the FL’s comment already put the no-lose ground for option 1-A, because the option 1-A is regarded as “default” one. Regarding the proposal: 1. we share view as Intel, same mechanism should apply, so we support option 1-B; 2. we did not agree the comments on the option 1-A to be default. The default behavior is that, still the 2LSB in the among MCS bit filed to indicate the 4 values configured by the gNB. That means for DCI scheduled msg3 retransmission, the middle bit will be ignored. But this is the same behavior for MCS reading for msg3 initial and re-transmission. FL: For re-transmission, we only agreed to use 2 MSB bits for repetition indication. It means the 3 LSB bits are left for indication of MCS. Note that we didn’t agree to use the same mechanism for MCS indication between initial transmission and re-transmission. |
| Ericsson | We still prefer Option 1-B. As mentioned in the first round, the SIB1 parameter, mcs-Msg3repetition, can have a valid length of 4 or 8 MCS indexes. If the configuration of 8 candidate MCS indexes is absent, it could be either the parameter is absent in SIB1, or the parameter has a sequence of 4 candidate MCS indexes. Therefore, we propose the following change in red. - [Option 1-B: The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration of the 8 candidate MCS indexes is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.] One SIB1 parameter for both Msg3 initial transmission and retransmission saves signaling overhead. It is up to gNB to configure 4 or 8 candidate MCS indexes in SIB1. If the gNB thinks no need to configure high MCS index, a configuration of 4 values enables the gNB to use the default MCS indexes for Msg3 retransmission. FL: I can understand your point, while I am afraid your proposed configuration method is not aligned with other proponents of Option 1-B. That is, if mcs-Msg3repetition is configured, it will always configure 8 candidate values and the first 4 are for initial transmission. |
| CATT | We do not support this proposal. Regarding the FL’s reply ‘only be used for the re-transmission of Msg3 re-transmission’, we have different understanding. The MCS indices 28≤IMCS≤31 or 29≤IMCS≤31 are typically applied for any PUSCH re-transmission, and can also be used for the first Msg3 re-transmission. FL: I copied related spec texts below. FL’s understanding is, if the reserved MCS indexes are used, the TBS will refer to a DCI transported in the latest PDCCH for the same transport block using . Msg3 initial transmission is scheduled by RAR UL grant instead of DCI. Therefore, the first Msg3 re-transmission cannot refer to Msg3 initial transmission. That is, the first Msg3 re-transmission cannot be scheduled with the reserved MCS indexes. the fallbackRAR shall be the same as the TB size used in the corresponding MsgA PUSCH transmission. else if - and transform precoding is disabled and Table 5.1.3.1-2 is used, or - and transform precoding is enabled, - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, - the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH. - the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 PUSCH.. Option 1-B, although not our first preference, at least leave a door for the possible utilization of 28≤IMCS≤31 or 28≤IMCS≤31 by explicit configuration in SIB1. Thus it is acceptable to us. |
| FL | Please find my inline comment above. @All, On one hand, proponents of Option 2 think Msg3 re-transmission needs to be scheduled with low MCS indexes when initial transmission fails. On the other hand, proponents of Option 1-B prefers more flexibility for re-transmission. Option 1-A is actually the middle ground between Option 1-B and Option 2. In addition, different proponents of Option 1-B still have different views on detailed singing structure. From FL perspective, each option can work, and we have to choose one way or another. For now, as FL, I still think Option 1-A supported by the majority is the most reasonable way out. |
| OPPO | We do not support this proposal. We share the views with Intel, Samsung and Ericsson. It is not clear to us why only way we can go is Option 1-A. The SIB1 parameter is optionally configured. The SIB1 overhead is not a concern. When configuration of 8 candidate MCS indexes is absent, MCS 0~7 can still be applied by default. We still prefer Option 1-B. |
| Qualcomm | Support the proposal. |
| LG | We can live with the Proposal for Issue#1. |
| vivo3 | Just to clarify that our intention of supporting option 2 is not simply allowing lower MCS value for retransmission, our intention is to have same range of MCS values for initial and retransmission of Msg3 without any additional changes specifically for retransmission of Msg3. |
| FL | It seems the situation is not changed. FL will ask for GTW discussion for this issue. Proposal for Issue#1: The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission. - Option 1-A: The 8 candidate MCS indexes are MCS 0~7. - Support/Can live with: Nokia/NSB, Sharp, Panasonic, Qualcomm, Apple, WILUS, China Telecom, NTT DOCOMO, CMCC, Spreadtrum, ZTE, Huawei/HiSilicon, LG - [Option 1-B: The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.] - Support/Can live with: Samsung, Intel, OPPO, Ericsson, CATT, [LG, CMCC, Nokia/NSB, ZTE] |
| Xiaomi | Support the proposal. |
| Ericsson | Regarding Option 1-B, we support that if mcs-Msg3repetition is configured, it will always configure 8 candidate values and the first 4 are for initial transmission. If companies have a concern of the signaling overhead of Option 1-B, there are always ways to design the parameter to reduce the overhead. |
| Samsung | Share with view of E///. |

    - [Paused] Text proposals for Issue#1

There are several companies ([1, Huawei, HiSilicon], [3, ZTE], [7, NTT DOCOMO],[15, Sharp]) provided the text proposals for Issue#1 based on Option 1-A. With taking all these text proposals into account, FL provides the updated version below for companies’ check.

Text Proposal for Clause 7.3.1.1.1 in TS 38.212 h00

##### 7.3.1.1.1 Format 0_0 **Unchanged parts are omitted** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - If the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], 5 bits as defined in Clause 6.1.2.1 and Clause 6.1.4.1 of [6, TS 38.214], - else, 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]. - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB **Unchanged parts are omitted**

Text Proposal for Clause 6.1.4.1 in TS 38.214 h00

#### 6.1.4.1 Modulation order and target code rate determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a fallbackRAR UL grant or for a MsgA PUSCH transmission, or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI, and if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3 - if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel; - elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI have an in-order one-to-one mapping with MCS index 0~7 in Table 5.1.3.1-1 to determine the MCS index IMCS. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. else - if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1. - elseif for PUSCH repetition type A the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI have an in-order one-to-one mapping with MCS index 0~7 in Table 6.1.4.1-1 to determine the MCS index IMCS. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. end <Unchanged parts are omitted>

This will be further discussed if Option 1-A is supported.

  - How to interpret the information filed

    - [Closed] Issue #2: How to interpret the information filed for Msg3 repetition

Regarding how a UE is managed to know whether to use legacy interpretation or new interpretation on the bit field indicating the number of repetitions, the following two options were agreed in RAN1#106-e.

Agreement Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. - Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. - Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

Companies’ support of each option is summarized below.

- Option 1:

  - Support: [1, Huawei, HiSilicon], [3, ZTE], [14, 	China Telecom], [15, Sharp], [17, CMCC], [18, LG]

  - Reason:

- Option 2 requires additional specification efforts to introduce new signaling for the indication.

- It was agreed that if the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}, which means Option 1 is supported. Similarly, Option 1 can also be applied in case the four candidate repetition factors are configured to achieve a unified solution.

- Option 2:

  - Support: [12, Apple]

    - For a UE capable of Msg3 transmission with repetitions, support dynamic indication of repetition factor using a reserved bit in DCI 1-0 to indicate repurposing MCS bit field in RAR UL grant.

    - For a UE capable of Msg3 transmission with repetitions, support dynamic indication of repetition factor using some of the reserved bit in DCI 0-0 to indicate repurposing MCS bit field in DCI 0-0.

First round

Based on the reasons of supporting Option 1 above, FL thinks it is more reasonable to go with Option 1 especially in maintenance phase.

Proposal 2: Regarding how a UE should interpret MCS information field for indication of the number of repetitions, Option 1 is supported.

- When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

| Company | Comments |
| --- | --- |
| Samsung | Agree. |
| Intel | We support the Proposal 2. |
| Nokia/NSB | Support. |
| Sharp | Support |
| Panasonic | We support the Proposal 2. |
| OPPO | Support |
| Qualcomm | Support |
| Apple | OK, as Option 2 needs more spec impact |
| vivo1 | Fine. |
| LG | Supportive. |
| Ericsson | Our understanding is that Proposal 2 is for CBRA, since UE interpretation of RAR for CFRA case is discussed in Issue#3. If so, we support, and it is better to clarify in Proposal 2. For example, Proposal 2: Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. |
| WILUS | Support |
| CATT | OK. |
| China Telecom | Support. |
| NTT DOCOMO | Support. |
| CMCC | Support FL proposal 2. |
| Spreadtrum | We support the Proposal 2. |
| Xiaomi | Generally agree with the proposal with slightly concern. In the case of four candidate values of repetition factor K configured by SIB1, does the repetition factor K=1 have to be one of the four candidate values? It means that only 3 remaining values need to be configured by SIB1, which are derived from the set e.g. {2, 3, 4, 7, 8, 12} given by Section 2.4.1. or is it possible that repetition factor K=1 is not always configured by SIB1? If so, all 4 values need to be configured by SIB1. And if K=1 isn’t configured, the UE with Msg.3 repetition request can only be scheduled with Msg.3 repetitions by gNB. Anyway, this is the configuration and scheduling scheme decided by gNB. @Xiaomi, If K=1 is not included in the candidate values, it would contradict previous agreements that gNB should be able to schedule Msg3 without repetition even UE requests Msg3 repetition. In addition, when configuring the candidate values, gNB would not be able to know whether all CE UEs would not require repetition (actually not a typical case) and then choose not to configure K=1. In this sense, not configuring K=1 would also impact the scheduling flexibility of gNB. |
| ZTE | Support |
| Huawei, HiSilicon | Support. |
| FL | @Xiaomi, Please find my inline reply above. @Ericsson, Your understanding is correct and your suggested updates look good. Based on the comments, the proposal is updated as follows. As this also has RRC impact (whether to include ‘1’ in RRC configuration), this proposal will also be prioritized in upcoming GTW session. Proposal for Issue#2: Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. |
| FL | The following agreements are reached during the GTW session. With this, the issue is clos Agreements Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. |

    - [Open] Text proposals for Issue#2

First round

There are several companies provided the text proposals for Issue#2. The main difference is whether to make it more general regarding the separate PRACH resource for Msg3 repetition. FL understanding is RAN2 has agreed to consider to use separate PRACH RO for new feature and/or feature combination, as the agreements copied below. Therefore, it’s better not to limit to separate PRACH preamble in RAN1 specification.

Agreements: Specification allows for use of Separate time-frequency resources, not defined through legacy RRC signalling, within Contention free preamble defined through legacy RRC signaling and the combination of these (i.e. using the reserved preamble at the end of SSBs like 2-step RACH)

Based on above, FL would like to collect companies’ view on the following TP:

Text Proposal for Clause 8.3 in TS38.213 h00

## 8.3 PUSCH scheduled by RAR UL grant <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted>

Companies are encouraged to provide views for above TP.

| Company | Comments |
| --- | --- |
| Samsung | Although we agree proposal 2. But we did not think the proposed TP here is necessarily needed. Actually, higher layer (MAC) will specify the RACH resource selection including deciding to have to do msg3 repetition and PHY is only being told which RACH resource/preamble to use. Whether such resource is really associated with msg3 repetition, it is already known by UE. So, if any wording is needed, we think only “if applicable” could do all the job. If applicable, the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, |
| Intel | We do not support this TP. For the issue in 2.2.1, we think NPUSCHrepeat>1 should be good enough to capture the Proposal 2. The update TP can be: If NPUSCHrepeat>1, Tthe UE repeats … |
| Nokia/NSB | Agree with Intel. |
| Sharp | The proposed TP is not clear. When the gNB schedules msg3 PUSCH with repetition factor = 1, then the resource on which the UE has transmitted a PRACH is not associated with msg3 PUSCH transmission with repetitions. We prefer the following text. “If the UE transmits a PRACH using a resource associated with requesting a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321],”. |
| Apple | Agree with Intel/Nokia_NSB |
| vivo1 | Not support. Whether the repetition is performed depends on the number of repetitions indicated, and the TP can be simply updated as following: If the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is if indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. |
| Ericsson | Agree with Intel, Nokia_NSB and vivo. The proposed TP means if a UE requests Msg3 repetition and the gNB configures K=1, the transmission occasion of Msg3 is based on available slot, which is different from legacy Msg3 transmission based on physical slot. |
| CATT | Generally fine. Since it is possible that K=1, our suggestion is to clarify that the PRACH resource is corresponding to ‘request of’ Msg3 PUSCH repetitions: “If the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with request of repetitions [11, TS 38.321]” |
| NTT DOCOMO | Support the update version by Intel. |
| CMCC | Fine with Intel's version. |
| Xiaomi | Fine with the version given by Intel. |
| ZTE | Prefer the proposed TP which is clearer. |
| FL | @Samsung, vivo and all, FL agrees that RAN2 spec will specify the separate resources for requesting Msg3 repetition during RACH procedure. However, it would not specify the relationship between request of Msg3 repetition and the repurposed interpretation of MCS field. The agreements reached in Tuesday’s GTW session is intent to clarify this, and FL thinks this should be captured in RAN1 specification. @Intel, and all, As commented by several companies, if UE requests Msg3 repetition, gNB may still indicate K=1. In such case, limiting the new interpretation of MCS filed to NPUSCHrepeat>1 only seems not correct. With above, FL would like to check whether the following TPs is agreeable. Text Proposal for Clause 8.3 in TS38.213 h00 ## 8.3 PUSCH scheduled by RAR UL grant <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource for requesting Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted>. |
| NTT DOCOMO | We understand the FL’s intention that the condition on msg3 repetition request is necessary for how UE determines NPUSCHrepeat from MCS field. Even in that case, we think it is better to mention UE repeats PUSCH transmission only if NPUSCHrepeat>1 for clarification. |

Second round

@Samsung, vivo and all, FL agrees that RAN2 spec will specify the separate resources for requesting Msg3 repetition during RACH procedure. However, it would not specify the relationship between request of Msg3 repetition and the repurposed interpretation of MCS field. The agreements reached in Tuesday’s GTW session is intent to clarify this, and FL thinks this should be captured in RAN1 specification.

@Intel, and all, As commented by several companies, if UE requests Msg3 repetition, gNB may still indicate K=1. In such case, limiting the new interpretation of MCS filed to NPUSCHrepeat>1 only seems not correct.

With above, FL would like to check whether the following TPs is agreeable.

Text Proposal for Clause 8.3 in TS38.213 h00

## 8.3 PUSCH scheduled by RAR UL grant <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource for requesting Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted>

| Company | Comments |
| --- | --- |
| Intel | We do not support the TP. As we mentioned previously, when NPUSCHrepeat is indicated as 1, UE does not need to repeat the Msg3 transmission. Given that the PRACH resource for requesting Msg3 repetition is defined in 321, the current TP seems redundant. In our view, we only need to mention the case when NPUSCHrepeat>1. FL: Please find the reply from Sharp below. |
| vivo2 | Thanks for the clarification from FL in the first round. Though we still do not think such wording in RAN1 spec. may not be necessary, we’re fine to capture this in RAN1 to make it more clear. However, considering the PUSCH should be the PUSCH scheduled by the RAR in response to the corresponding PRACH transmission and the NPUSCHrepeat slots could be only 1 slot, in which case there’s no repetition, it would be good to have following updates. <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource for requesting Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the corresponding PUSCH transmission over NPUSCHrepeat slots if NPUSCHrepeat>1 , where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| Panasonic | We agree with Intel. FL: Please find the reply from Sharp below. |
| Sharp | If the specification is updated with Intel’s proposal, the whole text is redundant since TS38.214 specifies all the above behaviors. Therefore, if the proposed change by FL is not agreed, we propose to delete the whole sentence from TS38.213. On the other hand, in our understanding, the motivation of the discussion is to introduce in what condition 2 MSBs of the MCS field in the RAR UL grant should be reinterpreted to determine the repetition factor. That is missing in the current specification. The proposed change by FL is intended to implement the condition. Therefore, we support the proposal. If K=1 is the issue, we propose further update as follows. If the UE transmits a PRACH using a resource for requesting Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat determines the number of repetitions K asis indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. |
| Samsung | We think our intention is not clearly reflected. Our previous comment saying that both MAC at UE side already decides to do msg3 repetition, and selects the msg3 repetition associated PRACH resource and indicated PHY layer to use. PHY layer at UE side just use it. Whether such PRACH resource is associated with msg3 repetition or not is known by UE already. Our concern on the proposal is that, by current wording, it seems you will ask UE PHY layer to explicitly compare the indicated resource by MAC is associated with msg3 repetition or not. We think this is not needed. This is like, PHY layer determine the valid PRACH resource, but we did not need to specify that MAC layer that when MAC layer selects PRACH resource from valid PRACH resource, because it is all at UE side. No need to further specification. . If applicable, Tthe UE repeats ….. , FL: FL has a different understanding here. It is true that MAC will determine the PRACH resource associated with Msg3 repetition. But it is PHY to determine whether to do repetitions and how many repetitions should do by decoding the MCS filed at PHY layer. As I replied before and also commented by Sharp above, the TP is to clarify the condition to apply repurposed MCS field. |
| Ericsson | According to the agreement in GTW, a UE should interpret MCS field in RAR in the Rel-17 repurposed way if it requests Msg3 repetition, regardless of the value of K indicated in RAR. But keeping the paragraph only about K>1 leaves the reader to think a UE should interpret MCS field in the legacy way if K is indicated as 1 in RAR. We think K=1 should be included in the specification, which conflicts with the words “UE repeats”. Therefore, we provide the following TP. <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource for requesting Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE performs the PUSCH transmission in one slot if NPUSCHrepeat =1 or repeats the PUSCH transmission over NPUSCHrepeat slots if NPUSCHrepeat > 1, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| Nokia/NSB | We tend to agree with FL, Sharp and Ericsson. At the same time we believe Samsung’s logic is sound. We suggest the following possible middle ground TP, where only the request by UE is mentioned (with reference to 38.321 for the whole higher-layer procedure, including all the checks on the ROs/preambles) and the word repeats is replaced with perform to capture any agreed value for NPUSCHrepeat (i.e., 1 as well). <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource for requests ing Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats performs the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| CATT | Once the UE requests Msg3 repetition, it needs to re-interpret the MCS field using a new method, although it is possible that K=1. Thus we support to make the spec more accurate and clear, and align with RAN2 specs. We are generally fine with FL’s TP. Vivo, Ericsson and Nokia’s TP are also fine, since they accommodate the following sentences. |
| FL | @Intel, Panasonic, Samsung, please find my reply inline above. Thanks vivo, Ericsson and Nokia for the good suggestion. Please check whether the following TP is acceptable. <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats performs the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| LG | We are OK with the FL’s TP. |
| vivo3 | “performs the PUSCH transmission over NPUSCHrepeat slots, …” seems like a TBoMS of Msg3 which should be avoided in our view. So we still prefer our earlier proposed TP. |
| FL | @vivo, it’s clear TMoMS is supported for Msg3 based on current specification. Your proposed TP would miss the case for NPUSCHrepeat =1. To move forward, let’s update the TP based on the version from Ericsson. <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE performs the PUSCH transmission in one slot if NPUSCHrepeat =1 or repeats the PUSCH transmission over NPUSCHrepeat slots if NPUSCHrepeat > 1, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| Apple | Support latest FL’s TP, based on Ericsson’s. |
| Sharp | The variable NPUSCHrepeat leads to inconsistency among specs. In TS38.214, the UE repeats the TB N * K times where K is indicated by the 2 MSBs of the MCS field, but, in TS38.213, the UE repeats/performs PUSCH transmission over NPUSCHrepeat (corresponding to N*K) slots where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field. Therefore, NPUSCHrepeat shouldn’t be indicated but determined. Our proposal is as follows. <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE performs the PUSCH transmission in one slot if NPUSCHrepeat =1 or repeats the PUSCH transmission over NPUSCHrepeat slots if NPUSCHrepeat > 1, where NPUSCHrepeat is determinedindicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| Samsung | We think Nokia’s revision could be more acceptable with change back to “repeat”, because even we say “repeat” but we allow NPUSCHrepeat=1, so nothing is wrong and all situation are included. <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource for requests ing Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted> |
| FL | @Gary, As I also Tomoki explained, MAC spec will not specify under which condition the UE should interpret the repurposed MCS information filed. The relationship between request of Msg3 repetition and the repurposed MCS field should be clarified in RAN1 spec. Considering you are the only company having concerns now, I hope you can live with this to make the spec clearer. @Tomoki, I can understand your point, while the update would look a bit weird... Instead, we can interpret corresponds to K in 38.214, and they are all indicated by RAR or DCI 0_0. Considering N=1 for Msg3 repetition, there would be no amgibuity. So, it may no need to refine on this point. Hope it is ok to you. BTW, I found N=1 is not clarified for Msg3 repetition in 38.214, and clarification is needed in 38.214. @Qi, The current TP might a bit clearer. Because, 'repeat transmission over N=1 slot' might be interpreted as transmiting in one slot and then repeating the transmission in one slot. As there is no ambiguity on this TP, let's keep it as it is. Hope it is ok to you. |
| Panasonic | We are fine with FL’s TP. |

    - [Paused] Issue #3: How to interpret the information filed for CFRA repetition

In addition, the following working assumption was reached for repetition of CFRA PUSCH.

Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

[3, ZTE], [14, 	China Telecom], [16, 	InterDigital], [17, CMCC] propose to confirm the working assumption, and they all have the following same understanding:

- After initial access and UE reports Msg3 repetition capability, repurposed MCS information field is applied for CFRA.

[2, vivo] also proposes to confirm the working assumption, while thinks RAN2 can discuss new signaling/method about how to interpret the information filed for CFRA repetition.

First round

Regarding how to interpret the information field for CFRA PUSCH repetition, the following two solutions can be considered.

  - Solution 1: After initial access and UE reports Msg3 repetition capability, repurposed MCS information field is applied for CFRA.

    - This is the natural interpretation based on the first note of the working assumption.

    - However, FL finds there might be an issue for this understanding. Assuming a scenario that a Rel-17 new UE in a legacy cell (the gNB is Rel-15/16), the gNB will not read the new capability reported from the Rel-17 UE. In such scenario, gNB and UE may have a different understanding on the MCS information field if MCS index larger than 3 is scheduled. The issue may not be that severe as gNB may not schedule large MCS index typically. Even if scheduled, gNB can then fall back to a low MCS index if gNB cannot successfully decode CFRA PUSCH due to different understanding on the MCS.

    - If supported, it can be specified in RAN2 to avoid RAN1 impact, e.g., capturing the following in TS 38.306.

      - ‘A UE supports msg3Repetition-r17 shall interpret RAR UL grant as indicating repetition for CFRA.’

  - Solution 2: After initial access and UE reports Msg3 repetition capability, repurposed MCS information field is applied for CFRA only if UE receives RRC configuration Msg3Repetitions-CB-PreamblesPerSSB-PerSharedRO.

    - If UE receives RRC configuration Msg3Repetitions-CB-PreamblesPerSSB-PerSharedRO, it means the gNB is a Rel-17 gNB.

    - If supported, it can be specified in RAN2 to avoid RAN1 impact, e.g., capturing the following in TS 38.331 for description of Msg3Repetitions-CB-PreamblesPerSSB-PerSharedRO.

      - ‘If Msg3Repetitions-CB-PreamblesPerSSB-PerSharedRO is configured, repurposed MCS information field is applied for CFRA for UEs reporting capability msg3Repetition-r17.

  - Solution 3: Introduce a new RRC parameter to indicate to apply legacy or new interpretation.

    - FL notices that this is also proposed in [25, Ericsson] in RAN2, though it is proposed to serve other purpose. FL suggestion is to leave to RAN2 about whether to introduce such RRC parameter or not, and no RAN1 impact is expected.

FL would like to check companies’ views on the following questions.

- Do you support to confirm the WA?

- If yes to 1), which solution do you prefer? Otherwise, please provide your reasoning.

- If Solution 1 or Solution 2 is adopted, do you agree that a corresponding RAN1 conclusion is needed (no need LS to RAN2)?

- Conclusion for Solution 1: From RAN1 perspective, a UE supports msg3Repetition-r17 shall interpret RAR UL grant as indicating repetition for CFRA.

- Conclusion for Solution 2: From RAN1 perspective, if Msg3Repetitions-CB-PreamblesPerSSB-PerSharedRO is configured, repurposed MCS information field is applied for CFRA for UEs reporting capability msg3Repetition-r17.

| Company | Comments |
| --- | --- |
| Samsung | 1) Not confirm. Pls see the second note: “The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.” To us, both solutions are obvious optimizations to CFRA PUSCH repetition. |
| Intel | It seems Samsung has good point. If there is any spec impact, this would naturally mean that the working assumption is not confirmed as indicated in the working assumption. |
| Nokia/NSB | We are not sure we understand why this discussion is needed. In our view, a UE would always be able to understand which gNB it is dealing with. Hence, we do not see any risk with possible misinterpretation of UL grants or UE capability reports. This does not need to be captured in RAN1 specification, since it is obvious. In this sense, we are favorable to confirm the WA and we think that neither Solution 1 nor Solution 2 is needed. |
| Sharp | 1) No. We shouldn’t confirm the WA. The working assumption can be confirmed once the solution is agreed. 2) Solution 2. The procedure should be captured in TS38.321. |
| Panasonic | We are OK to confirm WA. |
| OPPO | We are fine to confirm WA. |
| Qualcomm | We are fine to confirm WA. We prefer solution 3. |
| Apple | Similar view as Samsung |
| vivo1 | - We are fine to confirm the WA. - Detailed signaling aspects should be discussed in RAN2 assuming no RAN1 impacts should be introduced. - N/A. |
| LG | We have same opinion with Samsung, Intel, and Nokia. Obviously, this discussion is an optimization for CFRA PUSCH and seems to be an opposite way of the working assumption. |
| Ericsson | Support to confirm the WA and support solution 3, which has no additional RAN1 specification impact. Solution 1 and solution 2 have some flaws, but this can be solved by higher layer. We should leave it to RAN2 to implement a solution. For solution 1, a legacy gNB can’t understand Rel-17 UE capability. In additional, what if the neighbouring Rel-17 gNB does not support msg3 repetitions? For solution 2, as it will be up to the RACH indication and partitioning work item to implement signaling to support one or multiple preamble groups, it is likely that a preamble group for Msg3 repetitions and other features is configured, where Msg3Repetitions-CB-PreamblesPerSSB-PerSharedRO is absent. In general, the solution 1 and 2, where a UE interprets CFRA RAR in the repurposed way all the time or based on an optional parameter indirectly, cause ambiguity between gNB and UE. Solution 3 is simpler without ambiguity or RAN1 impact. |
| CATT | We are fine to confirm WA. The UE should be able to know whether gNB supports Msg3 repetition (either explicitly or implicitly inferred by any RRC parameters), and no RAN1 spec change is needed. RAN2 spec clarification is acceptable to us, if necessary. |
| China Telecom | We are fine to confirm the WA. |
| NTT DOCOMO | 1) Yes. PUSCH scheduled by RAR UL grant for CFRA and CBRA are not distinguished in RAN1 perspective. Also, there is specific use case for the PUSCH repetition scheduled by RAR UL grant in CFRA. 2) Solution 1. As FL pointed out, RRC configuration is better considering Rel-16 gNB deployment. 3) Agree |
| Spreadtrum | Fine to confirm the WA. Regarding how to interpret the information field for CFRA PUSCH repetition, solution 3 is preferred, and details can be discussed in RAN2, i.e., no additional RAN1 specification impact. |
| Xiaomi | Share the similar view as Samsung. |
| ZTE | Slightly prefer to confirm the WA. We can live with not confirming if no consensus can be reached in neither RAN1 nor RAN2. |
| Huawei, HiSilicon | Not confirm now. There are still some open issues discussed for initial transmission. Until the design for initial transmission is completed, we are not sure if the condition as stated in the last note of the WA can be fulfilled. |
| FL | @Samsung and all, When adding the last note in the last minute in RAN1#107-e, at least it’s FL understanding that ‘optimization specific for CFRA PUSCH’ is also limited to RAN1 only. On the other hand, FL agrees that current wording is not clear enough. The current situation is the majority support to confirm the working with the understanding that no RAN1 impact and RAN1 optimization. Whether/how it would impact RAN2 specification can be discussed in RAN2. On the other hand, a couple of companies prefer not confirm now. Based on current situation, FL would like to pause the discussion for now to wait a bit for the discussion in RAN2 and also other open issues. |
| InterDigital | We are ok to confirm the working assumption, but we are also ok with the FL’s decision to wait for the discussion in RAN2 to progress. |

  - Available slot determination for HD-FDD UE

    - [Open] Issue#4: Available slot determination for HD-FDD UE

At the RAN1#107-e meeting, it was agreed that for HD-FDD RedCap UEs supporting the counting based on available slots, ssb-PositionsInBurst is used in the first step of determining of available slots for both CG-PUSCH and DG-PUSCH.

For Msg3 PUSCH repetition, the available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst. This may indicate that for FDD system, all slots are considered as available slots for Msg3 repetition, which is also reflected in section 6.1.2.1 of 38.214 v17.0.0, as quoted below.

For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant.

For HD-FDD RedCap UEs, following options were considered for collision between SSB and Msg3 transmission, while no conclusion is reached yet, according to the discussion in AI8.6.1.2,

- Option 1: dynamically scheduled UL transmission is prioritized over SSB

- Option 2: SSB is prioritized over dynamically scheduled UL transmission

[2, vivo], [11, Intel] propose that, all slots are considered as available slots for Msg3 repetition for HD-FDD RedCap UEs, no matter the conclusion from RedCap WI.

- Reason: For HD-FDD RedCap UEs, it is more desirable to follow same rule as defined for FDD for Msg3 repetition. Given that gNB may not be able to differentiate whether the UEs are FDD or HD-FDD UEs during initial access, if the determination of available slots for Msg3 repetition for HD-FDD UEs follows that for TDD operation, this may introduce mis-alignment between gNB and UE on the slot for actual Msg3 repetition and may result in Msg3 decoding failure. Hence, in our view, for HD-FDD RedCap UEs, all slots are considered as available slots for Msg3 repetition.

- On the other hand, [20, Ericsson] proposes that HD-FDD UEs and FD-FDD UEs consider a slot that overlaps with a symbol of an SS/PBCH block with the index provided by ssb-PositionInBurst as an unavailable slot in the first step of determining available slots.

- Reason: If it is agreed that SSB is prioritized over Msg3 (re)transmission for a HD-FDD UE, the UE cancels the Msg3 transmission in the slot overlapping with SSB transmission by reusing legacy dropping rules, leaving gNB unaware of the cancellation and to blindly detect UL transmission, because the gNB doesn’t know the UE’s duplex capability until the RRC connection is established.

[15, Sharp] proposes that a configurability on available slot counting with/without referring to ssb-PositionsInBurst for HD-FDD RedCap UE for msg3 (re)transmission should be provided by new RRC parameter included in SIB1.

First round

Based on the input, FL’s understanding is the following:

- If Option 1 (i.e., Msg3 transmission is prioritized over SSB for HD-FDD RedCap UEs) is agreed in RedCap WI, all slots can be considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs. And there is no ambiguity issue for Msg3 transmission.

- If Option 2 (i.e., SSB is prioritized over Msg3 transmission for HD-FDD RedCap UEs) is agreed in RedCap WI, there could be the following three cases.

  - Case 1: All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs.

    - In this case, FD-FDD UEs will transmit Msg3 repetition even it overlaps with SSB in time while HD-FDD RedCap UEs need to cancel Msg3 repetition once it overlaps with SSB in time. Given gNB doesn’t know whether the UE is FD-FDD or HD-FDD, gNB would have ambiguity about whether there is actual transmission of Msg3 in the overlapping slot. It requires gNB to do blind detection or choose to ignore the overlapping repetition.

  - Case 2: All slots are considered as available slots for Msg3 repetition for FD-FDD UEs, while a slot that overlaps with SSB is an unavailable slot for HD-FDD RedCap UEs.

    - In this case, the situation seems worse. Because the available slot determination is different among FD-FDD UEs and HD-FDD UEs, and gNB would not know whether the Msg3 repetition would be transmitted.

  - Case 3: A slot that overlaps with SSB is an unavailable slot in the first step of determining available slots for both FD-FDD UEs and HD-FDD RedCap UEs.

    - In this case, it seems there would be no ambiguity issue. However, it has specification impacts as it is Case 1 in current specification.

Based on above analysis, FL thinks we can conclude that a unified solution between FD-FDD UEs and HD-FDD RedCap UE should be supported, i.e., either consider all slots are available for all FDD UEs, or consider the slot overlapping SSB is unavailable slot for all FDD UEs. For the first round of discussion, FL would like to collect companies’ view on this issue first.

| Company | Comments |
| --- | --- |
| Samsung | We think the resource is valid/available for both FD/HD-FDD; but only when UE operates in HD-FDD, the transmission in that available slot will be dropped if UE decides to measure SSB. |
| Intel | For HD-FDD RedCap UEs, our view is that we should follow same rule as defined for FDD for Msg3 repetition. As gNB may not be able to differentiate whether the UEs are FDD or HD-FDD UEs during initial access, if the determination of available slots for Msg3 repetition for HD-FDD UEs follows that for TDD operation, this would introduce mis-alignment between gNB and UE on the slot for actual Msg3 repetition and may result in Msg3 decoding failure. We think for HD-FDD RedCap UEs, all slots are considered as available slots for Msg3 repetition. |
| Nokia/NSB | We agree with Ericsson’s proposal. Having said this, we share FL’s view that a unified solution between FD-FDD UEs and HD-FDD RedCap UE should be supported |
| Sharp | After reading companies opinion, we think common solution for HD-FDD UEs and FD-FDD UEs seems better. Therefore, we support HD-FDD UE supports the same way as the FD-FDD UE (i.e., all slots are available) for all cases. |
| Panasonic | We think Case 1 in Option 1 is reasonable. |
| OPPO | A unified solution between FD-FDD UEs and HD-FDD RedCap UE is preferred for us. |
| vivo1 | HD-FDD UEs should follow legacy FD-FDD UEs, meaning that all slots are available for msg3 repetition. |
| Ericsson | If SSB is prioritized over Msg3 transmission for HD-FDD RedCap UEs, Case 1 and Case 3 are fine. Case 3 is our first preference. |
| CATT | In RedCap AI, there were long-heavy debates on whether dynamic UL or SSB should be prioritized. Finally, the group agreed that SSB is prioritized, because the HD-FDD UE may be, by implementation, NOT ABLE TO transmit any UL signal in SSB time, as insisted by many UE vendors. It is unclear Msg3 PUSCH can be an exception, and whether the HD-FDD UE can really support prioritizing Msg3 PUSCH over SSB. If this will lead to serious UE implementation issue, it is better to adopt Option 2. We can further discuss whether we should pick one of Case 1~3, or come up with a new solution. |
| NTT DOCOMO | Given that gNB cannot distinguish UE types, the unified solution for FD-FDD and HD-FDD UEs should be supported. Among three cases, we would rather support case 1, because it does not require any specification impacts for available slot determination. Also, smart resource scheduling by gNB can avoid the ambiguity issue. |
| CMCC | We think a unified solution between FD-FDD and HD-FDD UEs should be support. The gNB find out the UE’s capability for duplex after the RRC connection. A unified solution can avoid ambiguity for gNB about whether it is a available slots for UE. |
| Spreadtrum | A unified solution is preferred for us. |
| Xiaomi | Fine with case 1 and case 3 in Option 2, and slightly prefer case 1. |
| ZTE | Prefer Option 1. If Option 2 is supported in Redcap WI, we prefer Case 3. |
| Huawei, HiSilicon | Given the agreement below, we prefer the same solution for both Msg3 transmission and other PUSCH transmissions, i.e. we prefer Option 2. Agreement: [RAN1#107-e] - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission If option 2 is agreed for Msg3, we prefer case 1 because reduced number of available slots results in worse UL coverage which is opposite to this WI’s objective. With Case 1, a gNB can schedule more UL repetitions in order to preserve a better UL coverage performance for both FD-FDD and HD-FDD UEs. On the contrary, with Case 3, the performance of FD-FDD UEs is always degraded. |
| FL | Based on the input, it seems all companies agree to have a unified solution for FD-FDD and HD-FDD UEs. A clear majority of companies support or fine with the following proposal, regardless of which option is adopted in RedCap WI. Proposal for Issue#4: All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs. |

Second round

Proposal for Issue#4:

All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs.

Please comment only if you have strong concerns. If it is the case (hope not), please also provide an alternative agreeable proposal for all companies.

| Company | Comments |
| --- | --- |
| Intel | We are fine with the proposal. |
| vivo2 | Fine. |
| Samsung | support |
| Nokia/NSB | Can live with if this helps achieving consensus. |
| CATT | OK and seems this is the best we can achieve here. Moreover, we think this proposal means the ‘available slot determination’ (step 1) are the same for FD-FDD and HD-FDD. |
| FL | @CATT, Yes, your understanding is correct. |
| OPPO | Support. |
| LG | Fine with the proposal. |
| Xiaomi | For Proposal for Issue#4, we suggest to add an FFS: All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs. l FFS: collision handling rules for HD-FDD Redcap UEs |
| FL | We already have an FFS point in previous agreements as' FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s)'. So, let's not add it again here. We will anyway open the discussion once there is a conclusion from RedCap WI. |
| Panasonic | We are fine with the proposal. |

  - Number of repetitions for Msg3

    - [Open] Issue#5 Number of repetitions for Msg3

The main remaining issue for RRC parameter numberOfMsg3Repetitions is whether to also support repetition factors {12, 16}. Based on the discussion in RAN1#107-e and also the input in this meeting, a majority of companies prefer to support, while there are several companies don’t support.

  - Reason to support: 	It could be potentially useful and necessary for FR2 and Msg3 with Group B payload size. In addition, [8, Qualcomm] points out that they can be useful for NTN use cases, which is also included in Rel-17 CE WI.

  - Reason to not support: Lack of necessity; Causing more collisions when large repetition factor is used.

First round

Based on above, FL would like to propose a compromised way forward to support {12} while not support {16}. More specifically, the following proposal.

Proposal 1 for Issue#5: For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12}

Given this is the last meeting for RRC discussion, and FL doesn’t think this issue deserves such lengthy discussion during so many meetings, please be constructive and provide a agreeable way forward if you have strong concerns.

| Company | Comments |
| --- | --- |
| Samsung | First, the argument using NTN is not valid, self-explained by WID. Second, we start to question whether such value set is needed, because it is used for gNB to configure the repetition number, given that we said this is type A repetition, so as long as gNB is not configure some value outside the range/sets for type A repetition, then it should be fine. so we did not need to agree a new/separate value set, just one restriction in the RRC parameter for the repetition value set is enough, something like: UE is not expected to be configured with values that not supported for type A repetition. |
| Intel | We are fine with the Proposal 1 for Issue#5. In our view, large number of repetitions for Msg3 is also desirable for future proof. |
| Nokia/NSB | Agree with Samsung on the NTN argument, also because we have not performed any study in this sense during the SI phase. We would be aiming in the dark with no clue. Do not see a strong need of supporting up to 12 repetitions. |
| Sharp | At least ‘1’ should be included. |
| Panasonic | We are OK with the proposal. |
| OPPO | We are fine with the Proposal 1 for Issue#5. |
| Qualcomm | We do not support. Now that the set of 4 repetition numbers are configured, there is no benefit in excluding 16. |
| vivo1 | Both 12 and 16 can be supported so that 8 values will be allowed. Regarding the collision concern when larger number of repetitions is allowed, it can still be up to network to decide a reasonable number for the repetition of each specific transmission, network can decide a larger number when collision is less while more repetitions are needed. |
| LG | Same opinion with Nokia. |
| WILUS | We support the FL’s proposal. |
| CATT | We are fine with the proposal. |
| China Telecom | Support. |
| CMCC | Still don't support any numbers larger than 8 but can live with it if the group wishes. The factor ‘1’ is needed. We also think that if we only support 4 candidates for Msg3 repetition, for example, {1, 2, 4, 8}, then gNB and CE UEs can get the number of repetitions without additional configured candidates in SBI1 which seems easier and better for coverage. |
| Xiaomi | Share the similar view as Qualcomm and vivo. |
| ZTE | Fine with the proposal, though we prefer to support {12, 16}. |
| FL | It seems the situation is not changed, i.e., the majority support {12, 16}while a couple of companies think otherwise. In such case, FL thinks the proposal is still a good middle ground for all companies. As this is RRC impacted, the proposal will be prioritized in the upcoming GTW session. Proposal for Issue#5: For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12}. |

Second round

Based on the discussion during GTW session, no companies raised concerns on support of {12}. So, please do not make any comment on value {12}. Regarding {16}, the majority of companies are ok to support. From FL perspective, it has no harm to support this value, though we cannot reach consensus about the necessity. We have to decide one way or another, and FL’s suggestion is to go with the majority view, which is also aligned with the set for legacy PUSCH repetition type A.

Proposal-v1 for Issue#5: For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}.

Please comment only if you have strong concerns.

| Company | Comments |
| --- | --- |
| Nokia/NSB | We think that the argument used to defend the inclusion of {16} in the list of supported value is arbitrary and not based on evidence. The same argument could be used to defend the inclusion of larger values as well. Why wouldn’t they be acceptable? Where do we set the bar? For these reasons, we opposed the inclusion of {16} so far, and we still do. We propose to support only {1, 2, 3, 4, 7, 8, 12}. Having said this, and just to avoid misunderstandings, we are fine with FL’s suggestion of going with the majority view (whatever such view will be eventually). |
| FL | @Nokia/NSB, Thanks a lot for your constructive spirit! |

  - Spatial domain transmission relation

    - [Open] Issue#6: Spatial domain transmission relation

[6, Samsung]: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission. On the other hand, the UE can select the beam for msg3 re-transmissions.

[7, NTT DOCOMO]: It is beneficial to apply beam sweeping over repetitions for the better coverage performance. It’s up to UE implementation if the same spatial domain transmission filter is applied over the Msg3 repetition.

[8, Qualcomm]: Consider one of the following options on spatial domain transmission relation for Msg3 PUSCH transmission:

- Option 1: The UE transmits the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the same spatial domain transmission relation.

- Option 2: The UE may transmit the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the different spatial domain transmission relations.

==> FL: It seems no big issue to leave to UE implementation as legacy, or we can just define that the beam is the same among different Msg3 repetitions while doesn’t specify whether it is the same beam as the corresponding PRACH transmission.

[7, NTT DOCOMO]: In Rel-15/16, UE transmits the PUCCH using the same spatial domain transmission filter as one for the PUSCH transmission scheduled by a RAR UL grant if a UE does not have dedicated PUCCH resource configuration, such as when transmitting HARQ-ACK information corresponding to Msg4 PDSCH. On the other hand, it is up to UE implementation if the same spatial domain transmission filter is applied over the Msg3 repetition. For these reasons, it is unclear which spatial domain transmission filter is applied for PUCCH, when multiple spatial domain transmission filters are applied for PUSCH transmission scheduled by a RAR UL grant.

- Proposal: UE applies a specific spatial domain transmission filter for PUCCH transmission among the filters used for Msg3 repetition. (e.g., the spatial domain transmission filter used for Msg3 transmission in the first slot for PUSCH transmission).

==> FL: If the transmission beam of different Msg3 repetitions could leave to UE implementation, it seems no big issue to also leave to UE implementation for using one of the beam from Msg3 repetition for the associated PUCCH transmission. Or similar to above, we can just define that the beam is the same among different Msg3 repetitions while doesn’t specify whether it is the same beam as the corresponding PRACH transmission.

First round

For the first round of discussion, FL would like to collect companies’ view on the issues summarized in this section.

| Company | Comments |
| --- | --- |
| Samsung | We did not understand the argument that it’s beneficial to do beam sweeping among msg3 repetition. From msg1/msg2, UE has already determined a useful/qualified beam for UL transmission, why UE needs to do beam sweeping for msg3 repetition, which degrades the repetition benefits at all. Besides, without knowing the tx beam of msg3, it’s also difficult for gNB to plan the rx beam. We think, at least, the same beam should be used among repetitions. |
| Intel | Our view is that the existing mechanism can be extended for the Tx beam for Msg3 repetition. More specifically, - It is up to UE implementation for Tx beam for Msg3 repetition. - Same Tx beam is applied for Msg3 repetition. |
| Nokia/NSB | Samsung proposed a good argument. It is rather obvious to us that if there is a message that can benefit from beam sweeping to increase coverage is msg1 for FR2 deployments. The same Tx filter should be used across all Msg3 repetitions. We also need to ensure that Msg3 repetitions are compatible with spatial relationship between msg3 and PUCCH carrying HARQ-ACK for Msg4. |
| Sharp | In current specification, it is stated that PUCCH carrying HARQ-ACK for msg4 should use the same spatial filter for PUSCH scheduled by RAR UL grant. Therefore, it is clear from the current specification that all repetitions scheduled by a RAR UL grant should use the same spatial Tx filter. |
| OPPO | The beam selection for PRACH procedure is performed during Msg1 stage. It is not necessary to define beam sweeping for Msg3 repetition. Same beam with repetition can bring coverage enhancement for Msg3. We prefer same beam should be used among Msg3 repetition. |
| vivo1 | No need to optimize this. Msg3 beam determination is up to UE implementation and note that other UL signals before RRC connection (e.g. common PUCCH) will also use the TX beam same as Msg3. |
| CATT | We have similar view with vivo. For Msg3 repetition it-self, it seems OK to leave it to UE implementation. Same beam seems natural in FR1 while different beam may be beneficial in FR2. For spatial relationship between Msg3 and PRACH, Rel-15/16 did not specify the relationship between Msg3 and PRACH. It is up to UE implementation. This principle can be reused. |
| NTT DOCOMO | Given that even the current specification specifies which filter is used for PUCCH, Tx spatial filter for PUCCH carrying HARQ-ACK for Msg4 should be clarified for gNB Rx beam determination. It is a bad idea to keep the spatial domain filter for PUCCH up to UE. |
| CMCC | The UE transmits the Msg3 PUSCH repetitions within a transmission using the same beam. |
| Xiaomi | We recommend that the legacy mechanism for TX UL beam determination for Msg.3 in Rel-16 should be adopted for Msg.3 transmission in Rel-17, i.e., it is up to UE implementation for Tx beam for Msg3 transmissions. In addition, whether UE transmits the Msg3 PUSCH repetitions within a transmission using the same spatial domain transmission relation, should also be determined by UE implementation without any spec impact. |
| ZTE | We prefer to leave to UE implementation. |
| FL | Based on input, the majority of companies support to apply the same Tx beam for Msg3 repetition. This could ensure the reception at gNB side and also ensure a clear Tx beam for PUCCH before RRC connection. In such situation, FL would like to check whether companies can live with the following proposal. In addition, FL encourages companies to share views on whether specification changes are needed or a conclusion is sufficient for the following proposal. Proposal for Issue #6. - It is up to UE implementation for Tx beam for Msg3 repetition. - Same Tx beam is applied for Msg3 repetition. |

Second round

Based on the discussion in the first round, it seems companies’ view are convergent to the following proposal.

Proposal for Issue #6.

- It is up to UE implementation for Tx beam for Msg3 repetition.

- Same Tx beam is applied for Msg3 repetition.

Companies are encouraged to provide views on the proposal, and also whether there is any spec impacts for the second bullet.

| Company | Comments |
| --- | --- |
| Intel | We are fine with the proposal. |
| vivo2 | Support. |
| Panasonic | We are fine with the proposal. |
| NTT DOCOMO | We support the proposal. The proposal clarifies the spatial domain transmission filter for PUCCH carrying HARQ-ACK for msg4 as well. |
| Sharp | We support the proposal. We think no specification impacts on the 2nd bullet. |
| Samsung | We are ok with it. |
| Ericsson | The second bullet looks like the same Tx beam is applied for all Msg3 repetitions of initial transmission and retransmission. Not sure if this is the intent. |
| Nokia/NSB | We suggest a small correction to the second bullet: - Same Tx beam is applied for all Msg3 repetitions. Concerning the question asked by FL, we think it is not so trivial and may be better to add a simple sentence in the spec to ensure this aspect is captured properly (as it is the case, for instance, for PUCCH carrying HARQ-ACK for Msg4). |
| CATT | Although we understand the motivation of the proposal, we are a little hesitant to support it for both FR1 and FR2. Technically, it is fine for FR1. But we are not sure it is suitable for FR2. If this is the majority view, maybe OK. We can fix serious issue, if really arises, in the future. In addition, Ericsson’s comment is valid to us. In our understanding, same Tx beam is only applied for the Msg3 repetitions scheduled by the same RAR, or DCI with TC-RNTI. This proposal does not intend to couple the relationship between initial transmission and re-transmission. |
| FL | @CATT, thanks for your constructive spirit! Based on the comments from Ericsson, Nokia/NSB, CATT, the proposal is updated as follows to make it clearer. Proposal for Issue #6. - Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by the same RAR UL grant. - It is up to UE implementation for the Tx beam. - Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by the same DCI scrambled with TC-RNTI. - It is up to UE implementation for the Tx beam. |
| OPPO | We are fine with FL’s revised proposal. |
| Qualcomm | Support the revised proposal. |
| LG | We have similar concern with CATT. In our understanding, even within the Msg3 initial/retransmission it seems more reasonable to leave it up to the UE implementation. Now that the number of repetition {12, 16} was introduced in Issue #5, restricting the UE Tx beam to use only same beam may cause performance degradation. |
| FL | @LG, Based on views from the majority, leave to UE implementation would lead to two issues: 1) it makes very difficult for gNB reception if the beam of different repetitions could be different, 2) it would cause ambiguity for the beam of PUCCH before RRC connection. Based on current situation, hope you can live with the proposal. Proposal for Issue #6. - Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by the same RAR UL grant. - It is up to UE implementation for the Tx beam. - Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by the same DCI scrambled with TC-RNTI. - It is up to UE implementation for the Tx beam. |
| Samsung | We can live with FL’s proposal, we still believe keep same beam for all the repetition in one PUSCH transmission, is basically behavior for gNB reception. |
| Xiaomi | For Proposal for Issue #6, our concern is just the same as CATT and LG mentioned in the tdoc of FL’s summary. Why the TX beam must be kept the same across repetitions within a Msg.3 transmission? How much the performance gain can be obtained separately in FR1 and FR2? From our point of view, it seems more reasonable that the Tx beam for each repetition is also up to UE’s implementation without any spec effort, just the same as Msg.3 transmission without repetitions in Rel-16. |
| LG | For Proposal for Issue #6, we still have concern about that. We have similar view with CATT and Xiaomi. It seems too restrictive to use same beam. When the number of msg3 PUSCH repetitions is large, we think that it is necessary to allow changing the Tx beam according to the channel condition. Even if the Tx beam is changed, we think that there is no impact on the network. Also, we think spec change is unnecessary. In addition, we want to ask whether there is any spec impact if we agree to use same beam for transmitting msg3 PUSCH repetition. |
| Intel | For Proposal for Issue #6, we suggest the following change: Proposal for Issue #6. l Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by a the same RAR UL grant. n It is up to UE implementation for the Tx beam. l Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by a the same DCI scrambled with TC-RNTI. n It is up to UE implementation for the Tx beam. |
| FL | - @Yoonsuk, @Xuemei, As I replied before, leaving to UE implementation would lead to two issues: 1) it makes very difficult for gNB reception if the beam of different repetitions could be different, as gNB doesn't know the beam. It may not be a big issue in legacy as it has only one repeittion. 2) it would cause ambiguity for the beam of PUCCH before RRC connection. - Considring CATT has already stated that they are ok to follow the majority. Hope this could be also acceptable for you. - Regarding whether it has spec impact, we can further discuss in the next round. I only made minor editorial changes below based on Intel’s suggestion. Proposal for Issue #6. - Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by a the same RAR UL grant. - It is up to UE implementation for the Tx beam. - Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by a the same DCI scrambled with TC-RNTI. - It is up to UE implementation for the Tx beam. |
| Panasonic | We are fine with the proposal. |

  - [Open] Other issues

- Conditions of requesting Msg3 repetition

[8, Qualcomm]: The RSRP threshold depends on SS-RSRP and/or UE power class.

==> FL: RAN2 has already agreed to introduce a separate RRC parameter for rsrp-ThresholdSSB threshold for requesting Msg3 repetition. Further details are under discussion in RAN2, and no further action in RAN1 is needed unless any clarification/request from RAN2.

- Differentiation mechanisms for Msg3 repetition

[8, Qualcomm]: If the Msg3 PUSCH repetition request is indicated by a separate RO, RAN1 should discuss whether this RO can be shared for Redcap UE to send identification via PRACH or not.

[11, Intel]:

- Separate ROs can be additionally supported for requesting Msg3 PUSCH repetition.

- For separate PRACH preambles with shared ROs,

  - A unified design for PRACH resource partitioning for different use cases including coverage enhancement, network slicing, identification of RedCap UEs and RA-SDT is preferred.

  - PRACH mask is defined to indicate a subset of ROs associated with same SSB index for requesting Msg3 PUSCH repetition.

[23, xiaomi]:

Support PRACH resource sharing between RedCap requiring coverage enhancement and Non-RedCap requiring coverage enhancement

  - PRACH resource sharing on a separate initial UL BWP configured for RedCap can be considered

==> FL: Based on discusion in RAN1#107-E, we have already reached a common understanding that RACH related issues would be discussed in RAN2 as usual, and no further action in RAN1 is needed unless any clarification/request from RAN2.

- Support of a separate maximum number of preamble transmissions for CE UEs without msg.3 repetition requesting.

[13, Xiaomi] proposes to convert 4-step RACH without msg.3 repetition requesting into 4-step RACH with msg.3 repetition requesting for CE UEs when the failure of 4-step RACH without msg.3 repetition requesting happens, and configure a separate maximum number of preamble transmissions for CE UEs without msg.3 repetition requesting.

==> FL understanding is this could be discussed in RAN2 instead.

- Support of joint channel estimation for Msg3 repetition

[8, Qualcomm]: If JCE is supported for Msg3 PUSCH repetitions with subject to power consistency and phase continuity requirements, only back-to-back PUSCH transmission is supported and the UE indicates its capability of supporting JCE during RACH procedure. UE is not expected to monitor Msg2 PDCCH between the repetitions of Msg3. gNB should be able to expect no UL beam switching among repetitions of Msg3. Support transmission of PTRS inside Msg3 repetitions.

==> FL: This has been discussed before and no consensus has been reached. No further discussion would be pursued, meaning it will not be supported in Rel-17.

- Available slot determination

[19, WILUS] proposes not to use flexible symbol(s) indicated by tdd-UL-DL-ConfigurationCommon to determine available slot for Type A PUSCH repetition for Msg3 except the first repetition of Msg3 PUSCH to be scheduled.

==> FL understanding is we should not revisit what we have been agreed. The issue pointed out in [19] has been discussed before. It’s up to gNB whether to avoid such issue or bear blind decoding/interference if allowed. So, FL would not like to re-open the discussion unless requested by a clear majority of companies.

- Waveform indication for Msg3

[12, Apple]: Specify a UE-specific procedure to enable/disable transformprecoder for Msg3 transmission, via:

- Alt1: explicit indication, e.g., repurpose some bits in RAR UL grant (for initial Msg3 transmission) or DCI format 0_0 with CRC scrambled by TC-RNTI (for Msg3 retransmission) to indicate whether transformprecoder enabled or not

- Alt2: implicit indication, for example, transformprecoder is enabled if UE indicates to require coverage enhancement/recovery.

==> FL understanding is this may be out of scope as it is not specific for Msg3 repetition. In addition, it seems not a critical enhancement for Msg3 repetition.

- Support of qam64-LowSE MCS

In [13, Xiaomi]: QAM64-LowSE MCS table provides lower coding rate, which is benefit for Msg.3 coverage enhancement with lower required SNR. So, QAM64-LowSE MCS table can be used for Msg.3 transmission in bad coverage. Therefore, it proposes to support the use of QAM64-LowSE MCS table for Msg.3 transmission with repetitions.

==> FL understanding is this may be out of scope as it is not specific for Msg3 repetition. In addition, it seems not a critical enhancement for Msg3 repetition.

First round

FL would like to open the floor for discussion on above issues.

- If the majority agree FL’s assessment above, we can have a conclusion that issue (4~7) will not be discussed in RAN1 in future meetings, and issue (1~3) can only be discussed in RAN1 if requested by other WGs.

| Company | Comments |
| --- | --- |
| Sharp | We agree with FL. |
| OPPO | WE agree with FL. For issue 6, it seems within the scope of Rel-18 Coverage enhancement. We can revisit it in Rel-18. |
| vivo1 | Fine. |
| FL | The discussion on this issue is continue. |
| Ericsson | Agree. |
| CATT | OK. |
| FL | Based on the input so far, FL’s proposed conclusion seems agreeable. Proposed conclusion: For Rel-17 CE WI, Issue (4~7) in Section 2.6 of R1-2200712 will not be discussed in RAN1 in future meetings, and issue (1~3) in Section 2.6 of R1-2200712 can only be discussed in RAN1 if requested by other WGs. If any concerns, please provide your comment below. |

- Proposals for GTW session

Proposal for Issue#1: The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission.

  - Option 1-A: The 8 candidate MCS indexes are MCS 0~7.

    - Support/Can live with: Nokia/NSB, Sharp, Panasonic, Qualcomm, Apple, WILUS, China Telecom, NTT DOCOMO, CMCC, Spreadtrum, ZTE, Huawei/HiSilicon, LG

  - [Option 1-B: The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.]

    - Support/Can live with: Samsung, Intel, OPPO, Ericsson, CATT, [LG, CMCC, Nokia/NSB, ZTE]

- Proposals for email approval

Proposal-v1 for Issue#5: For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}.

Text Proposal for Clause 8.3 in TS38.213 h00

<Unchanged parts are omitted>

A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests Msg3 PUSCH transmission with repetitions [11, TS 38.321], Tthe UE performs the PUSCH transmission in one slot if NPUSCHrepeat =1 or repeats the PUSCH transmission over NPUSCHrepeat slots if NPUSCHrepeat > 1, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

<Unchanged parts are omitted>

Proposal for Issue#4:

All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs.

Proposal for Issue #6.

- Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by a the same RAR UL grant.

  - It is up to UE implementation for the Tx beam.

- Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by a the same DCI scrambled with TC-RNTI.

  - It is up to UE implementation for the Tx beam.

Proposed conclusion:

For Rel-17 CE WI, Issue (4~7) in Section 2.6 of R1-2200712 will not be discussed in RAN1 in future meetings, and issue (1~3) in Section 2.6 of R1-2200712 can only be discussed in RAN1 if requested by other WGs.

Appendix-A: Previous agreements

RAN1#104-e

Agreements: - For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below. - Option1: UL grant scheduling Msg3. - FFS details. - FFS fallbackRAR UL grant. - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported. - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI - FFS details. - Option3: SIB1 only - Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively Agreements: - For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below. Option1: DCI format 0_0 with CRC scrambled by TC-RNTI. FFS details. Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI. Option2: Can be determined based on the repetition number for Msg3 initial transmission Agreements: Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission. FFS details, e.g., signaling etc. Agreements: For Msg3 PUSCH repetition, the following options are considered, aiming for down-selection in RAN1#104b-e: - Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request, - A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions. - For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - FFS details if any. - Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request, - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition - For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways) - Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc. - gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition. - FFS details if any. - Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions. - Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS details if any. - Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways) - Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - FFS details if any. - Other options are not precluded.

RAN1#104b-e

Agreement: For Msg3 PUSCH repetition, support the following modified Option 2-1. - Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.). - Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual - FFS details if any. Agreements: For the determination of RV for Msg3 PUSCH repetition, - RV of the first repetition is determined in the same way as legacy. - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission. - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission. - FFS determination of the RV sequence. Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted. - FFS additionally using MAC RAR for indication. Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted. Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3. - FFS: the determination of available slots.

RAN1#105-e

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold. Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission. Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc. Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI. Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission. - The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214. - The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214. - FFS: The RV cycling for Msg3 is based on transmission occasions on available slot. Conclusion: - Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously) Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

RAN1#106-e

Agreement Do NOT support fallback RAR UL grant in 2-step RACH for indicating Msg3 repetition. Agreement The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition. Working Assumption Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors. - The new TDRA table is configured by SIB1, with selecting one of the two options below. - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor. - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor. - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied. - K=1. - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - X MSB bits of the MCS information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below. - Option 1: X LSB bits of the TPC information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Option 2: A predefined TPC command table with including repetition factor K is introduced. - FFS details. Agreements Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field. Agreement - Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition. - FFS whether to support other values, e.g., 8. - Note: K=1 is supported and how to support K=1 is FFS. Agreement - The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered. - If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot. Agreements: Do not support TBoMS for Msg3 in Rel-17 coverage enhancement WI.

RAN1#106bis-e

Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Conclusion There is no consensus to additionally support intra-slot frequency hopping for Msg3 PUSCH with repetition in Rel-17. Note: intra-slot FH is supported when a UE is scheduled Msg3 PUSCH without repetition. Agreement If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping. Agreement The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot. - FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17. - FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s) Agreement Include the following into the reply LS to R1-2108712(R2-2109195). RAN1 thinks at least the number of preambles per SSB per RO for request of Msg3 repetition, i.e., CB-PreamblesPerSSB, is needed. It’s up to RAN2 whether to indicate the start of preamble index for request of Msg3 repetition with shared RO. Agreement Include the following into the reply LS to R1-2108712(R2-2109195). - From RAN1 perspective, there is no need to separately configure the following legacy RACH parameters configured in RACH-ConfigCommon for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier. - prach-ConfigurationIndex - msg1-FDM - msg1-FrequencyStart - zeroCorrelationZoneConfig - totalNumberOfRA-Preambles - ssb-perRACH-OccasionAndCB-PreamblesPerSSB - FFS: rsrp-ThresholdSSB - rsrp-ThresholdSSB-SUL - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig - msg3-transformPrecoder Agreement Include the following into the reply LS to R1-2108712(R2-2109195) - From RAN1 perspective, it can be beneficial to separately configure rsrp-ThresholdSSB for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

RAN1#107-e

Agreement - Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition. - Note: whether and how to introduce other potential mechanisms to use the flexible symbols are separately discussed. - Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon. Conclusion - There is no consensus to additionally introduce explicit indication to indicate whether or not flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition. Agreement - RV cycling for Msg3 PUSCH repetition is based on transmission occasions on available slots. Agreement For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules. - The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused. - The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused. - The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused. Agreement - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Agreement Reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. - Note: there is no specification impact. Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. - Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

RAN1#107bis-e

Agreements Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Reference

R1-2200055 	Discussion on Msg3 repetition for coverage enhancement	 Huawei, HiSilicon

R1-2200090 	Remaining issues on Type A PUSCH repetitions for Msg3	 vivo

R1-2200115	 Discussion on remaining issues for Type A PUSCH repetitions for Msg3 	ZTE

R1-2200154	 Remaining issues on Type A PUSCH repetitions for Msg3	 CATT

R1-2200164	 Approaches and solutions for Type A PUSCH repetitions for Msg3 	Nokia, Nokia Shanghai Bell

R1-2200209	 Type A PUSCH repetitions for Msg3	 Samsung

R1-2200240	 Type A PUSCH repetitions for Msg3 	NTT DOCOMO, INC.

R1-2200305 	Type A PUSCH repetition for Msg3	 Qualcomm Incorporated

R1-2200323	 Discussion on Type A PUSCH repetitions for Msg.3	 Panasonic Corporation

R1-2200338	 Type A PUSCH repetitions for Msg3 coverage 	OPPO

R1-2200383 	Remaining details on Msg3 PUSCH repetition	 Intel Corporation

R1-2200424	 On remaining issues for Msg3 Coverage Enhancement	 Apple

R1-2200469 	Discussion on Type A PUSCH repetition for Msg3 	xiaomi

R1-2200489	 Remaining issues on type A PUSCH repetitions for Msg3 	China Telecom

R1-2200503	 Type A PUSCH repetitions for Msg3	 Sharp

R1-2200522	 Type A PUSCH repetitions for Msg3 	InterDigital, Inc.

R1-2200592 	Remaining issues on type A PUSCH repetitions for Msg3	 CMCC

R1-2200615 	Discussion on coverage enhancement for Msg3 PUSCH 	LG Electronics

R1-2200637	 Remaining issues on Type A PUSCH repetitions for Msg3	 WILUS Inc.

R1-2200659	 Remaining Issues for Type A PUSCH Repetition for Msg3 	Ericsson

R1-2200116	 Discussion on RRC parameters for coverage enhancement	 ZTE

R1-2200165 	Performance of Type A PUSCH repetitions for Msg3 for different MCS index configurations	 Nokia, Nokia Shanghai Bell

R1-2200470	 Other considerations for coverage enhancement 	xiaomi

R1-2200563	 Remaining issues on RRC parameters for NR coverage enhancement 	vivo

R2-2201598 On msg3 repetitions, Ericsson.