3GPP TSG RAN WG1 #107bis-e	R1-2200717

e-Meeting, Jan 17th – 25th, 2022

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

For this meeting, we will prioritize issues with impact to other WGs and in particular aim to progress on RRC issues by January 20th since the RRC email threads will start on that day.

Regarding TPs (text proposals), since we are entering maintenance phase we might need to work on TPs. For this first Rel-17 maintenance meeting, we will still mostly achieve agreements to be implemented by the spec editors.

There will also be some TPs to discuss. For TPs proposed by companies in input Tdocs, there may be two cases: either a more generic agreement on a functionality is needed before diving into a TP (or leaving it to the spec editors), or the TP can be discussed directly (e.g. an obvious correction to the specifications). For the “simple” type of TP, we will directly collect feedback on the TP itself in the discussion document and TPs can be possibly endorsed by email.

# ACK/NACK-based feedback specific

## Different CB types/PUCCH structures

### Other CB types for unicast

Submitted Proposals

- (vivo) Proposal 5:

  - When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot,

    - If enhanced type-2 codebook is used for unicast, UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

    - If type-3 HARQ-ACK codebook is used for unicast, UE only generates one type-3 codebook including both unicast HARQ-ACK and multicast information.

      - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI.

    - FFS when enhanced type-3 codebook is used for unicast

- (Samsung) Proposal 1:

  - If a UE would provide unicast and multicast HARQ-ACK CBs in a same PUCCH/PUSCH, the UE drops the multicast HARQ-ACK CB when the unicast HARQ-ACK CB is the Rel-16 Type-3; otherwise, the UE appends the multicast HARQ-ACK CB to the unicast HARQ-ACK CB.

- (DOCOMO) Proposal 2:

  - When a UE is indicated to report HARQ-ACK feedback with Type-3 HARQ-ACK codebook in a slot, the UE does not expect to receive an indication to report HARQ-ACK for multicast with different codebook type in the same slot.

- (DOCOMO) Proposal 3:

  - When a UE is scheduled to report HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot and enhanced Type-2 codebook is used for unicast, the UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

(LGE) Proposal 3:

  - Clarify whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast.

#### Round-1

FL’s Comments:

UE may be configured with different HARQ-ACK codebook types for unicast and multicast. It was agreed in the last meeting that in such a case and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot, UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast:

Agreement When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot, - UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast. - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI. - FFS: when Type-3 HARQ-ACK codebook or enhanced Type-2 codebook is used for unicast - Define a UE capability

One remaining FFS is for the case when Type-3 HARQ-ACK codebook or enhanced Type-2 codebook is used for unicast. It was viewed by some companies in the last meeting that such cases are not supported.

Based on the submitted proposals in this meeting, some details solutions are proposed for different codebook types for unicast. The discussion for this meeting can open from whether to support such cases and what the solutions are if supported.

FL’s proposal:

#### Proposal 2.1.1.1

Regarding whether to support multiplexing HARQ-ACK for unicast and multicast with the same priority in the same PUCCH slot when Type-3, enhanced Type-2, or enhanced Type-3 HARQ-ACK codebook is configured for unicast, down-select from the following options:

- Option1: Does not support.

- Option2: Support the following case:

  - When UE would provide enhanced Type-2 HARQ-ACK for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, UE generates two separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook.

  - When UE would provide Type-3 HARQ-ACK codebook for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, UE drops the HARQ-ACK for multicast.

  - FFS for the case when UE would provide enhanced Type-3 HARQ-ACK codebook for unicast.

Collect views:

| Company | Comments |
| --- | --- |
| - Nokia, NSB | - Support Option 2 with slight modification in wording: When the UE would provide is scheduled to multiplex enhanced Type-2 HARQ-ACK for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, the UE generates two separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook. - When the UE would provide is scheduled to multiplex Type-3 HARQ-ACK codebook for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, the UE drops the HARQ-ACK for multicast. - FFS for the case when UE would provide enhanced Type-3 HARQ-ACK codebook for unicast. - In our view, there is no technical reason to not to support the above proposal. Enhanced Type-3 codebook will be introduced in Rel-17 and we need further study on multiplexing schemes. Therefore, we may postpone multiplexing support to the next releases. |
| - ZTE | - The MBS feedback should not impact the configuration of unicast. All the Type-3, enhanced Type-2 and enhanced Type-3 HARQ-ACK codebook should be allowed for unicast. - For Type-3 HARQ-ACK codebook, it is clear all the feedbacks for multicast will be included in the Type-3 HARQ-ACK codebook, thus it is not necessary to transmit the multicast codebook. - For enhanced Type-3 HARQ-ACK codebook, only a subset of HPIDs will be feedback. Thus, we need to determine whether all the HPIDs of the multicast are included in the enhanced Type-3 HARQ-ACK codebook. If yes, then drop the multicast codebook; if no, then multiplexing them together. - Thus, we support the following updated Option2. - Option2: Support the following case: - When UE would provide enhanced Type-2 HARQ-ACK for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, UE generates two separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook. - When UE would provide Type-3 HARQ-ACK codebook for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, UE drops the HARQ-ACK for multicast. - When UE would provide enhanced Type-3 HARQ-ACK codebook for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, if all HARQ-ACK feedbacks for multicast have been included in the enhanced Type-3 HARQ-ACK codebook, UE drops the HARQ-ACK for multicast; otherwise, UE concatenates the codebook for multicast after the codebook for unicast. - FFS for the case when UE would provide enhanced Type-3 HARQ-ACK codebook for unicast. |
| - Samsung | - Support the proposal |
| - Qualcomm | - We are fine with the first subbullet about the multiplexing enhanced Type-2 HARQ-ACK for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast. - Regarding the second subbullet, it is not preferred to drop the multicast feedback. If the multiplexing of Type-3 HARQ-ACK for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast is not possible, gNB should avoid this case. |
| - CMCC | - Similar view as ZTE, the enhanced Type-3 HARQ-ACK codebook should be treated the same as Type-3 HARQ-ACK codebook. |
| - Lenovo, Motorola Mobility | - We have some questions for clarification: - When UE would provide enhanced Type-2 HARQ-ACK for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, UE generates two separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook. - When UE would provide Type-3 HARQ-ACK codebook for unicast and Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot, UE drops the HARQ-ACK for multicast. - FFS for the case when UE would provide enhanced Type-3 HARQ-ACK codebook for unicast. - Regarding the first bullet of Option 2, we are not sure whether there are two sub-codebooks for unicast and multicast in case the UE is configured to monitor more than one multicast services. If happens, then the total number of sub-codebooks may exceed 2. - For Type-3 codebook, it includes HARQ-ACK feedback for all the HARQ processes. As we agreed before, there is no HARQ process split for unicast and multicast. Upon reception of one-shot codebook triggering, UE shall report HARQ-ACK feedback for all the HARQ processes including both unicast and multicast. Simply saying the codebook for multicast is dropped is not quite clear. - The motivation of introducing Type-3 HARQ-ACK codebook is from Rel-16 NR-U to combat the unpredictable performance of unlicensed spectrum. Here, do we assume MBS in Rel-17 is supported on unlicensed spectrum? |
| - vivo | - Agree with Nokia and ZTE’s comment that all codebook types should be supported for unicast. We support option 2 with one small comment is for the wording in option 2. We think the intension is to say that UE only generates type 3 codebook since HARQ-ACK information for all HARQ processes, multicast HARQ-ACK are also included in the codebook, “UE drops the HARQ-ACK for multicast” is not very correct. “UE does not generate the type 1/2 HARQ-ACK codebook for multicast” may be better.” |
| - LG Electronics | - For Type-3 codebook, the intention of “UE drops the HARQ-ACK for multicast” is not clear to us. Considering that there is no HARQ process split for unicast and multicast, we wonder whether UE shall report HARQ-ACK feedback for all the HARQ processes including both unicast and multicast or for HARQ processes only for unicast. |
| - Spreadtrum | - Generally, we are fine with the main bullet, and the 1st sub-bullet for option 2. - Regarding the 2nd sub-bullet, we have one question for clarification. Does the original intention means that only HARQ process allocated for unicast would be considered for type-3 codebook for unicast, not all HARQ process? So UE only report HARQ-ACK for unicast. |
| - NTT DOCOMO | - We support Option 2. As for enhanced Type-3, we think it should be treated the same as Type-3. |
| - MediaTek | - Regarding the enhanced Type-2 HARQ-ACK for unicast, we are basically ok with the description. - Regarding the Type-3 HARQ-ACK for unicast, before making decision, some clarification is needed whether the Rel-17 MBS is supported on unlicensed spectrum as Lenovo commented. - Considering the enhanced Type-3 HARQ-ACK is introduced for Rel-17 URLLC topic, we suggest the HAQ-ACK multiplexing with multicast can be discussed in future release. |
| - CATT | - We are OK with ZTE’s version. But we are wondering, that is the UE behaviours when a subset of HARQ-ACK feedback is included in the enhanced Type-3 HARQ-ACK codebook? |
| - Ericsson | - OK with the proposal. |

### Text proposals for different PUCCH structures

#### TP 2.1.2.1 (Huawei)

# 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. If the UE multiplexes HARQ-ACK information associated with unicast DCI formats and HARQ-ACK information associated with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format. A UE is not expected to be configured with different PUCCH structures for unicast and multicast for which the HARQ-ACK are with the same priority and to be scheduled to multiplex the HARQ-ACK in the same PUCCH slot simultaneously.

##### Round-1

| Company | Your comments/suggestions to the text proposal TP 2.1.2.1 |
| --- | --- |
| - Nokia, NSB | - We suggest the following wording: - A UE is not expected to be scheduled with overlapping PUCCH transmissions for HARQ-ACK for unicast and multicast with the same priority, if the UE is configured with different PUCCH structures (slot/sub-slot based) for unicast and multicast for the corresponding priority. |
| - ZTE | - We tend to have the same handling for URLLC and MBS. For URLLC, we don’t have such statement in the specification. We don’t think we have to add this statement for multicast. - In fact, the specification doesn’t have the detailed UE behaviour for this case, it is the same as to say this case is not supported already. |
| - Samsung | - OK with the intention but similar view as ZTE. - If something is still desired to be captured, a more spec-consistent/compact rewording can be as below. - “A UE is not required to multiplex in a PUCCH multicast HARQ-ACK and unicast UCI of same priority if the UE is provided subslotLengthForPUCCH for the PUCCH with the unicast UCI”. |
| - Qualcomm | - Support |
| - CMCC | - Support Samsung’s version. |
| - Lenovo, Motorola Mobility | - Agree with Samsung’s wording. |
| - NTT DOCOMO | - We support Samsung’s version. |
| - MediaTek | - Support the proposal. |
| - CATT | - Samsung’s version is OK for us. |
| - Apple | - Fine with Samsung’s version. |
| - Ericsson | - Support |

## Multicast HARQ-ACK on PUSCH

Submitted Proposals

- (vivo) Proposal 6:

  - For PUCCH carrying multicast HARQ-ACK overlaps with PUSCH, reuse NR Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and PUSCH.

    - When both unicast HARQ-ACK and multicast HARQ-ACK, or HARQ-ACK for multiple G-RNTIs are multiplexed on one PUSCH, for UL-DAI indication, the following options can be considered.

      - Option 1: There is only one UL-DAI in UL grant, the UL-DAI is only for unicast HARQ-ACK and there is no UL-DAI for multicast HARQ-ACK.

      - Option 2: There is only one UL-DAI in UL grant, the UL-DAI indicates the total bits of unicast HARQ-ACK and multicast HARQ-ACK.

- (ZTE) Proposal 7:

  - The UL DAI filed in UL scheduling DCI is used to indicate the Type-2 codebook size of G-RNTIs.

    - Up to 2 additional UL DAI fields for multicast can be applied, where each UL DAI corresponds to a group of G-RNTIs configured by RRC.

- (Samsung) Proposal 6:

  - For multiplexing unicast and multicast HARQ-ACK in a PUSCH scheduled by DCI format 0_1/0_2

    - If the unicast and multicast HARQ-ACK codebooks are Type-1 or of different Type, DCI format 0_1/0_2 includes a Rel-16 DAI field for each corresponding HARQ-ACK codebook.

    - If both unicast and multicast HARQ-ACK codebooks are Type-2, the Rel-16 DAI field is applicable to all HARQ-ACK codebooks.

- (Samsung) Proposal 7:

  - A UE can receive multicast PDSCHs that are scheduled after a PUSCH and have an associated PUCCH transmission in a same slot as the PUSCH transmission. The UE does not include HARQ-ACK for those multicast PDSCHs in the PUSCH.

### Round-1

FL’s Comments:

When discussing Type-2 HARQ-ACK codebook for multicast, we agreed that DL DAIs are separately counted for unicast and multicast as follows:

Agreement: For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1, - DAI for unicast and DAI for multicast are separately counted. - Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported. - FFS details on the codebooks. - FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

UL-DAI is discussed in this section for multiplexing HARQ-ACK for multicast onto PUSCH with the same priority.

UE may be scheduled with PUSCH transmission which may collide with the HARQ-ACK feedback transmission for multicast with the same priority. As the legacy procedure of multiplexing HARQ-ACK feedback for unicast onto PUSCH, support UL-DAI indicating the number of HARQ-ACK bits for multicast as defined in Rel-16 seems straightforward.

Whether to support multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority has never been discussed. The discussion for this meeting can open from whether to support such cases and what the solutions are if supported.

FL’s proposal:

#### Proposal 2.2.1-1

When PUCCH carrying multicast HARQ-ACK overlaps with PUSCH with the same priority, support UL-DAI indicating the number of HARQ-ACK bits for multicast as defined in Rel-16.

Collect views:

| Company | Comments |
| --- | --- |
| - Nokia, NSB | - Support the intention of the proposal. - In Rel-16, UL-DAI is used to indicate the size of HARQ-ACK feedback for Type-2 codebook and to indicate “including” Type-1 codebook to PUSCH transmission for Type-1 codebook. In our view, same behavior can apply for multicast: #### Proposal 2.2.1-1 When PUCCH carrying multicast HARQ-ACK overlaps with PUSCH with the same priority, support UL-DAI indicating the number of HARQ-ACK bits for multicast as defined in Rel-16 for unicast HARQ-ACK. |
| - ZTE | - We support this proposal, which is the same as the legacy UE behaviour. |
| - Samsung | - The proposal is generally OK. |
| - Qualcomm | - One clarification question. - The proposal may need to clarify that the UL DAI for multicast is for all the G-RNTIs to be multiplexed or per G-RNTI. |
| - CMCC | - Support Nokia’s version, since we don’t have any HARQ-ACK codebook for multicast in Rel-16. |
| - Lenovo, Motorola Mobility | - We support the intention. - BTW, we have below questions for clarification: - whether the “UL DAI” is additionally introduced for multicast or not is not clear to us. - If UE is configured with multiple MBS, is the “UL DAI” applied to all the configured MBS? |
| - vivo | - Support in principle. It is better to clarify that “multicast HARQ-ACK” is for a certain G-RNTI or for multiple G-RNTIs. For the case of multiple G-RNTIs, DL DAI is separately counted, if no UL-DAI field is added, the UL-DAI should indicate the total number of HARQ-ACK bits for multicast. |
| - LG Electronics | - The proposal is generally OK. As Qualcomm mentioned, we need to clarify that UL DAI for multicast is per G-RNTI or for a group of G-RNTIs. |
| - Spreadtrum | - Fine with Nokia’s revision |
| - NTT DOCOMO | - We are fine with the proposal. |
| - MediaTek | - Generally ok with the proposal if it doesn’t introduce a new UL-DCI field for multicast |
| - CATT | - We have same questions with Lenovo. |
| - Apple | - General fine with the proposal, clarification is needed for UL DAI indication of single/multiple G-RNTI. |
| - Ericsson | - In legacy, UL-DAI indicates the number of bits for DL unicast. Then how to let it indicate number of bits just for multicast? Shall it be at least the total nrof bits including both unicast and multicast? |

#### Proposal 2.2.1-2

Regarding whether to support multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority, down-select from the following options:

- Option1: Does not support.

- Option2: Support the following case:

  - If the unicast and multicast HARQ-ACK codebooks are both Type-1, the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH.

  - If both unicast and multicast HARQ-ACK codebooks are Type-2, down-select from:

    - Option2-1: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks.

    - Option2-2: two 2-bit UL DAIs are included in DCI for unicast and multicast, respectively.

  - FFS the unicast and multicast HARQ-ACK codebooks are different Types.

Collect views:

| Company | Comments |
| --- | --- |
| - Nokia, NSB | - We support option-2-1. The gNB can indicate a total UL DAI for multicast and unicast. - If option-2-2 is applied, we would need a change in unicast DCI formats for an non-essential feature. |
| - ZTE | - We support to go with Option2 with some clarifications. - First of all, for the case “unicast and multicast HARQ-ACK codebooks are both Type-1”, the handling for UL DAI with value ‘0’ should be clarified. I understand the intention is to reuse the legacy handling for UL DAI with value ‘0’, but it is better to clarify it explicitly to avoid any further confusion. - The Type-2 codebook, we think Option 2-1 is simpler, which can avoid increasing the DCI size. |
| - Samsung | - Option 1 does not work and would require new timelines/procedures – in Rel-15/16, UE first resolves overlapping among PUCCHs (unicast/multicast HARQ-ACK is multiplexed) and then among PUCCH and PUSCH – option 1 would complicate that procedure and require new specifications. - For Type-1 CB, the proposal is functional but can be detrimental for the overall operation. We support, at least via RRC, to have separate DAIs for unicast and multicast – the overhead can be too much if a UE always reports both CBs, while the cost is 1 bit in the UL grant. If not always separate DAIs (i.e. 1 more bit), at least the NW should be allowed to choose between a joint DAI of separate DAIs. - For Type-2, we support option 2-1 (“can live” with option 2-2 but note that for N G-RNTIs, there will be N DAIs). - There is no reason for the FFS as there is nothing to FFS at this stage (i.e. the FFS should be resolved now – follow Rel-16 for the Type-1/Type-2 DAI). |
| - Qualcomm | - We prefer to have separate UL DAI for unicast and multicast. - But in the case of multiple G-RNTIs, it may be possible to avoid large UL grant overhead by distributing the feedback of multiple multicast in different slots. |
| - CMCC | - Prefer option 2-1 to avoid the impact on unicast DCI. |
| - Lenovo, Motorola Mobility | - We prefer Option 2-2 to have separate UL DAI for unicast and multicast since unicast and multicast have different sub-codebooks. - One thing needs to be decided is when UE is configured with multiple multicast services, the UL DAI for multicast is applied to all the multicast services or only to a single service. |
| - vivo | - We support option 2-1. Multicast HARQ-ACK can be for multiple G-RNTIs, two 2-bit UL DAIs in UL-grant may not be enough to indicate HARQ-ACK for different G-RNTIs. |
| - LG Electronics | - We prefer separate UL DAI for unicast and multicast. |
| - NEC | - Support Option2. For Option1, does it means a PUCCH carrying the multiplexed unicast HARQ-ACK bits and multicast HARQ-ACK bits will be dropped when it overlaps with a PUSCH with the same priority? Obviously it is not reasonable that unicast service will be affected due to the additional support of multicast service. |
| - Spreadtrum | - Support option 2-1. Unicast DCI should not be impacted by MBS. |
| - NTT DOCOMO | - We prefer Option 2-1 to avoid increasing the DCI size. |
| - MediaTek | - Share the similar view with CMCC/Spreadtrum/NTT Docomo and that unicast DCI should not be impacted by MBS. |
| - CATT | - Prefer Option 2-2. We think it is more reasonable that the UL DAI for unicast and multicast should be separated due to they may have different sub-CBs. |
| - Lenovo 2 | - In Option 2-2, there are two UL DAIs one for unicast and another for multicast. - Without UL DAI for multicast, unicast PUSCH may be impacted when UE and gNB have different understanding on multicast HARQ-ACK codebook size. |
| - Apple | - Option 2-1 is preferred to avoid impacting unicast DCI size. |
| - Ericsson | - Need clarification of Option2-1 in option2. What does it mean ‘UL DAI is applicable to both HARQ-ACK codebooks’? Does it mean it can be used to indicate total nrof HARQ feedback bits for unicast and multicast? |

## Type-1 HARQ codebook

### Fallback for Type-1 CB

Submitted Proposals

(Samsung) Proposal 9:

  - Extend fallback operation for Type-1 HARQ-ACK codebook to multicast PDSCH receptions.

- (Lenovo) Proposal 6:

  - Fallback operation is supported for Rel-17 Type-1 HARQ-ACK codebook determination. For a given G-RNTI, among the candidate PDSCH reception occasions, if UE detects only a SPS group-common PDSCH scrambled by the given G-RNTI, or only a group-common PDSCH scheduled by DCI format 4-1 with CRC scrambled by the given G-RNTI and counter DAI value of 1, or only a single DCI format 4-1 with CRC scrambled by the given G-RNTI and DAI value of 1 for activating SPS PDSCH, or only a single DCI format 4-1 with CRC scrambled by the given G-RNTI and DAI value of 1 for releasing SPS PDSCH, the UE determines the corresponding HARQ-ACK sub-codebook for the given G-RNTI comprises only a single HARQ-ACK information bit for the SPS group-common PDSCH, or the group-common PDSCH, or the single DCI format 4-1 for SPS PDSCH activation, or the single DCI format 4-1 for SPS PDSCH release; otherwise, the UE determines the corresponding HARQ-ACK sub-codebook for the given G-RNTI according to the K1 set and TDRA configuration as legacy.

#### Round-1

FL’s Comments:

For unicast, the fallback operation is defined for the Type-1 HARQ-ACK codebook as referred to TS38.213 as follows:

### 9.1.2 Type-1 HARQ-ACK codebook determination ## << Unchanged text is omitted >> If a UE reports HARQ-ACK information in a PUCCH only for - a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or - a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or - SPS PDSCH reception(s) associated with a CS-RNTI within the occasions for candidate PDSCH receptions as determined in clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for one SPS PDSCH reception according to corresponding occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to more than one SPS PDSCH receptions that the UE is configured to receive are ordered according to the following pseudo-code; otherwise, the procedures in clause 9.1.2.1 and clause 9.1.2.2 for a HARQ-ACK codebook determination apply. ## << Unchanged text is omitted >>

The proposal from two companies is to support the fallback operation for the Type-1 HARQ-ACK codebook for multicast by duplicating the above text in the box and replacing DCI format 1_0 with DCI format 4_1 and CS-RNTI by G-CS-RNTI (or by including DCI format 4_1 and G-CS-RNTI into the above text. The proposal seems straightforward but the discussion for this meeting can start from collecting views whether to support it since it has never been discussed.

FL’s proposal:

#### Proposal 2.3.1.1

Regarding whether to support fallback operation for the Type-1 HARQ-ACK codebook for multicast, down-select from the following options:

- Option1: Does not support.

- Option2: Support. Extending the fallback operation for Type-1 HARQ-ACK codebook to multicast PDSCH receptions.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - The fallback operation for Type-1 HARQ-ACK codebook is explained in the part of “FL’s comments” but not in the FL’s proposal, because the proposal will expand very long otherwise. |
| - Nokia, NSB | - We support Option-2, unless additional disadvantages are found for multicast operation than unicast. |
| - ZTE | - OK with the above proposal with Option2. |
| - Samsung | - We support Option 2 - direct extension of unicast operation and one of the many cases where that general principle (extend unicast to multicast) should apply. |
| - CMCC | - Support Option 2. |
| - Lenovo, Motorola Mobility | - We support fallback operation to reduce Type-1 codebook. |
| - vivo | - We support option 2. |
| - Spreadtrum | - Support option 2 |
| - NTT DOCOMO | - We support Option 2. |
| - Qualcomm | - We are open to consider Option 2. - For unicast, we only have PTP (re)transmission with one C-RNTI or one CS-RNTI per UE. - However, for multicast, the UE may be configured with multiple G-RNTIs and multiple G-CS-RNTIs and the PTP retransmission with C-RNTI or CS-RNTI can be associated with the PTM initial transmission with a G-RNTI or G-CS-RNTI, which may need to be considered if the fall-back operation for Type-1 HARQ-ACK codebook if supported. |
| - MediaTek | - Ok with the option 2. |
| - CATT | - Support Option2. |
| - Apple | - We support Option 2. |
| - Ericsson | - Support to downselect between the two options |

### Text proposals

#### TP for HARQ-ACK codebook for more than one G-RNTIs

##### TP 2.3.2.1.1 (Huawei)

#### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats - a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and - a serving cell is placed in a second set SM of NcellsDL,MMi serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, where Mi corresponds to the case of DCI format 4_1/4_2 scrambled by the ith G-RNTI if configured, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits. If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = 'mode1', the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook. < Unchanged parts are omitted >

##### TP 2.3.2.1.2 (Nokia)

## *** Unchanged text is omitted *** If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats - a serving cell is placed in a unicast set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and - serving cells are placed in a set first according to an ascending order of a serving cell index and then for the same serving cell index by concatenating the unicast set followed by the multicast set(s) according to an ascending order of the corresponding G-RNTI *** Unchanged text is omitted ***

##### TP 2.3.2.1.3 (Qualcomm)

- 9.1.2 Type-1 HARQ-ACK codebook determination ## *** Unchanged text is omitted *** If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI format associated with a G-RNTI - a serving cell is placed in a unicast set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for unicast DCI formats for scheduling on serving cell c, and - a serving cell is placed in a multicast set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for multicast DCI formats associated with a G-RNTI for scheduling on serving cell c, and - serving cells are placed in a set by concatenating the unicast set followed by the other multicast sets according to an ascending order of the corresponding G-RNTI the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits. ## *** Unchanged text is omitted ***

##### Round-1

| Company | Your comments/suggestions to the text proposals: TP 2.3.2.1.1, TP 2.3.2.1.2, and TP 2.3.2.1.3, including which one is more acceptable and any suggestions for that if any. |
| --- | --- |
| - ZTE | - It seems the TPs are trying to clarify the sub-codebook for multicast for FDMed case is performed per G-RNTI. - From our perspective, it seems better to go with the current specification to have only one sub-codebook for all the G-RNTIs. Otherwise the codebook size will be too large. - For example, if the TDRA table (with 8 SLIVs) is shared between unicast and multicast#1 and multicast#2. If FDMed reception is supported, - 1) Following the current specification, 8 (unicast) + 8 (multicast#1/multicast#2) =16bits will be generated; - 2) Following the TP above, 8 (unicast) + 8 (multicast#1) + 8(/multicast#2) =24bits will be generated. - Since the multicast#1 and multicast#2 can NOT be FDMed, it is ok to go with the current specification to reduce the codebook size from our perspective. - Thus, we tend to go with the existing specification. |
| - Samsung | - The TPs are not necessary and are actually incorrect in principle as there is no dependence of the multicast Type-1 HARQ-ACK codebook on the G-RNTI (as there is no dependence of the unicast Type-1 HARQ-ACK codebook on the C-RNTI or the MCS-C-RNTI). |
| - Qualcomm | - We are fine with the wording as long as the TP can correctly reflect the RAN1 agreement. The current spec to use single sub-codebook for all G-RNTIs is not aligned with the RAN1 agreement. |
| - Lenovo, Motorola Mobility | - We propose to delay the TPs until the decision on Type-1 codebook is made especially for fallback operation. |
| - Ericsson | - TP 2.3.2.1.2 seems clearer. |

#### TP for Type-1 CB for multicast

##### TP 2.3.3.2.1 (DOCOMO)

----------------------------------- Start of Text proposal to 9.1.2 of 38.213 ------------------------------------------------ ### 9.1.2 Type-1 HARQ-ACK codebook determination This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static or pdsch-HARQ-ACK-Codebook-Multicast = semi-static. <Unchanged text omitted> #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP - If the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static - If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 for serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6. - If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK - If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2 - If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-DCI-1-2 - If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c and configured with pdsch-HARQ-ACK-Codebook-Multicast = semi-static - if the UE is not provided type1-Codebook-Generation-Mode = 'mode1', K1 is additionally provided by the union of dl-DataToUL-ACK-ForDCI Format4_1 - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4_1, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - if the UE is provided type1-Codebook-Generation-Mode = 'mode1', the UE - determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by - if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static and pdsch-HARQ-ACK-Codebook-Multicast = semi-static, the row indexes R of the table are provided by - the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is not configured to monitor PDCCH for multicast DCI formats for serving cell c, or is not provided type1-Codebook-Generation-Mode = 'mode1', or, if any, for the first K1,UM set - the union of row indexes of time domain resource allocation tables for DCI format 1_0 and/or DCI format 1_1 and/or DCI format 1_2 for serving cell c for the second K1,U\M set, if any - the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c for the third K1,M\U set, if any - else if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static, the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for unicast DCI formats the UE is configured to monitor PDCCH for serving cell c - else the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c <Unchanged text omitted> If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats and the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static and pdsch-HARQ-ACK-Codebook-Multicast = semi-static <Unchanged text omitted> ----------------------------------- End of Text proposal to 9.1.2 of 38.213 ------------------------------------------------

###### Round-1

| Company | Your comments/suggestions to the text proposal TP 2.3.3.2.1 |
| --- | --- |
| - ZTE | - We prefer to leave this TP to editor since it seems this doesn’t need any new agreement. |
| - Samsung | - Fine to agree to the TP in principle. There is some repetitive/unnecessary text (could be abstracted in a general statement to keep the text compact) as the entire clause is for Type-1. |
| - Qualcomm | - Ok in principle |
| - CATT | - Prefer to leave this TP to editor. |
| - Lenovo, Motorola Mobility | - OK to leave it to editor. |
| - Ericsson | - Support |

## Others

(vivo) Proposal 4:

  - For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast,

    - For the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot, separate transmission of HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast is supported.

- (vivo) Proposal 7:

  - Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

(CATT) Proposal 5:

  - To achieve the common understanding on codebook size between gNB and UE, RAN1 should specify the following two aspects for generating Type-2 codebooks when UE joint an ongoing multicast transmission:

    - T0 indication: The slot that UE begins to monitor the PDCCH occasion and acquire scheduling information.

    - Missing DCI: The UE assume that DAI is not circled (e.g.  j=0) and (n-1) DCIs are missed when the DCI value is n (n is not equal 1) on T0 slot.

(CATT) Proposal 6:

  - For Type-2 HARQ-ACK feedback, nHARQ-ACK for PUCCH transmission can be modified as following:

nHARQ-ACK=nHARQ-ACK(unicast) +OACK(multicast),  in where nHARQ-ACK(unicast)  is computed as below formula which is same as R15/R16:

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c ;

    - OACK(multicast) is the total number of bit for all multicast traffic.

(Samsung) Proposal 2:

  - If a UE receives PDSCHs with multiple G-RNTIs that are scheduled by respective PDCCHs, the UE determines a resource for a PUCCH with corresponding HARQ-ACK based on the DCI format provided by the PDCCH that starts last.

(Qualcomm) Proposal 2:

  - For RRC_CONNECTED multicast UEs, it can be configured to generate the Type-1 HARQ-ACK feedback codebook for the G-RNTI(s) activated via L2 signaling.

(Intel) Proposal 1:

  - For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

(Intel) Proposal 3:

  - When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback.

(Apple) Proposal 3:

  - For the Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, if UE is configured with multiple G-RNTIs, only one sub-codebook is generated for all G-RNTIs, and is appended after the sub-codebook for unicast.

(NEC) Proposal 1:

  - In order to reduce the HARQ-ACK codebook overhead when multiple G-RNTIs are configured, the UE can bundle the HARQ-ACK bits corresponding to multicast candidate PDSCH occasions with the same SLIV to an allowed size.

(LGE) Proposal 1:

  - For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

# NACK-only based feedback specific

## NACK-only collides with other UCI

Submitted Proposals

NACK-only collides SR/UCI

(Huawei) Proposal 3:

  - When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority, adopt the procedures for multiplexing NACK-only and SR as in the following table:

|  | PF0 for SR | PF1 for SR |
| --- | --- | --- |
| PF0 for NACK-only | For positive SR: UE transmits PF0 NACK-only in the resources - for PF0 SR, by settingdifferent from that for PF0 SR only. - for PF1 SR, by setting b(0) or b(0)b(1) different from that for PF1 SR only depending on the number of TB(s) with NACK-only: - for one TB NACK-only, setting b0=1, - for two TBs (N,A) setting b(0)b(1)=(1,0), for (A,N) setting b(0)b(1)=(0,1), and for (N,N) setting b(0)b(1)=(1,1) For negative SR: UE transmits PF0 NACK-only in resources configured for NACK-only. |  |
| PF1 for NACK-only | Drop SR | For positive SR: UE transmits PF1 NACK-only in the resources for PF1 SR. For negative SR: UE transmits PF1 NACK-only in the resources for PF1 NACK-only |

- (vivo) Proposal 8:

  - When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, the following solutions can be considered.

    - Option 1: If SR is positive, NACK-only is dropped. If SR is negative, transmit NACK-only on its resource

    - Option 2: If SR is positive, transmit NACK-only on SR PUCCH resource. If SR is negative, transmit NACK-only PUCCH.

      - Option 2-1: only NACK-only is transmitted on SR PUCCH resource. If it is ACK, transmit SR only.

    - Option 3: When SR is positive, for the case that both SR PUCCH and NACK-only are PUCCH format 1, transmit NACK on SR using the same rule as NR R15/R16. Otherwise, NACK-only/SR is dropped. When SR is negative, transmit NACK-only PUCCH.

- (ZTE) Proposal 3:

  - On NACK-only PUCCH multiplexing with unicast PUCCH format 0/1 if they are indicated in the same slot,

    - When one NACK-only PUCCH is transformed into ACK/NACK bit due to multiplexing with unicast PUCCH format 0 carrying 1 bit HARQ-ACK, mapping of values for two HARQ-ACK information bits to sequences reuses what defined for unicast in Rel-16.

    - When one NACK-only PUCCH is transformed into ACK/NACK bit due to multiplexing with unicast PUCCH format 1 carrying 1 bit HARQ-ACK, transmission of two HARQ-ACK information bits reuses what defined for unicast in Rel-16.

    - When one NACK-only PUCCH is transformed into ACK/NACK bit due to multiplexing with unicast PUCCH format 0/1 carrying 2 bits HARQ-ACK, the total of 3 HARQ-ACK information bits are transmitted on one new PUCCH format 2/3/4.

    - In the above cases, the multiplexing result PUCCH is always determined based on the last unicast DCI.

- (ZTE) Proposal 4:

  - When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority,

    - For NACK only PUCCH carrying no more than 2 bits, convert NACK only bits to ACK/NACK bits, and determine CS based on HARQ-ACK bits and SR status, and always use SR PUCCH as the multiplexing result PUCCH.

    - For NACK only PUCCHs carrying more than 2 bits, convert NACK only bits to ACK/NACK bits, and convert SR to 1 bit, and always use a separate configured PUCCH resource as the multiplexing result PUCCH.

- (Nokia) proposal 5:

  - When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCC-H transmissions for SR for the same priority, support Alt1 from the following alternatives:

    - Alt1: up to two TBs of NACK-only feedback is transformed into ACK / NACK HARQ bits and is multiplexed with SR.

      - The UE uses the same UE-specific PUCCH resources that the UE is configured by RRC signalling to transmit multiplexed NACK-only bits, i.e., the scenario for more than B = 2 TBs in the corresponding proposal.

      - For the case of more than two TBs of NACK-only, SR is dropped.

    - Alt2:

      - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR;

        - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), [FFS: SR] is dropped.

      - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only.

- (Nokia) proposal 6:

  - When PUCCH transmission for the NACK-only based feedback for one MBS collides with PUCCH transmissions for ACK / NACK feedback for another MBS with the same priority, support UE multiplexing the NACK-only based feedback with the ACK / NACK feedback on PUCCH by transforming NACK-only into the ACK / NACK HARQ bit.

    - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

    - Converted NACK-only bits are concatenated to the ACK / NACK feedback.

- (Samsung) Proposal 4: Support the following

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support [Alt1] from the following alternatives:

Alt1: up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

- PUCCH resources configured for unicast will be used in concerned cases subject to legacy procedure as Rel-15/16.

- For the case of more than two TBs/HARQ-ACK bits of NACK-only, [FFS: SR] is dropped.

- (Samsung) Proposal 5:

  - If a PUCCH resource for unicast HARQ-ACK cannot accommodate multiplexing of multicast HARQ-ACK, the UE transmits a PUCCH with only unicast HARQ-ACK.

- (DOCOMO) Proposal 7:

  - When a PUCCH format 0 with NACK-only based feedback overlaps with a PUCCH with positive SR, the NACK-only based feedback is transformed into ACK/NACK based HARQ-ACK bits, multiplexed with SR, and transmitted in another PUCCH resource for ACK/NACK based feedback.

(Intel) Proposal 4:

  - When a PUCCH-SR for unicast collides with a PUCCH resource carrying NACK-only feedback for multicast with the same priority, the PUCCH-SR for unicast is prioritized.

- (Lenovo) Proposal 8:

  - For SR multiplexing with single NACK-only based feedback, positive SR multiplexing with NACK-only feedback is transmitted in SR resource.

- (Lenovo) Proposal 9:

  - For SR multiplexing with single NACK-only based feedback, different cyclic shifts are used to differentiate positive SR only and positive SR multiplexing with NACK-only feedback.

(NEC) Proposal 2:

  - Support Alt1 with some modifications for the case when PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority.

  - Alt1: up to two TBs of NACK-only based feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

    - PUCCH resources configured for unicast will be used in concerned cases subject to legacy procedure as Rel-15/16. The UE determines the PUCCH resource within PUCCH-Config/PUCCH-ConfigurationList configured for unicast by the PRI indicated in the GC-DCI.

    - For the case of more than two TBs of NACK-only, [FFS: SR] is dropped.

(CMCC) Proposal 2:

  - When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

(Ericsson) Proposal 17:

  - For the case when SR is PUCCH F1, NACK-only is PUCCH F1 or SR is PUCCH F1, NACK-only is PUCCH F0, for positive SR, and the NACK-only feedback consists to up to 2 bits, NACK-only is transformed into ACK/NACK feedback and transmitted on SR PUCCH resource.

(Ericsson) Proposal 18:

  - For the case when SR is PUCCH F0, NACK-only is PUCCH F0, or SR is PUCCH F0, NACK-only is PUCCH F1, for positive SR, and the NACK-only feedback consists to up to 2 bits, NACK-only is transformed into ACK/NACK feedback and transmitted on SR resource with an increased CS.

(Ericsson) Proposal 19:

  - For negative SR, NACK-only is transmitted on NACK-only PUCCH resource regardless the PUCCH format of SR and NACK-only.

(Ericsson) Proposal 20:

  - when NACK only PUCCH resources carry feedback of more than 2 bits conflict with positive SR, delay the SR transmission to the next available SR opportunity

### Round-1

FL’s Comments:

For convenient comparison of how much spec impact is for solving NACK-only collides with SR, it is good to first summarize what is specified for ACK/NACK-based feedback collides SR for legacy procedure.

Either PUCCH format 0 (PF0) or PUCCH format 1 (PF1) is configured for SR transmission. UE transmits a PUCCH only when the UE transmits a positive SR by setting  for PF0 and for PF1.

If UE would transmit a positive SR and PF0 for HARQ-ACK, the UE transmits the PUCCH in the resource using PF0 in PRB(s) for HARQ-ACK information by setting  different values from that used for PF0 for HARQ-ACK. If the UE would transmit a negative SR and PF0 for HARQ-ACK, the UE transmits the PUCCH in the resource using PF0 for HARQ-ACK information using the same for PF0 for HARQ-ACK.

If a UE would transmit PF0 for SR and PF1 for HARQ-ACK in a slot, the UE transmits only the PF1 for HARQ-ACK and SR is dropped. If the UE would transmit a positive SR using PF1 and a PF1 for HARQ-ACK in a slot, the UE transmits a PF1 for HARQ-ACK in the resources for PF1 SR. If a UE would not transmit a positive SR using PF1 and a PF1 for HARQ-ACK in a slot, the UE transmits a PF1 for HARQ-ACK in the resources for PF1 HARQ-ACK.

The above cases for multiplexing SR and HARQ-ACK are summarized in Table 1. Note that there are other cases of multiplexing, e.g., PF2/3/4 for HARQ-ACK and PF0/1 for SR that are not summarized in the table, because these are not the case of multiplexing NACK-only and SR since only PF0/1 are supported for NACK-only PUCCH.

Table 1: Legacy procedure for multiplexing SR and HARQ-AK for PF0/PF1.

|  | PF0 for SR | PF1 for SR |
| --- | --- | --- |
| PF0 for HARQ-ACK | For positive SR: UE transmits PF0 in the resources for HARQ-ACK by setting different values (3/0, 9/1, or 1/00, 4/01, 7/11, 10/10) from that used for PF0 for HARQ-ACK (0/0, 6/1, or 0/00, 3/01, 6/11, 9/10). For negative SR: UE transmits PF0 HARQ-ACK in the resources for HARQ-ACK using the same for PF0 for HARQ-ACK. |  |
| PF1 for HARQ-ACK | Drop SR | For positive SR: UE transmits PF1 for HARQ-ACK in the resources for PF1 SR. For negative SR: UE transmits PF1 for HARQ-ACK in the resources for PF1 HARQ-ACK. |

After several rounds of discussions in RAN1#107-e meeting, the last updated proposal from RAN1#107-e meeting was to support one of the alternatives identified as follows:

The last updated proposal in RAN1#107-e meeting: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support [Alt1] from the following alternatives: - Alt1: up to two TBs of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - PUCCH resources configured for unicast will be used in concerned cases subject to legacy procedure as Rel-15/16. - For the case of more than two TBs of NACK-only, [FFS: SR] is dropped. - Alt2: - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR; - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), [FFS: SR] is dropped. - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only.

The main differences between the two alternatives are whether first transforming NACK-only into ACK/NACK and using the PUCCH resources configured for unicast accordingly for the transmission when applicable per the procedure as R15/R16.

From the submitted proposals for this meeting, most companies expressed the views or suggested further updates based on the last proposal from RAN1#107-e and the majority is supporting Alt1 with some different views on what PUCCH resource is used for the transmission, e.g., always use PUCCH resource for SR, or use separately configured PUCCH resources.

Unlike the case of NACK-only transmission colliding with other UCI/PUSCH for which NACK-only transformed into ACK/NACK and then using PUCCH resources configured for unicast seems only practical because NACK-only resource is either PF0 or PF1 (and even configured to be shared with other UEs) which cannot accommodate the multiplexed UCI or UL-SCH data. If network configured PUCCH resources for NACK-only which is probably shared with other UEs, the resources is supposed to be used as much as possible for benefits of reducing PUCCH resources overhead or burden of detecting ACK/NACK. Then Alt2 is beneficial.

Alt1 not only needs to specify transforming NACK-only into ACK/NACK bits but also needs to specify which PUCCH resources will be used for the transmission. From moderator’s perspective, it does not seem obvious which Alternative has less specification impact. Given Alt1 has the majority support, which can be the starting point for this meeting and see whether/how the concern to support this alternative.

FL’s Proposal:

#### Proposal 3.1.1

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following:

  - For up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

    - PUCCH resources configured for unicast will be used subject to legacy procedure as Rel-15/16.

  - For the case of more than two TBs of NACK-only, SR is dropped.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support the proposal with minor changes. #### Proposal 3.1.1 When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two TBs/HARQ-ACK bits, of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of as Rel-15/16. - PUCCH resources configured for unicast will be used subject to legacy procedure as Rel-15/16. - For the case of more than two TBs of NACK-only, SR is dropped. |
| ZTE | We are generally fine with the proposal with the following clarifications. When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - PUCCH resources configured for unicast will be used subject to legacy procedure as Rel-15/16. - FFS the PUCCH resources, e.g, PUCCH resource for SR, or separately configured PUCCH resources - For the case of more than two TBs of NACK-only, SR is dropped. |
| Samsung | Do not support. We have been supportive of transforming NACK-only to ACK/NACK but, based on input from other Tdocs, we now realize that would not be a proper solution as PUCCH resource overhead, which was supposed to be avoided (hence the NACK-only), is moved to the SR resources. We prefer to leave the UE behavior undefined for the above case. The overall importance is insignificant as its probability is minor (incorrect TB reception + positive SR) while the demands on the NW are huge. If the UE has NACK and positive SR, the UE can choose whether to request TB retransmission (postpone SR) or whether to send the SR and deal with the incorrect TB reception – there is no impact on the NW. |
| CMCC | For the case of more than two TBs of NACK-only, it can be transformed into ACK/NACK based HARQ-ACK feedback as proposal 3.2.1, which may use PF 2/3/4 to transmit and legacy SR and HARQ-ACK multiplexing rule can be reused. |
| Lenovo, Motorola Mobility | We agree with the intention. One thing not clear to us is which resource is used for carrying the up to two bits? Is it SR resource or PUCCH resource configured for NACK-only? |
| vivo | Not support. If for the case of up to two TBs/HARQ-ACK bits of NACK-only feedback, NACK only can be transformed into ACK/NACK mode and PUCCH resources configured for unicast will be used subject to legacy procedure as Rel-15/16, we are wondering why the same rule can’t be applied for the case of more than two TBs of NACK-only. |
| NEC | Support this proposal generally. For the PUCCH resource selection and multiplexing between transformed ACK/NACK and SR, the order should be selected the PUCCH resource for transformed ACK/NACK first and then do the multiplexing operation between ACK/NACK and SR as legacy procedure as Rel-15/16, since the legacy procedure for multiplexing SR and ACK/NACK can appoint the final PUCCH resource, and we should ensure that the PUCCH resource should be UE-specific for case ACK/NACK with PF0, or ACK/NACK with PF1 and SR with PF1(the PUCCH resource for ACK/NACK will be used for transmitting ACK/NACK+SR for these cases). Thus we suggest modifying the proposal as: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - PUCCH resources configured for unicast will be used for transformed ACK/NACK subject to legacy procedure as Rel-15/16. - For the case of more than two TBs of NACK-only, SR is dropped. |
| NTT DOCOMO | We are generally fine with the proposal. |
| Qualcomm | In case of more than 2 TBs/HARQ-ACK bits, always dropping SR may not be a reasonable, considering NACK-only feedback may not be as urgent as SR. |
| Apple | We share the same view as CMCC and vivo. |
| Ericsson | Support |

## Multiple NACK-only without collision with other UCI (H)

Submitted Proposals

NACK-only->ACK/NACK

(Huawei) Proposal 4:

  - When more than one NACK-only feedback are available for transmission in the same PUCCH slot,

    - define up to 16 orthogonal PUCCH resources to select from per PUCCH resource set according to combinations of up to 4 TBs with NACK-only feedback.

    - for more than 4 TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

      - The PUCCH resource to be used for the transmission is based on the last DCI scheduling multicast

(ZTE) Proposal 2:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot, support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(vivo) Proposal 9:

  - When more than one NACK-only based feedback is available for transmission in the same PUCCH slot, Alt 1 is supported.

    - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(Nokia) proposal 2:

  - When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI:

    - define up to 4 orthogonal PUCCH resources to select from according to combinations of up to 2 TBs with NACK-only feedback,

    - for more than 2 TBs with NACK-only feedback and for more than 1 TB related with more than 1 G-RNTI, all NACK-only feedback are transformed into ACK / NACK based feedback, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

      - FFS: Which PUCCH resources UE uses.

(Nokia) proposal 3:

  - For more than B = 2 TBs with NACK-only feedback and for more than 1 TB related with more than 1 G-RNTI, the UE is configured by RRC signalling with specific dedicated PUCCH resource(s) that the UE utilizes to transmit multiplexed NACK-only bits, rather than selecting the PUCCH resource based on the PRI field of the last DCI.

    - Dedicated PUCCH resource(s) can be configured within the NACK-only or unicast PUCCH-config.

(Nokia) proposal 4:

  - For more than B = 2 TBs with NACK-only feedback and for more than 1 TB related with more than 1 G-RNTI, concatenation of the converted NACK-only bits is based on the ascending order of PUCCH resource IDs that are originally scheduled for NACK-only feedback.

(Spreadtrum) Proposal 1:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support Alt 1.

(Apple) Proposal 1:

  - Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(LGE) Proposal 5:

  - When different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

(LGE) Proposal 6:

  - For more than one NACK only based HARQ-ACK feedback changed to ACK/NACK based HARQ-ACK to be multiplexed, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI indicating PRI for NACK only based HARQ-ACK feedback based on PUCCH-config for ACK/NACK basesd HARQ-ACK feedback for multicast or PUCCH-config for unicast.

- Combination of NACK-only

(Samsung) Proposal 3: Support the following

- When more than one NACK-only feedback is available for transmission in the same PUCCH slot for a given G-RNTI,

  - define up to 15 [A] orthogonal PUCCH resources to select from according to combinations of up to 4 [B] TBs/HARQ-ACK bits with NACK-only feedback,

  - for more than 4 [B] TBs/HARQ-ACK bits with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

- (DOCOMO) Proposal 5:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support Alt 4.

    - If multiplexed payload size is over the maximum, bundling is applied.

(Qualcomm) Proposal 3:

  - For RRC_CONNECTED Ues, when more than one NACK-only based feedback are available for transmission in the same PUCCH slot,

    - for up to X TBs with NACK-only feedback, support Alt4 to select one from up to (2^X-1) PUCCH sequences based on cyclic shift/scrambling;

    - for more than X TBs with NACK-only feedback, support Alt1 to use a configured UE-specific PUCCH/PUSCH resource for multiplexing.

(OPPO) Proposal 1:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission (i.e., Alt 4).

(Lenovo) Proposal 10:

  - For multiple NACK-only feedback multiplexing in same slot, from per UE perspective, the PUCCH resource which corresponds to the first incorrectly decoded PDSCH is selected for transmitting the NACK-only feedback in the PUCCH slot.

(NEC) Proposal 3:

  - When more than one NACK-only feedbacks are available for transmission in the same PUCCH slot, the UE selects orthogonal PUCCH resources according to combinations of TBs with NACK-only feedback. If there are too many TBs with NACK-only feedback resulting insufficient resources, the UE does not report HARQ-ACK.

(CMCC) Proposal 1:

  - When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI,

    - define up to 16 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

    - for more than 4 TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, which UE reports HARQ-ACK as in Rel-16 using PUCCH resources for unicast determined by PRI in the last DCI. Otherwise, the UE does not report HARQ-ACK.

    - UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

(Ericsson) Proposal 3:

  - Alt4 is further clarified to mean that to convey N bits the UE transmits one selected PUCCH resource in a slot, from a bundle of M=2^N-1 configured PUCCH resources. The actually used PUCCH resource in a slot therefore carries the required N HARQ feedback information bits of data received with a single G-RNTI pointing to this UL.

(Ericsson) Proposal 4:

  - Change the wording of Alt4 to make it more clear: N HARQ feedback bits are mapped to one out of 2^N-1 PUCCH resources.

(Ericsson) Proposal 5:

  - Do not support Alt1 for the case when more than one NACK-only based feedback are available for transmission in the same PUCCH slot.

(Ericsson) Proposal 6:

  - When one or more NACK-only based feedback bits need be transmitted in same PUCCH slot, Alt 4 is supported.

- Other solution

(CATT) Proposal 4:

  - To simply the operation, UE is not expected to be scheduled more than one TBs of NACK-only feedback in the same PUCCH slot.

- “Last DCI”

(LGE) Proposal 7:

  - For multiplexing the NACK only based HARQ-ACK feedback for multicast and ACK/NACK-based HARQ-ACK feedback for unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(LGE) Proposal 8:

  - For multiplexing the NACK only based HARQ-ACK feedback for multicast and ACK/NACK-based HARQ-ACK feedback for multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI indicating PRI for ACK/NACK-based HARQ-ACK feedback for multicast.

### Round-1

FL’s Comments

For more than one NACK-only based feedback are available for transmission in the same PUCCH slot, two alternatives are still open based on the agreement reached in the last meeting:

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Defining combinations of NACK-only corresponding to a specific sequence or a PUCCH transmission as in Alt4 needs a limit because the number of PUCCH resources will be increased exponentially with the increase of the number of NACK-only for combinations. Some companies commented that Alt1 may discount the benefits of NACK-only because when NACK-only is configured the PUCCH resources for NACK-only are supposed to be used as much as possible. Hence, there could be a tradeoff between Atl1 and Alt4 for way forward, e.g., for more than four TBs with NACK-only feedback, Alt1 takes place. The combinations in Alt4 can be limited to four TBs with NACK-only.

This issue has been discussed in several meetings and the compromised solution in the last updated version in RAN1#107-e is as follows:

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI, - define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback, - for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK. - FFS: Which PUCCH resources UE uses. - FFS: values of A and B. - FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

The unclear points regarding the above proposal from the last meeting are mainly about the PUCCH resources and whether the proposal can be applied to the case of different G-RNTs.

When the resources for NACK-only are not shared with other Ues, it can be easily seen that network can differentiate which TB is not correctly decoded from the detected PUCCH resources. When the same PUCCH resources for NACK-only are configured to different Ues within the group, network can differentiate which TB is failed in decoding by indicating the same TB or TB combination to different Ues to report NACK-only in the same PUCCH slot by k1 included in the scheduling DCI, which can be done by configuring the same k1 set for different Ues in the group when configuring NACK-only based feedback per G-RNTI.

Some companies were concerned about the case of multiple G-RNTIs. Likewise, if all Ues are configured with the same set of G-RNTIs, the network can also differentiate which TB from which G-RNTI is failed in decoding by indicating the k1 which is common to all Ues in the group. On the other hand, Ues does not need to choose the PUCCH resources for the combinations of TBs from different G-RNTIs, which can be done by network indicating different k1 or PRI pointing to a different PUCCH resource set defined for NACK-only for different G-RNTIs, which can be up to Gnb implementation. Hence, the proposal works in general for a single or multiple G-RNTIs cases.

This issue has been discussed in several meetings and moderator suggests the last proposal from the last meeting can be the starting point and collecting concerns if any.

Regarding the number of [B] TBs and the number of [A] PUCCH resources, one company proposed 2 TBs corresponding to 4 PUCCH resources and some other companies proposed 4 TBs corresponding to 15/16 PUCCH resources. We can start the discussion from supporting 4TBs and collect concerns if any. Since UE only needs to transmit the PUCCH when some TB is failed in decoding, only 15 PUCCH resources needs to be defined to correspond to the combination of TBs.

FL’s Proposal:

#### Proposal 3.2.1

When more than one NACK-only feedback are available for transmission in the same PUCCH slot,

- define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

- for more than 4 TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

  - The PUCCH resource to be used for the transmission is based on the last DCI scheduling multicast.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| - Nokia, NSB | We support the intention of the proposal. As indicated by the FL, since different Ues may receive different combination of MBS services (different G-RNTIs), it would be hard to ensure a valid mapping between PUCCH resources to ACK / NACK combinations for all Ues, considering the PUCCH resources would be group-common, i.e., always the same number of TBs and the same TB combinations should be scheduled to different Ues that are to be transmitted in the same PUCCH slot. That would indeed require all the Ues receive the same G-RNTIs. If that is not the case, then the Gnb should ensure that Ues receiving different G-RNTIs are scheduled for feedback in different slots. It was observed that there are companies who cannot agree with this limitation in the previous meeting. In our view, Alt. 1 is a simple solution, however, we also agree that it is against the fundamental purpose of the NACK-only based HARQ-ACK feedback. In the light of the above observations, we propose a compromise solution where A and B are kept small, e.g., A = 4, B = 2, and the mapping configurations are made per G-RNTI. In addition, if TBs of different G-RNTIs are to be ACK / NACK-ed in the same slot, Alt. 1 shall be used. In addition, if the multiplexed feedback is to be transmitted using the PUCCH resource indicated by the last multicast DCI, considering last multicast DCI can be different for different services, it would make scheduling complex. Thus, PUCCH resources for that purpose should be configured by RRC signaling. Proposal: When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI: - define up to 3 orthogonal PUCCH resources to select from according to combinations of up to 2 TBs with NACK-only feedback, - for more than 2 TBs with NACK-only feedback or for more than 1 TB related with more than 1 G-RNTI, all NACK-only feedback are transformed into ACK / NACK based feedback, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK. - Dedicated PUCCH resources to transmit multiplexed feedback is configured by RRC signaling. |
| - ZTE | - Our first preference is to go with the previous Alt.1. But for progress, we are OK to go with the Proposal 3.2.1 if it is applicable to the multiple G-RNTIs case. - Another remaining issue for this proposal is the order of bits for the feedback. For example, if 4 TBs are received and needs to transformed into ACK/NACK bits, do we reuse the legacy mechanism to order these 4 bits? |
| - Samsung | Support the proposal. - One thing to note is that having multiple G-RNTIs for one UE is not an issue and is transparent to specifications (trivial to handle by the NW – no different than having multiple G-RNTIs for respective multiple Ues). Supporting ACK/NACK feedback for multiple G-RNTIs is far more challenging and requires discussion regardless (and is probably unlikely to be supported in practice). |
| - CMCC | Support the proposal. |
| - Lenovo, Motorola Mobility | - We don’t support this proposal which brings more issues. - It is weird with second sub-bullet where PUCCH resources are sufficient for A/N based feedback when the number of TBs is larger than 4 while PUCCH resource is problem for A/N based feedback when the number of TBs is not larger than 4. We can’t agree with it. - From our side, when the number of TBs is larger than 4, HARQ-ACK bundling should be performed to guarantee the number of NACK-only feedback is not larger than 4. |
| - vivo | Not support. - Alt. 1 is preferred because it is simple. The proposal proposed to define up to 15 orthogonal PUCCH resources to select and for the case with more than 4 TBs, NACK-only feedback is transformed into ACK/NACK based and the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. However, in proposal 3.1.1, it is proposed that for up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. We think the two proposals are not aligned. It makes us confused. |
| - LG Electronics | We do not support this proposal. We prefer Alt 1 for simplicity. |
| - NEC | Support the proposal generally. For the value of A and B, we agree with FL that the number of PUCCH resources will be increased exponentially with the value of B increasing linearly. However, there is no need to ensure the resources from different UEs in a group are orthogonal(the gNB only needs to know which TBs need to retransmit for the group instead of which UE needs), thus these orthogonal PUCCH resources can represent the different combinations of TBs with NACK-only feedback. The case for PUCCH format 0 and PUCCH format 1 is different. Anyway, we have no intention to overturn the value of A and B suggested by FL. If most companies think the value given by FL is normal, we are also to go with it. |
| - Spreadtrum | We slightly prefer Alt1 for its simplicity. But we have no strong preference between Alt 1 and Alt4, and either one is fine to us. The compromised solution is too complex for UE’s implementation. We not understand why one unified solution could not be considered especially we have agreed to transfer NACK-only into ACK/NACK when multiplexing with UCI. |
| - NTT DOCOMO | We are fine with the proposal. |
| - Qualcomm | Ok in principle |
| - CATT | Not support this proposal. According to the discussion during last RAN1 meeting, for a given G-RNTI, both Alt.1 and Alt.4 have some issues to be addressed. Moreover, for multiple G-RNTI, it becomes more complicated when more than one NACK-only feedback corresponding to different G-RNTIs is transmitted in the same PUCCH slot. Considering the remaining time in Rel-17 maintenance phase, we prefer not support more than one TBs of NACK-only feedback in the same PUCCH slot. |
| - Apple | If NACK-only feedback transforms into ACK/ACK based feedback is supported, why do we need to define the PUCCH resources combination. It only makes the implementation and specification complicated. For simplicity, Alt1 is preferred. |
| - Ericsson | - We still think “NACK-only feedback is transformed into ACK/NACK” (Alt1 in earlier agreement) is useless in practice since it requires dedicated ACK/NACK PUCCH resources for all UEs or can only be opportunistically used when unicast is scheduled. This is in contradiction with underlying assumed use case for NACK-only, which is that such ACK/NACK PUCCH resources are not available (if they are available, why use NACK-only in the first place?). - We recognize the need to have a hard limit on the number of bits supported by the “2^N-1 orthogonal PUCCH resources” solution (Alt4 in earlier agreement) but think this should rather be 5 bits, drawing some motivation from the fact that a PUCCH resource set can have up to 32 PUCCH resources When the hard limit (be it 4 or 5 bits) is exceeded we think it is more in line with NACK-only assumptions to use NACK bundling, to support any number of bits, with details to be further discussed. - We could live with a compromise according to the FL’s proposal (with 5 bits instead of 4 bits as the hard limit), provided the agreement also includes the possibility for an optional configuration of NACK bundling. The functionality would thus be: - If NACK bundling is not configured, the functionality will be according to the FL’s proposal (although with 4 or 5 bits) - If NACK bundling is configured, the fallback from Alt4 would be to NACK bundling instead of ACK/NACK, if sufficient UE-specific PUCCH resources for ACK/NACK HARQ feedback are not available. |

## PUCCH resource (H)

Submitted Proposals

Sequence

(Huawei) Proposal 5:

  - Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

(Huawei) Proposal 6:

  - The sequence cyclic shift for NACK-only is .

(Huawei) Proposal 7:

  - For PF1 NACK-only, set .

(ZTE) Proposal 1:

  - RAN1 clarifies the sequence determination of NACK only PUCCH F0/F1.

(TD Teck) Proposal 1:

  - Support PUCCH format 2/3/4 for NACK-only based HARQ-ACK feedback.

(TD Teck) Proposal 2:

  - If PUCCH format 2/3/4 is used to feed back the NACK-ONLY information for an MBS session, N is configured as a part of the configuration information of the MBS session. UE maps the NACK information of 1 bit long onto a sequence of N>2 bits long. Then send the NACK information with PUCCH format 2/3/4.

(Ericsson) Proposal 7:

  - Use the PUCCH format 0 or format 1 phase rotations (Cyclic Shifts) and for format 1 the orthogonal (DFT) spreading as dimension in addition to OFDM-symbol and PRB.

Resource sets/resources

(Nokia) proposal 1:

  - For the separate PUCCH-Config/PUCCH-ConfigurationList configured to UE for NACK-only based feedback:

    - 1 PUCCH resource set in each PUCCH-Config,

    - up to 32 PUCCH resources in each PUCCH resource set are configured.

    - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

(LGE) Proposal 4:

  - Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

(Lenovo) Proposal 11:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for NACK-only based feedback can’t be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(Lenovo) Proposal 12:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for NACK-only based feedback can’t be shared with PUCCH resource configuration for ACK/NACK based feedback for multicast.

(Lenovo) Proposal 13:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(Ericsson) Proposal 8:

  - PUCCH configuration for NACK-only contains a single PUCCH resource set

(Ericsson) Proposal 9:

  - The gNB has to ensure that it does not transmit more than r Nmax=log2(Z+1) transport blocks with dl-DataToUL-ACK pointing to the same uplink slot.

(Ericsson) Proposal 10:

  - The PRI in the DCI is interpreted by Ues in NACK-only mode to select one of the PUCCH resource bundles configurations for NACK-only.

(Ericsson) Proposal 11:

  - For PUCCH config for NACK-only, increase maxNrofPUCCH-ResourcesPerSet to 128.

(Ericsson) Proposal 12:

  - A UE configured with NACK only is always configured with a separate PUCCH configuration for NACK only if there is more than one NACK-only feedback bit.

(Ericsson) Proposal 13:

  - For a given UE, feedback for different G-RNTIs are TDM’ed in different UL slots.

(Ericsson) Proposal 14:

  - Each UE can be configured with one or more sets of PUCCH resource pools, where each set corresponds to one G-RNTI.

(Ericsson) Proposal 15:

  - When a UE needs to send NACK-only feedback for multiple G-RNTIs, it transmits this in multiple UL slots, with one slot per G-RNTI. The particular PUCCH resource to be transmitted corresponds, in each UL slot, to the NACK-only N-bit combination that applies for the G-RNTI associated with that UL slot to send HARQ feedback for N TBs of the G-RNTI.

(Ericsson) Proposal 16:

  - When the same UL slot is indicated for NACK-only feedback for more than one G-RNTI that the UE is receiving, the UL PUCCH conflict is resolved by delaying feedback of conflicting G-RNTIs to later UL slots, so that the transmitted PUCCH resource in each UL slot only carries HARQ feedback for one G-RNTI.

    - Delayed UL feedback is sent in first available UL slot

    - FFS details

- Power control for NACK-only

(OPPO) Proposal 2:

  - For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

### Round-1

FL’s Comments

As discussed in some contributions, what sequence is used and what modulated symbol is assumed for PF1 needs to be clarified. Since the NACK-only resource might be shared by UEs, a single sequence or a single bit for the modulated symbol is more beneficial for network decoding.

The number of PUCCH resource sets and the number of PUCCH resources depend on the decision for multiple NACK-only PUCCH transmission in the same PUCCH slot.

As discussed in previous meetings, to correspond to the proposal of multiple NACK-only transmission in the same PUCCH slot, the proposal from RAN1#107-e seems to converge and can be the starting point. The main comment received in the last meeting was extending the number of PUCCH resource in the resource set from 16 to 32 which is the same as the ACK/NACK based feedback. However, based on the proposal of 15 orthogonal PUCCH resources according to combinations of up to 4 TBs/HARQ-ACK bits with NACK-only feedback, keeping the same number of  32 PUCCH resources per set as that defined for ACK/NACK based feedback is not necessary leads to more specification efforts of associating the combinations of TBs with NACK-only with the PUCCH resource ID and also potential more blind decoding attempts from network.

FL’s Proposal:

#### Proposal 3.3.1-1

Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

  - The sequence cyclic shift for NACK-only is mcs=0.

  - For PF1 NACK-only, set b0=0.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support the proposal. |
| ZTE | We support the above proposal. |
| Samsung | Support. |
| CMCC | support |
| Lenovo, Motorola Mobility | OK |
| vivo | support |
| NEC | Support |
| Spreadtrum | Support |
| NTT DOCOMO | Support |
| CATT | Support |
| Ericsson | Not support 1. bullet: Although this would be the most robust use of PF0/PF1 for NACK-only HARQ feedback, we think this is unnecessarily restrictive. We do not see that interference between any pair of cyclic shifts is so strong that the error probability on the NACK-only feedback becomes inacceptable. For legacy ACK/NACK there is a flexibility for the network to configure how many cyclic shifts are allocated, depending on e.g. channel conditions. Furthemore, first we need to clarify whether the intention is that cyclic shift for NACK-only can be multiplexed with legacy cyclic shifts for ACK/NACK. If such multiplexing is supported, then supporting only one or two cyclic shifts for NACK-only could be acceptable, because the further cyclic shift opportunities can still be used for legacy ACK/NACK. Two bits for NACK-only could easily be supported using e.g. four Cyclic Shifts equally spread among the legacy 12 CSs. This would mean that for each used CS there is a gap of two “empty” CSs between each pair of adjacent CSs. We think the number of bits should be configurable to be 1 or 2. When more than 1 cyclic shift for NACK-only is supported per PF0/PF1 then a decision needs to be made how to combine the cyclic shifts of different resources. If e.g. 6 shifts are supported per resource then a single resource can still support only 2 bits, but 6 resources can support floor(log2(6*6-1))=5 bits. The 2. bullet of the proposal is ok with us. For PF1 we also need to agree how many and which orthogonal sequences are supported out of the total amount that is determined by the PUCCH length in number of OFDM symbols (NsymbPUCCH,1) . We propose that all the possible sequences from TS 38.211 shall be supported. |

#### Proposal 3.3.1-2

For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback,

  - 1 PUCCH resource set in each PUCCH-Config.

  - up to 16 PUCCH resources in each PUCCH resource set

  - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support with change in number of resources in PUCCH resource set. The reason why the proposal is made with 16 is that the initial proposal on multiplexing more than one NACK-only feedback included the mapping of 16 different ACK / NACK combinations to 16 orthogonal PUCCH resources. Even if that behaviour gets agreed, it applies only in case there are multiple NACK-only feedback available in a slot. When there is no such situation, i.e., only one NACK-only to be transmitted, keeping 32 PUCCH resource options to select from would keep the scheduling flexibility that the gNB currently has and minimizes the spec impact. #### Proposal 3.3.1-2 For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - 1 PUCCH resource set in each PUCCH-Config. - up to 1632 PUCCH resources in each PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback. |
| ZTE | We are tend to agree with Nokia, up to 32 PUCCH resources in the first PUCCH resource set is aligned with the legacy handling. |
| Samsung | Support. We do not see relevance with having 32 PUCCH resources – in Rel-15, having 32 resources was for prior to RRC configuration of PUCCH resources and only for 1-2 bits. Also, although after RRC configuration, more than 8 resources for unicast could be provided for 1-2 bits, that is also unrelated to 32 resources. If more than one set of 16 PUCCH resources is needed, that may be separately discussed. Rel-15 allows for a maximum of 128 resources per set but the current situation for NACK-only is not same. |
| CMCC | We also prefer to keep 32 PUCCH resources in the first PUCCH resource set. |
| Lenovo, Motorola Mobility | We are not sure why “up to 16 resources in each PUCCH resource set”. From our side, we think up to 16 resources are needed only for the first PUCCH resource set instead of “each”. |
| NEC | Support. We think 16 PUCCH resources in the PUCCH resource set for NACK-only is enough, and the different CS can be used for the TBs combination with NACK-only feedback. |
| Spreadtrum | Support. |
| NTT DOCOMO | We tend to agree with Nokia. |
| MediaTek | Ok with the proposal. |
| CATT | Similar view with Nokia, we prefer keep 32 PUCCH resources in the first PUCCH resource set.. |
| Apple | Ok with the proposal. |
| Ericsson | With 15 orthogonal PUCCH resources in Proposal 3.2.1, this proposal would be OK, but as we have commented on Proposal 3.2.1 we suggest to use 31 PUCCH resources to support 5 NACK-only bits. The number of PUCCH resources needs to be the same on Proposal 3.2.1 and this proposal. Furthermore, as described in our Tdoc, we propose to support the use of PRI. If 5 bits are supported and there are only 31 resoruces in a PUCCH resource set, then there is no opportunity to switch between PUCCH resource by means of the PRI. Therefore, we have proposed to support up to 128 PUCCH resources in the 1 PUCCH resource set. |

## NACK-only codebook

Submitted Proposals

- (DOCOMO) Proposal 6:

  - For NACK-only based HARQ-ACK feedback, support dynamic HARQ-ACK codebook.

(Ericsson) Proposal 1:

  - For NACK-only transmission of HARQ feedback for group scheduling, both semistatic and dynamic codebooks are supported

(Ericsson) Proposal 2:

  - The NACK-only codebook for a G-RNTI is constructed independently from codebooks constructed for other G-RNTIs that the UE may also receive and from the codebook that includes feedback for PDSCH scheduled by C-RNTI.

### Round-1

FL’s Comments

Normally HARQ-ACK codebook including semi-static and dynamic codebook is for ACK/NACK based feedback and whether there is codebook concept for NACK-only is not common understanding.

However, when associating the combination of TBs with NACK-only with the PUCCH resource ID, how to determine the number of TBs, there could be two options.

Option1: based on the k1 set and TDRA table configured by higher layer signaling.

Option2: based on C-/T-DAI included in the scheduling DCI.

This discussion can start from these two options and discuss whether need to down-select and which option is to be down-selected.

FL’s Proposal:

#### Proposal 3.4.1

For determining the number of TBs with NACK-only that corresponds to a PUCCH resource ID,

  - Option1: based on the k1 set and TDRA table configured by higher layer signalling.

  - Option2: based on C-/T-DAI included in the scheduling DL DCI.

  - FFS: whether/how to down select

Collect views:

| Company | Comments |
| --- | --- |
| FL’s suggestion | Companies can also express the view about the FFS. |
| Nokia, NSB | We prefer Option-2. In our view, a mapping between ACK/NACK combinations to PUCCH resource IDs should be configured with RRC signalling. The UE can directly use the mapping, in case there are multiple NACK-only feedback scheduled for the same slot. As we stated previously, the mapping should be per G-RNTI. In our view, the intention of the proposal is to make Option-1 a Type-1 codebook behavior and Option-2 a Type-2 codebook approach. In our view, k1/TDRA based approach has significant overhead that would always initiate NACK-only to ACK/NACK conversion and force usage of UE-specific resources. Thus, we prefer option-2. |
| ZTE | From our perspective, if NACK-only feedbacks are transformed into ACK/NACK based feedbacks. One type of feedback mode should be configured so that the UE can generate the codebook based on this type. From this perspective, the NACK-only should not be a separate feedback mode. Thus, the proposal should be updated as following. Propsal: For determining the number of TBs with NACK-only that corresponds to a PUCCH resource ID, UE follows the HARQ-ACK feedback mode (type-1 or type-2 codebook) defined for multicast. |
| Samsung | Option 2 (option 1 will practically result to certain NACK transmissions as some TDRA entries will not have PDSCH and there is no DCI dependence for Type-1). Note that there is no T-DAI as there is no CA for multicast. |
| CMCC | Support Option 2, from our perspective, NACK-only based HARQ feedback is much suitable for type-2 like HARQ-ACK codebook. For type-1 codebook, UE also needs to feedback NACK even doesn’t detect the PDCCH which causes the huge HARQ-ACK feedback overhead. |
| Lenovo, Motorola Mobility | To decide which option, we need to make it clear which HARQ-ACK codebook type is configured? Is it Type-1 or Type-2? |
| Spreadtrum | Is it for type-1 codebook or type-2 codebook, or both ? |
| NTT DOCOMO | We support Option 2. We think Option 1 is less useful because in many cases it will require processing of multiple NACK-only feedback regardless of the number of PDSCHs actually received. |
| Qualcomm | If Option 2 is selected, does it mean Type-1 codebook cannot be configured for NACK-only-based feedback? |
| MediaTek | Share the similar question with spreadtrum. |
| CATT | We have similar questions with Lenovo and Spreadtrum, the type of CB that can be configured should be clarified first. |
| Apple | We share the similar view with ZTE. The current proposal seems to define the TB combination for a PUCCH resource. The issue is still discussing in Proposal 3.2.1 |
| Ericsson | We wish to support both options. We also think the proposal should be expressed as Type I and Type II codebooks instead. |

# HARQ-ACK feedback common

## Text proposals

### TP for HARQ-ACK feedback priority

#### TP 4.1.1.1 (CMCC)

<Unchanged text is omitted> If a UE is provided pucch-Config-Multicast1 or pucch-Config-Multicast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-Config-Multicast1 or pucch-Config-Multicast2 for each G-RNTI or G-CS-RNTI that the UE provides associated HARQ-ACK information according to the first HARQ-ACK reporting mode or second HARQ-ACK reporting mode. <Unchanged text is omitted>

##### Round-1

| Company | Your comments/suggestions to the text proposal TP 4.1.1.1 |
| --- | --- |
| - ZTE | - OK, or can leave it to editor. |
| - Samsung | - OK |
| - vivo | - OK. Editorial correction. |
| - Qualcomm | - ok |
| - CATT | - OK |
| - Lenovo, Motorola Mobility | - OK |
| - Ericsson | - Support |

### TP for HARQ-ACK feedback timing

#### TP 4.1.2.1 (CMCC)

TS 38.213 section 9.2.3 <Unchanged text is omitted> For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a unicast DCI format, other than DCI format 1_0 or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1. For DCI format 4_1, the PDSCH-to-HARQ_feedback timing indicator field values are provided by dl-DataToUL-ACK-MulticastDciFormat4_1 or, if dl-DataToUL-ACK-MulticastDciFormat4_1 is not provided, by {1, 2, 3, 4, 5, 6, 7, 8}. For DCI format 4_2, the PDSCH-to-HARQ_feedback timing indicator field values are provided by dl-DataToUL-ACK. <Unchanged text is omitted>

##### Round-1

| Company | Your comments/suggestions to the text proposal. |
| --- | --- |
| - ZTE | - OK, or can leave it to editor. |
| - Samsung | - OK |
| - vivo | - OK |
| - Qualcomm | - ok |
| - CATT | - OK. To align the above expression, the text can be rewording as following: For DCI format 4_2, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK. |
| - Lenovo, Motorola Mobility | - Does it imply DCI format 4-2 is always configured with same K1 set with unicast? |
| - Ericsson | - We wish to support both options. We also think the proposal should be expressed as Type I and Type II codebooks instead. |

#### TP 4.1.2.1.1 (CMCC)

TS 38.213 section 9.1.2.1: <Unchanged text is omitted> e) If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c - if the UE is not provided type1-Codebook-Generation-Mode = ‘mode1’, K1 is additionally provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCI Format4_1 - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4_1, K1 is provided by the union of dl-DataToUL-ACK and slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - if the UE is provided type1-Codebook-Generation-Mode = ‘mode1’, the UE - determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM <Unchanged text is omitted>

##### Round-1

| Company | Your comments/suggestions to the text proposal. |
| --- | --- |
| - ZTE | - OK, or can leave it to editor. |
| - Samsung | - OK |
| - Qualcomm | - ok |
| - CATT | - OK |
| - Lenovo, Motorola Mobility | - Here, is dl-DataToUL-ACK same with dl-DataToUL-ACK for unicast? |
| - Ericsson | - Support |

## enable/disable HARQ-ACK (H)

Submitted Proposals

- (CATT) Proposal 1:

  - The “Enabling/disabling HARQ-ACK feedback indication” field can be included in DCI format 4_1.

- (CATT) Proposal 2:

  - UE shall ignore the Enabling/disabling HARQ-ACK feedback indication field if it does not support DCI indicating.

- (CATT) Proposal 3:

  - For multicast SPS activation/deactivation, HARQ-ACK feedback can be always enabled regardless of the RRC configurations.

- (Lenovo) Proposal 1:

  - Support one-bit enabling/disabling HARQ-ACK feedback indication in multicast DCI format 4-1.

- (MediaTek) Proposal 3:

  - Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

- (MediaTek) Proposal 4:

  - A new field of “enabling/disabling HARQ-ACK feedback indication” should be defined for DCI format 4_1.

- (MediaTek) Proposal 5:

  - A new field of “enabling/disabling HARQ-ACK feedback indication” should be defined for multicast DCI format scrambled by G-CS-RNTI.

(Samsung) Proposal 12:

  - Whether or not a UE provides a HARQ-ACK report for SPS PDSCH activation/release is configured to the UE by the serving gNB.

(Spreadtrum) Proposal 2:

  - For multicast SPS activation/deactivation PDCCH, support ACK/NACK based feedback, and the ACK/NACK based feedback can be enabled/disabled.

- (Lenovo) Proposal 2:

  - For group-common SPS configuration, ACK/NACK feedback is supported for confirming reception of activation/deactivation DCI.

- (Lenovo) Proposal 3:

  - For group-common SPS configuration, HARQ-ACK feedback enabling/disabling is independently configurable from that for SPS PDSCH without PDCCH

- (Lenovo) Proposal 4:

  - For group-common SPS configuration, ACK/NACK feedback for confirming reception of activation/deactivation DCI can’t be disabled.

- (Lenovo) Proposal 5:

  - For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK/NACK for the SPS PDSCH.

- (MediaTek) Proposal 6:

  - For multicast SPS activation/activation, disabling the HARQ-ACK feedback is not supported.

- (Ericsson) Proposal 21:

  - UE can be configured to monitor or ignore the enabling/disabling bit in group-common DCI.

    - UE configured to ignore the DCI indication for HARQ feedback type rely on the feedback type configured in RRC

    - UE configured to monitor the DCI indication for HARQ feedback type use the indicator in DCI to select one of two possible feedback types out of the possible three.

- (Ericsson) Proposal 22:

  - The UE is only expected to follow the enable / disable indicator for HARQ feedback in group-common DCI when the two following conditions are fulfilled:

    - If it is configured to use the enable/disable indicator present in group common PDCCH

    - If the HARQ feedback codebook type is of type 2.

### Round-1

FL’s Comments

The proposals submitted includes two aspects, whether the enabling/disabling HARQ-ACK feedback is included in DCI format 4_1 and whether the HARQ-ACK for multicast SPS activation/deactivation can be enabled/disabled. Both issues have been discussed for a couple of meetings with multiple rounds but no consensus. From moderator’s perspective, they are not the issues have to be solved and reaching consensus in maintenance phase is doubtful.

Another issue is regarding the size of DCI format 4_2 which seems pressing to be solve. Since it is group common DCI to be received by a group UE but whether UE is configured with DCI indicating the enabling/disabling HARQ-ACK is per UE, there could be the case that some UEs are configured but some other UEs are not. To make the DCI size for all the UEs in the group, there could be some options:

Option1: Enabling/disabling HARQ-ACK feedback indication is always included in DCI format 4_2. Update is needed to TS38.212.

Option2: Whether Enabling/disabling HARQ-ACK feedback indication is included in DCI is configured by another parameter. A new parameter is needed. When it is configured, for UE not supporting DCI indicating enabling/disabling HARQ-ACK or for UE supporting but when UE is not configured with DCI indicating enabling/disabling HARQ-ACK, UE just ignores the indication. For this option, update is also needed to TS38.212.

FL’s Proposal:

#### Proposal 4.2.1

For “enabling/disabling HARQ-ACK feedback indication” in DCI format 4_2,

  - Option1: this indication is always included in DCI format 4_2. It is a reserved field for UEs not supporting or for UE not configured with group-common DCI indicating HARQ-ACK enabled/disabled.

  - Option2: defining another RRC parameter configuring whether DCI format 4_2 includes this indication for all the UEs. When it is included in DCI, it is a reserved field for UEs not supporting or for UE not configured with group-common DCI indicating HARQ-ACK enabled/disabled.

  - FFS: which option is down-selected

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the proposal. Option-2 is more flexible. gNB can configure the field, if enabling/disabling dynamicity by DCI is needed. |
| ZTE | We support Option1. If this field is always in the DCI, the additional RRC signaling in Option2 is not necessary. |
| Samsung | Option 2. No apparent reason for option 1 as a NW (or even any UE) may not support the feature – same approach as for optional unicast fields. |
| CMCC | Option 2 |
| Lenovo, Motorola Mobility | Not support. We propose “enabling/disabling HARQ-ACK feedback indication” is also included in DCI format 4_1. Otherwise, when DCI format is missed, how can UE know this missed DCI is DCI 4-1 or 4-2? How can UE know the missed DCI enables or disables HARQ-ACK feedback? |
| vivo | Option 2. |
| LG Electronics | We prefer Option 1. |
| NEC | Support option 1. |
| Spreadtrum | Option 2 |
| NTT DOCOMO | We prefer Option 2. |
| Qualcomm | Option 1 is not preferred. In 8.12.1, we are discussing whether the size of DCI format 4_2 is configured by RRC. If yes, it can solve the ambiguity issue of configurable fields including the dynamic enabling/disabling. |
| MediaTek | Prefer Option 1. |
| CATT | Prefer Option 2 due to it is more flexible for network. |
| Apple | Option 2 is preferred. |
| Ericsson | We agree with further discussing these two options. We have a preference for Option 2, since we think it is unnecessary to transmit this DCI bit in cases the functionality is not configured. |

## Codebook for disabled HARQ-ACK

Submitted Proposals

“impact to codebooks”

(Huawei) Proposal 8:

  - For a given multicast with G-RNTI configured with HARQ-ACK disabled by RRC signaling, for Type-1 codebook generation,

    - if UE does not detect any PDSCH for unicast nor other multicast PDSCH with different G-RNTI with HARQ-ACK enabled by RRC signalling for the same PUCCH slot, UE does not report any HARQ-ACK information in the PUCCH slot; otherwise,

    - UE will constantly report NACK-only for the multicast G-RNTI with HARQ-ACK configured to be disabled regardless of decoding results of corresponding PDSCH.

(Huawei) Proposal 9:

  - UE is not expected to be configured with Type-1 HARQ-ACK codebook and to be configured with HARQ-ACK disabled by group-common DCI simultaneously.

(Huawei) Proposal 10:

  - For group-common DCI indicating HARQ-ACK enabled/disabled, for Type-2 codebook generation, UE expects the HARQ-ACK feedback for all PDSCHs to be transmitted in the same PUCCH slot are all enabled or all disabled.

- (ZTE) Proposal 5:

  - RAN1 clarifies how to handle HARQ-ACK enabling/disabling for type-1 codebook.

- (ZTE) Proposal 6:

  - Regarding DAI mismatch for type2 codebook, if RAN1 agrees to resolve this issue via specification, network performs separate DAI counting for “PDCCH indicating HARQ-ACK enabling” and “PDCCH indicating HARQ-ACK disabling”.

    - Note: No need to increase the DAI bits in DCI.

- (Samsung) Proposal 8:

  - A UE does not provide the Type-1 HARQ-ACK codebook if all multicast DCI formats that the UE detects indicate disabled HARQ-ACK report.

- (DOCOMO) Proposal 1:

  - Not include HARQ-ACK information disabled by DCI indication in the Type-2 HARQ-ACK codebook.

- (Intel) Proposal 2:

  - For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

- (Lenovo) Proposal 7:

  - Type-1 HARQ-ACK codebook and presence of 1-bit HEI in DCI should not be configured simultaneously.

“same enabled/disabled status for the same PUCCH slot”

- (MediaTek) Proposal 2:

  - The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

- (Ericsson) Proposal 23:

  - The enable/disable indicator in the PDCCHs scrambled by same G-RNTI which point to the same PUCCH for HARQ feedback must be the same.

### Round-1

FL’s Comments

How the codebook is generated for Type-1 and Type-2 and for RRC configuring or DCI indicating HARQ-ACK is disabled was discussed in RAN1#107-e, the views were divergent and the general comment was if HARQ-ACK is disabled UE should not report HARQ-ACK.

However, for example, for Type-1 codebook, the codebook size should not be affected by dynamic scheduling, so it makes sense that UE is not expected to be configured with Type-1 HARQ-ACK codebook and to be configured with HARQ-ACK disabled by group-common DCI simultaneously.

For the case of Type-1 codebook configured and HARQ-ACK disabled by RRC signaling for G-RNTI1, UE should not need to report anything when the codebook is constructed for this G-RNTI1 multicast only. However, if UE detected one PDSCH for unicast or one group-common PDSCH with another G-RNTI2 for which HARQ-ACK is enabled, the Type-1 codebook with the same priority from this UE should keep the same codebook size as if G-RNTI1 multicast is enabled so as to be aligned with network in case UE misses some scheduling DCIs. What should UE report for such a case? As agreed in NR NTN topic UE reports NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH, the same approach can be taken for NR MBS for this case.

For the case of Type-2 codebook configured, DAIs for unicast and multicast or for different multicast (G-RNTIs) if configured are independently counted and Type-2 HARQ-ACK codebook is constructed by sub-codebook for unicast appended by sub-codebook for multicast which can be a sub-codebook concatenating each sub-codebook of multicast configuration (G-RNTI) as shown in the following figure:

-	When the HARQ-ACK is disabled by RRC signaling per G-RNTI, UE does not need to report HARQ-ACK for the disabled G-RNTI and the Type-2 codebook can be deterministic and aligned between UE and network.

-	When the HARQ-ACK is disabled by group-common DCI, among the scheduled TBs with k1 pointing to the same PUCCH slot, if some TBs are indicated with HARQ-ACK enabled but others with HARQ-ACK disabled, UE still needs to report NACK for the TBs indicated with HARQ-ACK disabled, because network cannot differentiate which scheduling DCI is missed by UE otherwise. Alternatively, for all the TBs scheduled with k1 pointing to the same PUCCH slot, the status of “enabling/disabling HARQ-ACK feedback indication” in the scheduling DCI can be set to the same value, so that UE does not need to report HARQ-ACK for all TBs with the HARQ-ACK disabled, which can also address the C/T-DAI misalignment between different Ues.

FL’s Proposal:

#### Proposal 4.3.1-1

For UEs with multicast configured with HARQ-ACK disabled by RRC signalling for a given G-RNTI, for Type-1 codebook generation,

  - if UE does not detect any PDSCH for unicast nor other multicast PDSCH with different G-RNTI with HARQ-ACK enabled by RRC signalling for the same PUCCH slot, UE does not report any HARQ-ACK information in the PUCCH slot for the given G-RNTI; otherwise, UE will report NACK-only for the given G-RNTI with HARQ-ACK configured to be disabled regardless of decoding results of corresponding PDSCH.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Do not support the proposal. If the UE is configured by RRC signalling and HARQ-ACK for G-RNTI1 is disabled, there is no dynamicity in the scenario. If the UE does not report any feedback for G-RNTI1, there will be no ambiguity due to any DCI missing, as the Gnb would know that the UE does not report anything for G-RNTI1. We propose that the UE does not report any feedback, if HARQ-ACK is disabled by RRC signalling for that G-RNTI. |
| ZTE | We support this proposal. |
| Samsung | Do not support the proposal. It is strange that if a UE receives one unicast and no multicast, the Rel-16 fallback operation for unicast will change (no fallback) even though there was no multicast. A possible proposal should focus only on multicast and not affect the Rel-16 UE behavior for unicast. |
| CMCC | Support |
| Lenovo, Motorola Mobility | Not support. If UE is configured with disabled HARQ-ACK feedback, then UE shall not report HARQ-ACK feedback. |
| LG Electronics | We do not support this proposal. We think that UE with HARQ-ACK disabled by RRC signalling shall not report HARQ-ACK feedback for type 1 codebook generation. |
| Spreadtrum | Not support. We also think for type 1 codebook, UE with HARQ-ACK disabled by RRC signaling shall not report HARQ-ACK feedback. |
| NTT DOCOMO | Not support. We don’t see the need to report HARQ-ACK feedback for G-RNTI where feedback is disabled by RRC. |
| Qualcomm | More clarification is needed. |
| MediaTek | Not support. we are confused that why does the UE need to report the HARQ-ACK if the HARQ-ACK is disabled? |
| CATT | OK with this proposal. |
| Apple | Not clear why UE report NACK-only for the G-RNTI with disabled feedback. |
| Ericsson | Not support. G-RNTIs with HARQ-ACK disabled via RRC signaling should not be included in the Type I codebook, so feedback should never be transmitted for this G-RNTI. The Proposal is also unclear with respect to “NACK-only”. There is nothing else in the Proposal that restricts the use case to NACK-only HARQ feedback. Is the intention to say “[UE-specific] NACK HARQ feedback”? But if the UE sends UE-specific feedback, why not transmit the correct value, i.e. ACK in case of correctly received TB? |

#### Proposal 4.3.1-2 (H)

UE is not expected to be configured simultaneously with Type-1 HARQ-ACK codebook and with “enabling/disabling HARQ-ACK feedback indication” in DCI.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support. |
| ZTE | Maybe we should change the wording a little bit since the “enabling/disabling HARQ-ACK feedback indication” field may be always in the DCI format. Proposal: The“enabling/disabling HARQ-ACK feedback indication” in DCI is reserved if the UE is configured with Type-1 HARQ-ACK codebook for multicast. |
| Samsung | OK in principle but one issue with “UE does not expect …” statements in such cases is that the UE behavior is undefined and there is impact to the NW depending on what the UE does. It may be better to capture in the notes of the RRC parameters to RAN2 so that the misconfiguration is reflected in 38.331. |
| CMCC | Support |
| Lenovo, Motorola Mobility | Support. |
| vivo | ok |
| LG Electronics | OK |
| Spreadtrum | Ok |
| NTT DOCOMO | Support |
| Qualcomm | Ok in principle |
| MediaTek | Support. |
| CATT | OK |
| Apple | OK. |
| Ericsson | Support |

#### Proposal 4.3.1-3

For group-common DCI indicating HARQ-ACK enabled/disabled, for Type-2 codebook generation, UE expects the HARQ-ACK feedback for all PDSCHs to be transmitted in the same PUCCH slot are all enabled or all disabled for a given G-RNTI.

  - UE does not need to report HARQ-ACK for Type-2 codebook when DCI indicates HARQ-ACK is disabled.

Collect views/concerns:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support. |
| ZTE | From our perspective, this issue can be addressed by network implementation. However, if companies prefer to address this issue by specification, we propose the following way to allow the gNB to indicate different ‘enable’ or ‘disable’ for PDSCHs with the same PUCCH. For example, if five PDSCHs are transmitted {PDSCH#1(enable), PDSCH#2(disable), PDSCH#3(enable), PDSCH#4(disable), PDSCH#5(enable)}, the C-DAI value for PDSCHs indicated with ‘enable’ can be counted together. The C-DAI value for PDSCH indicated with ‘disable’ will be separately counted. In other words, there are two tracks of DAI counting, one for the DCI indicating ‘enable’ and another for the DCI indicating ‘disable’. Thus, the DAI value for the five PDSCHs will be the following: PDSCH#1(enable): C-DAI=1, T-DAI=1 PDSCH#2(disable): C-DAI=1, T-DAI=1 PDSCH#3(enable): C-DAI=2, T-DAI=2 PDSCH#4(disable): C-DAI=2, T-DAI=2 PDSCH#5(enable): C-DAI=3, T-DAI=3 For UEs configured as ‘disable’ by RRC, UE ignores all the DAI values; For UEs configured as ‘enable’ by RRC, UE combines C-DAI/T-DAI for both two tracks, i.e., C-DAI= (2+3)mod4=1, T-DAI= (2+3)mod4=1; For UEs configured as ‘dynamic indication’ by RRC, UE use the track of C-DAI/T-DAI for DCIs indicating ‘enable’. Proposal 6: Regarding DAI mismatch for type2 codebook, if RAN1 agrees to resolve this issue via specification, network performs separate DAI counting for “PDCCH indicating HARQ-ACK enabling” and “PDCCH indicating HARQ-ACK disabling”. Note: No need to increase the DAI bits in DCI. |
| Samsung | No apparent need for that proposal. So what if some DCIs indicate ‘enable’ and some DCIs indicate ‘disable’? All the specs need to say is that a UE does not report HARQ-ACK for a TB if the UE is indicated that HARQ-ACK for the TB is disabled. That would be same as in NTN where HARQ-ACK can be disabled for some HARQ processes but not for others. The rest is up to the NW and does not need to have spec impact for optimizations (the dynamic ‘enable/disable’ is anyway an optimization itself). |
| Lenovo, Motorola Mobility | Support. |
| vivo | We think the indication of “disabling/enabling” can be per PDSCH, for the DAI issue, we prefer to be addressed by gNB, |
| Spreadtrum | Generally we are fine. |
| NTT DOCOMO | We wonder if such a restriction is really necessary. It seems to us that the issue can be addressed by gNB implementation. |
| Qualcomm | It can be considered to let gNB avoid the case. |
| MediaTek | Ok with the proposal |
| CATT | Not support. Prefer this issue can be avoided by gNB implementation. |
| Apple | We share the views that this could be gNB implementation issue. In another way round, it could say UE doesn’t expect…. |
| Ericsson | Support |

## others

Submitted Proposals

- (MediaTek) Proposal 1:

  - The HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

- “retransmission vs. Codebook”

(LGE) Proposal 2:

  - HARQ-ACK to PTP retransmission is treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook.

- “retransmission vs. HARQ-ACK feedback mode”

(MediaTek) Proposal 7:

  - PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

(MediaTek) Proposal 8:

  - PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

# HARQ-ACK feedback for multicast SPS

## HARQ-ACK option for SPS multicast

### Text proposals

#### TP for ACK/NACK only for SPS activation

##### TP 5.1.1.1.1 (ZTE)

(ZTE) Proposal 9:

  - A GC-PDCCH activating a SPS transmission which has already been activated can be considered as a reactivation GC-PDCCH.

    - For the PDSCH associated with the reactivation GC-PDCCH, feedback information are generated for both feedback mode of SPS PDSCH and feedback mode of SPS PDSCH without PDCCH.

##### 18 Multicast Broadcast Services <Unchanged parts are omitted> A UE monitors PDCCH for scheduling PDSCH receptions or for activation/release of SPS PDSCH receptions for a corresponding SPS PDSCH configuration as described in clause 10.1.A UE can be configured by harq-Feedback-Option-Multicast for a G-RNTI or for a G-CS-RNTI to provide HARQ-ACK information for a transport block reception associated with the G-RNTI or with the G-CS-RNTI, respectively, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The second HARQ-ACK reporting mode is not applicable for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception and SPS PDSCH associated with the SPS activation. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. <Unchanged parts are omitted>

###### Round-1

| Company | Your comments/suggestions to the text proposal TP 5.1.1.1.1 |
| --- | --- |
| - Nokia, NSB | - Ok. |
| - ZTE | - The proposal and the TP are two different issues. - TP issue: - The TP is to cover the following agreements. Currently, the multicast SPS activation is not covered by the above spec. Agreement:(RAN1#106b) For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling. - FFS: Whether the configuration is per SPS configuration index or per G-CS-RNTI. - Note: Whether there is a UE capability for support of NACK-only based HARQ-ACK or not will be discussed as part of UE features discussion. Agreement:(RAN1#107-e) - For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported. - Proposal Issue: - Regarding the proposal, it is to address the issue on what kind of feedback mode should be used for the SPS retransmission. According to current agreements, only ACK/NACK feedback is supported for SPS activation/deactivation, while three feedback modes can be configured for SPS PDSCH without PDCCH, i.e., ACK/NACK, NACK-only, and disabled. - If type-2 codebook is configured for multicast, if the SPS re-activation is considered as the a normal SPS PDSCH without PDCCH, then the bit for this SPS re-activation (and its corresponding SPS PDSCH) will be located at the end of this codebook. If this re-activation is considered as a PDCCH activating SPS, then the location of this feedback bit should follow the DAI value indicated the DCI. Specification has to make it clear on how to generate the feedback for this SPS re-activation. - If the SPS re-activation PDCCH is missed, the UE can still decode the SPS PDSCH correctly. To handle this mis-detection issue, it is proposed to generate the feedback bit for SPS re-activation in both locations, i.e., 1 bit to consider it as the normal SPS PDSCH without PDCCH, another 1 bit to consider it as the SPS activation command. Proposal 9: - A GC-PDCCH activating a SPS transmission which has already been activated can be considered as a reactivation GC-PDCCH. - For the PDSCH associated with the reactivation GC-PDCCH, feedback information are generated for both feedback mode of SPS PDSCH and feedback mode of SPS PDSCH without PDCCH. |
| - Samsung | - OK in principle but not OK with the proposed text. - There is no such thing as “SPS PDSCH associated with the SPS activation”. A suggested text is “and the first SPS PDSCH reception after activation of SPS PDSCH”. |
| - Spreadtrum | - In principle we are fine with the TP, fine with text from Samsung. - For the proposal, the principle we are OK. Similar to the TP suggestion, the wording needs to be updated, e.g, ‘the PDSCH associated with the reactivation GC-PDCCH’ is not clear to us. |
| - CATT | - Ok with the TP. |
| - Lenovo, Motorola Mobility | - The TP is not clear to us. Does SPS PDSCH associated with the SPS activation mean the first SPS PDSCH after activation? |
| - Ericsson | - We do not think this is needed. For a given UE, it is enough to respond to the activation. |

#### TP for feedback option

##### TP 5.1.1.2.1 (CMCC)

for TS 38.214 section 18 <Unchanged text is omitted> A UE monitors PDCCH for scheduling PDSCH receptions or for activation/release of SPS PDSCH receptions for a corresponding SPS PDSCH configuration as described in clause 10.1. A UE can be configured by harq-Feedback-Option-Multicast for a G-RNTI or by sps-HARQ-Feedback-Option-Multicast for a G-CS-RNTI to provide HARQ-ACK information for a transport block reception associated with the G-RNTI or with the G-CS-RNTI, respectively, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The second HARQ-ACK reporting mode is not applicable for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. <Unchanged text is omitted>

###### Round-1

| Company | Your comments/suggestions to the text proposal TP 5.1.1.2.1 |
| --- | --- |
| - Nokia, NSB | - Support the proposal. |
| - ZTE | - OK, or can leave it to editor. |
| - Samsung | - OK. |
| - vivo | - OK |
| - Spreadtrum | - OK |
| - CATT | - OK |
| - Lenovo, Motorola Mobility | - OK |
| - Ericsson | - OK |

## Codebook for multicast SPS HARQ-ACK

Codebook related

(vivo) Proposal 1:

  - For the HARQ-ACK codebook for SPS PDSCHs regardless of unicast or multicast, UE adopts the same rule for codebook generation as defined in NR Rel-16.

(vivo) Proposal 2:

  - When HARQ-ACK for unicast SPS PDSCHs and multicast SPS PDSCHs are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

(vivo) Proposal 3:

  - When HARQ-ACK for unicast DG PDSCH and HARQ-ACK for multicast DG or SPS PDSCH are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList for unicast.

    - FFS when HARQ-ACK for unicast SPS PDSCH and HARQ-ACK for multicast DG PDSCH are multiplexed on the same PUCCH.

(Nokia) Proposal 8:

  - When NACK-only based HARQ-ACK feedback is used for multicast SPS PDSCH without PDCCH scheduling, the UE determines a priority index from the HARQ-ACK codebook index in the configuration for SPS multicast, using the same method with the one for ACK / NACK based feedback.

(Samsung) Proposal 10:

  - A UE generates a HARQ-ACK codebook for multicast SPS PDSCH receptions as for unicast SPS PDSCH receptions in Rel-16.

(Samsung) Proposal 11:

  - If a UE reports HARQ-ACK for unicast SPS PDSCHs and for multicast PDSCHs, the UE uses a PUCCH resource from sps-PUCCH-AN-List for the combined HARQ-ACK payload.

(Apple) Proposal 2:

  - For type 2 HARQ codebook construction,

    - HARQ-ACK bit ordering for MBS SPS activation/ deactivation is based on the DAI (counting with unicast DCI) and K1 indication

    - HARQ-ACK bits for one or more MSB SPS receptions without a corresponding PDCCH are appended after HARQ-ACK bits for group common PDSCH for multicast

(LGE) Proposal 11:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, UE determines a PUCCH resource based on one of the following options:

    - Option 1: The PRI of the last DCI with dynamic scheduling.

    - Option 2: Unicast SPS

    - Option 3: No multiplexing by dropping e.g. multicast HARQ-ACK

(LGE) Proposal 12:

  - For multiplexing HARQ-ACKs for multicast SPS and dynamically scheduled unicast, UE determines a PUCCH resource based on the last unicast DCI.

(LGE) Proposal 13:

  - For multiplexing HARQ-ACKs for multicast SPS and dynamically scheduled multicast, UE determines a PUCCH resource based on the last multicast DCI.

(LGE) Proposal 14:

  - For multiplexing ACK/NACK based HARQ-ACKs for group common SPS PDSCHs, UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for multicast as specified in 9.2.1 of 38.213.

    - If PUCCH-config for multicast is not configured, PUCCH-config for unicast is used.

(LGE) Proposal 15:

  - For group common SPS PDSCH(s) and unicast SPS PDSCH(s), UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for unicast as specified in 9.2.1 of 38.213.

    - If NACK only based HARQ-ACK is configured for group common SPS, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK.

### Round-1

FL’s Comments

How codebook is generated for multicast SPS was not discussed and concluded directly, though some companies may derive how it is generated based on the following agreements achieved:

Agreement: For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability - The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. - FFS: How to allocate the total SPS configurations between MBS and unicast. Agreement: For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total. - It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively.

The agreements state it is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively, and the total number of up to 8 configured SPS configurations in a BWP of a serving cell is kept unchanged. It may have implied that the codebook for SPS PDSCH is constructed in the same way as Rel-16 with multiple SPS PDSCHs regardless it is unicast SPS PDSCH or multicast SPS PDSCH. If there exists any potential ambiguity, we can try to make a clear agreement to confirm it.

In addition, we also agreed a separate SPS-PUCCH-AN-List for all SPS multicast configurations can be configured (for example when unicast SPS is not configured) and if it is not configured a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

Agreement: For support of ACK/NACK based HARQ-ACK feedback for SPS multicast, - the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast. - UE determines a priority index from the HARQ-ACK codebook index - UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

Then the question is which PUCCH resource configuration is used when both HARQ-ACK feedback for unicast SPS PDSCH and multicast SPS PDSCH are multiplexed on the same PUCCH. Given we have agreed the last unicast DCI is used to determine which PUCCH resource is used for the HARQ-ACK transmission for dynamic scheduling, the similar approach can be followed for SPS.

Another issue is which PUCCH resource is to be used when HARQ-ACK for unicast SPS PDSCH and HARQ-ACK for multicast PDSCHs are multiplexed on the same PUCCH slot. The legacy counterpart in Rel-15/R16 is PUCCH resource from PUCCH resource sets which is configured for DG HARQ-ACK when HARQ-ACK for SPS PDSCH and HARQ-ACK for DG PDSCH with the same priority are multiplexed on the same PUCCH. There is the proposal that PUCCH resource from sps-PUCCH-AN-List for the combined HARQ-ACK payload, which can be the starting point for the discussion.

FL’s Proposal:

#### Proposal 5.2.1-1

When UE is configured with unicast SPS and multicast SPS, the HARQ-ACK codebook is constructed as multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support the proposal with minor clarification: #### Proposal 5.2.1-1 When UE is configured with unicast SPS and multicast SPS, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs in Rel-16 regardless of unicast SPS PDSCH or multicast SPS PDSCH. |
| ZTE | We support this proposal. |
| Samsung | Support |
| CMCC | Support |
| Lenovo, Motorola Mobility | Agree with Nokia’s wording. |
| vivo | support |
| LG Electronics | OK |
| Spreadtrum | Ok |
| NTT DOCOMO | Support |
| Qualcomm | support |
| MediaTek | Ok |
| CATT | Support |
| Apple | Ok |
| Ericsson | OK |

#### Proposal 5.2.1-2

When HARQ-ACK for unicast SPS PDSCHs and multicast SPS PDSCHs are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support the proposal. |
| ZTE | We support this proposal. |
| Samsung | Support |
| CMCC | Support |
| Lenovo, Motorola Mobility | OK |
| vivo | support |
| LG Electronics | We agree with this proposal. We think that this proposal is applied to NACK-only as well as ACK/NACK based HARQ feedback for multicast SPS PDSCH. We think that if NACK only based HARQ-ACK is configured for group common SPS, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK. Accordingly, the proposal can be updated as follows: #### Proposal 5.2.1-2 When HARQ-ACK for unicast SPS PDSCHs and multicast SPS PDSCHs are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast. - If NACK only based HARQ-ACK is configured for group common SPS PDSCH, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK. |
| Spreadtrum | Ok |
| NTT DOCOMO | Support |
| Qualcomm | support |
| MediaTek | Ok |
| CATT | Support |
| Apple | Ok |
| Ericsson | OK |

#### Proposal 5.2.1-3

When HARQ-ACK for multicast DG PDSCHs and multicast SPS PDSCHs are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support the proposal. We should also indicate that the PRI in the last DCI is used. |
| ZTE | We support this proposal. |
| Samsung | Some general discussion is first needed on PUCCH resource allocation for multicast ACK/NACK-based HARQ-ACK in absence of unicast HARQ-ACK. For example, how will it work if some UEs have G-RNTI0 and no SPS, some UEs have G-RNTI0 and SPS, some UEs have G-RNTI0 and G-RNTI1 and no SPS, some UEs have G-RNTI0 and GRNTI1 and SPS, …, or if a UE has ACK/NACK based feedback for SPS but NACK-only based feedback for DG-PDSCH, … |
| CMCC | Support |
| Lenovo, Motorola Mobility | Agree with Nokia. |
| vivo | support |
| LG Electronics | We support this proposal. We think the proposal can be updated with the following: UE determines a PUCCH resource based on the last DCI. |
| Spreadtrum | Ok |
| NTT DOCOMO | Support |
| Qualcomm | support |
| CATT | Support |
| Apple | Ok |
| Ericsson | Support |

#### Proposal 5.2.1-4

When HARQ-ACK for unicast SPS PDSCHs and multicast PDSCHs are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support the proposal. |
| ZTE | We support this proposal. |
| Samsung | Support |
| CMCC | support |
| Lenovo, Motorola Mobility | OK |
| vivo | We are open to discuss this issue. If the PUCCH resource is determined from the SPS-PUCCH-AN-List configured for unicast, it is UE-specific. However, we are wondering whether the PUCCH resource is enough to transmit all the HARQ-ACK information since there are at most 4 PUCCH resources in the list, the number of HARQ-ACK for DG PDSCHs can be various. |
| LG Electronics | Support |
| Spreadtrum | Ok |
| NTT DOCOMO | Support |
| Qualcomm | Ok in general assuming here multicast PDSCH is multiple SPS PDSCH. |
| MediaTek | Generally Ok |
| CATT | Support |
| Apple | Ok |
| Ericsson | Duplicate proposal from 5.2.1.2 |

## others

(LGE) Proposal 9:

  - For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

(LGE) Proposal 10:

  - After group common SPS activation, all Ues autonomously release the group common SPS right after a pre-determined slot

    - The pre-determined time is determined by RRC and/or DCI.

(LGE) Proposal 16:

  - For multiplexing NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK for group common SPS PDSCHs, UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for multicast for ACK/NACK based HARQ-ACK as specified in 9.2.1 of 38.213.

    - If PUCCH-config for multicast is not configured, PUCCH-config for unicast is used.

(LGE) Proposal 17:

  - For multiplexing SR and HARQ-ACKs for group common SPS PDSCHs (and possibly unicast SPS PDSCHs), UE determines a PUCCH resource based on:

    - Option 1: UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for unicast as specified in 9.2.1 of 38.213

    - Option 2: HARQ-ACKs for group common SPS PDSCHs or SR is dropped.

# Slot-level repetitions

## Text proposals

### TP for slot-level repetition

#### TP 6.1.1.1 (Qualcomm)

(Qualcomm) Proposal 4:

  - Support pdsch-AggregationFactor configured in PDSCH-Config-Multicast for DCI format 4_1, similar as that of DCI format 4_2.

#### 5.1.2.1 Resource allocation in time domain ## *** Unchanged text is omitted *** When receiving PDSCH scheduled by DCI format 4_1/4_2 in PDCCH with CRC scrambled by G-RNTI or G-CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in the pdsch-Config-Multicast associated with the corresponding G-RNTI or in the associated SPS-Config-Multicast activated by the DCI format 4_1/4_2 with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 4_1/4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI with NDI = 0, or PDSCH without corresponding PDCCH transmission using associated [SPS-Config-Multicast] and activated by the DCI format 4_1/4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor, in associated SPS-Config-Multicast if configured, or 1 otherwise, consecutive slots. When receiving PDSCH scheduled by DCI format 4_0 in PDCCH with CRC scrambled by G-RNTI for MTCH, if the UE is configured with pdsch-AggregationFactor in the pdsch-Config-Broadcast, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception or G-RNTI, if the DCI field 'Time domain resource assignment' indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the PDSCH-Config-Multicast or PDSCH-Config-Broadcast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. When receiving PDSCH scheduled without corresponding PDCCH transmission using associated SPS-Config-Multicast and activated by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception, if the DCI field 'Time domain resource assignment' of the activating DCI indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the PDSCH-Config-Multicast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. ## *** Unchanged text is omitted ***

##### Round-1

| Company | Your comments/suggestions to the text proposal TP 6.1.1.1 |
| --- | --- |
| - Samsung | - OK. - However, RAN1 is doing an inconsistent design for DCI format 4_1 where configuration of fields has not been allowed (e.g. for priority indication) but configuration for what the fields indicate is pursued. |
| - NTT DOCOMO | - Support |
| - Qualcomm | - Support. - Considering slot-level repetition is supported for MTCH using DCI format 4_0, it is straightforward to also support it for multicast using DCI format 4_1 without increasing UE complexity. |
| - CATT | - OK |
| - Lenovo, Motorola Mobility | - OK |
| - Ericsson | - Support |

#### TP 6.1.1.1.1 (CMCC)

(CMCC) Proposal 5:

  - For multicast SPS PDSCH re-transmission, the pdsch-AggregationFactor in pdsch-Config-Multicast is applied as the repetition number.

for TS 38.214 section 5.1.2.1 <Unchanged text is omitted> When receiving PDSCH scheduled by DCI format 4_2 in PDCCH with CRC scrambled by G-RNTI or G-CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in the pdsch-Config-Multicast associated with the corresponding G-RNTI or G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI with NDI = 0, or PDSCH without corresponding PDCCH transmission using associated SPS-Config-Multicast and activated by the DCI format 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor, in associated SPS-Config-Multicast if configured, or 1 otherwise, consecutive slots. When receiving PDSCH scheduled by DCI format 4_0 in PDCCH with CRC scrambled by G-RNTI for MTCH, if the UE is configured with pdsch-AggregationFactor in the pdsch-Config-Broadcast, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. <Unchanged text is omitted>

##### Round-1

| Company | Your comments/suggestions to the text proposal TP 6.1.1.1.1 |
| --- | --- |
| - Samsung | - OK. |
| - Qualcomm | - ok |
| - Lenovo, Motorola Mobility | - OK |
| - Ericsson | - Support |

# Proposals for GTW on Jan. 18th

#### Proposal 2.2.1-1-r1

When PUCCH carrying multicast HARQ-ACK only overlaps with PUSCH with the same priority, support UL-DAI indicating the number of HARQ-ACK bits for multicast as defined in Rel-16 for unicast HARQ-ACK.

  - FFS it is applied to a single G-RNTI or applied to all configured G-RNTIs.

#### Proposal 2.2.1-2

Regarding whether to support multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority,

support the following case:

  - If the unicast and multicast HARQ-ACK codebooks are both Type-1, the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH.

  - If both unicast and multicast HARQ-ACK codebooks are Type-2, down-select from:

    - Option2-1: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks.

    - Option2-2: two 2-bit UL DAIs are included in DCI for unicast and multicast, respectively.

  - FFS the unicast and multicast HARQ-ACK codebooks are different Types.

#### Proposal 3.3.1-2 (H)

For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback,

  - 1 PUCCH resource set in each PUCCH-Config.

  - up to 32 PUCCH resources in each PUCCH resource set

  - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

#### Proposal 4.2.1 (H)

For “enabling/disabling HARQ-ACK feedback indication” in DCI format 4_2, support option 2 from:

  - Option1: this indication is always included in DCI format 4_2. It is a reserved field for UEs not supporting or for UE not configured with group-common DCI indicating HARQ-ACK enabled/disabled.

  - Option2: defining another RRC parameter configuring whether DCI format 4_2 includes this indication for all the UEs. When it is included in DCI, it is a reserved field for UEs not supporting or for UE not configured with group-common DCI indicating HARQ-ACK enabled/disabled.

#### Proposal 5.2.1-1-r1

When UE is configured with unicast SPS and multicast SPS with ACK/NACK based feedback, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH.

#### Proposal 5.2.1-2-r1

When HARQ-ACK for unicast SPS PDSCHs and multicast SPS PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

#### Proposal 5.2.1-3-r1

When HARQ-ACK for multicast DG PDSCHs and multicast SPS PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

#### Proposal 5.2.1-4-r1

When HARQ-ACK for unicast SPS PDSCHs and multicast PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH, the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

# References

R1-2200028	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2200095	Remaining issues on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2200118	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2200134	Remaining issues on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2200158	Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs supporting MBS	Nokia, Nokia Shanghai Bell

R1-2200214	Remaining issues on mechanisms to improve reliability	Samsung

R1-2200244	Remaining details on HARQ-ACK feedback procedure for multicast	NTT DOCOMO, INC.

R1-2200284	Discussion on the remaining issues on mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2200309	Maintenance on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2200351	Remaining issues on UL feedback for MBS of RRC-CONNECTED UEs	OPPO

R1-2200387	Reliability for RRC_CONNECTED UEs	Intel Corporation

R1-2200428	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2200472	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2200513	Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2200526	PUCCH formats for enhancement to NACK-ONLY based HARQ-ACK feedback	TD Tech, Chengdu TD Tech

R1-2200550	Remaining issues on improve multicast reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2200579	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2200597	Remaining issues on reliability improvement for RRC_CONNECTED UEs	CMCC

R1-2200666	Discussion on reliability mechanisms for NR MBS	Ericsson

# Appendix Agreements summary for AI 8.12.2

## 102e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

## 103e

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

## 104e

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on   the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

## 104be

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

## 105e

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Agreement:

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.

Agreement:

NR supports at least the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting.

Agreement:

For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- Agreement:

- Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Conclusion:

PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Agreement:

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

## 106e

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

Agreement:

When UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks.

- FFS: How UE is configured one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities.

Agreement:

For a UE configured with Type-1 HARQ-ACK codebook,

- If UE is not configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for TDM-ed unicast and multicast.

- If UE is configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for FDM-ed unicast and multicast.

Agreement:

For UEs supporting ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the following values are unchanged compared to unicast in Rel-16:

  - The maximum number of PUCCH resources sets in each PUCCH-Config,

  - The maximum number of PUCCH resources in a PUCCH resource set in each PUCCH-Config,

  - The maximum number of UCI information bits for the first PUCCH resource set.

  - The total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList.

  - Note:

    - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

    - The case of NACK-only based is discussed separately.

Agreement:

- When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for ACK/NACK based feedback for multicast, it is applied to all G-RNTIs configured to UE.

Agreement:

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

  - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

  - FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

The priority index is,

- for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format for GC-PDCCH, down-select from:

    - Alt1: Optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

    - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

  - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - Alt2: Support sub-slot based PUCCH for this case.

  - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

  - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - Alt5: NACK-only bundling

Agreement:

When UE supports and is configured with more than one G-RNTI,

  - for Type-2 codebook construction, DAI is separately counted per G-RNTI.

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

Agreement:

Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details.

- Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group

Agreement

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, select one or more of the following alternatives:

  - Alt1: HARQ-ACK feedback option is configured per SPS configuration index.

  - Alt2: HARQ-ACK feedback option is indicated in the SPS activation DCI.

  - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.

## 106be

Agreement:

The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit.

- This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

- Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

- FFS the case of PUCCH for SR.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Agreement:

Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback.

Agreement:

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives:

- Alt1: Reuse one existing field in the group-common DCI.

- Alt2: Introduce a new field in the group-common DCI.

Agreement:

For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling.

- FFS: Whether the configuration is per SPS configuration index or per G-CS-RNTI.

- Note: Whether there is a UE capability for support of NACK-only based HARQ-ACK or not will be discussed as part of UE features discussion.

Agreement:

- If configured, the pdsch-AggregationFactor for multicast dynamic scheduling is configured per G-RNTI.

- If configured, the pdsch-AggregationFactor for multicast SPS is configured per SPS-Config-Multicast.

Agreement:

For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

- Config A or Config B can be configured to UE:

    - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast.

    - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast.

    - If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH.

- For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is

    - Alt1: equal to 1.

Agreement:

For UE supporting both ACK/NACK based and NACK-only based feedback for multicast, for the same G-RNTI, support the following

- UE can be configured with either ACK/NACK based or NACK-only feedback for a single G-RNTI.

  - Note: Case1-1: if configured with ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config/PUCCH-ConfigurationList for multicast. Otherwise, PUCCH-Config/PUCCH-ConfigurationList for unicast applies (This has been agreed.)

  - Case 1-2: if configured with NACK-only based feedback, when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies.

Agreement:

For the priority index for the first DCI format for GC-PDCCH, support the following Alt2 from the previous agreement:

  - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Support of Alt. 1 is a UE capability

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the following Alt1 from the previous agreement:

- Alt.1: The last DCI for unicast

- FFS: Any details when last DCI is missed by the UE if it is necessary to make them different from current specifications for this case.

## 107e

Agreement

When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot,

- UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

  - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI.

  - FFS: when Type-3 HARQ-ACK codebook or enhanced Type-2 codebook is used for unicast

  - Define a UE capability

Agreement

For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported.

Agreement

UE is not expected to be configured with different PUCCH structures for unicast and multicast for which the HARQ-ACK are with the same priority and to be scheduled to multiplex the HARQ-ACK in the same PUCCH slot simultaneously.

Agreement

For a UE that supports multicast, the same TDRA table applies to all G-RNTIs if configured on a given serving cell.

Agreement

For a UE that supports multicast, when PUCCH-Config for ACK/NACK based feedback for multicast is configured separately from unicast, the PUCCH-Config is applied to all G-RNTIs with ACK/NACK based feedback with the same priority on a given serving cell.

- Note: The dl-DataToUL-ACK is included in PUCCH-Config

Agreement

At least for ACK/NACK based feedback, for obtaining a transmission power for a PUCCH, for Type-2 codebook, nHARQ-ACK is determined as follows:

- nHARQ-ACK=nHARQ-ACK(unicast) +nACK(multicast),  where

  - nHARQ-ACK(unicast)  is computed as in R15/R16.

  - nACK(multicast) is the total number of bits for all configured G-RNTIs.

Agreement

- For PTM retransmission,

  - if UE is configured to enable/disable HARQ-ACK per group-common DCI indication for initial transmission, whether HARQ-ACK is enabled/disabled for PTM retransmission also follows the indication in the group-common DCI scheduling the PTM retransmission.

  - if UE is configured directly whether the HARQ-ACK is enabled/disabled, it applies to both PTM initial transmission and retransmission.

- For PTP retransmission, the HARQ-ACK is always enabled.

Agreement

Support enabling/disabling HARQ-ACK for NACK-only based feedback.

    - The relevant agreements made for ACK/NACK based feedback can be extended for the support of NACK-only, including:

  - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI.

  - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI.

Agreement

HARQ-ACK feedback option is configured per G-CS-RNTI.

Agreement

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the “enabling/disabling HARQ-ACK feedback indication” is included in DCI format 1_1 scrambled by G-RNTI

- For DCI format 1_1 scrambled by G-CS-RNTI, it is discussed separately.

Agreement

For the DCI format including the field of “enabling/disabling HARQ-ACK feedback indication” for multicast scheduling, the field is a new field with 1 bit.

Agreement

For multicast SPS PDSCH without PDCCH scheduling, support the following:

- RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation.

  - The configuration is per G-CS-RNTI.

  - Separate UE capability is needed from that for dynamic scheduling for multicast.

- RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled.

  - The configuration is per G-CS-RNTI.

Agreement

For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value.

- The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure.

- FFS: whether/how to reduce the Type-1 codebook size when multiple G-RNTIs are configured.

- Note: The maximum number of G-RNTI(s) configured to UE for the FDMed unicast and multicast Type-1 codebook is up to UE capability which will be discussed in UE features.