3GPP TSG RAN WG1 #107bis-e	        		   R1-2200812

# e-Meeting, January 17th – 25th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the introduction for extending NR operation to 71 GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections/additions for extending of NR operation to 71 GHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Map the HARQ Feedback Timing Indicator to slots for SCS of 480/960 kHz in clause 8.2A. - Capture Tproc,2 for SCS of 480/960 kHz in clause 9. - Capture time domain bundling in clause 9.1.2.1. - Capture spatial domain bundling in clause 9.1.3.1. - Capture PDCCH monitoring pattern over consecutive slots within a group of slots in clause 10.1. - Capture PDCCH MOs for Type0-PDCCH CSS set with SS/PBCH block and CORESET multiplexing pattern 3 for 480/960 kHz in clause 13. - Other miscellaneous corrections/alignments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for NR extension to 71 GHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 8.2A, 9, 9.1.2.1, 9.1.3.1, 10.1, 13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.212, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 4	Synchronization procedures

## 4.1	Cell search

Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell.

A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211].

A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI.

For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the SCS of SS/PBCH blocks as follows, where index 0 corresponds to the first symbol of the first slot in a half-frame.

-	Case A - 15 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes of 2,8+14⋅n.

-	For operation without shared spectrum channel access:

-	For carrier frequencies smaller than or equal to 3 GHz, n=0,1.

-	For carrier frequencies within FR1 larger than 3 GHz, n=0,1,2,3.

-	For operation with shared spectrum channel access, as described in [15, TS 37.213], n=0, 1, 2, 3, 4.

-	Case B - 30 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 4,8,16,20+28⋅n. For carrier frequencies smaller than or equal to 3 GHz, n=0. For carrier frequencies within FR1 larger than 3 GHz, n=0,1.

-	Case C - 30 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 2,8+14⋅n.

-	For operation without shared spectrum channel access

-	For paired spectrum operation

-	For carrier frequencies smaller than or equal to 3 GHz, n=0,1. For carrier frequencies within FR1 larger than 3 GHz, n=0,1,2,3.

-	For unpaired spectrum operation

-	For carrier frequencies smaller than 1.88 GHz, n=0,1. For carrier frequencies within FR1 equal to or larger than 1.88 GHz, n=0,1,2,3.

-	For operation with shared spectrum channel access, n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9.

-	Case D - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 4,8,16,20+28⋅n. For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

-	Case E - 240 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 8,12,16,20,32,36,40,44+56⋅n. For carrier frequencies within FR2-1, n=0, 1, 2, 3, 5, 6, 7, 8.

-	Case F – 480 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 2, 9+14⋅n. For carrier frequencies within FR2-2, n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31.

-	Case G – 960 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 2, 9+14⋅n. For carrier frequencies within FR2-2, n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31.

From the above cases, if the SCS of SS/PBCH blocks is not provided by ssbSubcarrierSpacing, the applicable cases for a cell depend on a respective frequency band, as provided in [8-1, TS 38.101-1] and [8-2, TS 38.101-2]. A same case applies for all SS/PBCH blocks on the cell. If a 30 kHz SS/PBCH block SCS is indicated by ssbSubcarrierSpacing, Case B applies for frequency bands with only 15 kHz SS/PBCH block SCS as specified in [8-1, TS 38.101-1], and the case specified for 30 kHz SS/PBCH block SCS in [8-1, TS 38.101-1] applies for frequency bands with 30 kHz SS/PBCH block SCS or both 15 kHz and 30 kHz SS/PBCH block SCS as specified in [8-1, TS 38.101-1]. For a UE configured to operate with carrier aggregation over a set of cells in a frequency band of FR2 or with frequency-contiguous carrier aggregation over a set of cells in a frequency band of FR1, if the UE is provided SCS values by ssbSubcarrierSpacing for receptions of SS/PBCH blocks on any cells from the set of cells, the UE expects the SCS values to be same.

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to Lmax-1, where Lmax is determined according to SS/PBCH block patterns for Cases A through G. Lmax is a maximum number of SS/PBCH block indexes in a cell, and the maximum number of transmitted SS/PBCH blocks within a half frame is Lmax.

-	For operation without shared spectrum channel accessLmax=Lmax  in FR1 and FR2, and for operation with shared spectrum channel access in FR2-2, Lmax=Lmax

-	For operation with shared spectrum channel access in FR1, Lmax=8 for Lmax=10 and 15 kHz SCS of SS/PBCH blocks and for Lmax=20 and 30 kHz SCS of SS/PBCH blocks

For Lmax=4, a UE determines the 2 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211].

For Lmax>4, a UE determines the 3 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211]

-	for Lmax=10, the UE determines the 1 MSB bit of the candidate SS/PBCH block index from PBCH payload bit aA+7 as described in [5, TS 38.212]

-	for Lmax=20, the UE determines the 2 MSB bits of the candidate SS/PBCH block index from PBCH payload bits aA+6,aA+7 as described in [5, TS 38.212]

-	for Lmax=64, the UE determines the 3 MSB bits of the candidate SS/PBCH block index from PBCH payload bits aA+5,aA+6,aA+7 as described in [5, TS 38.212]

A UE can be provided per serving cell by ssb-periodicityServingCell a periodicity of the half frames for reception of the SS/PBCH blocks for the serving cell. If the UE is not configured a periodicity of the half frames for receptions of the SS/PBCH blocks, the UE assumes a periodicity of a half frame. A UE assumes that the periodicity is same for all SS/PBCH blocks in the serving cell.

For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index.

For operation with shared spectrum channel access, a UE assumes that transmission of SS/PBCH blocks in a half frame is within a discovery burst transmission window that starts from the first symbol of the first slot in a half-frame. The UE can be provided per serving cell by discoveryBurstWindowLength a duration of the discovery burst transmission window. If discoveryBurstWindowLength is not provided, the UE assumes that the duration of the discovery burst transmission window is a half frame. For a serving cell, the UE assumes that a periodicity of the discovery burst transmission window is same as a periodicity of half frames for receptions of SS/PBCH blocks in the serving cell. The UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that SS/PBCH block(s) within the discovery burst transmission window with candidate SS/PBCH block index(es) corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the SS/PBCH block(s) are not transmitted. If MSB k, k≥1, of inOneGroup is set to 1, and MSB m, m≥1, of groupPresence is set to 1, the UE assumes that SS/PBCH block(s) within the discovery burst transmission window with candidate SS/PBCH block index(es) corresponding to SS/PBCH block index determined by k and m may be transmitted; otherwise, the UE assumes that the SS/PBCH block(s) are not transmitted.

For operation with shared spectrum channel access in FR1, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214], if a value of NDM-RSPBCHmodNSSBQCL is same among the SS/PBCH blocks. NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, and NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-1 with kSSB<24 [4, TS 38.211]. The UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL where i is the candidate SS/PBCH block index. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one.

Table 4.1-1: Mapping between the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset to NSSBQCL for operation with shared spectrum channel access in FR1

| subCarrierSpacingCommon | LSB of ssb-SubcarrierOffset | NSSBQCL |
| --- | --- | --- |
| scs15or60 | 0 | 1 |
| scs15or60 | 1 | 2 |
| scs30or120 | 0 | 4 |
| scs30or120 | 1 | 8 |

For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one.

Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and spare to NSSBQCL for operation with shared spectrum channel access in FR2-2

| subCarrierSpacingCommon | spare | NSSBQCL |
| --- | --- | --- |
| scs15or60 | 0 | 16 |
| scs15or60 | 1 | 32 |
| scs30or120 | 0 | 64 |
| scs30or120 | 1 | reserved |

Upon detection of a SS/PBCH block, the UE determines from MIB that a CORESET for Type0-PDCCH CSS set, as described in clause 13, is present if kSSB<24 [4, TS 38.211] for FR1 or if kSSB<12 for FR2. The UE determines from MIB that a CORESET for Type0-PDCCH CSS set is not present if kSSB>23 for FR1 or if kSSB>11 for FR2; the CORESET for Type0-PDCCH CSS set may be provided by PDCCH-ConfigCommon.

For a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the PCell, or on the PSCell, or on an SCell if applicable as described in [10, TS 38.133], of the cell group for the serving cell.

## *** Unchanged text is omitted ***

## 8.2A	Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow.

In response to a transmission of a PRACH, if the PRACH preamble is not mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a HARQ Feedback Timing Indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} for μ≤3, from {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and from {13, 16, 24, 32, 40, 48, 56, 64} for μ=6 and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆+2μ∙Kcell,offset, where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], μ is the SCS configuration of the active UL BWP, and Kcell,offset is provided by Koffset in ServingCellConfigCommon; otherwise, if not provided, Kcell,offset=0

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI and receives a transport block within the window in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block the UE used for PRACH association, as described in clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

## *** Unchanged text is omitted ***

# 9	UE procedure for reporting control information

If a UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG.

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If a UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. If pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 is provided, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16. If harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup is provided, harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup. If harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup is provided, harq-ACK-SpatialBundlingPUSCH is replaced by harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup.

If a UE is provided pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookList-r16.

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = separate

the UE shall separately apply the procedures described in clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells, and the UE does not expect to be provided with subslotLengthForPUCCH or to be indicated by pdsch-HARQ-ACK-CodebookList to generate two HARQ-ACK codebooks on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE.

For NR-DC when both the MCG and the SCG operate either in FR1 or in FR2 and for a power headroom report transmitted on the MCG or the SCG, the UE computes PH assuming that the UE does not transmit PUSCH/PUCCH on any serving cell of the SCG or the MCG, respectively.

If a UE is configured for NR-DC operation, the UE does not expect to be configured with a PUCCH-SCell.

A PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, the priority index is 0.

If a UE is provided one PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in the PUCCH-Config is within the subslotLengthForPUCCH symbols in the PUCCH-Config

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config

If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority, a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority and a DCI format 1_1 can indicate SCell dormancy and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6

-	any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, 36 for μ=3, 144 for μ=5, and 288 for μ=6;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, 36 for μ=3, 144 for μ=5, and 288 for μ=6;

If a UE would transmit the following channels, including repetitions if any, that would overlap in time

-	a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or

-	a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or

-	a first PUCCH of larger priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s) and a second PUCCH of smaller priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or

-	a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or

-	a configured grant PUSCH of larger priority index and a configured PUSCH of smaller priority index on a same serving cell

the UE is expected to cancel a repetition of the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index if the repetition of the PUCCH/PUSCH transmissions of smaller priority index overlaps in time with the PUCCH/PUSCH transmissions of larger priority index.

When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index, including repetitions if any, after resolving the overlapping PUCCH other than PUCCH transmissions with SL HARQ-ACK reports and/or PUSCH transmissions, if the PUSCH includes no UCI, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index as described in clauses 9.2.5 and 9.2.6.

When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of larger priority index only, including repetitions if any, after resolving the overlapping PUCCH other than PUCCH transmissions with SL HARQ-ACK reports and/or PUSCH transmissions, the UE does not transmit the PUCCH with SL HARQ-ACK reports

where

-	the UE expects that the transmission of the PUSCH would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception;

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability.

The UE expects the PUCCH and PUSCH transmissions fulfill the conditions in clause 9 and clause 9.2.5 for UCI multiplexing replacing the reference time of "end of PDSCH" with "end of the last symbol of a last PSFCH reception occasion" as described in 16.5 and Tproc,1 with Tprep.

A UE does not expect that a PUCCH carrying SL HARQ-ACK reports overlaps with PUSCH with aperiodic or semi-persistent CSI reports.

A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH.

In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission as described in clause 11.1. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH.

In the remaining of this clause, the multiplexing or prioritization for overlapping channels are for overlapping channels with same priority index or for overlapping channels with a PUCCH carrying SL HARQ-ACK information.

In the remaining of this clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH, unless stated otherwise.

If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on a serving cell that includes positive SR information, the UE does not transmit the PUSCH.

If a UE would transmit CSI reports on overlapping physical channels, the UE applies the priority rules described in [6, TS 38.214] for the multiplexing of CSI reports.

If a UE has overlapping resources for PUCCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first follows the procedures described in clause 9.2.6 for resolving the overlapping among the resources for the PUCCH transmissions.

If a UE

-	would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and

-	the PUSCH and PUCCH transmissions fulfill the conditions in clause 9.2.5 for UCI multiplexing,

the UE

-	multiplexes only HARQ-ACK information, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH;

-	multiplexes only HARQ-ACK information and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.

A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration μ1 UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration μ2 if μ1<μ2.

A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs.

A UE does not expect a PUCCH resource that results from multiplexing overlapped PUCCH resources, if applicable, to overlap with more than one PUSCHs if each of the more than one PUSCHs includes aperiodic CSI reports.

A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, a DCI format 1_1 indicating SCell dormancy, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

If a UE transmits multiple PUSCHs in a slot on respective serving cells that include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

If a UE transmits a PUSCH over multiple slots and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent.

If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol.

If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

## *** Unchanged text is omitted ***

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

-	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 for serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6.

-	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK

-	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2

-	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-DCI-1-2

-	If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c

-	if the UE is not provided type1-Codebook-Generation-Mode = 'mode1', K1 is additionally provided by the union of dl-DataToUL-ACK-ForDCI Format4_1

-	if the UE is not provided dl-DataToUL-ACK-ForDCI Format4_1, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

-	if the UE is provided type1-Codebook-Generation-Mode = 'mode1', the UE

-	determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM

b)	on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by

-	the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is not configured to monitor PDCCH for multicast DCI formats for serving cell c, or is not provided type1-Codebook-Generation-Mode = 'mode1', or, if any, for the first K1,UM set

-	the union of row indexes of time domain resource allocation tables for DCI format 1_0 and/or DCI format 1_1 and/or DCI format 1_2 for serving cell c for the second K1,U\M set, if any

-	the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c for the third K1,M\U set, if any

-	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

c)	on the ratio 2μDL-μUL between the downlink SCS configuration μDL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in clause 11.1

e)	if ca-SlotOffset is provided, on Nslot,offset,cDL and μoffset,DL,c provided by ca-SlotOffset for serving cell c, or on Nslot,offsetUL  and μoffset,UL provided by ca-SlotOffset for the primary cell, as described in [4, TS 38.211].

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = joint

where

-	a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and

-	a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of OACK HARQ-ACK information bits.

If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats

-	a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and

-	a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = 'mode1', the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook.

If enableTimeDomainHARQ-Bundling is provided

-	set RT'=R

-	set RT to the set of row indexes that include the last SLIV of each row of set RT'

If the set of rows R includes a row with more than one entry as described in [6, TS 38.214] and enableTimeDomainHARQ-Bundling is not provided, the set of rows R and the set of slot timing values K1 are updated in this clause according to the following pseudo-code.

set R to the set of rows

set CR to the cardinality of R

set r=0 – index of row in set R

set RT=∅

set K1,T=∅

while r<CR

set Pr to the set of entries for row r

set K0,r to the set of K0 values of entries for row r

set ∆K0,r=maxK0K0,r-K0,r

set CPr to the cardinality of Pr

set C∆K0,r to the cardinality of ∆K0,r

set p=0 – index of element in Prset d=0 – index of element in ∆K0,r

while p<CPr

RT=RT∪Prp;

p=p+1;

end while

while d<C∆K0,r

K1,T=K1,T∪K1+∆K0,r(d)⋅2μUL-μDL∪K1+∆K0,r(d)⋅2μUL-μDL;

d=d+1;

end while

r=r+1;

end while

K1=K1,T;

For the set of slot timing values K1, the UE determines a set of MA,c occasions for candidate PDSCH receptions or SPS PDSCH releases or TCI state update according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. If a UE provides HARQ-ACK information corresponding to detection of a DCI format that provides TCI state update without scheduling PDSCH reception, as described in [6, TS 38.214], a location in the Type-1 HARQ-ACK codebook for the HARQ-ACK information is same as when the DCI format schedules a PDSCH reception with CBGs or with transport blocks that are correctly decoded.

Set j=0 - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update

Set B=∅

Set MA,c=∅

Set CK1 to the cardinality of set K1

Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 for serving cell c

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and enableTimeDomainHARQ-Bundling are provided for serving cell c

R=RT;

R'=RT';

elseif PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and enableTimeDomainHARQ-Bundling is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R

while R≠∅

Set r=0

while r<CR

if S≤m for start OFDM symbol index S for row r

br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r

R=R\r;

B=B∪br,k,nD;

else

r=r+1;

end if

end while

MA,c=MA,c∪j

j=j+1;

Set m to the smallest last OFDM symbol index among all rows of R;

end while

end if

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

while k<CK1

if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and enableTimeDomainHARQ-Bundling are provided for serving cell c

R=RT;

R'=RT';

elseif PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and enableTimeDomainHARQ-Bundling is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot  n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1 or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R

while R≠∅

Set r=0

while r<CR

if S≤m for start OFDM symbol index S for row r

br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r

R=R\r;

B=B∪br,k,nD;

else

r=r+1;

end if

end while

MA,c=MA,c∪j;

j=j+1;

Set m to the smallest last OFDM symbol index among all rows of R;

end while

end if

nD=nD+1;

end if

end while

end if

k=k+1;

end while

end if

If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,k,nD, where br,k,nD∈B, the UE does not expect to receive more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or TCI state update, or a PDSCH that is scheduled by a DCI format that does not support CBG-based PDSCH receptions and if

-	the UE is configured with one serving cell, and

-	CMA,c=1, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE generates HARQ-ACK information only for the transport block in the PDSCH, or only for the SPS PDSCH release, or only for the TCI state update.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or TCI state update, or a PDSCH that is scheduled by a DCI format that does not support CBG-based PDSCH receptions and if

-	the UE is configured with more than one serving cells, or

-	CMA,c>1, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE repeats NHARQ-ACKCBG/TB,max times the HARQ-ACK information for the transport block in the PDSCH, or for the SPS PDSCH release, or for the TCI state update.

A UE does not expect to detect a DCI format switching a DL BWP within N3 symbols prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where N3 is defined in clause 9.2.3.

If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-DCI-1-2, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6, and the set of slot timing values provided by K1 for the active DL BWP of a corresponding serving cell.

If maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block or a SPS PDSCH release or a TCI state update, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

A UE determines o0ACK, o1ACK,...,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set MA,c defines a total number Mc of occasions for PDSCH reception or SPS PDSCH release or TCI state update for serving cell c corresponding to the HARQ-ACK information bits.

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cells including, when applicable, cells in the set S0 and the set S1

Set j=0- HARQ-ACK information bit index

Set NcellsDL to the number of serving cells configured by higher layers for the UE

while c<NcellsDL

Set m=0 – index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update

while m<Mc

if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c

if the PDSCH is associated with the last SLIV in the TDRA row

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c;

j=j+1;

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c;

else

ojACK= NACK;

j=j+1;

ojACK= NACK;

end if

j=j+1;

elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c

if the PDSCH is associated with the last SLIV in the TDRA row;

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c

if the UE receives one transport block, the UE assumes ACK for the second transport block;

else

ojACK = NACK;

end if

j=j+1;

else

if the PDSCH is associated with the last SLIV in the TDRA row;

ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c

else

ojACK = NACK;

end if

j=j+1;

end if

else

if harq-ACK-SpatialBundlingPUCCH is not provided, PDSCH-CodeBlockGroupTransmission is not provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c,

ojACK= HARQ-ACK information bit corresponding to a first transport block of this cell;

j=j+1;

ojACK = HARQ-ACK information bit corresponding to a second transport block of this cell;

j=j+1;

elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c,

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell

if the UE receives one transport block, the UE assumes ACK for the second transport block;

j=j+1;

elseif PDSCH-CodeBlockGroupTransmission is provided, and NHARQ-ACK,cCBG/TB,max CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell c,

Set nCBG=0- CBG index

while nCBG<NHARQ-ACK,cCBG/TB,max

oj+nCBGACK = HARQ-ACK information bit corresponding to CBG nCBG of the first transport block;

if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c

oj+nCBG+NHARQ-ACK,cCBG/TB,maxACK = HARQ-ACK information bit corresponding to CBG nCBG of the second transport block;

end if

nCBG=nCBG+1;

end while

j=j+NTB,cDL∙NHARQ-ACK,cCBG/TB,max, where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell c;

else

ojACK = HARQ-ACK information bit of serving cell c;

j=j+1;

end if

end if

m=m+1;

end while

c=c+1;

end while

If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where

-	NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs

-	Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c

-	Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	if enableTimeDomainHARQ-Bundling is provided for serving cell c and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers as received only a PDSCH associated with the last SLIV

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH.

## *** Unchanged text is omitted ***

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause per G-RNTI and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values.

A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling PDSCH reception, on an active DL BWP of a serving cell c, as described in clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PDSCH-to-HARQ_feedback timing indicator field values, or a dl-DataToUL-ACK, dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-ForDCI-Format1-2 value if the PDSCH-to-HARQ_feedback timing indicator field is not present in a DCI format, for PUCCH transmission with HARQ-ACK information in slot n, as described in clause 9.2.3, in response to PDSCH receptions, or in response to a DCI format having associated HARQ-ACK information without scheduling PDSCH reception

-	slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions and by pdsch-AggregationFactor, or pdsch-AggregationFactor-r16, or repetitionNumber, when provided.

The set of PDCCH monitoring occasions for DCI formats scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling PDSCH reception, is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells. PDCCH monitoring occasions are indexed in an ascending order of their start times. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions, or HARQ-ACK information bits that are not in response for PDSCH receptions, associated with the DCI formats is present up to the current serving cell and current PDCCH monitoring occasion,

-	first, if the UE indicates by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, in increasing order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair,

-	second in ascending order of serving cell index, and

-	third in ascending order of PDCCH monitoring occasion index m, where 0≤m<M.

If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s), or HARQ-ACK information that does not correspond to PDSCH receptions, associated with DCI formats is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT-DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field.

If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

Set m=0 – PDCCH, with DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling a PDSCH reception, monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion

Set j=0

Set Vtemp=0

Set Vtemp2=0

Set Vs=∅

Set NcellsDL to the number of serving cells configured by higher layers for the UE

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs

-	if the UE indicates type2-HARQ-ACK-Codebook, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of type2-HARQ-ACK-Codebook

Set M to the number of PDCCH monitoring occasion(s)

while m<M

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

if PDCCH monitoring occasion m is before an active DL BWP change on serving cell c or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion m

c=c+1;

else

if there is a PDSCH providing a transport block for a HARQ process with enabled HARQ-ACK information on serving cell c associated with PDCCH in PDCCH monitoring occasion m, or there is a PDCCH providing a DCI format associated with HARQ-ACK information without scheduling PDSCH reception on serving cell c

if VC-DAI,c,mDL≤Vtemp

j=j+1

end if

Vtemp=VC-DAI,c,mDL

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL

else

Vtemp,2=VT-DAI,mDL

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell

o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell

Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

else

oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

end if

end if

c=c+1

end if

end while

m=m+1

end while

Vtemp=j mod4TD×4TD+Vtemp

if UE does not set Vtemp2=VT-DAIUL and TD=2

Vtemp2=Vtemp

end if

j=j×TD4

if Vtemp2<Vtemp

j=j+1

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

OACK=2⋅4⋅j+Vtemp2

else

OACK=4⋅j+Vtemp2

end if

oiACK=NACK for any i∈0,1,⋯,OACK-1\Vs

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits.

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits.

The UE generates HARQ-ACK information with ACK value in response to a detection of a DCI format that does not trigger a Type-3 HARQ-ACK codebook report and has associated HARQ-ACK information without scheduling a PDSCH reception.

For a PDCCH monitoring occasion with DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling a PDSCH reception, in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block, or detects a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

If a UE is not provided PDSCH-CodeBlockGroupTransmission for any serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c+g=0G-1VDAI,mlast,gDL-c=0Ncells,gDL-1UDAI,c,gmodTD,gNTB,max,gDL+c=0Ncells,gDL-1m=0Mg-1Nm,c,greceived+NSPS,c,g

where

-	NcellsDL is a number of serving cells where the UE is configured to receive unicast PDSCHs

-	Ncells,gDL is a number of serving cells where the UE is configured to receive multicast PDSCHs for a G-RNTI g or a G-CS-RNTI g

-	G is a total number of G-RNTIs or G-CS-RNTIs configured to the UE

-	M is the number of PDCCH monitoring occasions for unicast DCI formats

-	Mg is the number of PDCCH monitoring occasions for multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g

-	TD=2NC-DAIDL where NC-DAIDL the number of bits for the counter DAI field in unicast DCI formats

-	TD,g=2NC-DAI,gDL where NC-DAI,gDL the number of bits for the counter DAI field in multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, that the UE detects within the M PDCCH monitoring occasions.

-	if Ncells,gDL=1, VDAI,mlast,gDL is the value of the counter DAI in the last multicast DCI format with G-RNTI g, or G-CS-RNTI g, scheduling PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the Mg PDCCH monitoring occasions

-	if NcellsDL>1 or if Ncells,gDL>1

-	if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion

-	if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg, for G-RNTI g or G-CS-RNTI g, PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the total DAI in the at least one DCI format that includes a total DAI field

-	VDAI,mlastDL=0 or VDAI,mlast,gDL=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c in any of the M or Mg PDCCH monitoring occasions, respectively.

-	UDAI,c or UDAI,c,g, for G-RNTI g or G-CS-RNTI g, is the total number of DCI formats scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the M or Mg PDCCH monitoring occasions, respectively, for serving cell c. UDAI,c=0 or UDAI,c,g=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for serving cell c in any of the M or Mg, respectively, PDCCH monitoring occasions.

-	NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1.

-	NTB,max,gDL=1.

-	Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is the number of transport blocks the UE receives in a PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI formats that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c.

-	NSPS,c or NSPS,c,g, for G-RNTI g or G-CS-RNTI g, is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M or Mg PDCCH monitoring occasions, respectively.

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	a DCI format having associated HARQ-ACK information without scheduling PDSCH reception,

-	TCI state update, and

-	TB-based PDSCH receptions on the NcellsDL,CBG serving cells and on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ-ACK,maxCBG/TB,max HARQ-ACK information bits, where NHARQ-ACK,maxCBG/TB,max is the maximum value of NTB,cDL⋅NHARQ-ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ-ACK,cCBG/TB,max<NHARQ-ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ-ACK,maxCBG/TB,max-NTB,cDL⋅NHARQ-ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c

-	the pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook

If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions

-	UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH

If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and

-	the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and

-	instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that the UE does not receive in a slot as described in clause 11.1. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.

If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH receptions where a maximum number of PDSCH receptions, NHARQ-ACK,cTBG,max, scheduled by a DCI format on the serving cell is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax  HARQ-ACK information bits for serving cell c.

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI

| DAI MSB, LSB | VC-DAIDL or VT-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH generating a HARQ-ACK information bit without scheduling a PDSCH reception or providing TCI state update is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0,0 | 1 | Y-1modTD+1=1 |
| 0,1 | 2 | Y-1modTD+1=2 |
| 1,0 | 3 | Y-1modTD+1=3 |
| 1,1 | 4 | Y-1modTD+1=4 |

Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1

| DAI | VC-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH generating a HARQ-ACK information bit without scheduling a PDSCH reception or providing TCI state update is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0 | 1 | Y-1modTD+1=1 |
| 1 | 2 | Y-1modTD+1=2 |

## *** Unchanged text is omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or by searchSpaceZero in PDCCH-ConfigCommon when pdcch-Config-MCCH or pdcch-Config-MCCH is not provided, for a DCI format with CRC scrambled by a MCCH-RNTI or a G-RNTI, on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0B-PDCCH CSS set configured by searchSpaceBroadcast in pdcch-Config-MCCH and pdcch-Config-MTCH for a DCI format with CRC scrambled by a MCCH-RNTI or a G-RNTI, on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type1A-PDCCH CSS set configured by sdt-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI on the primary cell as described in clause 19.1

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or configured by SearchSpace-Multicast in PDCCH-Config-Multicast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI, or configured by sdt-CG-SearchSpace for DCI formats with CRC scrambled by C-RNTI or CS-RNTI as described in clause 19.1.

In the following, DCI formats with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI are also referred to as unicast DCI formats, DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI are also referred to as multicast DCI formats, and DCI formats with CRC scrambled by MCCH-RNTI or G-RNTI for MTCH scheduling PDSCH receptions are also referred to as broadcast DCI formats.

For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. If the active DL BWP and the initial DL BWP have same SCS and same CP length and the active DL BWP includes all RBs of the CORESET with index 0, or the active DL BWP is the initial DL BWP, the CORESET configured for Type0-PDCCH CSS set has CORESET index 0 and the Type0-PDCCH CSS set has search space set index 0.

For a DL BWP, if a UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1.

For a DL BWP, if a UE is not provided ra-SearchSpace for Type1-PDCCH CSS set, the UE does not monitor PDCCH for Type1-PDCCH CSS set on the DL BWP. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI and has been provided a Type1-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set. If the UE has not been provided sdt-SearchSpace for Type1A-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set as described in clause 19.2.

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is not provided peiSearchSpace for Type2A-PDCCH CSS set, the UE does not monitor PDCCH for Type2A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the maximum number of PDCCH candidates per CCE aggregation level for Type2A-PDCCH CSS set are given in Table 10.1-1.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or is not provided searchSpaceBroadcast, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in clause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or monitors PDCCH candidates for DCI formats with CRC scrambled by a MCCH-RNTI or a G-RNTI and the UE is provided a non-zero value for searchSpaceBroadcast in pdcch-Config-MCCH and pdcch-Config-MTCH for a Type0/0B-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set, or of the Type0/0B-PDCCH set, respectively, based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB, the DM-RS antenna port associated with corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214], if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. For operation without shared spectrum channel access in FR1 and FR2-1, a SCS is provided by subCarrierSpacingCommon in MIB. For operation with shared spectrum channel access in FR1 and for operation in FR2-2, a SCS is same as the SCS of a corresponding SS/PBCH block.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, or a CS-RNTI

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 1 |

For each DL BWP configured to a UE in a serving cell, the UE can be provided by higher layer signalling with

-	P≤3 CORESETs if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided

-	P≤5 CORESETs if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET

For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index p, by controlResourceSetId  or by controlResourceSetId-v1610, where

-	0<p<12 if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided;

-	0<p<16 if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

-	a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID;

-	a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity;

-	a number of consecutive symbols provided by duration;

-	a set of resource blocks provided by frequencyDomainResources;

-	CCE-to-REG mapping parameters provided by cce-REG-MappingType;

-	an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception;

-	an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or has associated HARQ-ACK information without scheduling PDSCH and is provided by a PDCCH in CORESET p, by tci-PresentInDCI or tci-PresentDCI-1-2.

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList, or with any RE of a SS/PBCH block.

If a UE is provided two TCI states indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a CORESET associated with a Type3-PDCCH CSS set, the UE may assume the quasi co-location information indicated in both of the two TCI states for the PDCCH reception in the CORESET.

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocations, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position RBs0+k,DLstart,μ  [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitorLocations if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. If a UE is provided RB sets in the DL BWP, the UE expects that the RBs of the CORESET are within the union of the PRBs in the RB sets of the DL BWP.

For a CORESET other than a CORESET with index 0,

-	if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure, or for a most recent configured grant PUSCH transmission as described in clause 19 for a same HARQ process;

-	if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure, or a SS/PBCH block the UE identified during a most recent configured grant PUSCH transmission as described in clause 19.

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command, μ is the SCS configuration for the PUCCH in the slot when the activation command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

If a UE is provided TCI-StateID_r17, a DM-RS antenna port for PDCCH receptions in a CORESET, other than a CORESET with index 0, associated only with USS sets and/or Type3-PDCCH CSS sets, and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State-r17 [6, TS 38.214].

If a UE is provided useIndicatedTCIState for a CORESET, other than a CORESET with index 0, associated only with CSS sets other than Type3-PDCCH CCS sets, and if useIndicatedTCIState is set as enabled, a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-state-r17.

If the UE is provided by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by one or two TCI-State each with same activated tci-StateID value, to CORESETs with a same index in all configured DL BWPs of all configured cells in a list determined from a serving cell index, where one or two tci-StateID, the CORESET index, and the serving cell index are provided by a MAC CE command.

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace:

-	a search space set index s, 0<s<40 , by searchSpaceId

-	an association between the search space set  s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a PDCCH monitoring pattern within a group of slots, indicating consecutive slots in the group of slots for PDCCH monitoring, by monitoringSlotsWithinSlotGroup-r17

-	a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration or by duration-r17

-	a number of PDCCH candidates Ms(L) per CCE aggregation level L by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set s is either a CSS set or a USS set by searchSpaceType

-	if search space set s is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates, or to monitor one PDCCH candidate per RB set if the UE is provided freqMonitorLocations for the search space set, for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	an indication by dci-Format2-4 to monitor PDCCH candidates for DCI format 2_4

-	an indication by dci-Format2-6 to monitor PDCCH candidates for DCI format 2_6

-	an indication by dci-Format4-0 to monitor PDCCH candidates for DCI format 4_0

-	an indication by dci-Format4-1, or dci-Format4-2, or dci-Format4-1-AndFormat4-2 to monitor PDCCH candidates for DCI format 4_1, or DCI format 4_2, or for both DCI format 4_1 and DCI format 4_2, respectively

-	a search space set index by searchSpaceLinking for another search space set that is linked to search space set s

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or an indication by dci-FormatsSL to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

-	a bitmap by freqMonitorLocations, if provided, to indicate an index of one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by RBs0+k,DLstart,μ+NRBoffset, where RBs0+k,DLstart,μ is the index of first common RB of the RB set k [6, TS 38.214], and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. For each RB set with a corresponding value of 1 in the bitmap, the frequency domain resource allocation pattern for the monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration.

If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol.

A UE does not expect to be provided a first symbol and a number of consecutive symbols for a CORESET that results to a PDCCH candidate mapping to symbols of different slots.

A UE does not expect any two PDCCH monitoring occasions on an active DL BWP, for a same search space set or for different search space sets, in a same CORESET to be separated by a non-zero number of symbols that is smaller than the CORESET duration.

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots.

A USS at CCE aggregation level L∈1, 2, 4, 8, 16 is defined by a set of PDCCH candidates for CCE aggregation level L.

If a UE is configured with CrossCarrierSchedulingConfig for a serving cell the carrier indicator field value corresponds to the value indicated by CrossCarrierSchedulingConfig.

For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if the UE is not configured with a carrier indicator field, the UE monitors the PDCCH candidates without carrier indicator field. For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if a UE is configured with a carrier indicator field, the UE monitors the PDCCH candidates with carrier indicator field.

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell. For the active DL BWP of a serving cell on which the UE monitors PDCCH candidates, the UE monitors PDCCH candidates at least for the same serving cell.

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI(L)  of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCI are given by

L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

where

for any CSS, Yp,ns,fμ=0;

for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537;

i=0,⋯,L-1;

NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set;

nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0;

ms,nCI(L)=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

the RNTI value used for nRNTI is the C-RNTI.

For search space sets si and sj that include searchSpaceLinking with value sj and si, respectively, a UE monitors, in monitoring occasions with same index according to each of search space sets si and sj in a slot, PDCCH candidates msi,nCI(L) and msj,nCI(L), with msi,nCI(L)=msj,nCI(L), for detection of a DCI format with same information. The UE expects ksi=ksj, osi=osj, Tsi=Tsj, Msi(L)=Msj(L), and a same number of non-overlapping PDCCH monitoring occasions per slot based on corresponding monitoringSymbolsWithinSlot, for search space sets si and sj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is provided tci-PresentInDCI or tci-PresentDCI-1-2 for either none or both of CORESETs pi and pj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is either not provided coresetPoolIndex value of 1 for any of the two CORESETs, or is provided coresetPoolIndex value of 1 for both CORESETs. The UE can indicate by countLinkedCandidates a capability for counting PDCCH candidates msi,nCI(L) and msj,nCI(L) either as 2 PDCCH candidates or as 3 PDCCH candidates. For search space sets si, sj, sk, and sl, that include searchSpaceLinking with values sj, si, sl, and sk, respectively, a UE expects to simultaneously monitor PDCCH candidates msi,nCI,1(8)=msj,nCI,1(8), and msk,nCI,2(16)= msl,nCI,2(16) only if a first CCE of msi,nCI,1(8) or msj,nCI,1(8) has different index than a first CCE of msk,nCI,2(16) or msl,nCI,2(16) in a CORESET configured with cce-REG-MappingType = 'nonInterleaved' and with duration of one symbol.

If a UE

-	is provided monitoringCapabilityConfig = r16monitoringcapability for a downlink cell,

-	is provided, by searchSpaceLinking, for search space sets si and sj on the downlink cell respective values sj and si, and

-	indicates three-BDforSSsetLinking

the UE counts each PDCCH candidate for the one of the search space sets si and sj that the UE monitors PDCCH in the later span, as two PDCCH candidates.

A UE does not expect to be provided freqMonitorLocations for a search space set s in a serving cell if intraCellGuardBandsDL-List indicates that no intra-cell guard-bands are configured for the serving cell.

A UE that

-	is configured for operation with carrier aggregation, and

-	indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and

-	has a PDCCH candidate with CCE aggregation level L in CORESET p associated with search space set si for detection of a first DCI format, other than DCI format 0_0 or DCI format 1_0, having a first size and scheduling

-	PUSCH transmission or configured grant Type 2 PUSCH release on serving cell nCI,2, or

-	PDSCH reception or having associated HARQ-ACK information  without scheduling PDSCH reception on serving cell nCI,2,

can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level L in CORESET p associated with search space set sj for detection of a second DCI format having a second size and associated with scheduling on serving cell nCI,1 if the first size and the second size are same and if neither of search space sets si and sj includes searchSpaceLinking.

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP.

A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, C-RNTI, CS-RNTI, or MCS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH reception and PSSCH transmission on a same serving cell.

A PDCCH candidate with index msj,nCI(L) for a search space set sj using a set of L CCEs in a CORESET p on the active DL BWP for serving cell nCI is not counted for monitoring if there is a PDCCH candidate with index msi,nCI(L) for a search space set si<sj, or if there is a PDCCH candidate with index nsj,nCI(L) and nsj,nCI(L)<msj,nCI(L), in the CORESET p on the active DL BWP for serving cell nCI using a same set of L CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index msj,nCI(L) is counted for monitoring.

For search space sets si and sj that include searchSpaceLinking with values sj and si, and for search space set sk that does not include searchSpaceLinking, when a UE

-	monitors PDCCH candidates msi,nCI,1(L)=msj,nCI,1(L) for detection of a first DCI format,

-	monitors PDCCH candidate msk,nCI,2(L) for detection of a second DCI format having a same size as the first DCI format,

-	the PDCCH candidate msi,nCI,1(L), or the PDCCH candidate msj,nCI,1(L), and the PDCCH candidate msk,nCI,2(L) have identical scrambling and use a same set of CCEs over same symbols in a slot in a CORESET p,

the PDCCH candidate msk,nCI,2(L) is not counted for monitoring and the UE assumes that a detected DCI format is the first DCI format. A UE may monitor PDCCH candidate msk,nCI,2(L) depending on a corresponding capability [16, TS 38.306]. For search space sets si and sj that include searchSpaceLinking with values sj and si, and for search space set sk that does not include searchSpaceLinking, when a UE

-	monitors PDCCH candidates msi,nCI,1(8)= msj,nCI,1(8) for detection of a first DCI format and monitors PDCCH candidate msk,nCI,2(16) for detection of a second DCI format, or monitors PDCCH candidates msi,nCI,1(16)=msj,nCI,1(16) for detection of the first DCI format and monitors PDCCH candidate msk,nCI,2(8) for detection of the second DCI format, and

-	one of the PDCCH candidates msi,nCI,1(8) and msj,nCI,1(8), and the PDCCH candidate msk,nCI,2(16), or one of the PDCCH candidates msi,nCI,1(16) and msj,nCI,1(16), and the PDCCH candidate msk,nCI,2(8), have a first CCE with same index and are simultaneously monitored in a CORESET p with cce-REG-MappingType = 'nonInterleaved' and duration of one symbol,

the UE assumes that a detected DCI format is the first DCI format.

For search space sets si, sj, sk, and sl that include searchSpaceLinking with values sj, si, sl, and sk, respectively, and for detection of DCI formats with same size, a UE expects different CCEs or different scrambling in a CORESET p for any of first PDCCH candidates msi,nCI,1(L) and msj,nCI,1(L), with msi,nCI,1(L)=msj,nCI,1(L), and any of second PDCCH candidates msk,nCI,2(L) and msl,nCI,2(L), with msk,nCI,2(L)=msl,nCI,2(L) that the UE would simultaneously monitor.

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, MPDCCHmax,slot,μ, per slot for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2: Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of monitored PDCCH candidates per slot and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-2A provides the maximum number of monitored PDCCH candidates, MPDCCHmax,X,Y,μ, per span for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2A: Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span for combination (X,Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 14 | 28 | 44 |
| 1 | 12 | 24 | 36 |

Table 10.1-2B provides the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2B: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 20 | 20 | - | - |
| 6 | 10 | 10 | 20 | 20 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, CPDCCHmax,slot,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of non-overlapped CCEs per slot and per serving cell CPDCCHmax,slot,μ |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

Table 10.1-3A provides the maximum number of non-overlapped CCEs, CPDCCHmax,X,Y,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell.

Table 10.1-3A: Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs in a span for combination X,Y for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 18 | 36 | 56 |
| 1 | 18 | 36 | 56 |

Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell.

Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 32 | 32 | - | - |
| 6 | 16 | 16 | 32 | 32 |

If a UE

-	does not report pdcch-BlindDetectionCA or is not provided BDFactorR, γ=R

-	reports pdcch-BlindDetectionCA, the UE can be indicated by BDFactorR either γ=1 or γ=R

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability but not provided CORESETPoolIndex, with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWPs of the scheduling cells,

-	more than MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,0DL,μ downlink cells, or

-	more than MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=γ∙CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

-	more than MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHmax,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

Ncellscap is replaced by Ncells,r15cap-r16, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability.

If a UE

-	is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability but not provided coresetPoolIndex,

-	with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and

-	a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell,

the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells. Ncellscap is replaced by Ncells,r15cap-r16 if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability.

For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot.

For each scheduled cell from the Ncells,1DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell

-	more than minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot

-	more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value

If a UE is configured with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16,  the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per span for each scheduled cell when the scheduling cell is from the Ncells,r16DL,(X,Y),μ downlink cells. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs

-	per set of spans on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells within every X symbols, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and any pair of spans in the set is within Y symbols, where first X symbols start at a first symbol with a PDCCH monitoring occasion and next X symbols start at a first symbol with a PDCCH monitoring occasion that is not included in the first X symbols

-	per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise

where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

For each scheduled cell from the Ncells,r16DL,(X,Y),μ downlink cells using combination X,Y, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(X,Y),μ,MPDCCHtotal,(X,Y),μ PDCCH candidates or more than minCPDCCHmax,(X,Y),μ,CPDCCHtotal,(X,Y),μ non-overlapped CCEs per span.

A UE does not expect to be configured CSS sets, except for CSS sets provided by searchSpace-Multicast, that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot, per group of Xs slots for a corresponding combination Xs,Ys, or per span that exceed the corresponding maximum numbers per slot, or per group of Xs slots for a corresponding combination Xs,Ys, or per span, respectively.

For same cell scheduling or for cross-carrier scheduling, a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot or per span on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot or per span, respectively. If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, except the first span of each slot, the UE does not expect a number of PDCCH candidates and a number of corresponding non-overlapped CCEs per span on the primary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the primary cell per span.

For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per span or per slot are separately counted for each scheduled cell.

For all search space sets within a slot n, or within a group of Xs slots for a corresponding combination Xs,Ys, or within a span in slot n, denote by Scss a set of CSS sets, except for CSS sets provided by searchSpace-Multicast, with cardinality of Icss and by Suss a set of USS sets and CSS sets provided by searchSpace-Multicast with cardinality of Juss. The location of search space sets sj, 0≤j<Juss, in Suss is according to an ascending order of the search space set index.

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates three-BDforSSsetLinking and is provided for search space set sj, by searchSpaceLinking, a search space set si with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

For the CSS sets in Scss, a UE monitors MPDCCHcss=i=0Icss-1LMScss(i)(L) PDCCH candidates requiring a total of CPDCCHcss non-overlapping CCEs in a slot, of in group of Xs slots for a corresponding combination Xs,Ys, or in a span.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, or in a group of Xs slots for a corresponding combination Xs,Ys if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.

In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(Xs,Ys),μ and CPDCCHmax,(Xs,Ys),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ respectively.

Denote by VCCESuss(j) the set of non-overlapping CCEs for search space set Suss(j) and by CVCCESuss(j); the cardinality of VCCESuss(j) where a UE determines the non-overlapping CCEs for search space set Suss(j) considering the allocated PDCCH candidates for monitoring for the CSS sets in Scss and the allocated PDCCH candidates for monitoring for all search space sets Suss(k), 0≤k≤j.

Set MPDCCHuss=minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ-MPDCCHcss

Set CPDCCHuss=minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ-CPDCCHcss

Set j=0

while LMSuss(j)(L)≤MPDCCHuss AND CVCCESuss(j)≤CPDCCHuss

allocate LMSuss(j)(L) PDCCH candidates for monitoring to search space set Suss(j)

MPDCCHuss=MPDCCHuss-LMSuss(j)(L);

CPDCCHuss=CPDCCHuss-CVCCESuss(j);

j=j+1;

end while

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs that have been configured with qcl-Type set to same 'typeD' properties as the CORESET, on the active DL BWP of a cell from the one or more cells

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band,

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells, and

-	is provided two-QCLTypeDforPDCCHRepetition

the UE monitors PDCCHs only in a first CORESET with qcl-Type set to first 'typeD' properties and, if any, in a second CORESET with qcl-Type set to second 'typeD' properties that are different than the first 'typeD' properties, and in any other CORESET from the multiple CORESETs with corresponding qcl-Type set to the first 'typeD' properties and/or to the second 'typeD' properties

-	the first CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS sets, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	excluding CSS sets and USS sets associated with CORESETs with qcl-Type set to first 'typeD' properties, the second CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS sets; if any; otherwise, to the USS set with the lowest index in the cell with lowest index, where the CSS set or the USS set includes searchSpaceLinking with a value indicating, respectively, any CSS set or any USS set associated with CORESETs with qcl-Type set to first 'typeD' properties

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band,

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells,

-	one or more CORESETs have two activated TCI states, and

-	reports twoTypeDcapabilityname

the UE monitors PDCCHs only in a CORESET with a first qcl-Type set to first 'typeD' properties and, if any, a second qcl-Type set to second 'typeD' properties that are different than the first 'typeD' properties, and in any other CORESET from the multiple CORESETs with corresponding qcl-Type set to the first 'typeD' properties or to the second 'typeD' properties

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

For the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL 'typeD' properties than a CSI-RS.

For the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL 'typeD' properties.

The allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells.

The number of active TCI states is determined from the multiple CORESETs.

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states configured with qcl-Type set to 'typeD',

the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs.

For a scheduled cell and at any time, a UE expects to have received at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol and at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol.

If a UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any serving cell, and

-	is not configured for NR-DC operation and indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for Ncellscap≥4 downlink cells and the UE is configured with NcellsDL>4 downlink cells or NcellsUL>4 uplink cells, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received at most 16∙Ncellscap PDCCHs for

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all NcellsDL downlink cells

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all NcellsUL uplink cells

If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for all serving cells, and

-	is not configured for NR-DC operation and indicates through pdcch-MonitoringCA a capability to monitor PDCCH candidates for Ncellscap-r16≥2 downlink cells and the UE is configured with NcellsDL>2 downlink cells or NcellsUL>2 uplink cells, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received at most 16∙Ncellscap-r16 PDCCHs for

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap-r16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all NcellsDL downlink cells

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap-r16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all NcellsUL uplink cells.

If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for at least one serving cell and is not provided monitoringCapabilityConfig = r16monitoringcapability for at least one serving cell, and

-	is not configured for NR-DC operation, and indicates a capability to monitor PDCCH candidates for Ncells,r15cap-r16≥1 downlink cells and Ncells,r16cap-r16≥1 downlink cells, and the UE is configured with  NcellsDL>1 downlink cell or NcellsUL>1 uplink cell, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received

-	at most 16∙Ncells,r15cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r15cap-r16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all serving cells that are not provided monitoringCapabilityConfig = r16monitoringcapability

-	at most 16∙Ncells,r15cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r15cap-r16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all serving cells that are not provided monitoringCapabilityConfig = r16monitoringcapability

-	at most 16∙Ncells,r16cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r16cap-r16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all serving cells that are provided monitoringCapabilityConfig = r16monitoringcapability

-	at most 16∙Ncells,r16cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r16cap-r16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all serving cells that are provided monitoringCapabilityConfig = r16monitoringcapability

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a CSS set and a second PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a USS set, where the CSS set and the USS set do not include searchSpaceLinking, in a CORESET with index zero on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_0/1_0 associated with the first PDCCH candidate.

If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.

A UE configured with a bandwidth part indicator in a DCI format determines, in case of an active DL BWP or of an active UL BWP change, that the information in the DCI format is applicable to the new active DL BWP or UL BWP, respectively, as described in clause 12.

For unpaired spectrum operation, if a UE is not configured for PUSCH/PUCCH transmission on serving cell c2, the UE does not expect to monitor PDCCH on serving cell c1 if the PDCCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10, TS 38.133]) on serving cell c2 and if the UE is not capable of simultaneous reception and transmission on serving cell c1and serving cell c2.

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception as described in [6, TS 38.214]. If a PDCCH candidate in a slot is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

A UE does not expect to be configured with dci-FormatsSL and dci-FormatsExt in a same USS.

## *** Unchanged text is omitted ***

# 13	UE procedure for monitoring Type0-PDCCH CSS sets

If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Tables 13-10A, 13-10B and 13-10C for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix.

For operation without shared spectrum channel access and for operation with shared spectrum channel access in FR2-2, a UE assumes that the offset in Tables 13-1 through 13-10C is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10 kSSB is defined in [4, TS 38.211].

For operation with shared spectrum channel access in FR1, a UE determines an offset from a smallest RB index of the CORESET for Type0-PDCCH CSS set to a smallest RB index of the common RB overlapping with a first RB of the corresponding SS/PBCH block

-	according to the offset in Table 13-1A or Table 13-4A, if the frequency position of the SS/PBCH block corresponds to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], and

-	according to a sum of a first offset and a second offset if the frequency position of the SS/PBCH block is provided by ssbFrequency in a measurement configuration associated with a reporting configuration providing reportCGI and does not correspond to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], where

-	the first offset is provided in Table 13-1A or Table 13-4A, and

-	the second offset is determined as the offset from a smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block indicated in the measurement configuration to a smallest RB index of the common RB overlapping with the first RB of a SS/PBCH block hypothetically located at the GSCN of a synchronization raster entry, where the single synchronization raster entry is located in the same channel as the SS/PBCH block used for the shared spectrum channel access procedure, as described in [15, TS 37.213]

where the offsets are defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set that is same as the SCS of the corresponding SS/PBCH block.

## *** Unchanged text is omitted ***

Table 13-15A: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {480, 480} kHz or {960, 960} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 31) |
| --- | --- | --- |
| 0 | SFNc=SFNSSB,i nc=nSSB,i | 2, 9 in i=2k, i=2k+1 |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search.

If a UE detects a SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for kSSB=31 for FR1 or for kSSB=15 for FR2, the UE determines that there is no SS/PBCH block having an associated Type0-PDCCH CSS set within a GSCN range NGSCNReference-NGSCNStart, NGSCNReference+NGSCNEnd. NGSCNStartand NGSCNEnd are respectively determined by controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1. If the GSCN range is NGSCNReference, NGSCNReference, the UE determines that there is no information for a second SS/PBCH block with a CORESET for an associated Type0-PDCCH CSS set on the detected SS/PBCH block.

If a UE does not detect any SS/PBCH block providing a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, within a time period determined by the UE, the UE may ignore the information related to GSCN of SS/PBCH locations in performing cell search.

Table 13-16: Mapping between the combination of kSSB and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to NGSCNOffset for FR1

| kSSB | 16×controlResourceSetZero +searchSpaceZero | NGSCNOffset |
| --- | --- | --- |
| 24 | 0, 1, …, 255 | 1, 2, …, 256 |
| 25 | 0, 1, …, 255 | 257, 258, …, 512 |
| 26 | 0, 1, …, 255 | 513, 514, …., 768 |
| 27 | 0, 1, …, 255 | -1, -2, …, -256 |
| 28 | 0, 1, …, 255 | -257, -258, …, -512 |
| 29 | 0, 1, …, 255 | -513, -514, …., -768 |
| 30 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |

Table 13-17: Mapping between the combination of kSSB and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to NGSCNOffset for FR2

| kSSB | 16×controlResourceSetZero +searchSpaceZero | NGSCNOffset |
| --- | --- | --- |
| 12 | 0, 1, …, 255 | 1, 2, …, 256 |
| 13 | 0, 1, …, 255 | -1, -2, …, -256 |
| 14 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |