3GPP TSG- RAN WG1 Meeting #108-e                                                  R1-2201763

e-Meeting, Feb 21st – Mar 3rd, 2022

Agenda Item:	8.1.5

Source:	Apple Inc.

Title:	On Further MIMO Enhancement Beyond Rel-17

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on further MIMO enhancement.

# AI based beam selection

Traditional beam selection requires gNB to transmit several periodic downlink reference signals, and UE performs beam measurement and report based on these reference signals. Based on the beam report, gNB can identify the best beam and provide TCI indication based on the selected beam.

The overall overhead and latency for traditional beam selection could increase as the number of gNB/UE beams increases. For example, if 64 beams are utilized at gNB side, 64 downlink reference signals may be needed. From UE side, to maintain a good gNB-UE beam pair, more power consumption and memory would be required as more number of beams are used.

For spatially correlated UEs, the beam network beam could be highly correlated. Based on spatial correlation, it is possible that the network can predict the best beam for some UEs without beam measurement and report. The spatial correlation can be determined based on some fast-fading parameters, e.g. delay/power/direction for each path. It is not easy to make a hard decision on whether two channels are spatially correlated or not. But with the help of machine learning, it is possible to predict whether two channels are spatially correlated, and they should share the same best network beam.

Usually, a UE can work in a FR1+FR2 CA mode. It is possible to determine the spatial correlation based on some FR1 feedback, and the network can predict the FR2 beam based on the FR1 feedback. In this contribution, we select the normalized channel impulse response (CIR) as the FR1 feedback to predict the FR2 beam. Figure 1 illustrates the procedure for FR1 assisted FR2 beam selection.

Figure 1: Procedure for FR1 assisted FR2 beam selection

The input for the neural network is the first 40 samples for the normalized CIR between UE and FR1 cell. 50000 UEs are dropped within a cell with 3 sectors, where 70% UEs are used for training and 30% UEs are used for validation. For FR2, it is assumed 32 network beams are unitized and each UE contains 2 panels with 4 beams per panel. Detailed simulation results are provided in Table A-1 in appendix.

Figure 2-5 illustrates some system level results for AI based beam selection for LOS only status.

Figure 2: RSRP distribution for AI selected beam for LOS only

Figure 3: Distribution of RSRP gap between ideal beam and AI selected beam for LOS only

Figure 4: Distribution of ideal beam for LOS only

Figure 5: Distribution of AI selected beam for LOS only

Figure 6-9 illustrates some system level results for AI based beam selection for mixed NLOS/LOS case.

Figure 6: RSRP distribution for AI selected beam for mixed NLOS/LOS

Figure 7: Distribution of RSRP gap between ideal beam and AI selected beam for mixed NLOS/LOS

Figure 8: Distribution of ideal beam for mixed NLOS/LOS

Figure 9: Distribution of AI selected beam for mixed NLOS/LOS

It can be observed that AI based FR1 assisted FR2 beam selection scheme can provide accurate beam selection with the accuracy of about 70% for mixed NLOS/NLOS case and 90% for LOS only case.

Observation 1: AI based FR1 assisted FR2 beam selection scheme can provide accurate beam selection with the accuracy of about 70% for mixed NLOS/NLOS case and 90% for LOS only case.

# Further enhancement on unified TCI framework

The unified TCI framework was introduced in Rel-17, which can reduce the beam indication signaling overhead. The gNB can update the beam for multiple channels based on a single TCI indication signaling. Moreover, the power control parameters can also be updated as a new TCI is indicated. For a UE with carrier aggregation mode, the TCI state for multiple CCs within a band can also be updated by a single TCI indication signaling. However, Rel-17 only supports unified TCI framework with sTRP operation, since gNB cannot indicate more than 1 TCI states.

Rel-17 also reduces the TCI update latency a bit by introducing DCI based TCI indication. The overall latency for known TCI indication with maintained pathloss reference signal is reduced by up to 3ms. But if the associated pathloss reference signal is not maintained for the new TCI state, the latency for TCI update could still be about 100ms, which is used for L3-RSRP measurement for the new pathloss reference signal, as shown in Figure 10 with the assumption of 20ms periodicity for the new pathloss reference signal. Thus, it is still necessary to reduce the latency for TCI update when extension of unified TCI is considered.

Figure 10: Action delay for joint/UL TCI state update when associated pathloss reference signal is not maintained

Observation 2: The latency for unified TCI update could be about 100ms for some cases, and it is still necessary to reduce the latency for TCI update when extension of unified TCI is considered.

To extend the unified TCI framework to support mTRP operation, it is necessary to study whether CCs with different sTRP/mTRP configuration is a typical configuration. Since Rel-17 supports common TCI ID update for multiple CCs, if different CCs can be configured with different sTRP/mTRP operation, the signaling could become complicated. In addition, Rel-17 mTRP operation is based on Rel-16 framework, where the transmission scheme for PUCCH/PUSCH is dynamically switched. But unified TCI is applied for all the PUCCH/PUSCH. A more flexible framework for unified TCI indication can be studied.

Observation 3: A more flexible framework for unified TCI indication can be studied with regard to different sTRP/mTRP operation in different CCs and dynamic sTRP/mTRP transmission scheme switching.

Two-TA operation was discussed in Rel-17 for inter-cell mTRP, but there was not good justification to support two TAs, and RAN1 made a conclusion that the propagation delay difference between two TRPs should be assumed to be within CP.  Usually, a UE with mTRP operation should be in the cell edge, and the propagation delay between two cells should be quite closed. To maintain two TAs would increase the UE complexity quite a lot, e.g. a UE may need to maintain two TAGs for some CCs and apply different timings to transmit uplink signals in different symbols, and it could also increase the scheduling complexity in gNB side.

Observation 4: To maintain two TAs per CC would increase the complexity for both gNB and UE quite a lot, and more evaluation is required for the justification of the necessity for two TAs.

# Consideration on CSI feedback for high-speed scenarios

Figure 11 Dominant spatial beams change with time

For CSI feedback targeting MU-MIMO, one serious issue is CSI aging. For high speed scenarios, typically the cell radius can be large and hence the number of UEs in a cell may not be so small. Open loop MIMO may not deliver high enough spectrum efficiency for that case. How to support MU-MIMO for high speed scenarios may merit some study.  In the illustration, it is shown with the pass of time, initially 4 spatial beams capture significant power emitting from the base station. After a while the spatial beams capture most of the power have moved to new beams as indicated by the green arrows. Similarly, there may be variations of relative delays relative frequency offsets with time. If the UE can estimate such drifts and includes them in the CSI feedback. With that, then the base station can track the variations without incurring frequent CSI feedback from the UE.

# Conclusion

In this contribution, we provided some discussion on further MIMO enhancement. Based on the discussion, the following observations have been achieved.

Observation 1: AI based FR1 assisted FR2 beam selection scheme can provide accurate beam selection with the accuracy of about 70% for mixed NLOS/NLOS case and 90% for LOS only case.

Observation 2: The latency for unified TCI update could be about 100ms for some cases, and it is still necessary to reduce the latency for TCI update when extension of unified TCI is considered.

Observation 3: A more flexible framework for unified TCI indication can be studied with regard to different sTRP/mTRP operation in different CCs and dynamic sTRP/mTRP transmission scheme switching.

Observation 4: To maintain two TAs per CC would increase the complexity for both gNB and UE quite a lot, and more evaluation is required for the justification of the necessity for two TAs.

# Appendix – Simulation assumption

Table A-1: Simulation Assumption

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban Macro |
| Number of cells | 1 |
| Number of sectors per cell | 3 |
| Number of UEs | 50000, where 70% for training and 30% for validation |
| Carrier frequency | FR1: 3GHz FR2: 28GHz |
| Subcarrier spacing | FR1: 30kHz FR2: 120kHz |
| Bandwidth | FR1: 20MHz FR2: 80MHz |
| gNB antenna configuration | FR1: (M, N, P, Mp, Np, Mg, Ng) = (2, 8, 2, 2, 8, 1, 1) FR2: (M, N, P, Mp, Np, Mg, Ng) = (4, 8, 2, 1, 1, 1, 1) |
| UE antenna configuration | FR1: (M, N, P, Mp, Np, Mg, Ng) = (1, 2, 2, 1, 2, 1, 1) FR2: (M, N, P, Mp, Np, Mg, Ng) = (2, 4, 2, 1, 1, 1, 2) |
| UE distribution | Uniformly distributed |
| UE orientation | Random |
| Slow fading model | 38.901 |
| Fast fading model | 38.901 with spatial consistency |
| FR2 gNB beam pattern | 4 beams in vertical * 8 beams in horizontal |
| FR2 UE beam pattern | 4 beams in horizontal per panel |