3GPP TSG RAN WG1#108		R1-22xxxxx

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.2.2

Source:	Moderator (Lenovo)

Title:	Email discussion [108-e-NR-52-71GHz-02] for B52.6 GHz PDCCH monitoring enhancements

Document for:	Discussion, Decision

# Introduction

As stated by the chairman:

[108-e-NR-52-71GHz-02] Email discussion for maintenance on PDCCH monitoring enhancements – Alex (Lenovo)

- 1st check point: February 25

- Final check point: March 3

Depending on the progress, new questions or proposals may be added for individual items.

# Discussion

FL NOTE: Excerpts from submitted documents are listed in Section 3.

## Topic A1: Blind Decoding Capability, Multi-slot monitoring

### (Closed) Issue A1-1: Configuration of multi-slot monitoring, general MSM capability

#### First round discussion

  - Proposal A1-1.1 (see R1-2201914): Adopt the following TP to cover the RAN1#107-e agreement: BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS.

TS 38.213 Clause 13 ============================= Unchanged part omitted ========================================= 13 UE procedure for monitoring Type0-PDCCH CSS sets If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Tables 13-10A, 13-10B and 13-10C for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. *<omitted text>*. For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, for FR1 and FR2-1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two consecutive slots starting from slot . For FR2, UE monitors PDCCH in the Type0-PDCCH CSS set over two slots, slot and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz. For SS/PBCH block with index , the UE determines an index of slot as that is in a frame with system frame number (SFN) satisfying if , or in a frame with SFN satisfying if . and are provided by Tables 13-11 and 13-12, and based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots and and is the first symbol index provided by Tables 13-11 and 13-12. For operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214]. For FR1 and FR 2-1, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two consecutive slots starting from slot n0 include the associated Type0-PDCCH monitoring occasions. For FR2, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two slots, slot n0 and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz, include the associated Type0-PDCCH monitoring occasions. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. M and O are provided by Table 13-11, and μ∈{0, 1} based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots n0 and n0+1 is the first symbol index provided by Table 13-11. The UE does not expect to be configured with M=1/2, or with M=2, when NSSBQCL=1. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over one slot with Type0-PDCCH CSS set periodicity equal to the periodicity of SS/PBCH block. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, if the active DL BWP is the initial DL BWP, the UE is expected to be able to perform radio link monitoring, as described in Clause 5, and measurements for radio resource management [10, TS 38.133] using a SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set. For a SS/PBCH block with index , the UE determines the slot index and based on parameters provided by Tables 13-13 through 13-15. ============================= Unchanged part omitted =========================================

| Company | Comment |
| --- | --- |
| Samsung | We believe the technical aspect of this TP has already been addressed by current specification, and the TP is not using the latest version of the specification. |
| Ericsson | Agree with Samsung that the agreement on monitoring in slots n0 and n0+X is already captured in the most recent version of 38.213. But we did notice a typo in Section 13 just now, and maybe we can agree on correcting it: For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two slots n0. For SS/PBCH block with index i, the UE determines an index of slot n0 as n0=O∙2μ+i∙MmodNslotframe,μ that is in a frame with system frame number (SFN) SFNC satisfying SFNcmod2=0 if O∙2μ+i∙MNslotframe,μmod2=0, or in a frame with SFN satisfying SFNcmod2=1 if O∙2μ+i∙MNslotframe,μmod2=1 where μ∈0,1,2,3,5,6 based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211] . - For μ∈{0, 1, 2, 3} and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+1. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+1 are provided by Table 13-11 and Table 13-12. - For μ=5 and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+4. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+4 are provided by Table 13-12A, where X=1.25. - For μ=6 and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+8. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+8 are provided by Table 13-12A, where X=0.625. |
| Qualcomm | Agree with Samsung’s comment. The latest version of TS 38.213 is already reflecting the changes required for the new Type0 CSS design. |
| vivo | Agree with Samsung and Ericsson |
| Nokia, NSB | Agree with Samsung. This TP is not needed. |
| Intel | Agree with Samsung and Ericsson |
| ZTE, Sanechips | We agree with Samsung and Ericsson and there is no need to agree on this proposal. |
| LG Electronics | Agree with Samsung’s comment. |
| Apple | Agree with previous companies. We are okay with Ericsson’s update to fix the typo. |
| Transsion | Agree with Samsung and Ericsson’s comment. |

| Futurewei | Agree with Samsung that the TP is not necessary. |
| --- | --- |
| Lenovo | Agree the typo identified by Ericsson should be fixed. |
| CATT | Agree with Samsung’s comment. |
| Huawei/Hisilicon | Agree with all companies above. Support fixing the typo pointed out by Ericsson. |

  - Proposal A1-1.2 (see R1-2202273): Adopt the following TP to cover the agreement: BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots.

  - FL Note: TP has been editorially modified by FL.

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE is not expected to monitor PDCCH for these CSS/USS sets in any other of the Xs slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. ...

| Company | Comment |
| --- | --- |
| Samsung | We didn’t see a strong need of this proposal, since it’s already implied by current text. |
| Ericsson | Agree with Samsung; this TP is not needed. The wording "… in any slot of the Ys slots" means that the monitoring is restricted to the Ys slots. No need to clarify further. |
| Nokia, NSB | This TP is not needed (wording is clear enough already) |
| ZTE, Sanechips | It is unnecessary to add that sentence as shown in the above TP. |
| Panasonic | The current spec only describes that what UE “can” do for any slot of the Ys slots, but does not describe any UE behavior for slots outside Ys slots. Without such restriction, it implies UE still needs to monitor slots outside outside Ys slots for Group (1) SSs if configured by gNB via search space configuration. Then the whole multi-slot monitoring mechanism of Xs,Ys becomes meaningless. Therefore, we think the TP is necessary. |
| LG Electronics | Same view with Samsung. |

| Futurewei | The TP is not needed |
| --- | --- |
| Lenovo | Current specification seems sufficiently clear on this topic. |
| CATT | No needed |

  - Proposal A1-1.3 (see R1-2202273): In case that MO of Group (1) CSS is changed, the location of Ys within Xs can be adapted accordingly to include all CSS MOs (of Group(1) SS) that are monitored by UE.

  - FL Note: TP has been editorially modified by FL.

TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots until a new monitoring occasion of Type1-PDCCH CSS set is provided by dedicated higher layer signalling, or of Type3-PDCCH CSS sets is configured or indicated to the UE. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively.

Please comment whether the proposal and TP are agreeable.

| Company | Comment |
| --- | --- |
| Moderator | If agreeable, perhaps the second part of the change could be modified for improved clarity: […], or until a new monitoring occasion of Type3-PDCCH CSS sets is configured or indicated to the UE. |
| Samsung | We don’t think this change is essentially needed, since there is no way of confusion in implementation. |
| Ericsson | Fundamentally, this TP is not needed. It is always the case that a configuration holds until a new configuration is received. If it was needed to always specify what happens upon reconfiguration, then the spec would need to be updated in many places. |
| Nokia, NSB | We also think that TP does not add value. |
| Intel | We share views from Samsung and Ericsson. Further, the change of position of the Ys slot may be triggered by the change of USS sets or Type-1/3 CSS sets. Why only Type-1/3 CSS sets are captured in the TP? |
| ZTE, Sanechips | We share the similar view with Samsung and it is not strongly necessary to add the explanation. |
| Panasonic | We support the TP as proposing company. It is true that there is no need to always specify what happens upon reconfiguration. However, the intention here is to prioritize CSS over USS considering the fact it might not be always feasible to restrict both CSS and USS within Ys (e.g. due to beam swtiching). In other words, location of Ys should follow the new configured/indicated MOs of CSS (of Group (1)) in order not to miss important control information. Certain USS MO would be dropped if it cannot be included in Ys . |
| LG Electronics | Agree with comments from Samsung and Ericsson. |
| Apple | This TP seems to try and address a problem that occurs because we are limited to Ys for all the Group (1) SSs. If I understand what this is saying, it means that “if there is a reconfiguration of one of the CSSs, then all of the Group (1) SSs move to the new location or any CSSs/USSs that are not should be dropped”. We may need to agree on the problem /solution and then decide on the TP. |
| Transsion | Agree with Samsung and Ericsson’s comment. |

| Futurewei | We do not see any confusion in the original text, therefore the TP is not necessary. |
| --- | --- |
| Lenovo | It seems ok without the suggested change. |
| CATT | Not needed |

#### First round discussion summary

Most companies think none of these TPs are necessary as the spec is sufficiently clear. There is no consensus to adopt Proposal A1-1.1, Proposal A1-1.2 or Proposal A1-1.3.

However, there is consensus to fix the following error in 38.213 clause 13, that should be corrected:

Text Proposal A1-1.4

For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two slots n0. For SS/PBCH block with index i, the UE determines an index of slot n0 as n0=O∙2μ+i∙MmodNslotframe,μ that is in a frame with system frame number (SFN) SFNC satisfying SFNcmod2=0 if O∙2μ+i∙MNslotframe,μmod2=0, or in a frame with SFN satisfying SFNcmod2=1 if O∙2μ+i∙MNslotframe,μmod2=1 where μ∈0,1,2,3,5,6 based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211] . - For μ∈{0, 1, 2, 3} and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+1. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+1 are provided by Table 13-11 and Table 13-12. - For μ=5 and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+4. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+4 are provided by Table 13-12A, where X=1.25. - For μ=6 and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+8. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+8 are provided by Table 13-12A, where X=0.625.

The following has been declared by email approval:

The following TP for Clause 13 of TS38.213v17.0.0 is endorsed

---- Start of TP ----

13  UE procedure for monitoring Type0-PDCCH CSS sets

--- unchanged text omitted ---

For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two slots .

--- unchanged text omitted ---

---- End of TP ----

### (Closed) Issue A1-2: [High Priority] Support of optional/FFS (X,Y) values

#### First round discussion

Proposal A1-2.1: Support Xs,Ys=(2, 1) as optional value for 480 kHz SCS with 10 maximum monitored PDCCH candidates and 16 maximum non-overlapped CCEs.

Proposal A1-2.2: Conclude that Xs,Ys=(2, 1) for 480 kHz SCS is not supported for multi-slot monitoring.

FL Summary: Only few companies showed active support for Xs,Ys=(2, 1) in their contributions. The discussion in RAN1#107bis-e showed a majority not supporting it, so it seems fair to say that there is no consensus to adopt Proposal A1-2.1. Therefore FL suggests to adopt Proposal A1-2.2: Conclude that Xs,Ys=(2, 1) for 480 kHz SCS is not supported for multi-slot monitoring.

| Company | Comment |
| --- | --- |
| MediaTek | We support FL’s suggestion on Proposal A1-2.2. |
| Samsung | We prefer to support Proposal A1-2.1. |
| Ericsson | We support Proposal A1-2.2. We think that (2,1) with BD/CCE budget of 10/16 is flawed in that the CCE budget is not enough for Type0-PDCCH monitoring which requires 4 + 8 + 16 = 28 non-overlapping CCEs according to Table 10.1-1. Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1 CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. |
| Qualcomm | We support Proposal A1-2.1. |
| InterDigital | We support Proposal A1-2.1. |
| LG Electronics | We support Proposal A1-2.1. Optional (2,1) can be supported for SCell without any flawness. |
| Sharp | Support FL’s suggestion. |
| Huawei/HiSilicon | We support A1-2.2 |
| Xiaomi | support FL’s suggestion on Proposal A1-2.2 |
| vivo | We support Proposal A1-2.1. |
| Nokia, NSB | We support Proposal A1-2.2 |
| Intel | We support Proposal A1-2.1. Given combination (4, 1) is already supported for SCS 960kHz, we would like to hear the comments on the complexity to further support combination (2, 1) for SCS 480kHz. |
| ZTE, Sanechips | We agree with FL’s suggestion on Proposal A1-2.2. |
| NTT DOCOMO | We support FL suggestion to adopt Proposal A1-2.2. In our view, Xs,Ys=(4, 2) for 480 kHz SCS has already agreed to support as optional and this value can provide a similar scheduling flexibility as Xs,Ys=(2, 1) for 480 kHz SCS. |
| Panasonic | We support Proposal A1-2.2. |
| Apple | We support Proposal Proposal A1-2.2. If Proposal A1-2.1 is selected, then the support for FG 3-5b within Ys is not needed and should be modified to FG 3-1. |
| Transsion | We support Proposal A1-2.2. |

| Futurewei | Support Proposal A1-2.2. |
| --- | --- |
| Lenovo | We support Proposal A1-2.2. |
| CATT | Support Proposal A1-2.2. |

#### First round discussion summary

6 companies indicated support of Proposal A1-2.1, while 14 companies indicated support of Proposal A1-2.2. Clearly, there is neither consensus nor a majority to support Xs,Ys=(2, 1) as optional value for 480 kHz SCS. Consequently, FL proposes the following:

Proposal A1-2.2: Conclude that Xs,Ys=(2, 1) for 480 kHz SCS is not supported for multi-slot monitoring.

The following has been declared by email approval:

Conclusion

for 480 kHz SCS is not supported for multi-slot monitoring.

### Issue A1-3: [High Priority] Determination of X in case of multiple supported X values for multi-slot monitoring

#### First round discussion

Proposal A1-3.1:

If the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations, choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring.

FL Note: Many companies see it as unnecessary to additionally choose according to the smallest Ys value, however this could be added if a majority of companies identifies a benefit.

Proposal A1-3.2:

Introduce an RRC parameter to indicate a (Xs,Ys) combination to be used for PDCCH monitoring. The parameter is UE-specific and has the value range {'xs4ys1', 'xs4ys2', 'xs8ys1', 'xs8ys4'}. If the parameter is absent, the UE uses (4,1) for 480 kHz and (8,1) for 960 kHz.

FL Summary: Discussion in RAN1#107bis-e and documents submitted to this meeting shows a majority supporting a rule-based approach as in Proposal A1-3.1. Therefore FL suggests to adopt Proposal A1-3.1.

| Company | Comment |
| --- | --- |
| MediaTek | For Proposal A1-3.1, we prefer to add Ys to the decision rule. We understand that Ys doesn’t play a role on BD/CCE budget but Ys is essential to determine whether a CC follows mandatory or optional capability. That is, a CC with (Xs,Ys)=(4,1) and a CC with (Xs,Ys)=(4,2) are handled differently from UE implementation perspective, e.g., power saving and number of BD within Ys slots. Therefore, it is necessary to distinguish CCs with same Xs but different Ys. For Proposal A1-3.2, we can accept it due to the simplicity. |
| Samsung | We support Proposal A1-3.1, and there is no need to choose a particular value of Ys, since it can be up to UE’s implementation. |
| Ericsson | Our first preference is to introduce an RRC parameter for simplicity (like Proposal A1-3.2). However, we agree with Samsung that there is no need to configure a particular Ys, since only Xs is needed for the UE to know the slot group size (Xs) for evaluating dropping rules. Furthermore, for 480 kHz there is only one supported value of Xs (assume (2,1) is not supported). In summary, we suggest the following amendment to proposal A1-3.2: Proposal A1-3.2a Introduce an RRC parameter to indicate the slot group size Xs for PDCCH monitoring. The parameter is UE-specific and has the value range {'xs4', 'xs8'}. If the parameter is absent, the UE assumes Xs = 4 for 480 kHz and Xs = 8 for 960 kHz. Regarding Proposal A1-3.1, we think that some further discussion would be needed to formulate a proper rule that takes into account compliance across all configured search spaces. |
| Qualcomm | We support Proposal A1-3.1. However, as will be discussed in Issue A2-1, separately from the rule in Proposal A1-3.1, an implicit indication of Xs (i.e., by the bitmap size of monitoringSlotWithinSlotGroup) per search space set is additionally needed. This implicit indication is to avoid ambiguity of SS set configuration, not for the indication of selected UE capability. |
| InterDigital | We support Proposal A1-3.1 and don’t see the necessity to introduce a new RRC parameter. |
| LG Electronics | We support the FL suggestion to adopt Proposal A1-3.1. The proper rule is already included in this proposal that choose an (Xs,Ys) combination from the complying reported set of combinations that results in the largest BD/CCE budget. In addition, we also think it is better to determine a particular value of Ys. Although Ys is not relevant to BD/CCE budget, it would be correct to describe that it operates according to a certain (Xs,Ys) combination rather than according to a certain Xs from the point of view of the specification description. In the current specification, the BD/CCE budget is defined according to a specific (Xs,Ys) combination, not according to a specific Xs. |
| Sharp | We share same view with MediaTek. |
| Huawei/HiSilicon | We support A1-3.1. Moreover, there is no need to determine Ys. In case that UE supports more than one (Xs,Ys) combinations AND the configured search space sets comply with more than one supported (Xs,Ys) combinations, determining the “active” Ys for the UE does not change UE monitoring behavior and has not impact on BD/CCE budget and, therefore, is not required. |
| Xiaomi | Prefer Proposal A1-3.2 but only indicate Xs explicitly, because only Xs will impact the BD/CCE processing capability. and Ys can be determined by the seachspace sets configuration.In fact, we don’t think UE need to determine Ys, it is gNB’s work to guarantee a suitable Ys based seachspace sets configuration. |
| vivo | We support the direction of Proposal A1-3.1 since it is aligned with the way of handling span-based capability in Rel-16. Agree with MTK, we also prefer to add Ys to the decision rule. |
| Nokia, NSB | We support Ericsson’s Proposal A1-3.2a We fail to see benefits in the rule based approach, compared to Proposal A1-3.2 (which is simple and straightforward). Furthermore, SSSG -based approach can be used for dynamic adaptation between multiple (Xs,Ys) combinations |
| Intel | We support proposal A1-3.1 and further support the determination of Ys for the active combination (Xs, Ys). Knowing the value Ys and the position of the Ys slots is critical in SSSG switching. For example, to drop certain USS/CSS if there is back-to-back monitoring which exceed UE capability for PDCCH detection. |
| ZTE, Sanechips | We prefer the Proposal A1-3.2a revised by Ericsson, in our understanding, the value of Xs should be configurable for more flexible operation in above 52.6 GHz band depending on UE capability. If Proposal A1-3.1 is approved, Xs = 4 and 8 slots for 480 and 960 kHz respectively are almost always chosen since those two values are mandatorily supported and associated with the largest BD/CCE budget. We can introduce new RRC parameter to configure the Xs,Ys with smaller value of Xs for low latency and more flexible scenarios. |
| NTT DOCOMO | We support Proposal A1-3.1. We share the same view as Samsung that it is not necessary to indicate or determine the rule to choose Ys since Ys is irrelative to BD/CCE budget and can be indicated via SS set configuration by gNB implicitly. |
| Panasonic | We support Proposal A1-3.1 in principle. However, additional rule for selecting Ys is needed. For example, if UE reports the support of (Xs,Ys) = (4,1) and (4,2). Both combinations would provide the same values of BD/CCE limits. But there is a need for common understanding between gNB and UE on the selected Ys value, because it may result in different USS dropping. We think Ys=2 (i.e. larger value) should be selected to ease the gNB’s scheduling restriction in this example. Therefore, we suggest to add the following to Proposal A1-3.1: - if multiple reported (Xs, Ys) combinations result in the same value of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ , select a (Xs, Ys) combination with a larger Ys. |
| Apple | We are fine with Proposal A1-3.1 and for this purpose we agree that the value of Ys is not needed given that M and C are independent of Ys. We assume that if a UE indicates no support for Ys ≥1, the gNB will not configure this and as such there will be no ambiguity. As has been mentioned, if the current working assumption is used i.e. “The size is 8 bits” then this rule is needed. However, if the FL’s suggestion is used “The size is Xs bits, where Xs is either 4 or 8”, then Xs will be implicitly signaled by the gNB and (a) this rule is not needed or (b) this rule should match with the expected estimate of Xs by the UE. |
| Transsion | We prefer Proposal A1-3.1 and we believe the determination of Ys is not necessary. |

| Futurewei | We prefer an explicit configuration , but we could accept Proposal A1-3.1 if the majority supports it. |
| --- | --- |
| Sony | Though the majority seems in favor of rule-based approach, but there seem different views on the detail of the rule setting. On the other hand, RRC-based approach is simpler and more straightforward in our view. Therefore, we prefer Proposal A1-3.2 |
| Lenovo | We don't think an explicit configuration is necessary. Proposal A1-3.1 looks good to us. If a majority sees a need to further add a condition for choosing according to the smallest Y value. |
| CATT | We support A1-3.1. Also we prefer not to add Ys to the decision rule. |
| Ericsson 2 | We want to check if there is common understanding on one point. Regardless of whether A1-3.1 or A1-3.2 is agreed, we think that the size of the monitoringSlotsWithinSlotGroup (4 or 8) is a separate issue. For example, let's say the UE is capable of both (8,1) and (4,1) for 960 kHz SCS. Further let's consider two configured search spaces each based on (8,1), but the Ys = 1 slot in the two different search spaces are offset by 4 slots from each other. In this case the gNB would configure both search spaces with monitoringSlotsWithinSlotGroup as a length-8 bitmap with a single '1'. However, since the UE is required to monitor Ys = 1 slot every 4 slots considering the two configured search spaces, then the "effective" Xs is 4. The UE would use this value of Xs for determining search space dropping. This is the Xs that Proposal A1-3.1 and Proposal A1-3.2 addresses, not the Xs used for configuration of monitoringSlotsWithinSlotGroup. Is this common understanding? |

#### First round discussion summary

A majority of companies supports overall a rule-based approach as in Proposal A1-3.1, where a small number of companies additionally thinks that also Ys needs to be included in the decision rule. Therefore, FL suggests to allow companies to continue discussion whether Ys is necessary or not, however prefers to have it just as a Note and not as FFS since we are in Rel-17 maintenance phase.

Additionally, Ericsson asks to confim that the determination of (Xs, Ys) for the purpose of calculating the BD/CCE budget (and consequently BD dropping) can be different from the Xs being discussed for the size of monitoringSlotsWithinSlotGroup (see Issue A2-1). This understanding seems to be shared by Qualcomm.

#### Second round discussion

In view of the first round discussion, FL suggests to adopt Proposal A1-3.1a.

Proposal A1-3.1a: (Red highlights show difference to Proposal A1-3.1)

- For the purpose of determining  MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring, if the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations, choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring.

- Note: Discussion whether additionally Ys is to be included in the decision rule can continue.

- Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration

Proposal A1-3.1b (new):

- For the purpose of determining  MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring, if the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations, choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring.

Proposal A1-3.1c (Proposed by Ericsson):

For μ=6, if more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations comply with all configured search space sets, then the UE assumes MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values.

- Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration

Please comment only if you have a strong concern with the proposal.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We support the Proposal A1-3.1a |
| ZTE, Sanechips | We still have concerns about the rule-based approach. In our understanding, we have madatorily supported capability in FR 2-2, the situation in multi-slot PDCCH monitoring is different from that in span-based PDCCH monitoring of Rel-16. In Rel-16 there is no mandatory capabilty and UE can report one or more combinations, However, for multi-slot PDCCH monitoring, (Xs, Ys) = (4, 1) and (8, 1) slots for 480 and 960 kHz respectively are almost always chosen since those two combinations are mandatorily supported and associated with the largest BD/CCE budget. The flexibility of multiple configurable (Xs, Ys) is essentially reduced. Can any companies who support Proposal A1-3.1a address our concern? |
| MediaTek | We would like to point out that the example Ericsson provided in first round discussion might not be valid due to the agreement we made in RAN1 #107e meeting: Agreement - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - The Y consecutive slots can be located anywhere within the slot group of X slots - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0 - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots Therefore, we still have the same question on the relation between the outcome of monitoringSlotsWithinSlotGroup-r17 discussion and determination of Xs and we don’t think the last note is needed. |
| Intel | We prefer to determine both Xs and Ys for the active combination. We are fine to agree on the determination of Xs first and continue discussion on Ys determination |
| Samsung | We support the proposal Proposal A1-3.1a. Response to ZTE: there is a condition expressed in the rule that “if the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations”, which can be utilized to be avoid always choosing the mandatory combination. For example in 960 kHz, if a NW provides configuration only complied with (4,1) but not (8,1), the UE will determine the combination as (4,1). Actually this behavior is exactly the same as span-based PDCCH monitoring, in the sense that the supported three sets have a common element (X,Y)=(7,3), which is effectively the same as the mandatory combination. |
| Moderator | I have added Proposal A1-3.1b above, which strips both notes. I hope this could find wider acceptance then. |
| Sharp | We support the RRC indication of (Xs,Ys) combination. If the budgets of (4,1) and (4,2) are different values at 960 kHz as discussed in 2.3.1.2, the UE and gNB need to recognize the same combination. Notification by RRC parameters is a simple implementation. |
| Qualcomm | We support the proposal. Regarding MediaTek’s comment, we are not sure why Ericsson’s example is not valid due to the agreement. The following is the illustration of Ericsson’s example. Another examples that, if a UE supports both (8,1) and (4,1), and the gNB wants to configure MOs according to (4,1), a SS set may be configured with monitoringSlotsWithinSlotGroup as a length-8 bitmap and another may be configured with length-4 bitmap. The resultant configuration will still satisfy the agreement, “BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots” |
| Ericsson | We still prefer Proposal A1-3.2a from the first round discussion (RRC parameter) since we think it is the simplest solution. However, if that cannot be agreed, then we can accept the rule based approach, but only if the decision rule depends only on Xs (and not Ys). Furthermore, the rule based proposal is only relevant for 960 kHz since the BD/CCE budget is invariant amongst th supported (Xs,Ys) combinations for 480 kHz. Hence we propose to add "For μ=6" into the proposal. Also, we find the wording of A1-3.1b and A1-3.1b is unclear and suggest the following simpler wording (Proposal A1-3.1c). Also, with this wording it becomes clear that if only one reported (Xs,Ys) combination complies with all configured search space sets, then clearly the UE would choose that combination as the "active" (Xs,Ys). This is inline with the comment from Samsung above. Proposal A1-3.1c For μ=6, if more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations comply with all configured search space sets, then the UE assumes MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values. - Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration We think the Note is still very useful to achieve common understanding. It emphasizes that there is a notion of an "active" Xs that is separate from the search space configuration. Please see the rationale for this note below in response to MediaTek. @MediaTek Thank-you for pointing out the highlighted bullet from the agreement. However, we disagree that the example we gave in our first round comments is invalid with respect to the agreement. We repeat the example here. Example 1: For example, let's say the UE is capable of both (8,1) and (4,1) for 960 kHz SCS. Further let's consider two configured search spaces each based on (8,1), but the Ys = 1 slot in the two different search spaces are offset by 4 slots from each other. In this case the gNB would configure both search spaces with monitoringSlotsWithinSlotGroup as a length-8 bitmap with a single '1'. However, since the UE is required to monitor Ys = 1 slot every 4 slots considering the two configured search spaces, then the "active" Xs is 4. The UE would use this value of Xs for determining search space dropping. In this example the "active" Xs is 4, and with slot group size 4, indeed BD attempts for all Group (1) SSs are within the same Y slots, even though each search space is configured according to Xs = 8. This example is chosen to illustrate that it is actually quite useful to stagger different SS because of the very limited number of CCEs for active Xs=4 with 960 kHz SCS. More specifically, in a 32-CCE CORESET, 4 AL=4 candidates cannot be configured to be at the same MO with 2 AL=8 candidates because the required # of CCE will exceed 16 which is the maximum CCE for Xs=4. As a result, the gNB is forced to configure the two different sets of candidates using Example 1 such that only 16 CCE is needed at each MO. Let's also consider another example: Example 2: Assume the following 2 search spaces are configured, and again, assume the UE is capable of both (8,1) and (4,1) for 960 kHz SCS. - SS1: Periodicity = 4 slots and monitoringSlotsWithinSlotGroup = [0 1 0 0] (bitmap length = 4) - Combination (4,1) complies with SS1 - Combination (8,1) does not comply since there is an MO every 4 slots - SS2: Periodicity = 8 slots and monitoringSlotsWithinSlotGroup = [0 1 0 0 0 0 0 0] (bitmap length = 8) - Both (4,1) and (8,1) comply with SS2 According to Proposals A1-3.1a/b/c above, the "active" Xs in this case is 4 even though SS2 is configured according to Xs = 8. Example 2 is also compliant with the agreement, i.e., all BD attempts are in the same Y consecutive slots of each group of Xs = 4 slots. |
| ZTE, Sanechips 2 | Thanks so much for Samsung’s clarification. We agree with your points, if all three sets include the common (X, Y) = (7, 3) which also corresponding to the largest BD/CCE budget, the behavior of span-based PDCCH monitoring is quite similar with that of multi-slot PDCCH monitoring. In this case, we can compromise to rule-based approach. We just think other configurable (Xs, Ys) is hardly to be chosen compared to the mandatory combination (only when the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations). |
| LG Electronics | We share the view with Intel that we prefer to determine both Xs and Ys for the active combination. The reason why Ys should also be specified is as follows. For example, when an active (Xs,Ys) is selected from among (8,1), (8,4), (4,1), or (4,2), one of two combinations with Xs=8 can be selected according to the rule of the proposal. But, it is still ambiguous whether the chosen combination is (8,1) or (8,4). Since the purpose of the rule is to determine MPDCCHmax,Xs,Ys,μ rather than MPDCCHmax,Xs,μ, the rule should clearly determine one (Xs,Ys). We think the rule should be to choose the (Xs,Ys) combination with largest Xs (i.e., largest BD/CCE budget) and Ys=1 as a simplest solution. However, we are also fine to agree on the determination of Xs first and continue discussion on Ys determination. For the last Note in Proposal A1-3.1a, we still have a question for clarification. Assuming that Xs1 indicates an Xs determined by an above rule and Xs2 is the size of the bitmap, it seems that the Note means that Xs2 can be different from Xs1, and then such difference may lead to additional ambiguity. For example, if Xs1=8 and Xs2=4 (or vice versa), how to map each bit in the bitmap to the slots of a slot-group is not very clear to us. |
| Xiaomi | We have two comment on P A1-3.1b, 1, from our understanding, UE does not need to determine the value of Ys, because Ys has no impact on the MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ. gNB needs to make sure the configured SS can conform to any defined Xs/Ys 2, there is parameter monitoringSlotsWithinSlotGroup-r17 is already agreed and details is in discussion, Is Xs can be determined from the bit size of monitoringSlotsWithinSlotGroup-r17? |
| MediaTek2 | Thanks to Qualcomm and Ericssson’s reply. As pointed out by Qualcomm in A2-1 second round, the confusion comes from the definition of “Xs”. Xs was used as slot-group size in previous agreements and Xs is used as the bitmap length of monitoringSlotsWithinSlotGroup-r17 in A2-1. However, even with this explanation, we still have similar question as LG. To avoid confusion, Xs means “effective” slot-group size and X means the bitmap length of monitoringSlotsWithinSlotGroup-r17 in the following description. For example, if gNB only configure one SS set and gNB would like to configure (Xs=4,Ys=1), can gNB use X=8 and monitoringSlotsWithinSlotGroup-r17=(10001000) (this is a similar question asked by Intel in A2-1)? If the configuration is allowed, then the definition of monitoringSlotsWithinSlotGroup-r17 should be modified. If the configuration is not allowed, then there is a relation between Xs and X. Either way, we should not include the following note. We hope this clarifies our view. - Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration Based on the above, we can support A1-3.1b with an FFS on how to determine Ys, which is also mentioned by LG. |
| NTT DOCOMO | We support Proposal A1-3.1c. Regarding the 2nd note in the Proposal A1-3.1a, it should be kept. In our understanding, the gNB would configure SS sets based on the reported UE capability of supporting (Xs, Ys) combination, and then the UE can determine “active” Xs based on the configured SS set. Especially when the multiple (Xs, Ys) combination can meet the SS set configuration by gNB, this (Xs, Ys) determination rule is necessary to determine “active” Xs. It is not necessary that the size of monitoringSlotsWithinSlotGroup and “active” Xs is always the same. |
| LG Electronics2 | After further thought with explanations from Qualcomm and Ericsson, we could see how active Xs (which is determined by the rule) and X (which is the size of the bitmap) can be determined differently and it does not create any ambiguity. As NTT DOCOMO commented, it is not necessary that Xs and X is always the same. Also, it should be noted that the UE cannot determine the active Xs (for dropping) by using the size of the bitmap for a certain SS set. In other words, to determine the active Xs (for dropping) the UE needs to look at the MOs from all configured SSs to evaluate which reported (Xs,Ys) combination should be the “active” one. With this understanding, we can support Proposal A1-3.1a. We are also fine with Proposal A1-3.1b with an FFS on how to determine Ys, as mentioned by MediaTek2. |
| Intel | With the condition ‘For the purpose of determining MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring’ in proposal A1-3.1a/b, the bullet on Ys determination is not necessary in the current proposal. However, it doesn’t mean Ys is not necessary for other functions. Our understanding is that determination of Ys can be discussed separately. We share the view from other companies that it is not necessary that the size of monitoringSlotsWithinSlotGroup and “active” Xs is always the same. Qualcomm had provided good figures for the illustration. We are also fine to limit it to SCS 960kHz as proposed in A1-3.1c. |
| Futurewei | We think that the 3-2a (RRC parameter configuration) would avoid above confusions and therefore it is preferable. However, we could compromise for A1-3.1a. We think that the size of “active” Xs and the size of monitoringSlotsWithinSlotGroup may no be the same, and agree with LG comment that to evaluate “active” Xs (for dropping) one may need to consider all the reported (Xs, Ys) combinations and at the MOs from all configured SSs. |

#### Second round discussion summary

A majority of companies (still) prefers to define a rule without RRC configuration. Proposal A1-3.1c seems to be acceptable by most companies.

#### Third round discussion

Proposal A1-3.1c

For μ=6, if more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations comply with all configured search space sets, then the UE assumes MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values.

- Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration

FL Note (not part of the proposal): I additionally suggest to modify the proposal in A2-1 for monitoringSlotsWithinSlotGroup to be simply 4 or 8 bits wide, without direct reference to Xs to avoid further confusion (suggested by several companies there). However as long as we don't have an agreement for A2-1, the Note above makes sense to keep.

Please comment only if you have a strong concern.

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We support Proposal A1-3.1c. |
| MediaTek | We can accept the main bullet of the proposal in general but the proposal is not complete. In particular, we have concerns on not determining Ys value from spec point of view. For example, (4,1) and (4,2) have the same BD/CCE limit and we still need to decide MPDCCHmax,4,1,μ or MPDCCHmax,4,2,μ even though they are the same. Furthermore, from UE perspective, Ys>1 and Ys=1 will be handled differently as mentioned by other companies, e.g., checking overbook, power saving, which is why we spent so much time on detabing which (Xs,Ys) is mandatory and which one is optional. Therefore, we still prefer to add FFS on how to determine Ys and we don’t see what will be the concern to have a decision rule on Ys? Regarding the Note on monitoringSlotsWithinSlotGroup, in our view, the definition of monitoringSlotsWithinSlotGroup is still open and we still need to check whether Xs and monitoringSlotsWithinSlotGroup are related or not after the definition is finalized. Therefore, the note is not precise and we suggest to remove it. Once monitoringSlotsWithinSlotGroup is finalized, the note is not needed anyway. |
| ZTE, Sanechips | We are ok with this proposal. |
| Futurewei | We are OK with the proposal A1-3.1c |
| LG Electronics | We still prefer A1-3.1a, but as a compromise, we can accept Proposal A1-3.1c on condition to add FFS on how to determine Ys, commented by MediaTek. |
| Samsung | We are ok with the proposal A1-3.1c. |
| Ericsson | We support Proposal A1-3.1c |
| Intel | We are not OK with Proposal A1-3.1c. The main bullet of the propsoal describes how to determine the maximum BD/CCE but does not talk about how to determine Xs. Therefore, the previous main bullet from A1-3.1a/b with are preferred. With the limitation ‘For the purpose of determining MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring’, it would be fine to not mention Ys in the proposal, with the understanding that determination of Ys can be discussed for other purpose. We provide an updated proposal Updated Proposal: For the purpose of determining MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring for μ=6, if the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations, choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring. - Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration |
| vivo | Agree with MTK and LGE that Ys is better to be determined due to BD/CCE budget checking, power saving and etc. We can accept Proposal A1-3.1c if ‘FFS on how to determine Ys’ is added. |
| Moderator | Seeing the recent discussion, I think the note doesn't really match Proposal A1-3.1b/-3.1c. First of all, in those versions the main bullet doesn't cover any determination of (Xs, Ys), so the note doesn't make much sense therewith; and some companies anyway feel that it is premature since we don't have a stable view on monitoringSlotsWithinSlotGroup. My suggestion is to change "… combination is independent …" to "… combination may be independent …". Consequently, I provide Proposal A1-3.1d (revision of Proposal A1-3.1c) as well as Proposal A1-3.1e (based on Intel's suggested update). My suggestion for -3.1d is to not include an FFS about determining Ys because the first bullet doesn't say anything about determining Xs or Ys anymore, and even if it did companies are free to bring up further proposals and motivations for determining Ys without an FFS. Proposal A1-3.1d (revision of Proposal A1-3.1c) For μ=6, if more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations comply with all configured search space sets, then the UE assumes MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values. Proposal A1-3.1e (based on Intel's suggested update) For the purpose of determining MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring for μ=6, if the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations, choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring. - Note: This determination of a (Xs,Ys) combination is may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration |
| LG Electronics2 | For the updated proposals by Moderator, we support A1-3.1e. We have been discussing how to determine an active (Xs,Ys) through Issue A1-3 from the first round of discussion untile now. It seems for us that Proposal A1-3.1e or the Intel's Updated Proposal above is a better proposal consistent with the discussion so far. Regarding the Note for monitoringSlotsWithinSlotGroup, we would like to present our previous comment once more and to confirm if this is a common understanding. - The active Xs (which is determined by the rule) and L (which is the size of the bitmap) can be determined differently and it does not create any ambiguity. It is not necessary that Xs and L is always the same. Also, it should be noted that the UE cannot determine the active Xs (for dropping) by using the size of the bitmap for a certain SS set. In other words, to determine the active Xs (for dropping) the UE needs to look at the MOs from all configured SSs to evaluate which reported (Xs,Ys) combination should be the “active” one. |
| Samsung | We are ok with Proposal A1-3.1e. |
| MediaTek | We slightly prefer A1-3.1e. Although we understand the intention of not including 480kHz in the proposal, it should at least capture this aspect as conclusion or simply add For the purpose of determining MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring for μ=5 and 6 , to complete the spec. Regarding the determination of Ys, the following sentence seems to imply Ys determination is needed. choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring Therefore, we suggest the following update by mergin the wording of A1-3.1d and adding a note for the determination of (Xs,Ys) for other purpose. Proposal A1-3.1e (based on Intel's suggested update) For the purpose of determining MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring for μ=6 and 5, if the configured search space sets comply with the requirements of more than one of the reported PDCCH monitoring (Xs,Ys) capability combinations, then the UE assumes MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values. choose a (Xs,Ys) combination from the complying reported set of combinations that results in the largest number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ for PDCCH monitoring. - Note: This determination of a (Xs,Ys) combination is may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration - Note: The determination of (Xs,Ys) for other purpose is a separated discussion. |
| Apple | We support A1-3.1e and agree with LG that a common understanding should be established based on their comment. |

### Issue A1-4: [High Priority] Multi-slot monitoring for IDLE UEs or prior to dedicated configuration (monitoringCapabilityConfig)

#### First round discussion

FL Summary: Most companies support to specify that Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 if monitoringCapabilityConfig is not provided. Discussion in RAN1#107bis-e showed that several companies don't see the need to change the wording from for a serving cell to for an active DL BWP of a serving cell in that context. FL provides two TPs with and without the change to active DL BWP of a serving cell and asks companies to show their preference.

FL Note: The TPs here assume that Issue A1-3 is resolved without introduction of an RRC parameter; otherwise, the text should change e.g. to "If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6} or is not configured with a combination Xs,Ys for μ ∈ {5,6}, […]"

Text Proposal A1-4.1: Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6}, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

Text Proposal A1-4.2: Change TS 38.213 Clause 10 as follows (change to Text Proposal A1-4.1 is highlighted):

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the the active DL BWP of serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

| Company | Comment |
| --- | --- |
| Samsung | We support TP A1-4.2. One typo correction for TP A1-4.2, the inserted “the active DL BWP of” should be before “the” to avoid duplicated “the”. If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs |
| Ericsson | We support TP A1-4.1. Furthermore, we think that TP A1-4.1 works regardless of whether or not Issue A1-3 is resolved by supporting an RRC parameter for indicating an "effective" Xs for which all search space sets comply. The intent of the clause "if the UE is not provided monitoringCapabilityConfig" is to cover the initial access case. It is our understanding that for 480/960 kHz the UE will always be configured with monitoringCapabilityConfig = r17monitoringcapability once in CONNECTED mode, since we have agreed that per-slot group monitoring is mandatory and per-slot monitoring is not supported for these SCSs (see Proposal A1-4.3 below). If an RRC parameter is agreed for Issue A1-3, it would be used once the UE is in CONNECTED mode only if a UE indicates support for multiple (Xs,Ys) combinations and there is a need to indicate the value of Xs that the UE should assume. A default can be defined if this parameter is absent, but that is a separate issue that can be handled in the field description in 38.331for the new parameter (if agreed). We don't support TP A1-4.2 since we see no need to change the current spec from "for a serving cell" to "for an active DL BWP of a serving cell." In both cases the UE is monitoring on a serving cell, so the behavior is clear. |
| Qualcomm | We are fine with A1-4.2. We believe that the TP will also clarify the legacy behavior (i.e., monitoringCapabilityConfig is per BWP configuration and the value doesn’t need to be the same across all BWPs of a serving cell). |
| vivo | We support TP A1-4.2. On adding “active DL BWP”, we support to add it since the spec is not accurate in Rel-17 case. For example, assuming a case that there are two BWPs existing in one serving cell, i.e. one BWP with 120KHz SCS where monitoringCapabilityConfig is not provided and the other BWP with 480KHz SCS where monitoringCapabilityConfig is provided with r17monitoringcapability. If the active BWP is 120KHz BWP, what’s the evaluation result of “ If a UE is provided monitoringCapabilityConfig for a serving cell” from UE? In our understanding, if strictly following current text, the answer should be ‘Yes’ since monitoringCapabilityConfig is actually provided for the serving cell (i.e. in 480KHz BWP). Then UE will perform the operation in this branch, which is not correct. |
| Nokia, NSB | Text Proposal A1-4.2 can be seen as a more accurate wording |
| Intel | We are now a bit confused by the two TPs. Are they proposed that only Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 are applicable for a UE after RRC connection if r17monitoringcapability is not configured? We prefer to define default combination only for a UE before RRC connection. Specifically, Xs,Ys=(4,1) applies for SCS 480kHz. The active combination Xs,Ys after RRC connection can be determined by all configured USS/CSS sets. Above all, we don’t think different behaviors are necessary for the case r17monitoringcapability is configured or not configured, since multi-slot PDCCH monitoring is the only applicable UE behavior |
| ZTE, Sanechips | We slightly prefer Text Proposal A1-4.1. Since the parameter monitoringCapabilityConfig included in PDCCH-Config is specified for one BWP in TS 38.331, we think it will not cause any ambiguity without “ active DL BWP of a serving cell”. |
| NTT DOCOMO | For both proposal A1-4.1 and 4.2, we are not sure whether it is necessary to define a default capability for 960 kHz SCS since it is not expected to be used during initial access, but we are fine with the proposals in general. |
| LG Electronics | We support TP A1-4.1 as our first preference, but TP A1-4.2 is fine too. |
| Apple | We are fine with TP A1-4.2 |
| Transsion | We support TP A1-4.2. |

| Futurewei | We prefer TP A1-4.2, and we could live with TP A1-4.1. |
| --- | --- |
| Lenovo | TP A1-4.1 is good enough and our first preference, but we can accept TP A1-4.2. |
| Huawei, HiSilicon | We cannot support either of the TPs. We think that default monitoring behavior for μ∈5,6 only applies to an RRC_IDLE UE or a UE during initial access. For an RRC_IDLE UE or a UE during initial access, following observations can be made: - UE does not monitor PDCCH for μ=6. - For μ=5, even if UE supports both Xs,Ys=(4,1)and Xs,Ys=(4,2), UE does not monitor Group (1) SS and, therefore, the considered value for Ys is irrelevant. Further, for both Xs,Ys=(4,1)and Xs,Ys=4,2, maximum BD and CCE budgets are equal to 20 and 32, respectively. - For μ=5, UE mandatorily supports Xs,Ys=4,1. Therefore, in our view, for μ=5, RRC_IDLE UE or a UE during initial access (i.e., when monitoringCapabilityConfig is not provided) may monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to either of the combinations Xs,Ys=4,1 or Xs,Ys=4,2 as either or the combinations have exactly the same BD and CCE budget. As such, it seems to be more accurate to specify that, for μ=5, RRC_IDLE UE or a UE during initial access (i.e., when monitoringCapabilityConfig is not provided) monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to Xs=4. However, as a convention, it may be preferable to alternatively specify that, for μ=5, RRC_IDLE UE or a UE during initial access (i.e., when monitoringCapabilityConfig is not provided) monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to combinationXs,Ys=4,1 simply because combination Xs,Ys=4,1 is mandatorily supported. Further, in our view, it should be clarified for μ∈5,6 that an RRC_CONNECTED UE is expected to be provided with monitoringCapabilityConfig= r17monitoringcapability so the UE does not fall back to its default monitoring mode during RRC_CONNECTED state. As such, we suggest the following alternative TP (we are open to change “a serving cell” to “active DL BWP of a serving cell” if such a change has the majority support alothough we don’t believe that such a change is essential): *** Unchanged text is omitted *** If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability, or - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability. For μ∈0,1,2,3, if UE is not provided monitoringCapabilityConfig for a serving cell, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ∈5,6, a UE that is provided dedicated higher layer parameters, expects to be configured with monitoringCapabilityConfig = r17monitoringcapability for a serving cell. For μ=5, if a UE is not provided with monitoringCapabilityConfig for a serving cell, UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to combination Xs,Ys=4,1. *** Unchanged text is omitted ***. |

Proposal A1-4.3 (see R1-2201735):

- Inform RAN2 that the value range for the existing parameter monitoringCapabilityConfig needs to be be extended to include the new value r17monitoringcapability, and that for 480 and 960 kHz SCS, the UE expects to be configured with this value. A note can be added to the RRC parameter spreadsheet to propose that RAN2 updates the field description of the parameter as follows:

monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1). When present, the UE expects to be configured with r17monitoringcapablity for 480 and 960 kHz SCS.

Please comment whether Proposal A1-4.3 is acceptable.

| Company | Comment |
| --- | --- |
| Samsung | Ok with the change. |
| Ericsson | Support the proposal as proponent. We emphasize that this would need to be added to the RRC parameter spreadsheet, so should be discussed prior to the RRC deadline on 2/24. |
| Qualcomm | We support the proposal. |
| vivo | Just a clarification: there maybe the following two understandings from current text: Understanding 1: For BWP with 480/960KHz SCS, monitoringCapabilityConfig is mandatory present with value r17monitoringcapablity Understanding 2: For BWP with 480/960KHz SCS, monitoringCapabilityConfig is optionally present. When present, r17monitoringcapablity should be the mandotary value. Besides, Rel-17 PDCCH monitoring capability should be added into the first sentence: Configures either Rel-15 PDCCH monitoring capability, or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell. |
| Nokia, NSB | We are also fine with the proposal |
| Intel | We are fine with the proposal. In fact, it would be fine to clarify r17monitoringcapablity is always configured for SCS 480/960kHz in either physical layer or high layer specification. |
| ZTE, Sanechips | We agree with this proposal and we can further perfect the wording (marked in blue) as follows: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1). When present, the UE expects to be configured with r17monitoringcapablity for 480 and 960 kHz SCS. |
| NTT DOCOMO | We are fine with the proposal. |
| LG Electronics | Not sure if this is an urgent issue, but we are OK with the proposed change. |
| Apple | Fine with the proposal and Vivo’s edits to the first line. Can be optionally present if we define the default behavior if absent. |

| Futurewei | We are OK with the proposed changes. |
| --- | --- |
| Lenovo | Generally fine including updates by vivo/ZTE. The last sentence could be rephrased for clarity. Our complete proposal is: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1). When present, a UE configured with 480 kHz or 960 kHz SCS expects to be configured with r17monitoringcapablity. |
| Ericsson 2 | To answer vivo's question, our view is Understanding #1. Perhaps the following clarification can capture this: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1). For a BWP with SCS configured as 480 or 960 kHz, When present, the UE expects to be configured with r17monitoringcapablity for 480 and 960 kHz SCS. |
| Huawei, HiSilicon | Similar to Ericsson, and as explained in our entry to issue A1-4, in our view, it should be clarified for μ∈5,6 that an RRC_CONNECTED UE is expected to be provided with monitoringCapabilityConfig= r17monitoringcapability so the UE does not fall back to its default monitoring mode during RRC_CONNECTED state (default monitoring mode is only used during RRC_IDLE state or initial access). In principle, we prefer a combination of Lenovo and Ericsson TP: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell or Rel-17 PDCCH monitoring capability for a BWP with SCS configured as 480 or 960 kHz. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1). For a BWP with SCS configured as 480 or 960 kHz, When present, the UE expects to be configured with r17monitoringcapablity for 480 and 960 kHz SCS. |

#### First round discussion summary

A majority prefers TP A1-4.2 (with fixed typo) TP A1-4.1, where supporters of TP A1-4.1 think that the addition of "active DL BWP" by TP A1-4.2 is not necessary.

Additionally, Intel thinks that defining default combinations after RRC connection (as both TPs do) is not necessary, it is only required for the case prior to RRC connection. FL thinks that this aspect could be covered by the description of monitoringCapabilityConfig in the RRC sheet.

For the description of monitoringCapabilityConfig in the RRC sheet, responding companies are overall fine with the direction. Considering also the discussion about the TPs, FL suggests to adopt Ericsson's latest suggestion with ZTE's addition in the first line. FL also moved the reference to 38.213 to apply for all Rel-15/Rel-16/Rel-17 PDCCH monitoring

FL suggests to adopt Text Proposal A1-4.2a and Proposal A1-4.3a:

Text Proposal A1-4.2a: Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

Proposal A1-4.3a (coloured text is just for highlighting changes):

- Capture the following in the RRC parameter spreadsheet to RAN2:

monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. For a BWP with SCS configured as 480 or 960 kHz, the UE expects to be configured with r17monitoringcapablity.

#### Second round discussion

In view of the first round discussion and online discussion, please continue discussion on the following items.

TP A1-4.2a includes monitoring behaviour for µ={5,6} without monitoringCapabilityConfig, while TP A1-4.2b includes monitoring behaviour only for µ=5 without monitoringCapabilityConfig.

Text Proposal A1-4.2a: Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

Text Proposal A1-4.2b: Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ = 5, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 as in Tables 10.1-2B and 10.1-3B.

Please state your TP preference or suggestion for 38.213 and "default" behaviour.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We think proposal A1-4.2b is in the right direction but, as discussed in the first round, it needs to be clarified that the default behavior monitoring behavior is only applicable to Initial Access UE or RRC_IDLE UE. That is why the default monitoring behavior in A1-4.2b does not include the case of mu=6. In other words, it needs to be clarified that “For μ∈5,6, a UE that is provided dedicated higher layer parameters, expects to be configured with monitoringCapabilityConfig = r17monitoringcapability an active DL BWP of a serving cell”. Otherwise, proposal A1-4.2b would be incomplete because, in current proposal A1-4.2b, a RRC_CONNECTED UE may be not provided with monitoringCapabilityConfig and, as such, should follow the default behavior but such a default behavior, although applicable ot a RRC_CFONNECTED UE, is not defined for mu=6. We suggest the following modification: Text Proposal A1-4.2b (modified): Change TS 38.213 Clause 10 as follows: If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ∈5,6, a UE that is provided dedicated higher layer parameters, expects to be configured with monitoringCapabilityConfig = r17monitoringcapability an active DL BWP of a serving cell. If the UE is not provided monitoringCapabilityConfig for μ = 5, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 as in Tables 10.1-2B and 10.1-3B.. Moderator: My intention was to achieve your suggested clarification by indicating monitoringCapabilityConfig as a mandatory field in the RRC sheet (See Proposal A1-4.3b, or-4.3c even more explicitly), will that be sufficient to resolve your concern (i.e. without your modification to 38.213)? |
| ZTE, Sanechips | We prefer Text Proposal A1-4.2a, we still need to clarify Xs,Ys=(8,1) for μ = 6 if the UE is not provided monitoringCapabilityConfig. If Text Proposal A1-4.2b is supported, we need further clarify that in CONNECTED mode the UE will always be configured with monitoringCapabilityConfig = r17monitoringcapability for 480/960 kHz. Moderator: My intention was to achieve that clarification by indicating monitoringCapabilityConfig as a mandatory field in the RRC sheet (See Proposal A1-4.3b, or -4.3c even more explicitly), will that be sufficient to resolve your concern? |
| Intel | After the agreement of A1-4.3b, our understanding is that proposal A1-4.2a or b only targets the operation before RRC connection. In such case, it would be fine only set a default combination for SCS 480kHz. therefore, A1-4.2b is preferred. |
| Samsung | With moderator’s response to Huawei, we are ok with TP A1-4.2b (better to approve this with the mandatory field in RRC together to get a full solution). |
| LG Electronics | We support A1-4.2a. We share the view with ZTE that it is needed to clarify Xs,Ys=(8,1) for μ = 6 if the UE is not provided monitoringCapabilityConfig. Regarding vivo’s question in the first round discussion, we think that monitoringCapabilityConfig can be optionally present if the default behavior is defined. In addition, we are not sure why the monitoringCapabilityConfig needs to be written in 331 spec so that it is indicated as mandatory only for 480kHz and 960kHz. For SCSs other than 480/960kHz, this parameter is not specified to be indicated mandatorily by the network. Also, in our understanding, r17monitoringcapability was introduced for the convenience of writing the specification, and there should be no additional impact from this in terms of multi-slot monitoring behavior. |
| Moderator | It seems there are still some diverging views whether it is necessary to cover µ=6 in the TP or not. I am closing this discussion and continue a new second round discussion merging the TP and RRC sheet discussion in sectin 2.1.4.4; please continue the discussion there. |

Also please comment on Proposal A1-4.3a (see above) or slightly modified Proposal A1-4.3b:

Proposal A1-4.3b:

- Capture the following description in the RRC parameter spreadsheet to RAN2, and indicate monitoringCapabilityConfig as a mandatory field:

monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. For a BWP with SCS configured as 480 or 960 kHz, the UE expects to be configured with r17monitoringcapablity.

Proposal A1-4.3c (new):

- Capture the following description in the RRC parameter spreadsheet to RAN2, and indicate monitoringCapabilityConfig as a mandatory field:

monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. For a BWP with SCS configured as 480 or 960 kHz, network configures this field with r17monitoringcapablity.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Generally OK but “UE expects to be configured” is a RAN1 language and is never used in RRC parameters field description. RAN2 uses “Network configures”. We suggest the following: Proposal A1-4.3b: (modified) - Capture the following description in the RRC parameter spreadsheet to RAN2, and indicate monitoringCapabilityConfig as a mandatory field: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. For a BWP with SCS configured as 480 or 960 kHz, the UE expects to be configured network configures this field with r17monitoringcapablity. Moderator: I added your suggestion as Proposal A1-4.3c above. |
| ZTE, Sanechips | Proposal A1-4.3b is related to Text Proposal A1-4.2a and Text Proposal A1-4.2b. If Text Proposal A1-4.2a is approved, we prefer the Lenovo’s comment in the first round discussion: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. When present, a UE configured with 480 kHz or 960 kHz SCS expects to be configured with r17monitoringcapablity.For a BWP with SCS configured as 480 or 960 kHz, the UE expects to be configured with r17monitoringcapablity. If Text Proposal A1-4.2b is approved, we do think Proposal A1-4.3b or Proposal A1-4.3b modified by Huawei should be supported. |
| Intel | We are fine with A1-4.3c |
| LG Electronics | We prefer A1-4.3a or Lenovo's comments in the first round discussion, as captured by ZTE above. We cannot accept A1-4.3c since we do not understand why monitoringCapabilityConfig needs to be written in 331 spec so that it is indicated as mandatory ONLY for 480kHz and 960kHz. For SCSs other than 480/960kHz, this parameter is not specified to be indicated mandatorily by the network. We think that monitoringCapabilityConfig can be optionally present if the default behavior is defined. |
| Moderator | It seems there are still some diverging views whether it is necessary to cover µ=6 in the TP or not. I am closing this discussion and continue a new second round discussion merging the TP and RRC sheet discussion in sectin 2.1.4.4; please continue the discussion there. |

#### NEW Second round discussion

This discussion merges the TP and RRC sheet discussion from 2.1.4.3.

Proposal A1-4.4a (combining Text Proposal A1-4.2b and a mash-up of Proposal A1-4.3a/-4.3c)

- Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ = 5, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 as in Tables 10.1-2B and 10.1-3B.

- Capture the following description in the RRC parameter spreadsheet to RAN2:

  - monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. For a BWP with SCS configured as 480 or 960 kHz, the network configures this field with r17monitoringcapablity.

FL Note (not part of the proposal): Since the RRC description indicates that "the network configures this field with r17monitoringcapablity" (addressing Huawei's preference), it is not required to designate monitoringCapabilityConfig as a mandatory field (addressing LG's preference).

Proposal A1-4.4b (combining Text Proposal A1-4.2a and revised mash-up of Proposal A1-4.3a/-4.3c)

- Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

- Capture the following description in the RRC parameter spreadsheet to RAN2:

  - monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. When present, For a BWP with SCS configured as 480 or 960 kHz, the network configures this field with r17monitoringcapablity for a BWP with SCS configured as 480 or 960 kHz SCS.

| Company | Comment |
| --- | --- |
| Qualcomm | For the TP, we support A1-4.2a. We share the same view as LGE’s second round comment. For the RRC sheet description, we are fine with the proposal. As LGE commented, we don’t see any reason to make the parameter monitoringCapabilityConfig mandatory only for 480 kHz and 960 kHz, while the parameter is already optional for other SCSs. Thus, with vivo’s suggested clarification in the first round, our understanding is ‘Understanding 2: For BWP with 480/960KHz SCS, monitoringCapabilityConfig is optionally present. When present, r17monitoringcapablity should be the mandotary value.” |
| Ericsson | We support that the TP for 38.213 and the field description for 38.331 need to be agreed together as a package. However, we do not support Proposal A1-4.4a since it is based on A1-4.2b. Instead it should be based on A1-4.2a since that TP establishes a default for both 480 and 960 kHz SCS in case monitoringCapabilityConfig is not present. Furthermore, the field description for 38.331 should contain "When present …". With these changes, it seems that the concerns from LGE, ZTE, and Lenovo would be addressed. Please see new Proposal A1-4.4b above. |
| ZTE, Sanechips | We share the same view with LGE. monitoringCapabilityConfig is an optional configuration and we do not necessarily need to make it mandatory for 480/960kHz in TS 38.331. In CONNECTED mode, It is possible that the UE is not configured with monitoringCapabilityConfig for μ = 6 . Approving Proposal A1-4.4a will not only have TP impact but also RRC parameter impact. We support the new Proposal A1-4.4b that Ericsson made, as long as we clarify Xs,Ys=(8,1) for μ = 6 if the UE is not provided monitoringCapabilityConfig, there will be no additional impacts on the RRC parameter. |
| LG Electronics | We are OK with Proposal A1-4.4b proposed by Ericsson above. |
| Xiaomi | Generally fine with the Proposal. |
| vivo | We support Proposal A1-4.4b if it is the following understanding. Understanding 2: For BWP with 480/960KHz SCS, monitoringCapabilityConfig is optionally present. When present, r17monitoringcapablity should be the mandotary value. |
| NTT DOCOMO | We support Proposal A1-4.4b by Ericsson, if vivo’s suggested clarification “Understanding 2: For BWP with 480/960KHz SCS, monitoringCapabilityConfig is optionally present. When present, r17monitoringcapablity should be the mandotary value.” is the common understanding. |
| Apple | We are fine the Proposal A1-4.4b with the condition highlighted by Vivo. |
| Intel | With Proposal A1-4.4b, does it mean gNB may not configure monitoringCapabilityConfig for a BWP with 480/960KHz SCS after RRC connection? Then, for CA operation, the specification needs to define behavior for both cases that monitoringCapabilityConfig is configured or not configured. If the understanding is correct, it seems complicate the specification. In this sense, Proposal A1-4.4a is preferred since it limitation no configuraton of monitoringCapabilityConfig to before RRC connection only. It is just single cell operation before RRC connection, which avoids additional specification impact. |
| Samsung | We agree to consider these two proposals as a package. We are ok with A1-4.4b with the understanding same as vivo. |
| Futurewei | We are OK with the Proposal A1-4.4b with vivo clarification. |
| Huawei, HiSilicon | We cannot agree with proposal Proposal A1-4.4b/A1-4.4c as it is. Proposal A1-4.4c extends the default monitoring behavior which concerns the initial access UE and RRC_IDLE to mu=6. It is understandable for us if companies want to extend the default behavior to the RRC_CONNECTED case and not to support monitoringCapabilityConfig as a mandatory field, however this should not come at the cost of having a RRC_IDLE UE or initial access UE to monitor PDCCH according to Xs,Ys=(8,1) for mu=6. We cannot agree with any text that opens the door for an Idle UE or initial access UE to monitor PDCCH for mu=6. As a compromise, we can suggest the following: Proposal A1-4.4c (modified) - Change TS 38.213 Clause 10 as follows: If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ=5 ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B. If the UE is not provided monitoringCapabilityConfig for μ=6 ∈ {5,6} and UE that is provided dedicated higher layer parameters, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.. - Capture the following description in the RRC parameter spreadsheet to RAN2, with monitoringCapabilityConfig being an optional field: - monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. When present, the network configures this field with r17monitoringcapablity for a BWP with SCS configured as 480 or 960 kHz SCS. |

#### Second round discussion summary

It seems proposal A1-4.4b with additional clarification is supported by many companies. The suggested clarification by vivo has been implemented in proposal A1-4.4c below, cleaning up change marks as well.

Proposal A1-4.4c

- Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

- Capture the following description in the RRC parameter spreadsheet to RAN2, with monitoringCapabilityConfig being an optional field:

  - monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. When present, the network configures this field with r17monitoringcapablity for a BWP with SCS configured as 480 or 960 kHz SCS.

#### Third round discussion

Huawei has voiced concerns on supporting µ=6 PDCCH monitoring for IDLE or initial access UEs, suggesting Proposal A1-4.4d as an alternative. Please comment whether that proposal can be agreeable.

Proposal A1-4.4d

- Change TS 38.213 Clause 10 as follows:

If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ ∈ {0,1,2,3}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ=5 ∈ {5,6}, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B. If the UE is not provided monitoringCapabilityConfig for μ=6 ∈ {5,6} and UE that is provided dedicated higher layer parameters, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B.

- Capture the following description in the RRC parameter spreadsheet to RAN2, with monitoringCapabilityConfig being an optional field:

  - monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability. When present, the network configures this field with r17monitoringcapablity for a BWP with SCS configured as 480 or 960 kHz SCS.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | We support Proposal A1-4.4c but we are also fine with Proposal A1-4.4d if Huawei insists to make sure a UE will not monitor PDCCH according to the combination (Xs, Ys)=(8, 1) for 960kHz in RRC_IDLE mode or during initial access. |
| Futurewei | We sare fine with the Proposals A1-4.4c and are fine with Proposal A1-4.4d. For Proposal A1-4.4d the purpose of the dedicated higher layer parameters should be stated. |
| Intel | By proposal A1-4.4d, our understanding is extend the default behavior to RRC connected state for SCS 480kHz. Then, depending whether monitoringCapabilityConfig is configured or not, UE has two corresponding behaviors, 1) always use combination (4,1), or 2) use one from (4,1) & (4,2). We are not conviced by the different behaviors. However, we are fine to go with majority view. |
| Huawei, HiSilicon | We support the proposal as the proponent company. To Intel and Futurewei: The default monitoring behavior for mu=5 in Proposal A1-4.4c and Proposal A1-4.4d are identical. The intention of Proposal A1-4.4d is that, unlike in Proposal A1-4.4c, the default monitoring behavior for mu=6 is restricted to the case of RRC_CONNECTED UE since an initial access UE or RRC_IDLE UE is not supposed to monitor PDCCH in mu=6 numerology altogether. Note to moderator: Our apologies but we noticed that a slight editorial modification in the last paragraph of the TP improves the TP language: If the UE is not provided monitoringCapabilityConfig for μ=6 ∈ {5,6} and the UE that is provided dedicated higher layer parameters, the UE monitors PDCCH on the active DL BWP of a serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ = 5 and Xs,Ys=(8,1) for μ = 6 as in Tables 10.1-2B and 10.1-3B. |
| Samsung | We actually didn’t see a technical difference between Proposal A1-4.4c and Proposal A1-4.4d. It’s a fact that the case “UE not provided with monitoringCapabilityConfig for μ=6 and not provided with dedicated higher layer parameter” does not exist based on current specification (i.e., 960 kHz is not used for initial access), then there is no issue with the wording in Proposal A1-4.4c. The specification really looks not clean in current wording of Proposal A1-4.4d, and technically it’s the same as Proposal A1-4.4c with the understanding that “960 kHz is not used for initial access” is already captured in the specification(s). |
| Qualcomm | We are generally fine with the proposal, at least for the progress. But we are still not convinced the differentiation between μ = 5 and μ = 6. monitoringCapabilityConfig is a parameter of PDCCH-Config IE. Then, what is the situation that an RRC_IDLE UE is monitoring SS sets configured by PDCCH-Config, other than PDCCH-ConfigCommon? RRC_IDLE UEs will monitor CSSs configured by PDCCH-ConfigCommon, so, in our understanding, the TP should only be applied to RRC_CONNECTED UEs. |
| Ericsson | We support Proposal A1-4.4c We can also support Proposal A1-4.4d, but we think more accurate language is "… and the UE is provided with a dedicated configuration …" instead of "… and the UE is provided dedicated higher layer parameters …" |
| vivo | Se support Proposal A1-4.4c. Agree with Samsung that Proposal A1-4.4c and Proposal A1-4.4d are the same technically. We are fine with Proposal A1-4.4d if majority supports. |
| LG Electronics | We don't see what malfunction to expect when "the UE that is provided dedicated higher layer parameters" is not present in the TP. |
| Apple | We are fine with Proposal A1-4.4c and Proposal A1-4.4d. |

### (Closed) Issue A1-5: Other multi-slot monitoring behaviour

#### First round discussion

Apple suggests the following proposal in R1-2201765:

For the slot group size (X) it should be concluded that:

- The configurable values for multi-slot PDCCH monitoring operation should be same as the reported X value(s). The  UE is not expected to handle a scenario in which they are different, and a UE can report its monitoring capability for more than one (X,Y) combination.

- For each SCS 480 kHz and 960 kHz, the minimum configurable multi-slot PDCCH monitoring periodicity is the smallest value X that a UE supports when reporting its PDCCH monitoring capabilities for the corresponding SCS and are UE specific.

- Both statements may be either explicitly stated in the specification or as a conclusion in the Chairman’s notes.

FL thinks that these points are or will be taken into account in the discussion e.g. for search space configuration parameters, and therefore a separate discussion/agreement on the proposal appears unnecessary.

## Topic A2: Search Space Configuration/Enhancement

### Issue A2-1: [High Priority] Open issues and revisions for agreement in RAN1#107bis-e

#### First round discussion

Agreement (RAN1#107bis-e) For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is { 8, 12, …, 20476} - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - FFS: need to revise the definition of duration - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs

FL Summary: Many companies have submitted their views especially on the FFS items and the working assumption. After review of the contributions, FL suggests the following update to the RAN1#107bis-e agreement. Note that aspects related to Group (2) monitoring are covered in Issue A2-2. The following picture (based on R1-2201471) is given for visualization of the parameters and their applicability.

Proposal A2-1.1: Revise the RAN1#107bis-e agreement as follows:

For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - FFS: Applicable value if this field is absent - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - The size is Xs bits, where Xs is either 4 or 8 - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - FFS: Applicable value if this field is absent

Please comment whether Proposal A2-1.1 is acceptable, and any suggestion you have for the case that are absent (e.g. it was proposed that a UE should assume a duration of 4 if duration-r17 is absent). Note that updates to the descriptions RRC parameter lists should be discussed after having reached consensus on the above.

| Company | Comment |
| --- | --- |
| MediaTek | Thanks to FL’s proposal and DOCOMO’s nice figure for visualization! We had one clarification question: how to determine Xs? Is it based on the bit-length of monitoringSlotsWithinSlotGroup-r17? If so, does it mean Issue A1-3 is resolved on the Xs part after we agree ProposalA2-1.1? |
| Samsung | We are ok with the change in principle, and have some further comments for clarification: - We understand the intention of equation 4(Xp-1)4, and want to comment that this equation is only for simply notation in RAN1 discussion, and capturing the offset in RAN2 specification may not need such equation. - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of Xs slots. - Value of duration-r17 should be 4 if this field is absent - The definition of “duration-r17” as “This field indicates the number of consecutive slots where a SearchSpace exists” may not be accurate. For example, in the figure provided by FL, duration-r17 is 8, but only 6 slots within the 8 slots includes SearchSpace. - We prefer to keep the bitwidth as 8 from last meeting, and could further clarify only the first Xs bit is applicable from a UE point of view. The gNB may not know what’s the Xs determined at the UE side. |
| Ericsson | The proposal is heading in a reasonable direction; however, we have some comments/concerns: Comment #1: Regarding the following changes: - {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - The configured offset is restricted to be an integer multiple of Xs slots If these are agreed, then it would effectively mean that both the periodicity and offset are restricted to an integer multiple of Xs slots, even for Group (2) SSs. We can agree to that, but ONLY if it is simultaneously agreed that for Group (2), monitoringSlotsWithinSlotGroup has no restriction on the number of '1's and position of '1's in the bitmap so as to be compliant with the following agreement from last meeting: Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. Hence we can agree to the above only if the following is added: - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs and are not restricted to be consecutive Comment #2: Regarding the following changes for duration-r17: - The value range is {8, 12, …, 20476} at least for Group (1) SSs - FFS: Applicable value if this field is absent Under the condition that monitoringSlotsWithinSlotGroup has no restriction on the number of '1's and position of '1's in the bitmap (see Comment #1), then we would be okay to remove "at least for Group (1) SSs" so that periodicity, offset, and duration are all restricted to integer multiple of Xs for both Group (1) and Group (2). Our view on the FFS is that if the field is absent, then the applicable value is the same as the configured length of the bitmap monitoringSlotsWithinSlotGroup (i.e., either 4 or 8). Comment #3: Agree with Samsung that the following description of the duration-r17 parameter still needs some work. - This field indicates the number of consecutive slots where a SearchSpace exists. Instead, wouldn't it make sense to define the duration such that the "configured number of slots for duration-r17 corresponds to the number of consecutive slot groups in which monitoring is performed within a period"? Comment #4: We are suggest the following change for clarity: - The size is Xs bits, where Xs is either configurable as 4 or 8 Regarding MediaTek's question on how to determine Xs. In our view, in terms of the configuration of an individual search space, it would make sense that Xs is given by the configured length of the bitmap monitoringSymbolsWithinSlot. To answer the second part of MediaTek's question, we don't think this resolves issue A1-3. That is a separate question, since from a UE perspective, if the UE reports capability for more than one value Xs for a given SCS, then it is needed to look over all configured search spaces to determine what is the "effective Xs" to which all search spaces comply. This "effective Xs" is the one that the UE would use to determine search space dropping if overbooking is employed. This can be done either (1) by explicit configuration with a separate RRC parameter (outside of the search space configuration) – this is our preference, or (2) a rule in the spec as proposed by some other companies, but this rule needs discussion. We think (1) is simpler, since the gNB must configure search spaces in such a way to comply with the UEs indicated capability, so the gNB is in a position to indicate what "effective Xs" for the UE to use. Comment #5: Regarding the following for the monitoringSlotsWithinSlotGroup-r17 parameter: FFS: Applicable value if this field is absent This seems to imply that the field can be absent. However, we think that the higher level question is whether or not the field can be absent. In our view, it makes more sense that it is always present since this parameter is an integral part of configuring a search space. Fundamentally 4 things are required in order to locate the MOs, especially when some parameters are restricted to an integer multiple of Xs: periodicity, offset, duration, and monitoringSlotsWithinSlotGroup. |
| Qualcomm | We are fine with the proposal. In particular for the bitmap size of monitorinSlotWithinSlotGroup, we think the size should be variable, i.e., 4 or 8 bits. Otherwise, when duration-r17 is configured, there could be ambiguity in the configuration of MOs. For example, in the figure below, periodicity = 32 slots, duration = 16 slots, and monitoringSlotWithinSlotGroup = ‘10000000’. If we assume Xs = 8, the MO configuration as the upper one in the following figure is obtained. However, if we assume Xs=4 and only the first 4 bits of the bitmap are used, the MO configuration as the bottom one in the following figure is obtained. Therefore, to avoid the ambiguity, the assumed value of Xs should be implicitly indicated by the bitmap size of monitoringSlotWithinSlotGroup. Note that the implicit indication of Xs is per SS set and separated from the discussion in A1-3. For example, even if a BWP is determined to use (Xs,Ys) = (4,1) according to Proposal A1-3.1, at least one SS set in the BWP may be configured with an 8-bit bitmap, while others are with a 4-bit bitmap. |
| LG Electronics | Regarding the periodicity, we agree with FL’s revised version, i.e., removing some values not an integer multiple of Xs from the list of supported periodicity values. Regarding the offset, we prefer to keep the previous agreement that the value range for the offset O is {0 .. Xp-1} slots. Regarding the duration-r17, - The definition of “This field indicates the number of consecutive slots where a SearchSpace exists” may not be sufficient. In order to accurately indicate the monitoring slot within the period, the duration-r17 should be defined as a consecutive candidate slots in which SearchSpace can exist, and the actual monitoring slot position among these candidate slots should be indicated with a bitmap monitoringSlotsWithinSlotGroup-r17. - For the default value of duration-r17, it is questionable whether the default duration of the SS set in which duration-r17 is not configured for UE operating with 960kHz SCS should still be 4 slots. Regarding monitoringSlotsWithinSlotGroup-r17, - We prefer to keep the bitwidth as 8 from the last meeting and the leftmost 4 bits can be valid for (Xs,Ys)=(4,Ys) combinations. - We are not sure that “The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs” needs to be agreed separately. If the number of 1's in this bitmap is greater than Ys, the UE could handle it as mis-configuration. - For the default value if this field is absent, we think the value of ‘10000000’ should be enough. In addition, for the mandatorily supported (Xs,Ys) combinations with Ys=1, this field can be absent. For these combinations, the monitoring slot within a period can be determined by offset given in monitoringPeriodicityAndOffset-r17. Regarding the Qualcomm’s comment of “the implicit indication of Xs is per SS set and separated from the discussion in A1-3”, it is unclear to us that the meaning of configuration with an 8-bit bitmap for a BWP whose (Xs,Ys) is determined to use (4,1). Does the 8-bit bitmap indicate MOs over two consecutive slot-groups operating in (Xs,Ys)=(4,1)? |
| Sharp | The offset should be set as a multiple of Xs, since Ys is the number of consecutive slots in Xs,. Therefore, we don't need 4(Xp-1)4, we can simply add the extended period and consider only multiples of Xs as valid settings. |
| Huawei/HiSilicon | Comment#1: Periodicity/Offset/Duration value range We think that for both Group(1) SS and Group (2) SS, the following should be supported: - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - The configured offset is restricted to be an integer multiple of Xs slots - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots where Xp the periodicity vlue - The configured duration is restricted to be an integer multiple of Xs slots - The value range is {8, 12, …, 20476} Comment#2: Duration definition We think the definition of duration should be modified as follows as SSs within a duration are not necessarily in consecutive slots but are in consecutive slot groups: Duration-r17 is the total number of slots in consecutive groups of Xs slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the monitoringPeriodicityAndOffset-r17 Comment#3: monitorinSlotWithinSlotGroup-r17 bitmap We can agree with the following (Based on Ericsson): - For both Group(1) SS and Group (2) SS, the size is Xs bits, where Xs is either 4 or 8. - Note: Determination of X in case of multiple supported X values for multi-slot monitoring should be separately discussed - For Group(1) SS, the 1s in monitoringSlotsWithinSlotGroup-r17 are consecutive and the number of 1s are no larger than Ys - For Group (2) SS, the 1s in monitoringSlotsWithinSlotGroup-r17 may be non-consecutive and the number of 1s may be up to Xs Also, we can agree with the following added bullet: - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. |
| Xiaomi | Generally fine with the update. 1, we have similar question as MTK, Can‘monitoringSlotsWithinSlotGroup-r17’ be used to indicate Xs? 2, for Comment 1 of Ericssion, we think Group (2) SS still reuse R15/16 search space configuration, monitoringSlotsWithinSlotGroup-r17 is only applied to Group (1) CSS, so we think 1s in monitoringSlotsWithinSlotGroup-r17 should still be consecutive. 3, for different searchspace set, each with a monitoringSlotsWithinSlotGroup-r17, the combined result of monitoring slots should comply to Xs/Ys pattern. |
| vivo | We are generally OK with the direction of this proposal with the following comments: Comment #1: On the following changes: - {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - The configured offset is restricted to be an integer multiple of Xs slots It should be applied to Group (2) SS only. Comment #2: Agree with Samsung that the description of duration-r17 is not accurate. |
| Nokia, NSB | This looks like a good starting point. Comment #1: We don’t see a need to change the previous the working assumption w.r.t 8 bits (“The size is 8 bits”). - If the size is either 4 or 8 bits, it should be configurable Comment #2: We think that the following bullet is not needed: - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs Comment #3: We support Huawei’s proposal for defining Duration-r17: - Duration-r17 is the total number of slots in consecutive groups of Xs slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the monitoringPeriodicityAndOffset-r17 |
| Intel | For SCS 960kHz, monitoringSlotsWithinSlotGroup-r17 has 8 bits. the 8 bits should be interpreted as the indication of a slot pattern for either one slot group with Xs=8 or two consecutive slot groups with Xs=4. Therefore, we propose the following revisions. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - The size is Xs bits, where Xs is either 4 or 8 - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group or two consecutive slot groups - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs in a slot group - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys in a slot group at least for Group (1) SSs - FFS: Applicable value if this field is absent |
| ZTE, Sanechips | For periodicity in monitoringPeriodicityAndOffset-r17 If we can further clarify in TS 38.213 or add a note in RRC parameter list that monitoringPeriodicityAndOffset-r17 is only used for 480kHz and 960kHz, we can accept the change: - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} Moreover, The configuration of periodicity/duration is intended not only for Group (1) SSs but also for Group (2) SSs, so we suggest changing the wordings in the agreement “The configured periodicity/duration at least for Group (1) SSs is restricted to be an integer multiple of Xs slots” to “The configured periodicity/duration is restricted to be an integer multiple of Xs slots for both Group (1) SSs and Group (2) SSs”. For offset in monitoringPeriodicityAndOffset-17 We can accept the idea to make sure the value of offset is restricted to be an integer multiple of Xs slots. If the offset is not restricted to be an integer multiple of Xs slots and the value range for the offset O is {0 .. Xp-1} slots for each periodicity value Xp. The offset indicates the slot offset between PDCCH monitoring occasion and frame boundary according to the current specification in TS 38.213, the configuration of parameter monitoringSlotsWithinSlotGroup-r17 should align with the configuration of offset in monitoringSlotPeriodicityAndOffset-r17. In other words, the offset indicated in monitoringSlotPeriodicityAndOffset-r17 is equal to the slot offset between the subframe boundary and the first slot in the slot group indicated in monitoringSlotsWithinSlotGroup-r17 for a PDCCH monitoring occasion. For duration-17 We don’t support the definition for duration-r17 in Proposal A2-1.1. We propose to clarify the definition of duration-r17 as “number of slots that a SearchSpace lasts for a couple of consecutive slot groups in every occasion”. In the curren specification of TS 38.331 for duration, “ If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0”. We prefer to extend this description for duration-r17: “If duration-r17 is absent, the UE applies the value 4 slots for 480kHz and 8 slots for 960kHz, except for DCI format 2_0.” For monitoringSlotsWithinSlotGroup If the size of monitoringSlotsWithinSlotGroup is Xs slots, the value of Xs depends on the solution of issue A1-3. |
| NTT DOCOMO | We support Proposal A2-1.1 in general. In our understanding, the first slot of slots which are indicated to apply monitoringSlotWithinSlotGroup-r17 needs to align with the boundary of a slot group. To ensure the alignment, therefore, periodicity, offset and duration needs to be the integer multiple of Xs. In addition, we believe that the proposal above especially for Group (1) SS with the restriction on periodicity, offset and duration can provide the same configuration flexibility as the legacy mechanism even for the Group (2) SS since slot-level offset or duration can be configured via monitoringSlotWithinSlotGroup-r17. Thus, we don’t see the need to specify different candidate values between Group (1) and (2) SS, i.e., periodicity, offset and duration should be restricted to integer multiple of Xs for both Group (1) and (2) SS. |
| Apple | We are in general fine with the proposal. Comment 1: We are fine with the removal of the non-integer multiples of Xs from the set of values for monitoringSlotPeriodicityAndOffset Comment 2: We are fine with the parameter monitoringSlotsWithinSlotGroup-r17 being modified as “The size is Xs bits, where Xs is either 4 or 8” Comment 3: For the duration, we think it should defined the # of slots corresponding to the # of slot groups i.e. 8 vs 6 (for Group (1) SSs. The parameter monitoringSlotsWithinSlotGroup-r17 would then define the acutal slots within the slot groups that are identified by the duration parameter. |
| Transsion | We are fine with the proposal in principle. Comment#1: Regarding duration-r17, we share the same view as Huawei. |

| Futurewei | We are fine with the proposal. We support that periodicity, offset and duration should be integer multiples of Xs for both groups Group(1) SS and Group(2) SS. We agree with previous companies that the definition of “duration” needs to be corrected. We are fine with Huawei definition. |
| --- | --- |
| Lenovo | Regarding the size of monitoringSlotsWithinSlotGroup-r17 we support that it is equivalent to the slot group size (Xs). However there is no need to configure it explicitly at least for the purpose of this particular parameter. The ASN.1 encoding includes the size implicitly, so when receiving the RRC message, a UE will inherently know whether the size of monitoringSlotsWithinSlotGroup-r17 is 4 or 8 bits. Regarding duration-r17 we think Huawei's suggestion goes to the right direction, though we think the wording of "upon every period" is not very clear. Perhaps it is sufficient to say: Duration-r17 is the total number of slots in consecutive groups of Xs slots that a SearchSpace lasts in every occasion as given by monitoringPeriodicityAndOffset-r17. |
| CATT | - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots This is because Xs is already multiple 4, so this restriction is enough to include the deleted text - The value range is {8, 12, …, 20476} at least for Group (1) SSs - FFS: Applicable value if this field is absent - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs These two are problematic statement. Isn’t that Ys is defined for slot group and there is no different Ys for group(1) and group(2) ? if yes, then there is only one Ys for both ss, what is the meaning of >Ys at least for group(1) SS? |

#### First round discussion summary

Taking into account the suggestions from the first round of A2-1 as well as A2-2 (for Group (2)), FL suggests to continue discussion based on revised Proposal A2-1.1a (green colour shows changes against Proposal A2-1.1).

Proposal A2-1.1a: Revise the RAN1#107bis-e agreement as follows:

For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - Value of duration-r17 should be 4 if this field is absent - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - The size is Xs bits, where Xs is either 4 or 8 - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per slot group at least for Group (1) SSs - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs - FFS: Applicable value if this field is absent

#### Second round discussion

In view of the first round discussion and online discussion, please continue discussion on the following proposal.

Note that this second round discussion includes Group (2) aspects from A2-2, and should include any proposals for further limitations e.g. as per Interpretation #2 in Issue A2-2.

Please note the following FL comments:

- Keeping both "value range for the offset O is {0, 4, 8,  …, 4(Xp-1)4} slots" as well as "The configured offset is restricted to be an integer multiple of Xs slots", since for Xs=8 not all the identified offset values are eligible. It is understood that the range expression is for convenience in the RAN1 agreement and not required to be refelected in this way in the RRC sheet for RAN2.

- For monitoringSlotsWithinSlotGroup-r17 where Xs is either 4 or 8, it is not necessary to make this number configurable, as the length is part of the ASN.1 encoding.

- Removed some of the restrictions "at least for Group (1) SS" or clarified that a restriction applies only to Group (1) (=removing "at least") in order to harmonize Group (1) and Group (2) where it seems possible.

Proposal A2-1.1a: Revise the RAN1#107bis-e agreement as follows:

For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - Value of duration-r17 should be 4 if this field is absent - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - The size is Xs bits, where Xs is either 4 or 8 - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per slot group at least for Group (1) SSs - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs - FFS: Applicable value if this field is absent

Proposal A2-1.1b (new): Revise the RAN1#107bis-e agreement as follows:

For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - If duration-r17 is absent, the UE applies the length of the slot group size in slots, except for DCI format 2_0. - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - The size is Xs bits, where Xs is either 4 or 8 - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per slot group at least for Group (1) SSs - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs - FFS: Applicable value if this field is absent

Please comment whether this proposal is agreeable.

| Company | Comment |
| --- | --- |
| Huawei, Hisilicon | Mostly Ok but still some changes are necessary as follows: - For duration-17, two definitions are provided. We think definition in “b” and not “a” is correct. - “This field indicates the number of consecutive slots where a SearchSpace exists.” - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - “At least for Group (1) SSs” in “The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs” needs to be removed because, according to the updated the definition of duration, “duration-r17 is the total number of slots in consecutive groups of Xs slots” for both Group (1) and Group (2) SS and, moreover, value range of duration is {8, 12, …, 20476} (multiple of Xs). Therefore we suggest the following change: - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - We suggest to add the following note: - Note: Determination of (Xs,Ys) combination from the complying reported set of combinations is independent from the size of monitoringSlotsWithinSlotGroup-r17. So, put together, we suggest the following update on A2-1.1a Proposal A2-1.1a (modified): Revise the RAN1#107bis-e agreement as follows: For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - Value of duration-r17 should be 4 if this field is absent - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - The size is Xs bits, where Xs is either 4 or 8 - Note: Determination of (Xs,Ys) combination from the complying reported set of combinations is independent from the size of monitoringSlotsWithinSlotGroup-r17 - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per slot group at least for Group (1) SSs - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs - FFS: Applicable value if this field is absent. Moderator: Your modifications look fine to me, I would like to hear a few more comments before consolidating to a revised proposal. |
| ZTE, Sanechips | We are generally ok with Proposal A2-1.1a except for the default value of duration-r17. In the curren specification of TS 38.331 for duration: “ If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0”. We can extend this description from one slot to one slot group for duration-r17: “If duration-r17 is absent, the UE applies the value of Xs slots , except for DCI format 2_0. Xs is either 4 or 8”. This can also align duration-r17’s default value with the size of monitoringSlotsWithinSlotGroup-r17. Moderator: Your modification looks generally fine to me, though I think in that case it would be better to refer to the slot group size explicitly: "If duration-r17 is absent, the UE applies the length of the slot group size in slots, except for DCI format 2_0.". Will that be ok? |
| MediaTek | Thanks to Moderator’s efforts for providing consolidated proposal. We still have following comments Comment1: the following bullet is applied to both 480kHz and 960kHz? Should it be 8 for 960kHz? - Value of duration-r17 should be 4 if this field is absent Moderator: Please see my comment to ZTE with suggestion above, I think it can resolve your comment 1? Comment2: do we need the following two bullets or we only need the last bullet? - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 Moderator: As commented to Huawei above, it seems fine to remove "This field indicates the number of consecutive slots where a SearchSpace exists." comment 3: regarding the bit-width of monitoringSlotsWithinSlotGroup-r17, we still have question on the implication to Issue A1-3, where we provided our view to Ericsson’s example. As for Qualcomm’s comment, Note that the implicit indication of Xs is per SS set and separated from the discussion in A1-3. For example, even if a BWP is determined to use (Xs,Ys) = (4,1) according to Proposal A1-3.1, at least one SS set in the BWP may be configured with an 8-bit bitmap, while others are with a 4-bit bitmap. We have different views on it and we are open to discuss. To us, there are two fundamental questions Q1:whether the SS sets in a BWP can be associated with different slot-group sizes? Based on the agreement we made (shown below), our understanding is all of the SS sets in a BWP should be associated with the same slot-group definition, i.e., slot-group size Xs is fixed for all the SS sets in a BWP. Otherwise, BD/CCE budget might have definitions for different SS set associated with different slot-group size. We hope this is a common understanding. If so, then we can discuss the Q2 below Q2: whether the SS sets in a BWP can have different bit-width of monitoringSlotsWithinSlotGroup-r17? Our view is all of the SS sets in a BWP should have the same bit-width of monitoringSlotsWithinSlotGroup-r17. In Qualcomm’s example, if all the SS sets in a BWP are associated with slot-group of Xs=4 slots, allowing 8 bits in monitoringSlotsWithinSlotGroup-r17 with (10000000) for one SS set seems to be optimization for configuration flexibility, which is not essential at this stage. Also, the meaning of “monitoringSlotsWithinSlotGroup” is the indication of monitoring slots within a slot group and having 8 bit for a slot group of size 4 is a little bit confusing to us. Agreement: - Multi-slot PDCCH monitoring is based on slots within a slot group - Each slot group consists of X consecutive slots - Slot groups are consecutive and non-overlapping - The start of the first slot group in a subframe is aligned with the subframe boundary - The start of each slot group is aligned with a slot boundary - Reporting the BD/CCE budget for X=4/8 slots (for 480/960 kHz resp.) is mandatory (if UE supports the corresponding SCS), and is optional for X=[2]/4 slots (for 480/960 kHz resp.) Agreement - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - The Y consecutive slots can be located anywhere within the slot group of X slots - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0 - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots Comment 4: regarding the added bullet - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs We still prefer to remove it and apply the same restriction as Group(1) SSs on Group(2) SSs to exploit improvement on UE power consumption, which is more aligned to FG 3-1, in our view. However, we are open to discuss this aspect. |
| Intel | On monitoringSlotsWithinSlotGroup-r17 One clarification question for the SCS 960kHz. Is it allowed to configure monitoringSlotsWithinSlotGroup-r17 with two non-consecutive blocks of ‘1’s in the 8-bit bitmap? E.g. ‘1100,1100’. This effectively means Xs=4 for the active combination (Xs, Ys). An related question is whether periodicity 4 is allowed in the SS set configuration for SCS 960kHz. Note: periodicity 4 for 960kHz means the absolute periodicity of half slot length of 120kHz. We are fine to allow such flexibity. On the other hand, it is also OK if majority companies don’t want to have such configurations. Assuming two non-consecutive blocks of ‘1’s is not allowed for monitoringSlotsWithinSlotGroup-r17, it would be reasonable to not allow periodicity 4 for 960kHz. Note: Such limitation doesn’t necessarily mean Xs=8 for the active combiantion (Xs, Ys). For example, the first SS set has bitmap ‘1000,0000’ and the second SS set has bitmap ‘0000,1000’, the combined PDCCH MOs of the two SS sets make a slot group size of Xs=4 Moderator: My understanding is that if the size of monitoringSlotsWithinSlotGroup-r17 is equivalent to Xs=8, a bitmap like '11001100' is not supported for a Group (1) SS. On limitation for Group (2) SS Though there is no consecensus on the clarification of ‘any of’ in FG 3-1, it is clarified in the GTW session that the limitation on Group (2) can continue. Per moderator’s request, we provide our comments here. The BD/CCE budget for a slot group is rather small. Particularly, the number of CCE is quite limited. If there is one or more TDMed MOs for Group (2) SS in a slot or slot group, it is likely to consume all BD/CCE budget, which blocks any Group (1) SS sets in the slto group. - We prefer to clarify the multiple Group (2) SS sets configured in the same slot have to be within the same span (interpretation #2), which allows the different PDCCH to share CCE estimation. Further, in a slot configured with both Group (1) and (2) SS sets, the maximum number of non-overlap spans should be still 2 to align with the same complexity in the agreement of Group (1) SS. - Then, in slot group level, we prefer to allow the simultaneous configuration of Group (2) SS sets in all/multiple slots in the slot group. note: this doesn’t mean UE has to monitor all configured MOs of Group (2) SS sets Moderator: If I understand your point correctly, the problem occurs as a function of the SS configuration. So can gNB be aware that a specific configuration will consume the BD/CCE budget as you say, and take that into account in its choice of SS configuration? |
| Samsung | Comment 1: We agree to remove “This field indicates the number of consecutive slots where a SearchSpace exists”. Comment 2: To respond to MediaTek’s comment: Our understanding is the Xs values for different SS in a BWP can be different from the gNB’s point of view (e.g. the SS configuration), but the UE determines a single value of Xs when monitoring the PDCCH based on all the SS configurations. In this sense, it doesn’t go against the agreement. |
| Apple | Given the online discussion are we concluding that for multi-slot monitoring - all configured Group (2) SS sets are limited to be within a single span of three consecutive OFDM symbols across multiple slots within a slot grouop or - each configured Group (2) SS sets is limited to be within its own single span of three consecutive OFDM symbols across multiple slots within a slot group the impact of (b) on UE power savings (especially for Type 1 CSS without dedicated RRC configuration when used in a RAR) can be quite negative. Moderator: Unless we can agree on an additional restriction for Group (2) SSs (e.g. as suggested by Intel's comment), my understanding is that (b) is the case. Then UE power saving may still be possible if the Group (2) SS is such that (a) is fulfilled, but the implementation would need to be ready for (b). |
| Moderator | I have added Proposal A2-1.1b to reflect Huawei's (except the Note for monitoringSlotsWithinSlotGroup-r17), ZTE's and some of Mediatek's suggestions. I would like to hear more views particularly on: - MediaTek's Comment 4. - Intel's suggestion after "On limitation for Group (2) SS" I took the liberty of highlighting them above in yellow. |
| Qualcomm | Comment 1: Default value of duration-r17 As ZTE and MediaTek commented, the default value should be 4 for 480 kHz and 8 for 960 kHz. Comment 2 (also response to MediaTek): As we commented for Proposal A1-3.1a, a UE capability value Xs is unique for a BWP, but SS sets within the BWP can be configured with any bitmap size of monitoringSlotWithinSlotGroup (4 or 8) as long as it is compatible with the Xs value. This would be clear from the examples we provided in Proposal A1-3.1a. In that sense the following sentences would be confusing: - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - The size is Xs bits, where Xs is either 4 or 8 As we commented above, the UE capability Xs and the bitmap size should be distinguished. As such, the following changes are suggested: - duration-r17 is the total number of slots in consecutive groups of X slots, where X is the number of bits in monitoringSlotWithinSlotGroup-r17, that a Search Space lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 - The size is X bits, where X is either 4 or 8 Comment 3: The discussion in A2-2 has not been concluded, although majority companies support Interpretation#2. Then, the following two bullets are not consistent with Interpretation#2, and seems premature at this stage: - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs The above two bullets actually follow Interpretation#1. Thus, if these two bullets are agreed, it will close the door to any further discussion on Interpretation#2. |
| Ericsson | Support Proposal A2-1.1b if the following comments are addressed. Comment #1 As we commented earlier, we are fine to restrict periodicity, offset, and duration to all be integer multiples of Xs for both Group(1) and Group(2) as long as the following bullets remain for the monitoringSlotsWithinSlotGroup bitmap - The size is Xs bits, where Xs is either 4 or 8 - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs We do not agree that the above two bullets in green close the door on Interpreation #1 vs. #2 mentioned by Qualcomm above. These bullets are fully consistent with the agreement from last meeting that the UE is mandatorily supports monitoring of Group (2) SSs within each of the Xs slots of a slot group. Which symbols within each of those slots (Interpreation #1 vs. #2) is a separate discussion. Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. Comment #2 Regarding our earlier comment about the monitoringSlotsWithinSlotGroup bitmap being configurable as 4 or 8, what we meant is that RAN2 will most likely implement the bitmap using the CHOICE syntax in ASN.1: monitoringSlotsWithinSlotGroup-r17 CHOICE { xs4 BIT STRING (SIZE (4)), xs8 BIT STRING (SIZE (8)) } Hence, we still prefer - The size is Xs bits, where Xs the size is configured as either 4 or 8 Furthermore, we think the note suggested by Huawei in their comment 3 would be good to add to establish common understanding. - Note: Determination of (Xs,Ys) combination from the complying reported set of combinations is independent from the size of monitoringSlotsWithinSlotGroup-r17 Comment #3 We think that the default value if duration-r17 is absent should be tied to the length of the monitoringSlotsWithinSlotGroup bitmap since that is what determines the configured value of Xs. Also, it is not clear what "except for DCI format 2_0" means. Does it mean there is no default duration for DCI 2_0? - If duration-r17 is absent, the UE assumes the duration in slots is equal to the length of the bitmap monitoringSlotsWithinSlotGroup applies the length of the slot group size in slots, except for DCI format 2_0. Comment #4 For clarity, we suggest the following change to the description of duration-r17 duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space lasts in every period occasion as given by monitoringPeriodicityAndOffset-r17 |
| LG Electronics | We are generally fine with Proposal A2-1.1b, but still have some comments for duration-r17. - Value of duration-r17 should be 4 for 480 kHz or 8 for 960 kHz, if this field is absent. But, we are also OK with the FL’s revised version in Proposal A2-1.1b, i.e., “If duration-r17 is absent, the UE applies the length of the slot group size in slots, except for DCI format 2_0” - One more thing we need to add for duration-r17 is “the maximum valid duration”. We propose to add the following restriction as a sub-bullet: - The maximum valid duration is Xp-Xs (where, Xs is either 4 or 8, and Xp is given in the monitoringSlotPeriodicityAndOffset-r17). Regarding MediaTek’s comments, - For Comment 3, we believe Q1 is important and should be clarified first. Moreover, Q1 is relavant with some other issues such as (1) the last Note in Proposal A1-3.1a and (2) SSSG switching in Proposal A2-6.1. We have the same understanding with MediaTek. - For Comment 4, we also prefer to apply the common restriction of the bitmap on both Group(1) and Group(2) like all other parameters. (i.e., whether to restrict consecutive '1's, and, whether to restrict the number of ‘1’s in the bitmap) Regarding Intel’s suggestion, - Basically our understanding is Interpretation#2 which is based on the previous agreement on Case 1-2 as we excerpted in the first round discussion of the issue A2-2. Therefore, in slot group level, even if the gNB can configures Group(2) SS sets in multiple slots within a slot group, it seems reasonable to limit the number of monitoring slots monitored by the UE. |
| Xiaomi | About MTK’s comment 4 and Intel’s suggestion for “On limitation for Group (2) SS”, we understand the intention is to have some degree of restriction on G(2) SS, Previously we already had a agreement that “For Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS SS monitoring locations can be anywhere within a slot group of X slots”, the above restriction seems reverts previous agreements. But we think some restriction is also reasonable to reduce UE complexity and are more prefer to take Intel’s suggestion. |
| MediaTek | (similar comment in A1-3)Thanks to Qualcomm and Ericssson’s reply. As pointed out by Qualcomm, the confusion comes from the definition of “Xs”. Xs was used as slot-group size in previous agreements and Xs is used as the bitmap length of monitoringSlotsWithinSlotGroup-r17. However, even with this clarification, we still have similar question as LG and Intel. To avoid confusion, assuming Xs means “effective” slot-group size and X means the bitmap length of monitoringSlotsWithinSlotGroup-r17 . For example, if gNB only configure one SS set and gNB would like to configure (Xs=4,Ys=1), can gNB use X=8 and monitoringSlotsWithinSlotGroup-r17=(10001000) (this is a similar question asked by Intel) or (10000000)? If the configuration is allowed, then the definition of duration-r17 and monitoringSlotsWithinSlotGroup-r17 should be modified as Qualcomm pointed out. If the configuration is not allowed, then there is a relation between Xs and X. Either way, we should not include the following note for now. We hope this clarifies our view. - Note: This determination of a (Xs,Ys) combination is independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration Based on the above, we prefer to align Xs=X, which is simpler and can save us some efforts for further discussion. However, if Xs=X can cause any issue, we are open to discuss other option |
| NTT DOCOMO | We support this proposal in general. For duration-r17, we think the current definition is not precice since SearchSpace may not be configured to all the slots indicated by this field. Thus, we suggest following modification; - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space can exists lasts in every occasion as given by monitoringPeriodicityAndOffset-r17 For monitoringSlotsWithinSlotGroup-r17, we share the similar view with FL that it is not necessary to make bitwidth configurable. |
| Apple | Baesd on the discussions so far, we support the following - Based on interpretation #2, all the CSSs are configured in the same span of 3 symbols for the slots. - As Intel/LG has mentioned, even if the gNB configures Group (2) SSs set in multiple slots in a slot group, the # of slots the UE has to monitor is limited (to 1 ideally). This could be set up as an UE capability if there is resistance to setting it to a specific value. Alternatively, MTK’s suggestion to use the same restriction as the Group (1) SS could also work. As has been mentioned, the current agreement is such that each configured Group (2) SS sets is limited to be within its own single span of three consecutive OFDM symbols across multiple slots within a slot group and any implementation has to be dimensioned for this, negating the entire design effort for Group (1) SSs. In this case as QC has mentioned, the following bullets should be removed: - The slots indicated in the bitmap are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs |
| LG Electronics2 | We can support NTT DOCOMO’s suggestion with further modification to clarify which slot is valid monitoring slot as belows, - duration-r17 is the total number of slots in consecutive groups of Xs slots that a Search Space can exists lasts in every occasion as given by monitoringPeriodicityAndOffset-r17, where the slot whose corresponding bit in monitoringSlotsWithinSlotGroup-r17 is set to ‘1’ is a valid monitoring slot. |
| Intel | Thanks moderator for the reply on our early questions. We found MTK has the same question on monitoringSlotsWithinSlotGroup-r17. Whether it is allowed to be e.g. ‘10001000’? This will result in Xs=4 for the combination (Xs, Ys). Moderator already replied it is not valid configuration. Hope this can be the common understanding. Or, it would be help to add one example under monitoringSlotsWithinSlotGroup-r17 definition to say ‘10001000’ is not valid configuration. On the potential limitation on Group (2) SS, we found some companies prefer to allow the flexibility to configure a Group (2) SS set in one, multiple or all slots in the slot group. On the other hand, some other companies prefer to have certain limitation to ease UE PDCCH monitoring. It would helpful to make a clear agreement or conclusion. We are open to hear more views. |
| Samsung | Based on the above comments, we agree with the comment that the configuration of SS should not use the wording “Xs”, and the following change could resolve the concern (no need to mention Xs in the configuration): - The size is Xs bits, where Xs is either 4 or 8 Based on this, it’s no need to explicitly forbid any bitmap values as invalid, since the UE can determine whether it’s complied with the reported combinations and determine the active Xs. |
| Futurewei | We have the same view with Samsung that we should not mention Xs in the configuration. In general we see a lot of confusion on the Xs term usage, and maybe it should be clearly defined ( maybe Xs the “active/effective” monitoring slot group as a subset of a slot group X ??) in respect to the slot group X ,as Mediatek suggested. |

#### Second round discussion summary

At this point, there is still no consensus on potential restrictions for Group (2) SSs (e.g. alignment with Group (1) restrictions). In order to progress for the RRC reelvant aspects, FL suggests to take an intermediate agreement for refinement of the various definitions and value limits, and continue discussion based on the listed options at the end of the proposal.

Regarding a suggested change to add a description of a valid monitoring slot (NTT DOCOMO, LG) to duration-r17, FL notes that this is already captured in the description of monitoringSlotsWithinSlotGroup-r17. FL therefore prefers to keep the current level of description.

Proposal A2-1.1c: Revise the RAN1#107bis-e agreement to the following:

For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - If duration-r17 is absent, the UE assumes the duration in slots is equal to the length of the bitmap monitoringSlotsWithinSlotGroup, except for DCI format 2_0. - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots where a SearchSpace exists. - FFS: need to revise the definition of duration - duration-r17 is the total number of slots in consecutive groups of Xs slots in which a Search Space can exist in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - Two sizes are supported for this parameter: 4 bits and 8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the slot groups as determined by the monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per slot group at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - For Group (2) SS: Continue discussion based on the following options - Option 1 - The configured periodicity is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs - Option 2 - Restrictions for Group (2) SS are as for Group (1) SS - FFS: Applicable value if this field is absent

#### Third round discussion

Please continue discussion on Proposal A2-1.1d, based on the latest status in the GTW discussion. Please note that I have cleaned up most colours and revision marks for a more concise list of items. Please note that I modified the bullet about the number of 1s for Group (1) SSs since L is not necessarily corresponding to the slot group size. If we can achieve quick consensus at least on the first part (i.e. not specific to Group (2)) I may ask for intermediate approval so that corresponding changes can be reflected in the RRC sheet.

Proposal A2-1.1d:Revise the RAN1#107bis-e agreement to the following:

For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {4, 8, 16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}, at least in the case of configuring a search space for Group (1) SSs - For each periodicity value Xp - The value range for the offset O is {0, 4, 8, …, 4(Xp-1)4} slots, at least in the case of configuring a search space for Group (1) SSs - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of L slots - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - If duration-r17 is absent, the UE assumes the duration in slots is equal to L, except for DCI format 2_0 - The configured duration is restricted to be an integer multiple of L slots at least for Group (1) SSs - duration-r17 is the total number of slots in consecutive groups of L slots in which a Search Space can exist in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Two sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per group of L slots for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than Ys for Group (1) SSs - For Group (2) SS: Continue discussion based on the following options - Option 1 - The configured periodicity is restricted to be an integer multiple of L slots - The configured offset is restricted to be an integer multiple of L slots - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs - Option 2 - Restrictions for Group (2) SSs are as for Group (1) SSs

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We are fine with the proposal. Regarding the last bullet for Group (2) SS, we support Option1. For Rel-17, monitoringSlotPeriodicotyAndOffset-r17 and duration-r17 indicates the slot group(s) which monitoringSlotWithinSlotGroup-r17 is applied. In that sense, the slot group boundary and the first slot indicated by monitoringSlotPeriodicotyAndOffset-r17 and duration-r17 should be aligned. This principle is common between Group (1) and Group (2) SS. The only difference in the configurability of SS set between Group (1) and Group (2) is whether it is restricted in Ys slot(s), and this differentiation can be expressed by monitoringSlotWithinSlotGroup-r17. Thus, monitoringSlotPeriodicotyAndOffset-r17 and duration-r17 should be restricted to the integer multiple of L for both Group (1) and Group (2). |
| MediaTek | We still prefer to align L=Xs, which is more clean and easy solution. The benefit of configuration flexibility by allowing L>Xs or L<Xs is not essential, in our view. Also, the naming monitoringSlotsWithinSlotGroup-r17 implies the bitmap should be defined within a slot group not two slot groups in some cases, i.e., Xs=4 L=8. However, if we are the only company preferring L=Xs, then we can accept the current format with an FFS: whether L should match Xs (in case some issues are identified in the future) We also have some editorial suggestion to avoid the wording “slot group” - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group each of the L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per group of L slots for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than Ys for Group (1) SSs Regarding Group (2) restriction, we prefer option 2. |
| Moderator | I think MediaTek's editorial changes to avoid "slot group" make sense. |
| ZTE, Sanechips | We are generally fine with Proposal A2-1.1d with option 1 for Group (2) SS. According to the agreement made in RAN1 #107-e, Group (2) SSs monitoring locations can be anywhere within a slot group of X slots, we don’t expect other restrictions for Group (2) SSs. In our understanding, as long as we make sure for Group (2) SSs the number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L and the slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive, the configured periodicity/offset/duration can be restricted to be an integer multiple of L slots for both Group (1) SS and Group (2) SS. In that case, the values of periodity and offset will not need to be modified anymore. |
| Apple | We are fine with MTK’s update to avoid the word slot-group. We prefer option 2 on the Group (2) restriction. |
| Futurewei | We are fine in principle with the proposal A2-1.1d Option 1 for Group(2) SS. Also fine with MTK proposal to replace slot group with “group of L slots” for a better reading. Under this change it would make sense to use in the third bullet the same wording as in the first bullet i.e. “group of L slots” and therefore change “A slot in the slot group each of the L slots is configured” to “A slot in the slot group each of the L slots of the group of L slots is configured” |
| LG Electronics | We are fine with the proposal in general and we are OK with MTK’s changes. Regarding restrictions for Group (2) SSs, we prefer Option 2 in term of periodicity/offset/duration. The only difference between Option 1 and Option 2 for periodicity/offset seems whether there is a restriction on the supported list of values. In addition, for duration-r17, Option 1 allows that any duration can be configured for Group (2) SS sets but Option 2 may allows the restricted duration can be configured such as to be an integer multiple of L slots. We don’t think more flexibility needs to be supported only for Group (2) SS in a situation that the perdiocity/offset/duration of Group (1) SS is restricted to be an integer multiple of L. However, for monitoringSlotsWithinSlotGroup-r17, we prefer Option 1. In our opinion, Option 1 makes more sense since Group (2) SSs monitoring locations can be anywhere within a slot group. |
| Intel | We are with FL proposal with MTK’s revisions. On Group (2) SS, common handling of periodicity and offset applies to Option 1 and 2, is it correct understanding? Then, the only difference is whether the configured slots with a Group (2) SS set can be inconsecutive and what is the maximum number of slots per L slots that can be configured with a Group (2) SS set. We slightly prefer Option 1 for configuration flexibility. On the other hand, UE complexity should also be considered. We prefer to clarifiy whether there is any restriction for multiple Group (2) SS sets in the same slot. We prefer to limit the multiple Group (2) SS sets to configured in a single span in the slot |
| Huawei, Hisilicon | Generally OK with the direction of the proposal. We think that L and Xs cannot be different (L=Xs) at least for Group (1) SS. This is due to the fact that if L and Xs are different, none of the following bullets are correct. - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of L slots - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of L slots - The configured duration is restricted to be an integer multiple of L slots at least for Group (1) SSs In the last GTW, a consensus was forming to avoid the wording of group of Xs slots and replace it with a greop of L slots. We don’t have any problem with this approach. However, the underlying principle that L=Xs cannot be violated in the agreement. Given that, we have the following suggestions: - OK in principle with MTK suggestion. However, we prefer the following change to aim using the consistent wording of “group of L slots” everywhere. - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group each of the a group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per group of L slots for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than Ys for Group (1) SSs - It should be clarified that for mu=5 only L = 4 is applicable. Suggest the following change: - Two sizes L are supported for this parameter: L=4 bits (for mu∈{5,6}) and L=8 bits (for mu=6) - We find it quite inconsistent that Xs is removed everywhere and replaced by L except in the following bullet: “The number of 1s in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than Ys for Group (1) SSs”. As such, Xs should be changed here to L with the following additional text: - The number of 1s in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots in group of L slots should be no larger than Ys for Group (1) SS where, for mu=5, Ys∈{1,2} and, for mu=6, Ys∈{1,2} if L=4 and Ys∈{1,4} if L=8. - In our view, the following bullet should also be added to Option 1 for Group (2) SS. In our view, if periodicity and offset are multiple of L slots, so should be the duration. - The configured duration is restricted to be an integer multiple of L slots Finally, as for Option 1 or Option 2 for Group (2) SS, we support Option 1. Note that if Option 1 is supported, the main bullet regarding monitoringSlotPeriodicityAndOffset and duration should be modified and the restriction of “at least for Group (1) SS” be removed. |
| Samsung | We are in general ok with the direction of the change, and provide some further comments: - Ys in the proposal should also need to be clarified by using another notation, similar to the reason for Xs - We agree with Huawei’s other changes - We also noticed the comment on the connection between L and Xs, which should be clarified especially when the UE capability is reported. - For Option 2, we believe it’s not proper to say it’s same as Group (1), since obviously the restriction within Ys slots is not applicable. In our understanding, the controversial part is only for whether the bits are consecutive or not. We modified Option 2 according to this understanding, and please feel free to further clarify. With the modified Option (2), we can be ok with either way, and prefer Option 2 for saving UE complexity. We further update the proposal as follow (didn’t keep the change mark since it looks too unclean): For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz: {4, 8, 16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}, at least in the case of configuring a search space for Group (1) SSs - For each periodicity value Xp - The value range for the offset O is {0, 4, 8, …, 4(Xp-1)4} slots, at least in the case of configuring a search space for Group (1) SSs - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of L slots - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - If duration-r17 is absent, the UE assumes the duration in slots is equal to L, except for DCI format 2_0 - The configured duration is restricted to be an integer multiple of L slots at least for Group (1) SSs - duration-r17 is the total number of slots in consecutive groups of L slots in which a Search Space can exist in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Two sizes L are supported for this parameter: L=4 bits (for μ=5 or 6) and L=8 bits (for μ=6) - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per group of L slots for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than K for Group (1) SSs, wherein K= 1 or 2 for L=4, and K= 1 or 4 for L=8. - (L, K) is selected from the combination(s) of (Xs, Ys), if the UE reports one or multiple combination(s) of (Xs, Ys) - For Group (2) SS: Continue discussion based on the following options - Option 1 - The configured periodicity is restricted to be an integer multiple of L slots - The configured offset is restricted to be an integer multiple of L slots - The configured duration is restricted to be an integer multiple of L slots - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs - Option 2 - The configured periodicity is restricted to be an integer multiple of L slots - The configured offset is restricted to be an integer multiple of L slots - The configured duration is restricted to be an integer multiple of L slots - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs |
| Qualcomm | We support the proposal. For Group (2) SS set, we support Option 2. For Option 1, we have concerns in the following cases: - For Type0A/2 CSS, the number of transmitted SSB could be very small, e.g., only one SSB, in some scenarios, e.g., indoor deployment - Type1 CSS may be configured in every slot In the above cases, the UE is required to monitor every slot back to back within a slot group. As Qualcomm, Intel, and Apple argued, such situations are not desirable and should be avoided. However, if majority companies support Option 1, we can also consider Option 1 with the following changes: - Option 1 - The configured periodicity is restricted to be an integer multiple of L slots - The configured offset is restricted to be an integer multiple of L slots - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs - Among the configured Group (2) MOs, UE is not required to monitor more than Z Group (2) MOs within a slot group - Z < Xs, FFS detailed values, e.g., Z = 1 |
| Ericsson | Agree to the direction of Proposal A2-1.1d, but with the below changes marked in blue. We call this A2-1.1d-revised. Additionally, we make the following comments Comment #1 We support Option 1. Regarding Option 2, we are okay with aligning some restrictions between Group (1) and (2), but only those for periodicity, offset, and duration, i.e., integer multiple of L. We would object to adopting the same restrictions for monitoringSlotsWithinSlotGroup as for Group (1) since that would revert the following agreement from last meeting where the UE manditorilty supports monitoring Group (2) SSs within each of the slots of a slot group: Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. Comment #2 Assuming that for Group (2) it is agreeable to restrict the periodicity, offset, and duration are restricted to integer multiples of L, our strong view is that we would then require the below two bullets at least for Type0/0A/2 CSS configured with non-zero search space ID in order to avoid making any changes to the legacy monitoring procedures which are defined per-slot. It is important to point out that even if the UE is configured to monitor every slot, the UE actually does not monitor every slot according to the procedures specified in 38.331 (for Type 0/0A) within the SI window and 38.304 (for Type 2) within the paging frame. The UE actually monitors only in slots that correspond to its preferred SSB, and those monitoring locations are separated by at least N slots where N is the number of transmitted SSBs. Clearly in FR2-2 N would be greater than L. It is beneficial from both network and UE perspective to be able to configure Type 0/0A/2 CSS in this way to minimize the time duration for which the UE is monitoring for SI updates and paging. - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs Comment #3 Regarding the changes in blue under monitoringSlotPeriodictyandOffset-r17 including the Note we suggest to add: as we indicated before, if we eventually agree that both Group (1) and Group (2) have periodicity, offset, and duration that are integer multiples of L, then the values from monitoringPeriodictyAndOffset that are not needed can be removed, and we would be fine with that. Procedurally, we just want to make sure an agreement on Group (2) is in place before those values are removed. Comment #4 Regarding the change we suggest for DCI 2_0, the reason for this is that it was unclear what "except for DCI 2_0" means. When looking into current version of 38.331, one can see that for DCI 2_0, the UE ignores the parameter duration. Hence, we think it is better to include a similar bullet in the proposal Proposal A2-1.1d-revised For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values for SCS 480kHz and 960kHz that can be configured is: {4, 8, 16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}, at least in the case of configuring a search space for Group (1) SSs - For each periodicity value Xp - The value range for the list of offset O values that can be configured is {0, 4, 8, …, 4(Xp-1)4} slots, at least in the case of configuring a search space for Group (1) SSs - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of L slots - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of L slots - Note: Periodicty values {1,2,5,10} and offset values that are not a multiple of 4 can be removed if it is later agreed that the configured periodicity and offset for Group(2) SSs is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - If duration-r17 is absent, the UE assumes the duration in slots is equal to L, except for DCI format 2_0 - The UE ignores duration-r17 for DCI format 2_0 - The configured duration is restricted to be an integer multiple of L slots at least for Group (1) SSs - duration-r17 is the total number of slots in consecutive groups of L slots in which a Search Space can exist in every occasion period as given by monitoringPeriodicityAndOffset-r17 - For 480 and 960 kHz SCS, monitoringSymbolsWithinSlot applies to each slot in a slot group of L slots configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Two sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each group of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per group of L slots for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs L slots should be no larger than Ys for Group (1) SSs - For Group (2) SS: Continue discussion based on the following options - Option 1 - The configured periodicity is restricted to be an integer multiple of L slots - The configured offset is restricted to be an integer multiple of L slots - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs - Option 2 - Restrictions for Group (2) SSs are as for Group (1) SSs |
| Apple | We agree with Ericsson that for Type 0/0A/2 CSS, 38.331 and 38.804 work so that the UE actually monitors only in slots that correspond to its preferred SSB, and those monitoring locations are separated by at least N slots where N is the number of transmitted SSBs with N most likely greater than L. However, there is no natural restriction for Type 1 CSS without dedicated RRC configuration. One possibility for compromise that we are open to is that we have Option 1 for Type 0/0A/2 CSS while we have Option 2 for Type 1 CSS without dedicated RRC configuration. This may be a way forward as it gives the flexibility demanded by one set of proponents while limiting complexity as demanded by the other camp. Note that in Comment 2, Ericsson has mentioned that “require the below two bullets at least for Type0/0A/2 CSS configured with non-zero search space ID ..” |
| Ericsson2 | Thank-you to Apple for this suggestion. We think putting some restriction on Type-1 CSS w/o RRC is reasonable, since according to current specs, it could happen that the UE could be be configured to monitor more than once per slot group. We could live with the following revision of Option 1 since this will ensure that the UE never needs to monitor any given SS more than once per L slots (even if ther are up to L '1's in the bitmap for Type0/0A/2). - For Group (2) SS: Continue discussion based on the following options - Option 1 - The configured periodicity is restricted to be an integer multiple of L slots - The configured offset is restricted to be an integer multiple of L slots - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L for Group (2) SSs - For Type1 CSS without RRC - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L should be no larger than Ys for Group (2) SSs Regarding Samsung's comment, we still think it is important to have the flexibility for non-consecutive '1's for Type0/0A/2 CSS since a configuration of [1 0 1 0 1 0 1 0] or [1 0 0 0 1 0 0 0] or would mean that the effective periodicity is 2 or 4 slots, respectively. As we commented before, this does not mean that the UE actually monitors every 2 or 4 slots since it monitors only MOs according to the preferred SSB, but this "effective" periodicity allows the total monitoring time for SI and paging to be minimized which is beneficial from a network perspective. Regarding Qualcomm's comment that the number of SSBs can be very small, we don’t think this is a practical scenario for FR2-2, but we could further discuss whether or not there is a need to handle the rare case where N < L where N is the number of transmitted SSBs according to ssb-PositionsInBurst. |
| Intel | We prefer updated Option 1 from Qualcomm for Group (2) SS. In general, it allows the flexibility for SS set configuration, subjected to a limitation of PDCCH monitoring of all Group (2) SS sets at UE. by this way, it provides a clear guideline on UE implementation. On the other hand, in the updated Option 1 from Ericsson2, it puts a restriction per SS set, however no overall limitation on the slots of all SS sets that should be monitored by UE. At a result, UE still needs to be ready for PDCCH monitoring in all/multiple of the Xs slots for the worst case. |
| Qualcomm2 | We thank Apple and Ericsson’s suggestion, and we think it’s a good step toward the convergence. Regarding Apple’s suggestion for applying Option 2 for Type1 CSS w/o dedicated RRC, we are generally fine with Ericsson2’s suggestion. But, since there is no ‘Ys’ for Type1 CSS w/o dedicated RRC (since it’s configured by SIB1 for RRC_IDLE UEs), we suggest the following change: - For Type1 CSS without RRC - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive for Group (2) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to L should be one no larger than Ys for Group (2) SSs For other CSS types, i.e., Type0A/2 CSS with searchSpaceId ≠ 0, we think additional restriction is needed for the number of “monitored” MOs within a slot group. As we have already commented above, there could be a deployment scenario where the number of transmitted SSBs, N, is very small, e,g., N < L. Unlike Ericssion’s view that such a scenario is impractical, we don’t think it is impractical, although it would not be typical. For example, some indoor deployment where the gNB only covers a very small room, we think it is a plausible assumption that the number of transmitted SSBs can be smaller than 4 or 8. Thus, we think special handling is needed so that a UE should expect to monitor up to one Type0A/2 CSS MO per slot grouop. For that purpose, we think Proposal A3-3.1 could be considered. Proposal A3-3.1 states that a UE does not expect to process information from more than one DCI format “per slot group” with CRC scrambled with SI-RNTI, P-RNTI, RA-RNTI, or MsgB-RNTI. Although some clarification may be required, in our understanding, the wording “process information” in the proposal includes blind decoding of PDCCH candidates. As such, the proposal implies that, irrespective of the configured MOs of Type0A/2 CSS within the slot group, the UE is required to actually monitor only up to one MO. The actually monitored MO would be determined by the legacy SSB-MO association rule. For the aforementioned case that the number of transmitted SSBs, N, is smaller than the slot group size, it should be up to network to configure MOs appropriately so that the UE is not required to monitor more than one MO per slot group (e.g., by using a periodicity Xp such that N·Xp > slot group size). |
| vivo | We are fine with direction of Proposal A2-1.1d. For Group (2) SS, we support option 1 since it provides more flexibility. Agree with Ericsson that Type 0/0A/2 CSS with searchspace not equal to 0 is mapped to multiple SSBs and UE only needs to monitor one of them. So there is no need to have the same restriction as Group (1) SS. |
| LG Electronics2 | We have seen comments from two companies that L=Xs should be. We don’t think it is right direction. If L=Xs for all SS sets, should gNB configure periodicity/offset/duration for all SS sets in one BWP using a common L? As we commented in the second round discussion on A1-3, we believe that UE cannot determine the active Xs (for dropping) by using the size of the bitmap for a certain SS set. That is, to determine the active Xs (for dropping) the UE needs to look at the MOs from all configured SSs to evaluate which reported (Xs,Ys) combination should be the “active” one. We think this is the basic assumption of the discussion of A1-3 so far. In addition, we have two additional comments as follows. - Regarding Ericsson’s Comment#4 for DCI 2_0, we support Ericsson’s Comment#4 that it is better to include “The UE ignores duration-r17 for DCI format 2_0” in the proposal. Similarly, we can add “The maximum valid duration is periodicity-L (periodicity as given in the monitoringSlotPeriodicityAndOffset-r17)” to the proposal. - Regarding Qualcomm2’s comment for the number of 1s in the bitmap, does the sub-bullet mean the number of 1s is always one for Type 1 CSS without RRC? Shouldn’t it be at most one? |
| Moderator | Thanks everyone for the constructive and technical discussion. For the moment, I would like to suggest the following revised proposal for consideration. Please note my following comments on modifications. It seems that there is still no common understanding whether L=Xs in all cases. Therefore I think this needs to be left open for the time being. I removed "in a slot group" from monitoringSymbolsWithinSlot, the bullet is sufficiently clear without the notion of a slot group. Regarding the restriction of 1s for Group (1) SSs, this restriction still applies per slot group Xs as defined and reported by (Xs,Ys) capabilities. However I have tried to rephrase the sentence to not focus on the number of 1s but rather on the configured slots. For Group (2) SSs, I have picked up some of the recent discussion regarding further differentiating Type1 CSS without RRC and Type0/0A/2 CSS. As Qualcomm mentions that A3-3.1 may be necessary for these updates, and most compnaies show support for A3-3.1 (extended with other RNTIs, I incorporate it at the end of the proposal. Overall I have tried to keep colour highlighting and strikethrough to a low level for better readability. Hopefully this proposal is a compromise that is acceptable to all, even if not necessarily everyone's first preference! Proposal A2-1.1e - For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of periodicity values that can be configured for SCS 480kHz and 960kHz: {4, 8, 16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}, at least in the case of configuring a search space for Group (1) SSs - For each periodicity value Xp - Offset O values that can be configured: {0, 4, 8, …, 4(Xp-1)4} slots, at least in the case of configuring a search space for Group (1) SSs - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of L slots - The configured offset at least for Group (1) SSs is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} at least for Group (1) SSs - If duration-r17 is absent, the UE assumes the duration in slots is equal to L - The UE ignores duration-r17 for DCI format 2_0 - The configured duration is restricted to be an integer multiple of L slots at least for Group (1) SSs - duration-r17 is the total number of slots in groups of L consecutive slots in which a Search Space can exist in every occasion as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Two sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the sloteach group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive per group of L slots for Group (1) SSs - For Group (1) SSs, the number of 1s in monitoringSlotsWithinSlotGroup-r17 slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE - For Group (2) SS: - The configured periodicity is restricted to be an integer multiple of L slots and is restricted to the same configurable values as for Group (1) SSs - The configured offset is restricted to be an integer multiple of L slots and is restricted to the same configurable values as for Group (1) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to Xs according to at least one of the Xs,Ys supported by a UE - For Type1 CSS without RRC - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than 1 [FL NOTE: The following is taken and expanded from Proposal A3-3.1 and part of this proposal] - If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot group. |
| Samsung | We don’t quite understand the discussion for Group (2) SS. - Even with current formulation, the UE may still need to monitor all slots in a slot group for Group (2) SS if multiple CSS are configured, then what’s the benefit to introduce further limitation on Type 1 CSS without RRC? - We already agreed a BD/CCE budget on all the SS configurations in the slot group, then we believe even without any restriction on Group (2) SS, there seems not feasible/beneficial to configure SS in all the slots within a slot group. Why a gNB would configure in such a way? Based on current framework on the UE capability, we believe the configuration of monitoring all slots within a slot group can be guaranteed already and no need to add any restriction further. We prefer to align the restrictions on Type0/0A/2 and Type 1 CSS. |
| MediaTek | We agree with Samsung’s comment but with different motivation. In our view, one of the motivation on the restriction for Group(2) SS sets is for UE power saving. Therefore, we still prefer to apply the same restriciton for both Group(2) and Group (1) SS set, which has more clear benefit. As mentioned by Samsung, it might not be feasible for UE to monitoring every slot for Group(2) SS sets under the BD/CCE budget then why don’t we design a configuration which is more aligned with such BD/CCE budget? One suggestion for the following bullet. Based on the wording, it sounds like duration-r17 size is L? We think the original wording is more clear. - duration-r17 is the total number of slots in groups of L consecutive slots in which a Search Space can exist in every occasion as given by monitoringPeriodicityAndOffset-r17 |

### (Closed) Issue A2-2: [High Priority] Multi-slot monitoring for Group (2) SS

#### First round discussion

R1-2201689 proposes to clarify the following:

Before discussing details of Group (2) SS handling, it is better to align the understanding of FG 3-1 especially ‘any of’ in the following bullet. - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot. There are two interpretations. - Interpretation #1: It is limited to single span in a slot for each Group (2) SS set, however the different Group (2) SS sets can be configured in different spans in the slot. - Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot. In our view, Interpretation #2 is the correct understanding. Interpretation #1 will not bring any real complexity reduction at UE side compared to Interpretation #2. With Interpretation #1, though a single Group (2) SS set is limited to single span per slot, there can still be multiple spans in a slot due to multiple configured Group (2) SS sets. Consequently, UE must prepare for the worst case, i.e., to decode multiple spans per slot for Group (2) SS sets. Proposal 2: - Clarify that Interpretation #2 is the right understanding for FG 3-1 - Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot.

Any comments on Intel's analysis and proposal? Is it necessary to formally agree on Interpretation #2?

| Company | Comment |
| --- | --- |
| MediaTek | We agree with Intel that Interpretation#2 should be the correct one. If we go with Interpretation #1, then it basically provides no restriction on Group (2) SS set occasions, as mentioned by Intel. Also, if span is specified for each SS set in Group (2) SS sets as interpretation #1, we wonder how to define the relation between them if those spans are overlapped? At least in Rel-16, the spans are not allowed to be overlapped. |
| Samsung | We support Interpretation #2 as the right understanding for FG 3-1. We can be ok with a conclusion in the chairman notes. |
| Ericsson (corrected by FL as per Ericsson's email via RAN1 reflector) | Clearly the interpretation is #1 #2. Rel-15 allows, e.g., Type0-PDCCH to be in the middle of the slot and Type-2 to be in the beginning of the slot. There no enforcement either in the spec or in UE capabilities that all CSSs in Group (2) are aligned in the same 3 symbol span. The wording "any of" applies to each SS individually to prevent multiple MOs for the same SS within the same slot. |
| Qualcomm | In our view, Interpretation #1 is the correct interpretation of the legacy specification. However, we can discuss whether Interpretation #2 should be applied for the multi-slot PDCCH monitoring, which we believe will resolve the issues that Intel pointed out. |
| LG Electronics | We agree with Intel’s interpretation. One clarification question: Is the description for Group (2) SS in the FG3-1 corresponding to the Case 1-2 of the following agreement at RAN#91? Agreements: (RAN1#91) - For information, the following cases are clarified: - Case 1: PDCCH monitoring periodicity of 14 or more symbols - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot - Case 2: PDCCH monitoring periodicity of less than 14 symbols - Note: this includes the PDCCH monitoring of up to three OFDM symbols at the beginning of a slot |
| Sharp | Support Interpretation#2 |
| Huawei/HiSilicon | We think that Interpretation#1 was the original intention of the agreement and follows the wording of FG 3-1. To Ericsson: Maybe I misunderstood your comment but I think Interpretation#1 is closer to your explanation. Would you please clarify further? To Samsung: Actually, we think Interpretation#2 restricts all Group (2) SSs to be configured on the same shared span in the slot and is not aliged with FG3-1. Clearly, Group (2) SSs in Rel-15 were not restricted to be sharing the same span in the slot. |
| Xiaomi | Support Interpretation#2 |
| vivo | We support Interpretation #2 |
| Nokia, NSB | We think that Interpretation#1 is the correct one |
| Intel | We support Interpretation #2 |
| ZTE, Sanechips | We are OK with this proposal and support interpretation#2. |
| NTT DOCOMO | We share the same understanding as Intel that Interpretation#2 is correct. |
| Panasonic | We support Interpretation#2. |
| Apple | Our understanding is interpretation #2 or based on the agreement LG highligted from RAN1 #91, for Group (2) SSs “For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot”. This span may be anywhere within the slot but ALL the Group (2) SSs fall within this span. Having a conclusion on this in the Chairman’s notes will be good to avoid the confusion that we are having now in the future. |
| Transsion | We support Interpretation#2. |

| Futurewei | We think that Interpretation # 1 is correct. We are no aware of any restriction in the spec that requires all CSSs in Group (2) are within the same 3 symbol span. If the majority’s understanding is different we are open to change the text in that direction. In any case the text needs clarifications to avoid misinterpretations. |
| --- | --- |
| Lenovo | When making the agreement our understanding was Interpretation #2. |
| CATT | We support Interpretation#2. |

Regarding alignment of Group (2) SSs with Group (1) SSs MSM configurations, several companies have expressed a preference to apply the same periodiicty and duration restriction to both Group (1) and Group (2) SSs, while others don't see a need for imposing any restriction.

Proposal A2-2.1 (see R1-2201735):

- For Group(2) SSs, the monitoring periodicity and duration are not restricted to be integer multiples of Xs slots.

- For Group(2) SSs, the slots indicated by the bitmap monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive.

Proposal A2-2.2: The configured periodicity and duration for Group (2) SSs is restricted to be an integer multiple of Xs slots (as for Group (1) SSs).

Proposal A2-2.3 (see R1-2201765): To limit the complexity based on the Group (2) SS location across multiple slot groups, one or more of the following could be considered:

- Group (2) SSs could be placed within the same slot group

- If spread across multiple slot groups, for CSSs Type 0 (SIB1), Type 0A (SIBx) and Type 2 (Paging), the CSS periodicity for 480 kHz and 960 kHz should not be shorter than that for 120 kHz to ensure that the wake-up period is intermittent and limit the impact on the UE’s power consumption.

- Limit the number of times a Group (2) SS may be configured within a duration of M slot groups e.g. N SSs within M slot-groups where the N SSs are in consecutive slot groups

Proposal A2-2.4 (see R1-2202130):

- For Type1 CSS without dedicated RRC configuration and for Type0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) within a slot, with the monitoring occasions for any of Type1 CSS without dedicated RRC configuration, or Types0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot group of X0 slots.

  - X0 = 4 for 480 kHz SCS and X0 = 8 for 960 kHz SCS

Please state whether you support one or more of the proposals above.

| Company | Comment |
| --- | --- |
| Samsung | So far, we didn’t see an essential need to introduce limitation on Group (2) SS sets, and it may be better to clarify the issue of existing search space configuration to be applied to multi-slot PDCCH monitoring. |
| Ericsson | In our contribution, we proposed that there should be no restriction for Group (2) SS sets, i.e., periodicity, offset, and duration need not be restricted to integer multiple of Xs. However, as we pointed out in Issue A2-1, if the following bullets from the FL proposal areagreed: - {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - The value range for the offset O is {0 .. Xp-1}{0, 4, 8, …, 4(Xp-1)4} slots - The configured offset is restricted to be an integer multiple of Xs slots then it would effectively mean that both the periodicity and offset are restricted to an integer multiple of Xs slots, even for Group (2) SSs. We can be okay to that, but ONLY if it is simultaneously agreed that for Group (2), monitoringSlotsWithinSlotGroup has no restriction on the number of '1's and position of '1's in the bitmap so as to be compliant with the following agreement from last meeting: Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. Hence we can agree to impose restrictions on periodicity, offset, and duration only if the following is added: - The number of 1s in monitoringSlotsWithinSlotGroup-r17 should be no larger than Ys at least for Group (1) SSs - The number of 1s in monitoringSlotsWithinSlotGroup-r17 can be up to Xs for Group (2) SSs and are not restricted to be consecutive This would then enable compliance with the above agreement. |
| LG Electronics | For Proposal A2-2.1 and Proposal A2-2.2, we prefer the same principle is applied for both Group (1) and Group (2) SSs. Namely, we prefer that the monitoring periodicity and duration for Group (2) SS are restricted to be integer multiples of Xs, since, in the last RAN1 meeting, it was agreed that those for Group (1) SS are restricted to be integer multiples of Xs. For Proposal A2-2.3 and Proposal A2-2.4, we share the view with Samsung that it may be better to first clarify potential issues that led to these proposals. |
| Huawei/HiSilicon | We support A2-2.2: “The configured periodicity and duration for Group (2) SSs is restricted to be an integer multiple of Xs slots (as for Group (1) SSs).” As discussed in our t-doc, if periodicities of Group (2) SS set is restricted to multiple of X_s slots, then, for μ=5, it means that periodicities of A1 = {1,2,5,10} slots cannot be used and, for μ=6, it means that periodicities of A1 = {1,2,5,10} slots cannot be used if both X_s={4,8} are supported and periodicities of A2 = {1,2,4,5,10,20} slots cannot be used if only X_s=8 is supported. Further, in absolute time, A1 = {1,2,5,10} slots for μ=5 is equal to B1 = {0.25, 0.5, 1.25, 2.5} slots for μ=3, and A2 = {1,2,4,5,10,20} slots for μ=6 is equal to B2 = {0.125, 0.25, 0.5, 0.625, 1.25, 2.5} slots for μ=3 and none of values in B1 or B2 is a supported value for a search space periodicity in Rel-15/16. Therefore, limiting Group (2) SS set to multiple of X_s slots is a mild restriction while greatly simplifies SS configuration as the same set of supported periodicities (and offsets/durations) for both Group (1) SS and Group (2) SS are used. |
| Nokia, NSB | We support Proposal A2-2.2: |
| Intel | We prefer to apply the same limitation on the periodicity for Group (1) SS and Group (2) SS We prefer to align understanding on issue A2-1 first, then come to remaining proposals under A2-2 |
| ZTE, Sanechips | We don’t see there is a need to introduce limitation on Group (2) SS sets. |
| NTT DOCOMO | For Group (2) SSs, the same configuration flexibility as the current specification should be maintained, thus we don’t see the need to restrict Group (2) SS configurations so far. |
| Apple | For Proposal A2-2.3, the motivation is to limit the # of times Group (2) SSs are transmitted over multiple multi-slots to help with UE power savings. From the agreement for Group (2) SSs, a specific CSS may be placed in any slot of the slot group. This may result in multiple slot groups with both CSSs and USSs and result in multiple slot groups over which the UE is unable to sleep thereby impacting the UE power consumption. To limit this effect, one or more of the following could be considered (in order of importance): - For Type 1 CSS without RRC configuration the combination of the agreement (every slot) with the possible size of the ra-ResponseWindow of up to 160 slots based on the conclusion in RAN1 #107-e. Conclusion: For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH. In this case, there is a possibility that the UE has to search every slot (in addition to the USS). Especially in this case the Proposal A2-2.2 “The configured periodicity and duration for Group (2) SSs is restricted to be an integer multiple of Xs slots (as for Group (1) SSs)” should be considered. - If spread across multiple slot groups, for CSSs Type 0 (SIB1), Type 0A (SIBx) and Type 2 (Paging), the CSS periodicity for 480 kHz and 960 kHz should not be shorter than that for 120 kHz to ensure that the wake-up period is intermittent and limit the impact on the UE’s power consumption. - Group (2) SSs could be placed within the same multi-slot group (as shown in Option 2 in Figure 1). |

| Futurewei | Support Proposal A2-2.2.We prefer the same restrictions for offset, periodicity and duration for both Group(1) SS and Group(2) SS in terms of integer multiples of Xs. |
| --- | --- |

#### First round discussion summary

A majority of companies thinks that interpretation #2 of FG 3-1 should be the common understanding.

FL suggests to note/conclude the following understanding for multi-slot monitoring in the chairman notes.

- For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot

Interpretation #2: "monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot" means that monitoring occasions are limited to be within a single span of three consecutive OFDM symbols within a slot considering all configured Group (2) SS sets in the slot.

Online discussion hasn't shown consensus about the interpretation of FG 3-1. Discussion about limiting all configured Group (2) SS sets are limited to be within a single span of three consecutive OFDM symbols can continue in the second round of Issue A2-1.

Regarding Proposal A2-2.2/-2.3/-2.4, most companies express a preference to have some alignment between Group (1) and Group (2) SSs. FL has updated the proposal in Issue A2-1 to capture such alignment. Discussion should continue based on Proposal A2-1.1a.

### (Closed) Issue A2-3: Periodicity restrictions for group-common DCI formats

#### First round discussion

Proposal A2-3.1 (see R1-2202130):

For group common DCI formats, only the following periodicities are applicable:

|  | 120 kHz (same as FR2) | 480 kHz | 960 kHz |
| --- | --- | --- | --- |
| DCI format 2_0 | sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20 | sl4, sl8, sl16, sl20, sl32, sl40, sl64, sl80 | sl8, sl16, sl32, sl40, sl64, sl80, sl128, sl160 |
| DCI format 2_1 | sl1, sl2, sl4 | sl4, sl8, sl16 | sl8, sl16, sl32 |
| DCI format 2_4 | sl1, sl2, sl4, sl5, sl8, sl10 | sl4, sl8, sl16, sl20, sl32, sl40 | sl8, sl16, sl32, sl40, sl64, sl80 |
| Highlighted: New periodicity values to be introduced for 480/960 kHz SCSs |  |  |  |

Can we agree Proposal A2-3.1?

| Company | Comment |
| --- | --- |
| Samsung | We are ok with adding more periodicities for 480 kHz and 960 kHz. |
| Ericsson | Support the above proposal |
| Qualcomm | We support the proposal as the proposing company. |
| vivo | Support |
| Nokia, NSB | We support this proposal |
| Intel | We are fine with proposal A2-3.1 |
| ZTE, Sanechips | We are ok with this proposal. |
| NTT DOCOMO | We are fine with the restriction for periodicity of common DCI which are the same absolute time for 120 kHz SCS. |
| LG Electronics | Support Proposal A2-3.1. |
| Apple | We are fine with the proposal |
| Transsion | We are fine with the proposal. |

| Futurewei | We are OK with the proposal. |
| --- | --- |
| Huawei, HiSilicon | We support the proposal |

#### First round discussion summary

All responding companies support Proposal A2-3.1, so FL suggests to agree Proposal A2-3.1.

For group common DCI formats, only the following periodicities are applicable:

|  | 120 kHz (same as FR2) | 480 kHz | 960 kHz |
| --- | --- | --- | --- |
| DCI format 2_0 | sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20 | sl4, sl8, sl16, sl20, sl32, sl40, sl64, sl80 | sl8, sl16, sl32, sl40, sl64, sl80, sl128, sl160 |
| DCI format 2_1 | sl1, sl2, sl4 | sl4, sl8, sl16 | sl8, sl16, sl32 |
| DCI format 2_4 | sl1, sl2, sl4, sl5, sl8, sl10 | sl4, sl8, sl16, sl20, sl32, sl40 | sl8, sl16, sl32, sl40, sl64, sl80 |
| Highlighted: New periodicity values to be introduced for 480/960 kHz SCSs |  |  |  |

The following has been declared by email approval:

Agreement

For group common DCI formats, only the following periodicities are applicable:

|  | 120 kHz (same as FR2) | 480 kHz | 960 kHz |
| --- | --- | --- | --- |
| DCI format 2_0 | sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20 | sl4, sl8, sl16, sl20, sl32, sl40, sl64, sl80 | sl8, sl16, sl32, sl40, sl64, sl80, sl128, sl160 |
| DCI format 2_1 | sl1, sl2, sl4 | sl4, sl8, sl16 | sl8, sl16, sl32 |
| DCI format 2_4 | sl1, sl2, sl4, sl5, sl8, sl10 | sl4, sl8, sl16, sl20, sl32, sl40 | sl8, sl16, sl32, sl40, sl64, sl80 |
| Highlighted: New periodicity values to be introduced for 480/960 kHz SCSs |  |  |  |

### Issue A2-4: SS set group switching minimum time

#### First round discussion

Proposal A2-4.1: Confirm the WA in RAN1 107-e, that is, support only search space set group switching processing capability 1 with with the following values

| μ | Minimum PswitchPswitch value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

FL Summary: All companies discussing the minimum P_switch value support the above proposal. Therefore FL suggests to formally agree on Proposal A2-4.1

| Company | Comment |
| --- | --- |
| MediaTek | We prefer to discuss this issue after the decision on Issue A2-5 and A2-6. |
| Samsung | We support confirming the WA. |
| Ericsson | We support confirming the WA |
| Qualcomm | We support the proposal. |
| Sharp | We are ok with FL proposal. |
| vivo | Support |
| Nokia, NSB | We support the FL proposal |
| NEC | We support confirming the WA. |
| Intel | We support the FL proposal |
| ZTE, Sanechips | We agree with FL’s suggestion to confirm the WA. |
| NTT DOCOMO | We support FL suggestion to confirm the WA. |
| LG Electronics | Support Proposal A2-4.1. |
| Apple | We support the proposal |
| Transsion | Support this proposal. |

| Futurewei | We are OK with the proposal. |
| --- | --- |
| Huawei, HiSilicon | We support the proposal |

#### First round discussion summary

Most companies support the proposal, however one company expressed a preference to come back to this after progressing on Issues A2-5 and A2-6.

### (Closed) Issue A2-5: SS set group switching behaviour

#### First round discussion

Proposal A2-5.1: For multi-slot PDCCH monitoring for 480/960 kHz SCSs, the boundary of SSSG switching is always aligned with the boundary of a slot group.

FL Note: In case SSSG switching supports different slot group sizes, the following should be added:

If the SSSGs before and after switching are associated with different slot group sizes Xs, the alignment is determined by the slot group boundary of the largest Xs value among the SSSGs.

FL Summary: Based on the submitted documents there seems to be almost consensus on the above proposal. Discussion in RAN1#107bis-e showed a majority support for the proposal. Therefore FL suggests to agree on Proposal A2-5.1

| Company | Comment |
| --- | --- |
| Samsung | We support Proposal A2-5.1. |
| Ericsson | We support Proposal A2-5.1, but we don't think an additional restriction as suggested in the FL note is needed. If Xs in the new SSSG is different, it can be handled by existing dropping rules (dropping per slot group). |
| Qualcomm | We support the proposal. |
| Sharp | We support FL proposal A2-5-1. |
| vivo | We support proposal A2-5.1. The note needs further discussion. |
| Nokia, NSB | We support FL proposal A2-5-1. |
| NEC | We support Proposal A2-5.1. |
| Intel | We prefer to add the bullet under ‘FL note’ in the proposal. We can compromise to the combined proposal though we still think it is beneficial to switch to new SSSG right at the time of Pswitch,. |
| ZTE, Sanechips | We support Proposal A2-5.1. |
| NTT DOCOMO | We support Proposal A2-5.1. |
| LG Electronics | We support Proposal A2-5.1 without FL Note. In other words, we only support that “For multi-slot PDCCH monitoring for 480/960 kHz SCSs, the boundary of SSSG switching is always aligned with the boundary of a slot group.”. We think above FL note can be formulated as a separate proposal since there are relavant discussions in A2-6 and the issue on SSSGs with different Xs may need to be discussed differently depending on the results of A2-6. |
| Apple | We are support the proposal (and the addition by the FL). |
| Transsion | We support this proposal. |

| Futurewei | We support Proposal A2-5.1 |
| --- | --- |
| Lenovo | We support Proposal A2-5.1. It may be better to first agree to it in its original form (i.e. without text in the FL note), and then consider if any additional rule is needed in case of switching between different slot group sizes in a later discussion round. No RRC impact is expected for that aspect. |
| Huawei, HiSilicon | As discussed on the reflector, we support Proposla A2-5.1 without any additional note. |

Text Proposal A2-5.2 to capture Proposal A2-5.1 (see R1-2200953):

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field for a serving cell in a DCI format 2_0, as described in clause 11.1.1; - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 - for the serving cell with μ∈0,1,2,3 at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and - for the serving cell with μ∈5,6 at the first slot of a slot group that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchTimer - for the serving cell with μ∈0,1,2,3 at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and - for the serving cell with μ∈5,6 at the first slot of a slot group that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchTimer - if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 - for the serving cell with μ∈0,1,2,3 at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0, and - for the serving cell with μ∈5,6 at the beginning of the first slot of a slot group that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 If a UE is not provided SearchSpaceSwitchTrigger for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format, the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set - for the serving cell with μ∈0,1,2,3 at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format, and - for the serving cell with μ∈5,6 at the first slot of a slot group that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set - if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 - for the serving cell with μ∈0,1,2,3 at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0, and - for the serving cell with μ∈5,6 at the beginning of the first slot of a slot group that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 *** Unchanged text is omitted ***

Please comment whether Text Proposal A2-5.2 is agreeable, provided proposal A2-5.1 is agreed.

| Company | Comment |
| --- | --- |
| Samsung | We are ok with the TP in general (there seems some typo on the change mark, which can be handled if we try to agree on the TP). |
| Ericsson | Agree with the intention of the TP, but maybe the spec implementation details can be left to the spec editor? The key point to agree on is that the UE starts or stops monitoring a SS not earlier than the first slot of a slot group that occurs after the slot in which the UE determines that a SSSG switch should occur (either by detecting DCI 2_0, detecting a DCI format in Group0, or by timer expiry). |
| Qualcomm | We support the TP. |
| Nokia, NSB | Agree in principle. |
| Intel | We prefer to wait until ‘FL note’ is resolved. |
| ZTE, Sanechips | We are generally ok with this TP. |
| NTT DOCOMO | We support the TP. |
| LG Electronics | Agree with the intention of the TP, but it would be good to discuss the TP after conclusion of Proposal A2-5.1. |
| Apple | We are fine with the TP in general. |

| Futurewei | We support the TP proposal |
| --- | --- |
| Lenovo | We are open to agree on a TP after finalising the technical agreement, but we can also give it as recommendation to the spec editor if that is preferred. |
| Huawei, HiSilicon | Support the TP |

#### First round discussion summary

Most companies support Proposal A2-5.1, although it seems further discussion is necessary whether an additional sentence is necessary to cover switching between different slot group sizes.

FL suggests to agree on Proposal A2-5.1b, and continue TP A2-5.2 discussion after progress in A2-6.

Proposal A2-5.1a:

- For multi-slot PDCCH monitoring for 480/960 kHz SCSs, the boundary of SSSG switching is always aligned with the boundary of a slot group.

- Note: If switching between different slot group sizes is supported, continue discussion on the following:

  - If the SSSGs before and after switching are associated with different slot group sizes Xs, the alignment is determined by the slot group boundary of the largest Xs value among the SSSGs.

Proposal A2-5.1b:

- For multi-slot PDCCH monitoring for 480/960 kHz SCSs, the boundary of SSSG switching is always aligned with the boundary of a slot group.

The following has been captured as agreement by email:

Agreement

For multi-slot PDCCH monitoring for 480/960 kHz SCSs, the boundary of SSSG switching is always aligned with the boundary of a slot group.

### Issue A2-6: SS set group switching configuration with same or different Xs,Ys

#### First round discussion

Proposal A2-6.1: SSSG switching is supported between SSSGs that correspond to the same or different Xs,Ys PDCCH monitoring combinations.

Proposal A2-6.2: SSSG switching is restricted to SSSGs that correspond to the same Xs,Ys PDCCH monitoring combinations.

FL Summary: A majority of submitted documents supports SSSG switching with different Xs,Ys combinations. Therefore FL suggests to agree on Proposal A2-8.1.

FL Note: Please see FL Note for Issue A2-5 in case Proposal A2-6.1 is agreed.

| Company | Comment |
| --- | --- |
| MediaTek | We support Proposal A2-6.2. At least the same (Xs, Ys) before and after SSSG switching should be supported and used as baseline to discuss A2-6.2. Also, such basic SSSG switching behavior implies simple solution to at least issue A2-5 and A2-4, which is preferred in maintenance phase. |
| Samsung | We prefer Proposal A2-6.1 for better flexibility. |
| Ericsson | Support Proposal A2-6.1 for better flexibility. |
| Qualcomm | We support Proposal A2-6.1. |
| Sharp | We support Proposal A2-6-1. |
| vivo | Just a clarification: this proposal implies that whether to comply (Xs, Ys) is checked per SSSG? However, in SSSG switching case, current BD/CCE dropping rule is defined for the whole search spaces, not per SSSG. If agreeing this proposal, does it mean that dropping rule should be changed to per SSSG? |
| Nokia, NSB | We support Proposal A2-6-1. |
| NEC | We support Proposal A2-6-1. |
| Intel | We support A2-6.1 and we prefer to make a joint proposal under A2-5.1 |
| ZTE, Sanechips | We prefer Proposal A2-6.2, in our understanding, the UE expects to monitor PDCCH according to the same combination Xs,Ys in every slot group on the active DL BWP of a cell. Therefore, we can conclude that SSSG is only supported between SSSGs that have the same Xs,Ys PDCCH monitoring capability on the active DL BWP of a cell. |
| NTT DOCOMO | We support Proposal A2-6.2 to avoid back-to back monitoring issue around SSSG switching boundary. |
| Panasonic | We support Proposal A2-6-1 for better flexibility. |
| LG Electronics | We support Proposal A2-6.2. Proposal A2-6.1 appears to be a way to provide additional flexibility to change (Xs,Ys) via SSSG switching in a certain BWP. By the way, it is ambiguous to us whether (Xs,Ys) combination is defined per BWP or per SSSG. If there is only one active (Xs,Ys) for a given BWP, then Proposal A2-6.1 need not be discussed. However, if (Xs,Ys) can be defined per SSSG, then Proposal A2-6.1 needs to be further discussed. So, we think this should be discussed first. |
| Apple | We are fine with Proposal A2-6.1 |
| Transsion | We prefer Proposal A2-6.2. We share the same view as LG that the (Xs,Ys) combination is defined per BWP. Therefore, Proposal A2-6.1 is not a valid case. |

| Futurewei | We prefer Proposal A2-6.1 |
| --- | --- |
| Sony | Prefer Proposal A2-6.1 as it provides better flexibility for the network. |

#### First round discussion summary

A majority of companies supports Proposal A2-6.1. However vivo, ZTE and LG have raised points for clarification that would be good to resolve before formally agreeing.

FL suggests to continue discussion, particularly addressing points raised by vivo, ZTE and LG.

#### Second round discussion summary

Opinions on this matter are still diverging. It seems further discussion in the third round is necessary.

#### Second round discussion

Please continue the discussion, particularly addressing points raised by vivo, ZTE and LG (copied for convenience).

| Company | Comment |
| --- | --- |
| vivo (from first round) | Just a clarification: this proposal implies that whether to comply (Xs, Ys) is checked per SSSG? However, in SSSG switching case, current BD/CCE dropping rule is defined for the whole search spaces, not per SSSG. If agreeing this proposal, does it mean that dropping rule should be changed to per SSSG? |
| ZTE, Sanechips (from first round) | We prefer Proposal A2-6.2, in our understanding, the UE expects to monitor PDCCH according to the same combination Xs,Ys in every slot group on the active DL BWP of a cell. Therefore, we can conclude that SSSG is only supported between SSSGs that have the same Xs,Ys PDCCH monitoring capability on the active DL BWP of a cell. |
| LG Electronics (from first round) | We support Proposal A2-6.2. Proposal A2-6.1 appears to be a way to provide additional flexibility to change (Xs,Ys) via SSSG switching in a certain BWP. By the way, it is ambiguous to us whether (Xs,Ys) combination is defined per BWP or per SSSG. If there is only one active (Xs,Ys) for a given BWP, then Proposal A2-6.1 need not be discussed. However, if (Xs,Ys) can be defined per SSSG, then Proposal A2-6.1 needs to be further discussed. So, we think this should be discussed first. |
| Huawei, HiSilicon | We support Proposal A2-6.1 and we don’t see a strong justification for the additional restriction in Proposal A2-6.2. Ys MO slots within a distance less than Xs slots on either side of SSSG switching boundary can be avoided by gNB using a proper switching trigger time or by UE using discarding the first MO in the second SSSG after switching. |
| ZTE, Sanechips | In Rel-16, the UE expects to monitor PDCCH according to the same combination (X, Y) in every slot on the active DL BWP of a cell (please refer to TS 38.213). Similarly, we think the (Xs,Ys) combination is defined per BWP, not per SSSG. We still prefer the following proposal: Proposal A2-6.2: SSSG switching is restricted to SSSGs that correspond to the same Xs,Ys PDCCH monitoring combinations. If SSSG switching is supported between SSSGs that correspond to the same or different (Xs,Ys) PDCCH monitoring combinations, the (Xs,Ys) needed to be defined per SSSG. |
| Intel | We support A2-6.1 for better flexibility. Since BD/CCE budget (20/32) is rather small, it would be helpful if a fine control on PDCCH monitoring can be allowed. |
| Samsung | Our understanding is (Xs, Ys) combination is associated with the BWP, and determined by the UE. At one time instance, only one valid (Xs, Ys) combination is valid, but the value may change e.g. due to the change of SS configuration or SSSG switch. |
| Sharp | We agree with Huawei and Intel’s view. |
| MediaTek | We support A2-6.2, which should be agreed first as the baseline. Changing PDCCH configuration with different monitoring capabilities, e.g., different (Xs,Ys), by SSSG switching in same BWP is not supported in current spec and the consequence is UE needs to adapt the dropping rule accordingly. Such UE behavior will need to be discussed along with the associated timelines, e.g., P_switch in A2-4, which is not desirable at this stage. |
| vivo | Before agreeing A2-6.2, the following issues should be discussed first: Whether (Xs, Ys) combination is defined per BWP or per SSSG as LG indicates. Whether BD/CCE budget is defined per BWP or per SSSG? In current SSSG switching, BD/CCE budget is defined per BWP, i.e. all the search spaces in the BWP follows the BD/CCE budget. |
| NTT DOCOMO | We are fine to support Proposal A2-6.1 if majority of campanies support it. However, in our understanding, it was agreed that the location of Ys in a slot group is maintained across consecutive slot groups to avoid back-to-back monitoring issue. If Proposal A2-6.1 is supported, it means that the location of Ys in a slot group would be maintained as long as the same (Xs, Ys) is applied but can be changed by switching. According to the agreement, it is unclear whether the location of Ys is fixed within BWP or SSSG. Thus, we think the previous agreement needs to be clarified if Proposal A2-6.1 is supported. |
| Apple | Based on the discussions so far, it seems there is a coupling between A-2-6 and A-3-4 and they should be discussed together. Also, if there is concern about UE’s needing to adapt their dropping rules, A-2-6.2 could be decided as baseline with A-2-6.1 as an enhanced capability. |
| LG Electronics | We share the view with MediaTek, vivo and DOCOMO that adopting A2-6.1 may create follow-up discussions (e.g., dropping rule to avoid back-to-back moitoring issue). This is not desired at this stage. |
| Futurewei | We agree with other companies we may need additional clarifications related to the BD/CCE budget per BWP or SSSG (the (Xs,Ys) combination is defined per BWP, or per SSSG ) before taking a decision |

#### Third round discussion

Please continue the second round discussion, I have copied some of the earlier comments for convenience. In addition to the two existing proposals, let's see if there is sufficient support for Apple's suggestion to define a UE capability for SSSG switching:

Proposal A2-6.3: UE indicates a capability whether it supports SSSG switching between SSSGs that correspond to different Xs,Ys PDCCH monitoring combinations. A UE not indicating the capability is capable of switching between SSSG that correspond to the same Xs,Ys PDCCH monitoring combination.

| Company | Comment |
| --- | --- |
| vivo (from second round) | Before agreeing A2-6.2, the following issues should be discussed first: Whether (Xs, Ys) combination is defined per BWP or per SSSG as LG indicates. Whether BD/CCE budget is defined per BWP or per SSSG? In current SSSG switching, BD/CCE budget is defined per BWP, i.e. all the search spaces in the BWP follows the BD/CCE budget. |
| NTT DOCOMO (from second round) | We are fine to support Proposal A2-6.1 if majority of campanies support it. However, in our understanding, it was agreed that the location of Ys in a slot group is maintained across consecutive slot groups to avoid back-to-back monitoring issue. If Proposal A2-6.1 is supported, it means that the location of Ys in a slot group would be maintained as long as the same (Xs, Ys) is applied but can be changed by switching. According to the agreement, it is unclear whether the location of Ys is fixed within BWP or SSSG. Thus, we think the previous agreement needs to be clarified if Proposal A2-6.1 is supported. |
| Apple (from second round) | Based on the discussions so far, it seems there is a coupling between A-2-6 and A-3-4 and they should be discussed together. Also, if there is concern about UE’s needing to adapt their dropping rules, A-2-6.2 could be decided as baseline with A-2-6.1 as an enhanced capability. |
| NTT DOCOMO | We agree with LGE and vivo that it should be clarified whether the combination of (Xs, Ys) and the location of Ys in a slot group is determined per BWP or per SSSG. Also we agree with LGE that Proposal A2-6.1 can avoid such discussion and our preference is supporting Proposal A2-6.1. |
| MediaTek | We don’t see the need to introduce a capability for different (Xs,Ys) before and after switching. As LG, vivo pointed out, further discussion on dropping rule might be necessary. Also, all the SSSG switching timelines need to be re-evaluated to accommodate such PDCCH capability changing. Therefore, we still prefer to support Proposal A2-6.2 as baseline. |
| ZTE, Sanechips | We prefer Proposal A2-6.2. According to the agreement made in RAN1 #107-e, the location of the Ys consecutive slots within the slot group of Xs slots is maintained across different slot groups. If Proposal A2-6.1 or Proposal A2-6.3 is supported, we can not maintain the location of Ys when the (Xs, Ys) is defined per BWP. To NTT DOCOMO: In our understanding, LGE support Proposal A2-6.2 too, do you type the wrong proposal number by accident? |
| LG Electronics | We still support Proposal A2-6.2 but not support A2-6.1. However, if companies views still don’t come to a consensus even in the 3rd round discussion, we think Apple's suggestion would be a better solution at this stage. Moreover, as many companies have said, Proposal A-2-6.1 needs to be discussed together with A3-4.3 and A3-4.4. Since A2-6.1 is understood as an enhanced capability of A2-6.2, we suggest to divide A2-6.3 into two separate proposals as belows, and continue the discussion on A2-6.3b after A2-6.3a. If we can easily agree on A2-6.3a, we can focus more on A2-6.3b. - Proposal A2-6.3a: SSSG switching between SSSGs that correspond to the same Xs,Ys PDCCH monitoring combinations is supported for 480 kHz and 960 kHz SCS. - Proposal A2-6.3b: UE indicates a capability whether it supports SSSG switching between SSSGs that correspond to different Xs,Ys PDCCH monitoring combinations. - Note: A UE not indicating the capability is capable of switching between SSSG that correspond to the same Xs,Ys PDCCH monitoring combination. - FFS: Whether to introduce additional dropping rule in Proposal A3-4.3 and Proposal A3-4.4. @NTT DOCOMO, we may have misunderstood your comment, but it seems A2-6.2 is closer to your explanation, as pointed out by ZTE. Could you please clarify further? |
| Samsung | In original understanding, supporting SSSG switching between SSSGs that correspond to different (X_s,Y_s ) PDCCH monitoring combinations may not need further specification impact, then there seems no need to distinguish it as UE capability. If companies have concerns on the spefication impact, we should discuss the potential impact first and then decide whether a capability is needed. |
| Qualcomm | We still support Proposal A2-6.1, and we don’t think a new capability is not needed. For the legacy Rel-16 specificaiton, we have different understanding from MediaTek. In our understanding, in Rel-16 span-based PDCCH monitoring, BD/CCE dropping rule is already accounted per SSSG, although the same Rel-16 (X,Y) is assumed for two SSSGs. Thus, evaluating BD/CCE per SSSG does not introduce additional effort. |
| Ericsson | We do not agree to introduce rules about SSSG configuration, e.g., restrictions on (Xs,Ys), nor do we agree to adding refined UE capabilities on SSSG switching. As always, the gNB must ensure SSS (and SSSG) configurations that comply with the reported UE capability on (Xs,Ys), and we have already agreed on dropping rules per slot group, and we have agreed that SSSG switching occurs on a slot group boundary. Hence we prefer not to introduce further restrictions or new UE capabilities. |
| Intel | We still support Proposal A2-6.1. Additional new optional capability is not preferred. |
| vivo | It seems that we still don’t get an answer on the question: Whether (Xs, Ys) combination is defined per BWP or per SSSG? If (Xs, Ys) combination is defined per BWP, there is no need to discuss the above proposal. If (Xs, Ys) combination is defined per SSSG, the first question should be: how to perform overbooking/SS dropping for SSSG switching case? In our understanding, current Rel-16 and Rel-17 SSSG switching, overbooking/SS dropping is done per BWP. If (Xs, Ys) combination is per SSSG, what’s the BD/CCE budget value for the BWP for overbooking/SS dropping? So the condition of (Xs, Ys) per SSSG is overbooking/SS dropping per SSSG. |
| Moderator | Opinions seeme to be virtually unchanged compared to earlier discussion.Overall, there isn't strong support for adding a UE capability. FL would like to hear views if the main concern against allowing SSSG switching between different (Xs, Ys) is the back-to-back monitoring issue, how would this concern be different for SSSG switching compared to e.g. RRC reconfiguration or n0 change? |
| MediaTek2 | Regarding Moderator’s question, we have another concern on the timeline to apply the change when different (Xs,Ys) are allowed. That is, switching between different (Xs,Ys) is similar to RRC reconfiguration but the P_swtich doesn’t take this aspect into account. Therefore, we still prefer to support Proposal A2-6.2 as baseline based on the WA on P_switch. |

### (Closed) Issue A2-7: [High Priority] Maximum value for searchSpaceSwitchTimer

#### First round discussion

Proposal A2-7.1 (see R1-2202336): For operation with shared spectrum channel access, define 40/160/320 slots as the maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS, respectively.

FL Summary: Discussion in RAN1#107bis-e showed a majority support for the proposal. Therefore FL suggests to agree on Proposal A2-7.1.

| Company | Comment |
| --- | --- |
| Samsung | We prefer to scaling the maximum value based on Rel-16 NR-U values: 20/40/80 for 15/30/60 kHz. We don’t think it’s reasonable to support a smaller maximum value for 120 kHz than 60 kHz. |
| Ericsson | Agree with Samsung's view |
| InterDigital | We agree with Samsung that supporting the scaled maximum values are reasonable. |
| LG Electronics | We support the proposal as a proponent. For 15/30/60 kHz, the maximum value of searchSpaceSwitchTimer is 20/40/80, respectively. These values are corresponding to the maximum COT duration, 20 msec. However, the maximum COT duration in FR2-2 is 5 msec, and the proposed values is derived from such 5 msec. That’s the motivation of the proposal. However, we do not intend to stick strongly with these values. We are open to discuss. |
| Sharp | The value scaled with respect to Rel-16 is a reasonable for searchSpaceSwitchTimer. |
| Huawei, HiSilicon | We prefer to follow the convention in Rel-17 and just scale the searchSpaceSwitchTimer values. |
| Xiaomi | Agree with Samsung's view |
| Ericsson | Agree with Samsung's view |
| Nokia, NSB | We support the FL proposal (and agree with Huawei). |
| Intel | Agree with Samsung's view |
| ZTE, Sanechips | We agree with LG’s comments and we can’t simply scale the maximum COT duration value based on Rel-16 NR-U as Samsung claims. The maximum COT duration in FR2-2 is 5 ms which is different from 20ms for 15/30/60 kHz. |
| Apple | We agree with Samsung/ Huawei |
| Transsion | Agree with Samsung’s and Huawei’s view |

| Futurewei | We agree with Samsung and Huawei |
| --- | --- |
| CATT | We agree with Samsung/ Huawei |

#### First round discussion summary

Most companies prefer to scale the maximum value for 120/480/960 kHz, although some companies think that MCOT for FR2-2 is 5 ms whereas it is 20 ms for 15/30/60 kHz. FL notes that even though regional regulations may impose an MCOT of less than 20 ms, there is no harm in allowing larger MCOT values in 3GPP specifications. Therefore FL suggests to agree on the scaled values as in Proposal A2-7.1a. Note that 160/640/1280 slots reflects scaled 15/30/60 kHz values.

Proposal A2-7.1a: For operation with shared spectrum channel access, define 160/640/1280 slots as the maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS, respectively.

#### Second round discussion

Please comment only if you have a strong concern with Proposal A2-7.1a (see above).

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We support Proposal A2-7.1a: |
| ZTE, Sanechips | We can understand FL’suggestion, even if the MCOT for FR2-2 is different from MCOT for 15/30/60kHz, it won’t cause additional trouble by defining a larger maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS. We are open with Proposal A2-7.1a. |
| Intel | We are OK with the propsoal |
| Moderator | As no strong concerns have been raised so far, I will suggest email approval for Proposal A2-7.1a. If there are any concerns, please raise them by email. |
| Ericsson | Support Proposal A2-7.1a |
| LG Electronics | With FL’s kind explanation, we are fine with Proposal A2-7.1a. The efficient use of the Timer value can be left up to the gNB implementation. |
| CATT | We support Proposal A2-7.1a |
| vivo | We support Proposal A2-7.1a |
| Futurewei | We support Proposal A2-7.1a |

#### Second round discussion summary

The following has been captured as agreement by email:

Agreement

For operation with shared spectrum channel access, define 160/640/1280 slots as the maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS, respectively.

### Issue A2-8: [High Priority] Harmonization of SSSG switching with Rel-17 power saving (e.g. searchSpaceSwitchTimer-r17, PDCCH skipping)

#### First round discussion

Several documents show support for supporting Rel-17 PDCCH skipping feature also in FR2-2.

Proposal A2-8.1 (see R1-2200953): In unit of slots, the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz are respectively 4x and 8x of their supported values for 120 kHz.

Proposal A2-8.2 (based on R1-22001663):

- Support PDCCH skipping feature also in FR2-2

- PDCCH Skipping lasts till the next slot group boundary after the skipping duration expires

- Support following skipping durations

  - {2,3,4,8,12,16,…636,640,720,…,1200,1280, 1440, 1600, 1760,…,3040,3200} for 480kHz SCS

    - Note: This is based on {2,3,[4:4:636],[640:80:1200],[1280:160:3200]}

  - {2,4,7,8,16,24,…1280,1440,1600,2400,2560,2880,3200,…,6080,6400 } for 960kHz SCS

    - Note: This is based on {2,4,7,[8:8:1280],[1440:160:2560],[2880:320:6400]}

Please comment whether you support any of proposals A2-8.1 and A2-8.2.

| Company | Comment |
| --- | --- |
| Samsung | We support Rel-17 power saving in FR2-2, and support Proposal A2-8.1. |
| Ericsson | Suppport Proposal A2-8.1. |
| Qualcomm | We are generally fine with the idea of scaling the values for 120 kHz by factor of 4 or 8, and support Proposal A2-8.1. However, the granularity of values may need further discussion. Since Xs = 4 is optionally supported for 960 kHz SCS, if we simply scale the values for 120 kHz by 8, it will restrict the skipping duration to be unit of two slot groups. |
| LG Electronics | For the Proposal A2-8.1, we agree the principle that the supported values for 480/960 kHz are respectively 4x and 8x of their supported values for 120 kHz. We had two clarification questions for the proposal. - Does the proposal mean to define the same set of values for searchSpaceSwitchTimer-r17 for both LBT case and non-LBT case? - What is the supported values for searchSpaceSwitchTimer-r17 for 120 kHz SCS? Does the proposal mean that the value for 480 kHz or 960 kHz should be scaled regardless of that value? For the Proposal A2-8.2, we support the concept of lasting PDCCH skipping till the next slot-group boundary after the kipping duration expires. However, we had two clarification questions for the proposal. - Why do we need values that are not integer multiples of X (e.g., {2,3} for 480 kHz)? - The skipping duration for 120 kHz in PS WI was determined as {1, 2, 3, …,160, 240, 320,400, 480, 640, 800}, and this set of values consists of a total of 166 values. However, the proposed set of values in the proposal consists of a total of 182 values for 480 kHz or 183 values for 960 kHz. Is this an intention of the proposal? |
| Sharp | We support Proposal A2-8-1. |
| Huawei, HiSilicon | Suppport Proposal A2-8.1. |
| Xiaomi | We support Rel-17 power saving in FR2-2, and support Proposal A2-8.1. |
| Nokia, NSB | Agree Proposal A2-8.1 in principle. However, we think that should be discussed in Rel-17 UE power saving AI (and not here) |
| Intel | We prefer proposal A2-8.1for simplicity |
| ZTE, Sanechips | We suppport Proposal A2-8.1. |
| Panasonic | We support Proposal A2-8.1. |
| Apple | We support Proposal A2-8.1. |
| Transsion | We support Proposal A2-8.1. |

| Futurewei | Support Proposal A2-8.1 |
| --- | --- |
| CATT | Support Proposal A2-8.1. |
| Moderator (Lenovo) | Per chairman's guidance: - Consolidate the email discussion for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration under [108-e-NR-52-71GHz-02]. This will also be announced by Xiaodong over [108-e-R17-PowSav-03]. - If there is a need to have GTW discussion for this topic, we will have it in the 60 GHz GTW session. In this case, the joint session will be announced in advance and we will find a time slot that does not overlap with UE power savings. - With regards the related working assumption in the UE power savings session, it is to be confirmed in 8.7. The discussion in the 60GHz agenda for this issue will continue after the WA is confirmed. |
| Moderator | The following agreements have been made in the power saving AI. Agreement - The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation. Agreement - It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values. If there are any additional comments, please continue your input here. |
| ZTE, Sanechips | We have several concerns about the Rel-17 PDCCH SSSG switching parameter, - We share the similar view with LGE addressed above, do we support the Rel-17 PDCCH SSSG switching for both LBT case and non-LBT case? - If we can support the Rel-17 PDCCH SSSG switching for LBT case, we need to make sure the candidate SSSG timer value is longer than the application delay (Pswitch). If the application delay of timer based SSSG switching is smaller than the application delay of DCI based SSSG switching, the UE will never monitor SSSG #1 and SSSG #2 as the SSSG timer expires before the end of application delay [please refer to the Tdoc R1-2201133]. According to the WA, for 480 kHz and 960 kHz, the Pswitch is 160/14≈11.4 slots and 320/14 ≈ 22.8 slots, respectively. Therefore, the SSSG timer value should be larger than 11.4 slots and 22.8 slots for 480kHz and 960kHz, respectively. We prefer to add one note as shown below (marked in red): Proposal A2-8.1 (see R1-2200953): In unit of slots, the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz are respectively 4x and 8x of their supported values for 120 kHz. Note: If Rel-17 PDCCH SSSG switching is supported for LBT case, the candidate SSSG timer value should be longer than Pswitch. |
| Huawei, Hisilicon | We can accept the proposal in its original form and without added note. We are not justified that such a note is necessary and cannot commit to it. If ZTE feels strong about it, the Note could be an FFS. |

Proposal A2-8.3 (see R1-2200953): A UE does not expect to be configured with Rel-16 SSSG switching parameters (such as searchSpaceSwitchTimer and SearchSpaceSwitchTrigger) and Rel-17 SSSG switching parameters (such as searchSpaceSwitchTimer-r17 and searchSpaceGroupIdList-r17) per cell simultaneously.

Some documents show support for proposal A2-8.3. Please comment whether it can be agreed.

| Company | Comment |
| --- | --- |
| Samsung | We support Proposal A2-8.3. |
| Ericsson | Fine with Proposal A2-8.3 |
| Qualcomm | We support the proposal. Although both Rel-16 and Rel-17 SSSG switching rely on the notion of search space set group, the configuration of Rel-16 SSSG switching is per-cell, while the SSSG timer for Rel-17 SSSG switching is per-BWP. Thus, they are not compatible to each other, and it is preferred to disallow the joint configuration. Furthermore, SSSG switching for FR2-2 is an extension of Rel-16 SSSG switching, mainly for the unlicensed band operation and, thus, the joint configuration with Rel-17 SSSG switching in FR2-2 should be disallowed. |
| InterDigital | We support Proposal A2-8.3. |
| LG Electronics | We support Proposal A2-8.3 |
| Huawei/HiSilicon | We support Proposal A2-8.3. |
| Nokia, NSB | Agree with Proposal A2-8.3 in principle. However, we think that should be discussed in Rel-17 UE power saving AI (and not here) |
| Intel | We agree with the proposal |
| ZTE, Sanechips | We are ok with Proposal A2-8.3. |
| Panasonic | We support Proposal A2-8.3. |
| Apple | We support the proposal. |
| Transsion | We support the proposal. |

| Futurewei | Support Proposal A2-8.3 |
| --- | --- |
| Lenovo | We support Proposal A2-8.3 |
| Moderator (Lenovo) | Per chairman's guidance: - Consolidate the email discussion for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration under [108-e-NR-52-71GHz-02]. This will also be announced by Xiaodong over [108-e-R17-PowSav-03]. - If there is a need to have GTW discussion for this topic, we will have it in the 60 GHz GTW session. In this case, the joint session will be announced in advance and we will find a time slot that does not overlap with UE power savings. - With regards the related working assumption in the UE power savings session, it is to be confirmed in 8.7. The discussion in the 60GHz agenda for this issue will continue after the WA is confirmed. |
| Moderator | The following agreements have been made in the power saving AI. Agreement - The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation. Agreement - It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values. If there are any additional comments, please continue your input here. |
| ZTE, Sanechips | As shown in the 38.331 CR of power saving enhancement in R2-2201814: searchSpaceGroupIdList List of search space group IDs which the search space is associated with. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1. if searchSpaceGroupIdList (i.e. without suffix) is included. The network configures at most 3 search space groups per BWP where the group ID is either 0, 1 or 2 if searchSpaceGroupIdList-r17 is included. And if searchSpaceGroupIdList-r17 is included, searchSpaceGroupIdList (i.e. without suffix) is ignored.. We noticed that the 38.331 CR can already solve the issue we are discussing now, we still support Proposal A2-8.3 but this discussion may not needed anymore. |
| LG Electronics | We support ZTE’s clarification that this proposal doesn’t to be needed agreed explicitly since 331 CR already covers the proposal. But, we can go with the majority view. |

#### First round discussion summary

There seems to be consensus to agree on Proposal A2-8.3, although ZTE commented that in view of a 38.331 CR for power saving, the proposal may not ne necessary. Still FL, suggests to agree on Proposal A2-8.3 by email unless there are concerns.

Most companies support Proposal A2-8.1 for the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz, though ZTE prefers to add a note as reflected in Proposal A2-8.1a.

Proposal A2-8.1a:

- In unit of slots, the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz are respectively 4x and 8x of their supported values for 120 kHz.

- Note: If Rel-17 PDCCH SSSG switching is supported for LBT case, the candidate SSSG timer value should be longer than Pswitch.

FL suggests to clarify if other companies think that the proposal including the note is agreeable.

The following has been captured as agreement by email:

Agreement

In unit of slots, the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz are respectively 4x and 8x of their supported values for 120 kHz.

## Topic A3: BD Budget/Dropping

### Issue A3-1: BD/CCE budget (e.g. for Xs=4 at 960 kHz) for a single serving cell

#### First round discussion

Proposal A3-1.1: Confirm the working assumption: BD/CCE budget of 960 kHz for Xs,Ys= (4,2), (4,1) is half that of X=8 (i.e. 10/16)

Proposal A3-1.2 (see R1-2201471): Adopt BD/CCE budget of 960 kHz for Xs,Ys= (4,2), (4,1) as 10/28

Proposal A3-1.3 (see R1-2202072): For Rel-17 960kHz multi-slot PDCCH monitoring, only (X,Y)=(8,1) and (8,4) are supported in a Pcell

FL Summary: Most companies seemed to be fine with the confirming the working assumption without changing parameters or adding restrictions. One company raised a concern that the budget according to the WA is insufficient for monitoring CSS. Another company suggests that a PCell supports only (X,Y)=(8,1) and (8,4).

FL asks companies if they are supporting the increased CCE budget (Proposal A3-1.2) and/or the suggested limitation for a PCell (Proposal A3-1.3). Otherwise it is suggested to just confirm the WA (Proposal A3-1.1) as is.

| Company | Comment |
| --- | --- |
| MediaTek | We support A3-1.3 and can be acceptable to A3-1.1. However, as mentioned in our Tdoc, the PDCCH monitoring BD/CCE budget for SS#0 has been agreed and we don’t see the feasibility to support SS#0 under the BD/CCE=10/16 requirement. The solution can be either increasing the BD/CCE limit budget as proposal A3-1.2 or simply restricting the use cases of X=4 for 960kHz not in Pcell. From the UE implementation point of view, we don’t prefer to optimize BD/CCE budget for such purpose at the huge cost of UE implementation complexity. Note that BD/CCE budget is highly replated to processing timelines, which have been agreed for both mandatory and optional PDCCH monitoring capability. Therefore, increasing BD/CCE budget should not be considered at this stage. |
| Samsung | We are open to increase the BD/CCE budget for (4, 2) and (4, 1) with 960 kHz, and the exact value can be further discussed. We didn’t see a strong motivation to restrict using (4, 2) and (4, 1) with 960 kHz for PCell. |
| Ericsson | These is a problem with Proposal A3-1.1. We think that (4,2) and (4,1) with BD/CCE budget of 10/16 is flawed in that the CCE budget is not enough for Type0-PDCCH monitoring which requires 4 + 8 + 16 = 28 non-overlapping CCEs according to Table 10.1-1. Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1 CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. To solve this issue, we suppport Proposal A3-1.2. With Proposal A3-1.2, there doesn't seem to be a need for Proposal A3-1.3. |
| Qualcomm | We support Proposal A3-1.1. However, we are also open to discuss Proposal A3-1.2, if there are sufficient interests. |
| Sharp | We generally suport Proposal A3-1.1 and A3-1.2. The specific budget values are worth discussing. |
| vivo | We are OK to confirm the working assumption. |
| Nokia, NSB | We support Proposal A3-1.2. |
| Intel | We prefer to confirm the WA |
| ZTE, Sanechips | We are fine with the intention to enlarge the CCE budge of 960kHz for (Xs, Ys) = (4,2), (4,1). However, we failed to understand why the CCE budge is calculated by simply adding, i.e. 4 + 8 + 16. |
| NTT DOCOMO | We support A3-1.2 and the exact values of increased CCE budget can be further discussed. In our view, Xs=4 slots for 960 kHz SCS can be supported as optional UE capability, so we don’t see the need to make CCE budget too small. Our concern on the current WA is that the available AL for the type0-PDCCH CSS can be limited, and/or type0-PDCCH monitoring can consume all the BD/CCE budgets in a slot group with Xs=4, then the UE cannot monitor other SSs including other CSSs than type0-PDCCH CSS in the same slot group. In addition, we agreed to distribute monitored slot for type0-PDCCH CSS into different slot groups, i.e., n0 and n0+X0 slots, to ensure the configuration flexibility for type0-PDCCH CSS monitoring. Thus, for the same reason as the previous agreement, we suggest increase the maximum number of CCEs even for Xs= 4 slot with 960 kHz SCS. |
| LG Electronics | Same view with Qualcomm. In addition, it should be noted that the BD/CCE budget for 960 kHz for (Xs,Ys)=(4,2) or (4,1) can be equally applied to a budget for 480 kHz for (Xs,Ys)=(2,1), if supported. |
| Apple | We support Proposal A3-1.1. |
| Transsion | We support Proposal A3-1.1. |

| Futurewei | We support Proposal A3-1.1 and open to discuss Proposal A3-1.2. |
| --- | --- |
| Lenovo | Similar view as NTT DOCOMO and Ericsson. We think Proposal A3-1.2 makes sense since it will allow better use of these optional slot group sizes. We shouldn't intentionally limit in the flexibility. |

#### First round discussion summary

A slight majority of companies prefers Proposal A3-1.1 (CCE budget = 16), a slight minority prefers Proposal A3-1.1 (CCE budget = 28). Several companies expressed interest in further discussing the detailed number for the CCE budget. There is no consensus to restrict 960kHz multi-slot PDCCH monitoring, only (X,Y)=(8,1) and (8,4) are supported in a Pcell. FL suggests to continue the discussion on BD/CCE budget of 960 kHz for (X_s,Y_s )= (4,2), (4,1) taking 10/16 and 10/28 as starting points.

#### Second round discussion

In view of the first round discussion, FL suggests to further discuss the BD/CCE budget for of 960 kHz for (X_s,Y_s )= (4,2), (4,1) taking 10/16 and 10/28 as starting points. If there is no consensus for a different value, it is suggested to adopt 16 CCEs as in the WA.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | As what we commented in the first round, we are fine with larger CCE budge of 960kHz but we just don’t think the calculation of 28=4+8+16 is reasonable. The PDCCH candidate can overlap and we only want to make sure the number of non-overlapped CCE is no less than 16. |
| MediaTek | Before making agreement on the BD/CCE budget, we suggest to align company views on whether BD/CCE=10/16 can be supported considering the SS#0 predetermined BD/CCE budget. In our view, based on the table below, we don’t think it is feasible to support SS#0 monitoring under BD/CCE=10/16 per slot group. Therefore, we can accept BD/CCE=10/16 with FFS on how to resolve BD/CCE insufficiency on SS#0 monitoring, if necessary. CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. As for 10/28, it will increase UE PDCCH detection complexity significantly since we only have one agreed processing timeline. Therefore, we can’t support such option. |
| Intel | For a CORESET with 32 CCEs (96 PRBs, 2 symbols), how many CCEs will be mapped for 4/2/1 PDCCH with AL 4/8/16? Our calculation based on the PDCCH to CCE mapping is that it will occupy 32 CCEs. We would like to hear other companies’ views. If the calculation is confirmed, shall we change the other candidate to 10/32? |
| Samsung | We are ok with increasing the CCE budget to 28. |
| Apple | We have the same view as MTK. |
| Intel2 | I would like to withdraw my early questions. The maximum value should be 28. If majority companies want to revise the WA, we are fine with the new value. |
| Sharp | We support different BD/CCE budget of 960kHz for (Xs,Ys) = (4,1),(4,2), since the purpose of multi-slot monitoring is to ensure a budget. |
| NTT DOCOMO | We support to increase CCE to 28 when Xs=4 for 960 kHz SCS. |
| LG Electronics | We can support to increase CCE budget for Xs=4 for 960 kHz. BTW, is a CCE budget of 28 enough? In order to define a new value correctly (and to speed up the progress), it would be better for proponents to provide the necessary value considering all possible cases. |

#### Second round discussion summary

Most companies support a CCE budget larger than 16 for Xs=4 with µ=6, where mainly 28 CCEs is being proposed and supported. However one company (MediaTek) seems to have strong concerns on increasing to 28 CCEs because of timeline reasons.

#### Third round discussion

FL suggests to check if MediaTek (or other companies) has sustained concerns in view of the majority of companies are supporting 28 CCEs.

| Company | Comment |
| --- | --- |
| MediaTek | We have strong concern on supporting 28 CCEs for Xs=4 under 960kHz. As we mentioned, processing timelines have been agreed and we see no room to increase BD/CCE budget at maintenance stage, even it is optional. When we evaluate timeline, we took the assumption of 120kHz BD/CCE limit, which is almost half of 28 CCEs within 4 slots under 960kHz (if we scaled time accordingly). To us, increasing BD/CCE budget only for the purpose of monitoring SS#0 is not a desirable solution. Note that we spent several meetings to discuss how to adjust SS#0 monitoring configuration under the BD/CCE=20/32 limit in WI phase and it is not acceptable to us to change the BD/CCE limit now to accommodate SS#0 monitoring in maintenance phase. Therefore, we can’t accept such proposal. We can confirm the WA on 10/16 BD/CCEs and FFS on the how to resolve SS#0 monitoring. |
| LG Electronics | We are fine with the proposal. |
| Ericsson | We support 28 CCEs for for Xs=4 with µ=6 |
| Moderator | There is a consistent strong concern against supporting 28 CCEs for Xs=4 for 960 kHz. Moderator would like to ask MediaTek (or other companies opposing 28 CCEs) whether another number greater than 16 CCEs could be acceptable. |
| MediaTek | Thanks to Moderator’s reply. Unfortunately, we still have strong concern on supporting more than 16 CCEs for Xs=4 at this stage where processing timeline designed has been finished. There are other simpler solution as mentioned in our Tdoc. For example, we see no harm to apply Xs=4 only on the cells which are not required to monitor Type-0 PDCCH. Even the discussion on the configuration restriction of Group(2) SS sets in A2-1 implies that UE should not monitor Type-0 PDCCH that often. We don’t see the strong motivation to increase the BD/CCE budget with the significant cost at UE implementation. |

### Issue A3-2: BD/CCE budget for MSM in multiple serving cells (e.g. carrier aggregation)

#### First round discussion

The documents submitted to RAN#108-e and the summary of the second round discussion in RAN1#107b-e identified the following alternatives (FL Note: Alt 1-1 marked as deleted as there was no support for it):

- Alt 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation

  - Alt 1-1: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs,Ys value are grouped together to follow a total BD/CCE budget

  - Alt 1-2: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget

- Alt 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation

  - Alt 2-1: A serving cell with SCS μ and multi-slot-based capability Xs,Ys is considered as an equivalent virtual cell with SCS μ-log2Xs and slot-based capability, where a slot group for the serving cell is considered as a slot for the virtual cell

  - Alt 2-2: A serving cell with SCS μ and multi-slot-based capability is considered as an equivalent virtual cell with SCS μ=3 and slot-based capability, where 4/8 slots for the serving cell with SCS μ=5/6 is considered as a slot for the virtual cell

- Alt 3: For multi-cell operation, if the number of configured DL cells is greater than the reported capability of supported DL cells

  - For a serving cell with mandatory X (i.e., X=4/8 for 480/960 kHz), BD/CCE budget is calculated by transforming the serving cell to the cell with 120 kHz SCS

  - For a serving cell with optional X (e.g., X=4 for 960 kHz), the serving cells with the same SCS and Xs value are grouped together for BD/CCE budget distribution.

FL Summary: Most companies showed support for Alt 1-2 or any Alt 2. Alt 3 covers the approach from both Alt 1 and Alt 2.

FL asks companies for their opinion on the identified alternatives. From the submitted documents, there seems to be a slight preference for Alt 1-2.

| Company | Comment |
| --- | --- |
| MediaTek | We apologize that we didn’t express our view clearly in our Tdoc. In fact, we support Alt1-1. As we commented in issue A1-3, a CC with (Xs,Ys)=(4,1) and a CC with (Xs,Ys)=(4,2) are handled differently from UE implementation perspective, e.g., power saving and number of BD per slot. Therefore, we prefer to take Ys into account when determining the BD/CCE limits for CA cases. We also like to point out that the BD/CCE budget specified for a group of cells might also have some impact on determining the minimum BD/CCE limit for a cell in the group of cells based on the following specification: For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot. For example, assume a UE can support one DL cell and is configured with 2 CCs, one with (4,2) and the other one with (4,1). If we only consider Xs regardless of Ys when grouping cells of CC1 with (4,2) and CC2 with (4,1), then the min BD/CCE per cell for CC1 and CC2 can be twice of the min BD/CCE limit when grouping cells with different Ys. |
| Samsung | We support Alt 1-2 to reuse the same principle for BD/CCE budget determination in CA mode in R15/16. |
| Ericsson | We also support Alt 1-2 to reuse the principle from Rel-15 per-slot monitoring and view it as the most straightforward solution. |
| Qualcomm | We support Alt 1-2. |
| vivo | Our first preference is Alt. 2 for simplicity. Otherwise, UE needs to handle 4 additional cases and reports many capability parameters as discussed in Issue C-1 |
| Nokia, NSB | We support Alt 1-2 |
| Intel | We prefer Alt 1-2. One clarification on Alt 2. Does Alt 2-1 map combination (4,1) for SCS 960kHz to a slot of SCS 240kHz? On the other hand, Alt 2-2 doesn’t cover combination (4,1) for SCS 960kHz. Is it correct understanding? Alt 3 sounds a variation of Alt 2. Since Alt 2/3 requires the sharing of BD/CCE among CCs with different SCS, we are afraid it causes complexity at UE side. Therefore, Alt 1-2 is preferred for simplicity. |
| ZTE, Sanechips | We prefer Alt 1-2, since the value of Ys in (Xs, Ys) does not impact the maximum BD/CCE budget, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget. |
| NTT DOCOMO | We support Alt 1-2 for the same reason as Samsung |
| LG Electronics | We support either Alt 2-1 and Alt 3. For a mixed situation of per-slot monitoring, per-span monitoring and per-slot-group monitoring, there would be too many combinations of different monitoring capabilities (as listed in Proposal C1-1.1). So, we support the concept of treating serving cell with per-slot-group monitoring capability as if they were cells with per-slot monitoring capability for BD/CCE budget distribution with multiple serving cells. We think it can be an efficient approach to avoid complicatedly handling all cases listed in Proposal C1-1.1. |
| Apple | We support Alt 1-2 to reuse the Rel-15/16 methodology. |
| Transsion | We prefer Alt 1-2. |

| Futurewei | Support Alt 1-2 |
| --- | --- |
| CATT | Support Alt 1-2 |

#### First round discussion summary

A minority of companies prefers Alt 2 or Alt 3 expressing a benefit in not requiring too many additional cases and capability parameters (Issue C-1). Nevertheless a majority prefers Alt 1-2.

FL suggests to agree on Proposal A3-2.1 (combining the general description of Alt 1 with the detail of Alt 1-2 to a single statement and editorial modification).

Proposal A3-2.1:

- For serving cells with the same PDCCH monitoring type including multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to determine a total BD/CCE budget for that group

#### Second round discussion

In view of the first round discussion, FL suggests to adopt Proposal A3-2.1 (see above).

Please comment only if you have a strong concern with the proposal.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | We support Proposal A3-2.1. |
| MediaTek | We still prefer to add Ys into the grouping rule. If the grouping rule doesn’t consider ys, then it will have some impact on determining the minimum BD/CCE limit for a cell in the group of cells based on the following specification: For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot. However, if we are the only company to object it, then we are ok to accept majority of view. |
| Moderator | Thanks MediaTek for being flexible if no other companies has a concern. Then I will suggest email approval for Proposal A3-2.1, where I will note "Though MediaTek has voiced a concern on the proposal, they are ok to accept the majority view if no other concerns are raised". If there are any additional concerns, please raise them by email. |
| Ericsson | We support Proposal A3-2.1, but the wording seems like it can be improved. Seems a bit ambiguous to say "include multi-slot based capability". Also, two things are determined in the grouping (1) the total budget for the group, and (2) the per-cell budget within the group. Proposal A3-2.1a: - For serving cells configured with 480 or 960 kHz SCS, with the same PDCCH monitoring type including multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to determine a total BD/CCE budget for that group and the per-cell BD/CCE budget within the group. With this high level proposal, it may be possible to leave the specification details to the spec editor. |
| vivo | We support Alt. 2 for simplicity. For Proposal A3-2.1, we can live with it if it is agreed together with Proposal C1-1.2 and Proposal C1-1.2 as a whole package. @Intel: Thanks for the question. Please fine our answer below: Does Alt 2-1 map combination (4,1) for SCS 960kHz to a slot of SCS 240kHz? [vivo] Yes, there will be a virtual cell with 240KHz. On the other hand, Alt 2-2 doesn’t cover combination (4,1) for SCS 960kHz. Is it correct understanding? [vivo] No, it covers the case. Even for (4,1) and SCS 960KHz, 8 slots are grouped together which mapped to a 120KHz cell. |
| Moderator | Please continue the discussion in the next section (2.3.2.4) |

#### NEW Second round discussion

Please consider the following proposal (based on Ericsson's modification and vivo's suggestion):

Proposal A3-2.1b:

- For serving cells configured with 480 or 960 kHz SCS, the serving cells with the same SCS and Xs value are grouped together to determine a total BD/CCE budget for that group and the per-cell BD/CCE budget within the group.

- Agree Proposals C1-1.1a (see section2.5.1.3) and C1-1.2 (see section 2.5.1.1).

| Company | Comment |
| --- | --- |
| Apple | We are fine with the proposal |
| Intel | We are OK with the package proposal |
| Samsung | We are ok with Proposal A3-2.1b. |
| NTT DOCOMO | We are fine with the Proposal. |
| MediaTek | We need more time to check the value in C1-1.1a and prefer to not include it in the proposal. |
| Apple | On the values used in the proposal, the key idea is that a Rel-16 PDCCH capability is 2ce the complexity (BD/CCE) of a Rel-15 PDCCH capability hence a minimum of 3 for (Rel-15, Rel-16) decided in Rel-16. A Rel-17 multi-slot PDCCH capability is equivalent in complexity (BD/CCE) to a Rel-15 PDCCH capability. As such, the minimum is equivalent to a Rel-15 PDCCH capability when it is by itself or combined with Rel-15. When combined with Rel-16, it is equivalent to the Rel-15 case (minimum = 3) and when combined with Rel-15 an Rel-16, we have 1 Rel-15, 1 Rel-16 (equivalent to 2 Rel-15) and 1 Rel-17 to give a minimum of 4. |
| Futurewei | We are OK with the Proposal A3-2.1b |
| LG Electronics | We are fine with Proposal A3-2.1b if majority companies agree on C1-1.1a and C1-1.2. |
| MediaTek | Thanks to Apple’s reply. Can you also clarify why pdcch-BlindDetectionCA-R17 is fixed to 4? - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - pdcch-BlindDetectionCA-R17 is equal to 4 {similar to Rel-15} Also can you pointed out where in the spec have the following related information? “a minimum of 3 for (Rel-15, Rel-16) decided in Rel-16.” We thought it should be 2. In conclsusion, we suggest the following change to reflect our comments and remove some redundant sentences. Proposal C1-1.1a (correcting two typos in C1-1.1): For Rel-17, 4 additional cases for UE capability signaling need to be defined: - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - pdcch-BlindDetectionCA-R17 is equal to [4] {similar to Rel-15} - Case 5: Capability on the number of CCS with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: [1:15] - The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: [4:16] - Case 6: Capability on the number of CCS with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: [1:15] - The minimum of pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 3 - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: [3:16] - Case 7: Capability on the number of CCS with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: [1:15] - The minimum of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : [4:16] |
| Samsung | To respond to MediaTek: The fields in TS 38.331 is captured in the following format: pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 SEQUENCE { pdcch-BlindDetectionCA1-r16 INTEGER (1..15), pdcch-BlindDetectionCA2-r16 INTEGER (1..15) } OPTIONAL And we further have: pdcch-BlindDetectionCA INTEGER (4..16) OPTIONAL, This should be the reasoning for proposal C1-1.1, but we are ok to further check the range values (in our view they should be in bracket for further discussion, and more importantly to agree on the feature first). In this sense, we are ok with Proposal C1-1.1a. |
| MediaTek2 | Thanks to Samsung’s information. We also found the same thing but pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 SEQUENCE { pdcch-BlindDetectionCA1-r16 INTEGER (1..15), pdcch-BlindDetectionCA2-r16 INTEGER (1..15) } seems to imply that - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R15(17): [2:16] However, since the range is in the bracket, we are fine with it. The main clarification we would like to ask is - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - pdcch-BlindDetectionCA-R17 is equal to [4] {similar to Rel-15} To us, 4 should be the min. If this aspect is clarified, then we can support the proposal with the modification we mentioned in the previous comments. Hope this clarified our view. |
| Samsung2 | Thanks to MediaTek for the detailed response, and we understand your concern better now. Sorry that we didn’t check the wording of the proposal in detail, and agree with you in the following aspects: - The crossed-out bullet in C1-1.1a is correct, and it’s not only duplicated, but also not fully correct in our view: e.g. we should discuss the value of min (r15+r16) instead of min r15 + min R16. - The value for pdcch-BlindDetectionCA-R17 should not be a fixed value as 4, and we believe the intention of the original proposal was trying to say its min value is following Rel-15 - The note “similar to Rel-15” seems not needed, since it’s actually the same range if we don’t revise it further; and if we revise it, then it’s not proper to say “similar to Rel-15” - Corrected several obvious typos. We updated the proposal based on above comments: Proposal C1-1.1b (modified by Samsung): For Rel-17, support UE capability signaling for 4 additional cases for UE capability signaling need to be defined: - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - Range of pdcch-BlindDetectionCA-R17 is equal to : [4:16] {similar to Rel-15} - Case 5: Capability on the number of CCSs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: [1:15] - The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: [4:16] - Case 6: Capability on the number of CCSs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: [1:15] - The minimum of pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 3 - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: [3:16] - Case 7: Capability on the number of CCSs with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: [1:15] - The minimum of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : [4:16] |
| Moderator | Thanks to all companies for elaborating and improving the proposal. I would like to check if companies have a common understanding about the notation e.g. [4:16]. Is it common understanding that this means all integer values from 4 to 16 (inclusive), but with squared brackets, i.e. the values are tentative and can be revisited? If so, my preference would be to use [{1, 2, …, 15}] for clarity. This may also allow us to clarify lower values without square brackets if these are agreeable, e.g. {1, 2, …, [15]}. If on the other hand the square brackets here are intended just as a range definition, it should be {1, 2, …, 15}. From earlier comments, I expect most companies have the former understandning (i.e. tentative value range). |
| Intel | We share moderator’s view that square bracket means the value is tenterive. With such a syntax, not all values or value range in the proposal need to be square bracket. As moderator commented, it should be fine to change [1:15] to {1:15}. On the other hand, there may be a different view on the minimum value of the range of the last sub-bullet of each case. It would be fine to put the into square bracket, i.e. {[4]:16}, {[4]:16}, {[3]:16}, {[4]:16}. |
| vivo | Agree with moderator on the notation clarification. Besides, we have the following two comments: Comment 1: it seems that the same parameter name is used for different cases. In our understanding, they should be different in final UE feature implementation. Maybe it is better to add a note to clarify this: Note: The above UE capabilities are defined as case-specific Comment 2: Agree with Samsung that we should discuss the value of min (r15+r17) instead of min r15 + min R17. Comment 3: The above proposal doesn’t take NR-DC into account. MCG and SCG specific capability should be defined as well when operation in NR-DC scenario. |
| LG Electronics | We share the same view with Moderator that the values are tentative and can be revisited. We also prefer {1:15} not [1:15] for clarity. |
| Moderator | Thanks for the feedback so far. Can we consider the following including notation update (based on Samsung's revision)? Please comment where you (don't) see the need for square brackets around inndividual values or whole ranges. Proposal C1-1.1c: For Rel-17, support UE capability signaling for 4 additional cases : - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - Range of pdcch-BlindDetectionCA-R17: {[4], …, 15, 16} - Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4], …, 15, 16} - Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3], …, 15, 16} - Case 7: Capability on the number of CCs with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : {[4], …, 15, 16} |
| MediaTek | Thanks to Moderator’s consolidated proposal. We prefer to keep all the values in bracket. For example, if we only put bracket on the values in the first bullet as follows, then it is possible that the summation of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17 will not reach 16. - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4], …, 15, 16} |
| Apple | Thank you everyone for fixing the typos. We are fine with the updates and the moderator’s proposal. Can you also clarify why pdcch-BlindDetectionCA-R17 is fixed to 4. For Rel-17, the PDCCH monitoring capability for a Rel-17 multi-slot is equivalent to that of a Rel-15 slot. As the minimum for Rel-15 is 4, then our thinking is that we can have the same for the Rel-17 multi-slot. Q: Also can you pointed out where in the spec have the following related information? “a minimum of 3 for (Rel-15, Rel-16) decided in Rel-16.” We thought it should be 2 - Please see 38.822: Also please find the agreements during the Rel-16 discussion RAN1 #99 Agreement - UE reports its PDCCH monitoring capability for the following cases: - Case 1: Capability on the number of CCs with Rel-15 monitoring capability only - n This capability already exists in Rel-15 - Case 2: Capability on the number of CCs with Rel-16 monitoring capability only - u pdcch-BlindDetectionCA-R16 can be smaller than 4 - Case 3: Capability on the number of CCs with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells - n pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - n pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - u Each of pdcch-BlindDetectionCA-R16 and pdcch-BlindDetectionCA-R15 can be smaller than 4 - u (The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R16) is not larger than 4 - l FFS (the minimum of pdcch-BlindDetectionCA-R15 + the minimum of pdcch-BlindDetectionCA-R16) can be smaller than 4 - pdcch-BlindDetectionCA-R15 and pdcch-BlindDetectionCA-R16 for the above three cases can be reported separately RAN1 #101 Agreement The following RAN1 agreement is revised (RED part) For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA: · The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1 - [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16 - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 |

### Issue A3-3: DCI processing

#### First round discussion

Proposal A3-3.1 (see R1-2201765, R1-2202130):

If a UE is provided

- one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, or ra-SearchSpace, or a CSS set by PDCCH-Config, and

- a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI,

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot group

Proposal A3-3.2 (see R1-2202130):

If PDCCH MOs of both Group (1) and Group (2) SS sets are configured in the same slot group, and there is at least one valid PDCCH MO of USS set(s) after overbooking and dropping, the UE does not monitor DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, and CS-RNTI in the Group(2) SS set(s).P

Proposal A3-3.3 (see R1-2201689):

- UE should be able to process one broadcast DCI for SI/RACH/paging in addition to the agreed number of processed unicast DCI in a slot group of X slots.

- To clarify whether a UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell with SCS 15kHz

- The limitation on number of detected DCIs in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz

FL Summary: Only few companies commented on this aspect in their documents, one company (R1-2201735) observes that it is not needed to specify that within a group of Xs slots, the UE is not be expected to process information from more than one DCI format with CRC scrambled by a given RNTI associated with Group (2) SSs, i.e., SI-RNTI, P-RNTI, RA-RNTI, MsgB-RNTI.

FL asks companies if they are supporting Proposal A3-3.1, Proposal A3-3.2 or Proposal A3-3.3.

| Company | Comment |
| --- | --- |
| Samsung | For Proposal A3-3.1: We are ok with Proposal A3-3.1. For Proposal A3-3.2: We don’t think the UE behavior for DSS should be directly applied to multi-slot PDCCH monitoring. For Proposal A3-3.3: Can revisit after multiple-cell operation is more clear. |
| Ericsson | Proposal A3-3.1: As we discusse in our contribution, we don't expect that the UE would need to process information from more than one DCI since regardless of the configured MO periodicity within SearchSpace, the MOs the UE actually monitors are separated at least by P*N slots where P is the configured periodcity, and N is the number of transmitted SSBs. Our expectation is that P*N is always greater than Xs in any practicaly deployment. Proposal A3-3.2: At least we are not aware that there is a similar restriction for a 120 kHz slot in the current spec. Hence, we don't think a restriction per slot group should be introduced since a 120 kHz slot and 480/960 kHz slot group have the same absolute time duration. Proposal A3-3.3: We are not aware of a similar restriction on processing only one broadcast DCI within a 120 kHz slot in the current spec. Hence, we don't think a restriction per slot group should be introduced since a 120 kHz slot and 480/960 kHz slot group have the same absolute time duration. |
| Qualcomm | We support Proposal A3-3.1, as a direct extension of the legacy slot-based PDCCH monitoring. We also support Proposal A3-3.2 as the proposing company, but this is also related to Issue A2-2. If Interpretation #2 is applied for multi-slot PDCCH monitoring, Proposal A3-3.2 is not needed. However, if Interpretation #1 is applied, in the worst case, it may result in per-slot monitoring of unicast PDCCH (i.e., C-RNTI, MCS-C-RNTI, and CS-RNTI), due to unicast PDCCH piggybacking on CSS: Section 10.1 in TS 38.213: If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, a CS-RNTI, a SL-RNTI, a SL-CS-RNTI, or a SL Semi-Persistent Scheduling V-RNTI the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.. For example, in the second slot group of Xs = 4 in the following figure (according to Interpretation #1 of A2-2), the UE should monitor unicast PDCCH in “every slot” within the slot group. |
| vivo | Proposal A3-3.1: We are OK with the proposal Proposal A3-3.2 and A3-3.3: WE don’t see the need to have such limitation |
| Intel | Proposal A3-3.1: OK Proposal A3-3.2: same view as Samsung. Proposal A3-3.3: OK to wait for more agreements/conclusions on Group (2) SS. |
| ZTE, Sanechips | We can only agree with proposal A3-3.1. |
| LG Electronics | For Proposal A3-3.1, we shar the view with Ericsson that we don't expect that the UE would need to process information from more than one DCI. For Proposal A3-3.2 and A3-3.3, we don’t see the need to have such limitation. |
| Apple | We support Proposal A3-3.1 |

| Futurewei | Support Proposal A3-3.1 , we do not see the need for A3-3.2 and A3-3.3 |
| --- | --- |
| CATT | Proposal A3-3.1: OK with the proposal Proposal A3-3.2 and A3-3.3: No need to specify these . |

#### First round discussion summary

Regarding Proposal A3-3.1, 7 companies support the proposal while 2 companies don't see a need in practical deployments. FL notes that even if it is not necessary in many cases, it seems the proposal doesn't have any negative consequences in those cases either.

Regarding Proposal A3-3.2, a majority of companies doesn't see a need to adopt the proposal. The supporting company clarified that the proposal is not needed if Interpretation 2 in Issue A2-2 is the common understanding.

Regarding Proposal A3-3.1, 2 companies are open to revisit it after more progress is achieved, while 4 companies didn't identify a need to adopt the proposal.

FL suggests to adopt Proposal A3-3.1.

#### Second round discussion

In view of the first round discussion, can we adopt Proposal A3-3.1? FL notes that even if it is not necessary in many cases, it seems the proposal doesn't have any negative consequences in those cases either.

If a UE is provided

- one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, or ra-SearchSpace, or a CSS set by PDCCH-Config, and

- a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI,

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot group

Please comment only if you have a strong concern with the proposal.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | We agree with the above Proposal A3-3.1. |
| Moderator | As no strong concerns have been raised so far, I will suggest email approval for Proposal A3-3.1. If there are any concerns, please raise them by email. |
| LG Electonics | With FL’s explanations, we are fine with Proposal A3-3.1. |
| Huawei, HiSilicon | The intention of the proposal is not clear for us and we cannot support it at this time. In Rel-15, the original rule is applied to CSS monitoring which includes (Group (2) SS + Type3-CSS): From 38.213: If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.. However, Proposal A3-3.1, removes “INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI”. It seems that the intention was not to adopt the legacy monitoring rule for Type3-CSS. The first question is why? The second question is, if it was the intention, why SFI-RNTI is not removed? |
| Ericsson | We don’t Proposal A3-3.1 is needed, and we do not support it. As we discusse in our contribution, we don't expect that the UE would need to process information from more than one DCI with CRC scrambled by any of the listed RNTIs, so why should rules be specified for cases that do not happen? SI-RNTI and P-RNTI: Even if the Type0/0A/2 SSs are configured with non-zero search space ID and the MO periodicity is as small as 1 slots, the MOs that the UE actually monitors are are separated at least by P*N slots where P is the configured periodcity, and N is the number of transmitted SSBs. Our expectation is that P*N is always greater than Xs in any practical deployment. RA-RNTI and MsgB-RNTI: Once the UE decodes the PDCCH schedulding the RAR, it would never need to receive/process another DCI scheduling another RAR within Xs slots. SFI-RNTI: This is a Group (1) SS, and we have already agreed that the monitoring periodicity cannot be less than Xs slots, so how could the UE be required to decode more than one DCI with CRC scrambled by SFI-RNTI per slot group? |
| vivo | We have similar question with Huawei, i.e. why to remove certain RNTIs from the original listed RNTIs |
| Moderator | Please continue the discussion to resolve comments and questions by Huawei, Ericsson, vivo. |
| Intel | As commented by email, we would like to ask for clarification on ‘any of’ in the proposal? Does it require UE to process only one DCI for the 5 listed RNTI in a slot group, or does it imply UE can decode up to 5 DCIs (one for each RNTI)? It may be similar to the use of ‘any of’ in FG 3-1. Our interpretation is in total one DCI per slot groups considering all listed RNTIs |
| Futurewei | We have same concerns as Huawei and vivo on removing of some specific RNTIs |

#### Second round discussion summary

Several companies are questioning why “INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI” are not part of the proposal. It is suggested to continue the discussion in a third round.

#### Third round discussion

Please continue the second round discussion on open questins (copied for convenience).

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon (from second round) | The intention of the proposal is not clear for us and we cannot support it at this time. In Rel-15, the original rule is applied to CSS monitoring which includes (Group (2) SS + Type3-CSS): From 38.213: If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.. However, Proposal A3-3.1, removes “INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI”. It seems that the intention was not to adopt the legacy monitoring rule for Type3-CSS. The first question is why? The second question is, if it was the intention, why SFI-RNTI is not removed? |
| Intel (from second round) | As commented by email, we would like to ask for clarification on ‘any of’ in the proposal? Does it require UE to process only one DCI for the 5 listed RNTI in a slot group, or does it imply UE can decode up to 5 DCIs (one for each RNTI)? It may be similar to the use of ‘any of’ in FG 3-1. Our interpretation is in total one DCI per slot groups considering all listed RNTIs |
| ZTE, Sanechips | We agree with Huawei’s comments, this proposal should be a simple extension of the legacy slot-based PDCCH monitoring by only changing “per slot” to “per slot group”, We may add the “INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI” in Proposal A3-3.1. |
| Futurewei | Agree with Huawei and ZTE that this proposal should just extend the legacy slot-based to group of L slots based and preserving “INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI”. |
| Ericsson | We don't agree that this legacy rule should be extended for the technical reasons we listed in the 2nd round. The scenario that is intended by the proposal won't happen. Furthermore, regarding preserving “INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI” these will all be in a Type-3 CSS which is part of Group (1), and we already agreed that the monitoring periodicity cannot be less than a slot group. So how would it even happen that the UE would be expected to process information from more than one DCI with CRC scrambled with these RNTIs within a slot group? |
| Qualcomm | As ZTE and Futurewei commented above, we think it’s agreeable to add other RNTIs that are in the legcy specification, but missing in the previous version of the proposal. To reply to Ericsson’s comment: - As we commented in A2-1, for Proposal A2-1.1d, there could be a deployment scenario where the number of transmitted SSBs, N, is very small, e,g., N < L. Unlike Ericssion’s view that such a scenario is impractical, we don’t think it is impractical, although it would not be typical. For example, some indoor deployment where the gNB only covers a very small room, we think it is a plausible assumption that the number of transmitted SSBs can be smaller than 4 or 8. Thus, we think a special rule is needed. - For Ericsson’s comment in the second round: Although some clarification may be required, in our understanding, the wording “process information” in the proposal (also in TS 38.213) includes blind decoding of PDCCH candidates. Thus, for Type 1 CSS without dedicated RRC config, the issues is that the UE may be required to monitor every slot in the slot group for DCI format with RA-RNTI, although the actual RAR PDCCH would be detected in only one of the slots. Likewise, for SFI-RNTI, with Ys=2, the UE may be required to monitor two consecutive slots although the actual SFI would be detected in only one of the slots. Therefore, to avoid such situations, we think Proposal A3-3. is needed. |
| LG Electronics | We share the view with ZTE. We also feel the need to clarify on Intel's comments as it may help to make this suggestion clearer. |
| Moderator | I have incorporated this proposal to A2-1 (section 2.2.1.5), please check there for further comments and discussion. |

### Issue A3-4: Dropping rules

#### First round discussion

One company (R1-2201352) observes that SS overbooking across different slot groups can be avoided by gNB implementation so that no dropping rule for this case is necessary.

Other companies have stated that due to UE mobility across beams or SSSG switching, the maximum number may be exceeded anyway, and this may need to be covered by a dropping rule in the specifications.

Proposal A3-4.1 (see R1-2201689):

- A span of SS sets configured in a slot group, if it is not monitored by the UE based on a semi-static rule, may not be counted in the number of monitored spans in the slot group.

- If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of at least the group (1) SS sets

Proposal A3-4.2 (see R1-2201914):

Additional dropping rules for PDCCH multi-slot monitoring should be defined to limit back-to-back SS monitoring between Group 1 and Group 2 SSs across multiple slot-groups. A window-based approach around the CSS maybe considered to determine whether or not a USS monitoring occasion is dropped or not for PDCCH monitoring.

Proposal A3-4.3 (see R1-2202130):

A dropping rule for PDCCH MOs may be applied for the first Ys consecutive slots after SSSG switching, if the separation between the two Ys consecutive slots before and after the SSSG switching boundary is less than Xs slots.

Proposal A3-4.4 (see R1-2202190):

To avoid the back-to-back problem, monitoring should not be done in the Xs slots before and after the SSSG boundary.

Proposal A3-4.5 (see R1-2202409):

When multi-slot PDCCH monitoring is applied with shifting of Group(2) SS due to n0 change, then dropping of Group (1) SS MOs and/or Group (2) SS MOs in the slot where the shift is first applied should be supported to avoid back-to-back monitoring issue.

FL asks companies for their comments to Proposal A3-4.1, Proposal A3-4.2, Proposal A3-4.3, Proposal A3-4.4 or Proposal A3-4.5.

| Company | Comment |
| --- | --- |
| Samsung | For Proposal A3-4.1, we don’t think “a span in a slot group” is well-defined in RAN1 specification, and wonder what’s the RAN1 impact of the proposal. For Proposal A3-4.2 and A3-4.5, we didn’t see the need of the proposals, and it has already been discussed to treat the back-to-back monitoring issue based on UE’s implementation when we made the comprise agreement. For Proposal A3-4.3 and Proposal A3-4.4, there is always a gap after SSSG switching, so we don’t think there is need to address the immediate change of the location of Ys. It can be handled by proper gNB’s implementation. |
| Ericsson | Proposal A3-4.1: We don't support this proposal – not clear what a "span of SS sets in a slot group" is. Proposals A3-4.2, 4.3, 4.4, and 4.5: Do not support. Our original compromise in supporting per-slot group monitoring with small values of Ys was that additional rules on dropping considering adjacent slot groups would not be entertained. Dropping should be evaluated per-slot group, with no coupling between slot groups. |
| Qualcomm | We think Proposal A3-4.1 is already the general principle of the legacy specification and is not required to be discussed. For Proposal A3-4.2 and A3-4.5, we agree with Samsung’s view and think no further discussion is needed. We support Proposals A3-4.3 and A3-4.4. The issue is relevant to the case that two SSSGs are configured with different (Xs,Ys) values and, thus, it could be revisited after Issue A2-6 discussion. |
| Sharp | Support Proposal A3-4-4. A drop around the SSSG switching boundary is necessary to allow for more flexible search space settings. If the gap before and after switching is guaranteed by the gNB, a statement should be added that the UE does not expect the MO to be present in Xs slots either before or after the SSSG switching boundary. |
| vivo | We don’t support the above proposals, i.e. there is no need to have additional dropping rules. |
| Intel | Proposal A3-4.1: we prefer to make an agreement or conclusion on this proposal. Specifically, if a MO of Type0A/2 CSS set with searchSpaceID non-zero is mapped by a beam that is different from the UE’s SSB, shall UE monitor the MO of Type0A/2 CSS set? The critical impact is if such MO is counted in total number of BD/CCE for the slot group, it likely consumes all BD/CCE budget. As a results UE cannot be scheduled at all. This is not desired. Proposal A3-4.2: we prefer to not consider further optimization due to MOs of Group (2) SS Proposal A3-4.3: we are supportive to the proposal Proposal A3-4.4: fine in principle, however, the Xs slot not for PDCCH monitoring should be either earlier or later than the switching boundary, but not both Proposal A3-4.5: we prefer to not consider further optimization due to MOs of Group (2) SS |
| ZTE, Sanechips | We don’t think additional dropping rules are needed. |
| NTT DOCOMO | For Proposal A3-4.3 and Proposal A3-4.4, it depend on the discussion results on Issue A2-6. If the different configuration of (Xs, Ys) and/or location of Ys in a slot group can be allowed for before and after SSSG switching, the additional dropping rule may be required around SSSG switching boundary. |
| Panasonic | We are open to further discuss Proposals A3-4.1, A3-4.3 and A3-4.4. For Proposals A3-4.2 and A3-4.5, we agree with Samsung that no enhancement is needed to handle back-to-back monitoring between Group (1) and Group (2). |
| LG Electronics | For Proposal A3-4.1, to further discuss on it, we think a proponent needs to elaborate the meaning of “a span of SS sets in a slot-group” on the 1st bullet and “the maximum number of spans” on the 2nd bullet. For Proposal A3-4.2 and A3-4.5, we have the same view with Samsung. For Proposal A3-4.3 and A3-4.4, we have the same view with Qualcomm. |
| Apple | Although we would like to further optimize the back-to-back interaction, we think that the issues in A2-2 optimizing Group (2) SSs (especially for Type 1 CSSs without dedicated RRC configuration) are more important. |
| Lenovo | Question to Samsung's (+others') view about Proposal A3-4.2 and Proposal A3-4.5: Do you intend to say that a UE is expected to apply dropping even though the specification doesn't allow dropping? Or is the point that existing dropping rules allow such dropping, especially for the cases of SSSG switching and n0 change? |

Another issue concerns the detailed dropping following up on the agreement in RAN1#107-e:

Agreement - SS set overbooking can be allowed with multi-slot PDCCH monitoring capability same as the current specification but applied per slot group, i.e., SS set overbooking is allowed for USS in PCell and PSCell, and UE expects no overbooking for CSS in PCell and PSCell and no overbooking in SCell. - The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group, i.e., a UE drops UE specific search space set(s) in a slot group with higher index when SS sets are overbooked. - Additional dropping rules are not precluded

Proposal A3-4.6 (see R1-2202130):

If a SS set to be dropped by overbooking has multiple MOs within a slot group, they are dropped as a whole.

From the discussion in RAN1#107-e, FL assumed that Proposal A3-4.6 was the understanding when arriving at the agreement esp. in the second bullet. Companies are invited to state their views whether the proposal is in line with their understanding, and if this should be clarified/agreed in this meeting.

| Company | Comment |
| --- | --- |
| Samsung | We share same understanding as FL, dropping per search space set was agreed in the second bullet, and we wonder the extra specification impact of the proposal. |
| Ericsson | We share the same understanding as the FL. 2nd bullet of the agreement already covers this issue. |
| Qualcomm | From the discussion in RAN1 #107-e, we assumed that Proposal A3-4.6 was not fully clarified and the third bullet is opening the door for further discussion. For clarification, we think it’s good to explicitly capture an agreement like Proposal A3-4.6. |
| Nokia, NSB | We agree with the FL view |
| Intel | We share the same understanding as the FL. |
| ZTE, Sanechips | We agree with FL’s understanding. |
| NTT DOCOMO | We share the same understanding as FL. |
| LG Electronics | We share the same understanding as the FL. |
| Transsion | We agree with FL’s understanding. |

| Futurewei | We agree with the FL view |
| --- | --- |

#### First round discussion summary

After the first round, it seems Proposal A3-4.1 could be further discussed after clarification. Additionally, there seems to be interest in further discussing especially for Proposals A3-4.3 and A3-4.4. Also, a clarification on current dropping possibility after SSSG switching or n0 change is requested.

Regarding Proposal A3-4.6, most companies don't see a need for any additional agreement. FL notes that even though the earlier agreement's third bullet allows discussion for further dropping rules, these further dropping rules would need to be agreed (like other proposals listed under this issue). Therefore FL thinks no further discussion on Proposal A3-4.6 is necessary at this point.

FL suggests to continue the discussion on Proposals A3-4.1 to A3-4.5 taking the open questions into account.

#### Second round discussion

In view of the first round discussion, continue the discussion on Proposals A3-4.1 to A3-4.5. Some first round discussion input is quoted below for convenience.

| Company | Comment |
| --- | --- |
| Qualcomm (from first round) | We support Proposals A3-4.3 and A3-4.4. The issue is relevant to the case that two SSSGs are configured with different (Xs,Ys) values and, thus, it could be revisited after Issue A2-6 discussion. |
| Intel (from first round) | Proposal A3-4.1: we prefer to make an agreement or conclusion on this proposal. Specifically, if a MO of Type0A/2 CSS set with searchSpaceID non-zero is mapped by a beam that is different from the UE’s SSB, shall UE monitor the MO of Type0A/2 CSS set? The critical impact is if such MO is counted in total number of BD/CCE for the slot group, it likely consumes all BD/CCE budget. As a results UE cannot be scheduled at all. This is not desired. |
| Lenovo (from first round) | Question to Samsung's (+others') view about Proposal A3-4.2 and Proposal A3-4.5: Do you intend to say that a UE is expected to apply dropping even though the specification doesn't allow dropping? Or is the point that existing dropping rules allow such dropping, especially for the cases of SSSG switching and n0 change? |
| ZTE, Sanechips | We don’t think additional rules are needed. |
| Intel2 | We share Qualcomm’s view, i.e. to support A3-4.3 and A3-4.4 in principle and fine to wait until issue A2-6 is concluded. For Proposal A3-4.1, we still have the above question. If the SS set is actually not monitored by UE which is also known to gNB, can UE just drop the SS sets so that more BD/CCE can be used for USS sets? Welcome for more technical comments on it |
| Samsung | Response to Lenovo: Our understanding is current specification can already handle the dropping in general (always consider a slot group as a unit), and the cases of SSSG switching and n0 change is already included. There is no need of special treatment on the back-to-back PDCCH monitoring issue, since it’s about across the slot groups if we understand correctly of the proposals. Response to Intel: We believe the example can be avoided by gNB’s implementation. We don’t think it’s reasonable to agree on a rule to exclude some MO from counting the BD/CCE budget. It’s true that the BD/CCE budget is not large for 480/960, but the agreement on that numbers should already implies restriction to gNB’s scheduling, and no special treatment to a particular SS should be further agreed. |
| Sharp | We share Qualcomm’s vies and support Proposal A3-4.4. Also, agree with Intel's point in the first round that an Xs slot either before or after the switching boundary is sufficient. |
| NTT DOCOMO | For Proposal A3-4.3 and A3-4.4, if Proposal A2-6.2 is supported as a result of IssueA2-6 discussion, such additional dropping rule is not necessary. In addition, even if Proposal A2-6.1 is supported in IssueA2-6, we tend to agree with Ericsson that USS dropping is conducted only per slot-group and sliding window-like checking, i.e., BD/CCE budget calculation across slot group boundary, should not be considered. Similarly for Proposal A3-4.2 and A3-4.5, we don’t think additional dropping rule is necessary. |
| Apple | Proposal A3-4.3 and A3-4.4 are dependent on the decision in A2-6. These should be discussed concurrently. If decided, we peref A3-4.4 |
| LG Electronics | As commented by Qualcomm in the first round, Proposals A3-4.3 and A3-4.4 are relevant to the SSSG switching with different (Xs,Ys) combinations. We can revisit this after A2-6 discussion. For Proposal A3-4.2 and A3-4.5, we don’t think additional dropping rule is necessary. We agree to Samsung’s clarifications. |
| Futurewei | We prefer to discuss Proposal A3-4.3 and A3-4.4 after A2-6 is concluded. For A3-4.2 and A3-4.5 we do not think additional dropping rules are necessary. |

#### Second round discussion summary

It seems there is no consensus on A3-4.2 and A3-4.5. Dicussion may continue for Proposal A3-4.3 and A3-4.4, though companies have mentioned that its outcome will depend on progress in Issue A2-6.

#### Third round discussion

Please continue the discussion focussing on Proposal A3-4.3 and A3-4.4.

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We are fine to discuss this issue after the progress on Issue A2-6 while we don’t see the need for the additional dropping rules as commented in the previous round. |
| ZTE, Sanechips | We prefer to revisit Proposal A3-4.3 and A3-4.4 after reaching an agreement on isssue A2-6. |
| LG Electronics | As we commented earlier, we think Proposals A3-4.3 and A3-4.4 could be valid only if it is agreed to support SSSG switching between SSSGs that correspond to different (Xs,Ys) PDCCH monitoring combinations. In this perspective, we would like to suggest that A3-4.3 and A3-4.4 should be discussed with A2-6.3 in Issue A2-6. Please refer to our comment of the 3rd round discussion on Issue A2-6. |
| Ericsson | We prefer not to introduce additional dropping rules. As we commented earlier, it was a compromise to support per-slot group monitoring with small values of Ys in the first place, and we made this compromise with the understanding that it would avoid the need to specify additional droppint rules. Dropping should be evaluated per-slot group, with no coupling between slot groups, and our view is that SSSG switching should not be a motivator for introducing additional rules. |
| vivo | We prefer not to introduce additional dropping rules. We are fine to discuss after the progress on Issue A2-6. |

## (Closed) Topic B: Multi-Beam Aspects

R1-2201266:

Proposal 2: the UE can share gNB COT only if the UL transmission resources are within the gNB COT and the UL transmission beam is covered by the gNB sensing beam for directional LBT.

Proposal 3: For higher layer configured CSI-RS reception, the UE performs the reception if the CSI-RS resources are within the gNB COT and the gNB’s sensing beam covers the CSI-RS beam.

Proposal 4: In FR2-2 unlicensed band, the pre-configured downlink reception is not only confirmed by the SFI indication but also by gNB’s sensing beam, e.g., UE should cancel the downlink reception within the gNB COT if the gNB sensing beam does not cover the downlink transmission beam.

Proposal 5: R17 should allow UE to skip PDCCH monitoring in the CORESET associated with a beam uncovered by the gNB sensing beam within the gNB COT.

FL Note: As per chairman's guidance in RAN1#107bis_e, the topics related to proposals in R1-2201266 were moved to AI 8.2.6 (channel access), therefore no discussion will occur in this AI.

## Topic C: Multi-Cell Operation, Cross-carrier scheduling (except BD aspects)

### (Closed/Merged) Issue C-1: [High Priority] MSM capability for multiple serving cells

#### First round discussion

Proposal C1-1.1 (see R1-2201765):

For Rel-17, 4 additional cases  for UE capability signaling need to be defined:

- Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

  - pdcch-BlindDetectionCA-R17 is equal to  4 {similar to Rel-15}

- Case 5: Capability on the number of CCS with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells

  - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

  - Range of  pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: [1:15]

    - The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R17) is equal to  4

      - Range of pdcch-BlindDetectionCA-R15 +   pdcch-BlindDetectionCA-R17: [4:16]

- Case 6: Capability on the number of CCS with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

  - pdcch-BlindDetectionCA-R16 for Rel-15 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

  - Range of  pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: [1:15]

    - The minimum of pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 3

      - Range of pdcch-BlindDetectionCA-R16 +   pdcch-BlindDetectionCA-R17: [3:16]

- Case 7: Capability on the number of CCS with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

  - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R16 for Rel-17 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

  - Range of  pdcch-BlindDetectionCA-R17,  pdcch-BlindDetectionCA-R16,  and pdcch-BlindDetectionCA-R15: [1:15]

    - The minimum of pdcch-BlindDetectionCA-R15  + pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4

      - Range of pdcch-BlindDetectionCA-R15  + pdcch-BlindDetectionCA-R16 +   pdcch-BlindDetectionCA-R17 : [4:16]

Proposal C1-1.2 (see R1-2201765):

For the case with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells (case 7) or any combination of 2 of the capabilities (i.e. case 5, and case 6), the UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) as UE capability. If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17), as in Rel-16, the gNB configures which combination for the UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability.

Please comment whether you agree to Proposal C1-1.1 and Proposal C1-1.2.

| Company | Comment |
| --- | --- |
| Samsung | For Proposal C1-1.1, we support it. For Proposal C1-1.2, we support it. |
| Ericsson | Proposaal C1-1.1 and 1.2: We have large reservations of taking the discussion in this direction. The intention of introducing mult-slot monitoring is to equate the UE processing requirements in a 120 kHz slot to the UE processing requirement in a slot group of Xs slots. Hence, we think splitting the BD/CCE budget across cells configured with per-slot or per-slot group monitoring should follow the same approach without considering so many different cases. |
| Qualcomm | We support the proposals. |
| vivo | This is related with Issue A3-2. If Alt. 2 in issue A3-2 is agreed, there is no need to have such proposal. |
| Intel | We support the proposals. On the other hand, since Rel-17 monitoring capability is bounded with SCS 480/960kHz, it is potentially possible to optimize the RRC signaling. anyway, we can leave it to RAN2. |
| NTT DOCOMO | We support the Proposal C1-1.1 (typo can be corrected as follows) and C1-1.2. Proposal C1-1.1 (see R1-2201765): For Rel-17, 4 additional cases for UE capability signaling need to be defined: - … - Case 6: Capability on the number of CCS with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-156 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: [1:15] - The minimum of pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 3 - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: [3:16] - Case 7: Capability on the number of CCS with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-176 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - … |
| LG Electronics | We share the view with vivo that this is related with Issue A3-2. If Alt. 2 in issue A3-2 is agreed, there is no need to have such proposal. |
| Apple | As the proposing company, we support both proposals. |
| CATT | We have similar view as Ericsson |

Proposal C1-1.3 (see R1-2202072):

For multi-cell operation, UE can report a capability on whether the location of the Y slots within a slot group of X slots is maintained across CCs associated with (X,Y) configuration.

Note: Figure 1 Example of non-aligned monitoring pattern across CCs

Please comment whether you agree to Proposal C1-1.3.

| Company | Comment |
| --- | --- |
| Samsung | We prefer to handle it in UE capability discussion. |
| Qualcomm | We are open for the discussion. We see the power saving benefit of aligning the Y slots across CCs. |
| Nokia, NSB | Depriorize this topic in RAN1 #108 |
| Intel | Since Group (2) SS can be in any slot in a slot group, the benefit of aligning the Y slots across CCs is limited. For simplicity, we prefer to not enforce such limitation. |
| LG Electronics | We think that it should be first discussed whether/why the Y location (or the location of the monitoring slot) within a slot-group needs to be aligned across CCs. We are open for the discussion. |
| Apple | We support the proposal. |

#### First round discussion summary

Most companies seem to be fine with proposals C1-1.1 and C1-1.2, however some companies note that the proposal is not needed if Alt 2 in Issue A3-2 is agreed. One company expressed a preference to not considering so many different cases. Regarding proposal C1-1.3, there hasn't been much feedback so far.

FL suggests to continue the discussion on proposals C1-1.1 and C1-1.2. It would be beneficial if companies could provide a smaller set of cases if that is their preference. Discussion on proposal C1-1.3 should continue to allow input from more companies.

#### Second round discussion

In view of the first round discussion, FL suggests to continue the discussion on proposals C1-1.1 (with typo corrected as identified by NTT DOCOMO) and C1-1.2. It would be beneficial if companies could suggest a smaller set of cases if that is their preference.

Proposal C1-1.1a (correcting two typos in C1-1.1):

For Rel-17, 4 additional cases  for UE capability signaling need to be defined:

- Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

  - pdcch-BlindDetectionCA-R17 is equal to  4 {similar to Rel-15}

- Case 5: Capability on the number of CCS with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells

  - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

  - Range of  pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: [1:15]

    - The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R17) is equal to  4

      - Range of pdcch-BlindDetectionCA-R15 +   pdcch-BlindDetectionCA-R17: [4:16]

- Case 6: Capability on the number of CCS with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

  - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

  - Range of  pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: [1:15]

    - The minimum of pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 3

      - Range of pdcch-BlindDetectionCA-R16 +   pdcch-BlindDetectionCA-R17: [3:16]

- Case 7: Capability on the number of CCS with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

  - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability

  - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

  - Range of  pdcch-BlindDetectionCA-R17,  pdcch-BlindDetectionCA-R16,  and pdcch-BlindDetectionCA-R15: [1:15]

    - The minimum of pdcch-BlindDetectionCA-R15  + pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4

      - Range of pdcch-BlindDetectionCA-R15  + pdcch-BlindDetectionCA-R16 +   pdcch-BlindDetectionCA-R17 : [4:16]

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | If Alt 1-2 in issue A3-2 is agreed, we support proposals C1-1.1 (with typo corrected) and C1-1.2. |
| Intel | We support the proposal. |
| Samsung | We support proposals C1-1.1 and C1-1.2. We believe these proposals are essentially needed to enable multi-cell operation, which is absent from current specification. |
| NTT DOCOMO | We support the proposal. |

Discussion on proposal C1-1.3 can continue to allow input from more companies.

| Company | Comment |
| --- | --- |
| Samsung (from first round) | We prefer to handle it in UE capability discussion. |
| Qualcomm (from first round) | We are open for the discussion. We see the power saving benefit of aligning the Y slots across CCs. |
| Nokia, NSB (from first round) | Depriorize this topic in RAN1 #108 |
| Intel (from first round) | Since Group (2) SS can be in any slot in a slot group, the benefit of aligning the Y slots across CCs is limited. For simplicity, we prefer to not enforce such limitation. |
| LG Electronics (from first round) | We think that it should be first discussed whether/why the Y location (or the location of the monitoring slot) within a slot-group needs to be aligned across CCs. We are open for the discussion. |
| Apple (from first round) | We support the proposal. |
| ZTE, Sanechips | We are open to discuss proposal C1-1.3, we agree with Samsung and this can be discussed in UE capability session. |
| MediaTek | We would like to point out that similar capability on span alignment was already introduced in Rel-16 URLLC and we still see the same benefit on power saving and PDCCH candidate dropping rule checking. |
| Samsung | To clarify, we are open to the discussion, but it seems more like a UE capability discussion, since we didn’t see the need to mandate aligning the location of Ys slots across CCs. |
| NTT DOCOMO | We share the same view with Intel that the benefit with the proposal is limited since Group (2) SS can configured in any slot and don’t see the need to restrict Group (1) SS configuration depending on th UE capabiility. |
| LG Electronics | If Proposal C1-1.3 is a simple extention of similar capability introduced in Rel-16 and, also is only about UE capability, we agree with companies’s comment that this can be discussed in UE capability discussion. Also, it seems valid that the gains from aligning Y slots across CCs is limited due to Group (2) SS, commented by Intel. For this reason, we also prefer to handle C1-1.3 in UE capability discussion. |

#### Second round discussion summary

Proposal C1-1.1a and Proposal C1-1.2 are suggested to be handled as part of Proposal A3-2.1b (see section 2.3.2.4). Discussion continues there as part of the second round.

FL concludes that if companies show continued interest in the topic/proposal, it should be raised as part of the UE capability discussion.

### (Closed) Issue C-2: Cross-carrier scheduling limitations by SCS difference

FL Note: Discussion in earlier meetings has not shown consensus to introduce a cross-carrier scheduling limitation as a function of |μPDCCH − μPDSCH|. RAN1#107bis-e has produced the following conclusion:

Conclusion

Potential indications of UE capability related to a limited support of cross-carrier scheduling e.g. as a function of |μPDCCH − μPDSCH| can be discussed as part of the UE capability discussion.

R1-2200953: Cross-carrier scheduling is only supported for μPDCCH-μPDSCH≤3.

R1-2201352: In order to better support cross-carrier scheduling of the new SCS, i.e. 480 kHz and 960 kHz, the difference of SCS of cross-carrier scheduling should not be limited.

FL Note: Due to the discussion in earlier meetings and the conclusion reached in RAN1#107bis_e, these proposals are not further discussed in RAN1#108-e.

# Contribution Details

The following sections show extracted discussion and proposals from the contributions submitted to this AI, by a pure subjective decision by the FL.

## Topic A1: Blind Decoding Capability, Multi-slot monitoring and corresponding (X,Y) values

List of issues, proposals, and suggestions for handling in the email discussion phase.

### R1-2200953 (Huawei, HiSilicon)

Observation 1: In case that UE supports more than one (Xs,Ys) combinations AND the configured search space sets comply with more than one supported (Xs,Ys) combinations, determining the “active” Ys for the UE does not change UE monitoring behavior and, therefore, is not required. Question 2: In case that UE supports more than one (Xs,Ys) combinations AND the configured search space sets comply with more than one supported (Xs,Ys) combinations, is it required to determine the “active” value of Xs? For μ=5, only (4,1) and (4,2) are supported and they have the same BD/CCE budget. Therefore, if the active value of Xs needs to be determined, there is no ambiguity that it would be Xs=4. For μ=6, a majority of companies argued in RAN1 107b-e that if UE supports more than one (Xs,Ys) combinations AND the configured SS sets comply with more than one reported (Xs,Ys) combinations, the “active” value of Xs should be the one that corresponds to the (Xs,Ys) combination that 1) all configured SS sets comply with; and 2) results in the largest BD/CCE budget. For instance, if UE supports (4,1) and (8,1) for μ=6 and all Group (1) SSs of the UE are configured within a single slot every 8 slots, it is technically possible to determine active Xs=4 which means that BD/CCE budget of the Group (1) and potential Group (2) SS in the first 4 consecutive slots are 10/16, while the BD/CCE budget in the second 4 consecutive slots within the 8-slot period cannot be used for group (1) SS monitoring in the first 4 consecutive slots. If Xs=8 is assumed in the same scenario, the total BD/CCE budget for both Group (1) SS and Group (2) SS within those 8 slots is 20/32. Therefore, we also think that if UE supports more than one (Xs,Ys) combinations AND the configured search space sets comply with more than one supported (Xs,Ys) combinations, the “active” value of Xs should be the one that corresponds to the (Xs,Ys) combination that all configured search space sets comply with and results in the largest BD/CCE budget (or, equivalently, the largest Xs in the set of complied (Xs,Ys) combinations). Such a choice for active Xs additionally provide more flexibility in monitoring group (2) SS. We propose the following: Proposal 2: If the configured search space sets comply with more than one reported (Xs,Ys) combinations, determine the “active” Xs as the one that corresponds to the (Xs,Ys) combination that all configured search space sets comply with and results in the largest BD/CCE budget (or, equivalently, the largest Xs in the set of complied (Xs,Ys) combinations). - Support TP 2 for 38.213. For an RRC_IDLE UE or a UE during initial access, following observations can be made: - UE does not monitor PDCCH for μ=6. - For μ=5, even if UE supports both Xs,Ys=(4,1)and Xs,Ys=(4,2), UE does not monitor Group (1) SS and, therefore, the considered value for Ys is irrelevant. Further, for both Xs,Ys=(4,1)and Xs,Ys=4,2, maximum BD and CCE budgets are equal to 20 and 32, respectively. - For μ=5, UE mandatorily supports Xs,Ys=4,1. Therefore, in our view, for μ=5, RRC_IDLE UE or a UE during initial access (i.e., when monitoringCapabilityConfig is not provided) may monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to either of the combinations Xs,Ys=4,1 or Xs,Ys=4,2 as either or the combinations have exactly the same BD and CCE budget. As such, it seems to be more accurate to specify that, for μ=5, RRC_IDLE UE or a UE during initial access (i.e., when monitoringCapabilityConfig is not provided) monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to Xs=4. However, as a convention, it may be preferable to alternatively specify that, for μ=5, RRC_IDLE UE or a UE during initial access (i.e., when monitoringCapabilityConfig is not provided) monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs according to combinationXs,Ys=4,1 simply because combination Xs,Ys=4,1 is mandatorily supported. Further, it can be clarified for μ∈5,6 that an RRC_CONNECTED UE is expected to be provided with monitoringCapabilityConfig= r17monitoringcapability. Note that this does not mean that monitoringCapabilityConfig= r17monitoringcapability should always be present in PDCCH-Config as monitoringCapabilityConfig is an optional field “Need M” and its value is stored by the UE and upon receiving a message with the field absent, the UE maintains the current value. As such, we propose the following. - Support either of TP 1 or TP 1A for 38.213 to describe the default PDCCH monitoring behavior for μ=5.

### R1-2200988 (Futurewei)

In RAN1#107bis-e it was concluded that r17monitoringcapability it should be maintained for consistency purposes. In this case the specs should define the interpretation of this configuration parameter as well as the UE behavior when this parameter is not provided. We propose the following text change in Proposal 4: Change the TS 38.213 Clause 10.1 as follows (changes are underlined): “If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability. If a UE is not provided monitoringCapabilityConfig - For μ ∈ {0,1,2,3}, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. - For μ=5 the UE applies Xs,Ys=(4,1) and for μ=6 the UE applies Xs,Ys=(8,1) “

### R1-2201033 (InterDigital)

For 480 kHz, in addition to the agreed values of Xs (i.e., Xs=4 for 480 kHz and Xs=4 or 8 for 960 kHz), whether to support Xs=2 with Ys=1 was further discussed. The technical motivation was to allow more flexible network implementation with the identical absolute time window with Xs=4 with 960 kHz. In addition, it should be noted that the combination (2,1) is not a mandatory value, but an optional value, so that only capable UEs would support. As required discussion and corresponding specification impact can be minimized by reusing the values for 960 kHz with (4,1), support the combination (2,1) would be beneficial for better network flexibility and reducing latency for FR 2-2. Proposal 2: Support the combination (Xs,Ys) = (2,1) for 480 kHz. For 960 kHz, there was a proposal to reopen the discussion on the supported combination of (Xs,Ys). The proposal was to support only (Xs,Ys) = (8,1) and (8,4) without (4,2) and (4,1). The raised concern was whether BD/CCE limit for Xs=4 in 960kHz can satisfy Type-0 PDCCH monitoring BD/CCE number specified in the specification. However, as (4,2) and (4,1) are optional values which can be used when only available UEs report their capability, it is preferred to keep the existing agreement. Proposal 3: Do not reopen the discussion for the combinations of (Xs,Ys) for 960 kHz. - Support the agreed combinations of (8,1), (8,4), (4,2) and (4,1) for 960 kHz. In RAN1#107bis-e [2], how to determine a combination of (Xs,Ys) values for PDCCH monitoring when UE reports multiple applicable (Xs,Ys) combinations as UE capability. To resolve this issue, the following two possible solutions are suggested: - Alt #1: Introduce RRC signaling to indicate a combination of (Xs,Ys) to be used for PDCCH monitoring. If the parameter is absent, the UE uses a default combination, i.e., (4,1) for 480 kHz and (8,1) for 960 kHz. - Alt #2: If there are more than one combination complying with the SS configurations among the reported combinations of (Xs,Ys) , the UE monitors PDCCHs according to the complying combination (Xs,Ys) that is associated with the largest maximum number of BD/CCE budget (largest value of Xs). Although Alt #1 provides a simplest solution to resolve the multi-capability issue, however, Alt #1 requires additional RRC specification impact to introduce the essential signaling. On the other hand, Alt #2 resolves the issue without introduction of any RRC signaling. In addition, it should be noted that Alt #2 is a simple way to extend Rel-16 UE behavior for span based PDCCH monitoring. Having said that, supporting Alt #2 is preferred to avoid unnecessary RRC specification impact. Proposal 1: UE monitors PDCCHs according to the complying combination (Xs,Ys) that is associated with the largest maximum number of BD/CCE budget (largest value of Xs), if there are more than one combination complying with the SS configurations among reported combinations of (Xs,Ys) via UE capability. In RAN1#107bis-e [2], whether to capture default UE behavior of (Xs,Ys) was discussed. As the UE receives the parameter monitoringCapabilityConfig in PDCCH-Config via UE specific configurations not in PDCCH-ConfigCommon for cell-specific PDCCH configuration, the UE shall perform per-slot PDCCH monitoring for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1. The UE behavior is not aligned with the agreement in RAN1#107-e [1]. Having said that, it is preferred to capture the UE default behavior and adopt the following TP in the specification. Proposal 4: Adopt Text proposal #1 to reflect the UE default behavior with (Xs,Ys) = (4,1) and (8,1) for 480 kHz and 960 kHz, respectively.

### R1-2201086 (vivo)

Based on the above agreement, it is obviously that multiple (Xs, Ys) values may be reported by UE, i.e. - 480KHz PDCCH: (4, 1) mandatory, (4, 2) optional - 960KHz PDCCH: (8, 1) mandatory, (8, 4), (4, 2) and (4, 1) optional If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, how to determine the value of (Xs, Ys) for a scheduling cell should be specified. First, UE should determine a set of (Xs, Ys) values according to which the search space configuration meets the limitation, i.e. configured Group (1) SSs are located within Y consecutive slots within a slot group of X slots where the location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups. Among the set of (Xs, Ys) value, determine one or more (Xs, Ys) with the largest Xs value (i.e. the largest BD/CCE budget) first and then select (Xs, Ys) with the smallest Ys value. Proposal 1: Adopt TP1 in Appendix to implement the following: Select one (Xs, Ys) value from multiple (Xs, Ys) combinations reported by a UE according to the following steps: 1) Determine a set of (Xs, Ys) values according to which the search space configurations meets the limitation, i.e. configured Group (1) SSs are located within Y consecutive slots within a slot group of X slots; 2) Determine one or more (Xs, Ys) with the largest Xs value (i.e. the largest BD/CCE budget) first; 3) Select (Xs, Ys) with the smallest Ys value from the above selected (Xs, Ys) combinations. According to current spec in TS 38.213, PDCCH monitoring capability type is determined according to the following text: If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. In TS 38.331, monitoringCapabilityConfig is provided in PDCCH-Config that is configured per BWP. However, according to the above text “If a UE is provided monitoringCapabilityConfig for a serving cell”, it seems monitoringCapabilityConfig is provided per serving cell. For NR Rel-16, it could be understood that the value of monitoringCapabilityConfig should be the same for all BWPs in a serving cell. However, this is not possible for NR Rel-17. For example, assuming a case that there are two BWPs existing in one serving cell, i.e. one BWP with 120KHz SCS and the other BWP with 480KHz SCS, these two BWPs can’t be configured with the same PDCCH monitoring capability type according to current agreement. In this case, for the BWP with 120KHz SCS, monitoringCapabilityConfig may not be provided or provided with r15monitoringcapability in PDCCH-Config; for the BWP with 480KHz SCS, monitoringCapabilityConfig should be provided with r17monitoringcapability. Thus the above text “If a UE is provided monitoringCapabilityConfig for a serving cell” is not accurate anymore. Another issue is the default PDCCH monitoring capability type when monitoringCapabilityConfig is not provided for a BWP, which depends on the BWP SCS. Namely, when the BWP SCS is 15/30/60/120KHz, default PDCCH monitoring capability type is slot-based monitoring. When the BWP SCS is 480/960KHz, default PDCCH monitoring capability type is multi-slot-based monitoring. Besides, if a UE doesn’t indicate any combination Xs,Ys, the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6. Proposal 2: Adopt TP2 in Appendix to implement the following: For NR Rel-17 UEs, PDCCH monitoring capability is defined per BWP; For NR Rel-17 UEs, default PDCCH monitoring capability type is adapted to BWP SCS.

### R1-2201352 (CATT)

In RAN1#107-e meeting, it has been agreed that multiple combinations (X,Y) were supported, including (4,1),(4,2) for 480 kHz SCS and (8,1), (8,4), (4,2), (4,1) for 960 kHz SCS. One remaining issue is how to determine the combination (X,Y) for PDCCH monitoring capability if a UE indicates a capability with multiple combinations (X,Y) of 480 kHz/960 kHz. A potential solution is to reuse the (X,Y) combination determination method of Rel 16. For 480kHz/960kHz, if the UE indicate a capability to monitor PDCCH according to multiple combinations (X,Y) and a configuration of search space sets results to a separation of every two consecutive Y slots of slot groups is equal or larger than a value of X from multiple combinations (X,Y), the UE will monitor the PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD/CCE. For example, if the UE indicates a capability with combination (8,1) and combination (4,1) for 960kHz SCS, and a configuration of search space sets results in a minimal separation of 9 slots between two Group(1) SSs in two consecutive slot groups, the UE will monitor PDCCH according to the BD/CCE limit of combination (8,1). If a configuration of search space sets results in a minimal separation of 7 slots between two Group(1) SSs in two consecutive slot groups, the UE will monitor PDCCH according to the BD/CCE limit of combination (4,1). Proposal 2: For 480kHz/960kHz, if the UE indicates a capability to monitor PDCCH according to multiple combinations (X,Y) and a configuration of search space sets results in a separation of every two consecutive Y slots of slot groups is equal or larger than a value of X from multiple combinations (X,Y), the UE will monitor the PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD/CCE.

### R1-2201389 (ZTE, Sanechips)

Proposal 1: Further clarify in TS 38.213 that if the UE is not provided monitoringCapabilityConfig or not configured with any monitoring capability combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively. ## TP for TS 38.213 (marked in red) If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability For μ ∈ {0,1,2,3}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ ∈ {5,6}, if the UE is not provided monitoringCapabilityConfig or not configured with a combination Xs,Ys, the UE assumes Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6.. Moreover, for SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to more combinations Xs,Ys, different Xs,Ys may correspond to different BD/CCE budget, we need to determine the Xs,Ys combination. However, the determination of Xs for multi-slot PDCCH monitoring is different from that in Rel-16. There is no mandatory capability among the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1 and a UE can indicate a capability to monitor PDCCH according to one or more of the X,Y combinations in Rel-16 . Assuming that a similar rule as Rel-16 is extended to Rel-17 and the UE monitors PDCCH according to the combination Xs,Ys that is associated with the largest maximum number of BD/CCE for the SCS. If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations and one or more multiple combinations Xs,Ys can meet the configuration of search space sets to the UE for PDCCH monitoring on a cell, Xs = 4 and 8 slots for 480 and 960 kHz respectively are almostly always chosen since those two values are mandatorily supported and associated with the largest BD/CCE budget. However, in our understanding, the value of Xs should be configurable for more flexible operation in above 52.6 GHz band depending on UE capability. We need to consider introducing new RRC parameter to configure the Xs,Ys with smaller value of Xs for some scenarios such as low latency traffic and flexible scenarios. Also, the UE expects to monitor PDCCH according to the same combination Xs,Ys in every slot group on the active DL BWP of a cell. - Proposal 2: If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations and one or more multiple combinations Xs,Ys can meet the configuration of search space sets to the UE for PDCCH monitoring on a cell, we need to consider RRC signaling to configure the Xs,Ys for flexible scheduling. Moreover, the UE expects to monitor PDCCH according to the same combination Xs,Ys in every slot group on the active DL BWP of a cell.

### R1-2201471 (NTT DOCOMO)

In our understanding, the dropped USS can be different depending on which BD/CCE budget to apply. Thus, it is necessary to define the rule to avoid misunderstanding between UE and gNB on which BD/CCE budget to apply, especially for the case when UE supports multiple combinations of (Xs, Ys) and multiple combinations of them can meet the SS set configuration by gNB. For multi-slot PDCCH monitoring, BD/CCE budget is defined depending on the value of X, and Y is irrelevant. In that sense, it is sufficient to specify the rule to determine Xs when multiple (Xs, Ys) meet SS set configuration and which Y to apply can be implicitly indicated by SS set configuration. At the last RAN1 meeting, some companies pointed out that (X, Y) determination rule already exists for URLLC in Rel-16. More specifically, when UE indicates a capability to monitor PDCCH according to one or more of (X, Y) combinations and multiple combinations of them meet a SS set configuration to the UE, the UE monitors PDCCH according to the combination (X, Y) which associates with the largest maximum number of BD/CCE for span PDCCH monitoring. This rule can be expanded for multi-slot PDCCH monitoring, i.e., the UE monitors PDCCH according to the X which associates with the largest maximum number of BD/CCE. Proposal 1: If a UE supports multiple combinations of (Xs, Ys) and more than one combination comply with SS set configuration by gNB, the UE monitors PDCCH on a cell with according to the Xs which associates with the largest maximum number of BD/CCE.

### R1-2201542 (Spreadtrum)

As pointed out by many companies in the last RAN1 meeting, the determination of (X, Y) when multiple combinations are reported by UE is similar to Rel-16 span-based capability reporting. Since Rel-16 already specified a rule for this issue(as follows), the same handling method as Rel-16 URLLC can be used, i.e., the UE monitors PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD and CCE. While some other companies prefer a simple way to determine the active combination (X, Y), that is configure the actual (X,Y) value to the UE via RRC. 38.213 10.1 If a UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more of the multiple combinations X,Y, the UE monitors PDCCH on the cell according to the combination X,Y, from the one or more combinations X,Y, that is associated with the largest maximum number of MPDCCHmax,X,Y,μ and CPDCCHmax,X,Y,μ defined in Table 10.1-2A and Table 10.1-3A. The UE expects to monitor PDCCH according to the same combination X,Y in every slot on the active DL BWP of a cell.. From our perspective, similar rules from Rel-16 should be adopted, since the existing rule is workable here and we don’t see the need to introduce additional RRC signaling. Proposal 1: For the determination of (X,Y) in case of multiple supported combinations(X, Y), adopt similar rules from Rel-16, i.e., the UE monitors PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD and CCE.

### R1-2201689 (Intel)

The following TP is proposed to capture additional X value 2 for SCS 480kHz. ------------------------------ TP#1: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** Table 10.1-2B provides the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell. Table 10.1-2B: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 10; 20; 20; -; -. 6; -; 10; 10; 20; 20. *** Unchanged text is omitted *** Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell. Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 16; 32; 32; -; -. 6; -; 16; 16; 32; 32. *** Unchanged text is omitted ***. Proposal 1: - X=2 can be optionally supported for SCS 480kHz, which corresponds to combination (X, Y) = (2, 1). The BD/CCE budget for (2,1) can be half that of X=4 - Agree on TP 1 to capture additional X value 2 for SCS 480kHz Based on the agreed framework, UE may report the capability to support multiple combinations (X, Y) for SCS 480/960kHz. The configured search space sets for the UE must satisfy at least one supported combination (X, Y). - For SCS 480 kHz: (X,Y) = (4,1), (4,2), (2,1)? - For SCS 960 kHz: (X,Y) = (8,1), (8,4), (4,2), (4,1) Further, if the configured search space sets of the UE satisfy two or more supported combinations (X, Y), the UE needs to identify the active combination (X, Y). Since the supported maximum numbers of BD/CCE is only determined by value X, UE can determine a combination (X, Y) with larger X which enables larger maximum numbers of BD/CCE for PDCCH monitoring for better flexibility. If multiple potential combinations (X, Y) have same value X, the active combination (X, Y) could be determined as the combination (X, Y) with smallest value Y. The following TP is proposed to determine the active combination (X, Y). ------------------------------ TP#2: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** 10 UE procedure for receiving control information *** Unchanged text is omitted *** For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell is allowed by one or more of the multiple combinations Xs,Ys, the UE monitors PDCCH on the cell according to the combination Xs,Ys with largest Xs and smallest Ys. *** Unchanged text is omitted ***. Proposal 7: - For a UE capable of multiple combinations (X, Y), if the configured SS sets are aligned with more than one combination (X, Y), the active combination (X, Y) is determined that is associated with the largest X and smallest Y. - Agree on TP 2 to determine the active combination (X, Y).

### R1-2201735 (Ericsson)

Inform RAN2 that the value range for the existing parameter monitoringCapabilityConfig needs to be be extended to include the new value r17monitoringcapability, and that for 480 and 960 kHz SCS, the UE expects to be configured with this value. A note can be added to the RRC parameter spreadsheet to propose that RAN2 updates the field description of the parameter as follows: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1). When present, the UE expects to be configured with r17monitoringcapablity for 480 and 960 kHz SCS. Adopt TP#1 which definines the default PDCCH monitoring behavior for 480/960 kHz SCS when the parameter monitoringCapabilityConfig is absent ----------------------------------------- Text Proposal (TP#1) for 38.213, Section 10 ---------------------------------------- *** Unchanged text omitted *** If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability For μ ∈ {0,1,2,3}, if If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ ∈ {5,6}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) for μ =5 and Xs,Ys=(8,1) for μ =6 as in Tables 10.1-2B and 10.1-3B. *** Unchanged text omitted *** ----------------------------------------------------------- End Text Proposal ----------------------------------------------------------- In the previous meeting, there was discussion on how to handle the case if a UE indicates it is capable of operating with multiple (Xs,Ys) combinations for a given subcarrier spacing. For 480 kHz, the supported combinations are (4,1) and (4,2). For 960 kHz SCS, the supported combinations are (8,1), (8,4), (4,1), and (4,2). One approach is to agree on a rule for which combination the UE should assume for multi-slot PDCCH monitoring, and this may require some discussion. Alternatively, we think a far simpler approach would be to support an RRC parameter that enumerates all possible (Xs,Ys) combinations. The gNB can then configure the UE with the desired combination according to the use case. For example, a combination with a small value of Xs could be configured for more latency demanding applications. Conversely, a combination with a larger value of Xs could be configured when lower latency is not critical to achieve. Based on this we propose Support an RRC parameter for indicating the (Xs,Ys) combination the UE shall employ for multi-slot PDCCH monitoring based on that UEs indicated capability. The parameter is UE-specific and has the value range {'xs4ys1', 'xs4ys2', 'xs8ys1', 'xs8ys4'}. If the parameter is absent, the UE assumes the default value 'xs4ys1' if 480 kHz SCS is used or 'xs8ys1' if 960 kHz SCS is used.

### R1-2201765 (Apple)

Proposal 1: For the slot group size (X) it should be concluded that: - The configurable values for multi-slot PDCCH monitoring operation should be same as the reported X value(s). The UE is not expected to handle a scenario in which they are different, and a UE can report its monitoring capability for more than one (X,Y) combination. - For each SCS 480 kHz and 960 kHz, the minimum configurable multi-slot PDCCH monitoring periodicity is the smallest value X that a UE supports when reporting its PDCCH monitoring capabilities for the corresponding SCS and are UE specific. - Both statements may be either explicitly stated in the specification or as a conclusion in the Chairman’s notes. Proposal 2: There is a need to define the UE default behavior in the case that the parameter r17monitoringcapability is absent or in RRC_IDLE mode. This can be defined as follows: - For an active DL BWP of the serving cell where a UE has not been configured with a combination the UE applies for μ=5 and for μ=6. Proposal 3: (X,Y)=(2,1) for 480 kHz SCS is not supported

### R1-2201914 (Xiaomi)

It is already agreed in R1#107 meeting that BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS. Related TP can be as follows, TP#2 for TS 38.213 Clause 13 ============================= Unchanged part omitted ========================================= 13 UE procedure for monitoring Type0-PDCCH CSS sets If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Tables 13-10A, 13-10B and 13-10C for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. *<omitted text>*. For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, for FR1 and FR2-1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two consecutive slots starting from slot . For FR2, UE monitors PDCCH in the Type0-PDCCH CSS set over two slots, slot and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz. For SS/PBCH block with index , the UE determines an index of slot as that is in a frame with system frame number (SFN) satisfying if , or in a frame with SFN satisfying if . and are provided by Tables 13-11 and 13-12, and based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots and and is the first symbol index provided by Tables 13-11 and 13-12. For operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214]. For FR1 and FR 2-1, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two consecutive slots starting from slot n0 include the associated Type0-PDCCH monitoring occasions. For FR2, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two slots, slot n0 and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz, include the associated Type0-PDCCH monitoring occasions. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. M and O are provided by Table 13-11, and μ∈{0, 1} based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots n0 and n0+1 is the first symbol index provided by Table 13-11. The UE does not expect to be configured with M=1/2, or with M=2, when NSSBQCL=1. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over one slot with Type0-PDCCH CSS set periodicity equal to the periodicity of SS/PBCH block. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, if the active DL BWP is the initial DL BWP, the UE is expected to be able to perform radio link monitoring, as described in Clause 5, and measurements for radio resource management [10, TS 38.133] using a SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set. For a SS/PBCH block with index , the UE determines the slot index and based on parameters provided by Tables 13-13 through 13-15. ============================= Unchanged part omitted =========================================

### R1-2202005 (Samsung)

Similar to Rel-16 span-based PDCCH monitoring, a UE can indicate multiple Xs,Ys combinations for multi-slot PDCCH monitoring in FR2-2, then the remaining issue is how to determine the combination that the UE will choose from the multiple indicated Xs,Ys combinations. A similar approach as Rel-16 span-based PDCCH monitoring can be reused, wherein the maximum values on the BD and non-overlapping CCE are replaced by MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, respectively. Proposal 1: If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, for a configuration of search space sets, a UE monitors PDCCH on the cell based on the following steps: - Step 1: Choose applicable Xs,Ys combinations based on the configuration of search space sets; - Step 2: Choose a Xs,Ys combination, from the applicable Xs,Ys combinations, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ; - Adopt TP#1 for TS 38.213. The discussion on multi-slot PDCCH monitoring was all based on UE capability, and one remaining issue is the UE behavior before the UE capability is reported. Since it has been agreed to not support single slot based PDCCH monitoring, a natural outcome is to support multi-slot PDCCH monitoring only before the UE capability is reported, and the only remaining issue which combination Xs,Ys should the UE choose for such multi-slot based PDCCH monitoring. Since we anyway need to specify the selection of combination Xs,Ys when multiple combinations Xs,Ys are reported, a simply way could be reuse the same scheme before the UE capability is reported. Proposal 2: Before a UE indicates a capability to monitor PDCCH, the UE monitors PDCCH on the cell based on the following steps: - Step 1: Choose applicable Xs,Ys combinations based on the configuration of search space sets; - Step 2: Choose a Xs,Ys combination, from the applicable Xs,Ys combinations, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ; - Adopt TP#2 for TS 38.213. There is a FFS on whether to support Xs,Ys=(2, 1) for 480 kHz SCS, as one of the reported UE capability. The intention of supporting a smaller value of Xs, comparing to the mandatory UE capability with Xs=4 for 480 kHz SCS, is to allow flexible network configuration on the SS sets. Also, other than adding the corresponding values of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ in the table, no other spec impact is expected, then it’s beneficial to support extra values of Xs for better flexibility. Proposal 3: Support Xs,Ys=(2, 1) for 480 kHz SCS. - Adopt TP#3 for TS 38.213.

### R1-2202190 (Sharp)

In RAN1#107-e meeting, the agreement for BD drop in multi-slot monitoring is achieved and it says “The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group”. Then, the slot group is just described as “group of Xs slots according to combination (Xs, Ys)” or “group of Xs slots for a corresponding combination (Xs, Ys)” in TS38.213. When a UE reports multiple (Xs, Ys) combinations, there is no determination rule such that which value of Xs is used for BD dropping per group. Therefore, we should make it clear what is meant by the statements in “according” and “corresponding”. Proposal 1: Introduce a new RRC parameter configuring Xs for allocation of PDCCH candidates when UE reports multiple (Xs, Ys) combinations. Proposal 2: Adopt Text proposal #1.

### R1-2202273 (Panasonic)

As mentioned in section 1, the current 38.213 describe the following behavior regarding Group (1) SSs monitoring: “If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, …” Our view is the above description is not sufficient to reflect the earlier agreement that BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots. The reason is that the spec only describes that what UE “can” do for Ys slots, but does not describe any UE behavior for slots outside Ys slots. Without such restriction, it implies UE still needs to monitor slots outside outside Ys slots for Group (1) SSs if configured by gNB via search space configuration. Then the whole multi-slot monitoring mechanism of Xs,Ys becomes meaningless. Therefore, we propose the following changes to 38.213: Proposal 1: To capture the earlier agreement that BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots in 38.213, adopt the following TP (with modifications highlighted in yellow). TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots and the UE is not expected to monitor any other slot of the Xs slots than Ys slots for PDCCH of the above-mentioned types, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. .... The earlier agreement that “the location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups” cannot be true when new monitoring occasion is configured or indicated to the UE. If this case is not allowed, the system cannot operate correctly. Considering the TDMed-beam transmission of CSS (of Group (1)), UE may need to monitor a different slot for CSS when UE’s serving beam is changed. However, for USS, gNB can simply change the serving beam without changing the MO location since USS MO is unicast to the UE. In this case, it can happen that CSS MO becomes far away from the USS MO of the new serving beam such that Ys cannot include both CSS and USS MOs anymore. The following Fig.1 illustrates above case more concretely. As shown, in the first slot group (UE is served by yellow beam), the location of Ys is the first slot, covering USS MO and CSS MO of the yellow beam. When UE moves from the coverage of yellow beam to green beam in the second slot group, the corresponding CSS MO location for the UE has to be changed as well because the previous CSS MO would still be used for other UEs covered by the yellow beam. Consequently, the new CSS MO for the UE becomes outside the Ys. According to the current spec, the UE may not receive such CSS because the location of Ys cannot be changed. Fig.1 Issue of fixed location of Ys due to beam switching To fix the above issue, the location of Ys should be allowed to change if new MO is configured or indicated to the UE. Considering the fact that Group (1) SSs include both CSS and USS, CSS should be prioritized over USS in order not to miss important control information. In other words, location of Ys should follow the new configured/indicated MOs of CSS (of Group (1)). Then USS MOs can be further included in Ys from USS with lower to higher indices. Certain USS MO would be dropped if it cannot be included in Ys . To reflect the above, the following TP is suggested: Proposal 2: In case that MO of Group (1) CSS is changed, the location of Ys within Xs can be adapted accordingly to include all CSS MOs (of Group(1) SS) that are monitored by UE. Adopt the following TP to 38.213 (with modifications highlighted in yellow). TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots until new monitoring occasion of Type1-PDCCH CSS set provided by dedicated higher layer signalling, or of Type3-PDCCH CSS sets is configured or indicated to the UE. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively..

### R1-2202336 (LG)

Among the supported combinations, X=8 for 960 kHz and X=4 for 480 kHz are the mandatorily supported X values for multi-slot PDCCH monitoring operations. These values can maintain the PDCCH monitoring burden of the UE at a level similar to that of the 120 kHz SCS since the slot-group length and BD/CCE limit for them are the same as for 120 kHz SCS. In addition, for 960 kHz SCS, there is an optionally supported X, i.e., X=4, which can allow more diverse and flexible monitoring operation than what the mandatory X can do. However, for 480 kHz SCS, whether additional X other than X=4 is supported has not been decided yet. For a flexible multi-slot monitoring operation like 960 kHz, support for additional optional X may be required. With X=2 for 480 kHz, a possible combination (X,Y) can be (2,1). At the last meeting, there were some concerns about (X,Y)=(2,1) combination support for 480 kHz SCS. One concern mentioned for example is that monitoring based on X=2 may greatly increase power consumption and UE implementation complexity compared to X=4. However, (X,Y)=(2,1) for 480 kHz has the similar monitoring behavior as (X,Y)=(4,1) or (4,2) for 960 kHz which RAN1 have already decided to support. Therefore, it is not expected that the complexity or power consumption of the UE additionally increases due to (2,1)-based monitoring. Rather, it may have an advantage of being able to schedule flexible monitoring operation for an aggressive UE for 480 kHz, by supporting (2,1) combination. Another concern is about BD/CCE budget for (X,Y)=(2,1) combination. Assuming that the BD/CCE budget for (X,Y)=(2,1) is half that of (X,Y)=(4,1), the raised issue is that it is not feasible to monitor Type-0 PDCCH twice during 4 slots. However, it is not a valid issue since only one MO for Type-0 CSS, according to the previous agreement, can be configured per 4 slots for 480 kHz SCS. Furthermore, when (X,Y)=(2,1) is supported as an optional combination for SCell, there will be no additional issue due to the BD/CCE budget. In summary, there seems to be no critical issue from technical point of view to support an optional X=2 for 480 kHz SCS. However, monitoring based on the optional X can allow more diverse/flexible monitoring operation than what the mandatory X can do. It should be noted that X=2 for 480 kHz and X=4 for 960 kHz share the same absolute time and their BD/CCE budget could be the same. These mean that the UE supporting X=4 of 960 kHz can support X=2 for 480 kHz without additional burden. In this regard, we support (X,Y)=(2,1) as an optional combination for 480 kHz SCS. Proposal #1: Support (X,Y)=(2,1) as an optional combination for 480 kHz SCS. For multi-slot monitoring operation, the length X of the slot-group becomes the criterion for BD/CCE budget management. For example, for 960 kHz SCS, the UE has the BD budget of 20 per X=8 slots or [10] per X=4 slots. Since the slot-based monitoring is not supported for 480/960 kHz SCS, multi-slot monitoring becomes a mandatory monitoring operation including IDLE mode. Since UE assumption on the (X,Y) can affect BD/CCE budget handling within X slots, the default (X,Y) combination should be specified in the specification. At the previous meeting, RAN1 agreed to have a mandatorily supporting (X,Y) combinations for 480 kHz and 960 kHz SCS, i.e., (4,1) for 480 kHz and (8,1) for 960 kHz. These can be the default (X,Y) combination for each SCS, respectively. We propose to adopt the following text proposal to Clause 10 in TS 38.213 specification. Proposal #3: Adopt the following text proposal in TS 38.213 Clause 10, to specify the default (X,Y) combination for multi-slot PDCCH monitoring. ============ Start of TP for TS 38.213 [1] ================== 10 UE procedure for receiving control information << Other parts are omitted >> If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig for μ=0, 1, 2, or 3, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ=5 or 6, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot group of Xs slots according to combination Xs,Ys=(4,1) for μ=5 or Xs,Ys=(8,1) for μ=6. << Other parts are omitted >> ============ End of TP for TS 38.213 ==================. When the UE reports multiple (X,Y) combinations for multi-slot monitoring, the UE should determine the (X,Y) combination based on the configured SS set configuration parameters (e.g., periodicity, offset, …). However, it may be ambiguous which (X,Y) the UE should assume to check BD/CCE budget if the configured monitoring occasions are aligned with multiple (X,Y) combinations. For example, when a UE reports (4,1) and (8,1) as capable (X,Y) combinations for 960 kHz and the configured monitoring occasions may be aligned with both (4,1) and (8,1) combinations, the UE should select an active (X,Y) combination to handle the BD/CCE budget. As a similar handling as Rel-16 URLLC, the active (X,Y) can be simply selected as that with the largest BD/CCE budget among the reported combinations which match the configured MOs. In multi-slot monitoring behavior, the largest X have the largest BD/CCE budget, so this rule is equivalent to selecting the combination with the largest X. Among the (X,Y) combinations with the largest X, it may be reasonable to select one with the smallest Y. Proposal #4: In the multi-slot monitoring, when the monitoring occasions corresponding to multiple (X,Y) combinations are configured, the UE should monitor PDCCH candidates based on the (X,Y) combination corresponding to the largest X (and the smallest Y that can be combined therewith) among the reported combinations that match the monitoring occasions.

## Topic A2: Search Space Configuration/Enhancement

### R1-2200953 (Huawei, HiSilicon)

Observation 3: For Type0/0A/2-PDCCH CSS in Group (2) SS set, if searchSpaceID is set to 0, the configured periodicity in monitoringSlotPeriodicityAndOffset-r17 for the search space is irrelevant as the UE obtains the monitoring occasions based on the associated SSBs. Observation 4: For Type0/0A/2-PDCCH CSS with searchSpaceID not equal to zero and Type1-PDCCH CSS, if periodicities is restricted to multiple of Xs slots, then, only following periodicities cannot be used: - For μ=5, periodicities of A1 = {1,2,5,10} slots - For μ=6, periodicities of A1 = {1,2,5,10} slots if both Xs={4,8} are supported and periodicities of A2 = {1,2,4,5,10,20} slots if only Xs=8 is supported. As, in absolute time, A1 = {1,2,5,10} slots for μ=5 is equal to B1 = {0.25, 0.5, 1.25, 2.5} slots for μ=3, and A2 = {1,2,4,5,10,20} slots for μ=5 is equal to B2 = {0.125, 0.25, 0.5, 0.625, 1.25, 2.5} slots for μ=3, none of the restricted periodicities in A1 or A2 is a supported value (in an absolute time) for a search space periodicity in Rel-15/16. In our view, the simplicity of using the same set of supported periodicities for both Group (1) SS and Group (2) SS along with above Observation 3 and Observation 4, strongly justify the following: Proposal 4: In addition to the configured periodicity for Group (1) SSs, the configured periodicity for Group (2) SSs is also restricted to be an integer multiple of Xs slots. Further, let us take a look at the offset in monitoringPeriodicityAndOffset-r17. The offset in monitoringPeriodicityAndOffset in Rel-15/16 indicates the starting slot index of monitoring occasions. However, in Rel-17, the indication of starting slot within a slot group is already provided by monitoringSlotsWithinSlotGroup-r17 and the offset in monitoringPeriodicityAndOffset-r17 needs only to indicate the starting slot index of the Xs slot group containing monitoring occasions. For example, suppose that the MOs start in the 5-th slot, as shown in Figure 1. In this case, the offset in monitoringPeriodicityAndOffset-r17 can be 4 slots while monitoringSlotsWithinSlotGroup-r17 = 0110000. e.g., monitoringPeriodicityAndOffset-r17 = (sl32, 4 slots) and duration-r17 = 12 slots offset = 4 slots duration = 12 slots = 3 slot groups periodicity = 32 slots Xs=4 slots 3 OSs 3 OSs Xs=4 slots Xs=4 slots … Xs=4 slots … monitoringSymbolsWithinSlot = 00111 00111 0000 Figure 1 An example of search space configuration for a UE that supports multi-slot monitoring for 480 kHz SCS with Xs,Ys=(4,2) with monitoringSlotsWithinSlotGroup-r17 = 0110000 Proposal 7: The configured offset in monitoringPeriodicityAndOffset-r17 is restricted to be an integer multiple of Xs slots and the invalid entries can be removed. As for the definition of duration-r17, it is worth noticing that, unlike Rel-15/16, the search space is not necessarily consist of a consecutive number of slots in a period (occasion) because the group (1) SS in only located within Ys consecutive slots of the Xs slot group. Therefore, a valid definition of duration-r17 is the total number slots in consecutive groups of Xs slots that a search space lasts in each period. Proposal 5: Support the following definition for duration-r17: - Duration-r17 is the total number of slots in consecutive groups of Xs slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the monitoringPeriodicityAndOffset-r17, for μ∈{5,6}. - Use the above definition of duration-r17 in RRC parameter list - Adopt TP3 for 38.213 We support the WA with a slight modification: Since the size of monitoringSlotsWithinSlotGroup-r17 is set to 8 while the size of slot group is either 4 or 8, only the Xs leftmost bits of monitoringSlotsWithinSlotGroup-r17 represent a slot in a slot group. Proposal 6: Confirm the working assumption on monitoringSlotsWithinSlotGroup-r17 with the following modification in the second sub-bullet: - Each of the Xs leftmost bits in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group. Adopt TP4 for 38.213. Given our earlier proposals (brought also above for the ease of reading) regarding the periodicity and offset values in monitoringPeriodicityAndOffset-r17 and the fact that Xs is only limited to 4 and 8 slots, every supported periodicity is an integer multiple of 4 slots. Further, for any periodicity of 4N slots, the permissible value range of offset is given by (0,3,…,4N-1). We propose to clarify this to RAN2 in the RRC parameter list as follows: Proposal 8: Supported value range for periodicity and offset in monitoringPeriodicityAndOffset-r17 is provided below and is included in RRC parameter list to RAN2: monitoringSlotPeriodicityAndOffset-r17 CHOICE { sl4 INTEGER (0), sl8 INTEGER (0,3), sl16 INTEGER (0,3..15), sl20 INTEGER (0,3..19), sl32 INTEGER (0,3..31), sl40 INTEGER (0,3..39), sl64 INTEGER (0,3..63), sl80 INTEGER (0,3..79), sl128 INTEGER (0,3..127), sl160 INTEGER (0,3..159), sl320 INTEGER (0,3..319), sl640 INTEGER (0,3..639), sl1280 INTEGER (0,3..1279), sl2560 INTEGER (0,3..2559), sl5120 INTEGER (0,3..5119), sl10240 INTEGER (0,3..10239), sl20480 INTEGER (0,3..20479) } Proposal 9: Confirm the WA in RAN1 107-e, that is, support only search space set group switching processing capability 1 with minimum Pswitch values 40/160/320 symbols for 120/480/960 kHz SCS. Since the BD/CCE budget of multi-slot PDCCH monitoring is defined per slot-group, it would be easier for a UE to monitor according to the BD/CCE budget if SSSG switching occurs at a slot-group boundary rather than in the middle of a slot-group. Therefore, it is beneficial to align the SSSG switching boundary with the boundary of a slot-group. Proposal 10: For multi-slot PDCCH monitoring for 480/960 kHz SCSs, a boundary of SSSG switching is always aligned with a boundary of a slot group. - Adopt TP 3 for 38.213. It has been discussed in RAN1 107b-e whether SSSG switching is only supported between SSSGs with the same Xs,Ys PDCCH monitoring combination. We do not see why the two different SSSGs have to support the same Xs,Ys combination. In our view, this is an unnecessarily conservative approach to guarantee that the distance between two consecutive Ys SS slots on either side of the switching boundary is not less than Xs. For instance (4,1) can switch to (8,1) as long as the distance between the last monitoring slot in (4,1) to the first monitoring slot in (8,1) is not less than 4. This can be easily supported by gNB implementation. Alternatively, UE can just drop monitoring the first SS in the “switched to” SSSG if the distance between two consecutive Ys SS slots on either side of the switching boundary is not less than Xs. Proposal 13: SSSG switching is supported between SSSGs that correspond to the same or different Xs,Ys PDCCH monitoring combinations. In power saving enhancements WI, the value range for both SSSG switching timer searchSpaceSwitchTimer-r17 and PDCCH skipping duration PDCCHSkippingDuration was agreed to be {1, 2, 3, …,160, 240, 320,400, 480, 640, 800} slots for120 kHz. Following other timeline related issues, it makes sense to scale the supported values of searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 120 kHz by 4 and 8 to obtain the supported values respectively for 480 kHz and 960 kHz. Proposal 11: In unit of slots, the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz are respectively 4x and 8x of their supported values for 120 kHz. Moreover, in addition to Rel-16 SSSG switching parameters, SSSG switching parameters searchSpaceSwitchTimer-r17 and searchSpaceGroupIdList-r17 were introduced in power saving enhancements WI. It is clear that when Rel-16 SSSG switching parameters are configured, a UE will perform the Rel-16 SSSG switching mechanism. Similarly, when Rel-17 SSSG switching parameters are configured, a UE will perform the Rel-17 SSSG switching mechanism. From our view, it does not make too much sense to allow a UE to perform both SSSG switching mechanisms simultaneously. Therefore, the gNB should not configure both Rel-16 and Rel-17 SSSG switching parameters at the same time. Proposal 12: A UE does not expect to be configured with Rel-16 SSSG switching parameters (such as searchSpaceSwitchTimer and SearchSpaceSwitchTrigger) and Rel-17 SSSG switching parameters (such as searchSpaceSwitchTimer-r17 and searchSpaceGroupIdList-r17) per cell simultaneously.

### R1-2200988 (Futurewei)

One remaining issue to be clarified in the above agreement is the “offset”. More precisely what is the offset basis? Following the existing specs we propose to maintain the same definition of the offset as presented in the TS 38.213. Proposal 2: In TS 38.213, Clause 10.1 add the following text (the proposed addition is underlined): “A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf. Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots. For µ = {5,6} ks and Ts are integer multiples of the slot group duration.” We propose to maintain the existing duration definitions for the multi-slot PDCCH monitoring where the duration it represents the time duration of the search space existence as an integer multiple of the basic monitoring pattern, which is the slot group. Proposal 1: In TS 38.213, Clause 10.1 add the following text (the proposed addition is underlined) “- a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration” to “- a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration. For µ = {5,6} the duration is an integer multiple of the slot group duration.” In RAN1#107bis-e it was agreed to introduce the RRC parameter monitoringSlotsWithinSlotGroup-r17. The design of this parameter was left as WA that needs to be confirmed. We support the WA confirmation. Proposal 3: Confirm WA regarding the parameter monitoringSlotsWithinSlotGroup-r17 design. Regarding the second bullet in above agreement, we prefer to allow full flexibility of the Group (2) SS. Besides, the necessity of further limitations on the number of spans per slot/slot group is not clear to us. Therefore, we prefer not to introduce other limitation for Group (2) SSs. Proposal 3: Do not introduce other limitation for Group (2) SSs monitoring.

### R1-2201033 (InterDigital)

In RAN1#107bis-e [2], RRC parameter duration-r17 was agreed with the value range {8, 12, …, 20476} slots. The configured duration should be an integer multiple of Xs slots at least for Group (1) SS. However, there is an FFS bullet on whether there’s a need to revise the definition of duration. In our company view, we couldn’t find any issues to revise the existing definition of duration. Proposal 5: Keep the agreed definition of duration and revise the agreement without the FFS bullet. In RAN1#107bis-e [2], RRC parameter monitoringSlotsWithinSlotGroup-r17 was introduced to indicate a monitoring slot position in a slot group. For example, UE can monitor PDCCH within a slot of a multi-slot group where a corresponding bit indicates. In our company view, we couldn’t find any serious issues. Having said that, we propose to confirm the working assumption. Proposal 6: Confirm the working assumption on monitoringSlotsWithinSlotGroup-r17. In RAN1#107-e [1], minimum Pswitch value for search space set group switching were discussed with the following issues: - Whether to support multiple capabilities (capability 1 and capability 2 as in Rel-16). - In Rel-16, UE cap report whether the UE supports search space set group switching Capability-2 via searchSpaceSwitchCapability2-r16. - Value(s) for 120 kHz - In Rel-16, Pswitch values for capability 2 are based on SPS release timeline. Having said that, 25 symbols were suggested for μ=3 with capability 2. - Whether to scale the value(s) of 120 kHz by 4 and 8 times for 480 and 960 kHz, respectively. - Most of UE capability values were determined based on scaled values by 4 or 8 times of 120 kHz value in NR 52-71. - Whether to consider additional margin. Based on the discussion, the following values with only one capability (i.e., UE processing capability 1) was agreed as a working assumption. μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. As the values do not have any serious issues, it is preferred to agree the working assumption as it is. Proposal 7: Confirm the working assumption on the minimum Pswitch value.

### R1-2201266 (OPPO)

The difference for FR2-2 is that the UE does not monitor PDCCH in all the slots configured by the duration-r17 parameter. This necessitates to introduce another parameter, i.e., monitoringSlotsWithinSlotGroup-r17, to indicate which of the slots to be monitored within the slots configured by duration-r17 parameter. However, we think that to make the search space configuration work, the restriction of the length of duration-r17 is integer of Xs slot should be modified. This because the Xs value is directly linked to the 8-bit of monitoringSlotsWithinSlotGroup-r17. The bit position should be aligned with Xs value in order for the UE to derive the monitored slots from a one-to-one mapping indication. If the Xs value is not equal to 8, the UE does not know how to interpret the bit-map. Given that for 480kHz SCS, UE only supports Xs=4 but for 960kHz SCS, UE may support Xs=4 or 8, we suggest that the duration-r17 is defined in a virtual slot group length of Xs=8, so that the duration-r17 should be an integer number of Xs=8. Thus the monitoringSlotsWithinSlotGroup-r17 can indicate the monitored slot based on Xs=8 slot basis and the corresponding configuration examples as shown in Fig. 2. Proposal 1: the value configured by duration-r17 should be an integer number of Xs=8 and make the previous working assumption as agreement.

### R1-2201352 (CATT)

The difference between option 1 and option 2 is how to use the monitoringSlotsWithinSlotGroup-r17 for 480kHz. It was agreed in the RAN1#107b-e meeting that the configured duration for 480 kHz is restricted to be an integer multiple of 4 slots at least for Group(1) SS. If option 2 is adopted and the value of duration is 12, the number of consecutive 8-slots group will be 1.5(12/8) that is not a integer. This may bring complexity to the definition of the specification. Proposal 1：For search space set configuration of multi-slot PDCCH monitoring, the following definition is suggested: - Offset: the value of offset can be configured as one of the value in {0 .. Xp-1} - Duration: It is used to configure the number of consecutive slots that a SearchSpace lasts in every periodicity. For 480kHz SCS, if the value of duration-r17 is N, the number of consecutive 4-slots group is N/4 that a SearchSpace lasts in every periodicity. For 960kHz SCS, if the value of duration-r17 is N, the number of consecutive 8-slots group is N/8 that a SearchSpace lasts in every periodicity. - monitoringSlotsWithinSlotGroup-r17: For 480kHz SCS, only the first four most significant(left) bit are applicable and can be used to configure the slot for PDCCH monitoring that a SearchSpace lasts in every 4-slots. For 960kHz SCS, all the 8 bits are applicable and can be used to configure the slot for PDCCH monitoring that a SearchSpace lasts in every 8-slots. The search space group set switching was introduced in Rel-16 NR-U with 15 kHz SCS, 30 kHz SCS and 30 kHz SCS for dynamic switching between different search spaces. Before the gNB obtains the COT, the frequent monitoring enables the gNB to transmit DCI as soon as possible if gNB’s LBT is successful. However, frequent monitoring is not conducive to power saving of the UE during the COT. When the search space group set switching is configured, the gNB can indicate to UE switching between a search space with long periodicity and a search space with short periodicity to meet different scheduling requirements. In RAN1#107bis-e discussion, it is generally the common understanding that the legacy SSSG switching mechanism should be reused for the 120 kHz SCS, 480 kHz SCS and 960 kHz SCS in 60GHz NR-U. Also the following was agreed Conclusion The SSSG switching timer is in units of slots.. The next issue is the switching boundary. To simplify the behavior and reduce specification effort, it is suggested to align the boundary with the boundary of a slot group. Proposal 4: The SSSG switching boundary is aligned with the boundary of a slot group.

### R1-2201389 (ZTE, Sanechips)

For monitoringSlotPeriodicityAndOffset-r17 and duration-r17, according to the agreement made in RAN1#107bis e-meeting, other appropriate periodicity values { sl32, sl64, sl128, sl5120, sl10240, sl20480} are added in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity/duration for multi-slot PDCCH monitoring in FR2-2 are an integral multiple of slot groups. The configuration of periodicity/duration is intended not only for Group (1) SSs but also for Group (2) SSs, so we suggest changing the wordings in the agreement “The configured periodicity/duration at least for Group (1) SSs is restricted to be an integer multiple of Xs slots” to “The configured periodicity/duration is restricted to be an integer multiple of Xs slots for both Group (1) SSs and Group (2) SSs”. Proposal 4: The configured periodicity in monitoringSlotPeriodicityAndOffset-r17 and duration in duration-r17 are restricted to be an integer multiple of Xs slots for both Group (1) SSs and Group (2) SSs. Moreover, the current definition of duration is “Number of consecutive slots that a SearchSpace lasts in every occasion”. For slot-group based PDCCH monitoring, duration is restricted to be an integer multiple of Xs of a slot group and its unit is slot based on the agreement made in RAN1#107bis e-meeting, we propose to clarify the definition of duration-r17 as “number of slots that a SearchSpace lasts for a couple of consecutive slot groups in every occasion”. If duration-r17 is absent, the UE applies the value 4 slots, except for DCI format 2_0. Proposal 5: Further clarify the definition of duration-r17 as: Number of slots that a SearchSpace lasts for a couple of consecutive slot groups in every occasion. If duration-r17 is absent, the UE applies the value 4 slots, except for DCI format 2_0. Basically, we are fine with introducing a new parameter monitoringSlotsWithinSlotGroup-r17 to indicate which slots have monitoring occasions within a slot group for SCS 480/960 kHz. However, we think the configuration of parameter monitoringSlotsWithinSlotGroup-r17 should align with the configuration of offset in monitoringSlotPeriodicityAndOffset-r17. The value range for the offset O is {0 .. Xp-1} slots for each periodicity value Xp and its unit is slot. The offset indicates the slot offset between PDCCH monitoring occasion and frame boundary according to the current specification in TS 38.213 as follows: - a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf. Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots.. In other words, the slot offset between the first slot in the slot group indicated in monitoringSlotsWithinSlotGroup-r17 for a PDCCH monitoring occasion and subframe boundary is equal to the offset indicated in monitoringSlotPeriodicityAndOffset-r17. Proposal 6: If new parameter monitoringSlotsWithinSlotGroup-r17 is introduced, the offset indicated in monitoringSlotPeriodicityAndOffset-r17 is equal to the slot offset between the subframe boundary and the first slot in the slot group indicated in monitoringSlotsWithinSlotGroup-r17 for a PDCCH monitoring occasion. Proposal 7: Confirm the working assumption: Support only search space set group switching processing capability 1 with the following values μ; Minimum PswitchPswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. The SSSG switching boundary is aligned with the slot boundary in Rel-15/16, the UE switches from one SSSG to another SSSG at the first slot after at least Pswitch symbols. Similar rule can be extended for multi-slot PDCCH monitoring in Rel-17, the SSSG switching boundary, i.e. the time of applying the switching, is aligned with the boundary of a slot group to avoid increasing complexity. Proposal 8: For multi-slot PDCCH monitoring in Rel-17, the SSSG switching boundary, i.e. the time of applying the switching, is aligned with the boundary of a slot group to avoid increasing complexity. Moreover, even though we think SSSG switching between two different capabilities of multi-slot PDCCH monitoring for 480/960kHz should be supported, in our understanding, the UE expects to monitor PDCCH according to the same combination Xs,Ys in every slot group on the active DL BWP of a cell. Therefore, we can conclude that SSSG is only supported between SSSGs that have the same Xs,Ys PDCCH monitoring capability on the active DL BWP of a cell. Proposal 9: Conclude that SSSG is only supported between SSSGs that have the same Xs,Ys PDCCH monitoring capability on the active DL BWP of a cell.

### R1-2201471 (NTT DOCOMO)

According to the agreement, an RRC parameter monitoringSlotsWithinSlotGroup-r17 is newly supported for multi-slot PDCCH monitoring which indicates monitored slot(s) within a slot group. However, given that monitoringSlotsWithinSlotGroup-r17 indicates monitored slot(s), the definition of duration-r17 needs to be revisited as captured as FFS since the RRC parameter duration in the current specification also indicates monitored consecutive slot(s) in every monitoring occasion indicated via monitoringSlotPeriodicityAndOffset. In our understanding, it makes more sense if duration-r17 is revised to indicate slots where monitoringSlotsWithinSlotGroup-r17 is applied in every monitoring occasion. The value range of duration-r17 was agreed as {8, 12, …, 20476} which are scaled from the values in the current specification based on the supported SCSs in FR2-2. According to the current specification, it is also specified that a UE applies the value 1 slot if duration is absent. We believe this default value should also be scaled in the same manner as for the exact values for duration-r17, i.e., a UE applies the value 4 slots if duration-r17 is absent. As per the above discussion, duration-r17 indicates multiple of Xs slots that monitoringSlotsWithinSlotGroup-r17 is applied. In that sense, the starting slot of duration-r17 should be aligned with slot group boundary, thus, the periodicity and offset also should be the multiple of Xs slots. Furthermore, it is preferable to support unified configuration principle between Group(1) SSs and Group(2) SSs. Therefore, both monitoringSlotPeriodicityAnd Offset-r17and duration-r17 should be an integer multiple of Xs slots regardless of SS type. Figure 1 shows an example as for SS set configuration with monitoringSlotPeriodicityAndOffset-r17, duration-r17 and monitoringSlotsWithinSlotGroup-r17 when a UE reports the support of (Xs, Ys)=(4, 2). Fig.1: SS set configuration for multi-slot PDCCH monitoring. Based on the discussion, we make the following proposal. Proposal 3: The following agreement at RAN1#107bis-e meeting should be supported with revision in red and the working assumption should be confirmed. For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The configured offset is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} - If this field is absent, a UE applies the value 4 slots. - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - This field indicates the number of consecutive slots that a SearchSpace lasts monitoringSlotsWithinSlotGroup-r17 is applied in every occasion, i.e., upon every period as given in the monitoringSlotPeriodicityAndOffset-r17. - FFS: need to revise the definition of duration - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs. As per the discussion in AI 8.2.2, most companies agreed that SSSG switching and PDCCH skipping feature introduced in Rel-17 UE power saving enhancement WI can be extended for NR operation in FR2-2 with 480 and 960 kHz SCS, then the exact values need to be discussed. We believe the scaled values from that for 120 kHz by 4/8 for 480/960 kHz SCS (shown below) can be a good starting point to determine the exact values for larger SCSs: - {[4,8,12,16,...,640,1280,1600,2560,3200]} for 480kHz SCS, - {[8,16,24,32,..., 1280,1600,2560,3200,6400]} for 960kHz SCS. We think it would be sufficient to just use the values above for larger SCSs in FR2-2, i.e., the values scaled based on SCS. From power saving perspective, we believe the same “absolute” time duration should be considered even when larger SCS is configured. The simplest solution to achieve this would be the simple scaling. Some companies argued that the values should be discussed separately for LBT and non-LBT cases since maximum COT duration should be considered for LBT case. However, the values which are scaled based on the values for Rel-17 power saving shown above includes the values for LBT case, i.e., 160/320 slots, and the single value set shown above seems sufficient configuring per BWP. Thus, we think it is not necessary to specify SSSG switching timer and PDCCH candidates skipping values for 480/960 kHz SCS as NR-U based value sets and Rel-17 power saving based value sets separately. Proposal 4: Support the candidate skipping values and SSSG switching initial timer values in slots as follows: - The candidate skipping values can be configured as - {[4,8,12,16,...,640,1280,1600,2560,3200]} for 480kHz SCS, - {[8,16,24,32,..., 1280,1600,2560,3200,6400]} for 960kHz SCS. - The value of the SSSG switching timer in slots can be configured as, - {[4,8,12,16,...,640,1280,1600,2560,3200]} for 480kHz SCS, - {[8,16,24,32,..., 1280,1600,2560,3200,6400]} for 960kHz SCS. Considering the discussion at last RAN1 meeting, while our first preference is to support 25/100/200 symbols for 120/480/960 kHz SCS (i.e., either Alt-1 or Alt-2b), we can accept the values captured as working assumption to SSSG switching even for the unlicensed band operation in FR2-2. Proposal 5: Confirm the following working assumption: Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320.

### R1-2201542 (Spreadtrum)

As multi-slot PDCCH monitoring is based on slots within a slot group, and the X is 4 and 8 slots for SCS 480 and 960 kHz, respectively. Therefore, the bit string size of monitoringSlotsWithinSlotGroup-r17 set to 8 to support both 480 and 960 kHz SCS is reasonable. For the monitoring symbols in such a slot, it can be done by monitoringSymbolsWithinSlot. However, monitoringSymbolsWithinSlot is used to determine the symbols within a slot. For multi-slot case, we think monitoring symbols in each slots can be the same. Thus, a justification can be added to the note, e.g., Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot, and monitoring symbols in each slot are the same. Proposal 2: Confirm the following revised working assumption. - Working assumption: - The size is 8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot, and monitoring symbols in each slot are the same. - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs

### R1-2201593 (TCL Communication)

For FR2-2, the PDCCH is monitored in a slot-group manner. The recent conclusion indicates this is the default capability of a UE. In the last RAN1 107bis-e meeting, there are intensive discussions about the design of the search space for PDCCH monitoring in FR 2-2, there are some pending parameters without agreement reached. The designs related to Group (1) SS is indicated. For example, monitoringPeriodicityAndOffset-r17 - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots Those for Group (2) SS is missing. We prefer a unified design for Both Group (1) SS and Group (2) SS. For duration-r17, there are two possible options. - Option 1: duration-r17 indicates a continuous range which should be monitored. This the same as FR1 and FR2-1, The PDCCH is monitored in a continuous range within the monitoring period. - Option 2: duration-r17 indicates dis-continuous ranges which should be monitored. This is useful for intermittent traffic or power saving case. A bitmap would complete this feature, whereas that brings more overhead. The monitored symbol within a slot group is configured by monitoringSymbolsWithinSlot. It is a tradeoff that whether monitored symbols in each monitored slot in the slot group should be same. That brings a balance between flexibility and signaling overhead. Since there are various UEs and the UE capability is different. The UE capability will cover this tradeoff. Proposal 1: For the search space configuration design, a unified framework for both Group (1) SS and Group (2) SS is preferred. That reduces the specification workload. Proposal 2: the PDCCH monitoring duration can be dis-continuous ranges in terms of slot group for FR 2-2. This flexibility gives more chances for power saving UE and intermittent traffics. Proposal 3: Whether the monitoringSymbolsWithinSlot is same for each monitoring slot in a slot group leads to a tradeoff. The UE capabilities will cover this tradeoff. We support the both designs of monitoringSymbolsWithinSlot. It can be either a bitmap of 14bits, or a bitmap with more bits mapped to each symbol in the slot group.

### R1-2201663 (Nokia, Nokia Shanghai Bell)

The working assumption is well inline with the principle applied widely for all the processing times (i.e., apply the same absolute time for all SCSs: 120 kHz, 480 kHz and 960 kHz). Based on that we just propose to convert the working assumption to an agreement. Proposal 5: Support only search space set group switching processing capability 1 with the following values Minimum value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. It was concluded in RAN1 #107bis-e that “The SSSG switching timer is in units of slots”. One of the open questions for SSSG switching for 480/960 kHz SCSs is when to perform the SSSG switching. We think that the switching should happen always at the slot group boundary. Otherwise, there can be ambiguity e.g. in the CCE/BD budget determination. Proposal 6: For multi-slot PDCCH monitoring for 480/960 kHz SCSs, a boundary of SSSG switching is always aligned with a boundary of a slot group. We think that it makes sense to support Rel-17 PDCCH skipping feature also in FR2-2. And it makes sense to scale the values the values corresponding for 120 kHz by 4 and 8 for 480 and 960 kHz SCS, respectively (as the corresponding working assumption has been done in UE power saving AI already there is no need to do a separate agreement in this agenda item). When considering the PDCCH skipping for higher sub-carrier spacings, we think that the skipping duration is always extend till slot group boundary. In this case the skipping duration granularity could be same as for X, and the minimum value, to enable skipping of one ‘slot group period’, would be X-Y+1 (i.e. at the second last slot on the group, min. 2 slots). Other approach is that skipping duration is not bound by slot group, thus skipping duration could end during the multi-slot monitoring of a slot group. This requires to support durations down to one slot for 960kHz. As this would result enabling ‘sub slot-group’ skipping for 960 kHz, it may not have much practical use from UE power saving perspective. Therefore it is proposed that the skipping duration would always assumed to be extended till slot group boundary. Proposal 7: Skipping duration is always extended till slot group boundary. Corresponding to this it proposed to support following skipping durations - {2,3,4,8,12,16,…636,640,720,…,1200,1280, 1440, 1600, 1760,…,3040,3200} for 480kHz SCS - I.e. {2,3,[4:4:636],[640:80:1200],[1280:160:3200]} - {2,4,7,8,16,24,…1280,1440,1600,2400,2560,2880,3200,…,6080,6400 } for 960kHz SCS - I.e. {2,4,7,[8:8:1280],[1440:160:2560],[2880:320:6400]} As discussed in RAN1 #107bis-e there are differences between Rel-16 and Rel-17 approaches: the SSSG timer configuration of Rel-16 SSSG switching is per-cell, while the SSSG timer for Rel-17 SSSG switching is per-BWP. Based on that, it makes sense to avoid simultaneous usage of Rel-16 feature (defined originally for LBT scenario) and Rel-17 feature (defined for non-LBT scenario). However, it would be more natural to make such decision in UE power saving WI. That would ensure that the agreement is not limited to FR2-2 & 480/960 kHz SCSs. Proposal 8: Agreements related to simultaneous usage of Rel-16 SSSG switching and Rel-17 SSSG switching are done in UE power saving WI. Proposal 9: Confirm the following working assumtion - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs Proposal 10: revise the definition of duration in the following way: - If duration ≤ 8, UE considers up-to “duration” bits as valid bits in the bitmap - If duration >8, UE creates a new bitmap by repeating the monitoringSlotsWithinSlotGroup-r17, and considers up-to “duration” bits as valid bits in the new bitmap. Proposal 11: revise the value range for offset as: - value range for offset is {0, 4, 8, …, Xp-1} slots.

### R1-2201689 (Intel)

Before discussing details of Group (2) SS handling, it is better to align the understanding of FG 3-1 especially ‘any of’ in the following bullet. - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot. There are two interpretations. - Interpretation #1: It is limited to single span in a slot for each Group (2) SS set, however the different Group (2) SS sets can be configured in different spans in the slot. - Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot. In our view, Interpretation #2 is the correct understanding. Interpretation #1 will not bring any real complexity reduction at UE side compared to Interpretation #2. With Interpretation #1, though a single Group (2) SS set is limited to single span per slot, there can still be multiple spans in a slot due to multiple configured Group (2) SS sets. Consequently, UE must prepare for the worst case, i.e., to decode multiple spans per slot for Group (2) SS sets. Proposal 2: - Clarify that Interpretation #2 is the right understanding for FG 3-1 - Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot. In general, it is not preferred to limit the maximum number of slots that can be simultaneously configured for PDCCH monitoring in a slot group for Group (2) SS configuration flexibility. That is, the Group (2) SS can be simultaneously configured in all slots in a slot group. On the other hand, the maximum number of configured spans in a slot may remain to be 2 to align with Group (1) SS for the Y=1 slot. Since UE mandatorily supports Y=1 for the multi-slot PDCCH monitoring capability, the design on handling group (2) SS can take existing agreement for Y=1 as baseline, i.e., FG3-5b extension. That is, UE must be capable to monitor up to two spans of Group (1) SS in a slot in a slot group. To avoid excessive increase of UE complexity, it is preferred that the maximum number of monitored spans per slot in a slot group for the two groups of SS sets remains to be 2. Further, the total number of monitored spans of SS sets in a slot group should be carefully designed. A larger number of spans provide a better flexibility for PDCCH transmission. However, it also increases UE complexity and power consumption. To balance various factors, it is preferred that UE should be capable to monitor up to 3 or 4 spans of all SS sets in a slot group which includes spans for all Group (1) and Group (2) SS sets. Proposal 3: - The limitation on Group (2) SS sets should be clarified from two aspects - Whether to limit the number of slots/spans on which the SS set(s) can be simultaneously configured in a slot group for a UE. - Whether to limit the number of slots/spans on which the SS set(s) can be simultaneously monitored in a slot group by a UE. Proposal 4: - For SS set configuration - The Group (2) SS sets can be simultaneously configured in all slots in a slot group. - The maximum number of configured spans in a slot remains to be 2 considering all SS sets. - For PDCCH monitoring at UE - The maximum number of monitored spans per slot in a slot group remains to be 2 considering all SS sets - UE is capable to monitor up to 3 or 4 spans of SS sets in a slot group considering all SS sets. - Discuss the exact number of limited configured/monitored spans in a slot or in a slot group in UE feature for WI NR_ext_to_71GHz The configured search space configuration in the two SSSGs may have different requirements on the PDCCH monitoring capability. Figure 1 provides two examples for the SSSG switching with corresponding switching between combination (4, 1) and (4, 2). In Figure 1A, at the time for SSSG switching, UE needs to decode more PDCCHs if no additional restriction is introduced. Since the agreed Pswitch values are much larger than the slot group size X=4/8, UE can know in a quite early time that there happens burst PDCCH detections at the time of SSSG switching. As a result, a simple solution is that UE can cancel the PDCCH detection in X slots before the time of SSSG switching. On the other hand, Figure 1B shows another example that there exists a distance between the slots for PDCCH monitoring before or after SSSG switching. There is practically no problem to monitor all PDCCHs around the time of SSSG switching. In this case, it is desired if PDCCH in the first SSSG in the X slots before the time of SSSG switching can still be detected. Based on Figure 1A/1B, a unified solution could be that UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of combinations (X, Y) used before and after SSSG switching. Figure 1: SSSG switching resulting in burst PDCCH detections The following TP is proposed to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y). ------------------------------ TP#4c: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** 10.4 Search space set group switching *** Unchanged text is omitted *** If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field for a serving cell in a DCI format 2_0, as described in clause 11.1.1; - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchTimer - if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 If a UE indicates a capability to monitor PDCCHs according to multiple combinations Xs,Ys, the search space sets with group index 0 and 1 are respectively associated with different combinations Xs0,Ys0 and Xs1,Ys1, the UE monitors PDCCHs of search space sets with group index g in the slot(s) that belongs to both the Ys0 slots of combinations Xs0,Ys0 and the Ys1 slots of combinations Xs1,Ys1 within the maxXs0,Xs1 slots prior to PDCCH monitoring according to search space sets with group index 1-g. *** Unchanged text is omitted ***. Proposal 9: - Dynamic SSSG switching is supported for all SCSs 120, 480 and 960kHz. - PDCCH monitoring following the second SSSG can start right after the time of SSSG switching - The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y) - UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching - Agree on TP 4 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y). According to the above WA, monitoringSlotsWithinSlotGroup-r17 has a size of 8 bits, which may exceed value X. for example, it is X=4 for SCS 480kHz, the monitoringSlotsWithinSlotGroup-r17 can indicate the slots with configured MOs in two consecutive slot groups of X=4. UE can expect that the configured MOs are in same position in both slot groups. The 8-bit bitmap allows full flexibility for SS set configuration, however, it is unnecessary for a typical SS set configuration if Y=1. Since the agreed value range for the offset O is {0 .. Xp-1} slots, the offset O can indicate a right slot index with configured MOs in a slot group, i.e., mod(offset, X). In this case, monitoringSlotsWithinSlotGroup-r17 can be omitted. One more remaining issue is on the definition of duration. duration indicates the number of consecutive slots for PDCCH monitoring according to a combination (Xs, Ys) upon every period as given in the monitoringSlotPeriodicityAndOffset. The following TP is proposed for SS set configuration. ------------------------------ TP#5: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: - a search space set index s, 0<s<40 , by searchSpaceId - an association between the search space set s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610 - a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17 - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot - a PDCCH monitoring pattern within a group of slots, indicating consecutive slots in the group of slots for PDCCH monitoring, by monitoringSlotsWithinSlotGroup-r17. If monitoringSlotsWithinSlotGroup-r17 is not configured, the slot for PDCCH monitoring is indicated by PDCCH monitoring offset os - a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration or by duration-r17 *** Unchanged text is omitted ***. Proposal 10: - In search space set configuration, - duration indicates the number of consecutive slots for PDCCH monitoring according to a combination (Xs, Ys) upon every period as given in the periodicityAndOffset - Revise the WA: - monitoringSlotsWithinSlotGroup-r17 can indicate the slots with configured MOs in two consecutive slot groups of X=4. UE can expect that the configured MOs are in same position in both slot groups. - if monitoringSlotsWithinSlotGroup-r17 is not configured, the SS set is only configured in slot modoffset,X in a slot group - Agree on TP 5 for SS set configuration

### R1-2201735 (Ericsson)

The following open issues from this agreement are identified: - For Group (2) SSs - Whether or not the monitoring periodicity and/or duration are restricted to be integer multiples of Xs slots - Whether or not the slots indicated by the bitmap monitoringSlotsWithinSlotGroup-r17 need to be consecutive - Details of the offset configured by the parameter monitoringPeriodicityAndOffset-r17 - Whether or not the definition of the parameter duration needs to be revised Regarding Issue 1a, according to the agreement for mandatory Group (2) monitoring capability below, the UE shall be capable of monitoring a particular Group (2) search space in a single span within each slot of the slot group. Hence, this agreement would already preclude that the monitoring periodicity and duration are restricted to be integer mulitples of Xs slots. Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. Regarding Issue 1b, we note that the legacy monitoring behavior defined in 38.331 and 38.304 for SI acquisition and paging (i.e., Type0A/2 SSs), respectively, are defined in terms of slots not slot groups. We prefer to avoid any changes to that legacy behavior, which may be required if configuration restrictions on monitoringSlotsWithinSlotGroup-r17 are introduced. We note that even if the monitoring periodicity for Type0A/2 SSs is configured to be quite small (e.g., 1 or 2 slots), the actual monitoring occasions within the SI window/paging frame are spaced much wider than the configured periodicity due to how the MOs are associated with transmitted SSBs (see discussion in Section 2.3). Based on the above we make the following pair of proposals: For Group(2) SSs, the monitoring periodicity and duration are not restricted to be integer multiples of Xs slots. For Group(2) SSs, the slots indicated by the bitmap monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive. According to the above agreement on search space configuration, the periodicity indicated by the parameter monitoringPeriodictyAndOffset-r17 is restricted bo be an integer multiple of Xs slots for Group (1) SSs. As we discussed previously (see Proposal 3), we propose that this restriction does not apply to Group (2) SSs. As agreed last meeting, for both Group (1) and Group (2), the value range of the parameter monitoringPeriodictyAndOffset is extended such that it allows for configuration of a number of slot groups that cover an equivalent time duration as the number of slots for 120 kHz. This needs to be captured in the field description of the RRC parameter. Furthermore, in the field description currently in 38.331, the allowed periodicities for monitoring for DCI formats 2_0, 2_1, and 2_4 are restricted to a certain number of slots, e.g., for DCI format 2_0 the periodicity is restricted to {1, 2, 4, 5, 8, 10, 16, or 20} slots. For the case of multi-slot PDCCH monitoring, it makes sense to extend the applicable values up to 160 slots to cover the case when either Xs = 8 or Xs = 4 is used. In the RRC parameter spreadsheet, provide a recommendation to RAN2 to update the field description of the parameter monitoringPeriodictyAndOffset as follows including additional allowed periodicities at least for DCI format 2_0 (DCI formats 2_1 and 2_4 can be separately discussed): monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. If the UE is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the UE is configured to monitor DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10) and additionally the values 'sl32', 'sl40', 'sl64', 'sl80', 'sl128', and 'sl160' are applicable if a UE monitors PDCCH according to a slot group combination (Xs,Ys) (see TS 38.213 [13], clause 10). If the UE is configured to monitor DCI format 2_4, only the values 'sl1', 'sl2', 'sl4', 'sl5', 'sl8' and 'sl10' are applicable. If a UE monitors PDCCH in a Type-1 CSS with dedicated RRC configuration, or a type 3 CSS, or a USS on a cell according to a slot group combination (Xs,Ys) (see TS 38.213 [13], clause 10), only values 'slN' are applicable where N=n*Xs and n is an integer greater than or equal to 1. For IAB-MT, If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_5, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10).. If a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. Regarding Issue 2 above, the following is already specified in 38.213 Section 10: This says that the location of the slot groups is fixed and aligned from the beginning of a subframe. Given the prior agreement that the Y consecutive slots in which PDCCH is monitored in Group (1) SSs can be located anywhere within a slot group, and that the slot(s) in which the UE monitors PDCCH in Group (2) SSs can be anywhere within a slot group, it is essential that the offset configured with the parameter monitoringPeriodicityAndOffset can be configured with slot level granularity, i.e., unrestricted. Based on this, we propose the following: For both Group (1) and Group (2) SSs, the offset configured by the parameter monitoringPeriodicityandOffset-r17 is not restricted to be an integer multiple of Xs slots. As shown in the above agreement on search space configuration, the value range {8,12, …, 20476} for the parameter duration-r17 supports integer multiples of Xs both for Xs = 4 and 8 which applies to Group (1) SSs. However, as discussed above for Group (2) SSs, the periodicity and duration should not be restricted to integer multiples of Xs (see Proposal 3). Hence, for the parameter duration-r17 to be usable for both Group (1) and Group (2) search spaces, the value range should be augmented to include all integer values from 2 .. 20479. If the value range is not augmented in this way, then a separate duration parameter would be needed for Group (2) SSs, and we don’t think this is a preferrable solution. Based on this discussion, we makes the following pair of proposals: For the parameter duration-r17, augment the agreed value range {8, 12, …, 20476} to include all integer values in the range {2 .. 20479}. For Group (1) SSs, the configured duration is restricted to be integer multiples of Xs as previously agreed. For Group (2) SSs, the configured duration is not restricted. In the RRC parameter spreadsheet, provide a recommendation to RAN2 to update the field description of the parameter duration as follows: duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). If a UE monitors PDCCH in a Type-1 CSS with dedicated RRC configuration, or a type 3 CSS, or a USS on a cell according to a slot group combination (Xs,Ys) (see TS 38.213 [13], clause 10), the duration can only be configured such that it is an integer multiple of Xs with minimum value 2Xs. If the field is absent, the UE applies the value Xs slots except for DCI format 2_0. The UE ignores this field for DCI format 2_0. For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).. Some companies expressed a concern that if the SSSG switch occurs early in the slot group and the new SSSG is based on a different location of the Y slots for Group (1) monitoring, then decisions on search space dropping may need to be re-evaluated. To avoid this complication, it was suggested to specify that SSSG switches are always aligned with a slot group boundary. This seems like a reasonable restriction to avoid specifying complicated UE behavior. Furthermore, it does not conflict with the conclusion above. It can be specified that upon timer expiry, the UE performs the switch at the next slot group boundary. For a serving cell, if if a UE is configured with monitoringCapabilityConfig = r17monitoringcapability and is provided a group index for a search space by searchSpaceGroupIdList, the UE starts or stops monitoring search spaces in the corresponding search space set group (SSSG) not earlier than the beginning of the first symbol of the first group of Xs slots that occurs after the slot in which the UE determines that a SSSG switch should occur. The two working assumptions Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8 Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. have been captured and approved in TR 38.213 [3]. In our view, both can be considered confirmed.

### R1-2201765 (Apple)

Proposal 6: To limit the complexity based on the Group (2) SS location across multiple slot groups, one or more of the following could be considered: - Group (2) SSs could be placed within the same slot group - If spread across multiple slot groups, for CSSs Type 0 (SIB1), Type 0A (SIBx) and Type 2 (Paging), the CSS periodicity for 480 kHz and 960 kHz should not be shorter than that for 120 kHz to ensure that the wake-up period is intermittent and limit the impact on the UE’s power consumption. - Limit the number of times a Group (2) SS may be configured within a duration of M slot groups e.g. N SSs within M slot-groups where the N SSs are in consecutive slot groups Proposal 8: To finalize the design of the Search Space configuration parameters for multi-slot PDCCH monitoring , the following should be agreed upon: - The configured value of the offset at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - The Working Assumption on the new parameter monitoringSlotsWithinSlotGroup-r17 should be accepted. Proposal 12: The following WA made in RAN1 #107-e should be confirmed: Working assumption The following values are adopted as minimum value of for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values Minimum value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. Proposal 13: Rel-16 and Rel-17 SSSG switching are not simultaneously configured

### R1-2201899 (NEC)

For operation in unlicensed band beyond 52.6GHz, in our understanding, the legacy SSSG switching can be reused for 120 kHz SCS, since it has been concluded: for 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. So the monitoring capability before and after the SSSG switch is the same, both are per-slot based. While for 480 kHz and 960 kHz SCS, it was agreed in RAN1#107-e [2], there are several supported combinations of (X, Y). If a UE supports multiple (X, Y) combinations, it will be more flexible to support SSSG switching between different (X, Y) combination. For example, to handle those use cases with low-latency, denser PDCCH monitoring occasion may be configured in some period, then PDCCH monitoring capability combination will be changed along with SSSG switching, e.g. for 960 kHz SCS, there are 2 configured SSSG, PDCCH is configured to be monitored per 8-slots by the first search space set group, and the second search space set configures PDCCH to be monitored per 4-slots. In this case, the monitoring capability is different before and after the switching. In our view, SSSG switching between SSSGs that have the different (X, Y) PDCCH monitoring combination should be supported. Proposal 1: For operation in unlicensed band with 480 kHz and 960 kHz SCS, support SSSG switching between SSSGs that have the different (X, Y) PDCCH monitoring combination. In Rel-16, the switching boundary is the first slot that is at least Pswitch symbols after some switch indication. For 480 kHz and 960 kHz SCS, PDCCH monitoring capability is multi-slot based, e.g. 4 slots for 480 kHz SCS and 4/8 slots for 960 kHz SCS, the switching boundary should be naturally extended to slot group based accordingly, and it can avoid monitoring two search space set groups in one slot group. Proposal 2: For operation in unlicensed band with 480 kHz and 960 kHz SCS, the switching boundary is aligned with the boundary of a slot group

### R1-2201914 (Xiaomi)

It was agreed that for duration-r17, the value range is {8, 12, …, 20476}, and values represent slots. The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs. Our understanding for duration-r17 is the number of consecutive slots that cover a SearchSpace in every periodicity, and search space can only exist within the slot(s) that is(are) indicated as ‘1’ by monitoringSlotsWithinSlotGroup-r17. For example, if duration-r17 is configured as 8, and Xs =4, monitoringSlotsWithinSlotGroup-r17=10001000.It is clear the intention is not to have consecutive 8 slots for the searchspace, rather, there is only 2 slots, 1 slot out of every 4 slots, for the search space within a periodicity. Proposal 2: duration-r17 is the number of consecutive slots that cover a SearchSpace in every periodicity, and search space can only exist within the slot(s) that is(are) indicated as ‘1’ by monitoringSlotsWithinSlotGroup-r17.

### R1-2202005 (Samsung)

In the last meeting, a framework for search space configuration applicable to multi-slot based PDCCH monitoring was agreed, and there are remaining issues to be resolved based on the framework. First, there is a FFS on the offset of search space set. Based on current framework, the periodicity and duration of the search space set are both integer multiple of Xs, and hence, the offset should also be an integer multiple of Xs such that the duration of a search space set starts from a boundary of a group of Xs slots. Also, there is a FFS on the definition of duration for the search space set. For legacy search space set configuration, the field duration refers to “number of consecutive slots that a SearchSpace lasts in every occasion”, which is obviously not applicable to Group (1) SSs. The description should be revised for multi-slot based PDCCH monitoring, and a UE should assume a SearchSpace may last in a subset of the consecutive slots according to the newly introduced bitmap monitoringSlotsWithinSlotGroup-r17. Lastly, it was agreed to have the limitation that the bits taking value of 1 in the bitmap for indicating slots including Group (1) search space set (i.e., monitoringSlotsWithinSlotGroup-r17) should be consecutive, and it should be further clarified that the number of bits taking value of 1 should be no larger than Ys in order to be compatible with the framework for multi-slot based PDCCH monitoring. Proposal 4: For search space set configuration of multi-slot PDCCH monitoring: - the configured offset at least for Group (1) SSs is restricted to be an integer multiple of Xs slots; - “duration-r17” refers to “number of consecutive slots that formulates a number of groups of Xs slots where a SearchSpace locates according to monitoringSlotsWithinSlotGroup-r17 in each of the group of Xs slots”; - The number of slots indicated by “monitoringSlotsWithinSlotGroup-r17” should be no larger than Ys.

### R1-2202072 (MediaTek)

In RAN1 #107-e meeting, the framework of multi-slot PDCCH monitoring and BD/CCE limit within a slot group in a single cell has been specified. Further in RAN1 #107bis-e meeting, enhancements of search space set configuration were discussed. Two IEs are introduced, duration-r17 and monitoringSlotsWithinSlotGroup-r17, to address the flexibility of multi-slot PDCCH monitoring configuration but no clear definition is captured. In our understanding, the purpose of duration-r17 is to specify the “duration” of slot-group and the purpose of monitoringSlotsWithinSlotGroup-r17 is to specify which slots within a slot group are configured to monitor. That is, for a search space set s, UE will monitor PDCCH in the slots indicated by monitoringSlotsWithinSlotGroup-r17 within a slot group of Xs slots and the same monitoring pattern within a slot group will be performed in consecutive Ms slot groups, where duration-r17=Ms×Xs. It was also discussed that the slots indicated in the bitmap should be consecutive at least for Group (1) SSs. In our view, the same restriction should be applied to Group (2) SSs since it is unlikely that UE will monitor a Group (2) SS more than once in a slot group. Proposal 2: For a search space set s configuration, UE is provided a PDCCH monitoring pattern within a slot group of size Xs slots indicating a duration of consecutive slots that a search set s exists by monitoringSlotsWithinSlotGroup-r17. The same monitoring pattern within a slot group will be performed in consecutive Ms slot groups indicated by duration-r17=Ms×Xs.

### R1-2202130 (Qualcomm)

Based on the agreement, associated with a SS set, slot(s) for PDCCH monitoring in a slot group is determined by the new parameter, monitoringSlotsWithinSlotGroup-r17. In the legacy SS set configuration, however, the monitored slot within a period (or with respect to SFN = 0) is determined by the offset (i.e., monitoringSlotPeriodicityAndOffset). As such, in Rel-17, there are two distinct parameters of similar purposes. To avoid the duplication and to keep the configuration consistent, it would be fair to split the roles. That is, if monitoringSlotsWithinSlotGroup-r17 is configured to indicate a slot offset, the offset parameter (indicated by monitoringSlotPeriodicityAndOffset-r17) may indicate a slot group offset, i.e., the starting slot of the slot group where the first MO in a duration (indicated by duration-r17) is located. Proposal 4: For multi-slot PDCCH monitoring, when monitoringSlotsWithinSlotGroup-r17 is provided, the offset parameter given by monitoringSlotPeriodicityAndOffset-r17 of the search space set configuration is restricted to be an integer multiple of Xs slots. - The offset parameter indicates a slot group offset, i.e., the first slot of the slot group where the first MO in a duration indicated by duration-r17 is located. For the granularity of the periodicity, since the new parameter monitoringSlotPeriodicityAndOffset-r17 is dedicatedly used for multi-slot PDCCH monitoring configuration, any values that are not integer multiples of Xs are unnecessary. Thus, the granularity of periodicity and offset parameters may be restricted to only support integer multiples of Xs. Proposal 5: monitoringSlotPeriodicityAndOffset-r17 is dedicatedly used for multi-slot PDCCH monitoring configuration. - The range of the periodicity is {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}. - For a periodicity value Xp, the range of the offset is {0,4,8,…,4(Xp-1)4}. In Rel-15/16, the periodicity configuration for some group-common DCI formats is restricted. Thus, for multi-slot PDCCH monitoring in Rel-17, similar restriction should be applied. To maintain the same level of flexibility as 120 kHz SCS in FR2-1, the restricted set of periodicities for 480/960 kHz SCSs may be determined by simple scaling. Proposal 7: For group common DCI formats, the following periodicities are only applicable: 120 kHz (same as FR2); 480 kHz; 960 kHz. DCI format 2_0; sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20; sl4, sl8, sl16, sl20, sl32, sl40, sl64, sl80; sl8, sl16, sl32, sl40, sl64, sl80, sl128, sl160. DCI format 2_1; sl1, sl2, sl4; sl4, sl8, sl16; sl8, sl16, sl32. DCI format 2_4; sl1, sl2, sl4, sl5, sl8, sl10; sl4, sl8, sl16, sl20, sl32, sl40; sl8, sl16, sl32, sl40, sl64, sl80. * Highlighted: New periodicity values to be introduced for 480/960 kHz SCSs. When a UE reports more than one (Xs,Ys) values for the multi-slot PDCCH monitoring capability, the search space set configuration should comply with at least one of the reported (Xs,Ys) values. At the same time, there should be no ambiguity in the search space set configuration between the UE and the gNB. In RAN1 #107b-e, a working assumption was made that the new parameter monitoringSlotsWithinSlotGroup-r17 is given by a bitmap of a fixed size, i.e., 8 bits. For Xs = 8 slots, the bitmap can indicate the consecutive Y slots within a slot group of 8 slots. Likewise, for Xs = 4 slots, only the first 4 bits of the bitmap may be used, while the rest 4 bits are left unused. However, depending on how the unused bits are marked, there could be an ambiguity between the UE and the gNB. For example, if the unused bits are set to ‘0’, a bitmap of ‘10000000’ may results in different MO configurations depending on whether it is interpreted with Xs = 8 slots or Xs = 4 slots, as illustrated in Figure 1. Figure 1: PDCCH MO configuration ambiguity for monitoringSlotsWithinSlotGroup-r17 = 10000000. To address the ambiguity issue, the bitmap, monitoringSlotsWithinSlotGroup-r17, can have a variable size, e.g., 4 bits or 8 bits. The size of the configured bitmap can implicitly indicate the selected Xs value by the gNB, among the multiple values supported by the UE. Proposal 6: The bitmap size of the new parameter, monitoringSlotsWithinSlotGroup-r17, is Xs bits, where Xs is either 4 or 8. In RAN1 #107b-e, the following has been agreed: Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. - Continue discussion on whether or not introducing other limitation for Group (2) SSs in RAN1#108-e.. FG 3-1, which is the basic slot-based PDCCH monitoring capability, restricts the MOs for Type1 CSS without dedicated RRC configuration and Type0/0A/2 CSSs within a slot. However, for multi-slot PDCCH monitoring, since it does not restrict the overall CSS MOs within a slot group, it may run into the same issues that we confronted with the design of SS set #0 in RAN1 #107-e: If a UE is required to monitor more than one MO for different types of CSS sets within a slot group, the BD/CCE budget may get exhausted. Thus, similar to the new design of SS set #0 (i.e., monitoring slot n0 and n0+X0), it would be desirable to limit the number of MOs for any of different types of CSSs within a slot group. During the RAN1 #107b-e discussion, it was argued that, at least for Type2 CSS, the MO is determined by the association with SSBs and, thus, the number of Type2 CSS MO per slot group is intrinsically limited without any explicit restriction. For example, if a MO for Type2 CSS is configured in every slot (i.e., searchSpaceId ≠ 0 for Type2 CSS) and the number of actually transmitted SSBs is N, the UE shall monitor the Type2 CSS only in a slot out of the N consecutive slots within a paging occasion, associated with the measured/preferred SSB. However, in some cases, the number of SSBs may be small, e.g., less than 4, and the UE may be required to monitor more than one Type2 CSS MO within a slot group. Furthermore, for Type1 CSS, when searchSpaceId ≠ 0, the UE can still be configured to monitor more than one MO per slot group. Proposal 2: For Type1 CSS without dedicated RRC configuration and for Type0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) within a slot, with the monitoring occasions for any of Type1 CSS without dedicated RRC configuration, or Types0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot group of X0 slots. - X0 = 4 for 480 kHz SCS and X0 = 8 for 960 kHz SCS When a UE reports more than one (Xs,Ys) combinations for its multi-slot PDCCH monitoring capability, the two SSSGs may be associated with different (Xs,Ys) values. For example, when a UE with 960 kHz SCS supports (4,1) and (8,1), SSSG#0 may be configured according to (4,1), and SSSG#1 may be configured according to (8,1). SSSG#0 may provide more frequent PDCCH MOs, once in 4 slots, although the BD and CCE budgets are limited (i.e., 10 BD and 16 CCEs). On the other hand, SSSG#1 may be configured with PDCCH MOs with a periodicity larger than or equal to 8 slots and provide improved power efficiency and scheduling flexibility (i.e., 20 BD and 32 CCEs). In the same situation, on the contrary, if the two SSSGs are restricted to have the same (Xs,Ys) value, (4,1) should be applied for both SSSGs. This will limit the BD and CCE budgets of both SSSGs to 10 BD and 16 CCEs and, as a results, may harm the scheduling flexibility of SSSG#1. Thus, it is beneficial to allow different (Xs,Ys) values, if supported, for different SSSGs. Proposal 8: When a UE supports more than one (Xs,Ys) combinations for multi-slot PDCCH monitoring, the applied (Xs,Ys) value is determined per SSSG. In the legacy design of search space set group (SSSG) switching, the switching boundary is aligned with a slot boundary. However, when SSSG switching and multi-slot PDCCH monitoring are jointly applied, the legacy SSSG switching design may lead to a complicated transient behavior. For example, if the SSSG switching occurs in the middle of a slot group and, as a result, if SS sets from both SSSG#0 and SSSG#1 are included in the same slot group, BD/CCE counting, overbooking, and dropping should be re-calculated. Therefore, it would be desirable to always align the SSSG switching boundary with the slot group boundary. In RAN1 #107b-e, it was concluded that the SSSG timer operates in a slot unit, which implies that the expiration of the timer may not always aligned with the slot group boundary. Thus, once the timer expires at a slot in the middle of a slot group, the actual SSSG switching can occur at the next slot group boundary. If the UE supports more than one (Xs,Ys) values, and the two SSSGs are associated with different Xs values, the slot group boundary would be determined by the largest Xs value between the two SSSGs. Proposal 9: For multi-slot PDCCH monitoring for 480/960 kHz SCSs, a boundary of SSSG switching is always aligned with a boundary of a slot group. - When the SSSGs are associated with different slot group sizes Xs, the slot group boundary is determined by the largest Xs value between the two SSSGs. Further related to the joint configuration of multi-slot PDCCH monitoring and SSSG switching, a situation shown in Figure 2 may be considered. That is, SSSG switching occurs from the first SSSG to the second SSSG. Then, at the boundary of SSSG switching, the locations of the two Y consecutive slots in the slot groups before and after the switching boundary may be different. In another case, the two SSSGs may be associated with different (Xs,Ys) values according to Proposal 8. Thus, in such cases, the separation between the two Y consecutive slots may be less than Xs slots. To avoid such a situation in Figure 2, a rule for dropping PDCCH MOs may be considered. For example, when the separation between the two Ys consecutive slots before and after the switching boundary is less than Xs slots, where Xs is according to the first (source) SSSG, some or all of the MOs in the Ys consecutive slots in the second (target) SSSG after the switching boundary may be dropped. Proposal 10: A dropping rule for PDCCH MOs may be applied for the first Ys consecutive slots after SSSG switching, if the separation between the two Ys consecutive slots before and after the SSSG switching boundary is less than Xs slots. Figure 2: An example of SSSG switching for multi-slot PDCCH monitoring.

### R1-2202190 (Sharp)

In RAN1#107-e meeting, the working assumption for SSSG switching at 120/480/960kHz is achieved. The working assumption was directly described in 38.213, and the minimum switching time Pswitch was defined as one value for each SCS. Here, we have one concern about the SSSG switching at 480kHz/960kHz, which is that it may cause a back-to-back problem when switching. It is important to avoid this problem, which we have been considering for a long time in past meetings. In the current specification, SSSG-related monitoring is stopped and started at the same time in the first slot after the Pswitch. However, in multi-slot monitoring, SSSG switching may be performed after the UE has monitored in a later slot of the slot group, and monitoring may start in the slot immediately after. In this case, a back-to-back problem occurs. To avoid this situation, it is possible to not monitor the Xs slots before and after the SSSG boundary. This can be achieved by dropping PDCCH candidates in the Xs slots before or after the switchover, or by setting different Pswitch values to stagger the stop and start of monitoring. Figure 1:Possible problem with SSSG switching with multi-slot monitoring. Proposal 3: To avoid the back-to-back problem, monitoring should not be done in the Xs slots before and after the SSSG boundary. Proposal 4: Adopt Text proposal #2-1 or #2-2.

### R1-2202234 (Transsion Holdings)

In RAN1#107-e meeting, it was agreed that the same units for parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot are reused in FR 2-2. Regarding the mechanism to determine monitoring slots within a slot group, sine the three parameters have been reused, a simple approach is to introduce a new parameter to indicate which slots within a slot group can be used to monitor the PDCCH instead of modifying the existing three parameters. Proposal 3: Confirm the working assumption that a new parameter monitoringSlotsWithinSlotGroup-r17 should be introduced. In unlicensed band, it is beneficial to access the channel as soon as possible, when gNB passes the LBT. For this reason, the PDCCH monitoring occasion needs to occur frequently in the time domain. However, frequent monitoring of PDCCH consumes a plenty of power on the UE side, which is not beneficial for UE. In order to resolve this issue, a mechanism of search space set group switching is introduced to balance the channel access possibility from gNB side and the power consumption on PDCCH monitoring of UE side. Regarding the parameter Pswitch, in NR-U the value of Pswitch is based on SPS release timeline. For 120kHz SCS, the value of SPS release timeline is 25 symbols. Considering the implementation margin, 40 symbols is reasonable for 120kHz SCS. Then, the values for 480/960kHz SCS can be directly scaled by a factor of 4/8. Furthermore, when SSSG switching operates on a slot group basis, it is also necessary to determine to which slot the switching operation applies. When reusing the Rel 16 rule, if the first slot after the time of Pswitch symbols is in the middle of the slot group, some potential issues may arise. Considering the SS set configuration can be switched in a slot group, the UE may be required to monitor two search space set groups during a slot group, in that case the BD/CCE budget may exceed the the limitation of the UE. As a result, unnecessary search space dropping occurs. Therefore, SSSG switching mechanism should take the slot group as the unit. Proposal 4: Confirm the working assumption that the value of Pswitch should be 40, 160, 320 respectively for 120/480/960 kHz SCS. Proposal 5: SSSG switching mechanism should take the slot group as the unit.

### R1-2202336 (LG)

In Rel-15, the definition of duration is the number of consecutive slots in which the SS set exists within ks (i.e., the configured period). However, for multi-slot monitoring in Rel-17, ks can be set across multiple slot-groups and UE can monitor Group (1) SS only in limited number of slots within a slot-group, so the slots in which the SS set exists within ks may be non-consecutive. In order to indicate such non-consecutive monitoring slots within ks, it is required to configure two different parameters jointly. One is the duration-r17 indicating the number of consecutive slots in which MO may exist within ks (which is a parameter similar to duration for Rel-15 except that MO does not always need to exist in the slot indicated by duration), and the other is monitoringSlotsWithinSlotGroup-r17 indicating the monitoring slots within each slot-group. With these two parameters, UE can determine the exact monitoring occasions within ks. Therefore, for multi-slot monitoring, duration-r17 can be defined as “the number of consecutive slots that a monitoring occasion may exist within periodicity ks indicated by monitoringSlotPeriodicityAndOffset-r17”. It is noted that not all slots indicated by duration-r17 need a valid monitoring occasion to exist. Proposal #5: For Group (1) SS, duration-r17 should indicate the number of consecutive slots (within periodicity ks indicated by monitoringSlotPeriodicityAndOffset-r17) in which a monitoring occasion can exist. Among these slots, a slot whose corresponding bit in monitoringSlotsWithinSlotGroup-r17 is set to ‘1’ can be a valid monitoring slot. In addition, the default value of duration-r17 for Group (1) SS sets should be discussed. According to the agreement above, the configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SS. So, the default value for Group (1) SS should be defined differently for each SCS. Considering that the default value of Rel-15 duration is 1 slot (which is the minimum value for duration), the default value of duration-r17 can be defined as 4/8 slots for 480/960 kHz, respectively. Proposal #6: For Group (1) SS, define the default value of duration-r17 as 4 slots for 480 kHz or 8 slots for 960 kHz. The parameter monitoringSlotsWithinSlotGroup-r17 is newly defined for multi-slot monitoring. As a working assumption, 8-bit bitmap was agreed to indicate the monitoring slots within a slot-group for 480 kHz or 960 kHz. Each bit in this parameter represents a slot in a slot-group and each bit is set to ‘1’ if MO exists in the corresponding slot. One thing to emphasize is that the monitoring slots configured by this parameter should be valid only for slots indicated by duration-r17. Proposal #7: monitoringSlotsWithinSlotGroup-r17 should be valid only for slots indicated by duration-r17. Among the supported (X,Y) combinations, monitoringSlotsWithinSlotGroup-r17 does not need to be set for the (X,Y) combinations that are mandatorily supported. This is because, for these combinations, a UE monitors PDCCH candidates only in one slot per slot-group, and the corresponding slot can be indicated by monitoringSlotPeriodicityAndOffset-r17. That is, even if this parameter is not set for the mandatorily supported (X,Y) combinations, the UE may recognize one slot for each X slots as a monitoring slot. Of course, the corresponding monitoring slot is only valid for slots indicated by duration-r17. Alternatively, default value of monitoringSlotsWithinSlotGroup-r17 can be used for the mandatorily supported (X,Y) combinations. For instance, the default value can be defined as '10000000'. If monitoringSlotsWithinSlotGroup-r17 is not configured when multi-slot monitoring is based on mandatorily supported (X,Y) combination, the first slot of each slot-group (only for slots indicated by duration-r17) can be determined as the monitoring slot according to the default value. Proposal #8: monitoringSlotsWithinSlotGroup-r17 doesn’t need to be configured for the mandatorily supported (X,Y) combinations. Proposal #9: Define the default value of monitoringSlotsWithinSlotGroup-r17 as ‘10000000’ for 480 kHz or 960 kHz. Regarding SSSG switching, in Rel-15/16 NR, one SSSG could be switched to another SSSG at the slot boundary after at least P_switch symbols from the switching triggering. However, for the multi-slot monitoring, if SSSG switching occurs at the slot boundary inside the slot-group rather than the slot-group boundary, the complexity of the UE may increase. For example, when a switch occurs within the X slot-group, it may operate as SSSG#0 before the switching and may operate as SSSG#1 after that. At this time, since the BD/CCE budget for 480/960 kHz SCS is checked in units of slot-group, SSSG change in the middle of slot-group may cause to increase UE complexity compared to that for the SSSG switching at slot-group boundary. In addition, given that the boundaries of slot-groups and SSSG switching are aligned and the location of Y within the X slot is maintained across slot-groups even if SSSG is switched, back-to-back monitoring issue would be avoided. In this regard, SSSG switching should be performed at the slot-group boundary. Proposal #11: For 480 kHz or 960 kHz multi-slot monitoring, SSSG switching should be performed at the slot-group boundary after at least P_switch symbols from the switching triggering. In Rel-16 NR-U, timer-based SSSG switching was introduced. A UE sets the timer value to the number of slots provided by searchSpaceSwitchTimer when SSSG switching is triggered from the default SS group (SSSG#0) to the other one (SSSG#1). The UE decrements the timer value by one after each slot based on the smallest SCS among all configured DL BWPs in the serving cell. The maximum value of searchSpaceSwitchTimer is 20/40/80 for 15/30/60 kHz, respectively. These are values derived from the maximum COT duration, 20 msec, for the NR-U in Rel-16. However, the maximum COT duration in FR2-2 was determined to be 5 msec, therefore the maximum value of searchSpaceSwitchTimer can be defined as the number of slots corresponding to that time for each SCS, at least for operation with shared spectrum channel access. Proposal #12: For operation with shared spectrum channel access, define 40/160/320 slots as the maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS, respectively.

### R1-2202409 (Lenovo, Motorola Mobility)

According to the working assumption, the size of the new parameter monitoringSlotsWithinSlotGroup-r17 is 8 bits regardless of the slot group size. In our opinion, the size of the parameter can be aligned with the slot group size for the corresponding search space configuration, so that for X=4 the bitmap is 4 bits wide and for X=8 the bitmap is 8 bits wide. This can be efficiently implemented in RRC. Proposal 2: Confirm the working assumption from RAN1#107bis-e with the following modification: - The size of monitoringSlotsWithinSlotGroup-r17 is equal to the number of slots per slot group (i.e. X).

## Topic A3: BD Budget/Dropping

### R1-2200953 (Huawei, HiSilicon)

The discussion on monitoring capability for multiple serving cells in RAN1#107b-e lead to the the formulation of the following two alternatives [1]: - Alt 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation - Alt 1-1: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs,Ys value are grouped together to follow a total BD/CCE budget - Alt 1-2: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget - Alt 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation - Alt 2-1: A serving cell with SCS μ and multi-slot-based capability Xs,Ys is considered as an equivalent virtual cell with SCS μ-log2Xs and slot-based capability, where a slot group for the serving cell is considered as a slot for the virtual cell - Alt 2-2: A serving cell with SCS μ and multi-slot-based capability is considered as an equivalent virtual cell with SCS μ=3 and slot-based capability, where 4/8 slots for the serving cell with SCS μ=5/6 is considered as a slot for the virtual cell Alt 1 is similar to the BD/CCE budget calculation for per-span PDCCH monitoring in Rel-16 and is straightforward. However, since the value of Ys in Xs,Ys does not impact the maximum BD/CCE budget, there is no need to calculate the BD/CCE budgets of serving cells with the same SCS and Xs yet different Ys separately. Hence, Alt 1-2 is least preferred. As pointed out by some companies in [1], there are the following 3 cell types in NR Rel-17: serving cells with slot-based PDCCH monitoring capability, serving cells with span-based PDCCH monitoring capability, and serving cells with multi-slot PDCCH monitoring capability. If Alt. 1 is adopted, i.e., the MOs of different PDCCH monitoring capabilities for each numerology are counted separately, it might be a burden on the UE side. Therefore, Alt 2 seems a better solution to us. Proposal 14: Support to transform the serving cell with multi-slot-based capability to an equivalent serving cell with slot-based capability for further BD/CCE budget calculation.

### R1-2201086 (vivo)

Therefore, in multi-cell operation scenario, BD/CCE budget calculation becomes more complex by introducing such multi-slot-based BD/CCE budget definition, i.e. more additional cases as described above. The following alternatives could be considered: Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation - Alt. 1-1: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and (Xs, Ys) value are grouped together to follow a total BD/CCE budget - Alt. 1-2: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget As a straightforward alternative, the BD/CCE budget calculation adopts the same way for NR Rel-16, i.e. serving cells with the same PDCCH monitoring type are grouped together for further handling. Particularly, the follows steps apply: - Determination of NcellsCap: UE needs to report respective NcellsCap for different cases, i.e. Case 1-7 as described above. For the case with mixed capability, L NcellsCap values need to be reported where L is the number of capability types in that case (e.g. 3 in case 7); - Determination of total limit for each group of serving cells: - If the group adopts slot-based or span-based capability, legacy way is used; - If the group adopts multi-slot-based capability, further divide the cell group into different parts depending on SCS and/or value of X/Y. Then BD/CCE budget for the serving cells will follow one total limit. Note that there may have certain limits in the group or part of serving cells. Alt. 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation - Alt. 2-1: A serving cell with SCS μ and multi-slot-based capability (Xs, Ys) is considered as an equivalent virtual cell with SCS μ-log2(Xs) and slot-based capability, where a slot group for the serving cell is considered as a slot for the virtual cell - Alt. 2-2: A serving cell with SCS μ and multi-slot-based capability is considered as an equivalent virtual cell with SCS μ=3 and slot-based capability, where 4/8 slots for the serving cell with SCS μ=5/6 is considered as a slot for the virtual cell As another alternative, the serving cell with SCS µ and multi-slot-based capability can be transformed to an equivalent virtual serving cell with SCS µ’ and slot-based capability, e.g. e.g. cell A with 480KHz SCS and BD/CCE budget per 4 slots is equivalent to a virtual cell A’ with 120KHz and BD/CCE budget per slot. After this, legacy operation as NR Rel-16 could be reused to calculate the BD/CCE budget. By comparing the above alternatives, Alt. 2 works well with less spec impact. Thus, Alt. 2 is preferred to handle multi-slot PDCCH monitoring capability for multi-serving cell case. Proposal 3: For multi-cell operation, support the following method to handle multi-slot PDCCH monitoring capability for multi-serving cell case, i.e. transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation.

### R1-2201352 (CATT)

In RAN1#107-e meeting, the definition for multi-slot PDCCH monitoring capability was agreed. The remaining issue on SS overbooking across different slot groups requires further study. Since Group (2) SS monitoring locations can be anywhere within a slot group, the total number of BD/ CCE for those Group(2) SSs within the current slot group and SSs within the next slot group may exceed the BD/ CCE budgets for a slot group, as shown in Figure 3. In our view, the SS overbooking across different slot groups can be avoided by gNB implementation. No additional dropping rule is needed. Figure 3: SS overbooking across different slot groups Proposal 3: It can be up to gNB implementation to avoid the overbooking issue across different slot groups.

### R1-2201389 (ZTE, Sanechips)

- Proposal 3: Confirm the working assumption: BD/CCE budget of 960 kHz for (4,2), (4,1) is half that of X=8. Table 1: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell. μ; (4, 1); (4, 2); (8, 1); (8, 4). 5; 20; 20; -; -. 6; 10; 10; 20; 20. Table 2: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (4, 1); (4, 2); (8, 1); (8, 4). 5; 32; 32; -; -. 6; 16; 16; 32; 32. In principle, we think serving cells with the same PDCCH monitoring type including multi-slot-based capability should be grouped together for further BD/CCE budget calculation. Since the value of Ys in (Xs, Ys) does not impact the maximum BD/CCE budget, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget. Proposal 10: For a group of serving cells with multi-slot PDCCH monitoring capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget.

### R1-2201471 (NTT DOCOMO)

Since RAN1 has agreed that overbooking is not allowed for CSS even for multi-slot PDCCH monitoring, by defining the maximum number of CCEs as 16, the available configurations for CSS monitoring are restricted. Since type0-PDCCH monitoring can consume all the BD/CCE budgets in a slot group with Xs=4, a UE cannot monitor other SSs including other CSSs than type0-PDCCH CSS in the same slot group, or configurations, e.g., number of CCEs or aggregation level for other SSs would be limited. In addition, at RAN1#107-e meeting, it was pointed out by companies that if two consecutive slots in the same slot group are monitored for type0-PDCCH CSS of SSB/CORESET#0 multiplexing pattern 1, the required number of BD/CCE would be at most 14/56 considering the above requirement for type0-PDCCH CSS monitoring, and thus UE may not be able to monitor the CSS with the BD/CCE budget for Xs=4/8 slots for SCS 480/960 kHz (i.e., 20 BD and 32 CCE). As a result, it was agreed that the monitored slots for type0-PDCCH CSS is n0 and n0+X0 slots to distribute the monitoring occasions for the CSS into different slot groups. In that sense, it seems fair to ensure enough PDCCH candidates/CCEs configuration flexibility even for Xs=4 slots for 960 kHz SCS which is supported as an optional UE capability. Therefore, regarding maximum number of CCE for 960 kHz SCS, we suggest considering larger value for Xs=4 slots, e.g., 28 CCE as maximum, than the simply halved value from that for Xs=8 slots. Text Proposal #1 -------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> ##### 10.1 UE procedure for determining physical downlink control channel assignment … Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell. Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (4, 1); (4, 2); (8, 1); (8, 4). 5; 32; 32; -; -. 6; 16 [28]; 16[28]; 32; 32. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------. At the last RAN1 meeting, some companies pointed out that BD/CCE budget for multi-cell operation, MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ, need to be specified since the definitions are unclear while they are used in TS 38.213. According to the specification, for Rel-16 URLLC, the total number of BD/CCE budget MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ is determined to be shared for the cells which are configured with same SCS and same (X, Y) value for span PDCCH monitoring, which should be considered as a baseline. For multi-slot PDCCH monitoring in Rel-17, considering that Ys does not have any impact on maximum BD/CCE budget for multi-slot PDCCH monitoring, the total number of BD/CCE can be shared for the cells which have same Xs configuration for multi-slot PDCCH monitoring. In addition, some companies proposed that the total BD/CCE budget can be shared for the cells which have same duration for BD/CCE budget, i.e., 1 slot for 120 kHz SCS and 4/8/ slots for 480/960 kHz SCS. However, it means that the total BD/CCE budget can be shared between the cells which have different SCSs (i.e., slot based PDCCH monitoring for 120 kHz SCS and multi-slot based PDCCH monitoring for 480/960 kHz SCS may share the BD/CCE budget) and may result additional complexity. Proposal 2: The total number of BD/CCE for multi-cell operation MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ is determined to be shared between the cells using the same SCS configuration and the same Xs for multi-slot PDCCH monitoring.

### R1-2201593 (TCL Communication)

The Multiple-Cell PDCCH monitoring capabilities have not reached any agreement in the last RAN1 meeting. In FR1 and FR2-1, the multiple-cell PDCCH monitoring budget are allocated to each cell proportionally. In FR2-2, the allocation of the multiple-cell PDCCH monitoring budget should follow same spirit. Proposal 4: In FR2-2, the allocation of the multiple-cell PDCCH monitoring budget should follow the same spirit as that in FR1 and FR2-1. The multiple-cell PDCCH monitoring budget are allocated to each cell proportionally. For FR2-2, the PDCCH monitored is monitored in slot group in 480KHz and 960KHz SCS. The MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ are noted in the CR for 38.213 [3]. The MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ can be are replaced by MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ respectively for μ=5 and μ=6. The underlying problem is how to map PDCCH monitoring capability of one active BWP of a cell with (X, Y) to the PDCCH monitoring in the time unit of a slot. The PDCCH monitoring capability for 120KHz SCS can be a reference when determining the mapping scheme. One feasible way is to consider the value of “X” in the (X, Y) pair. When the candidate values of MPDCCHmax,(Xs, Ys),μ and CPDCCHmax,(Xs, Ys),μ are determined, only the values of “X” is concerned in [4]. Proposal 5: The PDCCH monitoring capability for 120KHz SCS, in terms of maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP and maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per slot for a DL BWP , can be a reference when determining the mapping scheme from MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ to MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ.

### R1-2201663 (Nokia, Nokia Shanghai Bell)

We propose to convert the working assumption to an agreement. At the same time, we should agree the exact values for the BD/CCE budgets for different multi-slot PDCCH monitoring scenarios. We provide a proposal for the values in Table 1. Table 1. Proposed valus for BD / CCE budget per slot SCS; (X,Y); BD budget per X slots; CCE budget per X slots. 480 kHz; (4,1); 20; 32. 480 kHz; (4,2). 960 kHz; (8,1). 960 kHz; (8,4). 960 kHz; (4,1); 10; 16. 960 kHz; (4,2). Proposal 1: The maximum number of monitored PDCCH candidates per X slots for a single serving cell is 20 - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,1) - For SCS 960 kHz: (X,Y) = (8,4) Proposal 2: The maximum number of non-overlapped CCEs per X slots for a single serving cell is 32 - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,1) - For SCS 960 kHz: (X,Y) = (8,4) Proposal 3: The maximum number of monitored PDCCH candidates per X slots for a single serving cell is 10 - For SCS 960 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (4,2) Proposal 4: The maximum numberof non-overlapped CCEs per X slots for a single serving cell is 16 - For SCS 960 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (4,2)

### R1-2201689 (Intel)

The following TP is proposed to capture additional X value 2 for SCS 480kHz. ------------------------------ TP#1: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** Table 10.1-2B provides the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell. Table 10.1-2B: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 10; 20; 20; -; -. 6; -; 10; 10; 20; 20. *** Unchanged text is omitted *** Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell. Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 16; 32; 32; -; -. 6; -; 16; 16; 32; 32. *** Unchanged text is omitted ***. Proposal 1: - X=2 can be optionally supported for SCS 480kHz, which corresponds to combination (X, Y) = (2, 1). The BD/CCE budget for (2,1) can be half that of X=4 - Agree on TP 1 to capture additional X value 2 for SCS 480kHz When CA is considered, the total number of BD/CCEs that are shared by multiple cells/TRPs. MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ need to be determined considering both SCS configuration μ and the combination (X, Y) of the scheduling cells. Three options are discussed as below. - Option 1: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same combination (X, Y). Based on the agreed framework, the slot group always starts from the subframe boundary. Consequently, the slot groups of difference cells must be aligned if same SCS configuration μ are configured. On the other hand, though the group (1) SS is limited to the Y slots, the group (2) SS can be configured in any slot in a slot group. Since the PDCCH monitoring for the two scheduled cells may not be fully aligned due to the arbitrary MOs for group (2) SS, it is not necessary to align the Y slots in the slot group for multiple cells for BD/CCE sharing. Based on the above discussions, option 1 can be considered without alignment of the Y slots. - Option 2: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having SCS configuration μ and same value X in combinations (X, Y). In the option 1, the BD/CCE for the scheduling cells with combination (X, Y) of same value X and different value Y are separately handled. In fact, since the maximum numbers of BD/CCE is only determined by SCS configuration μ and value X, but not the value Y, it is possible to share the BD/CCE for the scheduling cells with same value X of the combinations (X, Y). This option can provide gNB more freedom to share the PDCCH scheduling capability for more cells. - Option 3: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having same duration of slot or slot group. A slot of SCS 120kHz and a slot group of X=4/8 slots for SCS 480/960kHz are fully overlapped in time with same the maximum numbers of BD/CCE. It is then expected the UE implementation on the PDCCH detection could be common in some extent. Consequently, it can be considered to share the BD/CCE for the scheduling cells with SCS 120kHz and for SCS 480/960kHz if X=4/8 slots apply. Option 3 can provide even more flexibility on PDCCH transmission than Option 2. However, Option 3 also requires UE to jointly share maximum BD/CCE budget across cells with different numerology, which results in additional complexity. Based on the above analysis, we prefer Option 2 since it provides a good balance between flexibility and complexity. The following TP is proposed to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ. ------------------------------ TP#3: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** If a UE - does not report pdcch-BlindDetectionCA or is not provided BDFactorR, γ=R - reports pdcch-BlindDetectionCA, the UE can be indicated by BDFactorR either γ=1 or γ=R If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ of the Ncells,0DL,μ+Ncells,1DL,μ downlink cells using combination Xs,Ys with same Xs for PDCCH monitoring, where μ=06Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, per a group of Xs slots on the active DL BWPs of the scheduling cells, - more than MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for each scheduled cell when the scheduling cell is from the Ncells,0DL,(Xs,Ys),μ downlink cells, or - more than MPDCCHtotal,(Xs,Ys),μ=γ∙MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=γ∙CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for each scheduled cell when the scheduling cell is from the Ncells,1DL,(Xs,Ys),μ downlink cells - more than MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,(Xs,Ys),μ downlink cells If a UE - is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and - with Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ of the Ncells,0DL,μ+Ncells,1DL,μ downlink cells using combination Xs,Ys with same Xs for PDCCH monitoring, where μ=06Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and - a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor, per a group of Xs slots on the active DL BWP with SCS configuration μ of the scheduling cells - for the Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ, downlink cells more than MPDCCHtotal,(Xs,Ys),μ=Ncellscap⋅MPDCCHmax,(Xs,Ys),μ⋅Ncells,0DL,(Xs,Ys),μ+γ∙Ncells,1DL,(Xs,Ys),μj=06Ncells,0DL,(Xs,Ys),j+γ∙Ncells,1DL,(Xs,Ys),j PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=Ncellscap⋅CPDCCHmax,(Xs,Ys),μ⋅Ncells,0DL,(Xs,Ys),μ+γ∙Ncells,1DL,(Xs,Ys),μj=06Ncells,0DL,(Xs,Ys),j+γ∙Ncells,1DL,(Xs,Ys),j non-overlapped CCEs. - for each scheduled cell from the Ncells,0DL,(Xs,Ys),μ downlink cells, more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs. - for each scheduled cell from the Ncells,1DL,(Xs,Ys),μ downlink cells, - more than minγ∙MPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minγ∙CPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs - more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs for CORESETs with same coresetPoolIndex value *** Unchanged text is omitted ***. Proposal 8: - MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ needs to be determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same value X in combinations (X, Y). - Agree on TP 3 to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ Proposal 6: - UE should be able to process one broadcast DCI for SI/RACH/paging in addition to the agreed number of processed unicast DCI in a slot group of X slots. - To clarify whether a UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell with SCS 15kHz - The limitation on number of detected DCIs in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz It is desirable if gNB can provide proper SS set configuration so that the total number of spans for all SS sets in any slot group does not exceed the defined maximum number of spans per slot group. However, there is also the case that the maximum number can be exceeded in a slot group. Further, if the SS/PBCH block for the UE is changed, the slot/symbol position of the SS set with searchSpaceId = 0 moves accordingly, which may impact the total number of spans in a slot group. A span of SS sets configured in a slot group, if it is not monitored by the UE based on a semi-static rule, may not be counted in the number of spans in the slot group. If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of at least group (1) SS sets so that the maximum number is not exceeded in the slot group. Proposal 5: - A span of SS sets configured in a slot group, if it is not monitored by the UE based on a semi-static rule, may not be counted in the number of monitored spans in the slot group. - If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of at least the group (1) SS sets

### R1-2201735 (Ericsson)

The two working assumptions Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8 Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. have been captured and approved in TR 38.213 [3]. In our view, both can be considered confirmed. In the following text from 38.213 Section 10.1 [3], the quantities MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ are used but not defined: The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, or in a group of Xs slots for a corresponding combination Xs,Ys if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(Xs,Ys),μ and CPDCCHmax,(Xs,Ys),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ respectively. The quantities can be defined following the same Rel-15 PDCCH monitoring capability allocation for carrier aggregation cases. The main difference for Rel-17 is that, in addition to difference in numerology μ, different cells may be configured with different Xs,Ys. Since slot groups start at a subframe boundary, the slot groups for serving cells with identical μ and Xs are aligned. The configuration of Ys does not impact the slot group alignment between different serving cells. Note further that MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ depend on Xs, but not Ys - In our view, the distribution of available BD/CE budgets amongst multiple cells can be properly done based on the grouping of numerology μ and configuration of Xs of cells as follows. If the number of cells is no more than than Ncellscap, MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,Xs,Ys,μ and CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,Xs,Ys,μ. - If the number of cells is more than than Ncellscap, the number of blind decodes Ncellscap⋅MPDCCHmax,Xs,Ys,μ is distributed to MPDCCHtotal,(Xs,Ys),μ based on a weighting of number of cells with numerology μ among all cells configured with the same Xs; and the total number of available channel estimation CCEs Ncellscap⋅CPDCCHmax,Xs,Ys,μ is distributed to CPDCCHmax,Xs,Ys,μ based on a weighting of number of cells with numerology μ among all cells configured with the same Xs. Both quantities are still upper bounded by MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, respectively. There is no need to further sub-divide this grouping based on the configuration of Ys. Furthermore, conversion of numerology μ and configuration of Xs to equivalent numerology with slot-based monitoring, as proposed by some sources, appears unnecessary optimization that complicates and impedes stabilization and finalization of the Rel-17 specs. For per-slot group monitoring, MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ should be defined in TS 38.213 following the same PDCCH monitoring capability allocation for carrier aggregation cases for the case of per-slot monitoring, except the grouping is based on numerology μ and configuration of Xs. Recommend TP#2 to the spec editor which defines how to allocate the BD/CCE budget to 480/960 kHs SCS cells configured for per-slot group monitoring. Note: this still requires that for cells with SCS ≤ 120 kHz configured for per-slot or per-span monitoring, adjustment of the Rel-15/16 allocation formulas will be needed. It is not needed to specify that within a group of Xs slots, the UE is not be expected to process information from more than one DCI format with CRC scrambled by a given RNTI associated with Group (2) SSs, i.e., SI-RNTI, P-RNTI, RA-RNTI, MsgB-RNTI.

### R1-2201765 (Apple)

Proposal 4: Accept the working assumption Based on the working assumption, the BD/CCE budget for X = 4 with 960 kHz, ie. (X,Y) = {(4,2), (4,1)} is half that of X=8 i.e. BD/CCE = 10/16. - The gNB should configure the aggregation levels to match the CCE budget e.g. only AL 4 and 8 allowed. Proposal 5: If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, or ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per multi-slot Proposal 7: Additional dropping rules for PDCCH multi-slot monitoring should be defined to limit back-to-back SS monitoring between Group 1 and Group 2 SSs across multiple slot-groups. A window-based approach around the CSS maybe considered to determine whether or not a USS monitoring occasion is dropped or not for PDCCH monitoring Proposal 11: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget

### R1-2201914 (Xiaomi)

From previous agreement, UE has a mandatory PDCCH monitoring capability and can also report an optionally supported capability to gNB. So it would be possible that gNB may configure optional PDCCH monitoring capability on some serving cells but leave other cells with mandatory capapbility. And there can be different SCS on different serving cells, so the applied multi-slot PDCCH monitoring capability can also be different. From our point of view, since we have very few time left, it is more preferred not to do PDCCH monitoring capability sharing among multiple serving cells. When UE is configured with multiple serving cells, within each serving cell, UE apply multi-slot PDCCH monitoring capability for a single serving cell independently. Proposal 1: When UE is configured with multiple serving cells, within each serving cell, UE apply multi-slot PDCCH monitoring capability for a single serving cell independently. Related TP can be as follows, TP#1 for TS 38.213 Clause 10 ============================= Unchanged part omitted ========================================= 10 UE procedure for receiving control information If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG - When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively. - When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG. A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies decoding each PDCCH candidate according to the monitored DCI formats. If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per multi-slot, as in Tables [xxx,TBD], if monitoringCapabilityConfig = r17monitoringcapability For FR1 and FR2-1, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For FR2-2, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per X multi-slot as in Tables [xxx,TBD], with X=4 for SCS configuration of and X=8 for SCS configuration of . For FR2-2, if the UE is configured with multiple serving cells, within each serving cell, UE apply multi-slot PDCCH monitoring capability for a single serving cell independently. A UE can indicate a capability to monitor PDCCH according to one or more of the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination X,Y, the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to Y. ============================= Unchanged part omitted =========================================

### R1-2202005 (Samsung)

When a UE supports CA or NR-DC operation in NR from 52.6 GHz to 71 GHz, UE can be configured with multiple downlink cells for multi-slot based PDCCH monitoring. It’s necessary to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that corresponds to Ncellscap downlink cells, which is in analogy to Ncellscap-r15 for slot-based PDCCH monitoring capability introduced in NR Rel-16 and Ncellscap-r16 for span based PDCCH monitoring capability introduced in NR Rel-17. The legacy mechanism for determine Ncellscap-r16 can be extended to determine a capability of Ncellscap-r17for multi-slot based PDCCH monitoring considering scenarios, such as - Case 1: UE is configured with CA but not NR-DC operation, and UE is configured with only R17monitoringcapability for all downlink cells - Case 2: UE is configured for NR-DC operation, and UE is configured with only R17monitoringcapability for all downlink cells When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per group of Xs slots according to combination Xs,Ys a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17. Proposal 5: Adopt TP#4 for TS 38.213 to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per Xs slots that corresponds to Ncellscap downlink cells, i.e. Ncellscap-r17. It was agreed in RAN1#107-e meeting to support PDCCH dropping per Xs slots based on combination Xs,Ys. The PDCCH dropping rule should be applied to both single cell case and CA. For UE configured with CA or NR-DC operation, the PDCCH candidates and non-overlapping CCE limits should be determined based on Ncellscap-r17downlink cells for all scheduling cells. The same scaling rule to determine MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ for R16monitoringcapability can be reused to determine MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ for R17monitoringcapability in CA mode when the number of downlink cells is larger than Ncellscap-r17. Proposal 6: Adopt TP#5 for TS 38.213 to determine maximum number of PDCCH candidates, MPDCCHtotal,(Xs,Ys),μ, and a maximum number of non-overlapped CCEs, CPDCCHtotal,(Xs,Ys),μ, per Xs slots in CA mode.

### R1-2202072 (MediaTek)

One remaining discussion is the BD/CCE budge for (X,Y)=(4,2) and (4,1) under 960kHz. In our view, the significance and usage of configuration (X,Y)=(4,1) and (4,2) are not clear compared to (8,1) and (8,4) and we prefer to remove such configurations. Furthermore, if the configuration (X,Y)=(4,1) and (4,2) were considered, based on the discussion so far, the associated BD/CCE limit should be around half of the ones for (X,Y)=(8,1) and (8,4). Consequently, the PDCCH scheduling will be impacted, especially when Type-0 PDCCH monitoring is involved where monitoring in consecutive slot-groups is needed. To alleviate the scheduling inflexibility due to the decreased BD/CCE limit, it is not desirable to configure X=8 in Pcell with 960kHz. Proposal 1: For Rel-17 960kHz multi-slot PDCCH monitoring, only (X,Y)=(8,1) and (8,4) are supported in a Pcell. Another essential discussion to complete the multi-slot PDCCH monitoring design is how to extend the framework to multi-cell operation and how to specify the BD/CCE limit for multi-cell operation. To address those aspects, it is necessary to discuss whether the fixed pattern of slot groups is the same across CCs with (X,Y) multi-slot PDCCH monitoring configuration. Based on the RAN1 #106bis-e agreement - The start of the first slot group in a subframe is aligned with the subframe boundary - The start of each slot group is aligned with a slot boundary it can be concluded that all the CCs with (X,Y) multi-slot PDCCH monitoring configuration share the same pattern of slot groups. Observation 1: All the CCs with the same subcarrier spacing 15×2μ and same (X,Y) in multi-slot PDCCH monitoring configuration share the same pattern of slot groups Consequently, it is natural to link the slot group notion in 480kHz or 960kHz and the slot notion in 120kHz, and it is desirable to reuse the multi-cell BD/CCE budget calculation method specified for slot-based PDCCH monitoring when determining the multi-cell BD/CCE budget calculation method for slot-group based PDCCH monitoring. That is, when the number of scheduled cells is less than or equal to the number of cells UE can support, UE is not required to monitor more than the BD/CCE limit per slot-group specified for a single cell on a DL BWP of scheduling cell. When the number of scheduled cells is larger than the number of cells UE can support, the multi-cell BD/CCE budget per slot group across CCs UE will follow can be derived based on the multi-cell BD/CCE budget calculation method specified for slot-based PDCCH. Proposal 3: The Rel-15/16 multi-cell BD/CCE budget calculation method specified for slot-based PDCCH monitoring should be considered as the baseline for the multi-cell BD/CCE budget calculation for slot-group based PDCCH monitoring in 480kHz and 960kHz.

### R1-2202130 (Qualcomm)

In RAN1 #107-e, a relevant issue was discussed in the dynamic spectrum sharing agenda, and the following has been agreed: Agreement - Following approaches for PDCCH monitoring and BD limit handling is supported for Type A UE - Additional simplifications to PDCCH monitoring - Type A UE as per RAN1#105-e agreement and - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’. In Short, if there is a USS sets monitored on the sSCell, the UE is not required to monitor DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in the CSS sets in the overlapping slot on the sPCell. The motivation behind the above agreement is removing redundancy and improving power efficiency. Therefore, in the same vein, when the UE is required to monitor both Group (1) and Group (2) SS sets in the same slot group, if there is at least one valid PDCCH MO of Group (1) SS sets, considering overbooking and dropping, the UE may refrain from monitoring DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in Group (2) SS sets. Proposal 1: If PDCCH MOs of both Group (1) and Group (2) SS sets are configured in the same slot group, and there is at least one valid PDCCH MO of USS set(s) after overbooking and dropping, the UE does not monitor DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, and CS-RNTI in the Group(2) SS set(s). In addition, in Rel-15, the following rule is applied for CSS monitoring within a slot: TS 38.213, Section 10.1: If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.. Thus, a similar design should be extended for multi-slot PDCCH monitoring. Proposal 3: Per slot group of X0 slots, a UE does not expect to process information from more than one DCI format with CRC scrambled by a RNTI from any of SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI. Further related to the joint configuration of multi-slot PDCCH monitoring and SSSG switching, a situation shown in Figure 2 may be considered. That is, SSSG switching occurs from the first SSSG to the second SSSG. Then, at the boundary of SSSG switching, the locations of the two Y consecutive slots in the slot groups before and after the switching boundary may be different. In another case, the two SSSGs may be associated with different (Xs,Ys) values according to Proposal 8. Thus, in such cases, the separation between the two Y consecutive slots may be less than Xs slots. To avoid such a situation in Figure 2, a rule for dropping PDCCH MOs may be considered. For example, when the separation between the two Ys consecutive slots before and after the switching boundary is less than Xs slots, where Xs is according to the first (source) SSSG, some or all of the MOs in the Ys consecutive slots in the second (target) SSSG after the switching boundary may be dropped. Proposal 10: A dropping rule for PDCCH MOs may be applied for the first Ys consecutive slots after SSSG switching, if the separation between the two Ys consecutive slots before and after the SSSG switching boundary is less than Xs slots. Figure 2: An example of SSSG switching for multi-slot PDCCH monitoring. In RAN1 #107-e, the following has been agreed: Agreement - SS set overbooking can be allowed with multi-slot PDCCH monitoring capability same as the current specification but applied per slot group, i.e., SS set overbooking is allowed for USS in PCell and PSCell, and UE expects no overbooking for CSS in PCell and PSCell and no overbooking in SCell. - The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group, i.e., a UE drops UE specific search space set(s) in a slot group with higher index when SS sets are overbooked. - Additional dropping rules are not precluded. For the detailed dropping rules in the third bullet of the agreement, two alternatives were identified: If a SS set has multiple monitoring occasions within a slot group, - Alt 1: All MOs of the SS set shall be dropped as a whole​. - Alt 2: Each MO of the SS set shall be dropped individually​. An example showing the difference of the two alternatives is illustrated in Figure 3. Figure 3: Alternatives of dropping rules. Between the two alternatives, Alt 2 has a benefit over Alt 1 in some cases, like the one shown in Figure 3. That is, Alt 2 may accommodate more PDCCH candidates than Alt 1 under the same BD/CCE budget. However, as an expense, the overbooking procedure may get complicated. Furthermore, the impact of Alt 1 on the existing specification is marginal, since it is a simple extension of Rel-15 overbooking. On the other hand, Alt 2 may require some changes of the specification. Therefore, to keep the design simple and reduce the burden of standardization, Alt 1 would be preferred. Proposal 11: If a SS set to be dropped by overbooking has multiple MOs within a slot group, they are dropped as a whole.

### R1-2202190 (Sharp)

In RAN1#107-e meeting, the working assumption for SSSG switching at 120/480/960kHz is achieved. The working assumption was directly described in 38.213, and the minimum switching time Pswitch was defined as one value for each SCS. Here, we have one concern about the SSSG switching at 480kHz/960kHz, which is that it may cause a back-to-back problem when switching. It is important to avoid this problem, which we have been considering for a long time in past meetings. In the current specification, SSSG-related monitoring is stopped and started at the same time in the first slot after the Pswitch. However, in multi-slot monitoring, SSSG switching may be performed after the UE has monitored in a later slot of the slot group, and monitoring may start in the slot immediately after. In this case, a back-to-back problem occurs. To avoid this situation, it is possible to not monitor the Xs slots before and after the SSSG boundary. This can be achieved by dropping PDCCH candidates in the Xs slots before or after the switchover, or by setting different Pswitch values to stagger the stop and start of monitoring. Figure 1:Possible problem with SSSG switching with multi-slot monitoring. Proposal 3: To avoid the back-to-back problem, monitoring should not be done in the Xs slots before and after the SSSG boundary. Proposal 4: Adopt Text proposal #2-1 or #2-2.

### R1-2202234 (Transsion Holdings)

Proposal 1: Confirm the working assumption that the maximum number of PDCCH candidates and the maximum number of non-overlapped CCEs should be 10 and 16 for combination (4, 2), (4, 1). In the current specification, slot-group based PDCCH monitoring capability has been specified and the overbooking rule is performed on slot group basis. With these methods, UE can handle the slot-group based PDCCH detection very well. However, when the PDCCH monitoring occasions change, the UE may need to perform PDCCH detection on consecutive slots belonging to different slot groups, which may result in the UE being unable to perform blind detection in time. For example, for SS/PBCH block and CORESET#0 multiplexing pattern 1, the PDCCH monitoring occasions are located within two consecutive slot groups which are associated with the SS/PBCH block. However, when the UE changes SS/PBCH block it tracks due to mobility, the relevant PDCCH monitoring occasions may be changed based on the slot index of n0. In a special case, for the Type0 CSS or Type0A/Type 2 CSS if “searchSpaceId=0”, the PDCCH monitoring occasions may be changed to the end of the slot group. If the PDCCH monitoring occasions of the Group (1) SS are located in the first slot of an adjacent slot group, then the requirement for the PDCCH detection capability of the UE is almost doubled, which may exceed the BD/CCE limit of the UE. Considering that it is difficult for gNB to avoid this back-to-back issue, it is better to drop the USS in these adjacent slot groups to ensure that the UE’s PDCCH detection budget does not exceed its BD/CCE limit. Proposal 2: Drop the USS with the higher index in these adjacent slot groups, when back-to-back issue happens.

### R1-2202336 (LG)

Regarding the BD/CCE budget for (X,Y) combinations with optional X, there is a working assumption for 960 kHz SCS. Considering UE implementation complexity and power consumption, current working assumption seems reasonable. There were concerns at the last meeting that the CCE budget of (X,Y) with X=4 for 960 kHz is too small to cover the Type-0 CSS monitoring budget, but from our understanding, such (X,Y) combination with optional X doesn't seem to be an issue when they applied to SCell. So, we support changing the above working assumption to an agreement. Similarly, BD/CCE budget for (X,Y) with X=2 can be simply defined as half that of X=4, for 480 kHz SCS. Proposal #2: Determine the BD/CCE budget for (X,Y) combination with optional X is half that of mandatory X. - For 960 kHz, BD/CCE budget for (X,Y) with X=4 is half that of X=8, i.e., confirm the WA. - For 480 kHz, BD/CCE budget for (X,Y) with X=2 is half that of X=4, if supported. According to this, the serving cells with the same PDCCH monitoring type can be grouped together for the BD/CCE budget distribution. That is, the serving cells for slot-based monitoring or span-based monitoring are treated as separate groups when distributing the BD/CCE budget. However, if three different monitoring capabilities associated to a carrier aggregation including multi-slot monitoring, the number of separate groups will be increased and the BD/CCE distribution mechanism becomes complicated, which may increase the complexity of the UE. Meanwhile, for the mandatorily supported X=4/8 for 480/960 kHz, the absolute time of the slot-group and the BD/CCE limit per X=4/8 (resp.) are the same as those for 120 kHz. In this case, when the multi-slot monitoring is configured with multiple serving cells with X=4/8 slots for 480/960 kHz, the BD/CCE budget distribution for serving cells with these SCS can be calculated as if they were serving cells with 120 kHz SCS. With this, BD/CCE budget distribution for CA can be simplified. Additionally, the number of DL serving cells for multi-slot monitoring may be reported by pdcch-BlindDetectionCA, whose value indicates total the number of DL serving cells for both single-slot based monitoring and multi-slot based monitoring. Through this, the UE will be able to further simplify BD/CCE budget distribution for the multi-cell operation. Proposal #13: For multi-cell operation, the number of configured DL cells is greater than the number of reported DL cells, consider followings for BD/CCE budget distribution, - For a serving cell with mandatory X (i.e., X=4/8 for 480/960 kHz), BD/CCE budget is calculated by transforming the serving cell to the cell with 120 kHz SCS. - For a serving cell with optional X (e.g., X=4 for 960 kHz), the serving cells with the same SCS and Xs value are grouped together for BD/CCE budget distribution.

### R1-2202409 (Lenovo, Motorola Mobility)

From the above agreement, there could be the issue of back-to-back monitoring across slot groups e.g. when the beam changes, e.g. with respect to the above proposal when n0 changes and also when the periodicity of Group(2) SS MO is every slot group. Observation 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, when multi-slot PDCCH monitoring is applied with shifting of Group(2) SS due to n0 change, then potential back-to-back monitoring issue can arise across slot groups where the shift is applied, and periodicity of Group(s) SS MOs is every slot group One potential solution could be to drop any Group(1) SS MOs and/or Group(2) SS MOs in the slot group in which shifting needs to be applied such that back-to-back monitoring issue can be avoided. Proposal 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, then dropping of Group(1) SS MOs and/or Group(2) SS MOs in the slot where the shift is first applied should be supported to avoid back-to-back monitoring issue.

## Topic B: Multi-Beam Aspects

### R1-2201266 (OPPO)

Since gNB can perform directional LBT and RAN1 has agreed that the sensing beam should cover the transmission beams, it implies that the transmission beams within the initiated gNB COT should be covered by the sensing beam. Moreover, the remaining COT duration indication in DCI 2_0 was used in R16 for the UE to determine whether the UE can share the gNB COT and also whether the UE should perform reception on the pre-configured downlink transmissions, e.g. CSI-RS. Thus, it would be more reasonable that the COT sharing as well as the legacy P-CSI-RS reception behavior should be checked by the sensing beam or the transmission beams. For COT sharing mechanism, in the legacy system, the UE can share gNB’s COT as long as the uplink transmission resources in time and frequency domain are within the gNB COT. However, when directional LBT is applied at gNB side, a beam level condition should be added such that the gNB LBT sensing beam should cover the UE’s transmission beam, otherwise, the gNB COT should not be considered sharable by the UE. The ‘cover’ definition can reuse the same definition in the directional LBT procedure. Proposal 2: the UE can share gNB COT only if the UL transmission resources are within the gNB COT and the UL transmission beam is covered by the gNB sensing beam for directional LBT. For P-CSI-RS reception within the gNB COT, in NRU system due to the FR1 frequency range, the UE only checks whether the CSI-RS resources are within the gNB COT to decide if the reception is to be canceled or not. But for FR2-2, if the gNB’s sensing beam does not cover the CSI-RS beam, the CSI-RS is not expected to be transmitted. In this case, if the CSI-RS resources are within the gNB’s remaining COT duration, as long as the gNB sensing beam does not cover the CSI-RS beam, the UE should also cancel the CSI-RS reception. Proposal 3: For higher layer configured CSI-RS reception, the UE performs the reception if the CSI-RS resources are within the gNB COT and the gNB’s sensing beam covers the CSI-RS beam. In the unlicensed spectrum, when COT duration indication is not configured in DCI 2_0, the COT duration is determined by the SFI periodicity. Moreover, the higher layer configured downlink reception, e.g. CSI-RS and PDSCH, is to be confirmed by the SFI indication. In FR2-2 with unlicensed spectrum where LBT is mandatory by the regional regulation, similar to the proposal 2, the confirmation should take into account the gNB sensing beam, because if the sensing beam does not cover the transmission beam, the pre-configured reception should also be canceled, as the gNB is not allowed to transmit in the direction uncovered by the directional LBT. Proposal 4: In FR2-2 unlicensed band, the pre-configured downlink reception is not only confirmed by the SFI indication but also by gNB’s sensing beam, e.g., UE should cancel the downlink reception within the gNB COT if the gNB sensing beam does not cover the downlink transmission beam. When it comes to SSSG switching, the motivation of introduction of this feature is to allow UE to reduce the PDCCH monitoring effort in the gNB COT. With the similar reasoning as described above, when the transmission beam is not covered by the gNB sensing beam, the gNB is not allowed to perform this transmission within the gNB COT. Therefore, if a CORESET beam is not covered by the gNB sensing beam, the UE should be allowed to skip the PDCCH monitoring in the CORESET. Proposal 5: R17 should allow UE to skip PDCCH monitoring in the CORESET associated with a beam uncovered by the gNB sensing beam within the gNB COT.

## Topic C: Multi-Cell Operation, Cross-carrier scheduling (except BD aspects)

### R1-2200953 (Huawei, HiSilicon)

The benefit to support scheduling between two carriers with large μPDCCH-μPDSCH is questionable. For example, for μPDCCH=1 and μPDSCH=6, a DCI with 15 kHz SCS would schedule 64 PDSCH slots with 960 kHz SCS. In such a case, the flexibility is poor and the latency is large. Therefore, we support to reuse the same limitation as in FR1 and FR2-1, i.e., μPDCCH-μPDSCH≤3. Proposal 15: Cross-carrier scheduling is only supported for μPDCCH-μPDSCH≤3.

### R1-2201352 (CATT)

In the RAN1#107bis-e, the maximum difference of SCS of cross-carrier scheduling, i.e.,uPDCCH-uPDSCH, has been discussed. In the Rel-16, the maximum difference of SCS is equal to 3. However, 480 kHz SCS and 960 kHz SCS have been supported for 52.6-71GHz. We believe the maximum difference of SCS of cross-carrier scheduling should be expanded to support of new SCS, and there is no motivation to limit the difference of SCS of cross-carrier scheduling. Proposal 5：In order to better support cross-carrier scheduling of the new SCS, i.e. 480 kHz and 960 kHz, the difference of SCS of cross-carrier scheduling should not be limited.

### R1-2201765 (Apple)

Proposal 9: For Rel-17, 4 additional cases for UE capability signaling need to be defined: - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - pdcch-BlindDetectionCA-R17 is equal to 4 {similar to Rel-15} - Case 5: Capability on the number of CCS with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: [1:15] - The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: [4:16] - Case 6: Capability on the number of CCS with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: [1:15] - The minimum of pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 3 - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: [3:16] - Case 7: Capability on the number of CCS with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-17 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: [1:15] - The minimum of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + The minimum of pdcch-BlindDetectionCA-R17) is equal to 4 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : [4:16] Proposal 10: For the case with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells (case 7) or any combination of 2 of the capabilities (i.e. case 5, and case 6), the UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) as UE capability. If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17), as in Rel-16, the gNB configures which combination for the UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability.

### R1-2202072 (MediaTek)

The other open issue is the location of the Y slots within a slot group of X slots across CCs. Compared with the fixed location of the Y slots within a slot group across CCs, non-aligned locations of Y slots can provide potential scheduling flexibility with the cost of less power saving. For example, without any restriction, it is possible that the monitoring slots in a cell with (X,Y) multi-slot PDCCH monitoring configuration might be close to the monitoring slots in another cell with the same (X,Y) multi-slot PDCCH monitoring configuration, which is illustrated in Figure 1. In this example, both CCs follow (X=4,Y=1) multi-slot PDCCH monitoring configuration and UE has to monitoring PDCCH in different slots across CCs without any chance of micro-sleep due to the non-aligned PDCCH monitoring pattern. On the other hand, if the PDCCH monitoring pattern is aligned across CCs, UE will monitor the same slots across CCs and improve PDCCH monitoring power consumption accordingly. Proposal 4: For multi-cell operation, UE can report a capability on whether the location of the Y slots within a slot group of X slots is maintained across CCs associated with (X,Y) configuration. Figure 1 Example of non-aligned monitoring pattern across CCs

## Topic D: Other

# List of submitted TDocs

The following TDocs have been used to compile above summary:

R1-2200953	Remaining issues of PDCCH monitoring enhancement for 52-71GHz spectrum	Huawei, HiSilicon

R1-2200988	On the remaining issues in multi-slot PDCCH monitoring  for Beyond 52.6GHz	FUTUREWEI

R1-2201033	Remaining issues for PDCCH monitoring enhancements	InterDigital, Inc.

R1-2201086	Remaining issues on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2201266	Discussion on remaining issue for PDCCH monitoring enhancement	OPPO

R1-2201352	Remaining issues on PDCCH monitoring enhancements for up to 71GHz operation	CATT

R1-2201389	Remaining issues on the PDCCH monitoring enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2201471	Remaining issues on PDCCH monitoring enhancements for NR in FR2-2	NTT DOCOMO, INC.

R1-2201542	Remaining issues on the PDCCH monitoring enhancements for 52.6 to 71GHz	Spreadtrum Communications

R1-2201593	Remaining Issues on PDCCH onitoring Enhancements in FR2-2	TCL Communication

R1-2201663	PDCCH monitoring enhancements	Nokia, Nokia Shanghai Bell

R1-2201689	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2201735	PDCCH Monitoring Enhancements	Ericsson

R1-2201765	On remaining issues for PDCCH Monitoring	Apple

R1-2201899	Remaining issues on PDCCH enhancement for NR operation from 52.6GHz to 71GHz	NEC

R1-2201914	Remaining issues on PDCCH monitoring enhancement for NR 52.6-71GHz	Xiaomi

R1-2202005	Maintenance on PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2202072	Remaining discussion on PDCCH monitoring enhancement for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2202130	PDCCH monitoring enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2202190	PDCCH monitoring enhancements	Sharp

R1-2202234	Remaining issues of PDCCH monitoring enhancements for above 52.6GHz	Transsion Holdings

R1-2202273	PDCCH monitoring for NR operation from 52.6 to 71 GHz	Panasonic

R1-2202336	PDCCH monitoring enhancements to support NR above 52.6 GHz	LG Electronics

R1-2202409	Remaining issues on PDCCH for NR from 52.6 GHz to 71GHz	Lenovo