3GPP TSG RAN WG1 #109-e		  R1- 2203322

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.1.2

Source:	Spreadtrum Communications

Title:	Discussion on CSI enhancement for coherent JT

Document for:	Discussion and decision

Introduction

In RAN#94 meeting, a new WI on MIMO Evolution for Downlink and Uplink in Rel-18 has been approved [1]. In this WI, CSI enhancement for coherent JT is part of the objectives, which can be found below.

Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we provide our view on CSI enhancement for coherent JT.

Discussion

In Rel16 and Rel-17, NCJT has been supported for DL as well as UL to increase user throughput especially for cell edge UEs. In Rel-17, CSI measurement and reporting for NCJT has been discussed and specified. UE is able to measure CSI-RS resources transmitted from two TRPs, estimate the CSI parameters for each TRP and report them together. During the CSI calculation, the measured channel corresponding to one TRP is considered as interference for another TRP, and only Type I single panel codebook is supported.

Coherent JT, as a new multi-TRP transmission scheme, can be considered as one potential enhancement to achieve better performance than NCJT. The coherence of antennas from multiple TRPs allows the NW to generate a single precoding matrix, where each TRP is considered as part of the data channel rather than interference. According to the Rel-18 MIMO WID, it is clear that the codebook design for coherent JT will take Rel-16/17 Type-II codebook refinement as a starting point.

Similar with NCJT, the target of coherent JT is also to increase user throughput for cell edge UEs. The maximum supported rank for NCJT is 4. Besides, the maximum supported rank for Rel-16/17 Type-II codebook is also 4. Therefore, we suggest to limit the maximum supported rank for Type II codebook for coherent JT as 4.

Proposal 1: Limit the maximum supported rank for Type II codebook for coherent JT as 4.

In order to enable channel measurement on multiple TRPs, NW needs to configure CSI-RS resource to UE. There are two possible ways to associate CSI-RS resource with different TRPs. The first way is to allow one to one mapping between CSI-RS resource and TRP, which means each TRP will transmit an individual CSI-RS resource. Another way is to allow a single CSI-RS resource being associated with all TRPs, which means different TRP will transmit different ports of the same CSI-RS resource.

According to TS 38.211, it is assumed that the antenna ports within a CSI-RS resource are quasi-colocated with QCL Type A, Type D (when applicable), and average gain. In order to not break the assumption, we prefer to configure different CSI-RS resources for different TRPs.

Proposal 2: Regarding the channel measurement for coherent JT, UE can be configured with different CSI-RS resources for different TRPs.

According to the Type II codebook structure, for W1, it is assumed that the antenna ports are equally spaced in two dimensions to form the 2D DFT basis. For coherent JT, since Type II codebook structure is reused as starting point, it is reasonable to assume that each selected SD basis is constructed by antenna ports from a single TRP. Besides, for W2 and Wf calculation, two different alternatives can be considered. The first one is per TRP operation, which means the FD basis selection and/or differential quantification are performed per TRP. The second alternative is across TRP operation. If the strong FD bases for different TRPs are assumed to be within a single intermediate FD basis subset, FD basis selection across TRPs can be considered. Besides, if the value range of the coefficient amplitudes corresponding to different TRPs are similar, differential quantification can be performed across TRP.

Proposal 3: For Type II codebook enhancement for coherent JT,

  - Each SD basis is constructed by antenna ports from a single TRP

  - Regarding FD basis selection and combinatorial coefficient reporting, consider either per TRP operation or across TRP operation

Conclusion

In this contribution, we provide our view on CSI enhancement for coherent JT. The following proposals are achieved:

Proposal 1: Limit the maximum supported rank for Type II codebook for coherent JT as 4.

Proposal 2: Regarding the channel measurement for coherent JT, UE can be configured with different CSI-RS resources for different TRPs.

Proposal 3: For Type II codebook enhancement for coherent JT,

  - Each SD basis is constructed by antenna ports from a single TRP

  - Regarding FD basis selection and combinatorial coefficient reporting, consider either per TRP operation or across TRP operation

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94, Samsung (Moderator)