3GPP TSG RAN WG1 #109-e		R1-2203362

e-Meeting, May 9th – 20th ,2022

Title: 	Additional consideration on inter-UE coordination

Source: 	ZTE, Sanechips

Agenda item:	8.11.3

Document for:	Discussion and decision

Introduction

In RAN1#108-e meeting, for the case that UE-B receives multiple non-preferred resource sets, the following agreements were achieved[1]:

Agreement

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A

  - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A

  - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A

  - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Agreement

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As,

  - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

- Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline)

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Accordingly, the behavior for using a received non-preferred resource set in current RAN1 specification was described as follows in TS38.214[2]:

- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement.

During the discussion in last RAN1 meeting, there are many understanding on UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same/different UE-As. In this paper, our understanding on this issue is provided.

Discussions

Understanding on UE-B’s behavior of receiving multiple non-preferred resource sets

Following current RAN1 specification in TS38.214, when UE-B receives one or multiple non-preferred resource sets from the same/different UE-As, UE-B uses a received non-preferred resource set as follows when performing resource (re-)selection:

-	the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set.

There may be three kinds of understandings:

Understanding 1: UE-B uses the latest received non-preferred resource set subject to UE processing timeline for its resource selection.

Understanding 2: UE-B uses all of the received non-preferred resource sets subject to UE processing timeline for its resource selection.

- Understanding 2-1: UE-B uses all of the received non-preferred resource sets one by one in physical layer subject to UE processing timeline for its resource selection.

- Understanding 2-2: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection.

Understanding 3: UE-B determines a final non-preferred resource set by UE implementation based on the received non-preferred resource sets subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection.

Following RAN1’s agreement, no RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e, that means either understanding 1 or 2 is not expected to be explicitly specified. In our point of view, understanding 3 should be the common understanding for the case that when UE-B receives one or multiple non-preferred resource sets from the same/different UE-As. With understanding 3, e.g. in MAC layer, UE can form a final non-preferred resource set based on received non-preferred resource sets, which can be the latest received non-preferred resource set, or can be a union of all or some of the received non-preferred resource sets. Then the final non-preferred resource set could be indicated to physical layer when UE performs sensing as described in TS38.214.

Based on above analysis, the following proposal is given:

When UE-B receives one or multiple non-preferred resource sets from the same/different UE-As, we should understand that (Understanding 3) UE-B can determine a final non-preferred resource set by UE implementation based on the received non-preferred resource sets subject to UE processing timeline. Then UE-B uses the final non-preferred resource set for its resource selection.

Conclusion

Based on above discussion, for the case that UE-B receives multiple non-preferred resource sets, the following proposal is given:

Proposal 1: When UE-B receives one or multiple non-preferred resource sets from the same/different UE-As, we should understand that (Understanding 3) UE-B can determine a final non-preferred resource set by UE implementation based on the received non-preferred resource sets subject to UE processing timeline. Then UE-B uses the final non-preferred resource set for its resource selection.

References

Chairman's Notes RAN1#108-e.

TS 38.214-h10.