3GPP TSG RAN WG1 #109-e		R1-2203567

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on solutions for integrity of RAT dependent positioning

Agenda Item:	9.5.2.1

Document for:	Discussion and Decision

Introduction

In RAN #94e Meeting, the SID on study on expanded and improved NR positioning RP-213588[1] was approved including the following objective related to integrity:

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2] - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

Identification of the error sources for RAT-dependent positioning

Based on TR 38.857[2], the following diagram is used for the error sources analysis of the GNSS case.

From this diagram, the 5 sources of feared events are identified and summarized in the following table:

| Feared Event Category | Feared Event | Examples of positioning integrity assistance information (FFS) |
| --- | --- | --- |
| 1. Feared events in the GNSS Assistance Data | Incorrect computation of the GNSS Assistance Data, e.g. software bug, corrupt or lost data | Validity or quality flags for existing assistance information |
|  | External feared event impacting the GNSS Assistance Data, e.g. satellite, atmospheric or local environment feared events (Category 3) impacting the GNSS reference stations in the GNSS correction provider's network. |  |
| 2. Feared events during positioning data transmission | Data integrity faults | Data corruption check, e.g. CRC |
|  |  | Data Authentication / Signature |
| 3. GNSS feared events | Satellite feared events e.g. bad signal-in-space or bad broadcast navigation data | Satellite health or quality flags |
|  | Atmospheric feared events | Ionospheric indicator |
|  |  | Tropospheric indicator |
|  | Local Environment feared events, e.g. Multipath, Spoofing, Interference | Assistance information: Trustable time reference, Data Authentication / Signature, Regionalized indicator of multipath, interference, jamming, spoofing, etc |
| 4. UE feared events | GNSS receiver measurement error | e.g., GNSS-MeasurementList |
|  | Hardware faults | * |
|  | Software faults | * |
| 5. LMF feared events | Hardware faults | * |
|  | Software faults | * |
| NOTE: The positioning integrity assistance information IEs are FFS as part of the WI. *NOTE: The UE or LMF are responsible for mitigating these feared events locally, outside the scope of the specifications. |  |  |

However, in Rel-17, only some error sources about SSR assistance date are specified in the following Table.

Table 8.1.2.1b-1: Mapping of Integrity Parameters [3]

| Error | GNSS Assistance Data | Integrity Fields |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Integrity Alerts | Integrity Bounds (Mean) | Integrity Bounds (StdDev) | Residual Risks | Integrity Correlation Times |
| Orbit | SSR Orbit Corrections | Real-Time Integrity (see Section 8.1.2.1.8) | Calculated according to Equation 8.1.1a-3 | Calculated according to Equation 8.1.1a-3 | Probability of Onset of Constellation Fault Probability of Onset of Satellite Fault Mean Constellation Fault Duration Mean Satellite Fault Duration | Orbit Range Error Correlation Time Orbit Range Rate Error Correlation Time |
| Clock | SSR Clock Corrections |  | Mean Clock Residual Error Vector | Standard Deviation Clock Error |  | Clock Range Error Correlation Time Clock Range Rate Error Correlation Time |
| Code Bias | SSR Code Bias |  | Mean Code Bias Error Mean Code Bias Rate Error | Standard Deviation Code Bias Error Standard Deviation Code Bias Rate Error |  |  |
| Phase Bias | SSR Phase Bias |  | Mean Phase Bias Error Mean Phase Bias Rate Error | Standard Deviation Phase Bias Error Standard Deviation Phase Bias Rate Error |  |  |
| Ionosphere | SSR STEC Correction | Ionosphere DNU | Mean Ionospherre Error Mean Ionospherre Rate Error | Standard Deviation Ionosphere Error Standard Deviation Ionosphere Rate Error | Probability of Onset of Ionosphere Fault Mean Ionosphere Fault Duration | Ionosphere Range Error Correlation Time Ionosphere Range Rate Error Correlation Time |
| Troposphere Vertical Hydro Static Delay | SSR Gridded Corrections | Troposphere DNU | Mean Troposphere Vertical Hydro Static Delay Error Mean Troposphere Vertical Hydro Static Delay Rate Error | Standard Deviation Troposphere Vertical Hydro Static Delay Error Standard Deviation Troposphere Vertical Hydro Static Delay Rate Error | Probability of Onset of Troposphere Fault Mean Troposphere Fault Duration | Troposphere Range Error Correlation Time Troposphere Range Rate Error Correlation Time |
| TroposphereVertical WetDelay |  |  | Mean Troposphere Vertical Wet Delay Error Mean Troposphere Vertical Wet Delay Rate Error | Standard Deviation Troposphere Vertical Wet Delay Error Standard Deviation Troposphere Vertical Wet Delay Rate Error |  |  |

Based on the error sources of GNSS, in the section, we provide our views on error sources for RAT-dependent positioning.

From this diagram, the 5 categories of feared events are identified:

1. Feared events in the Assistance Data

2. Feared events during positioning data transmission
3. TRP feared events

4. UE feared events

5. LMF feared events

Considering the progress of GNSS standardization and the characteristics of NR positioning, we propose that the error source of assistance data, and the error source of TRP/UE about signal transmission and measurement can be identified first.

- From the perspective of RAN1, error sources with respect to the assistance data and signal transmission and measurement can be identified first.

In addition, with different measurements, the corresponding error sources will be different. So,  in our view, the error source should be discussed per method, especially for the error sources with respect to measurement, e.g., the error source for timing-based and angle-based methods should be considered separately.

- RAN1 needs to study the error sources under various methods, including

  - Angle-based measurement and timing-based measurement

Error resources for timing-based positioning

According to the above analysis, we analyze error sources for timing-based positioning. In the section, we take DL-TDOA as an example and identify the following error sources which may impact positioning accuracy:

1. The error sources of assistance data

- The network timing synchronization

- The TRP location accuracy

- The GDOP of TRP location

2. The error sources of UE Measurement

- The timing measurement error

- The UE clock drifting

- The Rx timing error on the UE side

2. The error sources of the TRP side for signal transmission

- The reference signal (e.g., small bandwidth, low power)

- The Tx timing error on the TRP side

- The environment (e.g., Multipath, NLOS, Interference)

Error resources for angle-based positioning

In the section, we take UL-AOA as an example and identify the following error sources which may impact positioning accuracy :

The error sources of assistance data

- The TRP location accuracy

- The TRP beam/antenna information

- The GDOP of TRP location

2. The error sources of TRP Measurement

- The angle measurement error

3. The error sources of the UE side for signal transmission

- The reference signal (e.g., low power)

- The phase error between multiple antennas

- The environment (e.g., Multipath, NLOS, Interference)

Conclusion

In this contribution, we discuss potential error sources for Integrity for RAT-dependent positioning techniques with the following proposals.

- From the perspective of RAN1, error sources with respect to the assistance data and signal transmission and measurement can be identified first.

- RAN1 needs to study the error sources under various methods, including

  - Angle-based measurement and timing-based measurement

- References

- RP-213588 Study on expanded and improved NR positioning, Dec. 6 – 17, 2021

- TR 38.857 Study on NR Positioning Enhancements

- TS 38.305 User Equipment (UE) positioning in NG-RAN