3GPP TSG-RAN WG1 Meeting #109-e	R1-2203904

e-Meeting, May 9th – 20th 2022

Agenda Item:	9.3.2

Source:	Samsung

Title:	Subband non-overlapping full duplex for NR duplex evolution

Document for:	Discussion

Introduction

The SID for NR duplex evolution aims to identify solutions and evaluate their feasibility to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation in unpaired spectrum.

In the R18 study item, it is assumed that the duplex enhancements are deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation in a time-domain resource is limited to FDM, e.g., using non-overlapping subband allocations for simultaneous DL transmissions to UEs and for UL receptions from UEs respectively.

The SID for NR duplex evolution specifies the following RAN1 objectives regarding subband non-overlapping full-duplex and dynamic/flexible TDD operation [1]:

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1) - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1) - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1)

In this contribution, we first present our views on practical SBFD design requirements arising from both network deployment and UE implementation aspects when introducing simultaneous transmission and reception capability at the gNB for the NR TDD bands. Backwards compatibility and benefits of using gNB-side SBFD when legacy UEs are present in the NR TDD cell are considered in Section 3. UE enhancements for providing support to gNB-side SBFD operation are possible in later NR releases. We discuss several possible approaches for the SBFD design and the corresponding solutions for such potential UE enhancements, their feasibility, and their benefits in Section 4. Please see our accompanying contributions on CLI enhancements in the context of dynamic/flexible TDD [2][3].

Note that in the remainder of this contribution we will simply refer to SBFD for subband full-duplex operation using non-overlapping subband allocations, e.g., as assumed by the R18 SID.

SBFD design requirements

When SBFD is implemented at the gNB, the received UL signal at the gNB is subject to co-channel cross-link interference (CLI) from the gNB side transmitter. Methods to cancel the CLI include passive methods which rely on the antenna isolation between transmit and receive antennas, active methods which utilize RF or digital signal processing, hybrid methods using a combination of these, and filtering. It has been demonstrated that handling co-channel CLI at the gNB is feasible and within implementation range for FR1. For example, antenna SIC can be used to minimize the leakage power from the Tx ports to the Rx ports of the panel and digital SIC is used to handle any residual interference after antenna SIC. DL out-band signal power flowing into the UL receiver path can be effectively suppressed below the noise floor level to guarantee the UL receiver performance. Also, by combining digital pre-distortion (DPD) at the Tx path and digital SIC at the Rx path, the out-band interference from the DL signal to the UL signal can be effectively mitigated by the gNB such that the need for a guard band between the UL and DL signals is minimized [4]. In FR2, the use of separate antenna panels can provide additional spatial isolation

Using a single NR RF carrier, SBFD implements full-duplex transmission at the gNB in the BS channel bandwidth. In a slot where SBFD is supported and enabled, the gNB can schedule DL and UL resources to UEs using non-overlapping frequency-domain resources. The TDD UEs operate in half-duplex. When a UE is scheduled for DL reception in a slot, no simultaneous UL transmission from the same UE can occur using the same time-domain resources. The gNB must account for Rx-Tx and Tx-Rx switching times of the UE when scheduling DL and UL for the UE in different slots. UL transmission from a UE in a slot with SBFD occurs in the SBFD UL subband. DL transmissions to UEs in a slot occur in the SBFD DL subbands at the higher and lower range of the BS channel BW (Figure 1).

Figure 1: SBFD

In frequency-domain, it can be expected that the SBFD UL subband is often placed at the center of the BS channel BW which provides most frequency separation and isolation towards the adjacent channels in band segments owned by other TDD operator(s). In cases where an operator holds a band segment at the edge of the NR band, placing the SBFD UL subband at the edge of the BS channel BW is an alternative. There will then be only 1 DL subband. The size of the SBFD UL subband cannot be assumed fixed. At least for evaluation purposes, it may be assumed that the SBFD UL subband is 20 MHz wide, e.g., n78 and that the operator holds a 100 MHz band segment. However, many operators own band segments of smaller size in n78, e.g., 40 or 60 MHz, and some operators may even hold 2 disjoint band segments in n78. This makes it necessary to account for the need to configure an SBFD UL subband of smaller size than 20 MHz.

In time-domain, not all symbols and slots can be used for SBFD operation. DL symbols carrying the SSB(s), CORESET0 and PDSCH carrying SIB1 cannot be used for UL transmissions using the SBFD UL subband due to UE backwards compatibility considerations. Considering the UE Rx-Tx and Tx-Rx switching time requirements, this often implies that DL slots including SSBs are not available for SBFD operation (depending on the number of SSBs configured in the cell). SBFD operation in the UL slot(s) of the TDD UL-DL frame configuration, i.e., introducing DL subband in the UL slot(s), is possible in principle. However, any DL transmission in the UL slot is subject to inter-gNB co- and adjacent channel interference which makes the use of SBFD less meaningful in practice when considering the typically high DL Tx power levels and BTS-BTS interference scenarios in urban macro and micro cells. It should therefore be assumed that SBFD DL subband is not configured in UL slots or symbols.

gNB configurability of the frequency-domain occupation of the SBFD UL subband in the BS channel BW and the time-domain resources in which the SBFD UL subband can be scheduled by the gNB is therefore necessary. Note that this does not necessarily imply that the UE is aware of the SBFD UL subband configuration, although in our view there are several benefits to providing the UE with the SBFD UL subband configuration in the cell (as further discussed in Section 4).

The SBFD frequency-domain occupation, e.g., the location and bandwidth of the SBFD UL subband in the BS channel BW is configurable.

The SBFD time-domain resources, e.g., the symbols/slots of the TDD UL-DL frame configuration in which the SBFD UL subband can be scheduled are configurable.

When SBFD is supported by the gNB in the NR TDD cell, different approaches for the network to configure the UE (Figure 2) are possible. The simplest approach is that the gNB configures the SBFD UL subband and the DL subband(s) for the UE inside the UE channel BW, e.g., using single-carrier operation with SBFD in the serving cell (Figure 2). A possibility is to configure the SBFD UL subband for the UE by means of a SUL configuration. Another approach is the use of intra-band CA for SBFD (Figure 3).

Figure 2: SBFD using single carrier or configured as SUL

Figure 3: SBFD using intra-band CA

Using FR1 intra-band TDD carrier aggregation for SBFD, either 2 or 3 CCs need to be configured for the UE in the aggregated BS channel BW. With 2 CC contiguous intra-band TDD CA, the SBFD UL subband must be contained in the transmission BW configuration of one of the 2 CCs (approach 3a in Figure 3). It is possible to configure two non-contiguous CCs, but then parts of the available DL BW are not schedulable for the UE. With 3 CCs intra-band TDD CA, the SBFD UL subband is configured with an UL-heavy UL-DL frame configuration in one of the CCs and the DL subband(s) using are configured as the 2 remaining CCs (approach 3b in Figure 3). When intra-band TDD CA is configured for the UE, it is in principle possible to configure the DL BWPs corresponding to the DL subbands independently from the size of the SBFD UL subband, use features such as R16 SCell Dormancy for reception in the DL subbands, or configure the CCs with unaligned frame boundaries and the slot offset.

However, the use of FR1 intra-band CA for purpose of enabling SBFD operation in the NR band is significantly more challenging than using a single NR carrier. UE complexity is increased when FR1 intra-band TDD CA becomes prerequisite for SBFD support in the NR band. When 2 CCs are configured, 1 CC must still contain the SBFD UL subband and any transmission and reception behavior with SBFD on this CC then still corresponds to the single NR carrier case. Spectral efficiency when using CA instead of single carrier for SBFD operation is reduced. When including the mandated guard band(s) between the 2 (or 3) CCs, the number of schedulable RBs in the 2 (or 3) CCs of smaller channel BW is reduced by 5-6% in typical configurations when compared to the single carrier case. The resulting spectral efficiency loss from the use of TDD intra-band CA for SBFD will depend on the BS transmission BW in the band, the size of the operator band segment and the size and frequency-domain location of the SBFD UL subband. The introduction of additional intra-band CA configurations and BW combination (sets) for purpose of supporting SBFD would result in significant RAN4 work. For the UE DL receiver performance, the potential impacts of UL transmissions in the SBFD UL subband from UEs in proximity on DL reception must also be considered, in particular when using a wideband UE receiver implementation.

Using FR2 intra-band carrier aggregation, more flexibility to introduce SBFD operation exists when compared to FR1. The support of NR FR2 CA with Independent Beam Management (IBM) and Common Beam Management (CBM) with simultaneous DL reception on different CCs from the co-located and non-collocated TRPs has been introduced in R16 and R17. IBM implies that a UE is capable of DL simultaneous reception on different UE panels/chains using separate beams on different CCs and requires improved UE BB and RF capabilities, e.g., multiple BB chains and support of multiple antenna panels. R18 FR2 enhancements work is starting and will introduce requirements for FR2 UEs capable of multi-beam/multiple Rx chain simultaneous DL reception on a single CC to achieve improved RF, RRM and UE demodulation performance.

In summary, we think that the R18 study should focus on approach 1, e.g., SBFD operation using a single NR carrier, without precluding the possibility of later study and specification of TDD intra-band CA or SUL for purpose of configuring SBFD in the UE when possible and where meaningful to do so.

SBFD can be configured in the UE channel BW of a single NR carrier. Further study the potential benefits and impacts when using intra-band CA or configuring SUL for SBFD.

SBFD and legacy UEs

When SBFD operation is supported by the gNB, backwards compatibility for legacy UEs must be assured. UE backwards compatibility must be considered from the perspective of both functional and operational backwards compatibility.

Functional backwards compatibility implies that legacy R15-R17 UEs can still connect to and establish service in the NR TDD cell. Transmission and reception of data and RRC/NAS signaling should still be possible without undue performance impact. In addition, legacy UEs can benefit from the availability of SBFD in an NR TDD cell, e.g., when DFFFU is configured in the cell. Operational backwards compatibility means that legacy UEs should still be able to use existing deployed NR network features such as CA/DC when SBFD operation is introduced in a serving cell.

Legacy UEs in SA mode when operating on a TDD cell with SBFD at the gNB can access the TDD cell as their serving cell. These UEs must be able to acquire the TDD cell with SBFD during initial cell-selection, complete initial network access in the TDD cell with SBFD and later perform cell re-selection in RRC_IDLE or RRC_INACTIVE when camped on the TDD cell.

Using n78, channel BW = 100 MHz, SCS = 30kHz and DXXSU as example where gNB-side SBFD operation is enabled in the 2nd and 3rd slot (Figure 1), the gNB can configure SIB1 for the legacy UEs with tdd-UL-DL-ConfigCommon as {dl-UL-TransmissionPeriodicity, nrofDownlinkSlots, nrofDownlinkSymbols, nrofUplinkSlots, nrofUplinkSymbols} = {P = 2.5ms, 3 DL slots, 12 DL sym, 0 UL sym, 1 UL slot}, or DDDSU. Legacy NR UEs then consider only the last 2 symbols in slot #3 as flexible symbols (F). This disables the possibility to schedule any UL transmissions for legacy UEs using the SBFD UL subband when SBFD operation is supported in slots #1 and #2.

When SIB1 tdd-UL-DL-ConfigCommon for the legacy UEs is configured with {dl-UL-TransmissionPeriodicity, nrofDownlinkSlots, nrofDownlinkSymbols, nrofUplinkSlots, nrofUplinkSymbols} = {P = 2.5ms, 1 DL slot, 0 DL sym, 0 UL sym, 1 UL slot}, or DFFFU, the gNB can schedule UL transmissions to legacy UEs using the SBFD UL subband in flexible symbols of slots #1 and #2 (Figure 4). The UL BWPs of the legacy UEs must be configured inside the BS channel BW such that they contain the SBFD UL subband.

Figure 4: SBFD operation for legacy UEs without specification impact, e.g., transparent mode

Note that we assume here that no UE-dedicated, e.g., no UE-specific RRC UL-DL frame configuration is provided to the UEs by RRC. Similarly, SFI, e.g., DCI format 2_0 is not assumed as configured. This corresponds to typical NR TDD operation where FG 5-1 support is mandatory for the UE, but FG 5-1a (UE-specific RRC UL-DL frame configuration) and FG 3-6 (SFI) are optional UE features (and not usually implemented).

When DFFFU is configured, the gNB can transparently schedule UL transmissions to legacy UEs using the SBFD UL subband of slots #1 and #2, e.g., using UL resource allocation type 1. The R15 UE follows the gNB scheduling assignments for the semi-static symbols/slots configured as F. Limitations exist for example with respect to CSI-RS resource configurations when configured in these slots. UL scheduling by the gNB cancels UE reception for such RRC configured DL signals. Similar considerations apply to RRC configured UL signals/channels. Reception of a scheduling DCI by the UE will cancel the configured UL transmission and the UE transmits or receives the scheduled signal/channel. The R15 UE can be configured with multiple open loop power-control parameter sets when scheduled for UL transmissions in SBFD slots and the normal UL slot(s). The use of the DFFFU UL-DL frame configuration results in “transparent” SBFD operation, e.g., based on existing specifications and UE behavior.

- Legacy UEs can be scheduled in the SBFD UL subband by configuring flexible symbols in the SBFD slot using existing RAN1 specifications and UE behavior.

Another consideration is the BWP configuration of the UE. In NR TDD, a DL and UL BWP pair with the same configured bwp-id must have the same center frequency. Most UE implementations only support FG 6-0 (single BWP re-configurable by RRC). When UEs supporting only a single BWP are scheduled in the SBFD UL subband, the UE UL BWP must then be configured such that it comprises the entire SBFD UL subband. Depending on the frequency occupancy of the SBFD UL subband in the UE channel BW, the size of the UE DL BWP can then not always be selected by the gNB to fit the DL traffic profile of the UE under the condition that the center frequency of the DL and UL BWP pair remains aligned. This can negatively impact the UE power consumption during DL reception in RRC_CONNECTED. It is desirable for SBFD operation that a DL BWP of size from 20 to 40 MHz can be configured by the gNB without losing the ability to schedule the UE in the SBFD UL subband. When FGs 6-1 to 6-4 (2 or 4 BWPs with same or different SCS) are available in the UE implementation, the active BWP can be switched by DCI. However, DL switching delay of 2 or 5 slots in the SCS=30 kHz case is incurred depending on the UE capability bwp-SwitchingDelay. RRC re-configuration delay of a single BWP is even longer, e.g., 10 ms. Therefore, if BWP switching or re-configuration is used, the UE DL and UL UPT can be impacted.

Generally, it is very desirable that the introduction of gNB-side SBFD in the TDD cell does not increase the UE implementation complexity. SBFD operation must therefore be possible for UEs supporting only a single BWP. Moreover, single local oscillator implementation for the UE TDD implementation when operating in an NR band must remain possible. SBFD operation assuming that UEs implement multiple BWPs should not be the design objective.

SBFD operation is supported for UEs implementing only a single DL and UL BWP pair with aligned center frequency

When SBFD is supported in a TDD serving cell of an NR band, legacy NR UEs can use intra-band CA, inter-band CA, FR1-FR2 CA, EN-DC or NR-DC. The TDD cell with SBFD must be able to operate as PCell, SCell or SpCell for the legacy UEs.

Using the example of FR1-FR1 inter-band CA, it can be expected that the mid-band TDD cell with SBFD will often be configured as SCell (Figure 5). Relevant deployment cases are n3+n78 or n41+n78. The n78 (3.5 GHz) TDD cell with SBFD can use up to 100 MHz channel BW. The n3 (1.7 GHz) FDD carrier or the n41 (2.5 GHz) TDD carrier provide the coverage layer and are configured as PCell for the legacy UEs. Note that simultaneous Tx/Rx capability is mandated for the UE for n3+n78, but not for n41+n78. In this case, the PCell is in a different band and the TDD SCell with SBFD will then carry DL data. The TDD SCell with SBFD will then mostly be used for DL data, so the SCell UL does not benefit from the availability of SBFD.

Using the example of FR1-FR2 CA, it is possible that the mid-band TDD cell with SBFD is configured as PCell. One relevant case is n78+n257. For example, the n257 (28 GHz) TDD cell is then configured with 200 MHz and 120 kHz. Note that simultaneous Tx/Rx capability is mandatory for the UE for this combination (like for any FR1-FR2 combination). The FR1 TDD PCell with SBFD carries DL & UL control (L1, RRC, NAS) for both CCs. The FR1 TDD PCell with SBFD may carry both DL and UL data, so improved UL performance in the PCell with SBFD is beneficial.

Figure 5: SBFD on SCell for FR1 inter-band CA (n41+n78)

Examples for EN-DC or NR-DC are inter-band EN-DC within FR1, e.g., DC_1A_n78A or FR1-FR2 inter-band DC_n78A-n257A using 2 bands. Note that UL data is then present in both cases on the n78 TDD cell with SBFD, so any benefits (or penalties incurred) from introduction of SBFD apply.

Deployment of SBFD in a network region of an NR band by the operator should not prevent the operator from continuing to serve the legacy UEs in that band to avoid the need for segregation of the legacy and later release UEs through frequency layers in the deployment. Therefore, it is necessary to consider the use and impact of the SBFD UL subband when the TDD cell with SBFD is configured as PCell, SCell or SpCell using CA or DC. From the point-of-view of functional backwards-compatibility for the legacy UEs, it is also necessary to continue to support the case that the TDD cell with SBFD in band is configured as PCell, SCell or SpCell. In terms of operational backwards-compatibility, it is important to continue to support the case that the later release UEs providing improved support for SBFD at the gNB can still use CA or DC “as is” without changes to the operator’s deployment grid.

RAN1 to study the use and evaluate the potential impacts from SBFD when a TDD serving cell with SBFD is configured as PCell, SCell or SpCell for a UE.

UE enhancements for improved SBFD performance

Scheduling

One of the key features of NR is the support for dynamic TDD operation where the gNB scheduler can dynamically determine the transmission direction. NR uses 3 different signaling mechanisms to provide the information to the UE whether a transmission resource is used for UL or DL. The basic principle is that the UE monitors the configured PDCCH and then transmits or receives according to the received DCI. The UL-DL frame configuration can be signaled to the UE as cell-specific pattern, e.g., SIB1 tdd-UL-DL-ConfigCommon, and a UE-specific pattern can additionally be configured by tdd-UL-DL-ConfigDedicated. The third possibility is to configure SFI, e.g., DCI format 2_0.

When SBFD is introduced, these NR principles should be preserved. gNB scheduling decides if a transmission resource is used for UL or DL when SBFD is deployed in the cell. The frequency occupancy of the SBFD UL subband in the BS channel BW must be configurable to adjust SBFD according to deployment needs, e.g., depending on the operator band segment and according to the capabilities of the gNB-side SBFD implementation. UL transmissions from UEs in SBFD time-domain resources are then possible only inside the SBFD UL subband, but not in the DL subband(s) of the SBFD time-domain resource. gNB-side CLI cancellation restricts the possibility of UL transmissions outside the SBFD UL subband. However, the gNB scheduler can still schedule the SBFD UL subband for DL transmissions to UEs as needed, e.g., the SBFD time-domain resource is used for DL only. It is undesirable that the introduction of the SBFD feature results in hard UL-DL resource partitioning in the TDD cell. This principle is shown in Figure 6 where an UL transmission using the SBFD UL subband is scheduled for the UE in slot #1 but is then used for DL scheduling to the UE in slot #2.

For a UE, the transmission direction of a symbol in the SBFD UL subband is determined by gNB scheduling

Figure 6: Scheduling principles with SBFD

It must then be considered what kind of signaling is used by the gNB to assign the transmission direction of the SBFD UL subband and if the frequency occupancy of the SBFD UL subband in the UE channel bandwidth needs to be known by the UE.

Similar to the CSI-RS resource configuration, the frequency occupancy of the SBFD UL subband has a start RB and allocation bandwidth. In time-domain, UL transmissions in the SBFD UL subband may not be possible on all symbols in a slot, e.g., when PDCCH monitoring at the beginning of the slot is configured on the first symbol of the slot. In principle, it is possible to use the existing PDSCH and PUSCH resource allocation types 0/1 when scheduling the UE in DL and UL. A guard band between DL subband(s) and the UL subband in SBFD slots is implicitly provided by gNB scheduling. The gNB must ensure that the PUSCH is allocated or that PUCCH resource are configured for the UE in the SBFD UL subband.

If the frequency occupancy of the SBFD UL subband in the configured UE channel BW is not known to the UE, it is significantly harder to implement UL Tx and DL Rx filtering in the UE. While it should not be assumed that UE implementations with enhanced support for gNB-side SBFD must use more stringent Tx/Rx filtering than what is currently necessary to meet the existing NR UE RF requirements to improve their in-channel selectivity and in-band blocking performance, it is desirable to enable such UE functionality as capability.

Another consideration is to overcome existing limitations when using tdd-UL-DL-ConfigDedicated or SFI. Only flexible symbols in the tdd-UL-DL-ConfigCommon can be re-assigned as D or U. Providing the SBFD UL subband configuration to the UE in terms of the allowed symbols/slots where SBFD UL transmissions can be scheduled independently from SIB1 tdd-UL-DL-ConfigCommon has the advantage that legacy UE behavior in the TDD cell with SBFD can be independently configured from the transmission behavior of the UEs with improved support for gNB-side SBFD. For example, it is then possible to schedule UL transmissions in the SBFD UL subband even in the symbols/slots configured as D for the legacy UEs or to only schedule UL transmissions in the SBFD UL subband in the SBFD slots, but not the normal UL slot. It is then also possible to define separate UE behavior not currently possible in R17 with respect to CSI reception and SRS transmissions for purpose of inter-subband CLI measurements and associated reporting in the TDD cell with SBFD.

To signal the time-domain and frequency-domain configuration of the SBFD UL subband to UEs, the existing NR signaling can be extended, e.g., the SBFD UL subband is configured by cell-common or UE dedicated RRC signaling and/or provided to UEs by SFI. Note that R17 eIAB signaling already introduced signaling of hard, soft or unavailable type for frequency-domain resources, e.g., for an RB set in a symbol. Such existing and already specified functionality could be re-used for SBFD.

The cell-common or UE dedicated SBFD UL subband configuration, e.g., symbols/slots and the frequency occupancy in which UL transmissions can be scheduled by the gNB, is provided to the UE.

For UL and DL scheduling in a TDD cell with SBFD, both single slot and multi-slot scheduling of PUSCH and PDSCH must be considered. One important use case for TDD with SBFD is improved UL coverage in the existing deployment grid. SBFD enables to significantly increase the UL slot utilization ratio by making more time-domain resources available to UEs in bad coverage which transmit at maximum configured UE output power. In DDDSU, the UE can only transmit UL in 20% of the time-domain resources at maximum power, whereas with SBFD and DXXXU it can potentially transmit in up to 80% of the time-domain resources. R15 PUSCH aggregation and R16/R17 PUSCH repetition Type A features are important when scheduling the UE across multiple slots.

Several antenna and panel design options exist to support gNB-side full-duplex operation. Design options may be expected to change over time as technology evolves. For example, antenna panel to provide spatial separation of Tx and Rx paths and for gNB side SIC can be based on a fully split design where ports are either dedicated to DL transmission or to UL reception. Another possibility is shared design where a certain number of ports can be used for both DL transmission and UL reception with SBFD, but a number of ports is dedicated to either DL transmission or to UL reception respectively. Therefore, it cannot be expected that the number of TRXs and the available Tx or Rx aperture area used for Tx and/or Rx in normal DL or normal UL slots is necessarily the same when compared to the DL transmission or UL reception in SBFD slots. The link gain and resulting link budget when transmitting an UL signal or channel in the SBFD UL subband of a slot is different when compared to transmitting the same UL signal or channel in the normal UL slot. Similar considerations apply to transmission of DL signals/channels in the DL subbands of SBFD slots.

PUSCH transmissions of a R15 PUSCH aggregation and R16/R17 PUSCH repetition Type A when transmitted in the SBFD UL subband of SBFD slots and in the normal UL slot will therefore be subject to mis-matched link adaptation settings. It is desirable to restrict the PUSCH transmissions of a PUSCH repetition with respect to the time-domain resources in a TDD cell with SBFD. It should be possible to transmit such PUSCH repetitions using only the UL subband of the SBFD slots, but not in the normal UL slot. Note that even when PUSCH repetitions are limited to the use of the SBFD UL subband in DXXXU, the resulting UL slot utilization ratio is still higher than currently possible with DDDSU.

Multi-slot PUSCH, e.g., PUSCH repetition Type A, and PUCCH repetitions can be configured for transmission using only the SBFD slots/symbols.

An important design consideration for SBFD operation in the UL is that the separation of SBFD symbols/slots and the UL slot(s) with respect to the resource allocation in time-domain when introducing gNB-side SBFD operation also allows to increase the achievable UL spectral efficiency and UL cell throughput in the (normal) UL slot(s). Low-SINR UEs can be offloaded to UL transmissions in the SBFD slots, whereas UEs in good geometry can be scheduled high MCS using large contiguous RB allocations in the UL slots. Given that most existing UE implementations only support UL (almost) contiguous resource allocation, e.g., PUSCH allocation type 1, the offloading of low-SINR UEs using multi-slot allocations to the SBFD UL subband in the SBFD slots prevents resource fragmentation in the (normal) UL slot. It can be expected that the SBFD UL subband is often placed in the center of the NR carrier. During multi-slot PUSCH transmissions using the SBFD slots, the FDRA and FH if configured is then confined to the UL subband also in the (normal) UL slot. The gNB scheduler cannot allocate large contiguous BW segments for purpose of UL transmissions from UEs in good SINR conditions. Note that similar issues resulting in fragmentation of the UL scheduling BW in the (normal) UL slot can exist when single-slot PUSCH scheduling is used due to constraints arising from the UE DL/UL BWP pair configuration with respect to the channel occupancy of the SBFD UL subband.

In the DL, gNB-side SBFD operation can benefit from enhancements to PDSCH resource allocations and mappings, e.g., when rate match patterns are configured in the UE for RB level rate matching.

Using PDSCH resource allocation type 1, RBs and symbols of the SBFD UL subband in an SBFD slot can be masked when a R15 or R16 cell- or UE- specific RateMatchPattern is configured for the UE. Support of FG 5-26 (semi-static rate-matching resource set configuration for DL) is mandatory for legacy UEs, but dynamic signaling support of the bitmap 1/2/3 as by FG 5-27 is only optional. When the gNB uses the UL subband in the SBFD slot for DL transmissions to UEs, the RRC configured RateMatchPattern cannot be disabled by the scheduling DCI format 1_1. The gNB can then only schedule PDSCH using (non-interleaved) DL resource allocation type 1 either in the lower or upper DL subband of the SBFD slot, or PDSCH across both DL subbands cannot be mapped across using the UL subband at all (Figure 7).

RAN1 to study the impacts of SBFD operation on DL resource allocation and resource mapping in slots/symbols configured with an SBFD UL subband

Figure 7: DL scheduling with SBFD

CLI

The R16 WI CLI and RIM introduced several tools to mitigate inter-cell UE-to-UE CLI. gNBs can exchange and coordinate the intended TDD UL/DL configuration over the Xn and F1 interfaces. The gNB can then decide the transmission and reception pattern to either avoid creating CLI towards a neighbor cell or to incur CLI from a neighbor cell. Two different types of UE reportable CLI measurements were specified: CLI-RSSI and SRS- RSRP. For CLI-RSSI measurements, the victim UE measures the total received power over the configured CLI-RSSI measurement resource(s). For SRS-RSRP measurements, the victim UE measures the RSRP over configured SRS resource(s) which are transmitted from one or multiple aggressor UEs. These measurements are L3 filtered. Event triggered and periodic reporting are supported. A number of CLI mitigation techniques, including receiver-based design options were discussed and evaluated during the NR SI.

With respect to CLI measurements and associated UE reporting for SBFD, the R16 CLI reporting features can in principle be used “as is” for the case of inter-cell intra-subband CLI reporting when SRS transmissions from interfering (aggressor) UEs transmitting in the SBFD UL subband in a neighbor cell are reported by the victim UEs receiving in the DL subband of an SBFD slot in the serving cell. This case corresponds to unaligned SBFD UL subbands, e.g., differently configured frequency occupations of the SBFD UL subbands in the two TDD cells. This deployment case is not expected to occur often in practical deployments. It can be expected that the SBFD UL subband when deployed is then also aligned in the TDD cells of an operator network segment (Figure 8). Edge effects such as when the band segment of the operator changes exist but are rare.

A more frequently expected SBFD deployment case resulting in non-aligned SBFD UL subband(s) occurs at the edge of the operator network segment where SBFD is deployed at the gNB side. Here, the SBFD-capable TDD cells will need to inter-operate with legacy co-channel TDD cells of the operator. No SBFD UL subband is available in these neighbor cells (Figure 8).

Figure 8: SBFD inter subband CLI scenarios: aligned and unaligned cases

Forcing the UEs to transmit SRS in the DL subband(s) of an SBFD slot is not only detrimental to DL throughput but is also questionable in terms of its usefulness to the gNB scheduler. When SRS is transmitted by a UE in the DL subband(s) of the SBFD slot, DL transmissions from the gNB in these measurement resources are muted. SIC is not active. UE reported signal strength measurements, e.g., SRS-RRSP are therefore not much representative of actual UL-DL inter-subband interference in the SBFD slot.

In our view, introducing a UE reportable CLI-SINR measurement using a known configured DL measurement signal, e.g., NZP CSI-RS resource(s) is a better way to measure and report intra-cell inter-subband UL-DL interference.

Another limitation are UE capabilities. In the R16 CLI feature, simultaneous reception of DL signals/channels and CLI-RSSI or SRS-RSRP measurement resources by the UE is not required. For SBFD operation, DL scheduling in which the configured (RE-level) known DL measurement signal occurs is desirable.

A DL reference signal, e.g., NZP CSI-RS resource(s) can be configured for the UE to report inter-subband UL-DL interference.

With respect to CLI mitigation techniques, an important consideration for SBFD is that currently only a single UE configured maximum output power value p-Max limits the UE's UL transmission power on a carrier frequency. In addition, a single value for p-NR-FR1 and/or p-UE-FR1 can be configured for the total maximum configured output power of a cell group of which the serving cell is part or for all cells configured for the UE in FR1. A consequence is that the same UE configured maximum output power value is then used for UL transmit power control by the UE to determine the maximum transmission power value for PUSCH, PUCCH, SRS or PRACH irrespective of the slot type, e.g., normal UL slot or SBFD slot. For example, when p-Max is configured such that the UE is allowed 23 dBm nominal output power and adjusted by RF tolerances and A-MPR(s), the UE when scheduled will then transmit using up to the maximum configured value of 23 dBm in any time-domain resource of the serving cell.

It is desirable to separately set the allowed UE configured maximum output power for the normal UL slot and UL transmissions in the UL subband of the SBFD slots. The maximum allowed UL transmit output power of a UE determines its interference range with respect to the co-scheduled UEs in the DL subband(s) of the same cell. For SBFD operation in the serving cell, it is often useful to limit the interference range of the aggressor UE in average or good SINR conditions when transmitting in the SBFD UL subband of SBFD slots. The aggressor UE transmitting in the SBFD UL subband (mostly) interferes the victim UE receiving DL transmissions in the DL subband(s) of the same serving cell. The aggressor UE transmitting in the UL in the normal UL slot does (usually) not interfere with the DL transmissions to UE(s) in the same and in adjacent cells assuming the same TDD UL-DL frame configuration is configured for the TDD cells in the deployment and assuming that the guard period is configured sufficiently large.

The UE maximum output power is configurable per slot in a TDD serving cell with SBFD.

Other enhancements

When the SBFD UL subband is available for UL transmissions from the UE, consideration must be given to the type of UL signals or channels which can be configured or scheduled using the SBFD UL subband. In section 4.1 we discussed PUSCH and PUCCH scheduling aspects.

Allowing for configuration of PRACH in the SBFD UL subband and use of the SBFD UL subband for RRC and NAS signaling exchanges during the 5G Initial Attach procedure reduces UE initial acquisition time in NR TDD. It must be considered that about 8-10 DL and UL messages must be exchanged by network and UE during initial attach. These signaling exchanges are mostly “atomic” and “in sequence”, e.g., successful completion of a NAS procedure such as Authentication Request/Response is required before a subsequent NAS procedure such as NAS Identity Request can be initiated. NR TDD 30 kHz when compared to NR FDD 15 kHz in the NR low band is penalized by the average UL frame alignment delay incurred for each of the UL RRC and NAS messages due to limited number of UL slots. This typically results in 2x longer acquisition times.

Using the SBFD UL subband for PRACH transmissions has the additional benefit that fragmentation of the UL scheduling BW in the (normal) UL slot can now be avoided. The benefits of increased availability of contiguous UL scheduling BW in the normal UL slot are the same as discussed in section 4.1 in the context of PUSCH. It must be considered that RACH occasions in the UL slot are rarely configured at the carrier edges in practical NR TDD deployments for various reasons. RACH in the UL slot segments part of the schedulable UL bandwidth. Most UEs almost support (almost) contiguous PUSCH allocations, e.g., PUSCH resource allocation type 1. Another system-level aspect when placing PRACH ROs in SBFD slots is that PRACH is only used when a UE actually selects a random access resource and then transmits the RACH preamble. A configured, but not used PRACH RO in the SBFD slot does not create CLI towards other UEs in the slot. A configured, but not used PRACH RO in the (normal) UL slot can seldom be re-used for UL data scheduling and shared data/random access reception by the gNB, e.g., when configured, such a resource is not available for UL data.

Figure 9: PRACH configuration using the UL slot vs. the SBFD UL subband

Another important system design consideration for SBFD is that the SBFD UL subband suffers from inter-subband CLI from neighboring gNBs and possibly from self-interference during full-duplex transmission and reception at the gNB. As a result, the interference characteristics of RBs in the SBFD UL subband are different from those in the (normal) UL slot. The SBFD UL subband edge allocations can experience much higher interference levels when compared to the middle RB allocations of the SBFD UL subband, especially, when no guard band is configured or small-sized guard band are configured.

The guard band to avoid inter-subband CLI and self-interference can be explicitly configured by taking into account the interference levels in the SBFD UL subband. However, larger guard bands would reduce cell throughput since the resources in the guard band are then reserved and not used. Also, the guard band would then be fixed even if the neighbor cells do not use SBFD UL subband in the slot. Instead of providing an explicit guard band configuration, the gNB should be able to schedule DL signals and UL signals and account for need of guard band(s) as a result of its scheduling decision. For legacy R15 UEs operating in a TDD cell with SBFD, this kind of implicit guard band configuration must always be used by the gNB. When no explicit guard band configuration is provided by the gNB to UEs, this gives increased flexibility to the gNB implementation. For example, the gNB can then schedule UL signals and DL signals from/to UEs in the SBFD slot when much spatial isolation is possible using only small or no guard band. In cases where little spatial isolation between DL and UL UE pairings scheduled in the SBFD slot is available, larger guard bands be scheduled by the gNB. However, knowing the guard band size, possibly as part of the SBFD UL subband configuration then half-duplex TDD UEs can implement improved Tx/Rx side filtering when transmitting or receiving in SBFD slots.

Figure 10: Guard band configurations: Explicit configuration vs Scheduling based

For UL power control, RAN1 should evaluate limitations when using the legacy UL transmit power control procedure. For example, when the gNB configures higher open-loop power control parameters for a UE to overcome the inter-subband CLI and self-interference in the SBFD slot, this may result in higher UL interference in the full UL slot, where no inter-subband CLI and self-interference occurs. Similar considerations apply to the closed loop power control component.

RAN1 to study potential benefits and specification impacts of

  - Random Access and initial access when using the SBFD UL subband

  - Guard band configuration of SBFD UL subband and DL subband(s) provided to the UE

  - Enhancements to UL transmit power control, link adaptation procedures and QCL assumptions for transmissions using the SBFD UL subband

Conclusion

In this contribution, we made the following observations and proposals:

Legacy UEs can be scheduled in the SBFD UL subband by configuring flexible symbols in the SBFD slot using existing RAN1 specifications and UE behavior.

- The SBFD frequency-domain occupation, e.g., the location and bandwidth of the SBFD UL subband in the BS channel BW is configurable.

The SBFD time-domain resources, e.g., the symbols/slots of the TDD UL-DL frame configuration in which the SBFD UL subband can be scheduled are configurable.

SBFD can be configured in the UE channel BW of a single NR carrier. Further study the potential benefits and impacts when using intra-band CA or configuring SUL for SBFD.

SBFD operation is supported for UEs implementing only a single DL and UL BWP pair with aligned center frequency

RAN1 to study the use and evaluate the potential impacts from SBFD when a TDD serving cell with SBFD is configured as PCell, SCell or SpCell for a UE.

For a UE, the transmission direction of a symbol in the SBFD UL subband is determined by gNB scheduling

The cell-common or UE dedicated SBFD UL subband configuration, e.g., symbols/slots and the frequency occupancy in which UL transmissions can be scheduled by the gNB, is provided to the UE.

Multi-slot PUSCH, e.g., PUSCH repetition Type A, and PUCCH repetitions can be configured for transmission using only the SBFD slots/symbols.

RAN1 to study the impacts of SBFD operation on DL resource allocation and resource mapping in slots/symbols configured with an SBFD UL subband.

A DL reference signal, e.g., NZP CSI-RS resource(s) can be configured for the UE to report inter-subband UL-DL interference.

The UE maximum output power is configurable per slot in a TDD serving cell with SBFD.

RAN1 to study potential benefits and specification impacts of

  - Random Access and initial access when using the SBFD UL subband

  - Guard band configuration of SBFD UL subband and DL subband(s) provided to the UE

  - Enhancements to UL transmit power control, link adaptation procedures and QCL assumptions for transmissions using the SBFD UL subband

- References

RP-220633 “Study on Evolution of NR Duplex Operation,” RAN#95-e

R1-2203903 “Deployment scenarios and evaluation methodology for NR duplex evolution,” Samsung

R1-2203905 “Dynamic and flexible TDD for NR duplex evolution,” Samsung

Hyoungju Ji et al., “Extending 5G TDD Coverage with XDD: Cross Division Duplex,” IEEE Access, March 2021