3GPP TSG-RAN1 #109e	              R1-2203944

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.3.3

Source:	NEC

Title:	Views on enhancements of dynamic/flexible TDD

Document for:  	Discussion and decision

# 1 Introduction

In RAN #94 meeting, a new SID [1] on evolution of NR duplex has been approved. The objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios and use cases (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

This contribution provides NEC views on potential enhancements on dynamic/flexible TDD.

# 2 Discussion

## 2.1 Slot format enhancement for dynamic/flexible TDD operation

NR TDD specifications allows dynamic/flexible allocation of downlink and uplink in time. UE can be configured with tdd-UL-DL-ConfigurationCommon, or additionally provided tdd-UL-DL-ConfigurationDedicated, where the parameter tdd-UL-DL-ConfigurationDedicated overrides only flexible symbols per slot configured by tdd-UL-DL-ConfigurationCommon. A slot configuration period and a number of downlink symbols, uplink symbols, and flexible symbols in each slot of the slot configuration period are determined from tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated and are common to each configured BWP. Besides, a set of slot format combinations by slotFormatCombinations can be configured, where each slot format combination in the set of slot format combinations includes one or more slot formats. A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots.

For the flexible symbols, some new methods can be considered to improve configuration flexibility for NR TDD operations. For example, the flexible symbols per slot over the number of slots as provided by common-DCI can be further assigned to UE as DL or UL through another UE-specific DCI format x_y. This DCI can be scrambled by a specific RNTI, such as UE-SFI-RNTI, and it overrides only flexible symbols per slot over the number of slots as provided by common-DCI indicated. The priority of the slot format indicated by this DCI is higher than the DCI scrambled by C-RNTI, but lower than DCI 2_0 and tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated. UE-specific SFI indication in DCI format x_y provides the following information:

-  A slot index for a slot provided by slotIndex.

- nrofDownlinkSymbols provides a number of downlink symbols in the slot from the first flexible symbol.

- nrofUplinkSymbols provides a number of uplink symbols from the last flexible symbol in the slot.

The remaining symbols in the slot are still flexible symbols. Furthermore, for the flexible symbols, they can also be considered as shared resource between UL and DL, therefore, LBT scheme applied in shared spectrum can be considered to determine the flexible symbols used for DL or UL transmission.

Proposal 1: Enhancement for the flexible symbols allocation can be studied, such as:

- Methods to achieve different UE interpretation different slot format for flexible symbols can be studied.

- LBT scheme can be applied to determine the flexible symbols used for DL or UL transmission.

## 2.2 CLI management for dynamic/flexible TDD

As is well known, cross-link interference between different UEs or between gNBs exist in dynamic/flexible TDD. In the Rel-14 NR, a SI on dynamic TDD operation with mitigation of cross-link interference (CLI) was studied but without WI. And the CLI mitigation aspects for UE-to-UE CLI from the Rel-14 SI were addressed in the Rel-16 work item NR RIM afterwards, such as SRS-RSRP and CLI-RSSI measurements and reporting at the UE, and the network coordination mechanisms including exchange of intended DL/UL configuration among gNBs. However, the CLI between gNB/TRP was not specified due to the limited time, therefore, at least gNB-to-gNB CLI should be studied in Rel-18 evolution of duplex.

For gNB-to-gNB CLI, we think below aspects should be considered and further studied.

- The measurement metric for TRP-to-TRP measurement, such as the CLI sensitivity level, should be defined.

- How to support TRP-to-TRP CLI measurements, based on the RS or the IM resource. More specifically, how to define resource configuration and sequence for RS/IM resource to optimally handle TRP-TRP interference measurement.

- How to protect the resource for TRP-to-TRP IM. For UE, some indication is needed to resolve the issue when the PRACH/PUCCH/PUSCH resource overlapped or at least part overlapped with the TRP-TRP measurement RS resource. Such as do rate matching or reserve the resource.

- To ensure the CLI IM accuracy, if some blank resource is reserved, the spectrum efficiency will be effected.

Proposal 2:

- For gNB-to-gNB CLI measurement,

  - the measurement metric should be defined first, such as CLI sensitivity level.

  - Study the resource configuration and RS sequence properties for IM resources to optimally handle TRP-TRP interference measurement.

Moreover, to support gNB-to-gNB CLI measurement and interference mitigation, more information exchange is required between gNBs other than sharing UL/DL configurations. Some points to consider for the same are:

- CLI RS configuration needs to be implicitly or explicitly shared between gNBs for interference measurement

- Information exchange should allow victim gNB to identify the aggressor gNBs/TRPs identity from CLI RS measurement

- Assistance information sharing between gNBs to mitigate the interference observed by the victim gNB

Proposal 3:

- Following points need to be studied further for gNB-gNB interference mitigation using inter-gNB signaling

  - CLI RS configuration needs to be implicitly or explicitly shared between gNBs for interference measurement

  - Information exchange should allow victim gNB to identify the aggressor gNBs/TRPs identity from CLI RS measurement

  - Assistance information sharing between gNBs to mitigate the interference observed by the victim gNB

For UE-to-UE CLI, besides the interference and report scheme specified in R16, other scheme, such as TPC or new measurement methods can be considered. For example, new metrics (CLI sensitivity level and CLI tolerance level) for CLI management can be defined and gNB can adjust the transmission direction or the channel/traffic to reduce the CLI according to whether the CLI sensitivity level of the transmission resource meet the requirement of the CLI tolerance level of the channel/traffic.

Furthermore, when gNB-to-gNB and UE-to-UE are both considered simultaneously, the RS overhead should be considered if both RS are used for inter UE and gNB CLI measurement. And a common framework for cross-link interference mitigation schemes for both gNB and UE should be studied. If the DL and UL RS used for gNB-gNB or UE-UE IM occupy different time-frequency resource elements, and the REs for transmitting RS of one link should be blanked or muted on the other link to assure the reliability of the measurement, the spectrum efficiency will be effected. Therefore, the unified design for CLI RS should be considered to reduce the overhead for IM.

Proposal 4:

- Unified design for CLI RS for gNB-to-gNB and UE-to-UE measurement should be considered to reduce the RS overhead.

However, backhaul coordination, such as exchange the transmission direction information or RS configuration through Xn for inter-TRP may not be possible or could be limited (e.g. backhaul with large or unreliable latency, or multi-operator scenarios) for CLI management. And measurement report based long-term IM method cannot meet the requirement if the interference changed rapidly. Therefore, short-term method, such as sensing based scheme for interference measurement can also be studied.

Proposal 5:

- Sensing based scheme can be studied to avoid the CLI.

Finally, mechanisms to progressively mitigate interference based on measurement or report of measurement results should be studied. Basically, measurement can be conducted either by victim TRP or aggressor TRP when channel reciprocity holds. In this regard, interference mitigation can be initiated either by reporting of interference level from victim TRP or measurement of high interference level by aggressor TRP.

Proposal 6:

- Mechanisms to progressively mitigate interference based on measurement or report of measurement results should be studied.

# 3 Conclusion

Based on the discussion in the previous sections, this contribution is concluded with following proposals:

Proposal 1: Enhancement for the flexible symbols allocation can be studied, such as:

- Methods to achieve different UE interpretation different slot format for flexible symbols can be studied.

- LBT scheme can be applied to determine the flexible symbols used for DL or UL transmission.

Proposal 2:

- For gNB-to-gNB CLI measurement,

  - the measurement matric should be defined first, such as CLI sensitivity level.

  - Study the resource configuration and RS sequence properties for IM resources to optimally handle TRP-TRP interference measurement.

Proposal 3:

- Following points need to be studied further for gNB-gNB interference mitigation using inter-gNB signaling

  - CLI RS configuration needs to be implicitly or explicitly shared between gNBs for interference measurement

  - Information exchange should allow victim gNB to identify the aggressor gNBs/TRPs identity from CLI RS measurement

  - Assistance information sharing between gNBs to mitigate the interference observed by the victim gNB

Proposal 4:

- Unified design for CLI RS for gNB-to-gNB and UE-to-UE measurement should be considered to reduce the RS overhead.

Proposal 5:

- Sensing based scheme can be studied to avoid the CLI.

Proposal 6:

- Mechanisms to progressively mitigate interference based on measurement or report of measurement results should be studied.

# 4 References

- RP-220633, Revised SID: Study on evolution of NR Duplex Operation.