3GPP TSG RAN WG1 #109-e		R1-2204010

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Study on network energy saving techniques

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

In RAN #94e meeting, a new SI for study on network energy savings for NR was approved [1]. One objective of the study is the following:

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

In this contribution, we discuss some potential techniques on the gNB and UE side to improve network energy savings.

Discussion

The motivation of the SI is to study some potential network energy savings techniques to reduce the network energy consumption. When developing the network energy saving techniques, some principles should be kept in order not to introduce a large impact on current network. For example, the network energy saving techniques should avoid having a large impact on user perceived throughput and access latency, should avoid increasing additional burden on UE implementation complexity, and should avoid increasing UE power consumption and cost.

- The network energy saving techniques should avoid having a large impact on user perceived throughput and access latency.

- The network energy saving techniques should avoid increasing UE implementation complexity.

- The network energy saving techniques should avoid increasing UE power consumption and cost.

Cell on/off at network side

According to the SID [1], the study should prioritize idle/empty and low/medium load scenarios including both single-carrier and multi-carrier deployments, and some example scenarios are listed as:

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

For the multi-carrier deployment scenario, it is possible that the loads are different at different time periods. Take a shopping mall for example, the load may be heavy in the day time and may be light at night. Then when the load is low/medium in the night, the gNB may adopt some energy saving techniques to reduce the network power consumption without introducing a large impact on the system performance. One straightforward way is to put all the UEs into one serving cell, so that the gNB can turn off the other serving cells to save power. However, since the serving cells which the gNB decides to turn off could be the Pcell of some UEs, the gNB should not directly turn off these cells. Instead, the gNB should indicate the UEs to change their Pcell first.

Figure 1 gives an example on illustration of Pcell change procedure. Assume Cell1 is Pcell and Cell2 is Scell for a UE under multi-carrier deployment, and gNB decides to turn off Cell1, then the gNB may send a change cell trigger to the UE to ask for Pcell change, and the UE should perform Pcell change procedure in response to the change cell trigger. After the UE successfully change its Pcell from Cell1 to Cell2, and the gNB can make sure no UEs take Cell1 as Pcell, then the gNB can turn off Cell1. From UE side, the connection with the gNB can still be maintained.

Figure 1: Illustration of UE Pcell change procedure

- Study the Pcell on/off procedure at network side for power consumption.

Network PS due to periodic transmission

According to the SID [1], the power consumption of a radio access can be split into two parts: the dynamic part which is only consumed when data transmission/reception is ongoing, and the static part which is consumed all the time to maintain the necessary operation of the radio access devices, even when the data transmission/reception is not on-going. The periodic DL transmissions, e.g., SSB, system information and other non-unicast DL transmissions belong to the static part. To reduce power consumption for this part, one potential solution is to offload these common channels/signals of one cell to another cell, so that the gNB only needs to maintain minimum transmissions and can go to sleeping mode for that cell.

Figure 2 gives an example on illustration of common channel/signal offload procedure. Assume gNB decides to configure Cell1 in sleeping mode, then after that the gNB may only transmit SSB on Cell1 and transmit the system information, as well as the potential paging/broadcast/multicast channels associated with Cell1 on Cell2. For idle UEs accessing Cell1, they can be indicated to receive the system information for Cell1 on Cell2. And then Cell1 can stay in sleeping mode without introducing a large impact on system performance.

Figure 2: Illustration of common channel/signal offload

- Study the common channel/signal offload procedure for power consumption.

Network PS due to periodic reception

Similar to periodic DL transmissions, the periodic UL transmissions also belong to the static part. Take CG-PUSCH resources as example, once configured/activated, gNB should always blindly detect whether UE performs UL transmission, then unnecessary network power is consumed, as shown in Figure 3. But if the CG-PUSCH resources are not configured/activated, then UE may experience a large UL transmission delay due to the SR/BSR procedure for UL resource application.

Figure 3: Unnecessary power consumption due to blind detection for periodic UL transmission

One potential solution is that UE may provide some assistance information, e.g., UE may report information whether the configured UL transmission can be skipped for gNB reception. Figure 4 gives an example on illustration of UE reporting assistance information. Since UE may have additional information on the arrival of UL data, the UE may report deactivating CG-PUSCH resources information to gNB, to indicate that it does not plan to perform UL transmission on those CG resources, accordingly the gNB does not need to receive on those resources.

Figure 4: Illustration of assistance information report

- Study UE reporting assistance information for power consumption.

Conclusion

This contribution discusses some potential techniques on the gNB and UE side to improve network energy savings and provides the following proposals.

- The network energy saving techniques should avoid having a large impact on user perceived throughput and access latency.

- The network energy saving techniques should avoid increasing UE implementation complexity.

- The network energy saving techniques should avoid increasing UE power consumption and cost.

- Study the Pcell on/off procedure at network side for power consumption.

- Study the common channel/signal offload procedure for power consumption.

- Study UE reporting assistance information for power consumption.

Reference

[1] RP-213554, New SI: Study on network energy savings for NR, Huawei.