3GPP TSG-RAN WG1 #109-e	R1-2204032

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.1

Source:	Ericsson

Title:	Discussion on UE features for FeMIMO

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the UE features for feMIMO. We also recall the RAN2 LS “Guidelines on UE capability definitions” in section 3 to raise RAN1 attention on the expectation from RAN2.

The current UE feature list is included in the Appendix.

# 2	Discussion

## 2.1	Enhancements to multi-beam operation

The work in the multi-beam agenda targets to develop a more efficient TCI framework. Unnecessary flexible configurations are removed, and the signalling mechanisms for the critical parts are streamlined. However, this work has led to that some Rel-15/16 functionality has been replicated, and based on this replicated functionality, enhancements have been added.

It may be tempting to define a basic level of functionality that contains only the replicated functionality. This would bring the Rel-17 TCI framework on par with the Rel-15/16 functionality and would lead to a functional feature. However, there is no incentive for an operator to deploy only the replicated functionality, and there is no incentive for a NW vendor to implement only the replicated functionality:

There is no incentive for a NW vendor to implement only the parts of the Rel-17 TCI framework that replicated the Rel-15/16 functionality.

Based on Observation 1, we conclude that a UE that supports the Rel-17 TCI framework must also support enhancements on top of the replicated functionality:

A UE that supports the Rel-17 TCI framework must support enhancements relative the Rel-15/16 TCI framework: it is not sufficient to support only the replicated functionality.

To reiterate: no NW will implement only the duplicated functionality. There is no point that a UE only implements and advertises support for the replicated functionality. This would be a useless paper product.

A UE that supports the Rel-17 TCI framework would thus have to also support enhancements. Overall, the Rel-17 TCI framework supports two main performance enhancements:

- DCI-based TCI state update for all channels

- Inter-cell beam management

To make the Rel-17 TCI framework attractive for an operator to deploy and attractive for a NW vendor to implement, the basic level of UE support should include both DCI-based TCI state update and inter-cell beam management:

A UE that supports the Rel-17 TCI framework should support both DCI-based TCI state update and inter-cell beam management.

The support described in Proposal 1 could be on a minimum level: for example, DCI-based TCI state update requires that the UE supports two active TCI states – but not more than that. Inter-cell beam management would require that the UE can measure on SSBs associated with one PCI different from the serving cell PCI – but not more than one. More advanced UEs could then advertise larger values for these capabilities.

### 2.1.1	The FG 23-1-1 family

During RAN1#108-e, there was significant progress on content of the FG 23-1-1 family. The content is stabilizing, and we are starting to clean up FFSs and removing brackets.

For FG 23-1-1, there are one WA, one FFS and some brackets. Here we propose:

Confirm the WA The maximum number of configured joint TCI states.

Remove the FFS: how to count the MAC-CE activated joint TCI

The release 17 TCI framework is built upon the three-stage signalling: RRC+MAC-CE+DCI. TCI states that can be indicated by DCI are activated using DCI. There is no ambiguity.

In addition, FG 23-1-1 contain a number of brackets. Most of these statements in brackets can be removed – they are either wrong or unnecessary. Here we propose the following modifications:

- Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - One MAC-CE activated joint TCI state per CC [in a band] [in a band combination] - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band

For FG 23-1-1b, there is some yellow remaining highlighting on the definition of beam application time. RAN1 has spent a lot of time to define the beam application time, and component 2 is consistent with the RAN1 agreement. Component 4 is irrelevant – there is no need for such a component. Also in this case, most of the remaining brackets seem unnecessary:

- TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied]

For FG 23-1-1c, there is a strange component, which proposes to introduce a limitation on the total number of CCs configured with BFR. For this component, there is no relation to the Rel-17 TCI framework. The motivation that was brought forward was that this component would change the interpretation of a Rel-16 feature. This will not work: the NW will not check Rel-17 UE features to enable Rel-16 features. If necessary, a Rel-16 CR could be considered. In any case, component 2 should be removed:

1. Support of SCell BFR with unified TCI framework [2. Maximum number of CCs configured with SCell BFR with unified TCI framework [in a band with SpCell BFR]

For FG 23-1-d, there is an FFS on if a UE needs to support 23-1-1d if it supports 23-1-1, i.e., if it’s part of the basic support of the unified TCI. Support of TCI states per CC is part of the basic support in Rel-15, and should remain also for Rel-17:

A UE that supports 23-1-1 must also support 23-1-1d.

The corresponding FFS exists for 23-1-1e, but this is related to the new reference CC feature. In contrast to 23-1-1d, this is new functionality which should be optional:

FG 23-1-1d is optional, and a UE that supports 23-1-1 may or may not support 23-1-1e.

In FG 23-1-1g, there is an FFS on the applicability in FR1. Clearly, a spatial relation is only relevant in FR1, but it is not clear to us why the note is needed. There is no need for the UE to report it, and even if the UE reports that it supports beam misalignment in FR1, the NW will ignore the reported capability.

For FG 23-1-1h, there is a note in brackets if 23-1-1h is part of the basic functionality. Here we do not see that this is needed:

FG 23-1-1h is optional, and a UE that supports 23-1-1 may or may not support 23-1-1h.

In FG 23-1-1i, the open issues regarding SRS configuration have been resolved, and the brackets in 23-1-1i can be removed:

Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signaling/configuration design(s)

Also, there is a note in FG 23-1-1i that discusses if 23-1-1i should be part of the basic functionality. Here we do not see that this is needed.

For several of the FGs, there is a note on what FGs need to be part of the basic functionality. On this aspect, we propose

The following FGs in the 23-1-1- family are part of the basic functionality: FG 23-1-1, FG 23-1-1b, FG 23-1-1d, and FG 23-1-1f.

Support for the remaining FGs in the 23-1-1 family can be independently advertised by the UE.

In summary, we propose the following modifications in the components for the 23-1-1 family:

| 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - One MAC-CE activated joint TCI state per CC [in a band] [in a band combination] - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band |
| --- | --- | --- |
| 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] |
| 23-1-1c | SCell BFR with unified TCI framework | 1. Support of SCell BFR with unified TCI framework [2. Maximum number of CCs configured with SCell BFR with unified TCI framework [in a band with SpCell BFR] |
| 23-1-1d | Per BWP TCI state pool configuration for CA mode | 1. Support of TCI state pool configuration per BWP for CA mode |
| 23-1-1e | TCI state pool configuration with TCI pool sharing for CA mode | 1. Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC 2. The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band |
| 23-1-1f | Common multi-CC TCI state ID update and activation | Common multi-CC TCI state ID update and activation |
| 23-1-1g | Beam misalignment between the DL source RS in the TCI state | Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS |
| 23-1-1h | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS | For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS |
| 23-1-1i | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] | Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signaling/configuration design(s) |
| 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) |

### 2.1.2	FG 23-1-2

In FG 23-1-2, the inter-cell beam measurements are discussed, and several components are still highlighted.

The first highlighting is related to the name of the FG: should it be for inter-cell beam management only, or should inter-cell mTRP be included in the same FG? Here we propose to include both use cases in the same FG:

FG 23-1-2 includes both inter-cell BM and inter-cell mTRP.

The benefit of component 2 is that it could indicate the maximum number of beams that can be included in the RSRP report. However, the actual reporting of an inter-cell beam measurement is not more complicated than an intra-cell beam measurement, which is described by the intra-cell reporting beam reporting feature maxNumberNonGroupBeamReporting. Hence, component 2 can be reformulated:

2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] is associated with a PCI different from serving cell PCI can be reported] (FFS: if K is a component candidate value) K is equal to maxNumberNonGroupBeamReporting

For component 3, there are two FFSs: whether to split for FR1/FR2 and if to split for different types of reporting. Since the capability is reported per band, the FR1/FR2 differentiation exists automatically, and that FFS can be removed. Regarding the reporting type (periodic/aperiodic/semi-persistent), any potential difference in complexity would be captured by the general reporting capabilities, e.g., csi-ReportFramework. Hence, the second FFS can also be removed.

Component 5 proposes to limit the number of SSB resources that can be configured for inter-cell beam measurements. The target is to limit memory consumption of the UE.

The current assumption is that one additionalPCIIndex-r17 is configured for every SSB in a CSI-SSB-ResourceSet. Hence, the UE would at most be configured with 3 additional bits for SSB index for other PCIs. The additional memory that the UE needs for this purpose would thus be insignificant. Hence, we propose

Component 5 in FG 23-1-2 is removed, since the additional memory required is marginal.

The idea with component 6 is to define a baseline mode of operation where only the PCIs that are configured for L3 mobility measurements can be reported in an inter-cell beam report. In this baseline mode of operation, the UE could reuse the measurements it anyway performs for L3 mobility and report only those over L1. However, the default operation in NR (and LTE) is that the UE is not configured with specific PCIs for L3 mobility. Instead, the measurement object contains only an ARFCN, and the UE finds all the relevant PCIs. Since it is not clear how component 6 would be relevant for this normal configuration, we propose to remove component 6.

Remove component 6, since it is unclear how it would apply in case no PCIs are configured for L3 mobility measurements.

Regarding component 7, the input from RAN4 [2] is important: For FR2, RAN4 will not define requirements for inter-cell L1-RSRP measurements for the case when all the SSB of the cell with different PCI overlap with the SMTC window. For FR1, RAN4 will define requirements for any SSB-SMTC overlaps.

For almost all NR deployments, the SMTC window covers all SSB transmissions: it is a waste of resources to transmit SSBs that the UE is not able to measure. It is thus imperative to have support for that case. However, in FR2, the only measurement for which there will be requirements is the case where not all the SSBs overlap with the SMTC window. The important cases would thus be inside SMTC for FR1 and outside SMTC for FR2. To avoid fragmentation, we propose to remove the component.

All NR deployments rely on overlapping SSB transmissions: essentially it is assumed that all cells can transmit all SSBs. A UE that cannot perform measurements on overlapping SSBs would in practice not support inter-cell beam measurements. Hence, component 8 and 9 can be removed:

Component 8 and 9 in FG 23-1-2 can be removed since measurements on overlapping SSBs are required.

Since RAN4 assumes that the timing offset between the serving cell SSB and SSBs of a cell with another PCI is smaller than the CP, performing measurements is not associated with any significant additional complexity.

In summary, we propose the following modification of FG 23-1-2:

| 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] is associated with a PCI different from serving cell PCI can be reported] (FFS: if K is a component candidate value) K is equal to maxNumberNonGroupBeamReporting 3. The maximum number of [RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement] (FFS: whether to split this for FR1 and FR2) (FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement) 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC] [5. The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] [6. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [7. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [8. Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}] [9. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] |
| --- | --- | --- |

### 2.1.3	Separate DL/UL TCI

No components have been agreed for separate UL/DL TCI. The assumption is FG 23-1-1 can be used as a starting point to define such components.

Since it is unlikely that separate UL/DL TCI is implemented early, it would make sense that separate UL/DL TCI is a separate FG, to facilitate IODT:

Separate DL/UL TCI is a separate FG, and joint DL/UL TCI is a prerequisite FG.

A straightforward extension of component 1 is to replace “joint” with “separate”. However, one additional modification would make sense. Note that SRS is a valid source RS for UL TCI, and that SRS cannot be used as a PL RS. Defining beam alignment between SRS and a DL RS is non-trivial, since in the most realistic case, the SRS is configured without any relation to a DL RS. Based on this argument, we assume that a UE that supports separate DL/UL TCI would have to also support an arbitrary PL RS:

The basic component for ‘separate DL/UL TCI’ would be: “Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, support for beam misalignment)”

Some of the components discussed in subsection 2.1.1 are related to the number of TCI states, both configured and activated.

Since joint DL/UL TCI is a prerequisite FG for the separate DL/UL TCI, and a joint DL/UL TCI is identical to a DL TCI state from a configuration point of view, we propose

A UE that supports separate UL/DL TCI states supports the same number of configured DL TCI states as configured joint TCI states.

Of course, there is no need to introduce a separate component – the number reported under FG 23-1-1 can be reused. However, it is required that the UE reports how many configured UL TCI states it supports in addition to the joint UL/DL TCI states:

A UE that supports separate UL/DL TCI states reports how many configured UL TCI states it supports in addition to the number of joint/DL TCI states.

Note that the number of configured TCI states that are needed is not reduced with the introduction of separate DL/UL TCI. The same is true for the activated TCI states: the number of activated DL TCI states that are needed is not reduced with the introduction of UL TCI states. Hence, we have the corresponding proposals for the activated TCI states:

A UE that supports separate DL/UL TCI states supports the same number of activated DL TCI states as activated joint TCI states.

A UE that supports separate DL/UL TCI states reports how many activated UL TCI states it supports in addition to the number of activated joint TCI states.

The UL TCI states are now defined in the UL BWP. It would thus make sense to define UE features for the maximum number of UL TCI states per BWP and the total maximum number of UL TCI states across all CCs:

Introduce components for the maximum number of TCI states per BWP and across all CCs.

Regarding the beam application time, there is no need for a separate component for separate DL/UL TCI states – the value from FG 23-1-1b is reused.

Also, there is no need for a counterpart of FG 23-1-1c, since SCell BFR is not related to UL TCI states. The same is true for FG 23-1-1j.

Based on a cleaned-up version of the table in section 2.1.1, we propose the following FGs for separate DL/UL TCI:

| 23-1-5 | Unified TCI with separate DL/UL TCI update for intra- and inter-cell beam management | - Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, support for beam misalignment) - The maximum number of configured UL TCI states per BWP per CC - One MAC-CE activated UL TCI state per CC - TCI state indication: update and activation a) MAC CE based TCI state indication - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band |
| --- | --- | --- |
| 23-1-5b | Unified TCI with separate DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication: update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated UL TCI states per CC |
| 23-1-5d | Per BWP UL TCI state pool configuration for CA mode | 1. Support of UL TCI state pool configuration per BWP for CA mode |
| 23-1-5e | UL TCI state pool configuration with UL TCI pool sharing for CA mode | 1. Support of reference BWP/CC configured with reference UL TCI state pool shared by a set of BWP/CC 2. The maximum number of configured UL TCI state pools across all BWPs and all CCs in a band |
| 23-1-5f | Common multi-CC UL TCI state ID update and activation | Common multi-CC TCI state ID update and activation |
| 23-1-5h | Association between UL TCI state and UL PC settings for PUCCH, PUSCH, and SRS | For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS |
| 23-1-5i | Indication/configuration of R17 UL TCI states for SRS | Support of indication/configuration of R17 TCI states for SRS reusing the Rel-15/16 signaling/configuration design(s) |

### 2.1.4	FG	23-1-3

FG 23-1-3 describes enhancements to the P-MPR reporting. The basic capability would include support for a N (P-MPR,SSBRI/CRI) pairs in a new PHR MAC CE. The name of the FG is “MPE mitigation”, which is not really accurate: there are other means to perform MPE mitigation.

In addition to the naming of the feature, there are only small details to finish in this FG: a few brackets and candidate values. In the component description, we propose to remove the brackets:

1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation

For the name of the FG, we propose to describe what the feature does: “Enhanced PHR”, and the consequence if not supported would then simply be “Enhanced PHR not supported”:

Rename FG 23-1-3 “Enhanced PHR”, and update “consequence if not supported” accordingly.

Also, the candidate values for component 3 are highlighted. The proposed values seem sufficient: this would probably mean that all candidate RSs would be confined to the same cell: there are not enough candidate RSs to configure them in several cells. This is probably OK.

We propose the following changes:

| 23-1-3 | MPE mitigation Enhanced PHR | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  | Yes |  | [MPE mitigation is not supported] Enhanced PHR is not supported | Per Band | n/a | n/a | n/a | 2. Candidate value of {1,2,3, 4} 3. Candidate value [{8, 12, 16, 28, 32, 48, 64}] Note: FR2 only |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 2.1.5	FG 23-1-4

FG 23-1-4 describes the feature where the UE can include a capability index in its beam report. The current name of the feature is misleading since multi-panel UEs are inherently supported already in Rel-15. The consequence if not supported is also misleading.

We first note that the description includes [set]. RAN2 has now defined RRC signaling without “set”, so it would be appropriate to remove “[set]” from the description.

Furthermore, by reporting this quantity, the UE would also have to support to include the capability value in the beam report – otherwise the functionality would be useless. This functionality is enabled using the RRC parameter reportQuantity-r17- This should be part of the component description.

In summary, we propose

| 23-1-4 | MPUE support for UL Capability value reporting | 1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set] 2. Support reportQuantity-r17 |  | Yes |  | [MPUE support for UL is not supported] Capability value reporting is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: Up to 4 value [sets] each with one value of {[0,]1,2,4} Note: the reported list contains only unique value [sets] This FG is a working assumption |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 2.2	Enhancements to multi-TRP deployment

### 2.2.1 FG 23-4 (Inter-cell mTRP)

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs per CC is X1 (case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 | FFS 16-2a | Yes |  | IntCell-mTRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {[0,]1,2,3,[ 4, 5, 6,] 7} Component 3 candidate values: {0,1,2,3, [ 4, 5, 6,]7} Note: UE indicates a non-zero value for at least one of component 2 or component 3 FFS: how to count X1 and X2 [Note: case1 and case2 cannot be enabled simultaneously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The Rel-17 Inter-cell mTRP is an extension of Rel-16 Multi-DCI based Multi-TRP functionality. FG 16-2a which indicates UE is capable of CORESETPoolIndex configuration shall be supported as a prerequisite feature group for FG 23-4.

On FG 23-4, Add FG16-2a as prerequisite feature group for FG 23-4. Remove candidate values in yellow to avoid fragmented UE capability report.

### 2.2.2 FG 23-6-1 and FG 23-6-x (HST SFN)

| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 or 23-6-1b | Yes |  | Dynamic switching - scheme A is not supported | per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

On 23-6-1a, the prerequisite feature for dynamic switching-scheme A shall be confirmed, which is “23-6-1 or 23-6-1b”.

| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup for SFN | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold, if applicable 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | [23-6-1 or 23-6-2] | Yes |  | Default DL beam setup for SFN is not supported | Per band | n/a | n/a | n/a | Note: FR2 only for component 1 and 3 only | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

On 23-6-4, there’s a need to clarify what is the “Default DL beam setup for SFN”. In our understanding the “Default DL beam setup for SFN” refers to the UE behaviour enableTwoDefaultDCI-states is configured.

On 23-6-4, clarify the “Default DL beam setup for SFN” as “Default DL beam setup when enableTwoDefaultDCI-states is configured”.

# 3	Guidelines for UE capability definitions

It is beneficial to recall RAN2’s guidelines for UE capability definitions, as documented in the RAN2 LS Error! Reference source not found. and copied below:

1	Avoid defining “incapability” bits as they may cause interpretation issues

The definition of the capability should not say that “a UE setting the bit does not support Rel-16 feature X”. Such statements caused a lot of problems in Rel-15. One example was the pucch-F0-2WithoutFH that indicates that “the UE does not support PUCCH formats 0 and 2 without frequency hopping”.

2	Absence of a capability bit shall not imply support for something that is not yet defined

One example is “absence of channelBW means that the UE supports all BWs”. Since new BWs might be added later (and in a release independent manner) and maybe in some other specification, “all BWs” can become ambiguous. Any legacy UE would suddenly be expected to support the new bandwidth as well.

Another example is the default capability for UE power class and UL duty cycle. From signalling point of view, reducing the overhead is marginal (unless it is reported per-BandCombination or Per-Band-of-a-BandCombination as discussed in bullet 4 below). Instead, gNB faces difficulty in comprehending the implicit capabilities amongst UEs compliant with different versions in a release.

3 	Define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature

If a feature has a baseline feature, the baseline feature must be clearly defined. It is not enough to write that “if the UE does not set the bit it does not support feature group x-y”.  If a capability bit for a certain “feature X” has been defined, all UEs that support “feature X” shall set the bit, and additional parts of the feature may not be supported by all UEs.

Capability signalling and rules to determine whether a UE supports a feature should not be mixed. For example, avoid to say

“In TDD band combinations, only UEs that set the featureX capability bit support feature. In FDD-only band combinations all UEs support feature X even if they do not set the bit.”

If later FDD-only band combinations are defined, where UEs are not required to support the feature, band combinations will not be release independent. An example where this caused problem in Rel-15 was the capability simultaneousRx-Tx for EN-DC.

4 	Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination”

The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account.

It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered.

5 	Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

6	UE capabilities defined in specifications must be self-contained

RAN2 would also like to point out that once the “snapshot” of defined Rel-16 capabilities have been captured in RAN2 TR 38.822, RAN2 does not plan to update this TR. Discussions (on any WG capabilities) should be based on the actual specification text. RAN2 will document the capabilities in TS 3x.306, preferably with references to RAN1/RAN4 specifications and to corresponding RRC configurations (fields and information elements).

RAN2 understands that RAN1 uses the “feature IDs”, since they are the only available definitions during the initial stage of discussions. RAN2 would like to request that once the RRC signalling is in place and can be mapped to each “feature ID”, the actual field names used in the signalling are also utilized in the RAN1/RAN4 capability discussions (to avoid inter-WG confusion e.g. in LSs).

7	Rationale for necessity of both xDD and FRx differentiations for per-UE capability

RAN2 did not discuss the RAN1 LS on XDD-FRX Differentiation (R1-1913579/R2-2000013) at the RAN2#109-e meeting, but RAN2 would anyway appreciate to be provided with rationale in case Per-UE capability with both FDD/TDD and FR1/FR2 differentiations is deemed as necessary for a feature.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There is no incentive for a NW vendor to implement only the parts of the Rel-17 TCI framework that replicated the Rel-15/16 functionality.

Observation 2	A UE that supports the Rel-17 TCI framework must support enhancements relative the Rel-15/16 TCI framework: it is not sufficient to support only the replicated functionality.

1.	2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination

	List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination

1.	2. Support parameter combinations with M=2

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE that supports the Rel-17 TCI framework should support both DCI-based TCI state update and inter-cell beam management.

Proposal 2	Confirm the WA The maximum number of configured joint TCI states.

Proposal 3	Remove the FFS: how to count the MAC-CE activated joint TCI

Proposal 4	A UE that supports 23-1-1 must also support 23-1-1d.

Proposal 5	FG 23-1-1d is optional, and a UE that supports 23-1-1 may or may not support 23-1-1e.

Proposal 6	FG 23-1-1h is optional, and a UE that supports 23-1-1 may or may not support 23-1-1h.

Proposal 7	The following FGs in the 23-1-1- family are part of the basic functionality: FG 23-1-1, FG 23-1-1b, FG 23-1-1d, and FG 23-1-1f.

Proposal 8	FG 23-1-2 includes both inter-cell BM and inter-cell mTRP.

Proposal 9	Component 5 in FG 23-1-2 is removed, since the additional memory required is marginal.

Proposal 10	Remove component 6, since it is unclear how it would apply in case no PCIs are configured for L3 mobility measurements.

Proposal 11	Component 8 and 9 in FG 23-1-2 can be removed since measurements on overlapping SSBs are required.

Proposal 12	Separate DL/UL TCI is a separate FG, and joint DL/UL TCI is a prerequisite FG.

Proposal 13	The basic component for ‘separate DL/UL TCI’ would be: “Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, support for beam misalignment)”

Proposal 14	A UE that supports separate UL/DL TCI states supports the same number of configured DL TCI states as configured joint TCI states.

Proposal 15	A UE that supports separate UL/DL TCI states reports how many configured UL TCI states it supports in addition to the number of joint/DL TCI states.

Proposal 16	A UE that supports separate DL/UL TCI states supports the same number of activated DL TCI states as activated joint TCI states.

Proposal 17	A UE that supports separate DL/UL TCI states reports how many activated UL TCI states it supports in addition to the number of activated joint TCI states.

Proposal 18	Introduce components for the maximum number of TCI states per BWP and across all CCs.

Proposal 19	Rename FG 23-1-3 “Enhanced PHR”, and update “consequence if not supported” accordingly.

Proposal 20	On FG 23-4, Add FG16-2a as prerequisite feature group for FG 23-4. Remove candidate values in yellow to avoid fragmented UE capability report.

Proposal 21	On 23-6-1a, the prerequisite feature for dynamic switching-scheme A shall be confirmed, which is “23-6-1 or 23-6-1b”.

Proposal 22	On 23-6-4, clarify the “Default DL beam setup for SFN” as “Default DL beam setup when enableTwoDefaultDCI-states is configured”.

# References

R1-2202929, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.)

R1-2203021, Reply LS on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap, RAN4, RAN4#102e, Feb 2022

# Appendix

## Agreement from RAN108-e meeting [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable between UEs | Consequence if the feature is not supported by the UE | Type |  | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - WA: The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - One MAC-CE activated joint TCI state per CC [in a band] [in a band combination] - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band |  | Yes |  | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management is not supported | Per band |  | n/a | n/a | n/a | FFS: how to count the MAC-CE activated joint TCI | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] | 23-1-1 | Yes |  | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC is not supported | Per band |  | n/a | n/a | n/a | Component 3 candidate values: [{2, …}] Note: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signaled in 23-1-1, component 5 | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-1-1c | SCell BFR with unified TCI framework | 1. Support of SCell BFR with unified TCI framework [2. Maximum number of CCs configured with SCell BFR with unified TCI framework [in a band with SpCell BFR] |  | Yes |  | SCell BFR with unified TCI framework is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-1-1d | Per BWP TCI state pool configuration for CA mode | 1. Support of TCI state pool configuration per BWP for CA mode | 23-1-1 | Yes |  | Per BWP TCI state pool configuration for CA mode is not supported | Per band |  | n/a | n/a | n/a | FFS: A UE that supports 23-1-1 together with CA must indicate this FG is supported] | Optional with capability signaling |  |
| 23. NR_FeMIMO | 23-1-1e | TCI state pool configuration with TCI pool sharing for CA mode | 1. Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC 2. The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band | 23-1-1 | Yes |  | TCI state pool configuration with TCI pool sharing for CA mode is not supported | Per band |  | n/a | n/a | n/a | Component 2 candidate values: FFS FFS: A UE that supports 23-1-1 together with CA must support this FG] | Optional with capability signaling |  |
| 23. NR_FeMIMO | 23-1-1f | Common multi-CC TCI state ID update and activation | Common multi-CC TCI state ID update and activation | 23-1-1 | Yes |  | Common multi-CC TCI state ID update and activation is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signaling |  |
| 23. NR_FeMIMO | 23-1-1g | Beam misalignment between the DL source RS in the TCI state | Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS | 23-1-1 | Yes |  | Beam misalignment between the DL source RS in the TCI state is not supported | Per band |  | n/a | n/a | n/a | FFS: FR2 only | Optional with capability signaling |  |
| 23. NR_FeMIMO | 23-1-1h | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS | For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS | 23-1-1 | Yes |  | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS is not supported | Per band |  | n/a | n/a | n/a | [Note: A UE that supports FG 23-1-1 must indicate this FG is supported] | Optional with capability signaling |  |
| 23. NR_FeMIMO | 23-1-1i | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] | Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band |  | n/a | n/a | n/a | Note: This has no impact on detail signaling design for SRS TCI indication [A UE that supports 23-1-1 must indicate this FG is supported] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band |  | n/a | n/a | n/a | [A UE that supports 23-1-1 must indicate this FG is supported] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] (FFS: if K is a component candidate value) 3. The maximum number of [RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement] (FFS: whether to split this for FR1 and FR2) (FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement) 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC] [5. The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] [6. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [7. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [8. Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}] [9. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] | [2-24, 2-29] | Yes |  | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] is not supported |  | per band | n/a | n/a | n/a | [Note: Whether component 4 and/or 5 are also counted in FG16-1g/16-1g-1] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  | Yes |  | [MPE mitigation is not supported] | Per Band |  | n/a | n/a | n/a | 2. Candidate value of {1,2,3, 4} 3. Candidate value [{8, 12, 16, 28, 32, 48, 64}] Note: FR2 only | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | 1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set] |  | Yes |  | [MPUE support for UL is not supported] | per band |  | n/a | n/a | n/a | Component 1 candidate values: Up to 4 value [sets] each with one value of {[0,]1,2,4} Note: the reported list contains only unique value [sets] This FG is a working assumption | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FDM based PDCCH repetition for FR2)] including PDCCH repetition for Type 3 CSS 2. Required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  | Yes |  | PDCCH repetition is not supported | Per FS |  | n/a | n/a | n/a | Component 2 candidate values: 2 or 3 Component 3 candidate values: {1,2,3, FFS more} Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot Note: for component 3, each unique pair of overlaps is counted as one. | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] | Yes |  | Monitoring of individual candidates is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition [with non-SFN TDM and/or FDM sheme] | 23-2-1 | Yes |  | Two QCL TypeD for CORESET monitoring in PDCCH repetition is not supported | Per band |  | n/a | FR2 only | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-2-4 | Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | Support of simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | 23-2-1, 16-2a | Yes |  | Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP is not supported | Per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set | FFS | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for codebook based | per FS |  | n/a | n/a | n/a | Component 4 candidate values: {1,2 [,4]} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1-2 | Multi-TRP PUSCH repetition (type A) - non-codebook based | 1. Support of multi-TRP PUSCH repetition for non-codebook based PUSCH (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set | 2-15 | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for non-codebook based | per FS |  | n/a | n/a | n/a | Component 3: {1,2,3,4} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1-2a | Two associated CSI-RS resources | Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH | 2-15a, 23-3-1-2 | Yes |  | Two associated CSI-RS resources are not supported | Per Band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1-2b | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 4. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 5. UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously | 23-3-1-2a | Yes |  | CSI-RS processing framework for SRS with two associated CSI-RS resources is not supported | Per Band |  | n/a | n/a | n/a | [Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16} Component 5: {1,2}] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping for Multi-TRP PUSCH repetition | Support of cyclic mapping when the number of repetitions is larger than 2 | 23-3-1 or 23-3-1-2 | Yes |  | Cyclic mapping for Multi-TRP PUSCH repetition is not supported | Per Band |  | n/a | n/a | n/a | Candidate component values: {for repetition Type A, for repetition Type B, both} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field for Multi-TRP PUSCH repetition | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | 23-3-1 or 23-3-1-2 | Yes |  | Second TPC field for Multi-TRP PUSCH repetition is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs.) | 23-3-1 or 23-3-1-2 | Yes |  | Two PHR reporting is not supported | Per Band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 | Yes |  | A-CSI report on two PUSCH repetitions is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 | Yes |  | SP-CSI report on two PUSCH repetitions is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) | 23-3-1-1 or 23-3-1-2 | Yes |  | CG PUSCH transmission towards M-TRPs using a single CG configuration is not supported | Per Band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1-1 -codebook based | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ 3. Supported number of SRS resources in one SRS resource set |  | Yes |  | Codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC |  | No | No | No | Component 3 candidate values: {1,2[,4]} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-1-3 | Multi-TRP PUSCH repetition (type B) – non-codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. support of two SRS resource sets with usage set to ‘nonCodebook’ 3. supported number of SRS resources in one SRS resource set |  | Yes |  | Non-codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC |  | No | No | No | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition scheme 1 (inter-slot) | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition)- sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource | FFS | Yes |  | PUCCH repetition scheme 1 (inter-slot repetition) is not supported | Per band |  | n/a | n/a | n/a | Note: power control parameter sets only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-2b | Cyclic mapping for multi-TRP PUCCH repetition | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | 23-3-2 | Yes |  | Cyclic mapping for multi-TRP PUCCH repetition is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-2c | Second TPC field for multi-TRP PUCCH repetition | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | 23-3-2 | Yes |  | Second TPC field for multi-TRP PUCCH repetition is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions | FFS | Yes |  | PUCCH repetition scheme 3 (intra-slot repetition) is not supported | Per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI | 16-2a | Yes |  | IntCell-mTRP is not supported | Per band |  | n/a | n/a | n/a | Component 2 candidate values: {[0,]1,2,3,[4,5,6,]7} Component 3 candidate values: {0,1,2,3,[4,5,6,]7} Note: UE indicates a non-zero value for at least one of component 2 or component 3 FFS: how to count X1 and X2 [Note: case1 and case2 cannot be enabled simultaneously] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs |  | Yes |  | Group based L1-RSRP reporting enhancements are not supported | Per band |  | n/a | n/a | n/a | Component 1 candidate values: {1,2,3,4} Component 2 candidate values: FFS Component 3 candidate values: FFS Note: component 2 and 3 are also counted in FG 16-1g and 16-1g-1 | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR based on two BFD-RS sets | 1. Maximum number of supported BFD-RS resources per set per BWP 3. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP |  | Yes |  | MTRP BFR based on two BFD-RS sets is not supported | Per band |  | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 3 candidate values: {2,3,4} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Max number of PUCCH-SR resources for MTRP BFRQ per cell group |  | Yes |  | PUCCH-SR resources for MTRP BFRQ is not supported | Per UE |  | No | Yes | No | Component candidate values: {1, 2} [Note: A UE that supports FG 23-5-2 must indicate this FG is supported with at least component candidate value 1] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-5-2b | Association between a BFD-RS resource set on SpCell and a PUCCH SR resource | Support of association between a BFD-RS resource set on SpCell and a PUCCH SR resource | 23-5-2a | Yes |  | Association between a BFD-RS resource set on SpCell and a PUCCH SR resource is not supported | Per UE |  | No | Yes | No | [Note: A UE that supports FG 23-5-2a with candidate value 2 must indicate this FG is supported with at least component candidate value 1] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. Support of SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH |  | Yes |  | SFN scheme A (scheme 1) for PDSCH and PDCCH is not supported | per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-1-1 | SFN scheme A (scheme 1) for PDCCH only | Support of SFN scheme A for PDCCH scheduling single TRP PDSCH |  | Yes |  | SFN scheme A (scheme 1) for PDCCH only is not supported | Per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 or 23-6-1b | Yes |  | Dynamic switching - scheme A is not supported | per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH scheduled by single TRPPDCCH |  | Yes |  | SFN scheme A (scheme 1) for PDSCH only is not supported | per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. Support of SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH |  | Yes |  | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH is not supported | per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | [23-6-2 or] 23-6-2b | Yes |  | Dynamic switching – scheme B is not supported | per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH scheduled by single TRP PDCCH |  | Yes |  | SFN scheme B (TRP based pre-compensation) for PDSCH only is not supported | per FS |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 | Yes |  | Simultaneous activation of two TCI states for PDCCH across multiple CCs is not supported | Per UE |  | No | Yes | No |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup for SFN | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold, if applicable 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | [23-6-1 or 23-6-2] | Yes |  | Default DL beam setup for SFN is not supported | Per band |  | n/a | n/a | n/a | Note: FR2 only for component 1 and 3 only | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | [23-6-1 or 23-6-2] | Yes |  | Default UL beam setup for SFN is not supported | Per band |  | n/a | FR2 only | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection of mode 1 with X=0 and/or mode 2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] |  | Yes |  | CSI Enhancement for Multi-TRP is not supported | Per band and per BC |  | n/a | n/a | n/a | Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: { mode 1 with X=0, mode 2, both} Component 4 candidate values: - {2, 4, 8, 12, 16, 24, 32} - {2,3,4 … 64} - {2,3,4, …, 256} [Component 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-7-1b | Active CSI-RS resources and ports in the presence of multi-TRP CSI | 1. List of codebook combinations - 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-7-1 | Yes |  | [Active CSI-RS resources and ports in the presence of multi-TRP CSI is not supported] | Per band and per BC |  | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {[‘NCJT’,] NCJT+Type 1 SP (for sTRP)} {Codebook 2, Codebook 3} = {(NULL, NULL}), {“Rel 16 combinations in FG 16-8”}, {“New Rel17 combinations in FG 23-9-5”}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2, 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. Note2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 | Maximum value of numberOfSingleTRP-CSI-Mode1 | 23-7-1 | Yes |  | CSI report mode 1 with X=1, or X=2 is not supported | Per Band |  | n/a | n/a | n/a | Component 1 candidate value set: { X=1, X=2} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | Yes |  | Nmax=2 for Multi-TRP CSI is not supported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for Rel-17 Multi-TRP CSI enhancement and a single CMR configured for Single-TRP measurement in a CSI reporting setting | 23-7-1 | Yes |  | CMR sharing is not supported | Per band |  | n/a | FR2 only | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 | Yes |  | SRS triggering offset enhancement is not supported | Per band |  | n/a | n/a | n/a | Candidate 1 component values: {1, 2, 4} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | Yes |  | Triggering SRS only in DCI 0_1/0_2 is not supported | per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | Yes |  | SRS Antenna switching for >4Rx is not supported | Per FS |  | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: For any indicated value, x shall be equal to or smaller than the one associated with the largest y Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y. | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 | Yes |  | Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported | Per FS |  | n/a | n/a | n/a | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 | Yes |  | Increased repetition for SRS is not suported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 | Yes |  | Partial frequency sounding of SRS is not suported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 | Yes |  | Start RB location hopping for partial frequency SRS is not suported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | Yes |  | comb-8 for SRS other than for positioning is not suported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53, 2-55 | Yes |  | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R is not supported | Per FS |  | N/A | N/A | N/A | [Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports] | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=1 and R=1 - Support rank 1,2 - Support parameter combinations with M=1 | 2-35 | Yes |  | Further Enhanced Port-Selection Type II Codebook (FeType-II) is not suported | Per band and per BC |  | n/a | n/a | n/a | Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} Max # total ports: {4 to 256} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4 | Yes |  | Active CSI-RS resources and ports for mixed codebook types in any slot is not suported | Per band and per BC |  | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4 Note 3: Up to 4 combinations for component 1 | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 - 2. Support parameter combinations with M=2 | 23-9-1 | Yes |  | M=2 and R=1 for FeType-II is not suported | per band and per BC |  | n/a | n/a | n/a | Component 1 candidate values - Maximum 8 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | 23-9-1 | Yes |  | Rank 3, 4 for FeType-II is not suported | Per band |  | n/a | n/a | n/a |  | Optional with capability signalling |  |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=2 | 23-9-2 | Yes |  | R = 2 for FeType-II is not suported | per band and per BC |  | n/a | n/a | n/a | Component 2 candidate values: • Maximum 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256} | Optional with capability signalling |  |