3GPP TSG RAN WG1 #109-e		                                    		   R1-2204123

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.11.1

Source:	InterDigital, Inc.

Title:	Discussion on XR specific power saving enhancements

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, the contents of the SID for Rel. 18 NR XR Enhancements were agreed [1]. The objectives in the SID for XR-specific power savings are as follows.

Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

In this contribution, we discuss the potential enhancements and evaluations for power savings for XR applications/services.

# Discussion

During Rel-17 SI [2], the traffic models for different XR applications (AR, VR and cloud gaming) were defined and the power savings evaluations when supporting XR traffic in DL and UL were performed.

From the discussions on traffic models, the XR traffic in DL is characterized by high and variable PDU sizes (e.g. corresponding to I-frames, P/B frames), high number of PDUs per frame, quasi-periodic periodicity, and multiple flows/streams per application. In the case of UL, the XR traffic is characterized by high periodicity (e.g. for pose/control data), high data rate (e.g. for AR video), and multiple streams per application (e.g. aggregated video, audio). The XR traffic in both DL and UL require satisfying stringent QoS in terms of high data rate (e.g. 45Mbps), low latency (e.g. 10ms PDB) and high reliability (e.g. 99% success rate) for ensuring user satisfaction.

Given the small device form factor and limited battery capacity, achieving power savings in UE is important when supporting any of the XR applications. The typical XR traffic characteristics defined by non-integer periodicity (e.g. due to video frame rate), quasi-periodic periodicity (e.g. due to jitter) and variable data rates (e.g. due to variable frame sizes) makes it extremely challenging for achieving power savings with legacy power saving techniques (e.g. CDRX).

Observation 1: 	XR traffic characteristics, which are defined by non-integer periodicity, quasi-periodic periodicity and variable data rates, makes it extremely challenging for achieving power savings with legacy power saving techniques (e.g. CDRX)

The Rel-17 evaluations also indicate a tradeoff relationship between power savings gain and capacity achievable. This is mainly due to mismatch between the CDRX parameters configured in UE (e.g. cycle duration, ON/active duration) and the supported XR traffic characteristics with stringent QoS (e.g. low PDB, high data rate).

The scope of Rel-18 SI discussed during RAN#94-e includes the objectives for improving power savings based on CDRX enhancements and PDCCH monitoring enhancements. In this regard, whether and how the XR-specific traffic characteristics can be accommodated in any of the power saving enhancements is to be investigated. It is unclear what is the impact on the power saving gains achievable when accounting for such traffic characteristics in the power saving enhancements. The key XR traffic characteristics that should be further studied when considering the power saving enhancements are:

- Consideration of PDU sets/frames

- Consideration of multiple streams

Impact of handling of PDU sets/frames

A PDU set can consist of several PDUs that can be associated with a video/media frame. The number of PDUs per video frame or size of PDU set can vary depending on the type of video frame (e.g. I-frame, P/B-frame). The PDUs in a PDU set may be generated by the encoder at the application with a periodicity value proportional with the frame rate (e.g. 60fps, 90fps). Such frame rates can result in non-integer periodicity (e.g. 1/60) for the burst of PDUs in a PDU set which is transmitted in DL and UL.

During transmission in DL, it is possible that at least some of the PDUs in a PDU set to arrive at the gNB with certain offset values with respect to the periodicity due to delays in the network. It is also possible that the offset values, represented by the range for jitter, may vary depending on the number of PDUs in a PDU set. For example, a PDU set associated with an I-frame may result in wider jitter range due to presence of typically higher number of PDUs per PDU set compared to the PDU set associated with P/B-frames. Given the interdependencies between the PDUs, all PDUs in a PDU set need to be successfully delivered in DL to the UE within a PDU set-level latency requirement, irrespective of size of PDU set or jitter range.

When using a power saving scheme such as CDRX, the length of the ON/active time duration should at least span the duration for receiving all PDUs in the PDU set. In the case when the number of PDUs per PDU set vary and/or when the jitter range varies, the ON duration of the power saving scheme should also be varied accordingly to realize power savings. Using a semi-statically configured CDRX with fixed cycle duration and ON duration may allow certain power savings when the CDRX parameters are configured to accommodate a particular size of PDU set (e.g. average size) or jitter range. However, this comes at the expense of capacity loss due to increased latency for delivering some PDU sets.

Observation 2: 	Using a semi-statically configured power saving scheme (e.g. CDRX) to accommodate certain size of PDU set or jitter range may allow some power savings but comes at the expense of capacity

In this regard, how to adapt the CDRX configuration based on the variable size of PDU set and/or jitter range should be studied. Similarly, in the case of PDCCH monitoring, it is beneficial to study how the DCI-based PDCCH monitoring behaviour can be adapted for power savings based on awareness of changes in the number of PDUs in PDU set and change in jitter range. For any of the power savings enhancements, the impact of handling PDU sets on the power saving gains achievable should also be studied and evaluated.

Proposal 1: 	Capture in TR, XR-specific enhancements for power savings that support adaptations based on PDU set characteristics in UL and DL

Impact of handling of multiple streams

During Rel-17 evaluations [2], the power saving performance achievable with CDRX for XR applications with multi-streams (e.g. 2 stream AR) indicate only marginal gain compared to the always-on baseline case. In comparison with single stream case, the multi-stream case with CDRX indicate a reduction of almost 50% in the power saving gain. Such degradation is due to handling of high traffic load (e.g. for aggregated video) and high periodicity (e.g. for pose/control data), which require the use of long ON/active time duration. Similar issue may occur when handling multiple streams in DL, where there may be minimal opportunity for the UE to go into any sleep mode.

Regarding handling of multiple streams in DL, another key issue to be addressed for power savings is jitter. As an example, consider the case where the PDUs in different streams (e.g. I-frame stream, P/B frame stream) may be transported in DL from the application server to CN and RAN independently. Due to congestion and processing delays in network, the independent transport can result in inter-stream jitter where there may be a wide difference in the time at which the PDUs of different streams arrive at the gNB. From the application perspective, the PDUs of different streams need to be delivered to the UE in DL within a maximum inter-stream jitter value to be able to jointly perform processing/rendering at application. Otherwise, any of the PDUs which are not delivered to UE in time due to late arrival at gNB or misaligned ON time duration at UE can result in QoS degradation and capacity losses.

Observation 3: 	In multi-stream scenario, configuring a power saving scheme in UE that is not aligned with a maximum inter-stream jitter value required by the XR application can result in capacity losses

For balancing the tradeoff between power savings and capacity for multi-stream case, it is important to investigate how to any enhancements to power saving schemes can accommodate the inter-stream jitter (e.g. difference in the arrival times of PDUs in different streams). For example, how parameters of CDRX configuration can be flexibly adapted (e.g. adjusting start time offset of ON duration) or how to trigger the use of CDRX configurations with different cycle/ON durations based on inter-stream jitter should be studied. Similarly, whether and how adaptations can be effectively performed for DCI-based PDCCH monitoring (e.g. skipping or switching between different search space groups) when receiving PDUs in multiple streams should be studied. It is also important to investigate how the power savings enhancements for handling multiple streams can impact the power saving gains achievable.

Proposal 2: 	Capture in TR, XR-specific enhancements for power savings that support transmission of multiple associated/correlated traffic streams in UL and DL

Performance evaluations

For any of the enhancement techniques (e.g. adaptive CDRX and DCI-based PDCCH monitoring adaptations), it would be beneficial to investigate how the power saving and capacity performance compares with respect to baseline schemes (e.g. always-on, Rel-15/16 CDRX). For studying the impact of handling PDU sets, the power saving gain (PSG) achievable with an enhancement scheme along with any losses in capacity should be compared with the performance achieved when using a baseline power saving scheme that is agnostic to PDU set handling. For determining the PSG and capacity achievable with an enhancement technique that can handle multiple streams, the performance should be compared with a baseline scheme that is agnostic to association of multiple streams and inter-stream jitter.

Proposal 3: 	RAN1 to perform evaluations of XR-specific enhancement techniques (e.g. adaptive CDRX and DCI-based PDCCH monitoring adaptations) for evaluating the power saving performance

Given that the key system parameters were discussed during Rel-17 SI, we propose to reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) used in TR 38.838 as the starting point for the SLS performance evaluations for power savings. The SLS parameters and baseline traffic model parameters are provided in Section 5 (Annex).

Proposal 4: 	Reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) in TR 38.838 when evaluating performance of power saving gain

During Rel-17 SI evaluations, some of the considered enhancement schemes, such as eCDRX and use of multiple CDRX configurations, may accommodate adaptations for handling PDU sets, jitter and multiple-streams. For example, the UE can be configured with multiple CDRX configurations which may be associated with different traffic patterns or different types of PDU sets/frames. The UE may use a particular CDRX configuration based on PDU set attributes (e.g. size of PDU set, jitter range) expected in DL. For eCDRX, dynamically varying the start time offset can also minimize the mismatch between the CDRX cycle and arrival interval of XR traffic. Another enhancement scheme that may support handling of PDU sets and multiple streams include enhanced DCI-based PDCCH monitoring adaptations (e.g. use of multiple monitoring slot periodicity values). Such enhancement schemes can be considered as a starting point for Rel-18 enhancements and evaluations for power savings.

Proposal 5: 	Starting point for potential enhancement techniques for power savings can be those identified during Rel-17 SI (e.g. eCDRX, supporting multiple CDRX configurations, enhanced DCI-based PDCCH monitoring adaptations)

# Simulation Results

In this section we provide system level simulation results on power consumption performance for DL single traffic stream model. The power saving gain (PSG) is determined when using different CDRX configurations for Indoor Hotspot (InH) and Dense Urban (DU) deployments. For comparing whether there is tradeoff between power savings and capacity, the capacity is evaluated assuming the baseline scenario without power savings and with power savings when using CDRX. The parameters and CDRX configurations used in the simulations are shown in Table 4 and Table 5 (in Annex).

#### Indoor Hotspot

Power savings and system capacity performance for indoor hotspot is evaluated for FR1 (4 GHz) based on the parameter settings given in Tables 1, 2, 4 and 5 in Annex.

AR/VR @ 30 Mbps - InH

Figure 1: FR1 DL AR @ 30 Mbps results for Indoor Hotspot scenario

Figure 1 shows that the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is between 5 - 6 UEs/cell without any power saving scheme and up to 5 UE/cell with the implementation of the CDRX schemes. For AR/VR (30Mbps with 10ms PDB), the CDRX4 scheme (5,4,1) achieves highest number of satisfied UEs supporting up to 5 UEs per cell. Table I shows the performance of power saving gain vs the CDRX configurations employed for AR/VR at 30 Mbps in InH scenario. In terms of PSG, CDRX5 (4,2,2) shows the best performance but at the cost of low number of satisfied UEs.

Table I: Power savings gain for baseline vs. CDRX configs (Indoor – AR/VR – 30 Mbps)

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #Satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| No DRX | - | - | - | - | 4.84/5 |
| CDRX 1 | 6.136 | 2.601 | 6.104 | 10.090 | 2.90/5 |
| CDRX 2 | 10.4 | 0 | 10.292 | 21.273 | 4.00/5 |
| CDRX 3 | 6.551 | 3.307 | 6.773 | 8.916 | 4.14/5 |
| CDRX 4 | 10.559 | 0 | 10.631 | 20.816 | 4.56/5 |
| CDRX 5 | 15.50 | 10.213 | 15.574 | 21.20 | 2.88/5 |

AR/VR @ 45 Mbps – InH

Figure 2: FR1 DL AR @ 45 Mbps results for Indoor Hotspot scenario

In the case of AR/VR with 45Mbps shown in Figure 2, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is 3 UEs/cell without any power saving scheme. When using the CDRX4 scheme (5,4,1) the number of satisfied UEs are maintained at 3 UEs/cell, thereby overcoming the tradeoff between capacity and power savings. Table II provides the power saving gain performance vs the CDRX configurations employed for AR/VR at 45 Mbps in InH scenario.

CDRX2 (8,6,2) shows similar PSG performance with that of CDRX4 at slightly lower number of satisfied UEs when supporting AR traffic with 45Mbps. In comparison to the case when supporting AR traffic with 30Mbps (in Figure 1), the PSG for CDRX2 (8,6,2) is comparable with CDRX4 but comes at much lower number of satisfied UEs. In this regard, the traffic load supported (e.g. 30Mbps vs. 45 Mbps) can be impactful in determining the CDRX schemes or CDRX adaptations that can be supported to balances the tradeoff between power savings and capacity.

Table II: Power savings gain for baseline vs. CDRX configs (Indoor – AR/VR – 45 Mbps)

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #Satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| No DRX | - | - | - | - | 2.94/3 |
| CDRX 1 | 5.755 | 1.272 | 5.752 | 10.138 | 1.90/3 |
| CDRX 2 | 11.488 | 7.629 | 11.643 | 15.85 | 2.56/3 |
| CDRX 3 | 6.539 | 3.323 | 6.565 | 8.835 | 2.56/3 |
| CDRX 4 | 12.061 | 9.274 | 12.196 | 14.96 | 2.86/3 |
| CDRX 5 | 15.117 | 9.745 | 14.839 | 20.562 | 1.90/3 |

#### Dense Urban

Similar evaluation results for power savings are provided in the following for Dense Urban scenario for AR/VR (at 30Mbps).

AR/VR @ 30 Mbps – DU

Figure 3: FR1 DL AR @ 30 Mbps results for Dense Urban scenario

From Figure 3, it is observed that the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is at least 3 UEs/cell without any power saving scheme. When using CDRX2 (8,6,2) and CDRX4 (5,4,1) schemes, it is possible to maintain the number of satisfied UEs to 3 UEs/cell. Table III provides the power saving gain performance vs the CDRX configurations used for AR/VR at 30 Mbps in DU scenario.

Table III: Power savings gain for baseline vs. CDRX configs (Outdoor – AR/VR – 30 Mbps)

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #Satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| No DRX | - | - | - | - | 3.00/3 |
| CDRX 1 | 6.925 | 2.504 | 7.261 | 10.269 | 2.25/3 |
| CDRX 2 | 13.219 | 1.088 | 13.724 | 26.197 | 2.81/3 |
| CDRX 3 | 7.221 | 3.878 | 7.706 | 9.061 | 2.65/3 |
| CDRX 4 | 13.099 | 2.959 | 13.083 | 25.306 | 2.95/3 |
| CDRX 5 | 16.758 | 10.017 | 17.454 | 21.699 | 2.28/3 |

Observation 4: 	There is a tradeoff relation between PSG and capacity (i.e. ratio of satisfied UEs per cell). However, the tradeoff can be balanced to certain extent when using CDRX configurations with short cycle durations (e.g. CDRX 4 with (5,4,1))

Observation 5: 	Compared to all other PSG schemes evaluated, CDRX4 scheme with parameters (5,4,1) provides the best tradeoff in terms of PSG vs. satisfied UEs per cell in all simulated scenarios while CDRX1 scheme with parameters (16,12,4) provides the worst tradeoff. This is expected as the relatively short cycle duration of CDRX4 is only 31% of that of CDRX1, which is better aligned with XR traffic.

Observation 6: 	The DL traffic characteristics for XR (e.g. 30Mbps vs. 45 Mbps, with 10ms PDB) can significantly impact in determining the CDRX schemes or CDRX adaptations that can be supported for balancing the tradeoff between power saving gain and capacity

# Conclusion

In this contribution, the following observation are made:

Observation 1: 	XR traffic characteristics, which are defined by non-integer periodicity, quasi-periodic periodicity and variable data rates, makes it extremely challenging for achieving power savings with legacy power saving techniques (e.g. CDRX)

Observation 2: 	Using a semi-statically configured power saving scheme (e.g. CDRX) to accommodate certain size of PDU set or jitter range may allow some power savings but comes at the expense of capacity

Observation 3: 	In multi-stream scenario, configuring a power saving scheme in UE that is not aligned with a maximum inter-stream jitter value required by the XR application can result in capacity losses

Observation 4: 	There is a tradeoff relation between PSG and capacity (i.e. ratio of satisfied UEs per cell). However, the tradeoff can be balanced to certain extent when using CDRX configurations with short cycle durations (e.g. CDRX 4 with (5,4,1))

Observation 5: 	Compared to all other PSG schemes evaluated, CDRX4 scheme with parameters (5,4,1) provides the best tradeoff in terms of PSG vs. satisfied UEs per cell in all simulated scenarios while CDRX1 scheme with parameters (16,12,4) provides the worst tradeoff. This is expected as the relatively short cycle duration of CDRX4 is only 31% of that of CDRX1, which is better aligned with XR traffic.

Observation 6: 	The DL traffic characteristics for XR (30Mbps vs. 45 Mbps, with 10ms PDB) can significantly impact in determining the CDRX schemes or CDRX adaptations that can be supported for balancing the tradeoff between power saving gain and capacity

Based on these observations, the following proposals are made:

Proposal 1: 	Capture in TR, XR-specific enhancements for power savings that support adaptations based on PDU set characteristics in UL and DL

Proposal 2: 	Capture in TR, XR-specific enhancements for power savings that support transmission of multiple associated/correlated traffic streams in UL and DL

Proposal 3: 	RAN1 to perform evaluations of XR-specific enhancement techniques (e.g. adaptive CDRX and DCI-based PDCCH monitoring adaptations) for evaluating the power saving performance

Proposal 4: 	Reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) in TR 38.838 when evaluating performance of power saving gain

Proposal 5: 	Starting point for potential enhancement techniques for power savings can be those identified during Rel-17 SI (e.g. eCDRX, supporting multiple CDRX configurations, enhanced DCI-based PDCCH monitoring adaptations)

# Annex

The following show the parameters from TR 38.838 [2] used in system level simulations for XR.

Table 1: Assumptions for System-level simulations

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:64 Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,8,2,1,1;4,8), (dH, dV) = (0.5λ, 0.5λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | Reciprocity-based precoding |  |  |
| Scheduling Algorithm | DL: SU-MIMO with PF, FIFO, and resource sharing based scheduling UL: SU-MIMO with PF, FIFO, and resource sharing based scheduling |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 2: DL Traffic models for CG/AR/VR evaluations

|  | CG | VR | AR |
| --- | --- | --- | --- |
| Data Rate | 30Mbps (baseline) @60fps | 30Mbps (baseline), 45Mbps @60fps | 30Mbps (baseline), 45Mbps @60fps |
| FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
| Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
| Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
| Air Interface PDB | 15ms (baseline) | 10ms (baseline) | 10ms (baseline) |
| Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [[-4, 4]ms] |  |  |

Table 3: UL Traffic models for CG/VR/AR evaluations

|  | Stream 1 (pose/control data) (for CG/VR/AR) | Stream 2 (aggregated video) (for AR) |
| --- | --- | --- |
| Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
| Data rate | 0.2 Mbps | 10 Mbps |
| Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
| Air Interface PDB | 10ms | 30ms |
| Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,30ms] (baseline) |
| Jitter | Periodic: 4ms (no jitter) | Jitter: same model as for DL |

Table 4: Power model for DL

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table 5: Selected CDRX parameters for power consumption evaluations

|  | DRX cycle (ms) | On duration timer value (ms) | Inactivity timer value (ms) |
| --- | --- | --- | --- |
| CDRX1 | 16 | 12 | 4 |
| CDRX2 | 8 | 6 | 2 |
| CDRX3 | 10 | 8 | 2 |
| CDRX4 | 5 | 4 | 1 |
| CDRX5 | 4 | 2 | 2 |

# Reference

[1] RP-220285, New SID on XR Enhancements for NR, RAN#95e, Mar. 2022.

[2] TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17), Dec. 2021.