3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204740

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.2

Source:	Ericsson

Title:	Co-channel coexistence between LTE sidelink and NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

In RAN#94e, a new WID on NR sidelink evolution was agreed [1]. In this WID it has been agreed to study the co-channel coexistence between LTE sidelink and NR sidelink as stated in Justification and Objectives sections of WID as below.

## 3 Justification […] Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop. ## 4 Objective - Objective of SI or Core part WI or Testing part WI […] - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible […] Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.

In this paper, we present our views on this topic.

# 2	Background

## 2.1	ITS band

3GPP introduced LTE SL for V2X in Rel-14 and enhanced it in Rel-15. In Rel-16, 3GPP defined specifications for NR SL for V2X, with some enhancements being added in Rel-17.

When it comes to spectrum, it is generally assumed that the SL V2X services will be deployed on specific bands that are designated for ITS (Intelligent Transportation Systems). This ITS band, located around 5.9 GHz, is available in several regions around the world.

When it comes to spectrum, it is generally assumed that the SL V2X services will be deployed in the 5.9 GHz band. This band includes basic services (e.g., day-1 safety, etc.) as well as advanced services (e.g., cooperative driving, etc.). It is expected that UEs with both SL interfaces, i.e., LTE and NR SL, will coexist in this band.

## 2.2	3GPP sidelink evolution model

In terms of compatibility, the two LTE SL releases share a common set of features, but some aspects are not backwards compatible. Similarly, the two NR SL releases (and presumably the ones to be specified in the future) share a common set of features, but some aspects are not backwards compatible.

To enable coexistence between UEs without preventing technology evolution, 3GPP devised a SL evolution model in which:

- Different releases of a given RAT can be deployed in the same resources

- Each service is mapped to a specific RAT and release. For example, day-1 safety may use Rel-14 LTE SL and cooperative driving Rel-16 NR SL. Any vehicle meant to participate in a service must incorporate a UE implementing the required RAT and release.

In addition, 3GPP specified a basic framework for coexistence between LTE SL and NR SL in Rel-16. Note that most of the issues discussed and specified in Rel-16 referred to in-device coexistence. Only a few coexistence aspects at system level were incidentally considered (see Section 2.3).

Our view is that the different RATs (and their associated services) will go through: introduction, maturity, and decline phases. Resources will be typically dimensioned for the maturity phase and thus, be overprovisioned during the decline phase.

## 2.3	Rel-16 coexistence framework

LTE and NR sidelinks are configured independently. Although it is possible to configure their corresponding resource pools so that they share time-frequency resources, this is not desirable. The reason is that the coexistence mechanisms specified in Rel-16 rely in having the LTE and NR resource pools configured to use separate time-frequency resources. For example, NR and LTE sidelinks may be multiplexed in frequency (e.g., to use different channels; or to use different frequencies in the same channel), as shown in Figure 1. It is also possible that NR and LTE sidelinks are multiplexed in time (e.g., to use the same channels at different times, in different slots, subframes, etc.), as shown in Figure 2.

Figure 1. Coexistence of NR V2X and LTE V2X by FDM

Figure 2. Coexistence of NR V2X and LTE V2X by TDM

This configuration of LTE and NR resource pool on separate resources is typically static or semi-static. That is, it cannot be changed, or it can only be changed infrequently. Having a static split of the resources is usually not efficient as the needs for resources for one or the other technologies vary over time and space.

Static split of resources for LTE and NR is not efficient in resource utilization.

# 3	Solutions for co-channel coexistence between LTE and NR sidelink

In this section we discuss solutions for co-channel coexistence between LTE and NR sidelinks. First, we discuss a few baseline assumptions to ensure that the solution is useful in practice and to limit the scope of the discussions. Then, we discuss detect-and-vacate as a way of enabling coexistence. Finally, we present a solution for dynamic co-channel coexistence of LTE and NR SLs.

## 3.1	Baseline assumptions

At the start of Rel-18, we find the following types of legacy UEs:

- LTE SL UEs (Rel-14, Rel-15).

- NR SL UEs (Rel-16, Rel-17).

In our view, the following limitations are applicable regarding the solutions for co-channel coexistence studied in Rel-18.

- For LTE SL UEs:

  - Changes to the specification (signalling, procedures, etc.) are not possible.

  - Changes to the configuration (e.g., pool configuration) are possible but should be minimized.

- For NR SL UEs:

  - New procedures can be introduced, but these are only applicable to Rel-18 UEs.

  - The co-channel coexistence framework specified in Rel-18 should not create any backwards compatibility issue between Rel-16/Rel-17 NR SL UEs and Rel-18 NR SL UEs.

- The impact of having co-channel deployments of different RATs (e.g., interference from NR SL to LTE SL and vice versa) should be as limited as possible.

It is also important to clarify which type of devices and UEs the mechanisms should work for. In our opinion, the solutions agreed by 3GPP must be applicable to devices that implement NR SL alone. That is, for basic operation, it cannot be assumed that the device implements both NR SL and LTE SL. This should not preclude, however, optimizations for those devices that indeed implement both NR SL and LTE SL (e.g., optimizations using intra-device signalling, etc.).

RAN1 discusses solutions for co-channel coexistences that meet the following principles:

- For LTE SL UEs changes to the specification are not allowed. Changes to the configuration (e.g., pool configuration) are possible but should be minimized.

- For NR SL UEs, new procedures may be introduced for Rel-16 UEs, but without any additional constraints on inter-operability with legacy NR SL UEs (Rel-16/Rel-17).

- The impact of having co-channel deployments to the different RATs should be as limited as possible.

- The baseline solution is applicable for standalone NR SL UEs. Optimizations for devices implementing both NR SL and LTE SL may be considered too.

## 3.2	Detect-and-vacate solutions

It is clear from the principles above that Rel-18 NR SL UEs will carry the bulk of the co-channel coexistence mechanism. In essence this amounts to dynamically changing how the resources are used, depending on the presence of LTE SL UEs.

A common way to do this is to use “detect-and-vacate”. In “detect-and-vacate”, a first technology (here LTE SL) is given high priority (for using the spectrum) and a second technology (here NR SL) is given lower priority (for using the shared spectrum). The users of the second technology are required to attempt detecting the presence of users of the first technology.

- If users of the first technology are detected (e.g., by detecting their transmissions), then users of the second technology cannot use the spectrum or can only do it under restricted terms (e.g., using low power).

- If users of the first technology are not detected (e.g., by detecting their transmissions), then users of the second technology can use the spectrum without any additional restriction.

Typically, “detect-and-vacate” is operated at channel level. This approach is too coarse, not allowing to treat differently cases where there are many LTE SL UEs or just a few LTE SL UEs. For the case of SL technologies, it could be operated at resource pool level as well. Unfortunately, these approaches require a static partitioning of the resources, which is quite inefficient. For example, it is typically hard to know how many resources to allocate for the NR or LTE SL UEs/transmissions since their number and requirements might change over time causing that the resource pool ends up having a configuration which under or overprovisions the needed resources.

“Detect-and-vacate” solution in channel or resource pool level is not efficient in terms of resource utilization.

## 3.3	Dynamic co-channel coexistence between LTE and NR sidelink

Our solution for dynamic co-channel coexistence between LTE and NR sidelinks is based on:

- Defining different classes of resources within an NR SL resource pool.

- Dynamically using resources in the different classes in an NR SL resource pool, based on the presence/absence of LTE SL transmissions.

That is, to improve resource utilization we define multiple classes of resources within an NR SL resource pool.

The use of the resources in each class depends on the detected presence of LTE SL. As an example, illustrated in Figure 3, a resource pool is divided in time-domain into two classes of A and B. Then, Class A resources can always be used by NR UEs, while Class B resources are only used by a NR UE if LTE SL UEs are present (i.e., if they are not detected). Typically, the LTE SL would be configured so that the radio resources corresponding to Class B resources in the NR SL resource pool are the ones configured in the LTE SL resource pool, as illustrated in Figure 3 as well. In this way, a dynamic co-channel coexistence between NR and LTE sidelinks is enabled.

Note that from a SL signaling point of view, there are no changes w.r.t. to Rel-16/17 mechanism. It is possible to use a class A resource to reserve a class B resource and vice versa. This contrasts with having two separate pools of resources and performing detect-and-vacate on one of them. A legacy (Rel-16/17) NR SL UE would be configured with a resource pool that does not distinguish between class A and class B but would still be able to decode any transmissions.

Figure 3. Top: LTE resource pool. Bottom: NR resource pool with Class A and Class B resource mapping

NR SL supports:

- Configuring two different classes of resources within a resource pool

- Selecting resources from both classes if LTE SL transmissions are not detected

- Restricting selection of resources to a fixed class if LTE SL transmissions are detected

### 3.3.1	Detection of LTE SL transmissions

Detection of LTE SL transmissions is essential for dynamic LTE SL transmissions. We envision three ways to perform this:

- Direct detection. That is, the device is capable of detecting LTE SL signals. As said earlier, this is applicable to devices implementing both NR SL and LTE SL and should not be considered as a baseline.

- Indirect detection. That is, the device detects LTE SL transmissions without decoding the corresponding signals. For example, energy measurements such as RSSI may be used.

- Explicit signalling. In this case, the device is informed by others about the presence of LTE SL transmissions. Such signalling could come from another UE, the NW (e.g., a gNB), or OTT.

Detection of LTE SL transmissions can be direct, indirect or by means of explicit signalling.

We believe that RAN1 should study the different alternatives

RAN1 to study:

- Indirect detection and explicit signalling for detecting LTE SL transmissions without implementing LTE SL features.

- The use of direct detection, as an optimization, for devices implementing NR SL and LTE SL.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Static split of resources for LTE and NR is not efficient in resource utilization.

Observation 2	“Detect-and-vacate” solution in channel or resource pool level is not efficient in terms of resource utilization.

Observation 3	Detection of LTE SL transmissions can be direct, indirect or by means of explicit signalling.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 discusses solutions for co-channel coexistences that meet the following principles:

	For LTE SL UEs changes to the specification are not allowed. Changes to the configuration (e.g., pool configuration) are possible but should be minimized.

	For NR SL UEs, new procedures may be introduced for Rel-16 UEs, but without any additional constraints on inter-operability with legacy NR SL UEs (Rel-16/Rel-17).

	The impact of having co-channel deployments to the different RATs should be as limited as possible.

	The baseline solution is applicable for standalone NR SL UEs. Optimizations for devices implementing both NR SL and LTE SL may be considered too.

Proposal 2	NR SL supports:

	Configuring two different classes of resources within a resource pool

	Selecting resources from both classes if LTE SL transmissions are not detected

	Restricting selection of resources to a fixed class if LTE SL transmissions are detected

Proposal 3	RAN1 to study:

	Indirect detection and explicit signalling for detecting LTE SL transmissions without implementing LTE SL features.

	The use of direct detection, as an optimization, for devices implementing NR SL and LTE SL.

# References

RP-213678, New WID on NR sidelink evolution, OPPO and LG Electronics, RAN#94e, Dec.6-17, 2021.