3GPP TSG RAN WG1 #109-e                                                                              R1-2204934

e-Meeting, March 09– March 15, 2022

Source:	Mavenir

Title:	Timing Relationship Enhancements

Agenda Item:	8.14

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the NPDCCH monitoring modifications for IOT-NTN, and propose TP for Rel-17 specification.

# Discussion

In previous RAN1 meetings, the following agreement was reached on timing relationship enhancements of IOT NTN:

Agreement [1]

Modification of the designation of subframes with NPDCCH monitoring restrictions is needed for at least Cases 1 to 6.

In IoT NTN networks, because of large propagation delays, the duration for which the UE does not need to monitor any DL signal in response to UL transmission needs to be updated compared to traditional terrestrial networks. In legacy terrestrial networks, the propagation delay between UE and enB/gNB is less than 1 msec as the distance between UE and eNB would be around 40 KM for the NBIOT case. For non-terrestrial networks, the propagation delay between UE and eNB would be much larger than 1 msec, as the distance between UE and a GEO satellite and GEO satellite to eNB/gNB would be around 35786 Km, hence we need to update the DL monitoring restriction for the UE to improve UE performance.

Because of long propagation delays in non-terrestrial networks, the monitoring window for DL reception also needs to be updated. In subsequent sections, we provide the necessary changes for the NPDCCH monitoring restrictions which has not been discussed.

Note: UE monitors NPDCCH in DL subframe indices and eNB also schedules NDPCCH in DL subframe indices, so it is easier and less error prone to describe the timing in DL subframes only in the agreement, rather than mixing DL and UL subframes. This way, we can ensure the same understanding between UE and eNB for the scheduling timing.

- We propose following specification changes to 3GPP TS36.213 ver 17.1.0 section 16.6, to capture the impact of UE-eNB RTT properly for IoT-NTN scenarios. We suggest that following changes can be captured in TS 36.213 as an example of TP#1.

- It is incumbent to include the remaining cases of NPDCCH monitoring modification for IoT-NTN, as we had agreement for case1 to case6 in RAN1#107-e.

------------------------------------------------TP#1 TS 36.213 (in red)-----------------------------------------------

# 16 UE Procedures related to narrowband IoT

## 16.6  Narrowband physical downlink control channel related procedures

## **********************redundant portions are omitted***********************

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1- nTAUE to subframe n+3+Kmac;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1- nTAUE to subframe n+3+Kmac.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1- nTAUE to subframe n+3+Kmac.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5

otherwise

-	the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap.

For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n,

-	in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n- nTAUE to subframe n+40+Kmac,

-	otherwise, the UE is not required to monitor NPDCCH UE-specific search space from subframe n- nTAUE to subframe n+3 +Kmac.

## **********************redundant portions are omitted***********************

------------------------------------------------ end of TP#1-------------------------------------------------------------------

Proposal 1: Adopt TP#1 as modification in section 16.6 of TS36.213 for IoT-NTN .

# Conclusion

In this contribution, we discuss the remaining issues for the timing relationship enhancements for IOT-NTN as identified in RAN1 #106-bis-e meeting (case7 – case11).

Proposal 1: Adopt TP#1 as modification in section 16.6 of TS36.213 for IoT-NTN .

# Reference

Chairmain’s Notes, 3GPP RAN1 #107-e, 2021

Chairmain’s Notes, 3GPP RAN1 #106b-e, 2021

TS 36.213, version 17.1.0 Release 17 - section 16: UE Procedures related to narrow band IoT

R1-2202480, 3GPP TSG RAN WG1 #108-e, Mavenir.