3GPP TSG RAN WG1 #109-e	Tdoc R1-2204960

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.7.2

Source:	Ericsson

Title:	Maintenance for Rel-17 UE PS : PDCCH monitoring adaptation

Document for:	Discussion and Decision

# 1		Introduction

In this paper, several remaining maintenance aspects for Rel-17 PDCCH monitoring adaptation are addressed.

- application delay of PDCCH monitoring adaptation

- PDCCH skipping in relation to the overlap with outside active time

- PDCCH skipping in relation with SR and RACH

# 2	Discussion

## 2.1	Application delay

### 2.1.1	PDCCH-skipping

Following was agreed in RAN1#108-e meeting for application delay of PDCCH skipping commands.

Agreement

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes

  - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

    - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

PDCCH monitoring during retransmission

The UE skips PDCCH monitoring at a first slot after the last symbol of the PDCCH containing the skipping indication. This gives benefit in terms of the possible power-saving as UE could enter the skipping duration (and thus, sleep) faster.

However, if the last transport block (TB) is not successfully decoded by the UE, NW needs to wait until skipping duration ends before it could retransmit the corresponding TB. Thus, either it increases packet delays, or it can mean reduced opportunity for power savings as NW has to conservatively set the skipping duration. These are both undesirable.

It is preferred to have a mechanism where a NW has more flexibility in indicating long skipping durations without introducing unnecessary delays for already scheduled packets i.e. for retransmissions. DRX already includes mechanisms to ensure retransmissions are not affected by PDCCH monitoring reduction, and PDCCH skipping should ensure such mechanisms are not adversely overridden.

For DL, the DRX retransmission timer is triggered based on HARQ feedback (i.e. if NACK is generated for a TB). However, for UL the DRX retransmission timer is always triggered. There are at least two options to address the issue of retransmissions.

- Option 1 : UE will monitor PDCCH (even when it is indicated with PDCCH skipping), when DRX Retransmission timer DL or UL is running

- Option 2 : UE will monitor PDCCH (i.e. PDCCH skipping is cancelled) when DRX Retransmission timer DL is running

With Option 1, since for uplink the transmission timer is always triggered, this means the UE must monitor PDCCH always during the retransmission timer window. Option 2 limits the retransmission handling to only DL where the retransmission timer is triggered only when the packet is not successful. While both options address the retransmission handling, option 2 can be adopted as it can provide a good tradeoff between extra PDCCH monitoring vs impact on retransmissions.

- UE will monitor PDCCH (i.e., PDCCH skipping is cancelled) when DRX Retransmission timer DL is running.

Application delay for PDCCH-skipping when minimumSchedulingOffset is configured

Generally, it is up to UE implementation how fast it decodes the DCI to obtain the skipping information and adapt the PDCCH monitoring accordingly to save UE power. Having this in mind, there is no need to introduce additional delay in applying PDCCH skipping when minimumSchedulingOffset is configured. Even when the UE uses a relaxed PDCCH decoding mechanism, there will be no consequence if the UE is late in decoding the DCI containing the PDCCH-skipping indication. Given this, further clarification related to minimumSchedulingOffset  is not needed.

### 2.1.2	SSSG-switching

Following WA was made in RAN1#108-e meeting for application delay of SSSG switching commands.

Working Assumption

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Application delay value for different UE processing capabilities

It was agreed that for Rel. 17 SSSG-switching, the UE will apply the indicated SSSG at a first slot that is at least Pswitch symbols after the last symbols of the PDCCH containing the switching indication. The value of Pswitch itself will follow the value of Table 10.4-1 in TS38.213. It should be noted, however, the table consists of the values of 2 processing capabilities. A new FG (29-3e, Support Search space set group switching capability 2) is proposed in companion paper R1-2204963 to cover the capability2 support, analogous to capability 2 support for Rel-16 NR-U that covers unlicensed bands.

Another thing to clarify is that the Pswitch value will be configured. In Rel. 16 SSSG-switching, the UE is configured via an RRC parameter, searchSpaceSwitchDelay, where the configured value should be equal to or larger than the minimum Pswitch value shown in Table 10.4-1 from TS 38.213.

For Rel. 17 SSSG-switching, however, the agreement simply mentioned that the Pswitch value should follow Table 10.4-1 from TS 38.213. To clarify this, several options can be considered and the first approach is how Rel-16 SSSG switching works (and also how Rel-17 spec captures Pswitch for Rel-17 SSSG).

- Rel. 17 SSSG-switching uses the same mechanism as in Rel. 16 SSSG-switching, i.e., the Pswitch value is configured via searchSpaceSwitchDelay with the same value ranges, i.e., 10 to 52 symbols.

- Rel. 17 SSSG-switching uses the same mechanism as in Rel. 16 SSSG-switching, i.e., the Pswitch value is configured via searchSpaceSwitchDelay with different value ranges, i.e., it supports only values that appearing in Table 10.4-1 from TS 38.213.

- Confirm that RRC parameter searchSpaceSwitchDelay is used to configure the Pswitch value for Rel-17 SSSG switching.

Application delay for SSSG-switching when minimumSchedulingOffset is configured

According to the agreement in RAN1 #108, for the SSSG-switching feature, the UE will start monitoring PDCCH according to the indicated SSSG at a first slot that is at least Pswitch symbols after the PDCCH containing the DCI indicating the switching.

According to Rel. 16, a UE may be configured with a minimumSchedulingOffset parameter that inform the UE the minimum offset between the PDCCH and the corresponding scheduling. This feature is mainly beneficial in the sense of the UE may omit the PDSCH buffering during the PDCCH monitoring. By using this reasoning, the UE may already obtain the power-saving gain when the UE is configured with the minimumSchedulingOffset of 1 slot. Having this as the assumption, the application delay for the enhanced cross-slot scheduling feature will be 1 slot for the lower SCS and 2 slots for the higher SCS (note that the application delay for the cross-slot scheduling feature also has the Z factor on top of the minimumSchedulingOffset value). Seeing the value of Pswitch, the application delay for the SSSG-switching will also be around 1 slot for the lower SCS and 2 slots for the higher SCS. Thus, under this assumption, having a different application delay of SSSG-switching when the UE is configured and not configured with minimumSchedulingOffset may not be a critical issue.

It may be argued, however, some additional power saving gain of the enhanced cross-slot scheduling feature may come from the relaxed PDCCH decoding that may be used by the UE when the minimumSchedulingOffset value is sufficiently large. In this case, it may also be OK to have the SSSG-switching application delay depending on the enhanced cross-slot scheduling application delay. I.e., because it needs to make sure that the UE already finishes the decoding of the DCI containing the SSSG-switching indication. In a simple example, the applied application delay may be the max (X, Y) where X is the application delay of the SSSG-switching feature and Y is the application delay for the enhanced cross-slot scheduling feature.

- For SSSG-switching, minimumSchedulingOffset can be considered as a factor in the SSSG-switching application delay.

## 2.2	PDCCH skipping duration overlap with outside active time

In the previous meeting, RAN1 made following agreement to answer RAN2 LS question related to PDCCH skipping and DCI format 2_6 (or DCP) monitoring.

Agreement

         It is RAN1 understanding that for DRX operation,

  - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time.

Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

At least two options were discussed in last meeting.

- Option a : PDCCH skipping is cancelled when UE goes into to DRX (i.e. into outside active time).

- Option b : PDCCH skipping is applied for active time overlapping the indicated skipping duration. PDCCH skipping duration decrements by slot irrespective it is in active time or not.

A main issue with Option a is that it cancels PDCCH skipping even if the NW has no intention to schedule the UE for the indicated skipping duration, implying unnecessary increase in UE power consumption. For example, the UE may unnecessarily monitor PDCCH during subsequent DRX on durations. This power-saving loss will be more evident when the UE is not configured with WUS (i.e. DCI 2_6). Therefore, at least when WUS is not configured, option b should be used, i.e. PDCCH skipping should be applied for active time overlapping the indicated skipping duration even when there are some DRX slots in between (e.g. including Active time 1->DRX-> Active time 2). This then implies PDCCH skipping duration decrements by slot irrespective whether UE is in active time or not. When WUS is configured, option a can be used since subsequent on durations can be handled by DCI 2_6 though option b would also work. If a single solution is preferred, considering impact on both cases with and without WUS, we prefer option b.

- Select from below alternatives for application of PDCCH skipping duration outside active time

  - Alt1:

    - PDCCH skipping is applied for active time overlapping the indicated skipping duration. PDCCH skipping duration decrements by slot irrespective it is in active time or not.

  - Alt 2:

    - When DCI format 2_6 is configured,  PDCCH skipping is stopped when UE goes into DRX (i.e. into outside active time).

    - When DCI format 2_6 is not configured, PDCCH skipping is applied for active time overlapping the indicated skipping duration and PDCCH skipping duration decrements by slot irrespective whether UE is in active time or not.

## 2.3	PDCCH skipping in relation with SR and RACH

As per LS R1-2200884, RAN 2 agreed to the following and has captured the agreements into TS 38.300.

- UE ignores PDCCH skipping while the SR is pending.

- If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window.

- UE ignores PDCCH skipping while contention resolution timer is running.

RAN1 agreed to the following answer to RAN2 LS question#2.

RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification.

o    It is RAN1 understanding  PDCCH skipping  is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

- FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ).

Firstly, regarding where to capture the RAN2 agreements, since RAN2 seems to have captured the agreements in their spec, some further discussion is needed on what (if any) update are needed for RAN1. For example, the exact timing (as discussed in the FFS in above agreement) is one issue to address in RAN1.

For the SR pending case, the timing can be clearly defined relative to SR transmission, as shown by below proposal.

- At the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, UE stops PDCCH skipping  (i.e., PDCCH skipping is not activated).

Like the above, on the occasions such as during RAR/msgB window or when contention resolution timer is running, it will be beneficial if the UE could monitor PDCCH (in spite of NW indicated skipping command) as soon as possible to minimize the additional delay. In addition, the faster the UE is scheduled, the UE may finish the data transmission faster and may go to sleep earlier afterward.

- UE stops PDCCH skipping during the RAR/MsgB window, and while contention resolution timer is running.

# Conclusion

In section 2, the following observations and proposals were made:

Proposal 1	UE will monitor PDCCH (i.e., PDCCH skipping is cancelled) when DRX Retransmission timer DL is running.

Proposal 2	Confirm that RRC parameter searchSpaceSwitchDelay is used to configure the Pswitch value for Rel-17 SSSG switching.

Proposal 3	For SSSG-switching, minimumSchedulingOffset can be considered as a factor in the SSSG-switching application delay.

Proposal 4	Select from below alternatives for application of PDCCH skipping duration outside active time

  - Alt1:

- PDCCH skipping is applied for active time overlapping the indicated skipping duration. PDCCH skipping duration decrements by slot irrespective it is in active time or not.

  - Alt 2:

- When DCI format 2_6 is configured,  PDCCH skipping is stopped when UE goes into DRX (i.e. into outside active time).

- When DCI format 2_6 is not configured, PDCCH skipping is applied for active time overlapping the indicated skipping duration and PDCCH skipping duration decrements by slot irrespective whether UE is in active time or not.

Proposal 5	At the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, UE stops PDCCH skipping  (i.e., PDCCH skipping is not activated).

Proposal 6	UE stops PDCCH skipping during the RAR/MsgB window, and while contention resolution timer is running.

# References

RAN1#108-e chair notes

# Annex

Working Assumption (Excerpted from RAN#107bis-e)

- The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS

Agreement

-             The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation.

Agreement

- It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values.

Agreement

For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2,

- 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-     1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

Note: Same as in existing specifications, specifications allow gNB to configure a search-space set with zero PDCCH candidates.

Agreement

Answer 1 for R1-2200884 (R2-2201960):

Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e.

Agreement

Answer 2 for R1-2200884 (R2-2201960):

RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification.

o    It is RAN1 understanding  PDCCH skipping  is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

- FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ).

Agreement

Answer 4 for R1-2200884 (R2-2201960):

- It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration.

Agreement

- It is RAN1 understanding that for DRX operation,

  - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time.

Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

Agreement

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

- Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by

- Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI

- Otherwise, decrease the timer value by one after each slot.

- Note: There is no timer for SSSG switching is running for UE in SSSG#0

Agreement

[Draft] LS reply on PDCCH skipping, R1-2202896 is endorsed in principle.

Agreement

Final LS reply on PDCCH skipping, R1-2202905 is endorsed

Working Assumption

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Agreement

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes

  - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

    - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

Agreement

-     When the timer expires, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 in a slot

  - no earlier than [application delay] after the slot where timer expires and

  - if PDCCH skipping is being applied, no earlier than in slot where PDCCH skipping duration expires.

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.