3GPP TSG RAN WG1 #110bis                                                                                                   R1-2208868

e-meeting, October 10 – 19, 2022

Source:	OPPO

Title:	   Discussion on UE features (Topics 1)

Agenda Item:	8.16.1

Document for:	Discussion and Decision

# Introduction

After RAN1 #110 meeting, the RAN1 UE features list for Rel-17 NR was updated in [1]. In this contribution, we further elaborate our views on UE features, including IIoT/URLLC.

# Discussion

FG 25-3/3a/3b: PUCCH Repetition enhancements

For FG 25-3a, it is preferred to remove FG 30-5 from the prerequisite feature groups since there is no dependency for these two features although the mechanism of dynamic indication of repetition number is the same.

For FG 25-3b, the prerequisite feature groups should include FG 25-3 for support of sub-slot PUCCH repetition.

Proposal 1: Remove FG 30-5 from the prerequisite feature groups for FG 25-3a.

Proposal 2: The prerequisite feature groups for FG 25-3b includes FG 25-3.

FG 25-16: Intra-UE multiplexing with different priorities

Regarding to the prerequisite feature groups for FGs 25-16, FG 11-3 should be changed to FG 11-4 or one of {FG11-4, FG11-4a}. In addition, we prefer to remove FG 12-1 since it is unnecessary to couple Rel-16 intra-UE prioritization capability with Rel-17 intra-UE multiplexing capability. UE may choose to implement FG 25-16, while not to implement 12-1. This is because for a UE capable of intra-UE multiplexing almost no longer needs to perform cancelation between UL channels with different priorities.

Proposal 3: For the prerequisite feature groups of FG 25-16: change FG 11-3 to either FG 11-4 or one of {FG11-4, FG11-4a}, and remove 12-1.

FG 25-19a: RTT-based PDC based on DL-PRS and SRS

One pending issue is about the feature of PDC-PRS as spatial relation RS of PDC-SRS, which RAN1 agreed to discuss in UE feature session. It should be noted that the current TS 38.822 lists the spatial relation between positioning PRS and positioning SRS as a separate FG (FG 13-10b) for FR-2 only. It is also assumed in RAN1 PDC discussion that the benefit of such spatial relation for PDC is also mainly for FR-2.

Proposal 4: Adopt either of the following two options.

- Option-1: Add “Spatial relation for SRS for PDC based on PRS for PDC from the serving cell” as a new FG other than FG25-19a, with “N/A (FR2 only)” for FR1/FR2 differentiation.

- Option-2: Add “Spatial relation for SRS for PDC based on PRS for PDC from the serving cell. FR2 only.” as a new component to FG25-19a, and set “FR1/FR2 differentiation” of FG25-19a to “N/A”.

# Conclusion

In this contribution, we further discuss the UE features and the following proposals are made:

Proposal 1: Remove FG 30-5 from the prerequisite feature groups for FG 25-3a.

Proposal 2: The prerequisite feature groups for FG 25-3b includes FG 25-3.

Proposal 3: For the prerequisite feature groups of FG 25-16: change FG 11-3 to either FG 11-4 or one of {FG11-4, FG11-4a}, and remove 12-1.

Proposal 4: Relating to FG25-19a, adopt either of the following two options.

- Option-1: Add “Spatial relation for SRS for PDC based on PRS for PDC from the serving cell” as a new FG other than FG25-19a, with “N/A (FR2 only)” for FR1/FR2 differentiation.

- Option-2: Add “Spatial relation for SRS for PDC based on PRS for PDC from the serving cell. FR2 only.” as a new component to FG25-19a, and set “FR1/FR2 differentiation” of FG25-19a to “N/A”.

# Reference

- R1-2207923, Updated RAN1 UE features list for Rel-17 NR after RAN1 #110, Moderators (AT&T, NTT DOCOMO, INC.)