3GPP TSG RAN WG1 Meeting #110bis-e	R1-2208888

e-Meeting, October 10th – 19th, 2022

Title:	Draft reply LS on active TCI state list for UL TCI

Response to:	R4-2214972 / R1-2208354

Release:	Release 17

Work Item:	NR_feMIMO-Core

Source:	LG Electronics [RAN1]

To:	RAN2, RAN4

CC:

Contact Person:

Name:

E-mail Address:

Attachments:	None

Overall Description:

RAN1 would like to thank RAN4 for the question on active TCI state for UL TCI and pathloss reference RS (PL-RS). Answers from RAN1 are given as follows:

Question 1: What is the relationship between  qd and active UL (or joint) TCI state list? How does network configure the RS resource indexes qd?

Answer: If the UL (or joint) TCI state configuration includes PL-RS configuration, qd is same as the UL (or joint) TCI states that are activated by ‘TCI States Activation/Deactivation for UE-specific PDSCH MAC-CE’.

Question 2: Is UE expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or joint) TCI list?

Answer: Yes.

Question 3: What is the UE behavior when number of active UL (or joint) TCI states is larger than 4 if corresponding pathloss RSs need to be maintained?

Answer: When the number of active UL (or joint) TCI states is larger than 4, UE still expect that the total number of PL RSs included in active UL/joint TCI states is smaller than or equal to 4, meaning that same PL-RS may be repeated across multiple UL/joint TCI states.

Actions:

RAN1 respectfully requests RAN4 to take the above information into account.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#111,		November 14th - 18th, 2022			Toulouse, FR

TSG-RAN WG1 Meeting#112,		February 27th – March 3rd, 2023		Athens, GR