3GPP TSG RAN WG1 #110bis-e	                                                   	R1-2209046

e-Meeting, October 10th – 19th, 2022

Source:	Intel Corporation

Title:	Discussion on general aspects of AI/ML framework

Agenda item:	9.2.1

Document for:	Discussion and Decision

# Introduction

The new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study objectives includes terminology and description to identify common and specific characteristics for the AI/ML framework investigations. In RAN #110 meeting, the following was agreed on the general aspects of AI/ML framework:

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses. Agreement The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIsNote: Use-case specific KPIs may be additionally considered for the given use-case. Working Assumption Terminology; Description. Online training; An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.). Offline training; An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.. Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies. Working Assumption Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion. Terminology; Description. AI/ML model delivery; A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.. Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

In this contribution we discuss the terminology, general framework and evaluation methodology.

# General AI/ML Framework

## Functional framework

In terms of a functional framework for AI/ML we show below the framework from RAN3 TR37.817.

The operational mode for AI/ML can be categorized based on the side (NW or UE) at which the model inference is performed. They can be categorized as follows:

  - Single sided model at the Network side

  - Single sided model at the UE side

  - Two -sided model at the Network/UE

In the following, we present different functional frameworks for the 3 different cases as shown above – these are derived from 37.817 with small modifications.

## 2.2 Single sided model at Network

Figure 1 shows a single sided AI/ML model where the model is trained and inferred at the NW side. In this case, the NW may require a UE to assist in data collection for training and inference over the air-interface.

Figure 1 Single sided model at Network side

An example of this model could be MCS prediction where the scheduling unit acts as the actor and the action is selecting a certain MCS. The feedback from the actor could be for instance, the quality of prediction or the ACK/NACK resulting from the prediction. This model can be implemented for other AI/ML use cases e.g., CSI prediction, beam management.

- 2.3 Single sided model at UE

The below figure 2 shows a single sided AI/ML model where model training and inference occurs at the UE side.

Figure 2 Single sided model residing at the UE

In this case, some assistance information may be needed from the gNB to the UE (configuration/update) and some capability information from UE to gNB. Overall, the specifications impact to the air-interface can be relatively small. This model can be relevant for UE side channel prediction as an example.

- 2.4 Two-sided model

Figure 3 shows a two-sided model where the data collection and model training resides at the NW side. In this two-sided model, the model inference resides partly at the NW and partly at the UE. The NW may need assistance from UE for data-collection. In this case, a fully trained model could be delivered to the UE over the air interface (or other means). This model may have the most significant specification impact due to possibility of model exchange over the air interface. An example of this model is a CSI autoencoder where the encoder resides at the UE which compresses the CSI-RS based channel information and the decoder at the gNB that de-compresses the CSI.

Figure 3 Two- sided model

Proposal-1: The following functional frameworks are proposed based on NW-UE interaction (block diagrams, not agreed last time)

- Single sided model at NW (identical to RAN3 with small air-interface impact)

- Single sided model at UE (identical to RAN3 with small air-interface impact)

- Two-sided model (more significant air-interface impact)

# Collaboration Levels

In this section we provide an enhancement to the agreed NW-UE collaboration levels. In addition to categorizing NW-UE collaboration levels based on model transfer, the levels can be sub categorized based on single-sided or two-sided model.

The specification impact to having a single-sided model and two-sided model can be different. For instance, model monitoring, model deployment, model fine tuning etc will be different for single sided vs. double sided model.

Proposal-2: Consider the following network – UE collaboration levels as an enhancement to the agreed collaboration levels (split Level-1 and Level-2 of last agreement)

- Level 0: No collaboration

- Level 1a: Signalling-based collaboration for single-sided model without model transfer

- Level 1b: Signalling-based collaboration for two-sided model without model transfer

- Level 2: Signalling-based collaboration for two-sided model with model transfer

# Model life cycle management

The life cycle management of a ML model can be broadly categorized into data preparation, model creation/generation and model deployment/monitoring. The LCM of AI/ML models over the air interface is required for sustainable functioning of the AI/ML model and preventing any long-term performance degradation. In the following figure, we provide our high-level view of model LCM. This is based on the agreed upon terminologies from RAN1#110 as shown in the table below

Figure 4 Example of ML model lifecycle

Table 1 Different aspects of LCM

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| Model Training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| Model Deployment | Process of converting an AI/ML model into an executable form and delivering it to a target device where inference is to be performed. The conversion may happen before or after delivery |
| Model Registration | Model identifier for the network to identify the UE model version |
| Model Selection, activation, deactivation | Model selection is the process of selecting one model amongst many alternatives Model activation and deactivation is to enable and disable the model in case of non AI/ML model implementation |
| Model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model Update | Improve the model performance by updating the model and parameters or keeping the same model with new parameters |
| Model Transfer | Delivery of an AI/ML model over the air interface with 3GPP standardized mechanism to perform the transfer, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |

Proposal-3: Consider defining a Model LCM flow chart based on the agreed terminologies of data-collection, model training, model deployment, model registration, model selection/activation/deactivation, model inference, model monitoring, model update and model transfer

# Evaluation Methodology

## 5.1 Dataset Sharing

For AI/ML modelling, datasets are the most important. A common dataset could be beneficial to better align across different company’s results. However, generating this pool of data is not easy and therefore agreeing upon evaluation assumptions for dataset generation is another way to achieving common datasets.

Proposal-4: Consider the following options for achieving a common dataset

- Common dataset pool contributed by different companies

- Agreeing on evaluation assumptions to generate datasets

# Specification Impact

One-sided models:

For one sided models at the UE, the inference is performed in the UE side and correspondingly with one sided models at the NW, the inference is performed at the NW. Particularly for one sided models at the UE side, there should be assistance information exchanged between the NW and UE. As an example, UE capability, performance monitoring information from UE to the NW and model activation/deactivation/configuration from the NW to the UE.  Therefore, there could be specification impact from UE capability, performance monitoring, activation, de-activation of one or more models.

Two-sided models:

In the case of a two-sided model, a UE should be aware of the inference data and pre-processing information. For instance, in a CSI autoencoder use case, the pre-processing of inference data may include SVD, extraction of the strongest eigen vector followed by transformation from space-frequency domain to angular-delay domain, normalization etc. – all of which may have specification impact.

Proposal-5: Study specification impacts associated with one sided models (at least UE-side models) and two-sided models that may include UE capability exchange, performance monitoring, activation, de-activation, configuration of models.

# Conclusion

Proposal-1: The following functional frameworks are proposed based on NW-UE interaction (block diagrams, not agreed last time)

- Single sided model at NW (identical to RAN3 with small air-interface impact)

- Single sided model at UE (identical to RAN3 with small air-interface impact)

- Two-sided model (more significant air-interface impact)

Proposal-2: Consider the following network – UE collaboration levels as an enhancement to the agreed collaboration levels (split Level-1 and Level-2 of last agreement)

- Level 0: No collaboration

- Level 1a: Signalling-based collaboration for single-sided model without model transfer

- Level 1b: Signalling-based collaboration for two-sided model without model transfer

- Level 2: Signalling-based collaboration for two-sided model with model transfer

Proposal-3: Consider defining a Model LCM flow chart based on the agreed terminologies of data-collection, model training, model deployment, model registration, model selection/activation/deactivation, model inference, model monitoring, model update and model transfer

Proposal-4: Consider the following options for achieving a common dataset

- Common dataset pool contributed by different companies

- Agreeing on evaluation assumptions to generate datasets

Proposal-5: Study specification impacts associated with one sided models (at least UE-side models) and two-sided models that may include UE capability exchange, performance monitoring, activation, de-activation, configuration of models.

# Reference

| - [1] | - RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021. |
| --- | --- |
| - [2] | - Chairman notes of RAN1 #110 meeting |