3GPP TSG RAN WG1 Meeting #110bis-e 	 R1-2209517

e-Meeting, October 10th – 19th, 2022

Agenda Item: 9.10.1

Source: MediaTek Inc.

Title: On XR specific power saving techniques

Document for: Discussion and Decision

Introduction

During RAN1 #109e, the following highlights can be drawn from the RAN1 #109e chairman’s note [1].

For CDRX related power saving enhancements:

- High priority Issue 1-1: Mismatch between CDRX cycle and XR traffic periodicity for each flow

- High priority Issue 1-2: C-DRX enhancements to handle jitter

For PDCCH monitoring related power saving enhancements:

- High priority Issue 2-1: Enhancements to Rel-17 PDCCH monitoring adaptation

  - Focus on the techniques addressing XR-specific issues, e.g., jitter, re-transmission

During RAN1 #110, the following highlights can be drawn from the RAN1 #110 chairman’s note [2].

Agreement

RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity

Conclusion

All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference.

In this contribution, we further discuss the techniques for XR specific power saving enhancements.

Retransmission handling for Rel-17 PDCCH skipping

In Rel-17 the PDCCH skipping feature has been introduced under the power saving enhancements. The UE can be indicated to skip monitoring for a duration by the scheduling DCI format X-1/X-2. The feature brings good power saving gain while the scheduling is impacted especially for the retransmission requests.

Due to XR application’s stringent latency requirement, instant retransmission is critical. However, in the current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. In one way, NW can handle the situation by applying a low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission. Hence, further retransmission-aware mechanisms should be explored for XR traffic.

Observation 1: In current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. Due to XR application’s stringent latency requirement, further retransmission-aware mechanisms should be explored.

We have two solutions to address the retransmission following the PDCCH skipping indication.

Solution 1: If the UE is indicated to skip PDCCH monitoring for a specific duration based on the Rel-17 adaptation scheme, then if any NACK is transmitted the UE resumes the monitoring and cancel the indication of Skipping.

Proposal 1: (Solution 1) If the UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted the UE resumes the monitoring and cancel the indication of PDCCH.

Solution 2: In solution 2, when the UE is indicated to skip PDCCH monitoring for a duration by the Rel-17 adaptation scheme, then if any NACK is transmitted or if the scheduled UL data is transmitted, the UE has to resume monitoring for a configured duration.

Proposal 2: (Solution 2) If the UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted or if UL data is transmitted, the UE resumes the monitoring for a configured duration.

We have preference for solution 2 as it can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration.

Observation 2: Solution 2 can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration.

For Solution 2, we performed system level simulation (SLS) to evaluate the achievable power saving gain. The results are shown in Figure 1 below.

Figure 1 Power saving gain results by SLS for retransmission aware PDCCH skipping

In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling.

Observation 3: In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling.

At the same time, as shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping.

Observation 4: As shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping.

As mentioned before, in one way, NW can handle this retransmission issue by applying a very low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission.

Observation 5: In one way, NW can handle the retransmission issue by applying a low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission.

To provide a quantified analysis for Observation 4, we developed some analysis for robust MCS for last data scheduling to ensure a correct decoding based on SLS simulation results. The target is to use a low MCS to make sure that UE can decode last data transmission successfully. The resource impact of this low MCS scheme is modelled by the following formula:

s=2ps×[s-1s+1s×SEbaselineSEadap]              (1)

where

- 𝑠: number of scheduled data transmissions (segments) for one frame, s = 1, 2, 3, ….

- ps: probability of one frame is transmitted by 𝑠 scheduled data transmissions (segments)

  - The value of 𝑠 and ps are derived from SLS results as shown in Figure 2

- SEbaseline :  The spectral efficiency (SE) from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3], using the MCS calculated by gNB channel based on link adaptation algorithm in SLS

- SEadap :  The SE from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3], using a low MCS 0 to 5

Figure 2 MCS index and number of scheduled data transmissions (segments) for one frame in XR application

The results of resource impact using formula (1) is shown below in Figure 3 below. It can be seen that the robust MCS scheme can consume 1.72 times to 7.31 times of resources with MCS 0 to MCS 5, which is a large overhead for XR application.

Figure 3 Resource impact of robust MCS for last data scheduling to ensure a correct decoding

Observation 6: Figure 3 (using Formula (1) below) shows that the robust MCS scheme for last data scheduling to ensure a correct decoding can consume 1.72 times to 7.31 times of resources with MCS 0 to MCS 5, which is a large overhead for XR application.

The resource impact is modelled by the following formula:

s=2ps×[s-1s+1s×SEbaselineSEadap]              (1)

where

- 𝑠: number of scheduled data transmissions (segments) for one frame, s = 1, 2, 3, ….

- ps: probability of one frame is transmitted by 𝑠 scheduled data transmissions (segments)

  - The value of 𝑠 and ps are derived from SLS results as shown in Figure 2

- SEbaseline :  The spectral efficiency (SE) from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3], using the MCS calculated by gNB channel based on link adaptation algorithm in SLS

- SEadap :  The SE from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3], using a low MCS 0 to 5

CDRX enhancement for XR

In RAN1 #110 [2], the following is agreed:

Agreement

RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity

According to the companies’ contributions summarized in the RAN1 #110 moderator summary [4], we observed that semi-static enhancement of CDRX can achieve similar gain as dynamic enhancement of CDRX, while semi-static enhancement has less signaling overhead.

Observation 7: According to the companies’ contributions summarized in the RAN1 #110 moderator summary [4], semi-static enhancement of CDRX can achieve similar gain as dynamic enhancement of CDRX, while semi-static enhancement has less signaling overhead.

We hence have the following proposal:

Proposal 3: RAN1 to recommend semi-static CDRX enhancement for XR. The remaining discussions and spec impact can be left to RAN2.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: In current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. Due to XR application’s stringent latency requirement, further retransmission-aware mechanisms should be explored.

Proposal 1: (Solution 1) If the UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted the UE resumes the monitoring and cancel the indication of PDCCH.

Proposal 2: (Solution 2) If the UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted or if UL data is transmitted, the UE resumes the monitoring for a configured duration.

Observation 2: Solution 2 can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration.

Observation 3: In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling.

Observation 4: As shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping.

Observation 5: In one way, NW can handle the retransmission issue by applying a low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission.

Observation 6: Figure 3 (using Formula (1) below) shows that the robust MCS scheme for last data scheduling to ensure a correct decoding can consume 1.72 times to 7.31 times of resources with MCS 0 to MCS 5, which is a large overhead for XR application.

The resource impact is modelled by the following formula:

s=2ps×[s-1s+1s×SEbaselineSEadap]              (1)

where

- 𝑠: number of scheduled data transmissions (segments) for one frame, s = 1, 2, 3, ….

- ps: probability of one frame is transmitted by 𝑠 scheduled data transmissions (segments)

  - The value of 𝑠 and ps are derived from SLS results as shown in Figure 2

- SEbaseline :  The spectral efficiency (SE) from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3], using the MCS calculated by gNB channel based on link adaptation algorithm in SLS

- SEadap :  The SE from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3], using a low MCS 0 to 5

Observation 7: According to the companies’ contributions summarized in the RAN1 #110 moderator summary [4], semi-static enhancement of CDRX can achieve similar gain as dynamic enhancement of CDRX, while semi-static enhancement has less signaling overhead.

Proposal 3: RAN1 to recommend semi-static CDRX enhancement for XR. The remaining discussions and spec impact can be left to RAN2.

References

- Chairman’s Notes, RAN1 #109-e (9.11.1 XR-specific power saving techniques)

- Chairman’s Notes, RAN1 #110 (9.11.1 XR-specific power saving techniques)

- 3GPP TS 38.214 V17.3.0, 5G; NR; Physical layer procedures for data

- R1-2207833, “Final Moderator Summary on XR specific power saving techniques”, Moderator (Qualcomm Incorporated), RAN1 #110

Appendix

Table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3]