3GPP TSG-RAN WG1 #110bis-e							            R1-2209794

e-Meeting, October 10-19th, 2022

Agenda Item:	9.2.2.1

Source:	AT&T

Title:	Discussion on AI/ML for CSI feedback enhancement

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. One of the identified use cases include:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

For each of the use cases, one of the objectives is to

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

  - Note: Study of other sub use cases is not precluded.

  - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Conclusion

- Further discuss temporal-spatial-frequency domain CSI compression using two-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss improving the CSI accuracy based on traditional codebook design using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss CSI prediction using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss CSI-RS configuration and overhead reduction as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss resource allocation and scheduling as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss joint CSI prediction and compression as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

In RAN#109e [2], the following agreements were reached.

In RAN1#110[3] the following conclusion was reached

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

    - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

    - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

# Generic issues on evaluation methodology

## Channel estimation

In the previous meeting the following conclusion was reached:

Proposed Conclusion 2.3-2: For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

We want to update the note, regarding companies to report if ideal or realistic channel is used for simulations. We propose the following:

Proposal 1: For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modelling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate. Companies should report if ideal channel is used for simulations.

# Representative sub use cases for CSI feedback enhancement

In RAN1#109e, spatial-frequency domain CSI compression using two-sided AI model was selected as one representative sub use case for CSI feedback enhancement. In RAN1#110 it was concluded that CSI-RS configuration and overhead reduction and resource allocation and scheduling are not selected as representative use cases for CSI feedback enhancement.

With the RAN#98 deadline to finalize the sub-use cases, it is critical to complete the selection of sub-use cases on CSI feedback enhancements in this meeting, so that companies can concentrate their efforts on the approved sub-use cases. In RAN1#109 it was concluded that for any use case approval the EVM discussion first needs to be concluded therefore we propose.

Proposal 2: Finalize the EVM for the representative sub-use case to be considered for the CSI feedback enhancement.

We believe that the CSI prediction using a one-sided AI/ML model should be selected as a representative sub-use case for CSI feedback enhancements.  Unlike the CSI compression sub-use case that has been selected, which is a two-sided AI model, the CSI prediction use case is a one-sided model that is significantly unique from the CSI compression sub-use case and can considerably improve the overall performance of the system. Therefore, the EVM discussion for CSI prediction should be prioritized so that it can be agreed as a representative sub use-case and discussion on specification impact can begin.

Proposal 3: Finalize the EVM for the CSI prediction sub-use case for CSI feedback enhancements.

# Evaluation methodology for CSI compression sub use case

In the CSI feedback procedure deployed by the current systems, the CSI measurement and the DL transmission based on the CSI measurement are performed at different time steps. Using this out-of-date CSI information for downlink scheduling reduces the spectral efficiency for the DL transmission, especially for high mobility UEs. CSI prediction can be seen as a key enabler to combat channel aging for use cases such as MU-MIMO pairing and precoding, and massive MIMO deployments that require adaptive precoding across multiple often distributed transmission points and rely on accurate channel information for precoding. Therefore, an AI/ML based CSI prediction is crucial to be selected to predict the CSI at the time of transmission based on the past CSI.

While there is a concern regarding simultaneous study of AI based CSI prediction in this agenda item and rel. 18 type II codebook enhancements in the MIMO WI, we believe both have significant distinctions and have expressed our view in detail in [4]. However, to better understand the gain of AI/ML model over non Ai/ML model, we can use the same baseline for AI/ML CSI prediction as the R18 MIMO enhancement study item.

Proposal 4: Use same baseline for AI/ML CSI prediction as in R18 MIMO CSI enhancement.

Proposal 5: For the evaluation of the AI/ML based CSI prediction sub use cases,

- 100% outdoor UE is assumed for UE distribution.

- FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

- Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Furthermore, while there were some conclusions reached on the CSI prediction use case EVM conditioned on the CSI use case is accepted as a representative use case. We believe that we should finalize the remaining EVM items. Based on the conclusion and agreement from last meeting we have the following proposals.

Similar to the CSI compression sub-use case, the details of the models for evaluation should be reported for the CSI prediction model.

Proposal 6: For the AI/ML based CSI prediction sub use cases, companies are encouraged to report the details of their models for evaluation, including:

- Location of model (gNB or UE)

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Observation and prediction windows size

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Similarly, the evaluation metrics can be the average UPT, 5%ile UE throughput, and CDF of UPT.

Proposal 7:  For the evaluation of the AI/ML based CSI prediction sub use cases, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Furthermore, the evaluation metric for the AI/ML model should be discussed to understand inference complexity for using AI/ML models.

Proposal 8:  For the evaluation of the AI/ML based CSI prediction sub use cases, the inference complexity in the ‘Evaluation Metric’ includes

- Computational complexity of model inference: FLOPs

- Computational complexity for pre- and post-processing

- Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

Finally, the agreement for the verification of generalization for general CSI feedback enhancement must be discussed and refined for the CSI prediction use-case.

Proposal 9: For the evaluation of the AI/ML based CSI prediction sub use cases, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

# Conclusion

In this contribution, we discussed a sub-use case on CSI feedback enhancements related to MU-MIMO scheduling optimization. We made the following observations and proposals.

Proposal 1: For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modelling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate. Companies should report if ideal channel is used for simulations.

Proposal 2: Finalize the EVM for the representative sub-use case to be considered for the CSI feedback enhancement.

Proposal 3: Finalize the EVM for the CSI prediction sub-use case for CSI feedback enhancements.

Proposal 4: Use same baseline for AI/ML CSI prediction as in R18 MIMO CSI enhancement.

Proposal 5: For the evaluation of the AI/ML based CSI prediction sub use cases,

- 100% outdoor UE is assumed for UE distribution.

- FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

- Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Proposal 6: For the AI/ML based CSI prediction sub use cases, companies are encouraged to report the details of their models for evaluation, including:

- Location of model (gNB or UE)

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Observation and prediction windows size

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Proposal 7:  For the evaluation of the AI/ML based CSI prediction sub use cases, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Proposal 8:  For the evaluation of the AI/ML based CSI prediction sub use cases, the inference complexity in the ‘Evaluation Metric’ includes

- Computational complexity of model inference: FLOPs

- Computational complexity for pre- and post-processing

- Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

Proposal 9: For the evaluation of the AI/ML based CSI prediction sub use cases, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface, RAN#94e.

[2] RAN1#109-e Chair’s Notes, e-meeting May 9-20th, 2022

[3] RAN1#110 Chair’s Notes, August 22nd – 26th, 2022.

[4] R1-2209795, Discussion on Ai/ML for CSI feedback enhancement, October 10-19th, 2022