3GPP TSG RAN WG1 #110b-e	R1-2210167

e-Meeting, October 10 – 19, 2022

Agenda item:		9.14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Dynamic switching between DFT-s-OFDM and CP-OFDM

Document for:		Discussion and Decision

# Introduction

In RAN #94e a new work item description was approved on further NR coverage enhancements [1]. Three main objectives characterize the work item:

The detailed objectives of the work item are as follows: - Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

This contribution focuses on the last objective of the work item, i.e., dynamic switching between DFT-s-OFDM and CP-OFDM. Section 2.1 briefly recall definition and configuration of CP-OFDM and DFT-s-OFDM in Rel-15/16. Section 2.2 discusses dynamic waveform switching in Rel-18, including the scope, general observation on the design of the feature and detailed discussions on waveform switching solutions. Section 3 concludes the document by highlighting key proposals and observations.

# Discussion

## CP-OFDM vs. DFT-s-OFDM in NR Rel-15/16

NR supports two waveforms in UL, DFT-s-OFDM and CP-OFDM. DFT-s-OFDM supports single layer transmission while CP-OFDM can support multi-layer UL transmission. However, network can only configure waveform for UL transmission semi-statically. This means that even though a UE supports multiple Tx antennas, if configured with DFT-s-OFDM waveform it is expected to transmit only with a single layer (Rank=1). For this reason, network may configure CP-OFDM waveform for UL transmission to support more than one layer transmission (Rank ≥1) and to exploit UL MIMO transmission with spatial multiplexing. On the other hand, DFT-s-OFDM offers a lower PAPR compared to CP-OFDM, and hence larger deliverable power at the power amplifier. Therefore, when the UE is at cell edge, DFT-s-OFDM waveform provides better coverage due to higher power efficiency with up to 2dB smaller maximum power reduction (thanks to lower PAPR) compared to CP-OFDM (see e.g. Table 6.2.2.1-1 in TS 38.101-1).

DFT-s-OFDM is generated by adding a transform precoding block before the processing blocks used for generating CP-OFDM. In fact, the transform precoding block is a fast Fourier transform (FFT) block that converts a time domain signal into frequency domain signal. Due to the transform precoding block used in the transmitter, the waveform has to be separately configured so that receiver can perform the inverse operation (IFFT).

In current NR specifications, gNB configures the uplink waveform in semi-static manner via RRC signalling as follows:

- In case transformPrecoder in PUSCH-Config (or ConfiguredGrantConfig) is enabled, DFT-s-OFDM waveform is applied for the corresponding PUSCH transmissions. Otherwise, CP-OFDM is applied.

- In case msg3-transformPrecoder in RACH-ConfigCommon is configured, DFT-s-OFDM is applied for Msg3, or for the other PUSCH transmissions that are scheduled with DCI format 0_0 and the corresponding PDCCH is scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-CRNTI or SP-CSI-RNTI, or for the other PUSCH transmissions that are not scheduled with DCI format 0_0 in case transformPrecoder is not configured in PUSCH-Config and the corresponding PDCCH is scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-CRNTI or SP-CSI-RNTI. Otherwise, CP-OFDM is applied.

## Dynamic waveform switching in Rel-18

### Scope of the feature

Although objective of the WI does not explicitly mention the physical channel for the dynamic waveform switching to be applied, the scope of dynamic waveform switching in Rel-18 should be limited to only PUSCH because of the following reasons:

- First, from Rel-17 SI for coverage enhancements, only some PUCCH formats have coverage shortage in the evaluated scenario and link-budget of the PUCCH is always better than PUSCH.

- Second, considering dynamic waveform switching for both PUSCH and PUCCH may significantly increase the workload for RAN1, regardless of whether it is for finding a unified solution or specifying different solutions for different channels.

- Last but not least, currently waveform is fixed for each PUCCH format. However, each PUCCH resource set contains multiple PUCCH formats. Therefore, by dynamically selecting a PUCCH resource (associated with a PUCCH format) using PUCCH resource indicator (PRI), the gNB can already dynamically indicate the waveform for PUCCH.

In contrast to PUCCH, Msg3 PUSCH was identified as a channel suffering from coverage shortage in Rel-17 SI (which led to Msg3 repetition feature specified for coverage enhancement in Rel-17). Means for dynamic waveform switching for Msg3 does not exist. Moreover, some (if not all) specification efforts for specifying dynamic waveform switching for PUSCH can be leveraged for Msg3 PUSCH.

From the above analysis, we propose the following:

Proposal 1. The scope of dynamic waveform switching in Rel-18 is limited to only for PUSCH. FFS: Msg3 PUSCH.

### General observation on the design of the feature

It can be observed from the objectives of the WID that RAN1 must study and, if necessary, specify enhancements to reduce MPR/PAR. This objective may introduce multiple variations of at least DFT-s-OFDM waveform, e.g., legacy DFT-s-OFDM vs. DFT-s-OFDM with MPR/PAR reduction technique. We herein refer to CP-OFDM and the potential variants of DFT-s-OFDM as waveform operation modes for the sake of clarity since, strictly speaking, there are only two waveforms (CP-OFDM and DFT-s-OFDM), but the MPR/PAR reduction technique may introduce more than two waveform operation modes for at least DFT-s-OFDM. Therefore, means for indicating more than two waveform operation modes may be needed after the study on MPR/PAR reduction is completed. Although whether a dynamic switching is desired for switching among these waveform operation modes or not should be further discussed. RAN1 should consider this information when designing the framework for dynamic waveform switching in Rel-18. In other words, the framework for dynamic switching between CP-OFDM and DFT-s-OFDM in Rel-18 should be easily extensible for also accommodating the switching among more than two waveform operation modes. In any case it is worth noting that the waveform operation modes to be used are controlled by the gNB, therefore it is possible that the waveform switching is between e.g., legacy DFT-s-OFDM and DFT-s-OFDM with MPR/PAR reduction technique or some other pair of waveform operation modes as configured by gNB.

Observation 1. Given that the objectives of the WID also include possibility of specifying enhancements to reduce MPR/PAR, multiple variations of DFT-s-OFDM may be introduced. Therefore, means for indicating more than two waveform operation modes may be needed in the waveform switching after the study on MPR/PAR reduction is completed.

Proposal 2. The framework for dynamic switching between CP-OFDM and DFT-s-OFDM in Rel-18 should be easily extensible for also accommodating the switching among more than two waveform operation modes.

### Discussion on waveform switching solutions

DCI-based signaling vs. MAC-CE signaling:

It can be observed from Section 2.1 that currently waveform is RRC configured. Therefore, solution for dynamic waveform switching in Rel-18 should at least offer lower latency than RRC reconfiguration. Solutions for switching between DFT-s-OFDM and CP-OFDM waveforms had been discussed in the context of Rel-17 technical enhancement item (TEI), and the following was proposed without any conclusion [2]:

“Support for dynamic switching of waveform in UL

- Alt1: DCI signaling based dynamic UL waveform switching, it could be implicit or explicit

  - Alt1-1: Explicit signaling, e.g., by introducing 1 bit in DCI to indicate CP-OFDM or DFT-s-OFDM waveform to be used for PUSCH

  - Alt1-2: Implicit signaling, e.g., CP-OFDM or DFT-s-OFDM waveform to be used for PUSCH is identified by certain condition on the scheduling information in the DCI without changing DCI format.

- Alt2: MAC-CE signaling based dynamic UL waveform switching”

It is expected that the above two alternatives should be the baseline for the discussions in Rel-18 dynamic waveform switching. Each alternative above has its pros and cons, which can be summarized as follows.

- For Alt. 1-1 (explicit DCI-based signaling)

  - Pros:

    - This alternative is more flexible/dynamic than Alt. 1-2 and Alt. 2 in terms of scheduling (as mentioned below in the discussions of these alternatives).

    - This alternative may require less specification efforts compared to both Alt. 1-2 and Alt. 2.

  - Cons:

    - DCI overhead is increased if the 1 bit is always added in the DCI, once it is configured. The overhead issue would be even more crucial if more than 1 bit were added for accommodating the switching among more than two waveform operation modes, as discussed in Section 2.2.2.

- For Alt. 1-2 (implicit DCI-based signaling)

  - Pros:

    - This alternative is more flexible/dynamic than Alt. 2 in terms of scheduling. Indeed, if the gNB anticipates that a UE may be in coverage shortage by measuring e.g., UE’s scheduling request signal, the gNB can indicate waveform switching immediately using the scheduling DCI, instead of waiting for scheduling it via PDSCH (MAC-CE). This is also applied for Alt. 1-1.

    - This alternative does not introduce DCI overhead.

  - Cons:

    - This alternative may impose certain constrains on the scheduler, and therefore, being less flexible/dynamic compared to Alt. 1-1.

    - This alternative impacts the way UEs read scheduling information, and therefore, may introduce more specification efforts and system complexity compared to Alt. 1-1.

    - This alternative is not easily extensible for the switching among more than two waveform operation modes, if such extension is needed. Introducing support for such switching possibilities would require more specification efforts and could lead to considerable scheduling constraints.

- For Alt. 2 (MAC-CE signaling)

  - Pros:

    - This alternative does not introduce DCI overhead.

    - This alternative is easily extensible for accommodating the switching among more than two waveform operation modes.

  - Cons:

    - This alternative is less flexible/dynamic than Alt. 1-1 and Alt. 1-2 in terms of scheduling, as explained above.

    - This alternative introduces a significant latency for waveform switching, especially for dynamic grant (DG) PUSCH, compared to Alt. 1-1 and Alt. 1-2.

    - This alternative can only be applied for switching the waveform of all PUSCHs after the MAC-CE indication but cannot be applied for switching of a specific PUSCH, i.e., per PUSCH indication (note that this is also related to the cons on flexibility and latency above). Therefore, when the gNB needs to switch waveforms frequently, overhead in DL transmissions may exist.

    - This alternative may require more efforts from both RAN1 and RAN2 for specifying the MAC-CE indication compared to at least Alt. 1-1.

It can be observed from the above analysis that, although MAC-CE based signalling solution does not introduce DCI overhead (which may or may not be the case for DCI-based signalling solution), it has several disadvantages in terms of flexibility, latency, potential DL transmission overhead and specification efforts compared to DCI-based signalling solution, depending on the exact DCI-based signalling solution to be adopted. Therefore, RAN1 should prioritize DCI-based signalling solution for dynamic waveform switching in Rel-18.

Observation 2. Although MAC-CE based signalling solution does not introduce DCI overhead (which may or may not be the case for DCI-based signalling solution, depending on the exact DCI-based signalling solution to be adopted), it has several disadvantages in terms of flexibility, latency, potential DL transmission overhead and efforts from multiple working groups compared to DCI-based signalling solution.

Proposal 3. RAN1 to prioritize DCI-based signalling solution for dynamic waveform switching in Rel-18.

DCI formats to be considered for DCI-based signalling:

For DCI-based signalling, one aspect should be discussed is that which DCI formats should be considered for waveform switching indication. This includes whether both DL and UL DCI should be considered and what are the corresponding applicable DCI formats.

Concerning DL vs. UL DCI, it is straightforward that at least DCI-based signalling using UL DCI should be considered, since this not only offers the flexibility for dynamically indicating waveform for each scheduled PUSCH, but also offers low switching latency for at least dynamic grant (DG) PUSCH as well as for configured grant (CG) PUSCH when there are dynamic UL transmissions to be scheduled by UL DCI. However, limiting waveform switching indication to only UL DCI would have its drawbacks when there are no dynamic UL transmissions to be scheduled. There would be either considerable latency in indicating waveform switching for configured grant (CG) PUSCH or unnecessary UL scheduling overhead due to UL DCI sent for scheduling a PUSCH without data just for waveform indication. These drawbacks can be avoided by using DL DCI in addition to UL DCI for DCI-based signalling of waveform indication.

Observation 3. DCI-based signalling using UL DCI not only offers the flexibility for dynamically indicating waveform for each scheduled PUSCH, but also offers low switching latency for at least DG PUSCH.

Observation 4. DCI-based signalling using DL DCI not only offers lower latency for indicating waveform switching for CG PUSCH, but also avoid UL scheduling overhead in case there is no dynamic uplink transmission to be scheduled by UL DCI and there are DL transmissions to be scheduled by DL DCI.

Concerning the applicable DCI formats, at least the fallback DCI formats (DCI format 0_0 and DCI format 1_0) should be considered, since fallback DCI formats are relevant for coverage enhancements. Indeed, with a smaller payload compared to non-fallback formats, fallback DCI formats can have a smaller coding rate and hence being more reliable in coverage shortage scenario. On other hand, switching from DFT-s-OFDM waveform to CP-OFDM may be relevant when UE is not anymore in strict coverage shortage scenario. However, as the fallback DCI formats support only single-layer transmission, they may not be suitable for switching from DFT-s-OFDM to CP-OFDM where the latter can be used with rank greater than one. Therefore, non-fallback DCI formats should not be precluded for DCI-based signalling solution.

Observation 5. Fallback DCI formats can have a smaller coding rate and hence being more reliable in coverage shortage scenario. However, since fallback DCI formats support only single-layer transmission, it may not be suitable for switching from DFT-s-OFDM to CP-OFDM where the latter can be used with rank greater than one.

Proposal 4. RAN1 to consider both DL and UL DCI for DCI-based signalling solution for dynamic waveform switching in Rel-18. At least the fallback DCI formats 0_0 and 1_0 should be supported. FFS: Other non-fallback DCI formats.

Explicit vs. implicit DCI-based signalling:

For explicit DCI-based signalling (i.e., Alt. 1-1), an example of introducing one additional bit in DCI was mentioned in Rel-17 TEI. However, as aforementioned in the drawback of Alt. 1-1, DCI overhead is increased if the additional bit is always added in the DCI. The overhead issue would be even more crucial if more than 1 bit were added for accommodating the switching among more than two waveform operation modes, if specified.

Explicit signalling implies that UEs read explicit bit(s) in the DCI for waveform switching, however this direction does not always require additional bit(s) to be added. Indeed, another example of explicit DCI-based signalling is to repurpose certain bit(s) in a DCI field without changing the DCI size. UEs then read the repurposed bit(s) for waveform switching. The advantage of this approach is that it does not introduce DCI overhead. However, as a drawback, it limits the usage of the repurposed DCI field. Therefore, we see the need to consider solutions to tackle the drawbacks of increased DCI overhead or introduced scheduling constraints due to limiting the usage of the repurposed DCI field. solutions to tackle this drawback should also be considered, together with the discussion on the field to be repurposed. Similar to the case of explicit signalling using additional bit(s), one straightforward approach for reducing the impact on usage of the repurposed DCI field is to consider the field to be repurposed only for some specific PDCCH occasions.

Proposal 5. RAN1 to consider solutions to mitigate DCI overhead increase and to reduce impact on the repurposed DCI field for explicit DCI-based signalling of dynamic waveform switching.

For implicit DCI-based signalling (i.e., Alt. 1-2), the idea is to signal waveform switching based on one or multiple pre-defined conditions. Similar to repurposing bit(s) for explicit signalling, main benefit of using implicit signalling is that it does not introduce DCI overhead. However, as aforementioned in the drawback of Alt. 1-2, the predefined conditions impose certain constraints on the scheduler. During Rel-17 TEI discussion, several options were proposed for implicit signalling including:

- Option 1: Waveform is DFT-s-OFDM if contiguous RB allocation and multiple value of 2, 3, 5, else CP-OFDM.

- Option 2: Waveform is DFT-s-OFDM if MCS is lower than threshold, else CP-OFDM.

- Option 3: Waveform is CP-OFDM if PUSCH and DMRS is FDMed (based on ‘Number of DMRS CDM group(s) without data’), else DFT-S-OFDM.

- Option 4: Waveform is CP-OFDM if more than one layer/rank are indicated, else DFT-s-OFDM.

It can be observed that the predefined conditions in Option 1 to Option 4 are RB allocation size fulfilling DFT size condition, MCS, DM-RS structure, and MIMO rank, respectively. Although using a combination of some of these conditions might be a workable solution for signalling waveform switching, one should not forget that the objective of introducing dynamic waveform switching is for coverage enhancement. Therefore, the net gain of each waveform should be taken into account. For DFT-s-OFDM the gain come from low PAPR characteristic of the waveform which leads to lower MPR and higher UL power. However, the MPR is specified in RAN4 specifications for each waveform and the value is different for different RB regions and modulation orders. Specifically, for QPSK, the specified MPR values for edge/outer RB regions are higher than that for inner RB region for both DFT-s-OFDM and CP-OFDM, while the gap in terms of MPR in inner RB region between the two waveforms is smaller. This implies that the power gain offered by DFT-s-OFDM may be less significant, if not void, compared to CP-OFDM for inner RB region. In this case, CP-OFDM may be preferred as it could provide slightly better detection performance, which depends on implementation. Therefore, it is more relevant in views of coverage enhancements to consider at least the condition of RB regions and associated MPR for waveform switching, i.e., DFT-s-OFDM is used when the PUSCH is scheduled in the outer/edge regions and CP-OFDM is used when the PUSCH is scheduled in the inner RB region.

Observation 6. For a given modulation order, the specified MPR values for edge/outer RB regions are higher than that for inner RB region for both DFT-s-OFDM and CP-OFDM, while the gap in terms of MPR in inner RB region between the two waveforms is smaller. This implies that the power gain offered by DFT-s-OFDM may be less significant, if not void, compared to CP-OFDM for inner RB region.

Observation 7. It is more relevant in views of coverage enhancements to consider at least the condition of RB regions and associated MPR for waveform switching, i.e., DFT-s-OFDM is used when the PUSCH is scheduled in the outer/edge regions and CP-OFDM is used when the PUSCH is scheduled in the inner RB region.

In addition, from coverage enhancement point of view, power gain offered by DFT-s-OFDM is only needed if the UE cannot further boost its power (i.e., power limited UE). In other words, DFT-s-OFDM could be selected if the latest power headroom report (PHR) is sufficiently small.

Observation 8. Power gain offered by DFT-s-OFDM is only needed if the UE cannot further boost its power, i.e., if the latest power headroom report is sufficiently small.

Another aspect to be considered in coverage shortage is that the number of allocated RBs should not be so large. Indeed, the smaller number of RBs, the higher power per resource element and, thus, better link-budget can be achieved. Note that this is the main motivation for specifying TB processing over multiple slots (TBoMS) feature for coverage enhancements in Rel-17. Therefore, if a UE should switch to DFT-s-OFDM due to coverage shortage, it should not be allocated with a high number of RBs. In other words, DFT-s-OFDM should only be selected if the number of allocated RBs is sufficiently small.

Observation 9. If a UE should switch to DFT-s-OFDM due to coverage shortage, the number of allocated RBs should be sufficiently small.

From the above observations, at least the conditions of RB regions and associated MPR, PHR limitation, and total allocated RBs should also be considered for implicit waveform switching.

Proposal 6. RAN1 to further consider the following conditions for discussions on implicit DCI-based signalling for dynamic waveform switching indication:

- RB regions and associated MPR (i.e., DFT-s-OFDM is used when the PUSCH is scheduled in the outer/edge regions and CP-OFDM is used when the PUSCH is scheduled in the inner RB region),

- PHR limitation (i.e., DFT-s-OFDM is used when the latest PHR is sufficiently small, otherwise CP-OFDM),

- Total allocated RBs (i.e., DFT-s-OFDM is used when the total number of allocated RBs is sufficiently small, otherwise CP-OFDM).

From the identified conditions for implicit signalling (including the ones from Rel-17 TEI), scheduling flexibility would be reduced significantly if only one condition is considered for implicit signalling. For instance, if MCS condition is used, this would mean that CP-OFDM can never be used with low MCS levels, which significantly impacts the link adaptation procedure and may force the gNB to schedule waveform switching (e.g., to DFT-s-OFDM) even if it is not needed in practice. Therefore, to minimize the impact on scheduling flexibility, multiple conditions can be combined.

Observation 10. Scheduling flexibility at the scheduler would be reduced significantly if only one condition is considered for implicit DCI-based signalling for dynamic waveform switching.

Observation 11. Multiple conditions can be combined to minimize the impact of implicit DCI-based signalling for dynamic waveform switching on scheduling flexibility at the scheduler.

Proposal 7. RAN1 to consider the possibility of combining several conditions in the discussions on implicit DCI-based signalling for dynamic waveform switching indication.

Waveform selection in coverage shortage:

The WI objective of specifying enhancements to support dynamic switching should not be limited only to the waveform indication aspect but it also covers enhancements to support gNB in the selection of correct waveform. Indeed, it is not sufficient to only measure the UL signal strength for selecting a waveform. In contrast, other criterion should be considered such as UE power boosting capability, e.g., via PHR. Although PHR can be a good information given the current specification, the granularity of PHR is however quite coarse. Therefore, PHR cannot precisely reflect the actual UE power boosting capability/power level, which is only known by the UE. In addition, PHR is determined using the current transmission settings and waveform, which may not be correct for a new transmission with a new waveform. Therefore, we see that power headroom reporting should be enhanced for assisting gNB on selecting a suitable waveform. For example, PHR assuming CP-OFDM and PHR assuming DFT-s-OFDM could both be reported by the UE.

Observation 12. PHR cannot precisely reflect the actual UE power boosting capability/power level, which is only known by the UE.

Observation 13. PHR is determined using the current transmission settings and waveform, which may not be correct for a new transmission with a new waveform.

Proposal 8. RAN1 to study enhancements for power headroom reporting for assisting gNB on selecting a suitable waveform.

# Conclusion

In this contribution we have discussed enhancements to support dynamic switching between DFT-s-OFDM and CP-OFDM in Rel-18. The following observations can be noted:

Observation 1. Given that the objectives of the WID also include possibility of specifying enhancements to reduce MPR/PAR, multiple variations of DFT-s-OFDM may be introduced. Therefore, means for indicating more than two waveform operation modes may be needed in the waveform switching after the study on MPR/PAR reduction is completed.

Observation 2. Although MAC-CE based signalling solution does not introduce DCI overhead (which may or may not be the case for DCI-based signalling solution, depending on the exact DCI-based signalling solution to be adopted), it has several disadvantages in terms of flexibility, latency, potential DL transmission overhead and efforts from multiple working groups compared to DCI-based signalling solution.

Observation 3. DCI-based signalling using UL DCI not only offers the flexibility for dynamically indicating waveform for each scheduled PUSCH, but also offers low switching latency for at least DG PUSCH.

Observation 4. DCI-based signalling using DL DCI not only offers lower latency for indicating waveform switching for CG PUSCH, but also avoid UL scheduling overhead in case there is no dynamic uplink transmission to be scheduled by UL DCI and there are DL transmissions to be scheduled by DL DCI.

Observation 5. Fallback DCI formats can have a smaller coding rate and hence being more reliable in coverage shortage scenario. However, since fallback DCI formats support only single-layer transmission, it may not be suitable for switching from DFT-s-OFDM to CP-OFDM where the latter can be used with rank greater than one.

Observation 6. For a given modulation order, the specified MPR values for edge/outer RB regions are higher than that for inner RB region for both DFT-s-OFDM and CP-OFDM, while the gap in terms of MPR in inner RB region between the two waveforms is smaller. This implies that the power gain offered by DFT-s-OFDM may be less significant, if not void, compared to CP-OFDM for inner RB region.

Observation 7. It is more relevant in views of coverage enhancements to consider at least the condition of RB regions and associated MPR for waveform switching, i.e., DFT-s-OFDM is used when the PUSCH is scheduled in the outer/edge regions and CP-OFDM is used when the PUSCH is scheduled in the inner RB region.

Observation 8. Power gain offered by DFT-s-OFDM is only needed if the UE cannot further boost its power, i.e., if the latest power headroom report is sufficiently small.

Observation 9. If a UE should switch to DFT-s-OFDM due to coverage shortage, the number of allocated RBs should be sufficiently small.

Observation 10. Scheduling flexibility at the scheduler would be reduced significantly if only one condition is considered for implicit DCI-based signalling for dynamic waveform switching.

Observation 11. Multiple conditions can be combined to minimize the impact of implicit DCI-based signalling for dynamic waveform switching on scheduling flexibility at the scheduler.

Observation 12. PHR cannot precisely reflect the actual UE power boosting capability/power level, which is only known by the UE.

Observation 13. PHR is determined using the current transmission settings and waveform, which may not be correct for a new transmission with a new waveform.

In addition, the following proposals were made:

Proposal 1. The scope of dynamic waveform switching in Rel-18 is limited to only for PUSCH. FFS: Msg3 PUSCH.

Proposal 2. The framework for dynamic switching between CP-OFDM and DFT-s-OFDM in Rel-18 should be easily extensible for also accommodating the switching among more than two waveform operation modes.

Proposal 3. RAN1 to prioritize DCI-based signalling solution for dynamic waveform switching in Rel-18.

Proposal 4. RAN1 to consider both DL and UL DCI for DCI-based signalling solution for dynamic waveform switching in Rel-18. At least the fallback DCI formats 0_0 and 1_0 should be supported. FFS: Other non-fallback DCI formats.

Proposal 5. RAN1 to consider solutions to mitigate DCI overhead increase and to reduce impact on the repurposed DCI field for explicit DCI-based signalling of dynamic waveform switching.

Proposal 6. RAN1 to further consider the following conditions for discussions on implicit DCI-based signalling for dynamic waveform switching indication:

- RB regions and associated MPR (i.e., DFT-s-OFDM is used when the PUSCH is scheduled in the outer/edge regions and CP-OFDM is used when the PUSCH is scheduled in the inner RB region),

- PHR limitation (i.e., DFT-s-OFDM is used when the latest PHR is sufficiently small, otherwise CP-OFDM),

- Total allocated RBs (i.e., DFT-s-OFDM is used when the total number of allocated RBs is sufficiently small, otherwise CP-OFDM).

Proposal 7. RAN1 to consider the possibility of combining several conditions in the discussions on implicit DCI-based signalling for dynamic waveform switching indication.

Proposal 8. RAN1 to study enhancements for power headroom reporting for assisting gNB on selecting a suitable waveform.

# References

- RP-221858 	"New WI: Further NR coverage enhancements", China Telecom (moderator), June 2022.

- R1-2109722	“Summary on Rel-17 TEI”, NTT DOCOMO (moderator), October 2021.