3GPP TSG RAN WG1 Meeting #110bis-e	R1-2210397

Electronic, Oct. 10th – Oct. 19th, 2022

Agenda item:	9.4.2

Source: 	Qualcomm Incorporated

Title: 	Co-Channel Coexistence between LTE SL and NR SL

Document for:	Discussion/Decision

# Introduction

The work item for Release-18 NR sidelink enhancement includes multiple objectives [1]:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#98-e)

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to FR1 licensed spectrum and ITS band in FR1.

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

- Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

- Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18)

- Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

- Work is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible.

  - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A

- UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4]

- The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI.

- The exact frequency band combinations for both FR1 licensed and ITS-dedicated spectrum are to be determined based on company input during the WI.

- Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements.

- Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

- UE RRM core requirement for the new features introduced in this WI [RAN4]

Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.

In this contribution we propose a design to achieve co-channel coexistence between NR Sidelink (SL) and LTE Sidelink based on dynamically sharing a common resource pool. The proposed design focuses on the coexistence of dual-RAT (Type A) devices with collocated NR SL and LTE SL modules and legacy (Type C) devices with only LTE SL modules in a shared resource pool operating outside network coverage (Mode 2 NR SL + Mode 4 LTE SL).

The rest of the paper is organized as follows. In Section 2, we discuss the proposed scheme for the cochannel coexistence of NR SL and LTE SL UE-s based on dynamically sharing the shared resource pool. We discuss the possible solution techniques to enable cochannel coexistence of NR Sidelink (SL) and LTE SL in Section 3 and propose a potential solution. Corresponding evaluation results are presented in Section 4.

# Dynamic Resource Pool Sharing by NR SL and LTE SL

Assigning a set of transmission resources for the use of NR SL is necessary to enable cochannel coexistence. For the dynamic resource pool (RP) sharing between NR SL and LTE SL, this needs to be achieved in a manner such that (a) there are no changes required for existing (legacy) LTE devices, and (b) the resource set can adapt to the current system configuration.

## Information shared between collocated NR SL and LTE SL modules

For a Type A device, each NR SL UE, based on its own sensing, and based on inputs from collocated LTE SL module over a shared interface, independently estimates the ratio of NR SL traffic compared to the total traffic in the system. An example of this can be the following technique for the NR SL device to estimate the LTE resource pool occupancy.

The collocated LTE SL module indicates to the NR SL module the resources over which it receives packets from other LTE SL UE-s as well as the resources over which the LTE SL module itself transmits. The LTE SL module may additionally filter the resources it indicates to the NR SL module based on the parameters associated with the LTE SL resource reservation, for example, the received signal strength (RSRP) associated with the decoded LTE SL control information, the priority associated with the transmission, etc. The sharing of the resource information may be achieved over a shared interface like the sharing of transmission resource information of Rel-16 in-device coexistence.

The NR SL module aggregates the resource information shared by the collocated LTE SL module over a window of time (Twin) to estimate the fraction of the total resource in the shared RP occupied by LTE SL transmissions. We term this fraction of resources occupied by LTE transmissions as the SL LTE resource pool (RP) occupancy metric. We note here that this time window (Twin) over which LTE SL reservation information is aggregated by the NR SL module should be large compared to traffic periodicity over NR SL and LTE SL. For example, Twin is in the order of a few seconds when traffic periodicity is in the order of a hundred milliseconds, e.g., 2 sec or 5 sec, so that an accurate estimate of the traffic over the shared RP may be obtained. The value of Twin may either be a part of NR UE (pre)-configuration or may be computed by the NR SL UE based on the traffic pattern over NR SL and LTE SL.

Next, we note that any resource within this time window Twin that is not indicated by the collocated LTE SL module may either be occupied by NR SL transmissions and hence excluded by LTE SL UE-s in the system based on LTE SL RSSI ranking or is not used either for NR or for LTE transmissions. A proportion of resources available for NR transmissions can then be directly computed from the estimation of the LTE SL RP occupancy. In one example, this can be computed by subtracting the SL LTE RP occupancy from one. It is also possible that other techniques to estimate the LTE SL resource pool occupancy, for instance using LTE SL channel busy ratio (CBR) measurements, etc., may be designed to achieve dynamic RP sharing.

Proposal 1: The LTE SL module of a Type A device shares the resource reservation information decoded from LTE SCI as well the resources selected/used for its own transmissions (meeting certain constraints of RSRP, priority, etc.) over the shared interface with the collocated NR SL module.

Proposal 2: The NR SL module estimates a SL LTE resource pool occupancy metric by aggregating the LTE SL resource reservation information over a time window (Twin), where this time window is large compared to the periodicities of the traffic over the system.

Proposal 3: The size of the time window (Twin) may either be a part of the NR SL UE (pre)-configuration or may be calculated at the NR SL module based on NR and LTE traffic patterns.

Observation 1: The LTE SL module of a Type A UE is not precluded from sharing other sensing information in addition to or instead of the LTE SL resource reservation information with the collocated NR SL module.

Another key point to note here is that as the proposed dynamic RP sharing scheme depends on the aggregation of the LTE SL resource reservation over a large time scale, there need not be a strict timeline imposed for the sharing of the LTE sensing and reservation information by the LTE SL module of a Type A UE. In fact, this can lead to a potential easing of the implementation complexity of the Type A UE.

## Determination of PSFCH resources for NR SL

Hybrid Automatic Repeat Request (HARQ) based retransmissions are a fundamental feature of NR SL. To enable the NR SL UE to select the available resource set, it is crucial to have a common understanding of PSFCH resource across NR SL UEs. To achieve this, each NR SL UE may be (pre-)configured with a basic NR transmission resource set for cochannel coexistence where this basic set includes the common PSFCH resources. This basic resource set is derived from and is a sub-set of the periodic PSFCH resources configured for NR transmissions over the shared resource pool. Moreover, the NR SL UEs will include all the resources in these PSFCH slots in the set of available resources for NR transmissions. If this constraint is not implemented, there is an increased chance of LTE SL transmissions colliding with NR feedback transmissions. This would degrade the performance of both NR SL PSFCH and LTE SL transmissions, affecting the AGC, and rendering either reception potentially undecodable.

For example, as shown in Figure 1, the NR resource pool configuration configures every fourth slot to contain resources for transmission of PSFCH feedback (ACK/NACK or NACK-only). A Rel-18 NR SL UE coexisting with LTE SL UE-s over the same channel is (pre)-configured with a basic NR Tx resource set defined over {n, n+1, …, n+19} slots, where n is a reference slot, and repeating every 20 slots. In this example, the basic NR Tx resource set includes the (n+11)-th and the (n+19)-th slots which contains resources for PSSCH+PSCCH and PSFCH

Figure 1: Basic NR Tx resource set consisting of a periodically repeating set of slots where one or more slots within this periodically repeating set is a PSFCH slot.

- NR resource pool configuration with 1 slot containing resources for PSFCH (in orange) every 4 slots. NR SL will use all the resources in this pool when it need not coexist with LTE SL.

- A periodically repeating basic NR transmission resource set derived from the NR resource pool configuration.

transmissions. Further, as Figure 1 demonstrates, this basic NR transmission resource set is repeater periodically for every set of 20 slots starting from the n–th, n+20–th, n+40–th,…, slots.

Proposal 4: NR SL UE-s are (pre-)configured with a periodically repeating basic set of available resources for NR transmissions which are comprised of one or more PSFCH occasions.

## Determination of the available NR Tx resource set

Next, the NR SL UE uses the estimated SL LTE resource pool occupancy metric and the (pre)-configured basic NR transmission resource set to determine a set of available transmission (Tx) resources for NR SL transmissions. The resources not included in this set are determined by the NR SL UE to be for LTE SL transmissions only and UE does not use these resources for its NR SL transmissions. Moreover, a NR SL UE preforming dynamic resource set determination to coexist with LTE SL on the same channel will receive on all slots even if they are not part of the NR SL transmission resource set. Release 18 Type A device will continuously monitor the system and update the estimation of the SL LTE resource pool occupancy based on current sensing results as discussed in Section 2.1. Each NR SL UE adapts to the changing network conditions by using this estimated SL LTE resource pool occupancy metric to update the set of resources available for NR SL transmission. More resources become available for NR SL transmission when the UE detects a decrease in the SL LTE resource pool occupancy metric and vice versa.

Proposal 5: The NR SL UE-s determine the set of available transmission (Tx) resources for NR SL transmissions based on the estimated LTE channel occupancy metric and the (pre)-configured basic NR Tx resource set.

No change in the behaviour of LTE SL devices is needed to support cochannel coexistence using the adaptive NR SL transmission resource set update. Nearby NR SL devices will have same or similar estimation of the SL LTE RP occupancy metric. Their transmissions will thus be localized in time (and frequency if/when resources are FDM-ed). This leads to certain slots having a high RSSI when sensed by a LTE SL device. The LTE SL UE will assign a lower rank to these resources (based on the Rel. 14 RSSI-based resource ranking mechanism) preventing the LTE SL transmissions from colliding with the NR SL transmissions.

Figure 2: Example of adaptive set of available resources, starting from a basic set of resources with PSFCH resources, for NR transmissions.

Observation 2: The proposed dynamic resource sharing between NR SL and LTE SL will not require any change to current LTE specifications

An example of the NR SL UE determined set of available resources for NR SL transmissions is shown in Figure 2. Initially, a NR SL UE may estimate the proportion of NR resources available in the network (based on SL LTE resource pool occupancy) to be 20%. The UE determines that, for every 20-slot period, resources in the four slots marked in blue/orange in Figure 2 are in the available resource set for NR transmissions, while none of the resources in the rest of the 15 slots (marked in green) are in the available set. Next, in a later instance, the UE determines that due to a change in the network conditions, the SL LTE resource pool occupancy has decreased implying that more resources are potentially available for NR SL transmissions. The UE now adds, in this example, additional resources to the previously computed available resource set for NR transmissions. A NR SL receiver uses the PSFCH occasion that satisfies the minimum PSFCH time gap condition to transmit a HARQ feedback (ACK/NAK or NAK only). Note that, it is beneficial for the system performance if the additional resources are added in a manner such that a balanced number of resources are mapped to each PSFCH occasions.

Proposal 6: The NR SL UE, based on the estimation of SL LTE resource pool occupancy metric periodically updates the set of available Tx resources by adding resources to, or removing resources from the current set of available transmission resources for NR SL.

# Type A UE behaviour

For the study of NR SL and LTE SL cochannel coexistence, RAN 1 focuses on UE-s that support both NR SL and LTE SL modules referred to as a Type A UE. Type A UE-s will need to transmit and receive both NR SL and LTE SL packets, hence RAN 1 will need to specify a mechanism to avoid or resolve conflicts between NR SL Tx and LTE SL Tx, between NR SL Rx and LTE SL Rx, and between NR SL Tx and LTE SL Rx. One potential solution is to re-use the in-device coexistence mechanism as detailed in [2]. This may be adequate when one RAT has a higher priority than the other. On the other hand, when both the RAT-s (NR and LTE) have the same priority or when for both the applications over NR and over LTE require sufficiently high reliability, re-using the in-device coexistence mechanism for cochannel transmissions is not a good design choice as the de-prioritized RAT may have a very high packet drop rate leading to loss in reliability [4].

Observation 3: Type A devices, which contains both NR SL and LTE SL modules, need to implement a mechanism to resolve inter-RAT Tx-Tx and Tx-Rx conflicts for co-channel coexistence.

Observation 4: Dropping of transmission and reception based on a priority associated with a RAT will severely degrade the reliability of the transmission over the lower priority RAT.

The Type A UE may use one of the following inter-RAT conflict mitigation techniques to avoid and/or resolve Tx-Tx and Tx-Rx conflicts between the collocated NR SL and LTE SL modules:

- (Option 1) LTE SL module excludes resources in the NR Tx resource set: The NR module of a Type A UE determines the set of NR transmission resources as detailed in Section 2. This set of resources is shared with the collocated LTE SL module of the Type A device. The LTE SL module can either exclude the resources in the NR SL Tx resource set from its own set of transmission resources or avoid selecting resources in the NR SL Tx resource set. The LTE SL UE can receive on all slots, regardless of whether they are included in the NR Tx resource set, unless the slot is used by the collocated NR SL module to transmit data or feedback. We note that, similar to the procedures enabling NR SL and LTE SL in-device coexistence [2], this proposed device behaviour will not require any changed to the LTE SL specifications.

- (Option 2) Type A UE uses half-duplex constraint with Tx prioritized over Rx: In this case, the Type A SL UE prioritizes transmissions (NR or LTE) over receptions, except for the reception of NR SL feedback over PSFCH. For a conflict between NR SL Tx and LTE SL Tx, the NR SL transmission is prioritized as the NR SL module already restricts its transmission resource set following the procedure discussed in Section 2. Further, as PSFCH transmission and reception is a fundamental feature of NR SL and ensures reliable communication over NR, for slots where NR PSFCH resources are scheduled (according to Section 2.2), LTE SL transmission and receptions are dropped in the case of a conflict. As an optimization, a UE may re-transmit the dropped packet with potential resource reselection.

Proposal 7: RAN 1 specifies technique for inter-RAT conflict avoidance and mitigation for Type A devices.

In Section 4, we will show the performance of the proposed dynamic RP sharing scheme with Type A UE-s implementing either Options 1 or Option 2.

# Evaluation Results

In this section we provide evaluation results for the techniques for NR SL and LTE SL cochannel coexistence discussed in Section 2. We consider the highway scenario in our analysis where the inter-vehicle distance is determined by UE dropping model A [3]. In this set up, a UE may be a Type A UE with both NR SL and LTE SL capabilities or a Type C UE which supports only LTE SL. Detailed simulation assumptions are provided in the Appendix. To quantify the performance of the proposed dynamic RP sharing scheme, the study presented in this section considers the following scenarios:

- Type A and Type C UE-s operating over the same channel and are (pre-)configured with a NR SL resource pool and an LTE SL resource pool with the resources optimally partitioned (based on the traffic load over NR SL and LTE SL) in a TDM manner with no overlap between the NR SL RP and the LTE SL RP, and

- Type A and Type C UE-s operate over a shared resource pool with Type A UE-s performing the dynamic resource pool sharing mechanism based on the resource information received from the collocated LTE SL module. Inter-RAT Tx-Tx and Tx-Rx conflicts for a Type A UE are resolved based on one of the two options presented in Section 3. Type C UE-s operate based on Rel-14 mechanism with no change in LTE configurations.

In Figure 3, we consider a system where NR SL devices are transmitting NR aperiodic traffic and LTE SL devices transmit periodic BSM traffic (Table 1). A NR SL device is collocated with an LTE SL device in a Type A UE.  We consider two system configurations (a) 200 Type A UE-s and 400 Type C UE-s, and (b) 300 Type A UE-s and 300 Type C UE-s. We compare the packet reception rate (PRR) for both NR SL and LTE SL transmissions. We observe that the performance of the dynamic RP sharing scheme when at a Type A UE, the LTE SL module excludes the NR transmission resource set from its set of transmission resources (option 1) is similar to the performance of an optimally configured resource pool partition. On the other hand, when the Type A UE resolves inter-RAT Tx-Tx and Tx-Rx conflicts by dropping transmissions or receptions based on half-duplex constraints (option 2), we observe a slight drop in the LTE PRR at close range (<1%) for both the system configurations. For the 95% PRR point we observe approximately a 5% decrease in the LTE range in a system with 200 Type A and 400 Type C UE-s when a Type A UE resolves conflicts based on inter-RAT conflict mitigation option 2 as compared to Type A UE-s implementing Option 1. In the system with 300 Type A and 300 Type C UE-s, the loss in the 95% PRR range by Type A UE-s

Figure 3: Performance of the proposed dynamic resource pool (RP) scheme with NR SL transmitting aperiodic traffic and LTE SL transmitting periodic BSM traffic in a system with (a) 200 Type A and 400 Type C UE-s (top panel), and (b) 300 Type A and 300 Type C UE (bottom panel).

implementing inter-RAT conflict resolution option 2 instead of Option 1 is around 10%.

Next, in Figure 4, we consider the case when NR SL devices transmit NR periodic traffic with a periodicity of 100 ms and the LTE SL devices transmit periodic BSM traffic. Unlike in the previous setup with aperiodic traffic over NR, the NR SL UE-s in this case use semi-persistent scheduling (SPS) based resource reservations. In this set up, we observe that the dynamic RP sharing scheme with Type A UE-s operating based on inter-RAT conflict mitigation option 1 performs as well as the optimally configured semi-static RP partition. When Type A UE-s operate based on inter-RAT conflict mitigation option 2, we observe that there is <1% loss in reliability in the close range for both the system configurations. On the other hand, considering the 95% PRR range, there is little or no loss from Type A UE-s resolving conflicts based on option 2 instead of option 1.

Moreover, we note that for both the scenarios discussed in Figure 3 and Figure 4, NR performance is not affected by the choice between option 1 and option 2 at Type A UE-s to resolve inter-RAT Tx-Tx and Tx-Rx conflicts. In fact, the proposed dynamic resource pool sharing scheme has a performance that is same as that of an optimally TDM based partitioned resource pool between NR SL and LTE SL.

Figure 4: Performance of the proposed dynamic resource pool (RP) scheme with NR SL transmitting periodic traffic and LTE SL transmitting periodic BSM traffic in a system with (a) 200 Type A and 400 Type C UE-s (top panel), and (b) 300 Type A and 300 Type C UE (bottom panel).

Moreover, we emphasize the fact that the proposed dynamic RP sharing mechanism between NR SL and LTE SL UE-s adapts to various network and traffic conditions without the need for any change in configuration.

Observation 5: The proposed dynamic resource pool sharing scheme achieves a balance between NR SL and LTE SL performance when both NR SL and LTE SL share a common resource pool.

Observation 6: The proposed dynamic resource sharing scheme adapts to changing system and traffic conditions without requiring any update to its configurations.

Observation 7: The proposed dynamic resource pool sharing scheme can achieve high reliability SL transmissions for both NR and LTE under a range of system and traffic conditions.

Observation 8: Avoiding inter-RAT Tx-Tx and Tx-Rx conflicts in a Type A UE using option 1, i.e., LTE SL module excluding the NR SL Tx resource set from its set of transmission resources, performs better than Type A UE using option 2, i.e., resolving conflicts based on half-duplex constraints with Tx prioritized over Rx.

Proposal 8: Specify mechanism for dynamic resource sharing between NR SL and LTE SL cochannel coexistence.

## Comparison of the alternatives for NR PSFCH configuration

Feedback transmissions over the PSFCH channel is a fundamental feature of NR SL operations and guarantees high reliability communication required by services over NR SL. In his section we analyse the following alternatives for configuring NR PSFCH transmissions over the resource pool dynamically shared with LTE SL:

- Alternative 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions.

  - Option 1: The PSCCH/PSSCH TX UE avoids selecting resources where the corresponding PSFCH occasion overlaps with the LTE SL transmission.

    - Description: For Alt. 1 Option 1, the NR SL UE receives resource reservation information from the LTE SL module (both received SCI and own Tx resources). During the resource (re)selection procedure, the NR SL module excludes, from the NR candidate resource set, resources which either has a conflict with the received LTE reservations or whose PSFCH occasion has a conflict with an LTE SL reservation.

  - Option 2: The PSCCH/PSSCH RX UE does not transmit feedback on the PSFCH occasion that overlaps with the LTE SL transmission.

    - Description: For Alt. 1 Option 2, the NR SL UE does not transmit feedback on a PSFCH occasion if it has received resource reservation information from the LTE SL module (both received SCI and own Tx resources) indicate LTE SL transmission on that slot.

- Alternative 2: NR SL UEs use a periodically repeating set of PSFCH slots (Section 2).

In Figure 5, we show the distribution of the subframes over which LTE SL UE-s in the system transmit data. In the case when Alternative 1 based techniques are implemented at the NR SL UE, we observe that the LTE SL UE-s do not back off from the PSFCH slots (at slot numbers 3, 7, 11, 15, and 19). Thus, LTE SL transmissions are equiprobable across all subframes. On the other hand, when the NR SL UE-s restrict the transmission of PSFCH feedback to a basic set of PSFCH transmissions slots (at slot number 3, 11, and 19) we observe (from Figure 5) that LTE SL UE-s avoids the PSFCH slots through RSSI measurements at these slots.

Observation 9: When Alternative 1 is used to avoid NR PSFCH conflict with LTE SL transmissions, the LTE transmissions are equiprobable on all slots.

Observation 10: When a basic set of periodically repeating PSFCH slots are used by NR SL UE, LTE SL UE-s can efficiently avoid transmitting on these slots.

Next, in Figure 6, we plot the performance (in terms of PRR) of Alternative 1, options 1 and 2 and Alternative 2 (as discussed in Section 2. For Alt. 1 with Option 1, we observe that as LTE SL reservations are equiprobable across all subframes/slots, avoiding LTE SL resources such that transmissions over PSSCH/PSCCH and PSFCH do not collide with LTE SL transmissions leads to an extremely high degree of over-exclusion of SL resources. This leads to a severe degradation of NR performance. On the other hand, a NR SL UE using Alt. 1 with Option 2 that drops the transmission of feedback over PSFCH will, based on the analysis in Figure 5, will find most of the PSFCH slots being occupied by LTE SL. Thus, the NR SL UE-s end up dropping most of the PSFCH feedback transmissions which, as shown in Figure 6, cannot achieve reliable communication.

Figure 5: Distribution of LTE transmissions with Alternative 1 and Alternative 2

Observation 11: LTE SL UE-s avoiding NR SL PSFCH resources (Alternative 2) provides a good balance between LTE SL and NR SL performance in the shared pool.

Figure 7: Performance of dynamic resource sharing mechanism with collisions between NR PSFCH transmissions and LTE SL transmissions avoided based on (a) proposed Alternative 2 based technique, (b) Alternative 1 option 1, and (c) Alternative 1 Option 2. Traffic over NR SL UE is aperiodic.

Observation 12: Avoiding NR SL PSFCH collisions with LTE SL transmissions with Alternative 1 leads to degradation of NR SL performance.

Proposal 9: Support NR SL UEs use a periodically repeating set of PSFCH slots (Alternative 2).

# Conclusions

Observation 1: The LTE SL module of a Type A UE is not precluded from sharing other sensing information in addition to or instead of the LTE SL resource reservation information with the collocated NR SL module.

Observation 2: The proposed dynamic resource sharing between NR SL and LTE SL will not require any change to current LTE specifications

Observation 3: Type A devices, which contains both NR SL and LTE SL modules, need to implement a mechanism to resolve inter-RAT Tx-Tx and Tx-Rx conflicts for co-channel coexistence.

Observation 4: Dropping of transmission and reception based on a priority associated with a RAT will severely degrade the reliability of the transmission over the lower priority RAT.

Observation 5: The proposed dynamic resource pool sharing scheme achieves a balance between NR SL and LTE SL performance when both NR SL and LTE SL share a common resource pool.

Observation 6: The proposed dynamic resource sharing scheme adapts to changing system and traffic conditions without requiring any update to its configurations.

Observation 7: The proposed dynamic resource pool sharing scheme can achieve high reliability SL transmissions for both NR and LTE under a range of system and traffic conditions.

Observation 8: Avoiding inter-RAT Tx-Tx and Tx-Rx conflicts in a Type A UE using option 1, i.e., LTE SL module excluding the NR SL Tx resource set from its set of transmission resources, performs better than Type A UE using option 2, i.e., resolving conflicts based on half-duplex constraints with Tx prioritized over Rx.

Observation 9: When Alternative 1 is used to avoid NR PSFCH conflict with LTE SL transmissions, the LTE transmissions are equiprobable on all slots.

Observation 10: When a basic set of periodically repeating PSFCH slots are used by NR SL UE, LTE SL UE-s can efficiently avoid transmitting on these slots.

Observation 11: LTE SL UE-s avoiding NR SL PSFCH resources (Alternative 2) provides a good balance between LTE SL and NR SL performance in the shared pool.

Observation 12: Avoiding NR SL PSFCH collisions with LTE SL transmissions with Alternative 1 leads to degradation of NR SL performance.

Proposal 1: The LTE SL module of a Type A device shares the resource reservation information decoded from LTE SCI as well the resources selected/used for its own transmissions (meeting certain constraints of RSRP, priority, etc.) over the shared interface with the collocated NR SL module.

Proposal 2: The NR SL module estimates a SL LTE resource pool occupancy metric by aggregating the LTE SL resource reservation information over a time window (Twin), where this time window is large compared to the periodicities of the traffic over the system.

Proposal 3: The size of the time window (Twin) may either be a part of the NR SL UE (pre)-configuration or may be calculated at the NR SL module based on NR and LTE traffic patterns.

Proposal 4: NR SL UE-s are (pre-)configured with a periodically repeating basic set of available resources for NR transmissions which are comprised of one or more PSFCH occasions.

Proposal 5: The NR SL UE-s determine the set of available transmission (Tx) resources for NR SL transmissions based on the estimated LTE channel occupancy metric and the (pre)-configured basic NR Tx resource set.

Proposal 6: The NR SL UE, based on the estimation of SL LTE resource pool occupancy metric periodically updates the set of available Tx resources by adding resources to, or removing resources from the current set of available transmission resources for NR SL.

Proposal 7: RAN 1 specifies technique for inter-RAT conflict avoidance and mitigation for Type A devices.

Proposal 8: Specify mechanism for dynamic resource sharing between NR SL and LTE SL cochannel coexistence.

Proposal 9: Support NR SL UEs use a periodically repeating set of PSFCH slots (Alternative 2).

# References

- RP-221938, “WID revision: NR sidelink evolution,” OPPO, RAN 97-e.

- TS 38.213, “5G NR: Physical layer procedures for control,” v17.2.0, 2022.

- Chair Notes, RAN1 #109-e, electronic meeting, May 2022.

- R1-2207235, “Co-Channel Coexistence between LTE SL and NR SL,” Qualcomm Incorporated, 3GPP TSG RAN WG1, Meeting #110, Toulouse, France, Aug. 2022.

# Appendix A

The simulation assumptions for the results presented in Section 4 are detailed in Table 1.

Table 1: Simulation Assumptions

| Sidelink Frequency | 6 GHz |
| --- | --- |
| Traffic model for NR SL | Periodic traffic: Inter-packet arrival time: 100 ms Packet size: 800 bytes with probability 0.8 and 1200 bytes with probability 0.2. Latency requirement: 50 ms. Aperiodic Traffic: Inter-packet arrival time: 50 ms + exponential random variable with mean 50 ms. Packet Size: Uniformly distributed between [200, 400, 600, 800, 1000, 1200, 1400, 1600, 1800, 2000] bytes. Latency Requirement: 50 ms. |
| Traffic model for LTE SL | Periodic Traffic: One 300-byte packet followed by four 190-byte packets. Packet generation periodicity: 100 ms |
| Simulation environment | Highway |
| UE drop and mobility | Highway: Option A (140Kmph) (as per TR 37.885) with 600 vehicles, corresponding to 800 and 900 devices total, over 20 Km. UE Dropping Model A [3] Inter-vehicle distance between adjacent devices in the same lane given as: max{2 meters, 4 sec * Exponential random variable with average of speed} |
| Number of Tx/Rx antenna elements | 1Tx/2Rx |
| Antenna model | Option 1 |
| Simulation bandwidth | 20 MHz |
| SCS | 15 kHz |
| Pathloss, shadowing, blocking and dual mobility models | Enabled (as per TR 37.885) |
| Number of retransmissions (NR) | 1 Initial Transmission + up to 3 HARQ retransmissions |
| Number of retransmissions (LTE) | 1 Initial + 1 Retransmission |
| Communication Mode (NR) | Groupcast Option 1 |
| Required Communication Range (NR) | 240 m |