3GPP TSG RAN WG1 #110bis-e			R1-2208355

e-Meeting, October 10th – 19th, 2022

3GPP TSG-RAN WG4 Meeting #104-e	R4-2215029

Electronic Meeting, August 15 – August 26, 2022

Title:	Reply LS on SSB measurement for L1-RSRP on inter-cell beam management

Response to:	R1-2205640

Release:	Release 17

Work Item:	NR_feMIMO-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Yiyan Zhang

E-mail Address:	yiyan.zhang@samsung.com

1. Overall Description:

RAN4 thanks RAN1 for the LS in R1-2205640. In RAN4 #104-e meetings, RAN4 has discussed scheduling restriction for L1-RSRP measurements in the inter-cell beam management scenario. Based on discussion, RAN4 would like to provide the following reply:

Question 1:

RAN1 would kindly like to ask RAN4 to provide feedback on whether there are any requirements that are related to UE measurements of L1-RSRP and reception of PDSCH/PDCCH on the same RE in FR1 for inter-cell beam management.

RAN4 Reply:

From RAN4 requirement perspective, for FR1,

- In RAN4 we do not define any scheduling restriction on the PDSCH/PDCCH due to the L1-RSRP measurement for inter-cell beam management, provided

  - L1-RSRP measurement performed on SSB which has the same SCS as the PDSCH/PDCCH; and

  - L1-RSRP measurement performed on SSB which has different SCS as PDSCH/PDCCH and UE support the capability simultaneousRxDataSSB-DiffNumerology.

- Otherwise, the UE is not expected to receive PDCCH/PDSCH on OFDM symbols overlapped with SSB configured as L1-RSRP measurement RS for inter-cell beam management.

There is no RAN4 requirement or scheduling restriction that is related to UE L1-RSRP measurement and reception of PDSCH/PDCCH on the same RE in FR1 for inter-cell beam management. RAN4 only considers the case that whether the SCS of SSB and PDCCH/PDSCH are the same or not and defines scheduling restriction on the PDCCH/PDSCH symbols overlapping with SSB.

In RAN4 common understanding, if SSB has the same SCS as PDSCH/PDCCH or UE supports the capability simultaneousRxDataSSB-DiffNumerology, UE may be able to measure on SSBs while receiving PDSCH/PDCCH on overlapped REs simultaneously in FR1, though decoding performance degradations and/or additional UE receiver complexities may be expected if SSB and PDCCH/PDSCH overlap on the same RE. It is a UE implementation issue, and RAN4 does not have any requirement on the RE overlapping case.

2. Actions:

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to take the above information into account for related discussion in RAN1.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #104-bis-e		October 10 – October 19		Electronic Meeting

RAN WG4 Meeting #105		November 14 – November 18		Canada, CA