3GPP TSG RAN WG1 #110bis-e 			R1- 2210545

e-Meeting, October 10th – 19th, 2022

Title:	Reply LS to RAN4 for further information on DMRS bundling

Response to:	R1-2205715(R4-2211225)

Release:	Rel-17

Work Items:	NR_cov_enh-Core

Source:	RAN1, MediaTek Inc.

To:	RAN4

CC:

Contact Person:

#### Name:	Tao Chen

E-mail Address:	<tao.chen@mediatek.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment:	  none

1. Overall Description:

RAN1 would like to thank RAN4 LS in R1-2205715(R4-2211225) to provide the latest status and the questions on NR Coverage Enhancement. RAN1 had the further discussion and would like to provide more information below for consideration in addition to the ones provided in R1-2208212:

For CA/DC/SUL cases listed in RAN4 LS as extracted below:

- DL CA with “additional” UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured)

- FR1 inter-band UL CA with DMRS bundling

- SUL with DMRS bundling

DMRS bundling can be applied with the following conditions additional to the ones listed in RAN4 LS:

- Concurrent transmissions scheduled/configured over multiple carriers are not expected by UE

- Only configuration of a single TAG

- Only applicable for the back-to-back case (i.e., zero gap between two transmissions within an actual TDW)

- Only one band can be configured with DMRS bundling at a time

Note 1: Under the above conditions, phase continuity and power consistency within any actual TDW on one carrier is not impacted by operations on a different carrier.

Note 2: Under the above conditions, the events defined in section 6.1.7 of TS38.214 for the carrier with DMRS bundling are not triggered by any transmission within any actual TDW on the other carrier.

Note 3: If the modulation scheme higher than QPSK is scheduled for transmission on any carrier configured with DMRS bundling, DMRS bundling is not applicable according to UE feature 30-4 (i.e., the error case and up to UE implementation)

Note 4: No RAN1 specification impact

2. Actions:

To RAN WG4

ACTION:

- RAN1 kindly asks RAN4 to take into account the conditions listed in RAN4 LS and the additional conditions above in RAN4 spec.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #111	14 November– 18 November 2022	Toulouse, FR

TSG RAN WG1 Meeting #112	27 February– 3 March 2023	Athens, GR