3GPP TSG-RAN WG1 Meeting #111	R1-2210879

Toulouse, France, November, 14th – 18th, 2022

Agenda Item:	9.14.1

Source:	Huawei, HiSilicon

Title:	Discussion on PRACH coverage enhancements

Document for:	Discussion and Decision

- Introduction

In RAN#110-bis-e [1], the agreement regarding PRACH coverage enhancement was approved, which is given as follows.

Agreement - For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt. - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt. Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains. Agreement For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options. - Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design. - FFS: RA-RNTI. - Option 2: Only one RAR window for all of the multiple PRACH transmissions. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

- Discussion on multiple transmission with same beam

  - Enhanced PRACH resource

    - PRACH enhancement level of SSB beam

In practice, the channel states of different beams usually are quite different as they are transmitted and received with different angles and go through different environment, including trees, buildings, and distance, which makes different beams suffer quite different coupling loss. In high-frequency situation, the above difference becomes more significant as the path number is reduced and shelter problem easily arises, the path loss and penetration loss become larger [2][4]. In other words, there is a significant difference of coupling loss for enhanced UEs within different SSB beams.

There are two examples from electromagnetic wave experiment to verify the different coupling loss of beams due to the outdoor to indoor penetration and the tree penetration. The simulation results in [3] show the path gains for all measurement locations in the multi-story, brick building and outside this building. The mean path losses for outdoor and indoor locations are 117 dB and 139.7 dB, respectively. Hence mean excess loss due to outdoor to indoor penetration is 22.7 dB. In single family unit case, the excess loss due to penetration is as high as 21 dB. The penetration loss is quite high, which makes the coupling loss of beams covering buildings much larger than that does not. The simulation results in [5] show that the penetration loss of millimeter wave passing through trees is 15.1-26.1dB, which varies with the state of canopy leaf, such as full leaf, out-of-leaf. In full leaf scenario, signals at 3.5, 5.4 GHz travel through trees with dense branches and leaves to reach UEs, resulting in 22.6, 26.1 dB loss, respectively. In out-of-leaf scenario, when 3.5, 5.4 GHz signals travel through trees with sparse branches and leaves to reach the UE, a loss of 7.5, 9.4 dB is observed, respectively. Moreover, the more vegetation passes through, the greater the loss is. Since the tree penetration loss is large and varies with the state of vegetation, it makes the coupling loss of beams covering the area surrounded by tree much larger than that does not. Based on above experiment results, we can see that different beams require different coverage enhancement.

Figure1: Illustration of ranking beams covering different area and assigning different RO number for repetition.

Observation 1: Different beams covering different areas have different coupling loss due to the outdoor to indoor penetration and the tree penetration, which implies that different beams require different coverage enhancements.

To guarantee low PRACH latency, the repetition ROs for one RACH attempt should not distributed over a long period of time. And, considering the TDD configuration in practice, the uplink time resource in one frame is finite to balance the data transmission loads of downlink and uplink services, where downlink service usually requires more time resource than that of uplink. In additional, it is really hard for BS to generate multiple analog beams at the same time due to equipment cost and technique constraints. It implies that the numbers of repetition ROs in time domain associated with different SSB beams are quite small under given PRACH transmission latency. Thus, the scarcity of TDM-RO resource is the main problem for PRACH coverage enhancement that relies on assigning repetition TDM-ROs. This problem could be worse with the increase of SSB beams, while FR2 requires finer and more beams to cope with the larger power loss and the coverage area reduction of beams, respectively.

Observation 2: The numbers of available repetition ROs in time domain (TDM-RO) associated with SSB beams could be quite small under an acceptable PRACH transmission latency. The scarcity of TDM-RO resource might be the main challenge to enhance PRACH coverage by repetitions over TDM-ROs, and this problem would be more serious for communication in higher frequency.

To deal with this problem, it is a good idea that the scarce resource, i.e., TDM-ROs, should be allocated to different SSB beams reasonably according to their coverage enhancement requirement. It implies that different SSB beams should be divided into different coverage performance enhancement levels where SSB beams in different levels own different number of repetition ROs.

Proposal 1: SSB beams should be classified into different coverage enhancement levels, where SSB beams belonging to the same level are associated with the same number of ROs for repetition but SSB beams belonging to different levels are associated with different number of ROs for repetition.

BS can easily classify SSB beams into two or more coverage enhancement levels by analyzing history observation result or advanced models. Based on coverage enhancement levels, BS can adaptively assign different number of ROs to different SSB beams for multiple transmission.

More specifically, the observation result includes the usage frequency of beam’s enhancement resource and repetition times. Analyzing the statistical observation result, BS can estimate the probability density functions (PDF) of statistic variables representing beam’s enhancement resource and repetition times to support the decision of PRACH configuration. For example, illustrated in Figure 1, BS can simply divide SSB beams into different levels by the average of observation results (any one or more of received preamble’s RSRP, repetition RACH times, RACH success probability or others) and assign different numbers of ROs to SSB beams with different levels for multiple transmission.

Observation 3: History observation results can be used by BS to categorize SSB beams into multiple coverage enhancement levels. For example, divide SSB beams into different levels by the average RSRP of received preamble or RACH success probability.

    - Relation of enhanced PRACH and legacy PRACH

Relation of enhanced PRACH resource and legacy PRACH resource should be considered carefully as it determines the implementation complexity of UE and BS in RACH procedure and affects the compatibility of legacy UE.

Specifically, when the new PRACH and legacy PRACH are overlapped, the different preamble set should be used to identify new and legacy UEs, which could increase the collision probability of UEs and degrade the RACH performance. Because, new UEs require more PRACH resource to finish RACH successfully, where more preambles might be assigned to new UEs to guarantee a desired collision performance. Moreover, the complexity of BS to detect the preambles of new and legacy UEs could increase at overlapped ROs. More importantly, using the legacy PRACH for multiple PRACH transmission would increase the latency of RACH greatly for new UEs as a new UEs require the number of ROs for repetition is multiple times of that required by legacy UEs.

Thus, to reduce the implementation complexity and the impact on RACH performance of legacy UEs, it seems better to make the enhanced and legacy PRACH resource independent from each other.

Observation 4: The enhanced PRACH resource should keep the backward compatibility of legacy UEs, should not degrade RACH performance of legacy UEs, and should not increase PRACH transmission latency of new UEs significantly.

Proposal 2: For enhanced multiple PRACH transmission, a separate PRACH configuration should be supported. In case of the configuration resulting in ROs overlapped with legacy ROs in time domain due to limited uplink time resources or configuration patterns, additional handling to have non-shared ROs should be studied.

    - PRACH resource assignment

- Location relation of repetition ROs

The location of repetition RO in enhanced PRACH is closely related to the transmission latency and the implementation complexity of UE’s preamble transmission and BS’s preamble detection. As illustrated in Figure 2, when the repetition ROs are located continuously in time domain, UE and BS can persistently transmit and detect preamble in time duration, which can reduce PRACH latency and complexity. While, when the repetition ROs are located discretely in time domain, UE and BS have to switch to different time duration to transmit and detect preamble, which increases PRACH latency and complexity.

Figure 2: Illustration of different assigning pattern of repetition ROs in time domain.

Observation 5: The continuous assignment of TDM-ROs for repetition can reduce latency and complexity.

Proposal 3: The configuration with continuous repetition ROs in time domain should be supported for low latency and complexity.

- SSB-RO pattern

According to TS 38.213, Section 8.1, the association relation between the RACH occasions and SS/PBCH block (SSB), termed as (SSB-RO) for PRACH transmission is specified by parameter ssb-perRACH-OccasionAndCB-PreamblesPerSSB, which indicates two information: (i) the number of SSB indexes per RO and (ii) the number of contention-based preambles per SSB index. Moreover, UE maps the SSB indexes to the time-frequency grid of ROs in increasing order of frequency resource indices, time resource indices of the ROs within a PRACH slots, and the PRACH slots, sequentially.

To determine the specific SSB-RO mapping, parameters prach-ConfigurationIndex and Msg1-FDM are also necessary, through which UE can derive a set of available ROs associated with a specific SS/PBCH block index. Specifically, the time-domain resources for ROs are RRC configured by prach-ConfigurationIndex (in rach-ConfigGeneric), which indicates a row of a table specified in TS 38.211 (clause 6.3.3.2). From this table, UEs can determine the preamble format for PRACH and find the ROs in time-domain. The frequency-domain resources for ROs are RRC configured by Msg1-FDM (in rach-ConfigGeneric), which indicates the number of ROs in frequency during the same time instance. From these parameters, UEs can determine the SSB-RO pattern and their locations in time-frequency-domain.

Figure 3 illustrates an example of SSB-RO pattern, wherein the prach-ConfigurationIndex is 177 in table 6.3.3.2-4, the SSB number is 4, and ssb-perRACH-OccasionAndCB-PreamblesPerSSB=1, and Msg1-FDM=1. With above parameter assumptions, UE obtains the following information:

Time domain location: ROs are allocated at the system frame numbers (n_SFN) that satisfy n_SFN mod 8 =1,2 (i.e. the first two frame for every 8 frames). Within each of the determined SFNs, ROs are allocated at slots (SCS=60 KHz) number 9,19,29 and 39. It should be noted that each slot of SCS=60 KHz equals to two slots of SCS=120 KHz. For SCS=120 KHz, within each of the determined slots of SCS=60 KHz, the remaining parameters in the considered row indicate that ROs will start at symbol number 0, 6, 14, 20 and occupy 6 symbols.

Frequency domain location: each time instance only has one FDM-RO in frequency.

SSB-RO mapping: each RO only associates with one SSB, and four SSBs are mapped to ROs in sequentially and periodically.

Figure 3: An example of RACH occasions and SSB-to-RO mapping pattern for UL/DL configuration DDSUU in FR2

As above discussion in subsection 2.2.1, we can use these green blocks in Figure 3 to add repetition ROs for PRACH coverage enhancement, which is independent from the legacy PRACH. Moreover, we also can use the uplink slots of other frames (such as the last six frames) within every 8 frames except the former two frames, to add repetition ROs for PRACH coverage enhancement. The possible SSB-RO pattern A and B are both illustrated in Figure 4, where PRACH assumptions are the same with that in Figure 3 and introduce the same number of repetition ROs for SSBs but in different SSB and RO mapping sequence. Form Figure 4, we can see that SSB-RO pattern A has the same SSB and RO mapping sequence with legacy PRACH but repetition ROs are distributed at discrete time instance. It implies nonconsecutive phase, large transmission latency and high transceiver implementation complexity, which could be worse when the number of SSBs becomes larger. While, SSB-RO pattern B has consecutive repetition ROs and low transmission latency at cost of increased signaling cost. More important, in this case, all UEs in four SSB beams can transmit preamble repeatedly as many as for 12 times during 10ms, which seems good enough for PRACH coverage performance. However, in FR2, the number of SSB is usually set to 16, which means that all UEs in 16 SSB beams can only transmit preamble repeatedly for 3 times at most during 10ms. It really not good enough for PRACH coverage performance enhancement, especially for communication system working in higher frequency that requires more SSB beams.

Figure 4: SSB-RO mapping pattern A and B introduce the same number of repetition ROs for SSBs but have different SSB and RO mapping sequence.

Observation 6: If the same number of ROs has to be assigned to all SSB beams, then the configured number of ROs is restricted to that required by the worst SSB beam. As a result, the uplink resource within a SSB configuration period could be not enough yet for PRACH coverage enhancement, especially for higher frequency bands that require large number of SSB beams and larger number of PRACH repetitions.

Based on observation 2 and 6, it is necessary to allocate the repetition ROs of new PRACH to SSBs according to their coverage enhancement levels that reveals the SSB beams’ coverage enhancement requirement. As illustrated in Figure 5, the repetition ROs generated in uplink slots are assigned to SSB 0-3 by coverage enhancement levels, where SSB 0 and SSB 1 belong to level 1, SSB 2 and SSB 3 belong to level 2, level 1 and 2 are associated with 4 and 2 repetition ROs, respectively.

Figure 5: SSB-RO mapping pattern C introduces repetition ROs for SSBs by coverage enhancement levels.

In this case, UEs in SSB beams belonging to level 1 and level 2 can transmit preamble repeatedly as many as for 16 and 8 times during 10ms, respectively. When the number of SSB is set to 16, UEs in SSB beams belonging to level 1 and level 2 can transmit preamble repeatedly for 4 times and 2 times at most during 10ms. It really increases repetition ROs for SSB beams in level 1 that is evaluated to require more coverage enhancement and not affects the coverage performance of SSB beams in level 2 that is evaluated to require less coverage enhancement. In other example, when the number of SSB is set to 16, if all SSB beams except SSB 0 belong to level 2, then UEs in SSB beam belonging to level 1 and level 2 can transmit preamble repeatedly for 16 times and 2 times at most during 10ms. SSB-RO pattern C is very useful for scenario where only several SSB beams could suffer severe path loss, as discussed in subsection 2.1.

Observation 7: SSB-RO pattern C as illustrated in Figure 5, can make the best use of the scarce TDM-RO resource for coverage enhancement by multiple PRACH transmissions over TDM-ROs, especially for high frequency bands that require large number of SSB beams and larger number of PRACH repetitions.

There are two methods to achieve the SSB-RO pattern C in Figure 5. The first method is similar to IAB-PRACH configuration to introduce new PRACH but introduce extra signaling to configure the SSB-RO pattern in new PRACH. The second method is similar to eMTC-PRACH configuration and configure PRACH by coverage enhancement level, where different levels are permitted to configure different PRACH parameters and different SSB-RO pattern.

Specifically, for the first method, determining the location of new PRACH for repetition requires the signaling used in IAB-PRACH to indicate the time offset and duration of new PRACH relative to legacy PRACH and the period of new PRACH. Moreover, to generate the SSB-RO pattern in Figure 5, a new signaling is needed to indicate the coverage enhancement level of SSBs and the corresponding number of repetition ROs, where the numbers of repetition ROs associated with SSBs belonging to the same coverage enhancement level are same. For the second method, the coverage enhancement level of SSBs and the common PRACH parameters should be specified first, and then indicate the time-frequency location and number of repetition ROs and SSB-RO pattern for each coverage enhancement level.

Proposal 4: SSB-RO pattern C as illustrated in Figure 5 should be supported by RRC configuration, where different SSB beams can be associated with different number of ROs for PRACH repetition.

- On issue of Overlapped ROs

In some cases, it is inevitable that the ROs of new PRACH are overlapped with that of legacy PRACH as we have discussed in subsection 2.1.1 due to the limited uplink resource.

To deal with the overlapped RO problem, most used method is using different preamble sets to divide legacy UEs and new UEs but it results in a severe degradation on legacy UEs’ RACH collision performance. To cope with this problem, the ROs of new PRACH can be offset by a certain frequency offset when the ROs of new PRACH are overlapped with that of legacy PRACH. In this case, the starting RB of ROs could be different at different time instances for multiple PRACH transmissions. While, for low complexity, it seems no need to allow the different starting RB of ROs at different time instances for multiple PRACH transmissions when new legacy is not overlapped with legacy PRACH.

Observation 8: It is beneficial to offset new PRACH resource in frequency domain when it is overlapped with legacy PRACH in time domain. In this case, the starting RB of ROs at different time instances for multiple PRACH transmissions is different.

Moreover, the multiple PRACH transmissions over FDM-ROs located in the same time instance seems no big benefit for coverage performance as it could not accumulate power from different TDM-ROs and the performance improvement from frequency diversity is small.

Observation 9: It seems no necessary to support multiple PRACH transmissions over FDM-ROs in the same time instance due to small benefit and high complexity.

  - Repetition scheme

    - Repetition scheme type

There are two types of repetition schemes for coverage enhancement, which are fixed repetition scheme and flexible repetition scheme.

For fixed repetition scheme, as illustrated in Figure 6 (a), all UEs (UE1, UE2, UE3) use the repetition ROs with same number, in enhanced PRACH resources to transmit preamble. Although the worst case occurs with small probability, to guarantee the certain successful probability of random access of the worst UE (UE1), the fixed repetition number would be set to a large value, i.e., 4. Actually, other UEs maybe only require a small repetition number, i.e., 2. It is clear that assigning too much resource to such a small probability event is not efficient. All UE repeatedly transmitting preamble for a large number of slots is also power-consuming. Moreover, it causes more interference to UEs in the neighborhood cells.

For flexible repetition scheme, as illustrated in Figure 6 (b), UEs are permitted to select repetition level, where the repetition times of level 1, 2 are equal to 2, 4 and UE1, UE2, UE3 select repetition level 2, 1, 1, respectively. For the enhanced ROs associated with the same beam, the preamble sets used in different repetition levels should be different and indicated by BS through SIB1 message, and UEs randomly select preamble from the preamble set corresponding to its repetition level.

The flexible repetition scheme can improve efficiency by permitting UEs opportunistically using resource according to their needs. The improvement reasons are discussed as follows. First, the PDF of required enhancement for UEs in different beams are significantly different naturally, especially for the large cell and the cell with complex electromagnetic environment, such as the different areas of cell are heterogeneously surrounded by trees and buildings. Specifically, when different areas of cell are heterogeneously surrounded by trees and buildings, the multi-path components of different beams undergo different propagation paths, which makes the observed path gains vary significantly. Second, even if the locations of UEs within the same beam are close to each other, they may require different repetition times to achieve the same coverage performance due to channel fading. Exploiting above difference of channel state of UEs, flexible repetition scheme that permits UE to select its repetition level can save resource and power.

Figure 6: Illustration of fixed repetition scheme and flexible repetition scheme.

Observation 10: It is more efficient to permit UEs to select repetition level according to their coverage performance requirement.

Proposal 5: For repetition ROs associated with the same SSB, multiple repetition levels can be supported by configuring different preamble sets.

- UEs select repetition level based on RSRP thresholds.

Proposal 6: Using different preambles in different PRACH transmissions during the multiple PRACH transmissions in one attempt are not necessary.

    - Repetition level determination in flexible repetition scheme

In flexible repetition scheme, the number of PRACH repetitions is autonomously selected at the UE based on its channel conditions (e.g., compare its RSRP measurement with RSRP thresholds to determine RSRP level). This approach was already adopted in specification of preamble repetitions for LTE, so a similar framework could be specified for NR. Specifically, BS would need to configure several RSRP thresholds for UEs to determine repetition level, which are mapped to a specific repetition level and its number of preamble repetitions. An example of such configuration has been shown in Figure 6, wherein BS configures two different RSRP thresholds to determine two repetition levels and one non-repetition level, level 1 and 2 associated to 2 and 4 repetitions, respectively.

The selected repetition level should be adjusted when transmission power exceeds the maximum transmission power in retransmission, since the RACH success probability is too low at the selected repetition level. Moreover, without increasing repetition level, enhanced UEs would persistently transmit preamble when RACH attempt fails until exceeds the maximum retransmission time, which might increase collision and interference. The RACH procedure that permits repetition level increasing is illustrated in Figure 7.

Figure 7: Illustration of repetition level determination and update in flexible repetition scheme.

Observation 11: It is beneficial to permit UEs to increase selected repetition level when its transmission power exceeds the maximum transmission power but still fails.

Proposal 7: The selected repetition level should increase in next retransmission when the transmission power of retransmission reaches the maximum transmission power.

  - Power control

The power control is important to interference control and power saving. The power control formula for multiple transmission with same beam has been specified in eMTC PRACH coverage enhancement. There are multiple power control formulas that are correspondingly used in different cases in eMTC. The calculation procedure for PRACH power control in eMTC is given as follows. The calculation procedure consists of three steps: calculating the receive target power, dividing the target power for multiple PRACH transmission and calculating the target power for one single transmission, and calculating the actual transmit power for one single PRACH. Specifically,

Step 1: Calculate the total receive target power.

PREAMBLE_RECEIVED_TARGET_POWER=preambleInitialReceivedTargetPower+DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep   (1)

where PREAMBLE_RECEIVED_TARGET_POWER is the total receive target power value, preambleInitialReceivedTargetPower is the initial power value of preamble indicated by BS, PREAMBLE_TRANSPORT_COUNTER represents the total transmission number times of preamble, powerRampingStep indicates the power increase step when RACH fails.

Step 2: Calculate the target receive power for one single transmission.

PREAMBLE_RECEIVED_TARGET_POWER=PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt)  (2)

where numRepetitionPerPreambleAttempt indicates the repetition transmission times of preamble. Note that PREAMBLE_RECEIVED_TARGET_POWER has been updated. Now, it represents the receive target power that needs to be reached for one single transmission.

Step 3: Calculate the transmission power for one single transmission.

P_PRACH = min{P_CMAX, PREAMBLE_RECEIVED_TARGET_POWER + PL_c} [dBm] (3)

where P_CMAX is the configured maximum transmit power at UE, and PL_c is the path loss. According to the above formulas (1)-(3), the transmission power of each single PRACH transmission can be obtained. It should be noted that the highest PRACH coverage enhancement level 3 in eMTC directly uses the maximum transmission power, P_CMAX, to transmit preamble.

Observation 12: The power control of multiple transmission specified in eMTC PRACH coverage enhancement can be starting point.

Proposal 8: The same estimation of path loss should be used in multiple PRACH transmission using the same beam pair during one RACH attempt, and the power ramping should not be used.

Enhanced UEs would persistently transmit preamble when RACH fails until achieving the maximum retransmission times, which might degrade the collision performance and cause interference. To avoid this interference, the power or power ramping could be changed for enhanced UEs. For example, when RACH fails, the value of power ramping can be increased or directly using the maximum power to retransmit preamble in next attempt. It is also beneficial to introduce other methods, including increasing the repetition number, setting frequency hopping, to assist the enhanced UEs failed last time to increase success probability, which can reduce the duration of interference lasting.

Observation 13: It is necessary to enhance PRACH retransmission by increasing power ramping and increasing repetition levels to improve success probability.

For few UEs in the extremely worst channel, above methods may be not able to improve their RACH performance, which causes long-term interference and resource wasting. In such case, limiting the maximum retransmission times for enhanced UE could be helpful to end this harmful lasting transmission.

Observation 14: The retransmissions of enhanced UEs using large repetition level may cause a serious and long-time interference for other cells.

Proposal 9: The methods to improve the success probability of next RACH attempt should be supported, such like increasing power ramping and increasing repetition level. To deal with the harmful retransmission, the maximum retransmission time of large repetition levels should be reduced.

  - RAR window and RA-RNTI

After UE transmit PRACH, UE tries to detect a PDCCH (DCI) with the corresponding RA-RNTI within the period of RAR-Window. The RAR-Window is configured by rar-WindowLength information element (IE) in a SIB message. If UE successfully decoded the PDCCH, it decodes PDSCH carrying RAR data. After decoding RAR, UE checked if random access preamble identifier (RAPID) in RAR matches its RAPID. For multiple PRACH transmissions, single RAR-Window is enough for RAR detection. RAR window should be designed as simple as possible for low complexity and UE power-saving.

RA-RNTI calculation should also be considered for multiple PRACH transmissions. The RA-RNTI associated with the RO in which preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

If PRACH repetitions transmit in different RO, then each of the PRACH transmission is associated with a different RA-RNTI. While, multiple RA-RNTIs seem unnecessary even if these repetition ROs are reused at different repetition levels, since the RAPIDs of UEs at different repetition levels are different. Single RA-RNTI can work well in the case that UEs at different repetition levels use the same repetition ROs to transmit different preambles with different multiple transmissions. This single RA-RNTI can be computed by a certain RO, such like the last repetition RO.

Observation 15: The simple design of RAR window and RA-RNTI is beneficial to low complexity.

Proposal 10: For enhanced UEs repeating PRACH during one RACH attempt, one RAR window starting from the last repetition RO is supported. RA-RNTI can be calculated based on one of transmitted repetition ROs, such like the last repetition RO.

- Discussion on multiple transmission with different beam

  - Coverage enhancement advantages and disadvantages

Multiple transmission with same beam is better than multiple transmission with different beams on coverage performance when channel is constant during multiple transmissions where UEs can use the best beam measured for multiple transmission. While, it might not happen when the best beam measured by UE is unprecise or the channel changes rapidly. In the case of rapid channel changes, multiple transmission with different beams can go through more paths and improve robustness performance naturally, compared to multiple transmission with same beam. However, the occurrence of above assumption is rare in practice, and it is not worth to change specification for a rare case.

Observation 16: Multiple transmission with the same beam is better than multiple transmission with different beams regarding coverage performance when channel condition is constant during multiple transmissions as UEs can use the best beam measured for multiple transmission.

Proposal 11: The standardization of multiple transmission with different beams should not be supported.

- Conclusion

With the discussion above, we have the following observations and proposals.

Observation 1: Different beams covering different areas have different coupling loss due to the outdoor to indoor penetration and the tree penetration, which implies that different beams require different coverage enhancements.

Observation 2: The numbers of available repetition ROs in time domain (TDM-RO) associated with SSB beams could be quite small under an acceptable PRACH transmission latency. The scarcity of TDM-RO resource might be the main challenge to enhance PRACH coverage by repetitions over TDM-ROs, and this problem would be more serious for communication in higher frequency.

Observation 3: History observation results can be used by BS to categorize SSB beams into multiple coverage enhancement levels. For example, divide SSB beams into different levels by the average RSRP of received preamble or RACH success probability.

Observation 4: The enhanced PRACH resource should keep the backward compatibility of legacy UEs, should not degrade RACH performance of legacy UEs, and should not increase PRACH transmission latency of new UEs significantly.

Observation 5: The continuous assignment of TDM-ROs for repetition can reduce latency and complexity.

Observation 6: If the same number of ROs has to be assigned to all SSB beams, then the configured number of ROs is restricted to that required by the worst SSB beam. As a result, the uplink resource within a SSB configuration period could be not enough yet for PRACH coverage enhancement, especially for higher frequency bands that require large number of SSB beams and larger number of PRACH repetitions.

Observation 7: SSB-RO pattern C as illustrated in Figure 5, can make the best use of the scarce TDM-RO resource for coverage enhancement by multiple PRACH transmissions over TDM-ROs, especially for high frequency bands that require large number of SSB beams and larger number of PRACH repetitions.

Observation 8: It is beneficial to offset new PRACH resource in frequency domain when it is overlapped with legacy PRACH in time domain. In this case, the starting RB of ROs at different time instances for multiple PRACH transmissions is different.

Observation 9: It seems no necessary to support multiple PRACH transmissions over FDM-ROs in the same time instance due to small benefit and high complexity.

Observation 10: It is more efficient to permit UEs to select repetition level according to their coverage performance requirement.

Observation 11: It is beneficial to permit UEs to increase selected repetition level when its transmission power exceeds the maximum transmission power but still fails.

Observation 12: The power control of multiple transmission specified in eMTC PRACH coverage enhancement can be starting point.

Observation 13: It is necessary to enhance PRACH retransmission by increasing power ramping and increasing repetition levels to improve success probability.

Observation 14: The retransmissions of enhanced UEs using large repetition level may cause a serious and long-time interference for other cells.

Observation 15: The simple design of RAR window and RA-RNTI is beneficial to low complexity.

Observation 16: Multiple transmission with the same beam is better than multiple transmission with different beams regarding coverage performance when channel condition is constant during multiple transmissions as UEs can use the best beam measured for multiple transmission.

Proposal 1: SSB beams should be classified into different coverage enhancement levels, where SSB beams belonging to the same level are associated with the same number of ROs for repetition but SSB beams belonging to different levels are associated with different number of ROs for repetition.

Proposal 2: For enhanced multiple PRACH transmission, a separate PRACH configuration should be supported. In case of the configuration resulting in ROs overlapped with legacy ROs in time domain due to limited uplink time resources or configuration patterns, additional handling to have non-shared ROs should be studied.

Proposal 3: The configuration with continuous repetition ROs in time domain should be supported for low latency and complexity.

Proposal 4: SSB-RO pattern C as illustrated in Figure 5 should be supported by RRC configuration, where different SSB beams can be associated with different number of ROs for PRACH repetition.

Proposal 5: For repetition ROs associated with the same SSB, multiple repetition levels can be supported by configuring different preamble sets.

- UEs select repetition level based on RSRP thresholds.

Proposal 6: Using different preambles in different PRACH transmissions during the multiple PRACH transmissions in one attempt are not necessary.

Proposal 7: The selected repetition level should increase in next retransmission when the transmission power of retransmission reaches the maximum transmission power.

Proposal 8: The same estimation of path loss should be used in multiple PRACH transmission using the same beam pair during one RACH attempt, and the power ramping should not be used.

Proposal 9: The methods to improve the success probability of next RACH attempt should be supported, such like increasing power ramping and increasing repetition level. To deal with the harmful retransmission, the maximum retransmission time of large repetition levels should be reduced.

Proposal 10: For enhanced UEs repeating PRACH during one RACH attempt, one RAR window starting from the last repetition RO is supported. RA-RNTI can be calculated based on one of transmitted repetition ROs, such like the last repetition RO.

Proposal 11: The standardization of multiple transmission with different beams should not be supported.

# References

- R1-2210660, “FL summary #4 of PRACH coverage enhancements”, China Telecom, 3GPP RAN1 #110bis-e, October 10th – 19th, 2022

- Samimi M K, Rappaport T S. Characterization of the 28 GHz millimeter-wave dense urban channel for future 5G mobile cellular[J]. NYU Wireless TR, 2014, 1.

- C. U. Bas et al., "Outdoor to Indoor Penetration Loss at 28 GHz for Fixed Wireless Access," 2018 IEEE International Conference on Communications (ICC), 2018, pp. 1-6, doi: 10.1109/ICC.2018.8422819.

- H. Zhao et al., “28 ghz millimeter wave cellular communication measurements for reflection and penetration loss in and around buildings in new york city,” in 2013 IEEE International Conference on Communications (ICC), June 2013, pp. 5163–5167

- Adegoke A S. Measurement of propagation loss in trees at SHF frequencies[D]. University of Leicester, 2015.