3GPP TSG-RAN WG1 Meeting #111 	R1-2210899

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.5

Source:	Huawei, HiSilicon

Title:	Remaining issues of Rel-17 positioning enhancements

Document for:	Discussion and decision

# Introduction

In this paper, we discuss the maintenance aspects for Rel-17 NR positioning on the following issues.

The number of RSRP reporting for DL-TDOA and Multi-RTT

# The number of RSRP reporting for DL-TDOA and Multi-RTT

In RAN1 #107[1], the DL PRS-RSRPP reporting for DL-TDOA and multi-RTT are agreed to mitigate impact of NLOS for timing-based positioning method.

Agreement - Support the LMF to request DL PRS-RSRPP together with timing measurement as part of DL-TDOA and multi-RTT reporting enhancements - Note: This applies to the first path and also to additional paths. - Support the LMF to request UL SRS-RSRPP together with timing measurement as part of UL-TDOA and multi-RTT reporting enhancements - Note: This applies to the first path and also to additional paths.

And the number of RSRP/first path RSRPP measurements reporting for DL-TDOA and Multi-RTT has been extended to 32 when the TEG feature is introduced, i.e. up to 4 PRS resources per Rx/RxTx TEG times 8 Rx/RxTxTEG. In the meantime, the maximum number of measurements reporting already included in TS37.355 6.5.10.4 and 6.5.12.4[2]:

6.5.10.4 NR DL-TDOA Location Information Elements – NR-DL-TDOA-SignalMeasurementInformation NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16 OPTIONAL, ..., [[ nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17) OPTIONAL, nr-DL-PRS-FirstPathRSRP-Result-r17 INTEGER (0..126) OPTIONAL, nr-los-nlos-Indicator-r17 CHOICE { perTRP-r17 LOS-NLOS-Indicator-r17, perResource-r17 LOS-NLOS-Indicator-r17 } OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL, nr-DL-TDOA-AdditionalMeasurementsExt-r17 NR-DL-TDOA-AdditionalMeasurementsExt-r17 OPTIONAL ]] } NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 …… 6.5.12.4 NR Multi-RTT Location Information Elements – NR-Multi-RTT-SignalMeasurementInformation NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-UE-RxTxTimeDiff-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-Multi-RTT-AdditionalMeasurements-r16 NR-Multi-RTT-AdditionalMeasurements-r16 OPTIONAL, ..., [[ nr-UE-RxTx-TEG-Info-r17 NR-UE-RxTx-TEG-Info-r17 OPTIONAL, nr-DL-PRS-FirstPathRSRP-Result-r17 INTEGER (0..126) OPTIONAL, nr-los-nlos-Indicator-r17 CHOICE { perTRP-r17 LOS-NLOS-Indicator-r17, perResource-r17 LOS-NLOS-Indicator-r17 } OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL, nr-Multi-RTT-AdditionalMeasurementsExt-r17 NR-Multi-RTT-AdditionalMeasurementsExt-r17 OPTIONAL ]] } NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16 NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16 … 6.6 Multiplicity and type constraint values … ... maxAddMeasTDOA-r17 INTEGER ::= 31 maxAddMeasAoD-r17 INTEGER ::= 23 maxAddMeasRTT-r17 INTEGER ::= 31 ...

However, in TS38.214, it is only mentioned that the UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP and/or up to 24 DL PRS-RSRPP measurements on DL PRS resources associated with the same dl-PRS-ID. No other texts explicitly explain the different of the maximum number of DL PRS-RSRPP/DL PRS-RSRPP reporting between DL-AOD and DL-TDOA/Multi-RTT. The UE may wrongly interpret that the upper limit of 24 also applies to DL-TDOA and Multi-RTT.

During the discussion in RAN1#110bis-e, companies expressed that there should be no such confusion from specification in the email discussion [3], however, we still think it is necessary to clarify that the up to 24 RSRP/RSRPP measurements only apply to DL-AoD positioning and the maximum number of RSRP/RSRPP per TRP is 32 for DL-TDOA and Multi-RTT.

- Proposal 1: Conclude that the maximum number of DL PRS-RSRPs and DL PRS-RSRPPs for the first path per TRP is 32 for DL-TDOA and Multi-RTT.

# Conclusion

In this contribution, we discuss the remaining issues of RSRP and RSRPP reporting for DL-TDOA and Multi-RTT. With the discussion, we have the following proposal for conclusion.

- Proposal 1: Conclude that the maximum number of DL PRS-RSRPs and DL PRS-RSRPPs for the first path per TRP is 32 for DL-TDOA and Multi-RTT.

# Reference

- RAN1, Chair’s Notes RAN1#107, RAN1#107, e-Meeting, 11th – 19th Nov., 2021.

- TS 37.355, LTE Positioning Protocol (LPP).

- R1-2210478, Feature Lead Summary #2 for maintenance on accuracy improvements for DL-AoD positioning solutions, Moderator (Ericsson), RAN1#110bis-e, 10th – 19th Oct., 2022