3GPP TSG-RAN WG1 Meeting #111	R1-2210985

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FDM unicast PDSCH and multicast PDSCH to TS38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022/10/31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There is a typo in G-GS-RNTI. - In the last RAN1 meeting, it was agreed that FG 33-3-2 does not support FDMed SPS, which was not captured in the current specification. Agreement: Add a note to FG 33-3-2 as below - Note: this FG does not support FDMed SPS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change ‘G-GS-RNTI’ to ‘G-CS-RNTI’. - If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled with C-RNTI/CS-RNTI and a PDSCH scheduled with G-RNTI for multicast/G-CS-RNTI that partially or fully overlap in time in non-overlapping PRBs except that both of the unicast PDSCH and multicast PDSCH are scheduled without a corresponding PDCCH transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is not specfied if network scheduled FDMed SPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, G-RNTI, MCCH-RNTI, G-CS-RNTI or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time.

< Unchanged parts are omitted >

The maximum number of PDSCHs scheduled per slot per component carrier with C-RNTI/CS-RNTI and G-RNTI/G-CS-RNTI/MCCH-RNTI that the UE shall be able to decode is the same as the indicated UE capability for the number of unicast PDSCHs per slot per component carrier. If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled with C-RNTI/CS-RNTI and a PDSCH scheduled with G-RNTI for multicast/G-CS-RNTI that partially or fully overlap in time in non-overlapping PRBs except that both of the unicast PDSCH and multicast PDSCH are scheduled without a corresponding PDCCH transmission. If the UE is capable of receiving FDMed unicast and broadcast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled with C-RNTI/CS-RNTI and a PDSCH scheduled with G-RNTI for broadcast/MCCH-RNTI that partially or fully overlap in time in non-overlapping PRBs.

If the UE is configured by higher layers to decode a PDCCH with its CRC scrambled by a CS-RNTI or G-CS-RNTI, the UE shall receive PDSCH transmissions without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration for those PDSCHs.

The UE it is not expected to support reception of FDMed MCCH PDSCH and MTCH PDSCH in PCell or SCell, or FDMed multiple MTCH PDSCHs in PCell or SCell, or FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell, or FDMed multicast PDSCHs in Pcell or Scell, or FDMed multicast PDSCH and MCCH/MTCH for broadcast in Pcell or Scell, or FDMed MCCH/MTCH/multicast PDSCH and paging PDSCH, or FDMed MCCH/MTCH/multicast PDSCH and SIB1 PDSCH that partially or fully overlap in time in non-overlapping PRBs.

< Unchanged parts are omitted >