3GPP TSG RAN WG1 #111	R1-2211085

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	9.7.1

Source: 	Fujitsu

Title:	Discussion on NW energy savings performance evaluation

Document for:	Discussion/Decision

Introduction

A study item on network energy savings for NR was approved in RAN#94 and is planned to be finished in RAN#98 [1]. In RAN1#109-e, RAN1#110 and RAN1#110b-e meetings, the base station energy consumption model and evaluation methodology were discussed and finalized. In this contribution, we provide evaluation results for time domain and spatial domain network energy techniques. Our views on network energy saving techniques are provided in companion contributions [2].

System Level Simulation Results

In this contribution, we give our simulation results of network energy saving techniques in time domain and spatial domain, based on the agreements made by RAN1 [3]. Detailed parameters are provided in appendix.

Evaluation results for time domain network energy saving techniques

The system level simulations are performed for the following cases. In the simulations, it is assumed that the base station goes into micro sleep for the symbols on which neither transmission nor reception is performed. The performance evaluation results of energy saving gain compared with the baseline scheme are summarized in Table I.

- Case 1: Baseline scheme, where adaptation of SSB/SIB1 is not performed

- Case 2: SSB/SIB1 adaptation scheme, where SSB and SIB1 are transmitted with the periodicity larger than 20 ms

- Case 3: SSB-less scheme where simplified SSB which contains SSS and PSS is transmitted with periodicity of 160 ms

Table I. Energy saving gain for SSB/SIB1 adaptation scheme and SSB-less scheme

| Reference configuration | System load | SSB/SIB1 transmission periodicity |  |  | SSB-less |
| --- | --- | --- | --- | --- | --- |
|  |  | 40 ms | 80 ms | 160 ms |  |
| Set 1 | zero | 21.8% | 36.8% | 45.7% | 52.7% |
| Set 2 |  | 18.2% | 27.4% | 31.9% | 36.1% |
| Set 1 | low | 13.7% | 20.6% | 24.1% | 27.0% |
| Set 2 |  | 12.5% | 18.8% | 22.0% | 24.6% |
| Set 1 | light | 11.1% | 16.6% | 19.4% | 21.7% |
| Set 2 |  | 9.4% | 14.1% | 16.5% | 18.4% |
| Set 1 | medium | 8.5% | 12.8% | 14.9% | 16.7% |
| Set 2 |  | 6.9% | 10.4% | 12.1% | 13.5% |

From the evaluation results in Table I, we could observe that:

- By increasing the SSB and SIB1 transmission periodicity, 18.2% ~ 45.7% network energy saving gain can be obtained for zero system load and 6.9% ~ 24.1% for non-zero system loads

- By SSB-less scheme, 31.6% ~ 52.7% network energy saving gain can be obtained for zero system load and 13.5% ~ 27% for non-zero system loads

Observation 1. By increasing the SSB and SIB1 transmission periodicity, 18.2% ~ 45.7% network energy saving gain can be obtained for zero system load and 6.9% ~ 24.1% for non-zero system loads.

Observation 2. By SSB-less scheme, 31.6% ~ 52.7% network energy saving gain can be obtained for zero system load and 13.5% ~ 27% for non-zero system loads.

Evaluation results for spatial domain network energy saving techniques

In this section we give our simulation results of network energy saving techniques in spatial domain, based on the agreements made by RAN1 [3].

We simulated the following two cases and evaluated the impact on energy consumption and UPT with different levels of traffic loads. More detailed parameters were provided in appendix.

- Case 1: Baseline scheme, where adaptation of TxRU is not performed.

- Case 2: Dynamic TxRU adaptation scheme, where gNB dynamically turns out half of the TxRUs if it finds out that all the DL data in the buffer is expected to be transmitted within 2 slots

The performance evaluation results of energy saving gain and UPT loss for dynamic TxRU adaptation are summarized in Table II. The UPT performance for reference configuration set 1 and set 2 with low, light and medium loads are shown in Figure 1 ~ Figure 6 in the appendix, respectively. From the evaluation results in Table II, we could observe that:

- By dynamic TxRU adaptation, 12.0% ~ 26.5% network energy saving gain can be obtained with UPT loss of 0.0% ~ 1.9% for 5% UPT and 0.3% ~ 5.7% for average UPT

- For the case with light system load in FDD, the reason for the improvement in 5% UPT is that the TxRU reduction also reduces interference for neighbor cells. But UPT loss can still be observed for average UPT.

Table II. Energy saving gain and UPT loss for dynamic TxRU scheme

| Reference configuration | System load | Resource utilization rate (RUR) | Energy consumption gain | 5% UPT loss | Average UPT loss |
| --- | --- | --- | --- | --- | --- |
| Set 1 | low | 11.1% | 24.0% | 0.0% | 5.7% |
| Set 2 |  | 14.8% | 26.5% | 1.9% | 0.6% |
| Set 1 | light | 24.8% | 18.6% | 0.4% | 4.1% |
| Set 2 |  | 28.5% | 20.0% | -0.4% | 0.5% |
| Set 1 | medium | 46.2% | 12.0% | 1.5% | 2.3% |
| Set 2 |  | 48.3% | 12.8% | 1.6% | 0.3% |

Observation 3. By dynamic TxRU adaptation, 12.0% ~ 26.5% network energy saving gain can be obtained with UPT loss of 0.0% ~ 1.9% for 5% UPT and 0.3% ~ 5.7% for average UPT

Conclusions

In this contribution, we provide performance evaluation results for spatial domain network energy saving technique. Based on the evaluation results, the following observation can be drawn.

Observation 1. By increasing the SSB and SIB1 transmission periodicity, 18.2% ~ 45.7% network energy saving gain can be obtained for zero system load and 6.9% ~ 24.1% for non-zero system loads.

Observation 2. By SSB-less scheme, 31.6% ~ 52.7% network energy saving gain can be obtained for zero system load and 13.5% ~ 27% for non-zero system loads.

Observation 3. By dynamic TxRU adaptation, 12.0% ~ 26.5% network energy saving gain can be obtained with UPT loss of 0.0% ~ 1.9% for 5% UPT and 0.3% ~ 5.7% for average UPT

- References

[1] RP-213554, “Study on network energy savings”, RAN#94e, Dec. 6-17, 2021

[2] R1-2211086, “Discussion on Network Energy Saving Techniques”, RAN1#111, Nov. 14-18, 2022

[3] R1-2210744, “Discussion Summary #5 for energy saving techniques of NW energy saving SI”, Oct. 10-19 2022

- Appendix

Table III. Simulation parameters for reference configuration set 1

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | 5GCM Uma, Indoor rate = 80% (v=3[km/h]), low-loss O2I penetration model |  |
| Cell layout | Hexagonal Grid, ISD=500m, 7*3 sector |  |
| Carrier frequency | 4.0 GHz |  |
| System bandwidth | 100MHz (272RB) |  |
| Subcarrier spacing | 30kHz |  |
| Antenna array configuration | Cell: (M, N, P, M_g, N_g; Mp, Np)=(8, 8, 2, 1, 1; 4, 8) User:(M, N, P, M_g, N_g)=(1, 2, 2, 1, 1) |  |
| Antenna spacing | (dH, dV) = (0.5, 0.8)λ |  |
| Transmission power | 55dBm |  |
| Traffic model | FTP model 3, packet size=0.5MB, mean inter-arrival time = 200ms |  |
| UE dropping | 4 / 8 / 13 UE per cell for low / light / medium load |  |
| Scheduler | Proportional Fairness |  |
| Number of layers | Rank adaptation (Max rank =4) |  |
| Number of maximum retransmit | 3 |  |
| Modulation | QPSK, 16QAM, 64QAM, 256 QAM |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| Codebook | Non-PMI transmission |  |
| DL CSI measurement | Precoded CSI-RS based |  |
| SRS transmission period | 40 ms |  |
| CSI feedback period and delay | Period = 20 ms, delay = 4 ms |  |
| Antenna adaptation delay | Immediate |  |
| Receiver / Channel estimation | MMSE-IRC / Non-ideal |  |
| SSB periodicity | 20 ms |  |
| SSB time resource | 2 SSB per slot, 4 symbols for each SSB, Case B for time-domain mapping |  |
| SSB frequency resource | 20 RB |  |
| SIB1 transmission periodicity | 20 ms, 1 SIB1 per SSB |  |
| SIB1 time resource | 1 slot |  |
| SIB1 frequency resource | 24 RB |  |
| BS category | Cat 2 |  |
| BS power consumption mode | Baseline: PstaticDL=P3, A=0.4, ηsf,sp=1 |  |

Table IV. Simulation parameters for reference configuration set 2

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | 5GCM Uma, Indoor rate = 80% (v=3[km/h]), low-loss O2I penetration model |  |
| Cell layout | Hexagonal Grid, ISD=500m, 7*3 sector |  |
| Carrier frequency | 2.1 GHz |  |
| System bandwidth | 20MHz (104RB) |  |
| Subcarrier spacing | 15kHz |  |
| Antenna array configuration | Cell: (M, N, P, M_g, N_g; Mp, Np)=(8, 8, 2, 1, 1; 2, 8) User:(M, N, P, M_g, N_g)=(1, 2, 2, 1, 1) |  |
| Antenna spacing | (dH, dV) = (0.5, 0.8)λ |  |
| Transmission power | 49dBm |  |
| Traffic model | FTP model 3, packet size=0.5MB, mean inter-arrival time = 200ms |  |
| UE dropping | 1 / 2 / 3 UE per cell for low / light / medium load |  |
| Scheduler | Proportional Fairness |  |
| Number of layers | Rank adaptation (Max rank =2) |  |
| Number of maximum retransmit | 3 |  |
| Modulation | QPSK, 16QAM, 64QAM, 256 QAM |  |
| Codebook | Type I codebook |  |
| DL CSI measurement | Non-precoded CSI-RS based |  |
| CSI feedback period and delay | Period = 20 ms, delay = 4 ms |  |
| Antenna adaptation delay | Immediate |  |
| Receiver / Channel estimation | MMSE-IRC / Non-ideal |  |
| SSB periodicity | 20 ms |  |
| SSB time resource | 2 SSB per slot, 4 symbols for each SSB, Case B for time-domain mapping |  |
| SSB frequency resource | 20 RB |  |
| SIB1 transmission periodicity | 20 ms, 1 SIB1 per SSB |  |
| SIB1 time resource | 1 slot |  |
| SIB1 frequency resource | 24 RB |  |
| BS category | Cat 2 |  |
| BS power consumption mode | Baseline: PstaticDL=P3, A=0.4, ηsf,sp=1 |  |

Figure 1. Comparison of UPT of dynamic TxRU adaptation and baseline for reference configuration set 1 with low system load

Figure 2. Comparison of UPT of dynamic TxRU adaptation and baseline for reference configuration set 1 with light system load

Figure 3. Comparison of UPT of dynamic TxRU adaptation and baseline for reference configuration set 1 with medium system load

Figure 4. Comparison of UPT of dynamic TxRU adaptation and baseline for reference configuration set 2 with low system load

Figure 5. Comparison of UPT and power consumption of dynamic TxRU adaptation and baseline for reference configuration set 2 with light system load

Figure 6. Comparison of UPT of dynamic TxRU adaptation and baseline for reference configuration set 2 with medium system load