3GPP TSG RAN WG1 Meeting #111	R1-2211089

Toulouse, France, November 14th – 18th, 2022

Source:	Fujitsu

Title:	Discussion on Dynamic switching between DFT-s-OFDM and CP-OFDM

Agenda Item:	9.14.3

Document for:	Discussion and Decision

# Introduction

In this contribution, we share our view on the open issues for dynamic switching between DFT-s-OFDM and CP-OFDM.

# Agreements in RAN1#110b-e

The agreements made in RAN1#110b-e are captured below for the reference [1].

Agreement Dynamic waveform switching enhancement in R18 is only applicable to PUSCH channel. Working Assumption Support at least one of the following options for the dynamic waveform indication in R18: Alt 1: Indication from an UL scheduling DCI - Alt 1-A: New field in scheduling DCI - Alt 1-B: Reuse existing field in scheduling DCI - Alt 1-B-1: Explicit indication by repurposing field, e.g. - Add one column to TDRA table - Add one column to MCS table(s) - Other solutions not precluded - Alt 1-B-2: Implicit determination from condition(s) on scheduling information, e.g. - RA type, MSB of RA - Number of RBs (below threshold or multiple of 2,3,5) - Location of RB allocation within carrier and the associated MPR - MCS below threshold - Number of PUSCH repetitions (or whether PUSCH repetition is used) and/or TBoMS - Number of DMRS CDM group(s) without data - Precoding information and number of layers - SRI - Condition over multiple types of scheduling information - Other types of scheduling information not precluded - Indicated waveform applies at least to the scheduled PUSCH transmission - FFS: Whether it also applies to subsequent transmissions, and of which type - FFS: DCI formats can contain the indication - FFS: Indication applies only if condition(s) are satisfied (e.g. PDCCH occasion, /RNTI, /Search space of the scheduling DCI, latest PHR reported by the UE, etc.) Alt 2: Indication from a non-UL scheduling DCI - FFS: DCI formats that can provide the indication (e.g. Downlink DCI, UE-group common DCI) - FFS: Types of subsequent transmissions to which indication is applicable Agreement To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as: - Reporting power headroom related information - Other solutions are not precluded Agreement Dynamic waveform switching enhancement in R18 is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1. - Note: The above does not imply that dynamic switching enhancement in R18 is applicable or not applicable to other cases of PUSCH (e.g. PUSCH transmission with a Type 1 or Type 2 configured grant, PUSCH scheduled by DCI format 0_0).

# Discussions

## Dynamic waveform indication

In the working assumption, two options are considered for the dynamic waveform indication, i.e., Alt 1: Indication from an UL scheduling DCI and Alt 2: Indication from a non-UL scheduling DCI. Alt 1 is a straightforward approach, and Alt 1-A and 1-B have their own benefit and drawback. For example, Alt 1-A is quite straightforward while the negative performance impact by the introduction of additional bits needs to be further investigated in RAN1. From our understanding, a new field with one bit wouldn’t deteriorate the system performance given the fact that PDCCH with DCI format 0_1/0_2 is not the bottleneck channel comparing to UL channels. On the other hand, Alt 1-B is preferable if Alt 1-A requires many additional bits in the DCI formats. However, repurposing the existing field may take the scheduler flexibility away, and hence careful discussion in RAN1 is necessary. Especially, we have a concern that so many potential solutions (as in Alt 1-B-2) suddenly show up in RAN1 meeting, and our precious time is consumed for clarification and down-selection discussions. Our preference is to preclude the solutions that are associated with other functionalities, e.g., PUSCH repetition and/or TBoMS, in order to avoid an unnecessary dependency among UE features.

Observation 1:

- Alt 1 is a good approach for RAN1 to move to the next step discussion

  - For Alt 1-A, necessary number of bits for new field should be clarified

  - For Alt 1-B, unnecessary dependency among UE features should be avoided

On the other hand, Alt 2 requires more complicated handling of DCI and waveform, which brings less benefit to both UE and gNB. For example, we don’t see any benefit to use DL DCI to switch the UL waveform because there might be a negative impact to DL transmission. We don’t think this is a good system design. In addition, we don’t see any necessity to be “UE-common” for this functionality since only a part of UEs (e.g., for UE suffering from lack of coverage) is needed to switch waveform. Also, from gNB perspective, UE-group common DCI requires additional monitoring of PDCCH, which is requiring additional complexity for the gNB if it does not use group common DCI at all. Furthermore, if non-UL DCI used for UL waveform switching, we should consider additional technique to avoid the negative impact of false alarm/DTX, which just make the system design complicated. In summary, no benefit can be seen for Alt 2 over Alt 1.

Observation 2:

- No benefit is foreseen for Alt 2.

Given the above analysis, we would propose the following:

Proposal 1:

- Confirm the working assumption on the dynamic waveform indication, and Only Alt 1 (Indication from an UL scheduling DCI) should be supported in Rel-18

## Enhancements to assist the scheduler in determining waveform SW

In our view, the use cases for using DFT-S-OFDM instead of CP-OFDM are: (1) the UE is under power limited situation, (2) the channel condition does not offer any throughput gain by higher rank transmission, and (3) the scheduling gain by non-contiguous resource allocation (by CP-OFDM) is limited. The necessary information related to (1) (2) and (3) can be obtained by the existing mechanisms, such as PHR, SRS and DMRS. Therefore, we don’t see the strong necessity to introduce further enhancements, and the following proposal can be made:

Proposal 2:

- Existing mechanism of power headroom reporting is sufficient to assist the scheduler in determining waveform switching

# Conclusion

In this contribution, we provide our views on dynamic switching between DFT-s-OFDM and CP-OFDM as below.

Proposal 1:

- Confirm the working assumption on the dynamic waveform indication, and Only Alt 1 (Indication from an UL scheduling DCI) should be supported in Rel-18

Proposal 2:

- Existing mechanism of power headroom reporting is sufficient to assist the scheduler in determining waveform switching

# References

- Chair’s note RAN1 #110b-e meeting