3GPP TSG RAN WG1 #111			R1-2211299

Toulouse, France, November 14th – 18th, 2022

Title:	Draft reply LS on further questions on feMIMO RRC parameters

Response to: 	R1-2210808 (R2-2211028)

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	3GPP TSG-RAN WG1(Lenovo)

To:	3GPP TSG-RAN WG2

Contact Person:

#### Name:

E-mail Address:

- Answers on questions from RAN2

Question 1

According to above RAN2 understandings, RAN2 would like to ask RAN1 to explain what the association of BWP and cell ID for srs-TCIState is.

Answer: This field is configured to indicate the DL or joint TCI state, or the UL TCI state the SRS resource follows. Only the ID of the TCI state is configured here. The TCI-StateID or TCI-UL-State-ID-r17 points to the actual TCI-State or TCI-UL-State-r17 state, where the ServCellIndex and BWP-Id of the reference RS is defined. It’s sufficient to determine the reference RS ID for SRS transmission. When the ServCellIndex and BWP-Id is not configured in the TCI-State or TCI-UL-State -r17, the configured reference RS is on the same serving cell and the same BWP as the target SRS.

Question 2

RAN2 would like to ask RAN1

- Is there a need to add the parameter codebookMode which selects mode1 or mode2 in the IE CodebookConfig-r17?

- If a) is needed, which types of codebook the parameter is needed, for example:

  - typeI-SinglePanel-Group1-r17

  - typeI-SinglePanel-Group2-r17

  - typeII-PortSelection-r17

- Additionally, if a) is needed, for each type of codebook that needs a parameter codebookMode, please confirm if a separate parameter is needed or if they share the same configuration of codebookMode.

Answer:

a) Yes, an additional parameter codebookMode is needed for CodebookConfig-r17

b) The parameter codebookMode is needed for both typeI-SinglePanel-Group1-r17 and typeI-SinglePanel-Group2-r17 codebook groups

c) Both typeI-SinglePanel-Group1-r17 and typeI-SinglePanel-Group2-r17 codebook groups share the same configuration of codebookMode

Question 3

RAN2 assumes RAN1 informs RAN2 in case the above described pathloss reference signal configuration for unified TCI state is not what RAN1 has intended.

Answer: RAN1 confirms RAN2’s understanding is correct.

2. Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #111 	November 2022    Toulouse

TSG-RAN WG2 Meeting #112	February 2023      Athens