3GPP TSG RAN WG1 Meeting #111					                       R1-2211587

Toulouse, France, Nov. 14th – 18th, 2022

Agenda Item:  	9.1.2

Source:             	Fraunhofer IIS, Fraunhofer HHI

Title:                  	CSI enhancements for medium UE velocities and coherent JT

Document for: 	Decision

# Introduction

This contribution discusses issues related to Rel. 18 codebook enhancements for medium/high speed UEs and coherent JT based on the agreements made in RAN1#110-bis-e [1] and prioritized for RAN1#111.

# CSI enhancements for medium velocity UEs

5G NR codebooks that have been specified until Rel. 17 are mainly for pedestrian UEs scenarios. For fast moving UEs, the channel between the BS and the UE changes rapidly. As a result, Doppler shift and Doppler spread increases resulting in a reduced channel coherence time which in turn results in a drastic performance loss when Rel-15-Rel.-17 Type II codebooks are used. One way to overcome this problem is to increase the CSI update rate via more frequent CSI reporting and measurements. However, this has the disadvantage that enormous DL and UL resources are utilized and results in high complexity at the UE due to increased number of channel measurements and PMI calculations.

One way to overcome this problem is to exploit Doppler-domain information of the channel in the CSI report that allows to predict the future channel behavior. In [2], time-delay spectrum and delay-Doppler spectrum for different UE mobility scenarios are shown. By observing the channel variations in both time-delay spectrum and delay-Doppler spectrum, it is shown that the channel remains invariant for a longer time interval in the delay-Doppler domain compared to the time-delay domain. The time interval over which the channel remains constant in the delay-Doppler spectrum is several folds higher than the coherence time. Therefore, by incorporating the Doppler-domain information in Type II CBs, the need for frequent CSI updates can be alleviated as the delay-Doppler spectrum remains invariant for a longer time interval.

Regarding the work scope of Type-II codebook refinement for high/medium velocities, enhancements on Rel. 17 FeType-II PS codebook should be supported as the workload is small as only delay reciprocity is assumed, and Doppler reciprocity is not assumed. In our view, only some of the parameter combinations need some refinement as all other codebook aspects are straightforward extensions.

Proposal 1: Support Rel. 18 Type-II codebook enhancements on Rel. 17 FeType-II PS codebook as the workload is minimal as all codebook aspects are straight forward extensions.

The Type II Rel. 16/17 codebook structure is given by W=W1W2WfH, where W1 is an NT ×2L matrix comprising 2L spatial domain DFT basis vectors, W2 is a 2L ×M matrix comprising up to 2LM non-zero precoder coefficients, and Wf is a N3×M matrix comprising M frequency domain DFT  basis vectors. Each precoder coefficient is associated with an angle-delay pair. The 2L spatial domain DFT basis vectors are selected from a 2D-DFT matrix of size Nt/2×O1O2Nt/2 and the M delay domain DFT basis vectors are selected from a DFT matrix of size N3×N3, where N3 is the number of sub-bands. Using Rel. 16/17 Type II CB structure as a baseline, the Doppler information can simply be incorporated by extending the precoder equation to the time domain. For this, an additional codebook component is used for determining the Doppler components. The extended codebook or precoder can be expressed as a function of W1, W2, Wf and Wd. Here, W1 is a NT ×2L matrix comprising 2L spatial domain DFT basis vectors, Wf is a N3×M matrix comprising M frequency domain DFT basis vectors, Wd is a N4×Q matrix comprising N time domain basis vectors, and W2 is a 2L ×M ×Q matrix comprising 2LMQ precoder coefficients. Here, N4 refers to the number of time units for which the PMI is reported.

Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, study the supported values for Q from (but not limited to) the following candidates, in conjunction with the supported values of N4 and DD units: - Alt1. Q is determined as a function of N4, e.g., Q=2 for N4=2, and Q=ceil(N4/2) for N4>2 - Alt2. Q is selected from multiple candidate values, e.g., {2, 3, 4, …,} (or a subset thereof, e.g. {2, 3}), the maximum value is FFS Alt3. Only single value is supported, e.g. Q=2 only or Q=4 only

According to the agreement, the enhanced Type II CB is expressed as W=W1W2Wf⨂WdHand the DD components are selected commonly for all SD and FD components. However, from our evaluations, each beamformed channel/SD component is associated with a different Doppler spread/shift and hence the dominant DD components for each SD-FD pair are different. Moreover, we also observed that one or two dominant DD components per SD component seems to be sufficient to achieve a reasonable performance gain compared to the legacy Rel. 16 codebook. Taking the union of all dominant DD components selected across all SD and FD components results in a minimum of Q=3 DD components. Therefore, in addition to Q=2, larger values of Q need to be supported. Table 1 shows the cell averaged throughput gain for different values of commonly selected Q DD components across all SD components. It can be clearly observed that for different WCSI values, the throughput gain of the enhanced Type II CB increases with increasing number of DD components.

Table 1: Throughput gain [%] of the Rel. 18 CB over the baseline for different values of  WCSI  and different number of DD components for fixed non-zero precoder coefficient K = 24.

|  | Q = 2 |  | Q = 3 |  |
| --- | --- | --- | --- | --- |
| Speed | WCSI=20 | WCSI=40 | WCSI=20 | WCSI=40 |
| 20 Kmph | 5.6% | 4.8% | 6.8% | 5.7% |
| 60 Kmph | 3.4% | 2.9% | 4.3% | 3.6% |

Observation 1: The throughput gain of the enhanced Type II CB increases with increasing number of DD components for different WCSI values.

Proposal 2: Support one another value of Q in addition to Q = 2.

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling: - For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H - For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH - Only Q (denoting the number of selected DD basis vectors) >1 is allowed - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - FFS: Whether Q is RRC-configured or reported by the UE Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, down-select from the following alternatives (by RAN1#111) for the orthogonal DFT DD basis: - Alt1. No rotation factor - Alt2. A rotation factor is selected for each SD basis vector - FFS: Supported values of rotation factor Note: At least two companies opine that Alt2 is not aligned either with the agreement in RAN1#110bis-e or WID objective #1

Prediction is an important step that decides the performance of the Rel. 18 codebook for medium/high speed UEs. For a significant performance gain compared to the legacy, the Doppler components need to be captured at least with a moderate resolution. An oversampled DFT basis significantly increases the accuracy of the prediction step and results in a reduction of the number of selected Doppler components which in turn reduces the number of non-zero precoding coefficients. Based on our evaluations, we observed an increasing gain with increasing oversampling factors. For reasonable performance gains compared to the baseline, we observed that an oversampling factor of four is sufficient for the DD basis. When oversampling is considered for the DD basis, the size of the codebook increases from N4 to N4O4 and hence the feedback overhead to report the selected DD basis vectors increases as it is conditioned on N4O4 instead of N4. To keep the overhead reasonable, one method is to report the selected rotation factors per SD component. As mentioned before, the channel associated with each SD component experiences a different Doppler shift. It has been observed that the DD components associated with different SD components are associated with different orthogonal subgroups i.e., different rotation factors. Therefore, to reduce feedback overhead, the rotation factors for each SD component can be indicated separately followed by the DD component indication conditioned on N4.

According to the agreements made in RAN1#110-bis-e, DD basis components are commonly selected for all SD/FD components from an orthogonal DFT basis. However, the use of a rotation factor for each SD component is yet to be decided. For an oversampling factor O4, there exists O4 orthogonal DFT matrices of size N4 comprising indices {0,…, N4-1}. For the use of a rotation factor, the agreement supports two possibilities 1) Orthogonal DFT basis set associated with the same rotation factor for all SD components and 2) Orthogonal DFT basis set associated with different rotation factors for each SD component. The rotation factor can be incorporated for each SD component using the following steps. In the first step, UE selects Q DD components commonly across all SD/FD components from {0,…, N4-1} DD components. In the second step, the selected Q DD components of each beam are rotated with respect to a rotation factor. The second step can be seen as an additional step performed by the UE on top of step 1. Note that the DD basis index from the first step is not changed due to the addition of the rotation factor in the second step. For example, for N4=4 and Q=2, if the selected DD basis indices are {0,2}, the DD basis indices remain the same after step 2. Note that this is one way of incorporating the rotation factor for each SD component. Table 2 shows the cell averaged throughput gain using different rotation factors. It can be observed that the throughput gains increase with increasing rotation factor.

Table 2: Throughput gain [%] of the Rel. 18 CB over the baseline for different values of WCSI  for Q= 3 common DD components for different rotation factors for K = 24 non-zero precoder coefficients.

|  | Rotation factor - 1 | Rotation factor – 2 | Rotation factor - 4 |
| --- | --- | --- | --- |
| Speed | WCSI=20 | WCSI=20 | WCSI=20 |
| 20 Kmph | 6.8% | 9.1% | 12.2% |
| 60 Kmph | 4.3% | 5.8% | 8.1% |

Observation 2: The throughput gains increase with increasing rotation factors.

Observation 3: A rotation factor of 2 seems to be sufficient to achieve a significant gain compared to the legacy Rel. 16 codebook.

Proposal 3: Support a single rotation factor of 2 for the Rel. 18 DD basis.

Agreement For the Type-II codebook refinement for high/medium velocities, down-select from the following alternatives: - Alt1. Q different 2-dimentional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimentional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. - Alt2. A DD-basis-common per-layer 2-dimensional bitmap for indicating the location of NZCs used in Rel-16/17 Type-II is used - This implies that for each layer, the location of NZCs in SD-FD is common across all the Q selected DD basis vectors FFS: Further overhead reduction on bitmap(s). FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector.

In the following the different alternatives are compared and some enhancements are discussed.

Alt 1: In Rel. 16 Type II CBs, the non-zero coefficient selection is indicated to the gNB via a 2LM-sized bitmap. For the enhanced Rel. 18 Type II CBs, it is natural to consider a 2LM-sized bitmap for each DD component. Compared to the legacy CB, the size of the bitmap increases by Q-fold for Q DD components resulting in a 2LMQ-sized bitmap represented by Alt 1 (see Figure 1 which is a representation of the 2D-bitmap of Alt1).

Alt 2: Alt 2 on the other hand supports a DD-basis-common bitmap for the indication of the non-zero coefficients. The term “DD-basis-common bitmap” implies that the non-zero coefficient selection is common across all DD components. For Rel. 18 Type II CB, the channel is “compressed” in the angle-delay-Doppler domain. Hence, the channel is sparse. This means the main energy of the precoder coefficients is concentrated only in few SD-FD-DD pairs and not in all 2LMQ SD-FD-DD pairs. Instead of reporting a 2LMQ-sized bitmap, the channel/precoder

Figure 1: 2-dimensional bitmap of size 2LMQ (Alt 1) comprising MQ = 12 FD-DD pairs when M = 4 and Q = 3.

sparsity in the angle-delay-Doppler domain should be exploited to reduce the feedback overhead. There are several methods to exploit this channel/precoder sparsity and Alt 2 is one of such methods exploiting sparsity, however, only in the SD-FD domain and not in the TD domain. In our understanding, for Alt 2 the UE selects T SD-FD pairs out of the 2LM SD-FD pairs and hence a first 2LM-sized bitmap is needed to indicate the T SD-FD pairs. The value of T is selected by the UE. As the SD-FD pairs are common for all DD components, the indication of the non-zero coefficients for the Q DD components is provided by a second bitmap of size TQ bits. As these two bitmaps are reported in CSI part 2, the size of CSI part 2 varies with the value of T. Therefore, the value of T  needs to be additionally reported in CSI part 1 per layer using a log2(2LM)-bit indicator. Hence, the total feedback for non-zero coefficient indication for Alt 2 is given by 2LM+TQ+log2(2LM) bits per layer. The main drawback of Alt 2 is that the feedback overhead heavily depends on the number of selected SD-FD pairs T. Selecting a small value of T to reduce the feedback overhead results in throughput degradation, whereas selecting a large value of T results in similar feedback overhead to that of Alt 1. The throughput gains and feedback overhead for different values of T are shown in Table 4. Moreover, as the sparsity is exploited only in the angle-delay domain, restricting the number of SD-FD pair selection for achieving overhead reduction results in a further restriction of the SD component selection which can be main reason for the throughput degradation.

Observation 4: Alt 2 exploits channel sparsity only in the angle-delay domain.

Observation 5: Exploiting sparsity in the angle-delay domain introduces further restriction on the SD components on which the performance is heavily dependent on.

Figure 2: High level representation of Alt 3 for M=4, Q=3, and R=4.

Table 3: Feedback overhead in bits for bitmap reporting for Alt 1, Alt 2 and Alt 3.

|  | Alt 1 | Alt 2 | Alt 3 |
| --- | --- | --- | --- |
| Overhead bits for indication of the location of non-zero coefficients per layer | 2LMQ | 2LM+TQ | 2LR |
| Indication of selected number of SD-FD pairs among 2LM SD-FD pairs |  | log2(2LM) | R = M is fixed and not reported. |
| Indication of FD components and DD components for the R selected FD-DD pairs |  |  | 2LR+R(M+Q) or Rlog2M+Rlog2Q |
| Total feedback overhead | 2LMQ | 2LM+TQ+log2(2LM) | 2LR+R(M+Q) or 2LR+Rlog2M+Rlog2Q |

In the following, we discuss one potential feedback overhead reduction scheme that exploits the sparsity in the delay-Doppler domain without sacrificing the performance and achieving a significant feedback reduction. For clarity, we call this scheme as Alt 3.

Alt 3: According to the agreement, the FD and DD components are commonly selected across all SD components. Therefore, for M FD components and Q DD components, there are in total MQ FD-DD component pairs which are common across all SD components (see Figure 1 which is a representation of the 2D-bitmap of Alt1). Each column of the bitmap is associated with an FD-DD component pair. As mentioned before, an important observation is that the energy of each SD component/beamformed channel is only associated with very few (either one or two) dominant FD-TD component pairs and not with all MQ FD-DD component pairs. As the FD-DD components pairs are commonly selected across all SD components, for the number, R, of dominant common FD-DD pairs, it holds that R≪MQ. Hence, the feedback overhead can be greatly reduced when reporting only the dominant R FD-DD pairs associated with the precoder. The overall feedback overhead of Alt 3 is depicted in Figure 2 and summarized in Table 3. For Alt3, R common FD-DD pairs from MQ FD-DD pairs across all 2L SD components are selected. Each FD-DD pair is associated with an FD component and DD component m,q, where m=0,…,M-1 and q=0,…,Q-1. The selected R common FD-DD pairs m0,q0,…,(mR-1,qR-1) are indicated in the CSI report. For this, an M-sized bitmap indicating the selected FD component and a Q-sized bitmap indicating the selected DD component per FD-DD pair is reported. This results in a total of R(M+Q) bits for the indication of the R selected FD-DD pairs. Alternatively, for each FD-DD pair log2M+log2Q bits can be used which results in a total feedback of R(log2M+log2Q ) bits for the R FD-DD pairs. The non-zero coefficients are indicated by a reduced-size bitmap of size 2LR. The total feedback overhead for Alt3 results is 2LR+R(M+Q) or 2LR+Rlog2M+Rlog2Q bits.

Special case of Alt3:

Based on our evaluations, we observed that R = M pairs are sufficient to obtain a similar throughput to that of Alt 1 (see Table 4). Therefore, only M FD-TD pairs are considered for the reporting of the non-zero coefficient selection. As all FD components are present in the M FD-DD pairs, the association of FD component to each FD-DD pair is implicitly given by the FD component indication. However, the association of DD components to each FD-DD pair needs to be reported in addition via a Q-sized bitmap resulting in a total of MQ bits for M FD-DD pairs. The feedback overhead of Alt1, Alt2 and Alt 3 is summarized in Table 3.

Table 4 shows the cell averaged throughput gain of Alt 1, Alt 2 (T SD-FD pairs) and Alt 3 (with M FD-DD pairs). It can be observed that for smaller values of T i.e., 8 SD-FD pairs, Alt 2 results in a 1.5% throughput loss compared to Alt1, whereas for higher values of T, i.e., 16 and 24 pairs, the achieved throughput gain is closer to that of Alt 1. Although Alt 2 results in overhead reduction for smaller number of SD-FD pairs values, for increasing SD-FD pairs, the feedback overhead becomes closer or more than that of Alt 1 especially when T≥22 SD-FD pairs. Alt 3 on the other hand results in a massive feedback reduction of 56 bits with only a minor loss compared to that of Alt 1. Therefore, considering the throughput and feedback overhead trade-off, Alt 3 shall be studied further as a potential alternative before taking a decision.

Table 4: Throughput gain and feedback overhead for Alt 1, Alt 2 and Alt 3 for L = 4, M = 4, Q = 3 for K = 24 non-zero precoder coefficients.

|  | Alt 1 | Alt 2 (T = 8 SD-FD pairs) | Alt 2 (T = 16 SD-FD pairs) | Alt 2 (T = 24 SD-FD pairs) | Alt 3 (R= M = 4 FD-DD pairs) |
| --- | --- | --- | --- | --- | --- |
| Overhead bits for bitmap reporting | 96 | 61 | 85 | 109 | 2LM + RQ = 40 |
| Throughput gain UE speed – 20 Kmph | 6.8% | 5.3% | 6.4% | 6.6% | 6.3% |
| Throughput gain UE speed – 60 Kmph | 4.3% | 2.9% | 3.6% | 4.2% | 4.1% |

Observation 6: For smaller number of SD-FD pairs, Alt 2 results in a throughput loss of 1.5% compared to Alt 1 with a feedback overhead reduction of 35 bits.

Observation 7: For larger number of SD-FD pairs. Alt 2 results in a similar throughput to that of Alt 1, however with only a minor feedback reduction of 7 bits for 16 SD-FD pairs and an overhead increase of 13 bits for 24 SD-FD pairs.

Proposal 4: Decision shall be taken by comparing Alt 1 and Alt 2 with other potential feedback reduction schemes instead of simply choosing between the legacy bitmap scheme (Alt 1) with no feedback reduction and Alt 2 with small feedback reduction.

Proposal 5: Include the following alternative for further study.

Alt 3: Single 2-dimensional bitmap of size 2LR (similar as in R16) for indicating the location of the NZCs, and a single bitmap of size RQ to report the association of each DD component to each FD component. As a special case, consider R=M.

Regarding the number of non-zero coefficients (K), in order to exploit the sparsity of the channel/precoder in the angle-delay-Doppler domain, the non-zero coefficient selection shall be performed on all DD basis components jointly i.e., on all 2LMQ precoder coefficients and not per DD basis component. We see no meaningful reason as to why the non-zero coefficient selection must be performed on the 2LM coefficients associated with each DD basis component separately.

Proposal 6: Support non-zero coefficient selection across all DD basis components jointly i.e., on all 2LMQ precoder coefficients.

# Rel-16/17 Type-II codebook refinement for C-JT targeting FDD

In RAN1#109-e, it was agreed to specify Type-II codebook refinements for coherent joint transmission (CJT) multi-TRP (mTRP). For CJT mTRP, multiple geographically separated TRPs or RRHs are assumed to be well synchronized in time and frequency as well as the phase and amplitude of their antenna arrays are calibrated, so that the UE can coherently combine the data streams or multiple layers simultaneously transmitted from the TRPs/RRHs. In the following, based on the agreements from RAN1#110-bis-e and offline discussion, additional details related to codebook structure and CSI reporting for Type-II CJT mTRP are discussed.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

For coherent joint transmission, it is the common understanding that the precoders for all TRPs are calculated jointly. In this case, the strength of the channels between each TRP and UE is reflected in the power of the precoder coefficients. As the UE selects K non-zero strongest coefficients jointly across all TRP precoders, weaker precoder coefficients are anyway discarded. Increasing the number of pol-specific amplitudes from 2 to 2N increases the resolution of the weaker coefficients. However, we do not expect any meaningful gain by increasing the resolution of few weaker precoder coefficients among the K selected non-zero precoder coefficients. Therefore, for simplicity and keeping the specification complexity reasonable, we prefer only one quantization scheme to be supported for the Type-II codebook refinement for CJT mTRP.

Proposal 7: For simplicity, support Alt 1.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding the bitmap(s) for indicating the locations of NZCs, down-select from the following alternatives for the size of the bitmap for CSI-RS resource n (Bn) (by RAN1#111): - Alt1. Analogous to legacy, Bn =2LnMv,n (Mv,n=Mv for mode 2) - Alt2. Non-rectangular bitmap, i.e., NZC bitmap allowing different lengths for different SD/FD basis vectors. - TBD: How to determine the lengths for different SD/FD basis vectors

In Alt 1, the selected non-zero precoder coefficient location is indicated to the gNB by a 2LnMv,n length bitmap, where Ln and Mv,n are the number of SD and FD components, respectively, per layer and TRP, whereas in Alt 2, the selected non-zero precoder coefficient location is indicated to the gNB by a reduced length bitmap of size less than 2LnMv,n bits. The length of the bitmap is reduced by using different length bitmaps for the SD/FD basis. However, it is not clear on what basis the length of the bitmap for each SD/FD basis is determined and how much overhead reduction is achieved without degrading the performance. Therefore, for simplicity, we prefer Alt1.

Proposal 8: Support Alt1 i.e., legacy bitmap Bn= 2LnMv,n  for the reporting of the non-zero coefficient location.

Offline proposal 1.D.2: On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources from a gNB-configured set of FD basis candidates - Alt2. Wf,n independently selected across N CSI-RS resources from a gNB-configured set of FD basis candidates (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources from a gNB-configured set of FD basis candidates. For all the above alternatives, the legacy FD basis selection indication scheme (combinatorial-based for N3≤19, window-based for N3>19) is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

For the Type-II codebook refinement for CJT mTRP, two modes have been agreed in RAN1#110. The only difference between the two modes is the selection of the FD basis. For Mode 1, the FD basis selection is performed per TRP separately and hence NTRP FD indicators need to be reported per layer, whereas for Mode 2, the FD basis selection is performed across all TRPs jointly and hence a single FD indicator is used per layer. In order to reduce the feedback overhead associated with the FD basis indication for Mode 1, three alternatives have been agreed by the companies for further study and down-selection.

One way of reducing the feedback overhead is to align the PDP of each TRP with respect to a reference TRP (see Figure 4). By doing so, the selected FD basis for each TRP will be concentrated in a few FD components N3'. If N3' is configured such that N3' is always less than N3, the FD basis reporting will be conditioned on N3' instead of N3 and hence feedback overhead reduction can be achieved. Alt 1 and Alt 3 support the use of a per-CSI-RS resource FD basis offset with respect to a reference TRP and support the FD basis selection from a gNB configured set of FD basis N3'.  The only difference between the two alternatives is the FD basis selection per TRP. For Alt1, the FD basis after cyclic shifting the PDPs with respect to a reference TRP is commonly selected for all TRPs, whereas for Alt 2, the FD basis after cyclic shifting the PDPs with respect to a reference TRP is independently selected. However, as the PDPs for the TRPs are different, using a common FD basis for all TRPs as in Alt 1 may results in performance degradation.  Alt 2 on the other hand does not support the use of a per-CSI-RS-resource FD basis offset as the FD basis selection is performed from a gNB configured FD basis set. Such an FD basis selection from a restricted FD basis N3' without aligning the PDPs of cooperating TRPs may result in a performance degradation.

Observation 8: Assuming the gNB configured FD basis set size N3' is less than N3, Alt 2 results in a performance degradation when not using per-CSI-RS-resource FD basis offset.

Observation 9: As the FD bases of the cooperating TRPs are not identical, selecting common FD basis for all TRPs as in Alt 1 may result in performance degradation compared to Alt 3.

Proposal 9: Support Alt 3.

Figure 3: Alignment of FD basis of TRP 2 and TRP 3 with respect to TRP 1 to obtain a reduced size window of size N3'.

# Conclusions

Based on the above discussion, we have the following observations and proposals.

Proposal 1: Support Rel. 18 Type-II codebook enhancements on Rel. 17 FeType-II PS codebook as the workload is minimal as all codebook aspects are straight forward extensions.

Observation 1: The throughput gain of the enhanced Type II CB increases with increasing number of DD components for different WCSI values.

Proposal 2: Support one another value of Q in addition to Q = 2.

Observation 2: The throughput gains increase with increasing rotation factors.

Observation 3: A rotation factor of 2 seems to be sufficient to achieve a significant gain compared to the legacy Rel. 16 codebook.

Proposal 3: Support a single rotation factor of 2 for the Rel. 18 DD basis.

Observation 4: Alt 2 exploits channel sparsity only in the angle-delay domain.

Observation 5: Exploiting sparsity in the angle-delay domain introduces further restriction on the SD components on which the performance is heavily dependent on.

Observation 6: For smaller number of SD-FD pairs, Alt 2 results in a throughput loss of 1.5% compared to Alt 1 with a feedback overhead reduction of 35 bits.

Observation 7: For larger number of SD-FD pairs. Alt 2 results in a similar throughput to that of Alt 1, however with only a minor feedback reduction of 7 bits for 16 SD-FD pairs and an overhead increase of 13 bits for 24 SD-FD pairs.

Proposal 4: Decision shall be taken by comparing Alt 1 and Alt 2 with other potential feedback reduction schemes instead of simply choosing between the legacy bitmap scheme (Alt 1) with no feedback reduction and Alt 2 with small feedback reduction.

Proposal 5: Include the following alternative for further study.

Alt 3: Single 2-dimensional bitmap of size 2LR (similar as in R16) for indicating the location of the NZCs, and a single bitmap of size RQ to report the association of each DD component to each FD component. As a special case, consider R=M.

Proposal 6: Support non-zero coefficient selection across all DD basis components jointly i.e., on all 2LMQ precoder coefficients.

Proposal 7: For simplicity, support Alt 1.

Proposal 8: Support Alt1 i.e., legacy bitmap Bn= 2LnMv,n  for the reporting of the non-zero coefficient location.

Observation 8: Assuming the gNB configured FD basis set size N3' is less than N3, Alt 2 results in a performance degradation when not using per-CSI-RS-resource FD basis offset.

Observation 9: As the FD bases of the cooperating TRPs are not identical, selecting common FD basis for all TRPs as in Alt 1 may result in performance degradation compared to Alt 3.

Proposal 9: Support Alt 3.

# References

| [1] | “Chairman’s Notes 3GPP RAN1 #110bis-e,” October 10th– 19th, 2022. |
| --- | --- |

Table 5: Simulation parameters

| - Parameter |  | - Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | - Rel. 18 Type-II Doppler |  | - Rel. 18 Type-II CJT |
| - Duplex, Waveform |  | - FDD, OFDM |  |  |
| - Multiple access |  | - OFDMA |  |  |
| - Scenario |  | - Outdoor2 - - 1 TRP per sector, 3 sectors per site, 19 sites - - Number of TRPs (NTRPs) – 2,3 - - Both inter- and intra-site selection of TRPs - - Urban Macro |  | - - Single TRP - - Urban Macro for 20, 60 Kmph - Mobility model – Spatial consistency procedure A with 50m decorrelation distance from TS 38.901 - Urban Macro |
| - ISD |  | - 500m |  | - 200m |
| - Frequency range |  | - FR1 only, 2 GHz |  |  |
| - Channel generation |  | - According to the TR 38.901 |  |  |
|  |  | - Difference in propagation delays between UE and NTRP TRPs is considered in the composite Channel Impulse Response (CIR) for CJT. |  |  |
| - Antenna setup and port layouts at gNB |  | - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |  |  |
|  |  | - Total #ports = NTRP×32 |  |  |
| - Antenna setup and port layouts at UE |  | - 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |  |  |
| - BS Tx power |  | - 44dBm |  |  |
| - BS antenna height |  | - 25m |  |  |
| - UE antenna height & gain |  | - Follow TR36.873 |  |  |
| - UE receiver noise figure |  | - 9dB |  |  |
| - Modulation |  | - Up to 256QAM |  |  |
| - Coding on PDSCH |  | LDPC - Max code-block size=8448bit |  |  |
| - Numerology | - Slot/non-slot | - 14 OFDM symbol slot |  |  |
|  | - SCS | - 15kHz |  |  |
| - Simulation bandwidth |  | - 10 MHz |  |  |
| - Frame structure |  | - Slot Format 0 (all downlink) for all slots |  |  |
| - UE distribution |  | - 100% outdoor (20 Km/h) |  |  |
| - UE receiver |  | - MMSE-IRC |  |  |
| - MIMO scheme |  | - SU/MU-MIMO with rank adaption |  |  |
| - MIMO layers |  | - 2 |  |  |
| - CSI feedback |  | - CSI feedback periodicity: 5 ms - Scheduling delay: 4 ms | - CSI feedback periodicity for R16 (baseline): 5 ms - CSI feedback periodicity: W ms - W = (20, 40) |  |
| - UE distribution |  | - 80% indoor (3 Km/h), 20% outdoor (30 km/h) | - 100% outdoor |  |
| - Feedback assumption |  | - Realistic |  |  |
| - Channel estimation |  | - Realistic |  |  |
| - Measure |  | - Cell-averaged Throughput |  |  |