3GPP TSG RAN WG1 #111	                                    		           		R1-2211843

Toulouse, France, Nov 14 – Nov 18, 2022

Agenda Item:	9.10.2

Source:	InterDigital, Inc.

Title:	Discussion on XR-specific capacity enhancements techniques

Document for:	Discussion and Decision

# Introduction

The list of agreements and conclusions made during RAN1#110bis-e [1] are provided in Annex A. In the summary R1-2210413, the following topics were recommended for discussion for next meeting:

Please prioritize the following discussion topics for decision on candidate enhancement techniques: - On CG enhancements: - Please focus on the discussions in sections 2.1, 2.2 and 2.3 captured with the following agreement: Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration - On DG enhancements - Please focus on the discussions in sections 3.5 with respect to proposals 3-5-5/6 captured the following agreement: Agreement - For further study the mechanisms to enable HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission for CA operation on TDD cells, consider at least the following: - Capacity performance evaluation results - Complexity analysis and RAN2 impact

In this contribution, we discuss the potential CG enhancements and provide evaluations for XR capacity improvements.

# Configured Grant Enhancements

One of the key XR-specific aspects to consider for any capacity enhancements is on delivering of data in the form of PDU sets. In TR 38.835 [2], the PDU Set is defined as follows:

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926 [6]). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can still recover parts or all of the information unit, when some PDUs are missing.

In the UL, the video encoder at the UE may generate different types of video frames (e.g. I-frame, P/B-frame) at a particular frame rate (e.g. 60fps, 90fps). The generated video frames or the corresponding PDU sets may consist of variable payload sizes at each periodic occasion, e.g. depending on the encoding applied at the application layer. During transmission multiple PDUs in a PDU set may need to be delivered within a PDU set delay bound (PSDB) [2].

Observation 1:	PDU sets generated at each periodic occasion can consist of variable payload sizes per PDU set. A PDU set is expected to be delivered within PSDB

During UL transmissions, the generation of different types of video frames at the application (e.g. for AR) at UE can cause inter-PDU set jitter. For example, some PDU sets associated with base video frames (e.g. I-frames) can incur longer delays compared to the differential frames (e.g. P-frames), hence causing inter-PDU set jitter. While the impact of jitter is more prominent in the DL, due to processing of different types of video frame the presence of inter-PDU set jitter in UL is non-negligible.

Observation 2:	Due to application processing of different types of video frame the presence of inter-PDU set jitter in UL is non-negligible

Using DG for transmitting PDU sets in UL may allow flexible allocation of resources that may align with that of the variable payload sizes and jitter. However, the delay incurred for sending SR/BSR for requesting DG may result in not being able to meet the delay bound for sending the PDU sets. Alternatively, configuring the UE with CG can be useful for supporting periodic transmissions of the PDU sets (e.g. video traffic and pose traffic). During RAN1#119bis-e an agreement was made to study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity.

In the following, enhancements to CG for addressing are discussed for addressing the following issues:

- Mismatch between the CG occasions/resources and the data payload size

- Mismatch between the start time of each CG occasion and XR traffic arrival

- 2.1 Adaptations to number of PUSCHs and parameters per CG occasion

As discussed above, the PDU set payload size may vary in each periodic occasion due to application layer factors (e.g. type of video frames generated). When CG is configured semi-statically to handle UL transmissions, there is a likelihood of the resources configured per CG occasion to be either insufficient or more than necessary. Any reconfiguration to CG via RRC signaling can cause additional latency.

For ensuring that the CG resources are not under-provisioned or over-provisioned, mechanisms for supporting multiple PUSCHs per CG occasion and dynamic adaptations to CG based on the XR traffic patterns can be considered. The different PUSCHs per CG occasion can be configured with different parameters (e.g. number of PRBs, MCS) to allow efficient use of resources during UL transmissions. In RAN1#119bis-e, the mechanism where the UE can dynamically indicate any unused CG occasions or resources was agreed as one of the candidate CG enhancement techniques. A similar technique that can be considered is for the UE to dynamically request to increase the CG occasions or resources, when the configured resources are insufficient for transmitting the XR traffic (e.g. PDU sets) within the delay bound.

For periodic UL transmissions, the UE can be configured with an initial number of PUSCHs per CG occasion. When the number of TBs carrying the PDUs of PDU set are larger than the initial PUSCHs, the UE can send an indication to gNB (e.g. via UCI or in the first PUSCH) for increasing the resources or number of PUSCHs per CG occasion. The UE can decide to send the indication when the initial PUSCHs or resources are insufficient and none of the PDUs of the PDU set can be delayed to the next CG occasion, for example. The gNB can then dynamically signal (e.g. in DCI) the increase in the resources or the number of PUSCHs per CG occasion. The gNB can also indicate adjustments to the CG parameters such as changes to the number of PRBs or MCS applied to the different CG PUSCHs. Although it was agreed during RAN1#119bis-e that one of the candidate CG enhancement techniques include increase CG PUSCH occasions in a duration, it is not clear whether the technique also applies to changes to CG parameters. In this regard, a dynamic approach which increases CG PUSCHs and/or allows changes the CG parameters can be beneficial for flexibly (re)allocating the CG occasions and resources when needed. In Section 3, evaluation results are provided showing the capacity gains, compared to baseline DG, for enhanced CG scheme that supports dynamic adjustments to CG parameters.

Proposal 1: 	Support dynamic indication from the UE to increase the CG PUSCH occasions or resources

Proposal 2:	Support dynamic indication to the UE (e.g. via DCI) to adjust CG parameters (e.g. number of PRBs, MCS) per CG occasion

- 2.2 Adaptations for time shifting the CG occasions

When PDU sets are generated at an application frame rate (e.g. 60 fps), the non-integer periodicity results in misalignment with the integer periodicity of the CG configuration. As a result, the periodic occasions of the CG configurations should be adjusted to realign with the XR traffic pattern. Similarly, the arrival of the PDU sets may also be impacted by jitter in UL as discussed earlier. Depending on how close the PDU sets arrive with respect to the CG occasions, certain adjustments may be needed for realignment of the occasions.

To address the timing misalignments, the UE can be configured with multiple time shift offset values that can be used for time shifting the CG PUSCHs. Based on the PDU set arrival, adaptations in terms of advancing/delaying the CG PUSCHs by any of the configured offset values can be dynamically signaled by the UE. To minimize the overhead, especially when time shifting the resources for a large data burst, single DCI can be considered for time shifting the PUSCHs for multiple CG occasions.

Proposal 3:	Support dynamic indication from the UE for time shifting (e.g. advancing or delaying) the CG PUSCH occasions by an offset value

Proposal 4: 	Support dynamic indication to the UE (e.g. via DCI) to apply a time shift offset to the CG PUSCH occasions

- 2.3 Multiple CG configurations

In Rel-16 multiple active SPS configurations are supported. Such approach can be extended for CG for supporting XR traffic in UL. For example, the UE can be configured to use multiple CG configurations, where each CG can be configured with different set of parameters (e.g. different number of PUSCHs, MCS, PRBs, start time offset). Such CG configurations can be beneficial for transmitting data with different traffic patterns (e.g. variable payload sizes), without having to incur additional overhead and latency due to reconfiguration or scheduling (e.g. for sending SR/BSR for DG).

Proposal 5:	UE can be configured with multiple CG configurations with different parameters (e.g.  different number of PUSCHs, MCS, PRBs, start time offset)

When transmitting UL data (e.g. one or multiple PDU sets) using an initial CG configuration over a number of CG occasions, the UE may determine the traffic pattern in UL in next set of CG occasions, e.g. based on application/higher layer information. The UE can identify another preconfigured CG configuration (e.g. with different number of PUSCHs per occasion) that matches with the traffic pattern of the next UL data. The UE can send in a dynamic indication to gNB on the identified CG pre-configuration for the next UL data. The gNB can then dynamically activate/deactivate the CG configurations in the UE for supporting the UL transmissions.

Proposal 6:	Support dynamic indication from the UE to request activation/deactivation of preconfigured CG configurations

# 3 Simulation Results

In this section, SLS results on capacity performance for the single stream traffic model in UL are provided. The SLS results are provided for Indoor Hotspot (InH) deployment scenario using baseline evaluation assumptions and parameters as listed in TR 38.383 (Section 5).

System capacity is defined as the maximum number of supported UEs out of which at least Y% of UEs are satisfied (e.g., PDB and PER requirements are met for UL traffic), where Y=90 (baseline) is used in the simulations. In all scenarios, a UE is declared satisfied if more than 99% of packets are successfully transmitted within the PDB values for cloud gaming and AR/VR applications.

The system capacity achieved with CG (baseline) and enhanced CG schemes are compared with baseline DG. For the baseline DG the scheduling delay incurred due to the transmission of SR/BSR and PDCCH is assumed to have a mean value of 5ms, while no scheduling delay is assumed for the CG (baseline) and enhanced CG schemes. The capacity performance when using DG, CG and enhanced CG is evaluated for UL AR traffic with 10Mbps, and PDB values of 30ms and 10ms.

The baseline schemes used in the evaluations are:

- DG (with PF scheduling)

  - Single PDCCH schedules 1 PDSCH

  - Overhead: 2 PDCCH symbols per PDSCH

  - Mean scheduling delay: 5ms

The CG schemes evaluated in the simulations are:

- Baseline CG

  - Configured with 1 PUSCH per CG occasion

  - Periodicity: 10ms

- Enhanced CG with flexible resource (re)allocation

  - Configured with at least 1 PUSCH per CG occasion

  - Periodicity: 10ms

  - Overhead: 1 PDCCH symbol per CG allocation

  - For this scheme, we assume the UE can indicate to gNB unused CG PUSCH resources. The gNB can re-assign these resources to (other) UEs that need additional resources.

UL system capacity for Indoor hotspot (with parameters as defined in Table 1 in Annex) is evaluated for FR1 (4 GHz) using SU-MIMO with DG (baseline), CG, and enhanced CG with flexible resource allocation. A UE is declared satisfied if more than 99% of packets are successfully transmitted within PDB values of 30ms and 10ms, respectively.

Figure 1 shows the UL results for system capacity for AR in an InH deployment where a single stream is simulated and PDB is 30ms. It is observed that for the Uplink AR case, where aggregated video is sent in the UL, baseline DG performs better than baseline CG, however, enhanced CG yields the best performance. System capacity for baseline CG is 6, whereas both baseline DG and enhanced CG can support 7 UEs per cell.

Figure 2 shows the UL results for system capacity for AR with PDB of 10ms in an InH deployment. It is observed that for the Uplink AR case, where aggregated video is sent in the UL, baseline CG as well as enhanced CG outperforms the baseline DG scheme. Baseline DG can support 2 UEs per cell, baseline CG can support 4 UEs per cell, while enhanced CG outperforms both with a capacity of 5 UEs per cell. Baseline DG is severely impacted by scheduling delay of 5 ms, which heavily eats into the stringent 10ms PDB requirement.

Figure 1: FR1 UL AR 10 Mbps (30ms PDB) for Indoor Hotspot scenario

Figure 2: FR1 UL AR 10 Mbps (10ms PDB) for Indoor Hotspot scenario

Observation 3: 	Enhanced CG scheme with flexible resource (re)allocation outperforms baseline DG schemes. The performance improvement offered by enhanced CG over baseline DG is more pronounced when the PDB requirements are more stringent.

# Conclusion

In this contribution, the following observation are made:

Observation 1:	PDU sets generated at each periodic occasion can consist of variable payload sizes per PDU set. A PDU set is expected to be delivered within PSDB

Observation 2:	Due to application processing of different types of video frame the presence of inter-PDU set jitter in UL is non-negligible

Observation 3: 	Enhanced CG scheme with flexible resource (re)allocation outperforms baseline DG schemes. The performance improvement offered by enhanced CG over baseline DG is more pronounced when the PDB requirements are more stringent.

Based on these observations, the following proposals are made:

Proposal 1: 	Support dynamic indication from the UE to increase the CG PUSCH occasions or resources

Proposal 2:	Support dynamic indication to the UE (e.g. via DCI) to adjust CG parameters (e.g. number of PRBs, MCS) per CG occasion

Proposal 3:	Support dynamic indication from the UE for time shifting (e.g. advancing or delaying) the CG PUSCH occasions by an offset value

Proposal 4: 	Support dynamic indication to the UE (e.g. via DCI) to apply a time shift offset to the CG PUSCH occasions

Proposal 5:	UE can be configured with multiple CG configurations with different parameters (e.g.  different number of PUSCHs, MCS, PRBs, start time offset)

Proposal 6:	Support dynamic indication from the UE to request activation/deactivation of preconfigured CG configurations

# Annex A

The following show the list of agreements and conclusions that were made on XR-specific capacity enhancement techniques during RAN1

Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration Conclusion No further discussion in RAN1 for Rel-18 XR to extend the support of legacy single DCI scheduling multi-PDSCHs for FR2-2, to other SCS in FR1/FR2-1. Conclusion The capacity gain performance results in R1-2208661, R1-2209658 and R1-2209198 corresponding to enhancements based on multi-PDSCH scheduling by a single DCI are captured in XR SI TR Conclusion Study on enhancement for CBG based HARQ-ACK feedback reporting is down-priorotized in RAN1 XR SI. Conclusion The following proposed enhancements techniques to improve XR capacity performance are down-priorotized in RAN1 XR SI: - (P3-5-3) Study on PHR enhancement based on XR traffic arrival periodicity or UL pose periodicity. - (P3-5-4) Study mechanism of packet dropping based on the PDB requirement, to avoid resource waste due to the out-of-date packets. Agreement - For further study the mechanisms to enable HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission for CA operation on TDD cells, consider at least the following: - Capacity performance evaluation results - Complexity analysis and RAN2 impact Conclusion - Study of soft HARQ-ACK and Delta MCS in RAN1 XR SI for improving XR capacity is down-priortized. - Note: The corresponding capacity gain performance results in R1-2210003, R1-2208377 and R1-2203607 are captured in XR SI TR. Conclusion - Study on enhanced CQI based on CBG transmission, and study on enhanced CQI based on DMRS for improving XR capacity are down-priortized in RAN1 XR SI. - Note: The corresponding capacity gain performance results in R1-2208402 and R1-2209536 are captured in XR SI TR. R1-2210412 Moderator Summary#3 - Study on XR Specific Capacity Improvements Moderator (Ericsson) Conclusion Study on Cooperative MIMO via DL interference probing based on SRS enhancement for improving XR capacity is down prioritized in RAN1 XR SI. Note: The corresponding capacity gain performance results in R1-2208377 are captured in XR SI TR. Conclusion No consensus to continue study on differentiation of XR multiple flows based on CG enhancement in RAN1 XR SI. Conclusion No consensus to continue study of multi-bits SR mechanisms for capacity improvement of XR traffic in RAN1 XR SI.

# Annex B

The following show the parameters from TR 38.838 [2] used in system level simulations for XR.

Table 1: Assumptions for System-level simulations

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:64 Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,8,2,1,1;4,8), (dH, dV) = (0.5λ, 0.5λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | Reciprocity-based precoding |  |  |
| Scheduling Algorithm | DL: SU-MIMO with PF |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 2: DL Traffic models for CG/AR/VR evaluations

|  | CG | VR | AR |
| --- | --- | --- | --- |
| Data Rate | 30Mbps (baseline) @60fps | 30Mbps (baseline), 45Mbps @60fps | 30Mbps (baseline), 45Mbps @60fps |
| FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
| Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
| Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
| Air Interface PDB | 15ms (baseline) | 10ms (baseline) | 10ms (baseline) |
| Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [[-4, 4]ms] |  |  |

Table 3: UL Traffic models for CG/VR/AR evaluations

|  | Stream 1 (pose/control data) (for CG/VR/AR) | Stream 2 (aggregated video) (for AR) |
| --- | --- | --- |
| Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
| Data rate | 0.2 Mbps | 10 Mbps |
| Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
| Air Interface PDB | 10ms | 30ms |
| Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,30ms] (baseline) |
| Jitter | Periodic: 4ms (no jitter) | Jitter: same model as for DL |

# Reference

[1]	RAN1 chairman notes, RAN#110bis-e, Oct 2022

[2]      3GPP TR 38.835, Study on XR enhancements for NR (Release 18), Oct 2022

[2] 	3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17), Dec. 2021

[3]	3GPP TR 23.700-60, “Study on XR (Extended Reality) and media services (Release 18), v0.3.0, May 2022