3GPP TSG RAN WG1 meeting #111		                                   R1-2211905

Toulouse, France, November 14th – 18th, 2022

Source: 	ZTE, Sanechips

Title:	Evaluation on XR specific power saving techniques

Agenda item:	9.10.1

Document for:	Discussion

Introduction

The Rel-18 XR SID was approved in RAN#94 [1]. And the RAN1’s work was expected to be finished in RAN1#111 meeting. Based on the agreements we’ve achieved, the remaining work focuses on discussing the solutions for periodicity alignment between eCDRX and XR traffic. Moreover, the PDCCH monitoring adaptions including alternatives for jitter handling should be evaluated and further discussed. Basically, these techniques are potential solutions to power saving of XR terminals.

Discussion

Periodicity alignment between enhanced CDRX and XR traffic

In RAN1#110bis-e, semi-static solutions were prioritized for alignment between CDRX cycle and XR traffic periodicity. Meanwhile, the dynamic approaches were extensively discussed and not concluded. In following section, we continue to discuss the necessity of applying dynamic enhancements.

Dynamic CDRX enhancement

If a semi-static mechanism had been used to fulfill CDRX enhancement to align XR traffic periodicity, where a constant offset between the jitter center of packet arrival and start of drx-Onduration would be assumed per RRC configuration, because the data burst arrival is not known at the start.

A clarification question is whether or not the constant offset is an appropriate value that match the CDRX to XR traffic perfectly. If the answer is no, then the offset needs to be adjusted via the parameter of CDRX StartOffset, which can be implemented either via RRC or dynamic indication. For RRC re-configuration, it brings additional latency thus may be not preferred in this case. On the other hand, if the answer is yes, there seems no reason to dynamically adjust the offset.

In our view, it is preferred to make gNB able to dynamically configure CDRX, since that can not only provide more flexibility e.g., dynamic adjustment of configuration, but also is beneficial for extending XR applications. For example, this function can ensure robustness to more variety of frame generation rates. In addition, the generation of video frame can be adjusted according to XR applications, when RAN awareness is enabled at CN side, the corresponding change of CDRX configuration is needed to accommodate various video data rates. In short, we think that dynamic mechanism can be beneficial for these use cases.

W.r.t potential specification impacts of dynamic CDRX enhancement, we think a signaling can be designed for adjustment of CDRX StartOffset, and even for adjusting/ending onDuration based on the packet arrivals, which should not cost much signaling overhead if the adjustment is not frequent. And from UE perspective, a UE changes the CDRX configuration according to the received indications. Moreover, the details on how UE changes the CDRX configuration can be discussed in normative phase, which needs the cooperation between RAN1 and RAN2 as well.

- Proposal 1:   Support dynamic CDRX enhancement to adjust at least the parameter of CDRX StartOffset.

Semi-static CDRX enhancement

As discussed in [2][3], RAN1 studied and evaluated the detailed semi-static CDRX alternatives for improving power saving performance as listed below:

- Alt 1:Non-uniform CDRX cycle pattern

For a legacy CDRX configuration, drx-LongCycle is configured as an integer value, which may not be adapted to XR traffic periodicity. Thus, non-uniform CDRX cycle pattern can be considered. In this way, multiple small CDRX cycles are used to compose a longer cycle, e.g.,50 ms. For example, a CDRX cycle set with three small CDRX cycles, e.g., (17ms, 17ms, 16ms) or (16ms,16ms,18ms), can form a long cycle. And the alignment between XR traffic arrival and CDRX cycle is achieved at the start of each long cycle.

- Alt 2:Uniform non-integer CDRX cycle

The formula defined in TS 38.321 can be reused with small modification as follows, i.e., start the drx-onDurationTimer if floor([(SFN×10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset, wherein the value of drx-LongCycle can be a non-integer CDRX cycle (e.g., 1000/FPS ms). Although a non-integer CDRX cycle value is introduced, this option actually achieves a CDRX cycle pattern as e.g., (17ms, 17ms, 16ms).

Moreover, once SFN wraparound mismatch is addressed by RAN2, the formula can be revisited at SFN wrap around.

- Alt 3:Multiple CDRX configurations

In our understanding, enhanced multiple CDRX configurations are proper to handle multiple XR flows. And it can also be used to align the CDRX cycle with XR traffic. For example, the common CDRX parameters include drx-LongCycle, drx-onDurationTimer, drx-InactivityTimer, and separate CDRX parameter such as staggered start offsets can be used in order to achieve alignment per long CDRX cycle.

The performance evaluations of these solutions were provided in Table 1~Table 3 in [3]. The scenarios include indoor hotspot scenario, and dense urban. And results of both joint UL&DL and DL only are provided. Basically, for different cases, CDRX alignment schemes achieve power saving gain as follows.

- For FR1, Indoor Hotspot, [-4,4] ms jitter range, VR30Mbps

- For DL+UL, the CDRX alignment schemes have 23.85%~24% power saving gain.

- For DL only, the CDRX alignment schemes have 33.8%~34% power saving gain.

- For FR1, Dense Urban, [-4,4] ms jitter range, VR45Mbps, DL only, the CDRX alignment schemes have 16%~23% power saving gain.

Proposal 2:	Capture the evaluation results of solutions of matching periodicities between XR traffic and CDRX into TR 38.835.

More discussions on the differences among these solutions are highlighted in Table 1. For example, detailed differences from specification/ implementation aspects, are given for better understanding of these options and evaluations.

Table 1 Comparison among semi-static RRC configuration approaches.

|  | 1)Non-uniform CDRX cycle pattern | 2)Uniform non-integer CDRX cycle | 3)Multiple CDRX configurations |
| --- | --- | --- | --- |
| Performance | Similar power saving gain with acceptable capacity loss. |  |  |
| New CDRX parameters/ RRC signaling | Low/medium RRC signaling overhead, with new parameters, Form flexible DRX cycle pattern, e.g., (17,17,16), (16,16,18) etc. | a set of CDRX parameters are required with new parameters Form uniform DRX cycle pattern, e.g., (17,17,16) | Multiple sets of CDRX parameters are required with new parameters |
| Specification impact | Complex switching among multiple sub-cycles | Modified the formula. The cycle in the formula can be non-integer, e.g., 50/3 ms | Complex management of multiple sets of timers Support multiple CDRX in one serving cell |

Alt2 and alt3 are preferable from specification impact perspective. For example, it would be difficult to describe the UE behavior of PDCCH monitoring if we go with alt 1. Then for alt 2, the change is quite simple, i.e., the framework of the UE’s behavior is reused, and only minor changes to the formula are needed. In addition, for alt 3, the complexity could be acceptable for UE managing multiple CDRX configurations. More importantly, alt 3 is potentially beneficial for multiple flows as well as mixed XR and non-XR cases.

As studied in both RAN1 and RAN2 [4], we think the details of RRC-based mechanisms are useful for discussion. The alternative 2 and alternative 3 can be recommended for the reasons e.g., small specification impact, small complexity.

Proposal 3:	RRC-based mechanism e.g., uniform non-integer CDRX cycle, multiple CDRX configurations, is recommended due to small specification impact, small complexity.

Jitter handling

As discussed in previous meetings, for some realistic DL video traces, the instantaneous jitter value can be larger than 4ms. And we agreed in RAN1#110bis that [-8,8] ms truncation jitter range can also be optionally evaluated in this SI. In this case, the power saving gain of aligned CDRX is too limited as shown in [3], it means that’s essential to identify solutions to handle the low PSG cases with given statistical jitter parameters.

If the CDRX onDuration was extended e.g., up to 12ms under a CDRX configuration, e.g.,(16,12,4), UE could monitor PDCCH for arbitrary packet arrival. However, the power consumption will increase because of such a long time duration for PDCCH monitoring. Some companies propose to consider PDCCH skipping to save power by reducing the extended onDuration or InactivityTimer. More specifically, a scheduling DCI before the end of the packet transmission or a DCI with dummy PDSCH after receiving acknowledge to last PDSCH transmission, can be used for the PDCCH skipping indication.

In our study, the evaluation results in Table 2~4 show that certain power saving gain improvement can be achieved via using PDCCH skipping, in the meanwhile the capacity degrades nearly 5%~15.48% in these cases; one of the reasons is the PDCCH skipping may delay potential retransmission scheduling.

Table 2 FR1 power consumption results in Dense Urban scenario (VR45Mbps, fps=60, DL only, jitter=[-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 10 | 5 | 7 | 7 | 90.48% | 16% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 10 | 5 | 7 | 7 | 84% | 24% |

Table 3 FR1 power consumption results in Dense Urban scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 85% | 4.5% |
| PDCCH skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 74% | 9.6% |

Table 4 FR1 power consumption results in Indoor scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 88.10% | 4.60% |
| PDCCH skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 72.62% | 10.84% |

Observation 1: The evaluation results show certain power saving gain improvement via using PDCCH skipping, in the meanwhile the capacity degrades nearly 5%~15%.

On the other hand, if normal/shorter onDurationTimer is configured, e.g., (16, 6, 4), the capacity could deteriorate as well, because the random packet arrival can be outside of DRX active time. As a result, the packet would endure delay of being transmitted till next CDRX cycle. In order to address this issue, according to what we’ve discussed so far, additional active time based on current CDRX design is considered, which more likely has minor specification impact.

Additional DRX active time

This candidate was extensively discussed in RAN1#110bis [5], in this case, an additional PDCCH monitoring time on demand can be applied to monitor the scheduling which is out of CDRX onDuration (it happens when normal CDRX onDuration is configured and there is jitter impact). That is, if a data packet arrives later than the end of onDuration, an additional active time can be activated for receiving this data packet. For instance, the UE can be implicitly/explicitly indicated to trigger another active time in response to no data reception before the end of original CDRX onDuration.

From RAN1’s evaluations perspective, the proposed solutions can be supported for both (-4ms,4ms) and (-8ms,8ms) jitter value range. The performance evaluations of these solutions are provided in Table A3~Table A5 in Appendix A. It was observed flexible additional active time can achieve better performance in comparison with the performance of aligned CDRX solutions. That basically justifies the enhancement of CDRX mechanism to accommodate to transmission with jitter impact. And in general, from commercial perspective, we think the CDRX feature is mature, and new feature with small modification on the current CDRX mechanism can be deployed fast, thus the CDRX enhancement is preferred.Regarding the concern to UE autonomous extension of DRX active time, for example, if another data is scheduled first, the additional DRX active time may not be triggered, thus UE may falsely miss the XR packet. This issue refers to how UE can recognize specific period XR packets. It was explained that the packet can be recognized via higher layer identification (e.g. characteristics of PDU sets), and the XR packet scheduling can be differentiated via RNTI.

Similarly, DCI can trigger additional DRX active time for Rel-18 XR-specific power saving enhancements. The explicit signaling can provide similar benefit for handling jitter problem. While an additional concern was pointed out, i.e., legacy dummy grant can trigger an InactiveTimer, and it can extend the active time. In our view, dummy grant is not flexible due to the fixed length of InactiveTimer. Of course, the dummy grant also costs UE’s additional power/complexity/latency by decoding PDCCH+PDSCH.

Proposal 4:	Capture solutions and evaluation results for jitter handling into TR 38.835.

- Proposal 5:  Support flexible additional active time based on current CDRX design, with minor specification change.

When PDCCH skipping/ SSSG switching functions are taken into account, further evaluation results are given in Table 5 and Table 6. W.r.t the evaluation assumptions, aligned CDRX with XR traffic periodicity are assumed to be applied for all other power saving schemes. Besides, legacy R17 PDCCH skipping is used, for PDCCH skipping indicated by a scheduling DCI at the last slot of initial traffic transmission, the PDCCH skipping duration can’t be very large, because the PDCCH skipping may delay potential retransmission scheduling. And R17 PDCCH skipping can be PDCCH skipping indication via dummy grant after gNB receiving the Acknowledge to last PDSCH transmission of a packet. In this case, UE is assumed to receive the dummy grant DCI and decode a blank PDSCH. Hence, the relative power of the slot in which UE receives the dummy grant DCI is set to 300 (i.e., it is equivalent to the relative power of PDCCH+PDSCH).

Table 5 FR1 power consumption results in Indoor scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 88.10% | 4.60% |
| Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 5 | 7 | 7 | 80% | 9.46% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 4 | 7 | 7 | 89.48% | 19% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 72.62% | 10.84% |
| PDCCH skipping with dummy grant (duration = 4ms,8ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 80.95% | 13.03% |

Table 6 FR1 power consumption results in Dense Urban scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 85% | 4.50% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 4 | 7 | 7 | 84% | 18% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 74% | 9.6% |
| PDCCH skipping with dummy grant (duration = 4ms,8ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 80% | 12% |

Figure 1. Power saving gain results for 45Mbps XR traffic with [-8,8]ms jitter range

The results are summarized as shown in Figure 1. Different PSG of legacy PDCCH skipping solutions can be observed. As to PDCCH skipping with dummy grant (black bar in Figure 1), it doesn’t achieve a large PSG, and the capacity is negatively affected. Because in this case a large onDuration is used, and the power consumption of PDCCH monitoring after the last packet and before the dummy grant cannot be saved, and additional power is consumed for PDSCH decoding and ACK/NACK transmission for the dummy grant. Moreover, the selected skipping duration may not be most proper to achieve good balance between capacity and power saving gain.

Regarding the PSG differences of these power saving schemes as shown in Figure 1, flexible additional active time has additional 9.54%~13.5% PSG over the aligned CDRX, and outperforms legacy PDCCH skipping about 6% PSG as well as 4%~8.53% capacity gain.

In addition, if the PDCCH is miss-detected, the UE would waste power consumption for monitoring PDCCHs. What’s worse, only one timer is configured for FR1 or FR2, if a long onDurationTimer in CDRX configuration is configured, there will be a problem for other services in the cell group.

- Observation 2:   For FR1, Indoor, [-8,8]ms jitter range, VR45M, DL only,

- Flexible additional active time has 19% PSG with 89.48% of satisfied UE

- PDCCH skipping with dummy grant has 13.03% PSG with 80.95% of satisfied UE

- PDCCH skipping indicated in last transmission slot has 10.84% PSG with 72.62% of satisfied UE

- Observation 3:   For FR1, Dense Urban, [-8,8]ms jitter range, VR45M DL only,

- Flexible additional active time has 18% PSG with 84% of satisfied UE

- PDCCH skipping with dummy grant has 12% PSG with 80% of satisfied UE

- PDCCH skipping indicated in last transmission slot has 9.6% PSG with 74% of satisfied UE

Observation 4:   For evaluation of power saving techniques, the limitations of legacy PDCCH skipping should be accounted.

- PDCCH skipping is indicated in a scheduling DCI

- The PDCCH skipping duration may delay potential retransmission scheduling

- PDCCH skipping with dummy grant costs additional power consumption due to decoding PDSCH and transmitting HARQ-ACK to the dummy grant

Rel-17 PDCCH monitoring adaption enhancement applied to XR scenarios

As mentioned in section 2.2.1, the long PDCCH skipping duration may delay potential retransmission scheduling, which limits the potential power saving gain from PDCCH skipping. Therefore, in this section, we discuss whether or not the PDCCH skipping enhancements is needed in order to bring more power saving gain.

For example, as shown in Figure 2, the PDCCH skipping is indicated in a scheduling DCI for last slot of packet transmission, a large duration can be adopted for saving more power, if UE can resume PDCCH monitoring after UE sends the NACK, so retransmission scheduling can be performed within the indicated PDCCH skipping duration in this case. Thus, the actual power saving time spans from the receiving of PDCCH skipping to the start of UE resuming PDCCH monitoring.

Figure 2. UE resumes PDCCH monitoring after PDCCH skipping starts

As shown in Table 7 and Table 8, in case of Indoor scenario, VR 45Mbps, jitter= [-8,8]ms, enhanced PDCCH skipping is evaluated, which achieves 3.28% power saving gain compared with PDCCH skipping with dummy grant. And in case of VR 45Mbps, jitter= [-8,8]ms, enhanced PDCCH skipping achieves 4% power saving gain compared with PDCCH skipping with dummy grant.

Table 7 FR1 power consumption results in Indoor scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 88.10% | 4.60% |
| Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 5 | 7 | 7 | 80% | 9.46% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 72.62% | 10.84% |
| PDCCH skipping with dummy grant (duration = 4ms,8ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 80.95% | 13.03% |
| PDCCH Skipping with retransmission (duration = 3ms,7ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 84.52% | 16.31% |

Table 8 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 85% | 4.50% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 74% | 9.6% |
| PDCCH skipping with dummy grant (duration = 4ms,8ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 80% | 12% |
| PDCCH Skipping with retransmission (duration = 3ms,7ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 84% | 16% |

Observation 5:  In case of XR traffic, jitter = [-8,8]ms, PDCCH skipping with retransmission achieve a small ( i.e., 3.29%~4%) power saving gain compared with PDCCH skipping with dummy grant.

Based on above results, the PDCCH skipping enhancement brings limited additional power saving gain, and is considered not attractive. Apart from the power saving benefit, the mentioned concerns should be addressed. For example, according to the discussions in Rel-17 UE power saving, both UL-retransmission and DL-retransmission should be considered in this case. However, this discussed PDCCH skipping enhancement purely handles in terms of resuming PDCCH monitoring for scheduling DL-retransmission, thus is not preferred. In addition, there is no interaction between re-transmission and PDCCH skipping if PDCCH skipping duration stops before HARQ-ACK transmission, or be terminated by outside DRX active time, i.e., RTT timer.

- Proposal 6:   There is no need to enhance Rel-17 PDCCH skipping for re-transmission.

Conclusion

In this contribution, we discuss the XR-specific power saving techniques and we have the following observations and proposals.

Observation 1: The evaluation results show certain power saving gain improvement via using PDCCH skipping, in the meanwhile the capacity degrades nearly 5%~15%.

- Observation 2:   For FR1, Indoor, [-8,8]ms jitter range, VR45M, DL only,

- Flexible additional active time has 19% PSG with 89.48% percentage of satisfied UE

- PDCCH skipping with dummy grant has 13.03% PSG with 80.95% of satisfied UE

- PDCCH skipping indicated in last transmission slot has 10.84% PSG with 72.62% of satisfied UE

- Observation 3:   For FR1, Dense Urban, [-8,8]ms jitter range, VR45M DL only,

- Flexible additional active time has 18% PSG with 84% of satisfied UE

- PDCCH skipping with dummy grant has 12% PSG with 80% of satisfied UE

- PDCCH skipping indicated in last transmission slot has 9.6% PSG with 74% of satisfied UE

Observation 4:   For evaluation of power saving techniques, the limitations of legacy PDCCH skipping should be accounted.

- PDCCH skipping is indicated in a scheduling DCI

- The PDCCH skipping duration may delay potential retransmission scheduling

- PDCCH skipping with dummy grant costs additional power consumption due to decoding PDSCH and transmitting HARQ-ACK to the dummy grant

Observation 5:  In case of XR traffic, jitter = [-8,8]ms, PDCCH skipping with retransmission achieve a small ( i.e., 3.29%~4%) power saving gain compared with PDCCH skipping with dummy grant.

Proposal 1:  Support dynamic CDRX enhancement to adjust at least the parameter of CDRX StartOffset.

Proposal 2:	Capture the evaluation results of solutions of matching periodicities between XR traffic and CDRX into TR 38.835.

Proposal 3:	RRC-based mechanism e.g., Uniform non-integer CDRX cycle, Multiple CDRX configurations is recommended due to small specification impact, small complexity.

Proposal 4:	RAN1 suggests to capture solutions and evaluation results for jitter handling into TR 38.835.

- Proposal 5:  Support flexible additional active time based on current CDRX design, with minor specification change.

- Proposal 6:   There is no need to enhance Rel-17 PDCCH skipping for re-transmission.

References

RP-220285, Study on XR Enhancements for NR.

R1-2207061, Evaluation on XR specific power saving techniques, ZTE, Sanechips, RAN1#110.

R1-2209197, Discussion on XR specific power saving techniques, ZTE, Sanechips, RAN1#110bis.

RAN2#119bis-e Chair Notes.

RAN1#110bis-e Chair Notes.

- Appendix A

Simulation parameters and power model adopted in our simulation refer to [4], and the assumptions are in line with TR 38.838.

In our contribution, evaluations of power consumption in terms of both joint UL&DL and DL only are provided.

More simulation results from [3] are also provided in the Appendix.

Table A1 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL + pose/control, jitter = [-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.18% | - |
| R15 CDRX | 10 | 8 | 4 | 11 | 11 | 90.15% | 6.65% |
|  | 16 | 10 | 5 | 11 | 11 | 81.82% | 13.9% |
| Non-uniform CDRX cycle | (17,17,16) | 6 | 4 | 11 | 11 | 90.15% | 23.86% |
| Uniform non-integer CDRX cycle | (1000/60) | 6 | 4 | 11 | 11 | 90.15% | 23.85% |
| Enhanced multiple CDRX (3 CDRX configurations) | 50ms DRX cycle | 6 | 4 | 11 | 11 | 90.11% | 24% |

Table A2 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL only, jitter = [-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.18% | - |
| R15 CDRX | 10 | 8 | 4 | 11 | 11 | 90.15% | 7% |
|  | 16 | 10 | 5 | 11 | 11 | 81.82% | 18.47% |
| Non-uniform CDRX cycle | (17,17,16) | 6 | 4 | 11 | 11 | 90.15% | 33.9% |
| Uniform non-integer CDRX cycle | (1000/60) | 6 | 4 | 11 | 11 | 90.18% | 33.8% |
| Enhanced multiple CDRX (3 CDRX configurations) | 50ms DRX cycle | 6 | 4 | 11 | 11 | 90.1% | 34% |

Table A3 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only)

| Traffic | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VR45M Jitter = [-4,4]ms | Always On | - | - | - | 7 | 7 | 91.16% |  |
|  | Aligned CDRX with XR traffic | Aligned every 50ms | 8 | 5 | 7 | 7 | 89.8% | 23% |
|  | Aligned CDRX with XR traffic | Aligned every 50ms | 10 | 5 | 7 | 7 | 90.48% | 16% |
|  | Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 5 | 7 | 7 | 91.84% | 26.24% |
|  | Flexible additional active time + aligned CDRX + PDCCH skipping(duration = 2ms) | Aligned every 50ms | 2 | 5 | 7 | 7 | 91.16% | 33.5% |
|  | WUS(start at the beginning of jitter range, WUS is monitored every 1 slot) | Aligned every 50ms | 5 | 5 | 7 | 7 | 90.48% | 24.87% |
|  | PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 10 | 5 | 7 | 7 | 84% | 24% |
| VR45M Jitter = [-8,8]ms | Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 87% | 4.86% |
|  | Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 5 | 7 | 7 | 90.48% | 20.53% |
|  | WUS(start at the beginning of jitter range, WUS is monitored every 1 slot) | Aligned every 50ms | 5 | 5 | 7 | 7 | 86% | 20.84% |
|  | PDCCH skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 82% | 9.2% |

Table A4 FR1 power consumption results in Indoor Hotspot scenario (VR,CG, fps=60, DL + pose/control)

{additional active time = original DRX On duration}

| Traffic | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VR45M Jitter = [-8,8]ms | Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 5 | 7 | 7 | 90% | 7.5% |
|  | Flexible additional active time + aligned CDRX | Aligned every 50ms | 6 | 5 | 7 | 7 | 88.1% | 15.16% |
| VR30M Jitter = [-8,8]ms | Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 4 | 11 | 11 | 88.36% | 7.5% |
|  | Flexible additional active time + aligned CDRX | Aligned every 50ms | 5 | 4 | 11 | 11 | 90% | 14.18% |
| CG30M Jitter = [-4,4]ms | CDRX alignment | 16 | 6 | 4 | 12 | 12 | 84% | 21.2% |
|  | Flexible additional active time + aligned CDRX | 16 | 6 | 4 | 12 | 12 | 88.19% | 21.3% |

Table A5 FR1 power consumption results in Indoor Hotspot scenario (VR, CG, fps=60, DL only)

| Traffic | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VR45M Jitter = [-8,8]ms | Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 5 | 7 | 7 | 90% | 8.47% |
|  | Flexible additional active time + aligned CDRX | Aligned every 50ms | 6 | 5 | 7 | 7 | 88.1% | 20% |
| VR30M Jitter = [-8,8]ms | Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 4 | 11 | 11 | 88.36% | 10.4% |
|  | Flexible additional active time + aligned CDRX | Aligned every 50ms | 5 | 4 | 11 | 11 | 90% | 30% |
| CG30M Jitter = [-4,4]ms | CDRX alignment | 16 | 6 | 4 | 12 | 12 | 84% | 30.9% |
|  |  | 16 | 8 | 6 | 12 | 12 | 88.89% | 20.6% |
|  | Flexible additional active time + aligned CDRX | 16 | 6 | 4 | 12 | 12 | 88.19% | 32.4% |

- Appendix B

Agreement and conclusions in RAN1#109

Agreement For power saving study of Rel-18 XR SI, CDRX enhancements to evaluate in this study item are to be selected from the following: - High priority Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity for each flow - High priority Issue 1-2: C-DRX enhancements to handle jitter - Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows [Note 2] - Low priority Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate - Note: Some companies think the adjustment for variable burst sizes can be realized by existing spec already - Low priority Issue 1-5: low latency handling - Low priority Issue 1-6: SFN wraparound mismatch (if handled in RAN1) FFS: how the solutions or the combination of the solutions can handle all the identified issues. Note 1: Other considerations are not precluded Note 2: It can also be adopted for addressing issue 1-1 Note 3: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group. Additional details can be found in R1-2205411. Agreement For power saving study of Rel-18 XR SI, PDCCH monitoring enhancements to evaluate in this study item are to be selected from the following - Low priority Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. - Note: some companies think Rel-17 PDCCH monitoring adaptation can solve issue 2-1 or achieve similar intended outcome - Note: Solutions proposed for Issue 2-1 and those proposed for Issue 1-1 are motivated by the same issue, namely non-integer XR traffic periodicity. It is to be studied how they compare in in terms of power saving gain and capacity, (a) solutions proposed for Issue 1-1; (b) solutions proposed for Issue 2-1. - Low priority Issue 2-2: XR-dedicated PDCCH monitoring window to supplement CDRX for multi-flow traffic. - Note: some companies think Rel-17 PDCCH monitoring adaptation can solve issue 2-2 or achieve similar intended outcome - Note: Solutions proposed for Issue 2-2 and those proposed for Issue 1-3 are motivated by the same issue, namely multiple XR traffic flows. It is to be studied how they compare in in terms of power saving gain and capacity, (a) solutions proposed for Issue 1-3; (b) solutions proposed for Issue 2-2. - High priority Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. - Note: Discussion on some enhancements may depend on the outcome of Rel-17 PDCCH monitoring adaptation maintenance - Note: The study on enhancement to R17 PDCCH monitoring adaptation should focus on the techniques that are used for addressing XR-specific issues, e.g., jitter - Note 1: Other considerations are not precluded - Note 2: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group. Agreement For Rel-18 XR power saving enhancements, RAN1 further discusses by RAN1 #110 whether the issues below are to be addressed, and if so, which solutions should be selected for evaluation in this study item. These issues are low priority. - Issue 3-1: Misaligned UE transmission and reception. - Issue 3-2: Power saving by XR-aware scheduling. - Note 1b: XR SI objective has XR-awareness in RAN listed as a specific topic of RAN2 study - Issue 3-3: Unnecessary data transmission in allocated resources. Note 1: Rel-18 XR SI objective only has CDRX enhancements and PDCCH monitoring enhancements explicitly listed as focus of RAN1 study Note 2: Other considerations are not precluded Agreement Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements Agreement Companies are encouraged to compare performance of the following Rel-15/16/17 features with the proposed enhancements for Rel-18 XR power saving evaluations. Power saving gain is calculated w.r.t. the AlwaysOn baseline. - Rel-15/16 CDRX including long DRX cycle, short DRX cycle and DRX command MAC CE and DCP - Rel-17 PDCCH adaptation including PDCCH skipping and SSSG switching Note: up to companies to report the configuration of the Rel-15/16/17 features Conclusion - If no evaluation result is provided by any company for an issue, the issue is deprioritized. The issue and proposed enhancements for the issue will not be captured by RAN1 in TR 38.835. - If no evaluation result is provided by the proponent company for a proposed enhancement, the proposed enhancement is deprioritized. The proposed enhancement will not be captured by RAN1 in TR 38.835. - If multiple enhancement techniques are proposed for the same issue, there can be down selection among them for the consideration of candidate enhancement for study item recommendation by RAN1 at least based on performance (power saving and capacity), spec impact, signaling overhead and implementation complexity. - Companies are encouraged to provide detailed information for both the proposed enhancement and the existing power saving features used as the performance reference so that the evaluation results for both can be reproduced by other companies. - When using existing power saving features as the performance reference, companies are encouraged to configure the existing power saving features to achieve the best performance. - For evaluation of a proposed enhancement and evaluation of the existing power saving features as performance reference, companies are encouraged to provide the high load case (as defined in TR 38.838, Section A.2) results. Results for low load case can also be reported optionally.

Agreement and conclusions in RAN1#110

Agreement

RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity

Conclusion

Conclude that “SFN wraparound mismatch” is a RAN2 issue. It can be left to RAN2 to address. RAN1 does not further study it.

Conclusion

RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA.

Conclusion

All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference.

Conclusion

UE transmission and reception alignment for Issue 3-1 is deprioritized for power saving in Rel-18 XR SI.

Conclusion

RAN1 does not assume dynamic switch of different XR video data rates or frame rates for Rel-18 XR power saving study before further input is provided by SA.

For future meetings

Companies are encouraged to account the enhancement of CDRX to align with XR traffic periodicity in their further evaluations for XR power saving enhancements.

Conclusion:

Companies are requested to use the Excel sheet attached with TR 38.838 in RP-213652 for recording the simulation results that are provided in their contributions.

Agreement and conclusions in RAN1#110-bis

Agreement

For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1)

- Prioritize semi-static solutions

  - FFS: Whether dynamic solutions will be also needed

R1-2210338	Moderator Summary#2 on XR specific power saving techniques	Moderator (Qualcomm Incorporated)

Conclusion

“Retransmission-less CG for UL pose transmission (Item 3.3-5)” is a RAN2 issue, leave the discussion to RAN2, RAN1 does not further investigate the issue.

- Note: how to capture evaluation results and findings will be separately discussed

Conclusion

RAN1 does not further study jitter handling by LP-WUS (Item 2.2-4) in Rel-18 XR SI

- Note: how to capture evaluation results and findings will be separately discussed

Conclusion

In addition to the values for jitter in Table 5.1-2 in TR 38.838, the following statistical parameters for jitter can also be optionally evaluated in Rel-18 XR SI.

- Note: This optional assumption is not applicable to the evaluation of 90 FPS and above

| Parameter | unit | Optional value for evaluation |
| --- | --- | --- |
| Mean | ms | 0 |
| STD | ms | 5 |
| Truncation range | ms | [-8, 8] |

Conclusion

RAN1 deprioritizes DCP indicated SSSG switching (Item 3.3-4)

- Note: how to capture evaluation results and findings will be separately discussed