3GPP TSG RAN WG1 meeting #111		                                   R1-2211906

Toulouse, France, Nov. 14th – 18th, 2022

Title: 	XR specific capacity enhancements

Source: 	ZTE, Sanechips

Agenda item:	9.10.2

Document for:	Discussion and decision

Introduction

In RAN1#110bis-e meeting, the study scope of candidate capacity enhancement techniques for XR traffic based on CG has been agreed as follows [1]:

Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration

In this paper, we present the evaluation results of following candidate capacity enhancement techniques: multiple CG PUSCH transmission occasions in a duration with unused resource release. Moreover, the evaluation results of other candidate capacity enhancement techniques, including e.g., BSR enhancement are summarized.

Discussion on uplink CG enhancement

CG transmission for XR traffic

CG mechanism in NR is adopted to support URLLC services. The key reasons are that CG transmission is capable of reducing overhead of PDCCH, more importantly, extra latency is caused by both SR transmission and BSR procedure.

Then for XR services which require low latency (e.g. 10ms,15ms,30ms), the CG-based solutions were considered and evaluated to meet the requirement. On the other hand, when conserved dynamic grant with SR-BSR-PUSCH procedure was adopted, a lot of packets transmission might exceed PDB and therefore got dropped, which would deteriorate the performance seriously.

In the case of Pose/Control traffic model transmission, since the packets arrival is periodic and the packet size is constant, CG mechanism can be applied directly without any enhancements.

The Pose/Control packets arrival is periodic and the packet size is constant, thus CG is suitable for Pose/Control traffic transmissions.

For AR video traffic model, i.e., AR aggregating scene/video/data/audio, which is characterized by large and various packet size. In this case, one solution is to use uplink dynamic grant for flexible resource scheduling. However, more time would be needed for transmitting the large packet for a UE. Thus DG would encounter performance penalty, due to the procedure of SR-BSR-PUSCH transmission, especially in the scenario of more stringent latency requirement.

For AR video traffic model, uplink dynamic grant would confront performance penalty especially in the scenario of stringent latency requirement.

On the other hand, configuring multiple CG PUSCHs transmission occasion(s) in a duration is studied for transmitting AR UL video traffic. Due to the packet variability, the flexibility of using CG resources should be taken into account. Basically Type-2 CG can configure/re-initiate the resource allocation, moreover, UE can skip the unused resources, and inform the information to gNB(this is consider as an enhancement). When gNB is aware of the unused resource, the resource can be released and recycled for other users or other traffic. To this end, the solution of unused resource release/recycling is proposed for handling the problem of flexibility of resources usage for UE.

The resource allocation can be configured/re-initialized for legacy type 2 CG, in the meanwhile unused resource release can be considered to solve the problem of low efficiency of resource usage for UE.

Potential XR-specific CG enhancement

Increase CG PUSCH transmission occasions in a duration

As to CG, although it’s beneficial from undoing the procedure of SR-BSR-PUSCH, the problem is CG PUSCH resource may not be able to transmit the whole packet. To this end, multiple CG PUSCH transmission occasions in a duration is considered to transmit the XR packet. For example, multiple CG PUSCH transmission occasions in a duration are pre-configured, when a packet arrives, UE can transmit the packet in these occasions directly.

Increasing CG PUSCH transmission occasions in a duration can be considered to deal with the characteristic of large and variable packet size for UL AR video traffic model.

According to the email discussion in RAN1#110bis-e meeting, increased CG PUSCH transmission occasions in a duration can be achieved by either Alt 1 or Alt 2 as follows:

- Alt 1: single CG configuration with multiple PUSCH occasions per CG period

- Alt 2: single DCI based activation of multiple CG configurations

For Alt 1, several parameters should be considered to determine the length of the CG period, location of CG PUSCHs in one CG period, and the number of CG PUSCHs in one CG period. As depicted in Figure 1, multiple CG PUSCH transmission occasions are pre-configured in one CG period.

Figure 1 Single CG configuration with multiple PUSCH occasions per CG period

For Alt 2, a DCI signaling is considered to jointly activate multiple CG configurations, and the association between different CG configurations should also be considered.

Figure 2 Single DCI based activation of multiple CG configurations.

Since the increased CG PUSCH transmission occasion(s) in a duration is capable of transmitting the XR packet in time, it can be considered as one of candidate techniques in Rel-18 XR WI.

Support increased CG PUSCH transmission occasion(s) in a duration in Rel-18 XR WI.

Moreover, since this is the last meeting for Rel-18 XR SI, we should focus on drafting TR38.835 within the limited TU. As a result, we suggest that details of Alt 1 and Alt 2 should be discussed in Rel-18 XR WI.

Owing to limited TU, both single CG configuration with multiple PUSCH occasions per CG period (Alt 1) and single DCI based activation of multiple CG configurations (Alt 2) should be included in Rel-18 XR WI and down-selected late.

As to the number of pre-configured CG occasion(s) in a period, there are two cases for configuration.

For one case, if gNB is aware of periodicity of XR traffic and packet statistic information, including e.g., maximum/minimum/mean packet size, the configuration is able to be based on the XR-awareness information. Moreover, the periodicity of CG configuration is able to align with that of periodicity of XR traffic if XR traffic is obtained.

For another case, if gNB is not aware of the XR-awareness information, gNB needs to configure multiple CG PUSCH transmission occasions in a duration according to the statistic information of previous received packet to cover the range of packet variability as far as it can. In our opinion, there are limited impact for this case if XR-awareness is not aware of.

XR-awareness information is helpful for configuration of increased CG PUSCH transmission occasion(s) in a duration

Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE

No matter what cases for configuration mentioned before, over-configuration may happen. And when over configuration happens, there are several unused resources in each period, which causes resource waste and degrade capacity performance.

CG PUSCH transmission occasions may be over-configured in a duration, which causes resource waste and leads to capacity performance loss.

Therefore, unused resource release mechanism, as mentioned in section 2.1, is proposed in order to assist network to save the unused resource and improve resource usage efficiency. In this case, the redundant CG PUSCHs can be released and recycled for other users or other traffic transmission so that the capacity performance can be improved.

Figure 3 Unused resource release indication

For example, as shown in Figure 3, UE estimates that the 1st CG PUSCH and the 2nd CG PUSCH are used for packet 1 transmission, while the 3rd CG PUSCH and the 4th CG PUSCH are regarded as redundant resources and these two resources can be re-used for other XR service transmission. The procedure of resource release can be summarized as follows:

Step 0: Network configuration

- Network configures increased CG PUSCH occasion(s) by either Alt 1 or Alt 2 as in Chapter 2.2.1. Moreover, time/frequency domain resource allocation, MCS level, the number of layers are determined by gNB.

Step 1: UE estimates the CG occasions usage.

- From the beginning of the packet transmission, UE has already known the actual packet size, and the actual CG occasions usage can be estimated, which is based on the resource related parameters set by semi-static configuration of gNB or activation DCI, including e.g., time/frequency domain resource allocation, MCS level and the number of layers.

Step 2: UE decides whether or not report Resource Release

- UE compares the CG occasions usage and the pre-configured CG occasions in Step 0 in a CG period. If the CG used occasions estimated by UE is less than the number of pre-configured CG occasions in a CG period, UE reports the resource release indication to gNB.

Step 3: UE transmits the resource release reporting to gNB through L1 or L2 signaling.

- The UCI or the MAC CE can be used for carry the resource release indication. And it is better to specify a new dedicated UCI or MAC CE to carry this kind of indication. When it comes to UCI signaling, if the signaling is transmitted in PUCCH, the resource release indication is SR-like report. While if the signaling is carried by PUSCH, the resource release indication can be CG-UCI-like report.

Step 4: gNB schedules/recycles/re-uses these unused resources after the resource release reporting is received.

- When the resource release indication is transmitted, gNB should firstly decode the resource release indication and schedule the unused resources at least if the resource release indication is decoded correctly. It means that the unused resources can be released and reused, in the condition that there is Downlink slot/symbols between the Uplink slot/symbol for resource release indication reception and the indicated Uplink unused slots/symbols. It is obvious that current TDD configuration in reality, i.e. DDDSU, DDDUU, can meet the condition.

The procedure can be summarized in the following figure for simplicity:

Figure 4 The procedure of resource release indication transmission

Based on the resource configuration of the BS, the UE can estimate the CG occasions usage, and UE decides whether or not report Resource Release.

Release indication for unused CG PUSCHs can be carried by L1/L2 signaling.

In summary, the procedure of resource release indication is helpful for gNB to align the understand of occasion usage in a CG period with UE, saving unused resource and improving the capacity performance.

Support dynamic indication of unused CG PUSCH occasion(s) or resource(s) by the UE for Rel-18 XR in order to address the problem of resource waste in increased CG PUSCH occasion(s) in a period.

Evaluation results for CG enhancement

Evaluation assumptions and evaluation results

In this part, we present the evaluation results for multiple CG PUSCH transmission occasions in a duration with unused resource release. And we assume that the periodicities of packet arrival and CG PUSCH have been aligned in our simulations. Moreover, the simulation assumptions are shown as below:

- Traffic model: use the uplink AR video traffic model in TR 38.838 [2]

Table 1 Traffic model for uplink simulation

| Traffic Model No. | Bit rate (Mbps) | Periodicity (FPS) | Mean Packet size (Byte) | PDB (ms) |
| --- | --- | --- | --- | --- |
| Traffic Model 1 | 20 | 60 | 41667 | 30 |
| Traffic Model 2 | 20 | 60 | 41667 | 15 |
| Traffic Model 3 | 20 | 60 | 41667 | 10 |
| Traffic Model 4 | 10 | 60 | 20833 | 15 |
| Traffic Model 5 | 10 | 60 | 20833 | 10 |

- Simulation cases: Following case 1~ case 3 include the legacy CG, dynamic grant, and enhanced CG.

  - Case 1: legacy CG

Figure 5 Legacy CG

As shown in Figure 5, orthogonal CG configurations are configured among different UEs in one cell.

  - Case 2: uplink dynamic grant

- Agreed UL dynamic grant baseline (Case 2-1)

- Another UL dynamic grant baseline (Case 2-2)

Figure 6 Dynamic grant with Tx delay (Note that DCI-x schedules PUSCH-x, while DCI-B schedules PUSCH only for BSR)

In previous meeting, we agreed when DG is used as the baseline scheme, both BSR and UL data can be transmitted using the UL grant after SR, the procedure is depicted in Figure 6(a).

While in for XR use case, it seems challenging for gNB to predict the size of resources by the first UL grant. For simplification, we assume accurate size of resource for the first UL grant can be obtained with the ability of XR-awareness. For case 2, it should be noted that we have an assumption of ideal BSR reception, implying that BSR is always received successfully.

  - Case 3: CG enhancement (eCG)

Figure 7 multiple CG PUSCH transmission occasions configuration with resource release

As shown in Figure 7, the used resources release is introduced in CG enhancement. And we think the parameters of CG configuration can be re-initialized.

The evaluation assumptions have been listed in Appendix A.

The simulation results with different traffic models are shown below:

| (a) Traffic model 1: 20Mbps@60fps, 30ms PDB | (b) Traffic model 2: 20Mbps@60fps, 15ms PDB | (c) Traffic model 3: 20Mbps@60fps, 10ms PDB |
| --- | --- | --- |

Figure 8 Capacity comparison among Case 1, Case 2 and Case 3 for 20Mbps@60fps traffic model with different PDB in Dense Urban scenario

| - Traffic model 4: 10Mbps@60fps, 15ms | (b) Traffic model 5: 10Mbps@60fps, 10ms |
| --- | --- |

Figure 9 Capacity comparison among Case 1, Case 2 and Case 3 for 10Mbps@60fps traffic model with different PDB in Dense Urban scenario

It can be observed in Figure 8 that with traffic model of 20Mbps@60fps, the system capacity of CG enhancement is [3.9], [3.5] and [2.6] UEs for 30ms PDB, 15ms PDB and 10ms PDB, respectively. The system capacity of uplink DG with the assumption that both BSR and PUSCH are transmitted after SR is triggered is [3.6], [2.1] and [0] UEs for 30ms PDB, 15ms PDB and 10ms PDB, respectively. While the system capacity of uplink DG with the assumption that only BSR is transmitted after SR is triggered is [3.5], [0] and [0] UEs for 30ms PDB, 15ms PDB and 10ms PDB, respectively.

It can be observed in Figure 9 that with traffic model of 10Mbps@60fps, the system capacity of CG enhancement is [8.2] and [5.4] UEs for 15ms PDB and 10ms PDB, respectively. The system capacity of uplink DG with the assumption that both BSR and PUSCH are transmitted after SR is triggered is [7.0] and [0] UEs for 15ms PDB and 10ms PDB, respectively. While the system capacity of uplink DG with the assumption that only BSR is transmitted after SR is triggered is [4.0] and [0] UEs for 15ms PDB and 10ms PDB, respectively.

For more easily comparing these cases the above simulation results are summarized in Figure 10.

| - 20Mbps@60fps | (b) 10Mbps@60fps |
| --- | --- |

Figure 10 Capacity summary of Figure 8 and Figure 9

It can be observed in Figure 10 that CG enhancement is beneficial to increase capacity gain compared to the UL DG solutions and legacy CG. Moreover, the capacity gain increases when PDB requirement becomes stringent with the same data rate. And the capacity also increases when data rate increases with the same PDB limitation.

CG enhancement is capable of bringing significant capacity gain compared to UL dynamic grant and legacy configured grant. Moreover, capacity gain increases when the required PDB decreases or the data rate increases.

Additional evaluation results for CG enhancement

- Hybrid CG+DG

In the email discussion of RAN1#110bis-e meeting, some companies proposed that hybrid CG+DG can be one of transmission scheme for carrying XR traffic. In order to make a comprehensive comparison between eCG and other legacy schemes, we provide the evaluation results of hybrid CG+DG based on the following assumptions:

  - Case 4 Hybrid CG+DG

Figure 11 Hybrid CG+DG

The CG periodicity is assumed to align the packet arrival and CG can be re-initialized. In addition, the CG PUSCH is restricted to size of 100k bits. The CG configuration in hybrid CG+DG is legacy CG. Moreover, the evaluation assumptions have been listed in Appendix A.

Then the simulation results with different traffic models are shown below:

| (a) Traffic model 1: 20Mbps@60fps, 30ms PDB | (b) Traffic model 2: 20Mbps@60fps, 15ms PDB | (c) Traffic model 3: 20Mbps@60fps, 10ms PDB |
| --- | --- | --- |

Figure 12 Capacity comparison among DG, hybrid CG+DG and eCG for 20Mbps@60fps traffic model with different PDB in Dense Urban scenario

| - Traffic model 4: 10Mbps@60fps, 15ms | (b) Traffic model 5: 10Mbps@60fps, 10ms |
| --- | --- |

Figure 13 Capacity comparison among DG, hybrid CG+DG and eCG for 10Mbps@60fps traffic model with different PDB in Dense Urban scenario

Then the above simulation results are summarized in Figure 14.

| - 20Mbps@60fps | (b) 10Mbps@60fps |
| --- | --- |

Figure 14 Capacity summary of Figure 12 and Figure 13

According to the summary of capacity performance in Figure 14, hybrid CG+DG provides marginal capacity performance gain compared to DG baseline. Although hybrid CG+DG is capable of reducing the scheduling delay caused by SR-BSR-PUSCH procedure, hybrid CG+DG may cause resource waste in some cases (e.g., as shown in Figure 11, the UL slot before the Downlink slot/symbols at the time when dynamic UL grant is transmitted, can’t be scheduled), which results in capacity performance loss compared to eCG.

The capacity performance of hybrid CG+DG is slightly better than that of DG baseline.

The capacity performance of eCG outperforms that of hybrid CG+DG.

- BSR transmission reliability

In Chapter 2.3.1, the evaluation assumptions are ideal for BSR transmission, without failure transmission. However, towards the transmission schemes relying on BSR signaling, including e.g., dynamic grant, hybrid CG+DG and so on, their capacity performances are relevant to BSR reliability.

For exapmle, if BSR fails to be received, gNB is not able to schedule the following transport blocks, thus causes large transmission delay. More specifically, as illustrated in Figure 15, if the first transport block (data + BSR signaling) fails to be received correctly, implying gNB is not aware of the packet size in the UE buffer, therefore gNB cannot schedule transmission for this packet immediately. This leads to certain transmission delay as shown in Figure 15.

Figure 15 The impact of BSR failure transmission

To evaluate the impact of BSR reliability, we introduce the BSR transmission model in following simulation, and provide the evaluation results considering BSR reliability. And the evaluation results with traffic model 1: 20Mbps@60fps, 30ms, and traffic model 2: 20Mbps@60fps, 15ms are shown below:

| - Traffic model 1: 20Mbps@60fps, 30ms | (b) Traffic model 2: 20Mbps@60fps, 15ms |
| --- | --- |

Figure 16 Capacity comparison w/wo BSR error for DG baseline

| - Traffic model 1: 20Mbps@60fps, 30ms | (b) Traffic model 2: 20Mbps@60fps, 15ms |
| --- | --- |

Figure 17 Capacity comparison w/wo BSR error for Hybrid CG+DG

| - Traffic model 1: 20Mbps@60fps, 30ms | (b) Traffic model 2: 20Mbps@60fps, 15ms |
| --- | --- |

Figure 18 Capacity summary of Figure 16 and Figure 17

According to the capacity performance in Figure 18, it can be observed that the capacity loss occurs when considering the BSR transmission error, especially for the XR traffic with stringent PDB.

For the transmission schemes that rely on BSR signaling, including e.g., dynamic grant, hybrid CG+DG, the BSR failure transmission enlarges the transmission delay and causes performance loss, especially for the traffic with stringent PDB.

In addition, BSR processing delay(e.g.,both in L1 and L2) should be also considered. In fact, gNB may need time to decode the BSR information from MAC PDU that contains BSR. Therefore, if the BSR processing delay is considered, the overall scheduling delay will increase, and the capacity performance will be worse.

BSR enhancement

For UL AR video transmission, data packets are characterized by large amount and variability. The reported buffer size is actually larger than the actual buffer size. Thus, legacy buffer status reporting (BSR) will waste radio resource and decrease the UL system capacity. To this end, precise buffer size determination through enhanced buffer status reporting (BSR) is considered to enhance system capacity.

To address capacity performance loss caused by overestimated packet sizes based on legacy BSR indication in uplink transmission, precise BSR indication can bring capacity performance gain.

Moreover, according to our contribution in [3], the evaluation results of the impact on BSR are summarized in the following table.

Table 2 Capacity performance in Indoor HotSpot scenario

| Traffic model | Case | Capacity |
| --- | --- | --- |
| 10Mbps@60fps, 30ms | Legacy BSR | 9.5 |
|  | Precise BSR | 10.9 |
| 20Mbps@60fps, 30ms | Legacy BSR | 3.4 |
|  | Precise BSR | 5.1 |

Although the granularity enhancement for BSR is more likely RAN2’s work, we believe the evaluation results in RAN1 for proving the capacity gain brought by this kind of enhancement should be captured in TR 38.835. It is suggested that RAN2’s colleagues can consider the evaluation results provided by RAN1. As a result, we have the following proposal:

Suggest to capture the evaluation results of BSR enhancement provided by companies in RAN1 into TR 38.835.

Other Enhancements on Capacity Performance for XR

SPS enhancement

According to the conclusion in RAN1#110 meeting [4], SPS enhancement has been deprioritized. However, since SPS has benefit in reducing overhead of downlink control signaling, transmission delay and power consumption, SPS enhancement is valuable for further study.

SPS transmission is characterized by fixed MCS level, periodic and fixed time-frequency resource, which fits to the periodical service with small packet size and stringent latency, including e.g. URLLC services. Compared to dynamic grant transmission, SPS transmission is capable of reducing overhead of downlink control signaling, as well as transmission latency which is required for PDCCH transmission/decoding in DG mechanism.

XR service is characterized by quasi-periodicity, large and various packet size as well as stringent latency. In order to reduce transmission latency and save overhead of downlink control signaling, SPS transmission may be one of potential transmission schemes for XR service due to the mentioned advantages.

Given the advantages of reducing overhead of downlink control signaling and transmission latency, SPS mechanism can be further studied for XR-specific capacity enhancement.

According to our contribution in [3], we provide the capacity performance and power consumption of enhanced SPS (with resource release indication for unused resource) and legacy SPS, and the evaluation results are summarized in the following table.

Table 3 Capacity performance and power consumption comparison in Indoor HotSpot scenario

| Traffic model | Case | Capacity | Power consumption |
| --- | --- | --- | --- |
| 45Mbps@60fps, 10ms | Legacy SPS | 0 | 195.43 |
|  | Enhanced SPS | 4 | 52.88 |
| 60Mbps@60fps, 10ms | Legacy SPS | 0 | 195.37 |
|  | Enhanced SPS | 2.4 | 61.06 |

Suggest to capture SPS enhancement techniques as well as their evaluation results proposed by companies into TR38.835.

Intra/inter UE prioritization/multiplexing enhancements

According to the conclusion in RAN1#110 meeting [4], intra/inter UE multiplexing enhancements have been deprioritized. However, since intra/inter UE multiplexing enhancements are capable of bringing significant capacity gain in the scenario of multi-stream transmission and mixed traffic transmission, intra/inter UE multiplexing enhancements are valuable for further study.

Inter-UE multiplexing enhancement can be used in the case of coexistence between XR traffic and eMBB or URLLC traffic. While intra-UE multiplexing enhancement can be considered in the case of multi-stream transmission. Both of them are capable of ensuring the stream with high priority transmission in time without sacrificing the performance of the stream with low priority in case of resource starvation.

Given the advantages of guaranteeing streams with both high priority and low priority transmission for multi-stream model in case of resource starvation, adaptive intra/inter UE prioritization/multiplexing enhancement can be further studied for XR-specific capacity enhancement.

Moreover, according to our contribution in [5], the evaluation results for intra/inter UE multiplexing are summarized in the following table.

Table 4 Capacity performance comparison of preemption in Indoor HotSpot scenario

| Traffic model | Case | Capacity | Capacity gain |
| --- | --- | --- | --- |
| audio/data + video multi-stream model | Without preemption | 4.9 | - |
|  | Rel-15 preemption | 5.7 | 47.37% |
|  | Enhanced preemption | 8.4 | 71.43% |
| Option 1A: slice-based multi-stream model | Without preemption | 4.5 | - |
|  | Rel-15 preemption | 7.1 | 43.66% |
|  | Enhanced preemption | 10.2 | 126.67% |

Suggest to capture the evaluation results of adaptive inter-UE/intra-UE multiplexing enhancement provided by companies into TR38.835.

Link adaptation enhancement for re-transmission

XR service is characterized by large packet size and stringent latency, which result in high resource occupation and limited re-transmission times when re-transmission occurs. If the scheduled MCS level is too aggressive in a poor channel condition, XR packets call for frequent re-transmissions and are prone to be dropped.

To this end, the UE feedback information for re-transmission is considered, for example, with delta MCS feedback information adaptive HARQ transmission can be achieved.

Given the advantages of guaranteeing the reliability of re-transmission and reduce the re-transmission time, delta MCS can be further studied for XR-specific capacity enhancement.

Moreover, according to our contribution in [5], the evaluation results for delta MCS are summarized in the following table.

Table 5 Capacity performance of delta MCS in Indoor HotSpot scenario

| Traffic model | Case | Capacity |
| --- | --- | --- |
| 60Mbps@60fps, 10ms | Delta MCS information feedback | 0.0 |
|  | No delta MCS information feedback | 3.3 |

According to the conclusion in RAN1#110 meeting [4], delta MCS has been deprioritized. However, since delta MCS is capable of bringing significant capacity gain, delta MCS is valuable for further study. As a result, we have the following proposal:

Suggest to capture the evaluation results of delta MCS provided by companies into TR38.835.

Network Coding

Due to the requirement of low latency and high reliability of XR traffic, the ARQ functionality in RLC layer needs to be de-activated or limited, which leads to insufficient reliability.

To this end, network coding is considered. UE does not need to successfully receive all source packets. If the number of successfully received packets equals to or are larger than the original packets, it can be successfully received by UE. Network coding provides more coding gain and increases the system reliability.

Given the advantages of guaranteeing the transmission reliability by providing coding gain with better error detection and correction performance, network coding can be further studied for XR-specific capacity enhancement.

Moreover, according to our contribution in [5], the evaluation results for network coding are summarized in the following table.

Table 6 Capacity performance of network coding in Indoor HotSpot scenario

| Traffic model | Case | Capacity |
| --- | --- | --- |
| 30Mbps@60fps, 10ms | PDCP duplication | 7.9 |
|  | Network coding | 8.9 |

According to the conclusion in RAN1#109-e meeting [6], network coding has been deprioritized. However, since network coding is capable of bringing significant capacity gain, network coding is valuable for further study. As a result, we have the following proposal:

Suggest to capture the evaluation results of network coding provided by companies into TR38.835.

Conclusion

In this contribution, we have the following observations/proposals:

Observation 1: The Pose/Control packets arrival is periodic and the packet size is constant, thus CG is suitable for Pose/Control traffic transmissions.

Observation 2: For AR video traffic model, uplink dynamic grant would confront performance penalty especially in the scenario of stringent latency requirement.

Observation 3: The resource allocation can be configured/re-initialized for legacy type 2 CG, in the meanwhile unused resource release can be considered to solve the problem of low efficiency of resource usage for UE.

Observation 4: Increasing CG PUSCH transmission occasions in a duration can be considered to deal with the characteristic of large and variable packet size for UL AR video traffic model.

Observation 5: XR-awareness information is helpful for configuration of increased CG PUSCH transmission occasion(s) in a duration

Observation 6: CG PUSCH transmission occasions may be over-configured in a duration, which causes resource waste and leads to capacity performance loss.

Observation 7: Based on the resource configuration of the BS, the UE can estimate the CG occasions usage, and UE decides whether or not report Resource Release.

Observation 8: Release indication for unused CG PUSCHs can be carried by L1/L2 signaling.

Observation 9: CG enhancement is capable of bringing significant capacity gain compared to UL dynamic grant and legacy configured grant. Moreover, capacity gain increases when the required PDB decreases or the data rate increases.

Observation 10: The capacity performance of hybrid CG+DG is slightly better than that of DG baseline.

Observation 11: The capacity performance of eCG outperforms that of hybrid CG+DG.

Observation 12: For the transmission schemes that rely on BSR signaling, including e.g., dynamic grant, hybrid CG+DG, the BSR failure transmission enlarges the transmission delay and causes performance loss, especially for the traffic with stringent PDB.

Observation 13: To address capacity performance loss caused by overestimated packet sizes based on legacy BSR indication in uplink transmission, precise BSR indication can bring capacity performance gain.

Observation 14: Given the advantages of reducing overhead of downlink control signaling and transmission latency, SPS mechanism can be further studied for XR-specific capacity enhancement.

Observation 15: Given the advantages of guaranteeing streams with both high priority and low priority transmission for multi-stream model in case of resource starvation, adaptive intra/inter UE prioritization/multiplexing enhancement can be further studied for XR-specific capacity enhancement.

Observation 16: Given the advantages of guaranteeing the reliability of re-transmission and reduce the re-transmission time, delta MCS can be further studied for XR-specific capacity enhancement.

Observation 17: Given the advantages of guaranteeing the transmission reliability by providing coding gain with better error detection and correction performance, network coding can be further studied for XR-specific capacity enhancement.

Proposal 1:	Support increased CG PUSCH transmission occasion(s) in a duration in Rel-18 XR WI.

Proposal 2:	Owing to limited TU, both single CG configuration with multiple PUSCH occasions per CG period (Alt 1) and single DCI based activation of multiple CG configurations (Alt 2) should be included in Rel-18 XR WI and down-selected late.

Proposal 3:	Support dynamic indication of unused CG PUSCH occasion(s) or resource(s) by the UE for Rel-18 XR in order to address the problem of resource waste in increased CG PUSCH occasion(s) in a period.

Proposal 4:	Suggest to capture the evaluation results of BSR enhancement provided by companies in RAN1 into TR 38.835.

Proposal 5:	Suggest to capture SPS enhancement techniques as well as their evaluation results proposed by companies into TR38.835.

Proposal 6:	Suggest to capture the evaluation results of adaptive inter-UE/intra-UE multiplexing enhancement provided by companies into TR38.835.

Proposal 7:	Suggest to capture the evaluation results of delta MCS provided by companies into TR38.835.

Proposal 8:	Suggest to capture the evaluation results of network coding provided by companies into TR38.835.

References

“RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #110bis-e	e-Meeting, October 10th–19th, 2022.

3GPP TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

R1-2209198, “XR specific capacity enhancements”, ZTE, Sanechips, 3GPP TSG RAN WG1 meeting #110bis-e e-Meeting, Oct. 10th–19th, 2022

“RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #110, Toulouse, France, August 22nd–26th, 2022

R1-2203607, “Discussion on XR specific capacity enhancements techniques”, ZTE, Sanechips, 3GPP TSG RAN WG1 #109-e, e-Meeting, May 9th–20th, 2022

“RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #109-e e-Meeting, May 9th–20th, 2022.

- Appendix

- A Simulation Parameters

Table A.1 simulation parameters for FR1 DL

| Parameters | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors | Scenario-3: Urban Macro hexagonal layout with 7, 3 Sectors |
| Inter-BS distance | 20m | 200m | 500m |
| Carrier frequency | 4 GHz |  |  |
| Duplex Mode / Simulation bandwidth | 100 MHz |  |  |
| SCS | 30KHz |  |  |
| TDD pattern | DDDSU |  |  |
| BS Antenna Configuration | 32 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 32TX:(M, N, P, Mg, Ng; Mp, Np) = (4,4, 2, 1, 1; 4, 4); The antenna tilt is 90 degrees. | 64 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 64TX:(M, N, P, Mg, Ng; Mp, Np) = (8,8, 2, 1, 1; 4, 8); The antenna tilt is 12 degrees. |  |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |  |
| Transmit Power | 24 dBm per 20M at TRP, and 23 dBm at UE | 44 dBm per 20M at TRP, and 23 dBm at UE |  |
| BS Height | 3 m | 25 m |  |
| UE Height | 1.5m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |
| Antenna Element Gain | 5dBi for BS and 0 dBi for UE | 8dBi for BS and 0 dBi for UE |  |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |  |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |  |
| Number of UEs per cell | up to 15 |  |  |
| Scheduling Algorithm | SU-MIMO+ PF |  |  |
| HARQ/repetition | HARQ retransmission |  |  |
| Channel estimation | Realist |  |  |
| Target BLER | 10% for first transmission |  |  |
| PHY processing delay | UE PUSCH processing Capability #1,N1=10 |  |  |
| PDCCH overhead | 1/7(2symbols per 14symbol) |  |  |
| DMRS overhead | 1 symbol per 14symbol |  |  |
| BS receiver | MMSE-IRC |  |  |

Table A.2 Simulation assumption for FR1 UL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban |
| Carrier frequency | 4GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) for UL interaction/pose information delivering |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64R: (8,8, 2, 1, 1; 4, 8) (dH,dV) = (0.5λ, 0.5λ); |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (1,1,2,1,1;1,1) (dH, dV)=( 0.5, N/A)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | Ceiling-mount pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi | Omnidirectional, 0 dBi |
| UE max Power | 23 dBm |  |
| Power control | P0 = -80 dBm, alpha = 0.8 |  |
| Noise Figure | 5dB for BS and 9dB for UE |  |
| Scheduler | MU-MIMO+PF / SU-MIMO CG |  |
| Device deployment | 100% indoor | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |
| Down-tilt | 90 degrees | 12 degrees |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% for first transmission |  |
| UE speed | 3 km/h |  |
| HARQ/repetition | HARQ retransmission,Maximum number of retransmissions is 3 |  |
| PHY processing delay | UE PUSCH processing Capability #1, N2=12 |  |
| DMRE overhead | 4RE |  |

- B Power model

Table B.1 Power model for DL

| Power state | Relative Power (1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table B.2 Power model for UL

| Power state | Relative Power (1 slot) |  |  |
| --- | --- | --- | --- |
|  | 0dBm | 23dBm | Other Tx power |
| UL(long PUCCH or PUSCH) | 250 | 700 | Obtained by linear function or step function |
| PDCCH+PUSCH | Sum(PDCCH-only, PUSCH) |  |  |
| PDCCH+PDSCH+PUSCH | Sum(PDCCH+PDSCH, PUSCH) |  |  |

Note: Power model of “PDCCH+PUSCH” is the sum of “PDCCH-only” power consumption and “PUSCH” power consumption. Power model of “PDCCH+PDSCH+PUSCH” is the sum of “PDCCH+PDSCH” power consumption and “PUSCH” power consumption.