3GPP TSG RAN WG1 #111			R1-2211983

Toulouse, France, November 14th – 18th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for: 	Discussion and Decision

- Introduction

At RAN#95-e meeting, revised SID on NR duplex operation enhancements was approved with the objective as follows [1]:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhance - ement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

At RAN1#110bis-e meeting, following agreements were achieved.

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands. Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step) Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency. Agreement Study whether SBFD operation in SSB symbols is supported or not. Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline. Agreement The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible. Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance. Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above. LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671. Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

In this contribution, possible solutions for subband non-overlapping full duplex and impact to legacy operation are discussed.

- Discussions

2.1 DL/UL time and frequency resource allocation

In Rel-15/16, gNB can configure/indicate a symbol as “DL” or “UL” or “flexible”, via time domain slot format indication. In Rel-18, with subband non-overlapping full duplex (SBFD) at gNB, both DL subband(s) and UL subband(s) can be configured/indicated on the same symbol.

Indication of SBFD DL/UL subband time and frequency location

During RAN1#110bis-e meeting, it was agreed that at least UL subband frequency domain location should be explicitly indicated. Besides UL subband, the frequency location of DL subband also needs to be known by UE. Explicit indication of DL subband frequency location is the most straightforward solution. Implicit indication is also possible. For example, if guard band between DL and UL subbands is not required, the DL subband frequency location can be implicitly derived based on the UL subband frequency location. If guard band is required, the DL subband frequency location can be determined based on UL subband frequency location and guard band location (or size). For this case, explicit indication of guard band size or location should be supported.

Proposal 1:

- If guard band between DL and UL subband in SBFD symbol is required, either DL subband  frequency location or guard band size should be explicitly indicated.

- If guard band between DL and UL subband in SBFD symbol is not required, DL subband  frequency location can be implicitly determined based on UL subband frequency location.

In RAN1#110 meeting, it was agreed that semi-static configuration of subband time and frequency location will be studied as baseline. Regarding whether to support dynamic indication of DL/UL subband allocation, we think it can be separately discussed for indication of subband frequency domain allocation and subband time domain location. Dynamic indication of DL/UL subband frequency domain allocation will make it more difficult for higher-layer configured channels/signals to avoid overlapping with unavailable frequency resources, and also more difficult for UE to filter interference. Therefore, dynamic indication of DL/UL subband frequency domain allocation is not preferred. Regarding dynamic indication of DL/UL subband time domain location, it can provide more flexibility for gNB to adjust SBFD symbol location according to timely DL/UL traffic characteristics, e.g. dynamically converting a symbol configured for SBFD operation into non-SBFD symbol.  It also provides more flexibility to better avoid collision with certain signals/channels, e.g. SSB or other multi-cast/broadcast channels/signals. Therefore, dynamic indication of DL/UL subband time domain location should be studied.

Proposal 2:

- Study dynamic indication of DL/UL subband time domain location, at least for converting an SBFD symbol into non-SBFD symbol.

- NOT support dynamic indication of DL/UL subband frequency domain location.

Symbol type for SBFD operation

At RAN1#110bis-e meeting, what type(s) of symbol can be supported for SBFD operation with UL subband was discussed. It seems common understanding that SBFD operation with UL subband can be supported at least for symbol configured as DL by TDD-UL-DL-ConfigCommon. The main controversial point is whether/how to support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon. In our understanding, support of SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon is beneficial for co-existence of R18 SBFD aware UE and legacy UE. On the other hand, interaction with TDD-UL-DL-Dedicated should be considered for symbol configured as flexible by TDD-UL-DL-ConfigCommon. For example, for a symbol configured as flexible by TDD-UL-DL-ConfigCommon:

- If the symbol is configured as DL by TDD-UL-DL-Dedicated, SBFD operation with UL subband in the symbol can be supported without additional specification impact, based on support of SBFD operation with UL subband in symbol configured as DL by TDD-UL-DL-ConfigCommon.

- If the symbol is configured as UL by TDD-UL-DL-Dedicated, SBFD operation should be de-prioritized, according to RAN#97 guidance that SBFD operation in UL symbol is de-prioritized.

- If the symbol is configured as flexible by TDD-UL-DL-Dedicated, or if TDD-UL-DL-Dedicated is not configured, regarding whether/how to support SBFD operation with UL subband in the symbol, interaction with DCI 2_0 (i.e. dynamic SFI indication) should be considered. Possible options include:

  - Option 1: Not support SBFD operation with UL subband in the symbol, i.e. the symbol should be non-SBFD DL/UL/flexible symbol as legacy.

  - Option 2: Whether to support SBFD operation with UL subband in the symbol depends on indication by DCI 2_0, e.g. when DCI 2_0 indicates DL (and/or flexible) for the symbol, SBFD operation with UL subband in the symbol is supported, otherwise, SBFD operation with UL subband in the symbol is not supported.

  - Option 3: Support SBFD operation with UL subband in the symbol, regardless of the direction indication by DCI 2_0.

Proposal 3:

- Support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon, at least for symbol configured as DL by TDD-UL-DL-Dedicated.

- Study whether/how to support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon and TDD-UL-DL-Dedicated, with considering interaction with dynamic SFI indication.

Link direction of DL/UL subband for SBFD operation

At RAN1#110 meeting, whether to allow DL/UL reception/transmission within/outside UL subband was discussed, with the following agreement.

Proposed Agreement: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

In our understanding, the issue can be discussed from two perspectives separately, i.e. whether to allow DL reception within UL subband in SBFD symbol, and whether to allow UL transmission outside UL subband in SBFD symbol. The first thing to be clarified is, DL and UL subbands should be used for DL reception and UL transmission respectively, which is the principle of legacy FDD/TDD or enhanced Rel-18 SBFD.

According to proponents of allowing DL transmission in UL subband in SBFD symbol, the motivation lies in the flexibility that gNB can make use of the UL subband in SBFD symbol for DL or UL based on DL and traffic load. However, such flexibility can be achieved by flexibly indicating the symbol as SBFD symbol or non-SBFD symbol. In other words, it can be achieved by dynamic indication of SBFD time domain location, instead of violating the principle that only DL reception in DL subband and only UL transmission in UL subband. Therefore, we don’t think DL reception in UL subband in SBFD symbol should be allowed.

Since UL transmission is not allowed in DL subband, whether to allow UL transmission outside UL subband in SBFD symbol depends on whether dynamic indication of DL/UL frequency domain resource allocation is supported. For example:

- If dynamic indication of DL/UL frequency domain allocation is not supported, i.e. the subband(s) outside UL subband should be DL, UL transmission outside UL subband in SBFD symbol is not supported.

- If dynamic indication of DL/UL frequency domain allocation is supported, i.e. the subband(s) outside UL subband is possible to be indicated as UL by explicit or implicit indication, UL transmission outside UL subband in SBFD symbol may be possible.

As discussed above, dynamic indication of DL/UL subband frequency domain resource allocation is not preferred to be supported. Therefore, UL transmission outside UL subband is not supported.

Proposal 4:

- Support only DL reception in DL subband (i.e. not support UL transmission in DL subband).

- Support only UL transmission in UL subband (i.e. not support DL reception in UL subband).

- Not support UL transmission outside UL subband in SBFD symbol.

- Not support DL transmission outside DL subband in SBFD symbol.

2.2 DL and UL collision for SBFD operation

In Re-15/16/17, UE transmission/reception behavior based on slot format indication is specified in TS38.213. Fig 1 shows the example of Re-16 UE reception or transmission based on slot format indication. For dynamically scheduled/triggered DL reception or UL transmission, collision with UL or DL symbol can be avoided by gNB scheduling. For higher layer configured DL receptions or UL transmissions, DL reception or UL transmission will be cancelled if colliding with UL or DL symbol, except that UL collision for SSB/type-0 PDCCH and DL collision for valid PRACH occasion are not expected.

Fig 1: Example of Re-16 UE reception or transmission based on slot format indication

Since DL subband(s) and UL subband(s) may be configured on the same symbol for SBFD operation, UE transmission and reception behavior needs to be clarified, e.g. whether/how to receive/transmit DL/UL channel/signal overlapping with SBFD symbol.

Three cases can be considered for DL and UL collision for SBFD operation:

- Case 1: DL channel/signal vs. UL subband in SBFD symbol

- Case 2: UL channel/signal vs. DL subband in SBFD symbol

- Case 3: DL channel/signal in DL subband vs. UL channel/signal in UL subband in the same SBFD symbol

Fig 2: Example of DL and UL collision cases for SBFD operation

For case 1 and case 2, the key point is how to resolve the issue of unavailable resource collision in SBFD symbol for DL/UL channel/signal. For case 3, the key issue is how to determine whether UE should perform DL reception or UL transmission in the same SBFD symbol. The two types of collision cases will be separately discussed in section 2.2.1 and 2.2.2.

2.2.1 Possible enhancements for unavailable resource collision cases in SBFD operation

Collision case 1 and case 2 will be discussed in this section.

For dynamically scheduled/triggered DL reception or UL transmission without repetition, except the case of multiple PDSCHs/PUSCHs scheduled by single DCI, collision with UL/DL subband in SBFD symbol can be avoided by gNB scheduling, as shown in Fig.3.

Fig 3: Example of dynamically scheduled channels

For the case of multiple PDSCHs/PUSCHs scheduled by single DCI, single FDRA is applied to each scheduled PDSCH/PUSCH. It may be difficult to avoid collision with UL/DL subband in SBFD symbol. For such collision, one possible enhancement is to introduce an additional FDRA field in the DCI scheduling multiple PDSCHs/PUSCHs. Another possibility is to interpretate the FDRA field differently on SBFD symbol and non-SBFD. It is also possible that the PDSCH/PUSCH overlapping with UL/DL subabnd is simply dropped.

Fig 4: Example of single DCI scheduling multiple PDSCHs

Proposal 5: For single DCI scheduling multiple PDSCHs/PUSCHs, study possible enhancements when any of the multiple scheduled PDSCHs/PUSCHs overlapping with UL/DL subband.

Channel/signal with repetitions or higher layer configured channel signal

For higher layer configured DL reception or UL transmission, or for PDSCH/PUSCH/PUCCH repetitions, as shown in Fig.5 and Fig. 6, UE behavior for collision handling needs to be discussed.

Fig 5: Example of higher layer configured channels/signals

Fig 6: Example of PDSCH/PUSCH repetitions

Since there are too many signals/channels, priority should be clarified for each channel.

- Unlike data channels which is possible to be dynamically scheduled, PDCCH monitoring completely follows higher layer configuration. Discussions for PDCCH collision handling should have highest priority.

- PUCCH report based on higher layer configuration is important, e.g. P/SP-CSI report PUCCH for channel condition, SR PUCCH for UL scheduling, and/or SPS HARQ-ACK PUCCH or DL data transmission, etc, so they can be considered as middle priority.

- Periodic or semi-persistent reference signals (e.g. P/SP CSI-RS, TRS, SRS, etc.) are important for MIMO operation (e.g. beam management, CSI feedback, etc.), which may affect link performance.

- SPS PDSCH and/or CG PUSCH are important for IIoT/URLLC/XR services, which may be important use case for subband non-overlapping duplex so that it can be considered as middle priority.

- PDSCH/PUSCH/PUCCH repetitions are important for reliability and coverage.

Proposal 6:

- For collision handling of higher layer configured channels/signals with SBFD DL/UL subband, study possible enhancements for channels/signals with higher priority first, e.g. PDCCH as highest priority for study.

- Study collision for PDSCH/PUSCH/PUCCH repetitions with UL or DL subband in SBFD symbol.

For collision handling with SBFD symbol for higher layer configured DL reception or UL transmission, or for PDSCH/PUSCH/PUCCH repetitions, generally three alternatives can be considered. And different alternative may be applicable to different collision cases.

- Alt 1: UE does not expect collision of higher layer configured DL reception or UL transmission with UL subband or DL subband in SBFD symbol.

- Alt 2: Separate higher layer DL reception or UL transmission configuration for SBFD symbols and non-SBFD symbols, e.g. separate PDCCH monitoring configuration or PUCCH resource configuration.

- Alt 3: No separate configuration, but special handling is needed if higher layer configured DL/UL channel/signal colliding with UL/DL subband in SBFD symbol, e.g. the DL/UL channel/signal is canceled or rate matched around DL/UL subband, or different interpretations are applied for frequency resource allocation for SBFD symbol and non-SBFD symbol.

For Alt 1, no specification impact is needed. It is up to gNB to guarantee no collision with DL/UL subband in SBFD symbol. It may lead to too restricted scheduling/configuration. For Alt 2, there is little impact on legacy UE operation, and special handling for collision with SBFD symbol or unavailable resource on SBFD symbol is not needed. But it may bring complexity to both NW and UE due to multiple sets of configurations, with different configurations applicable to different set of symbols/slots. For Alt 3, special handling needs to be defined.

Table 1: Analysis for possible solutions

|  | Pros | Cons |
| --- | --- | --- |
| Alt 1 | No specification impact. | Too restricted scheduling/configuration. |
| Alt 2 | Little impact on legacy UE operation; no need to define special handling for collision with SBFD symbol | Complexity due to multiple sets of configurations, with different configurations applicable to SBFD and non-SBFD symbol. |
| Alt 3 | Common configuration always applicable. | Special handling needs to be defined, and new special handling may have impact on broadcast channels/signals reception/detection for legacy UE. |

Proposal 7: For Rel-18 subband non-overlapping full duplex, at least for higher layer configured channels/signals, three alternatives can be considered:

- Alt 1: UE does not expect collision of higher layer configured DL reception or UL transmission with UL subband or DL subband in SBFD symbol.

- Alt 2: Separate higher layer DL reception or UL transmission configuration for SBFD symbols and non-SBFD symbols, e.g. separate PDCCH monitoring configuration or PUCCH resource configuration.

- Alt 3: No separate configuration, but special handling is needed if higher layer configured DL/UL channel/signal colliding with UL/DL subband in SBFD symbol, e.g. the DL/UL channel/signal is canceled or rate matched around DL/UL subband, or different interpretations are applied for frequency resource allocation for SBFD symbol and non-SBFD symbol.

Besides above general discussion for different channels/signals, some specific enhancements can be considered for specific channel/signal.

Segmentation for PUSCH repetition type B

For PUSCH repetition type B, nominal repetition may be segmented into actual repetitions around semi-static DL symbol, SSB symbol or RRC configured invalid symbol, as shown in Fig 7. Considering SBFD operation with UL subabnd in semi-static DL symbol, impact on PUSCH repetition type B segmentation should be studied.

Figure 7: Example of PUSCH repetition type B

Proposal 8: Study SBFD impact on PUSCH repetition type B segmentation.

SPS HARQ-ACK deferrring

For PUCCH, SPS HARQ-ACK deferring is introduced in Rel-17. When a PUCCH for SPS HARQ-ACK overlaps with semi-static DL symbol or SSB symbol, UE will defer the PUCCH until no overlapping with semi-static DL symbol or SSB symbol, as shown in Fig 8. Considering SBFD operation with UL subabnd in semi-static DL symbol, impact on SPS HARQ-ACK deferring should be studied.

Figure 8: Example of SPS HARQ-ACK deferring

Proposal 9: Study SBFD impact on SPS HARQ-ACK deferring.

CSI-RS resource (set) and CSI reporting

For CSI-RS, only wideband contiguous RB allocation (number of RBs no smaller than 24) is supported currently.

For SBFD operation with DL-UL-DL subband pattern in SBFD symbol, it is possible that contiguous frequency band allocation of a CSI-RS resource may collide with UL subband in SBFD symbol. In order to better avoid overlapping with UL subband, non-contiguous RB allocation for CSI-RS or narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs) can be considered. For such case, to cover the wideband CSI reporting band, different narrow band CSI-RS resources may need to be configured. Based on current restrictions (e.g. same frequency band allocation for CSI-RS resources in one resource set), it might require three CSI resource sets/settings/reporting configuration and/or three CSI reports in one or multiple CSI reporting instances to report measurement covering the wide reporting band. Therefore, it would introduce much overhead on CSI resource/CSI resource set/CSI resource setting configuration and CSI reporting configuration, and also on CSI report overhead. Potential enhancement can be considered to reduce RRC configuration overhead and/or CSI reporting overhead. One example is to allow different frequency band configurations (i.e. different starting PRB and number of RBs) for CSI-RS resources in the same CSI-RS resource set. Another example is to associate different CSI-RS resources with different frequency band configurations and treat them as one “effective CSI-RS”.

Fig 9: Example of associated CSI-RS resources

On the other hand, considering the interference level at SBFD symbol and at non-SBFD symbol may be different, whether to separately report CSI for non-SBFD symbol/slot and for SBFD symbol/slot may need to be considered.

Fig 10: Example of CSI reporting on SBFD and non-SBFD symbol

Proposal 10: Study possible enhancements on CSI-RS configuration and CSI reporting, including:

- Study non-contiguous RB allocation for CSI-RS or narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs).

- Study possible enhancements for CSI reporting overhead reduction for SBFD, e.g. to allow different frequency band configurations (i.e. different starting PRB and num-ber of RBs) for CSI-RS resources in the same CSI-RS resource set, or to associate different CSI-RS resources with different frequency band configurations and treat them as one “effec-tive CSI-RS”.

- Study whether to separately report CSI for non-SBFD symbol/slot and for SBFD symbol/slot.

2.2.2 Possible enhancements for SBFD symbol direction determination

Collision case 3 (i.e. DL channel/signal in DL subband vs. UL channel/signal in UL subband in the same SBFD symbol) will be discussed in this section.

Fig 11: Example of DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol

The simplest solution would be to avoid such case. However, it is too restricted, especially for higher layer configured channels/signals.

Another solution is that UE determines whether DL reception or UL transmission in the symbol based on explicit indication of SBFD symbol direction. For example, gNB signals an explicit indication that the direction of the SBFD symbol is DL. In this case, UE will determine to cancel the UL transmission in the UL subband, and perform DL reception in DL subband of the SBFD symbol. This solution requires additional signal overhead for SBFD symbol direction.

Another solution is that UE determines whether DL reception or UL transmission in the symbol based on predefined rules. For example, if the DL channel/signal in DL subband in the symbol is higher layer configured, while a PUSCH in UL subband in the symbol is scheduled by DCI, UE can determine to transmit the PUSCH in UL subband in the symbol, and cancels DL reception in DL subband in the symbol. Considering another example when semi-persistent CSI-RS in DL subband and periodic SRS in UL subband in the same SBFD symbol, a rule to prioritize semi-persistent channel/signal over periodic channel/signal can be applied. Another example can be SPS PDSCH in DL subband and type 2 CG PUSCH in UL subband in the same SBFD symbol. For this case, a simple rule may be that the SPS PDSCH in DL subband will be prioritized over CG PUSCH in UL subband. This solution doesn’t require explicit indication for direction of SBFD symbol. And it is flexible to separately consider different rules for different cases. For defining the rules, some legacy principles can be reused, e.g. dynamic overriding semi-static signal, legacy handling for higher layer configured DL and UL channels/signals in the same flexible symbol, etc.

Proposal 11: For DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol, study predefined rules to determine whether UE will receive in DL subband or transmit in UL subband in the same SBFD symbol.

- To define the predefined rules, following general principles can be considered:

  - Whether the channels/signals are scheduled by DCI

  - Whether the channels/signals are periodic or semi-persistent

  - Whether the channels/signals are with repetitions or without repetitions

2.2.3 SBFD operation in RRC idle/in-active mode

SSB and type-0 PDCCH are cell specific channel/signal for both legacy UEs without SBFD capability and SBFD capable UEs. Assuming DL and UL subbands on SSB or type-0 PDCCH symbol, it may cause issue for legacy UE without SBFD capability. Therefore, SBFD operation on SSB or CORESET#0 symbol should be avoided.

Proposal 12: Not support SBFD operation with UL subband on SSB symbol or CORESET #0 symbol.

In legacy RACH procedure, valid RO is determined based on semi-static TDD pattern configuration. RO overlapping with semi-static DL symbol would be regarded as invalid RO. For SBFD, when UL subband  is configured in a DL symbol, PRACH and Msg 3 PUSCH transmission in UL subband of the SBFD symbol would be beneficial for RACH procedure latency reduction. In legacy RACH procedure, UE doesn’t monitor RAR in symbol(s) with valid RO. If RO in UL subband in SBFD symbol is regarded as valid, it should not be precluded that UE can monitor RAR in DL subband of the symbol, when UE doesn’t transmit PRACH on any valid RO in the symbol.

Proposal 13:

- Support PRACH and Msg 3 PUSCH transmission in UL subband in SBFD symbol.

- Support UE monitoring RAR in DL subband in SBFD symbol, if UE doesn’t transmit PRACH in any valid RO on the symbol.

2.2.4 Other aspects

For UL cancellation DCI format 2_4, the time domain position for the indicated resources is obtained by excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon.

## 11.2A Cancellation indication If a UE is provided UplinkCancellation, the UE is provided, in one or more serving cells, search space sets for monitoring the first PDCCH candidate with a CCE aggregation level of LCI CCEs of each search space set for detection of a DCI format 2_4 [5, TS 38.212] with a CI-RNTI provided by ci-RNTI as described in clause 10.1. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier, if the serving cell is configured with a SUL carrier - an information payload size for DCI format 2_4 by dci-PayloadSize-ForCI - an indication for time-frequency resources by timeFrequencyRegion For a serving cell having an associated field in a DCI format 2_4, for the field denote by - NCI a number of bits provided by ci-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, from a number of symbols that - is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or - is equal to the PDCCH monitoring periodicity, otherwise. - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

Considering SBFD operation with UL subband in DL symbol, there may be UL transmission in DL symbol. With the legacy rule, UL transmission in UL subband in SBFD symbol configured as DL can’t be cancelled by DCI 2_4. Therefore, impact on UL cancellation DCI format 2_4 should be studied.

Proposal 14: Study SBFD impact on UL cancellation DCI format 2_4.

2.3 Cross-link interference for SBFD

As guided by RAN1#109-e agreements, interference management enhancements common for dynamic TDD and SBFD will be discussed in AI 9.3.3, and interference management enhancements specific for SBFD will be discussed in AI 9.3.2, including gNB self-interference, UE-to-UE inter-subband CLI, and gNB-to-gNB inter subbband CLI.

UE-to-UE inter-subband CLI

In Rel-16 dynamic TDD, CLI-RSSI and SRS-RSRP measurement/reporting are supported for UE-to-UE inter-cell CLI management. However, only CLI-RSSI measurement and reporting is applicable for both inter-subband and intra-subband CLI, while SRS-RSRP is not applicable for inter-subband CLI (since legacy Rel-16 SRS-RSRP can be measured only in DL subband). Therefore, legacy SRS-RSRP measurement and reporting is not applicable for SBFD, and only CLI-RSSI measurement and reporting can be reused for SBFD.

Observation 1: Rel-16 legacy SRS-RSRP measurement and reporting is not applicable for SBFD.

Regarding possible enhancements for CLI-RSSI measurement/reporting for SBFD, enhancements like beam specific CLI measurement/reporting and Layer 1 based CLI measurement/reporting as discussed in companion contribution [3] in AI 9.3.3 are also applicable for SBFD.

Observation 2: Enhancements like beam specific CLI measurement/reporting and Layer 1 based CLI measurement/reporting are also applicable for SBFD.

Some SBFD specific aspects can also be considered. For example, for the legacy CLI-RSSI measurement, one contiguous subband can be configured for CLI-RSSI measurement resource. With SBFD operation, there may be non-contiguous DL subbands in SBFD symbol. On the other hand, with SBFD operation, CLI interference level at different RB locations may vary. CLI reporting with finer granularity is beneficial to identify different interference levels at different RB locations. Therefore, multiple non-contiguous or contiguous subbands for CLI-RSSI measurement resource would be beneficial.

Proposal 15: Study CLI-RSSI measurement and reporting with multiple subbands for CLI-RSSI measurement resource.

Inter-gNB coordination

In Rel-16 inter-gNB coordination, TDD patterns can be exchanged. With SBFD operation, exchanging more detailed DL/UL subband allocation would be beneficial for interference mitigation. Therefore, DL/UL subband allocation information also needs to be exchanged.

Proposal 16: Support inter-gNB coordination enhancement with DL/UL subband allocation exchanged between gNBs.

- Conclusion

In this contribution, we discussed subband non-overlapping full duplex. We have the following observations and proposals:

Observation 1: Rel-16 legacy SRS-RSRP measurement and reporting is not applicable for SBFD.

Observation 2: Enhancements like beam specific CLI measurement/reporting and Layer 1 based CLI measurement/reporting are also applicable for SBFD.

Proposal 1:

- If guard band between DL and UL subband in SBFD symbol is required, either DL subband  frequency location or guard band size should be explicitly indicated.

- If guard band between DL and UL subband in SBFD symbol is not required, DL subband  frequency location can be implicitly determined based on UL subband frequency location.

Proposal 2:

- Study dynamic indication of DL/UL subband time domain location, at least for converting an SBFD symbol into non-SBFD symbol.

- NOT support dynamic indication of DL/UL subband frequency domain location.

Proposal 3:

- Support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon, at least for symbol configured as DL by TDD-UL-DL-Dedicated.

- Study whether/how to support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon and TDD-UL-DL-Dedicated, with considering interaction with dynamic SFI indication.

Proposal 4:

- Support only DL reception in DL subband (i.e. not support UL transmission in DL subband).

- Support only UL transmission in UL subband (i.e. not support DL reception in UL subband).

- Not support UL transmission outside UL subband in SBFD symbol.

- Not support DL transmission outside DL subband in SBFD symbol.

Proposal 5: For single DCI scheduling multiple PDSCHs/PUSCHs, study possible enhancements when any of the multiple scheduled PDSCHs/PUSCHs overlapping with UL/DL subband.

Proposal 6:

- For collision handling of higher layer configured channels/signals with SBFD DL/UL subband, study possible enhancements for channels/signals with higher priority first, e.g. PDCCH as highest priority for study.

- Study collision for PDSCH/PUSCH/PUCCH repetitions with UL or DL subband in SBFD symbol.

Proposal 7: For Rel-18 subband non-overlapping full duplex, at least for higher layer configured channels/signals, three alternatives can be considered:

- Alt 1: UE does not expect collision of higher layer configured DL reception or UL transmission with UL subband or DL subband in SBFD symbol.

- Alt 2: Separate higher layer DL reception or UL transmission configuration for SBFD symbols and non-SBFD symbols, e.g. separate PDCCH monitoring configuration or PUCCH resource configuration.

- Alt 3: No separate configuration, but special handling is needed if higher layer configured DL/UL channel/signal colliding with UL/DL subband in SBFD symbol, e.g. the DL/UL channel/signal is canceled or rate matched around DL/UL subband, or different interpretations are applied for frequency resource allocation for SBFD symbol and non-SBFD symbol.

Proposal 8: Study SBFD impact on PUSCH repetition type B segmentation.

Proposal 9: Study SBFD impact on SPS HARQ-ACK deferring.

Proposal 10: Study possible enhancements on CSI-RS configuration and CSI reporting, including:

- Study non-contiguous RB allocation for CSI-RS or narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs).

- Study possible enhancements for CSI reporting overhead reduction for SBFD, e.g. to allow different frequency band configurations (i.e. different starting PRB and num-ber of RBs) for CSI-RS resources in the same CSI-RS resource set, or to associate different CSI-RS resources with different frequency band configurations and treat them as one “effec-tive CSI-RS”.

- Study whether to separately report CSI for non-SBFD symbol/slot and for SBFD symbol/slot.

Proposal 11: For DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol, study predefined rules to determine whether UE will receive in DL subband or transmit in UL subband in the same SBFD symbol.

- To define the predefined rules, following general principles can be considered:

  - Whether the channels/signals are scheduled by DCI

  - Whether the channels/signals are periodic or semi-persistent

  - Whether the channels/signals are with repetitions or without repetitions

Proposal 12: Not support SBFD operation with UL subband on SSB symbol or CORESET #0 symbol.

Proposal 13:

- Support PRACH and Msg 3 PUSCH transmission in UL subband in SBFD symbol.

- Support UE monitoring RAR in DL subband in SBFD symbol, if UE doesn’t transmit PRACH in any valid RO on the symbol.

Proposal 14: Study SBFD impact on UL cancellation DCI format 2_4.

Proposal 15: Study CLI-RSSI measurement and reporting with multiple subbands for CLI-RSSI measurement resource.

Proposal 16: Support inter-gNB coordination enhancement with DL/UL subband allocation exchanged between gNBs.

References

- RP-220633, Revised SID: Study on evolution of NR duplex operation, March 2022, CMCC

- R1-2209901, Discussion on evaluation on NR duplex evolution, October 2022, NTT Docomo

- R1-2209903, Discussion on potential enhancements on dynamic/flexible TDD, October 2022, NTT Docomo

- 3GPP, RAN1#109-e meeting, Chairman’s notes

- 3GPP, RAN1#110 meeting, Chairman’s notes

- 3GPP, RAN1#110bis-e meeting, Chairman’s notes