3GPP TSG RAN WG1 #111			     R1-2211986

Toulouse, France, November 14th – 18th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on channel design framework in SL-U

Agenda Item:	9.4.1.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#110bis-e meeting [1], there was discussion on channel design framework in SL-U. In this contribution, we share our further views on channel design framework in SL-U.

- Discussions

  - Resource pool

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.T Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission Agreement At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

    - Intra-cell guard band (ICGB)

At the last RAN1 meeting, it was agreed that ICGB of two adjacent RB sets is used when a transmission is performed across the two adjacent RB sets. For example, let us assume 10 PRBs of sub-channel size with SCS = 30 kHz and 40 MHz bandwidth. In this example, 50 PRBs are available for each RB-set and 6 PRBs are set to ICGB as the following illustration, according to 38.101-1. 5 interlaces are defined and thereby 5 sub-channels can be composed for each RB-set. Then, how to use the 6 PRBs needs to be discussed. Now there are 10 sub-channels in total and 10 PRBs are included in each. Each of the 6 PRBs would be associated with one of the 10 sub-channels. Here we believe that there are at least 5 issues for ICGB usage: 1) unequal sub-channel size, 2) unsupported sub-channel size, 3) TBS determination, 4) PSFCH/S-SSB handling.

Fig.1: An example of two RB sets with an ICGB.

For issues 1)2), if a PRB within ICGB is treated as a part of a sub-channel, this means that some sub-channels are composed of 11 PRBs and the other sub-channels are of 10 PRBs. The resource pool includes two different sub-channel sizes, which has not been supported so far. In addition, supported sub-channel sizes are 10, 12, 15, 20, …, 100 PRBs. 11 PRBs have not been supported as a sub-channel size. To solve this issue without large spec impact, we believe that (pre-)configured sub-channel size is maintained as in R16/17 SL regardless of ICGB configuration and whether ICGB is used for each transmission or not. Meanwhile, only for the purpose of 2nd-SCI/SL-SCH rate-matching and 2nd-SCI/SL-SCH/RS mapping of a transmission, the PRBs within ICGB that are associated with sub-channels where the transmission is performed are used with the sub-channels so that appropriate signal can be mapped with appropriate information amount. Note that definition of the associations between sub-channels and PRBs within ICGB is dependent on issues on frequency-domain resource indication. If the same interlace shall be used across RB-sets, the associations mean just belonging to the same interlace; otherwise, explicit definition on the associations is necessary.

For issue 3), the above direction implies that TBS determination is performed according to the (pre-)configured sub-channel size and thereby TBS is constant regardless of whether ICGB is used or not. Maintaining TBS is essential for retransmissions of a TB; thus this mechanism would be the most reasonable.

For issue 4), PSFCH/S-SSB should be defined as a channel/signal within an RB-set. There is no reason to wider bandwidth, which leads to more frequent LBT failure.

Fig.2: Sub-channel definition excluding ICGB.

Proposal 1:

- (Pre-)configurable sub-channel size is kept from R16/17 SL regardless of ICGB configuration, and TBS is calculated according to the (pre-)configured sub-channel size.

- Only for 2nd-SCI/SL-SCH rate-matching and 2nd-SCI/SL-SCH/RS mapping of a transmission, the PRBs within ICGB that are associated with sub-channels where the transmission is performed are used with the sub-channels.

- PSFCH/S-SSB transmission does not use PRBs within ICGB.

    - Additional S-SSB slot vs Resource pool

It was agreed that conventional S-SSB slots are excluded from SL RP as in R16/17, but additional candidate S-SSB occasions have not been concluded yet. In our view, the additional S-SSB slots should also be excluded from SL RP as the conventional S-SSB slots. If additional S-SSB slots are included in RP and thereby multiplexing/prioritization with PSSCH is performed, resource allocation mechanism becomes quite complicated. Besides, a lot of UEs may perform S-SSB transmission at the additional S-SSB slots; that is, they cannot communicate each other. Benefit to use the slots for e.g. data transmission is highly questionable.

Proposal 2:

- Additional S-SSB slots are excluded from SL resource pool.

    - RP with subset of PRBs belonging to an RB set

Although a resource pool with one or more RB sets were agreed at the previous meeting, a resource pool with subset of an RB set is FFS. In our understanding, this FFS was added to consider multiple resource pools within an RB set. In Rel-16/17 SL, there is no restriction on (pre-)configuring multiple resource pools for a set of RBs, as long as the sub-channel size is aligned with the specification. In this sense, it should be possible to (pre-)configure multiple resource pools within an RB set.

Fig. 3: Multiple resource pools within an RB set

This operation is illustrated above. In this figure, three interlaces (i.e., three sub-channels) are belonging to resource pool#1, and the remaining two interlaces are included in resource pool#2. As we can see, this operation is possible and feasible, when interlaced RB-based approach is applied. Detailed structure would need to be discussed including a case of multiple RB sets. On the other hand, in contiguous RB-based structure, such a subset-based structure would not be allowed since OCB/PFD requirement is not satisfied.

Observation 1:

- Multiple resource pools within an RB set are possible and feasible when interlaced RB-based structure is applied.

Proposal 3:

- Support a resource pool including sub-set of PRBs belonging to an RB set in interlaced RB-based structure.

  - Not supported for contiguous RB-based structure.

  - PSCCH/PSSCH structure

    - Channel/signal mapping

Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details

For interlaced structure, how to map PSCCH/PSSCH including RS needs to be clarified.

- Control/Data/RS mapping

In interlaced RB-based structure, how to map control/data/RS is an issue to be solved. In short, our view is that the conventional mapping is applied to IRBs and then IRB-to-PRB mapping is performed. We do not see any issue on this simplest way. Other procedure would be large spec impact; thus the above procedure should be supported unless critical issue is found.

Fig. 4: Control/Data/RS mapping for interlaced RB-based structure

Proposal 4:

- For interlaced RB-based structure, PSCCH/PSSCH is mapped on IRBs as on PRBs in Rel-16/17, and then IRB-to-PRB mapping is performed.

- PSCCH location

One issue on PSCCH mapping that was raised at the last meeting is whether PSCCH should be mapped on every RB set of corresponding PSSCH. Motivation of this mechanism is to support a new resource pool where UEs supporting different bandwidths can communicate each other. In R16/17 resource pool, all UEs using this resource pool shall support its full bandwidth so that especially sensing mechanism can work well and complicated resource selection behavior can be avoided.

Although it was argued that the same resource pool should be available by UEs supporting different bandwidths e.g., for commercial services, it is not aligned with 3GPP sidelink specification so far, even R17 SL, where commercial UEs are one of the target services

Proposal 5:

- Do not support Option 2 on PSCCH location.

    - Sub-channel definition

Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets

There was discussion on relationship between sub-channel and interlace at the last meeting, and K values for each sub-channel were agreed. Meanwhile, whether each sub-channel is confined withing a single RB set or can be defined across multiple RB sets has not been concluded yet. Alt 1 vs Alt 2 can be illustrated as below.

Fig.5: Alternatives for sub-channel definition.

In Alt 1, each transmission with a small number of sub-channels is performed at an RB-set. After LBT only at the LBT channel corresponding to the RB-set, the UE can perform the transmission. In other words, LBT at other LBT channel is unnecessary for this transmission. Probability of LBT failure can be lower and excessive channel occupation can be avoided, compared to Alt 2 since any PSCCH/PSSCH transmission in Alt 2 will use multiple RB sets and thereby LBT at multiple LBT channels is always required.

Observation 2:

- A sub-channel composed of PRBs belonging to only a same RB set can achieve lower probability of LBT failure for each transmission and can avoid excessive channel occupation.

Proposal 6:

- For interlaced RB-based structure in a resource pool with multiple RB sets, each sub-channel is composed of PRBs belonging to only a same RB set (i.e., support Alt 1).

    - Maximum 2 candidate starting symbols

Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

Maximum 2 candidate starting symbols were agreed as a working assumption in order to solve resource efficiency issue due to transmission cancellation due to LBT failure. For this mechanism, we think there are at least four issues to be discussed.

- Availability

Although a mechanism of max 2 candidate starting symbols was agreed, our understanding is that allowing TX from the 2nd starting position without any restriction was not intended. For example, allowing TX from the 2nd starting position without TX attempt from the 1st starting position is not reasonable way in consideration of the abovementioned motivation of this feature. Besides, defining structure for all sl-LengthSymbols values is not preferred from workload perspective compared to efficiency. In SL-U, sl-LengthSymbols = 14 is the typical situation and hence there is no/little motivation to support this feature for sl-LengthSymbols != 14. From the same reason, this mechanism of max 2 candidate starting symbols is unnecessary at PSFCH-slot.

Proposal 7:

- 2nd starting symbol is available only when the following conditions are met.

  - The UE attempted the transmission from the 1st starting symbol but the LBT was failed.

  - (Pre-)configured sl-LengthSymbols value is 14.

  - The slot does not include a PSFCH occasion.

- AGC symbols

Whether 2nd AGC symbol within a slot is necessary for the case where transmission starts from the 1st starting symbol or not was discussed but has not been concluded yet. The key point would be whether it is feasible that there are two transmissions at the same slot where one transmission starts from the 1st starting symbol and the other transmission starts from the 2nd starting symbol, and whether such a case leads to large impact on reception performance.

In our view, the above situation is feasible and the impact is not so small. For example, let us assume there are three UEs. The 1st UE can detect the 2nd UE and the 3rd UE, but the 2nd UE and the 3rd UE cannot detect each other. Then the 2nd UE starts transmission from the 1st starting symbol and the 3rd UE starts transmission from the 2nd starting symbol as illustrated below. In this case, definitely power level detected at the 1st UE will change at the 2nd starting symbol and thus AGC issue would occur at the 1st UE. If the number of UEs starting transmission from the 2nd starting symbol is larger, the AGC issue becomes more significant. We discussed such a hidden-node issue in R17 IUC agenda, this implies that this situation is not rare.

Fig.6: Typical situation of AGC issue caused by the 2nd starting symbol.

Observation 3:

- Due to hidden-node issue, it could occur that some UEs start from 1st starting symbol and other UEs start from 2nd starting symbol within the same slot.

According to the above analysis, we believe that the 2nd AGC symbol is necessary when 2 candidate starting symbols are available. As the 1st AGC symbol, signal copy from the next symbol should be applied.

Proposal 8:

- If 2 candidate starting symbols are (pre-)configured, AGC is applied twice within a single slot. The 2nd starting symbol is copied from the next symbol.

- Processing time constraints

When TX is started from the 2nd starting symbol, timing of completing decoding of each channel would change. For example, PSSCH decoding timing may be later compared to the case where TX is started from the 1st starting symbol since decoding completion of 1st SCI and 2nd SCI will be later. Correspondingly, PSCCH/PSSCH to PSFCH slot offset should be larger than the existing sl-MinTimeGapPSFCH values. Besides, PSCCH decoding timing will be later since PSCCH location becomes later. Timing of obtaining reservation information becomes later; hence ending timing of sensing window should be modified so that excessively small processing time is not required.

Fig.7: Issue - existing processing time constraints with 2nd starting symbol
(upper: PSSCH to PSFCH, lower: sensing to exclusion)

Proposal 9:

- Update the following processing time constraints.

  - PSCCH/PSSCH to PSFCH slot offset is sl-MinTimeGapPSFCH + 1.

  - The last monitored PSCCH occasion for a resource selection is the first PSCCH occasion at the last slot within the sensing window.

- Untransmitted CB

When TB includes multiple CBs, TX from the 2nd starting symbol may not help to obtain positive acknowledgment since CB with higher index is not mapped in the transmission. This issue can be illustrated as below. Here there are two CBs in a TB. When the TX is started from the 1st starting symbol, both CB#0 and CB#1 will be transmitted. However, when the TX is started from the 2nd starting symbol, only CB#0 would be transmitted due to lack of available REs. When an initial TX is performed from the 2nd starting symbol, obviously the retransmission from the 2nd starting symbol with the same CB mapping is quite inefficient and any other mechanism should be introduced.

Fig.8: Issue - Untransmitted CB when 2nd starting symbols is used.

Observation 4:

- When a TX of a TB is started from the 2nd starting symbol, some CBs of the TB would not be transmitted.

Proposal 10:

- Study how to perform efficient transmission of a TB with multiple CBs when 2 starting positions are available.

    - Resource indication

Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details

A controversial issue on PSCCH/PSSCH is frequency-domain resource indication. In our understanding, the main point is whether the same interlace index(s) of multiple RB-sets shall be used for a PSSCH transmission or not. It was argued that the motivation is lower PAPR and thus NR-U introduced this restriction. However, there is trade-off between PAPR and resource allocation flexibility. Compared to Uu, SL resource allocation flexibility is lower; hence discussion from both aspects would be necessary.

In our view, Option B + Option 2 is the best way to achieve both lower PAPR and better resource allocation flexibility. This is illustrated in the following figure. Here sub-channel indices are defined per RB set, and association between sub-channel indices and interlaces are the same. Then UE can indicate sub-channel index(s) per RB set. The same interlace is associated with the same sub-channel index across RB sets so that the same interlace(s) across RB sets can be indicated in the existing mechanism of contiguous sub-channel indices indication. When lower PAPR is required, the same interlaces can be selected; if congestion level is high, the flexible resource usage is beneficial.

Fig.9: Freq-domain resource indication for both lower PAPR and better flexibility.

Observation 5:

- A mechanism to enable both indication of the same interlace index(s) across RB sets and indication of different interlace indices across RB sets should be introduced.

Proposal 11:

- For frequency-domain resource indication in a RP with multiple RB sets,

  - Option B + Option 2 are supported with the following sub-bullets.

    - Sub-channel indices are defined per RB set.

    - Among RB sets, the same sub-channel index is associated with the same interlace index.

    - Each SCI indicates sub-channel indices per RB set.

  - PSFCH structure

Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives

    - Too small PSFCH capacity

One issue on PSFCH format with interlaced structure as PUCCH format 0 in NR-U is small PSFCH capacity. In Rel-16/17, when X PRBs and Y CSs are available for PSFCH, there are XY PSFCH resources in a PSFCH occasion. However, if the interlaced structure of PUCCH format 0 is reused for PSFCH format 0 without any enhancement (labeled Alt 0), each PSFCH resource occupies multiple PRBs and hence available PSFCH resources become much less than XY. Study to solve this issue would be necessary, and the above three alternatives were identified at the last meeting. In NR-U, the capacity issue is not critical since any time-frequency resources are available for PUCCH; but this is not the case in SL-U, where PSFCH can be transmitted at only (pre-)configured and associated PSFCH occasion.

Here we will discuss the alternatives in an example of 20MHz channel bandwidth with 50 PRBs and 5 interlaces. Alt 3 is skipped since it seems that Alt 3 is not using interlace and hence is not aligned with

- Alt 0 (No enhancement)

In this alternative, as abovementioned, the maximum PSFCH capacity in a single PSFCH occasion is 5 interlaces x 6 CSs = 30. Compared to Rel-16 PSFCH capacity (= 50 x 6 = 300), definitely the capacity is much less.

- Alt 1

This alternative occupies a common interlace. The interlace will not be used for user multiplexing, thereby the remaining PRBs and CSs are utilized for the purpose. The number of the remaining PRBs is 40. Then the PSFCH capacity would be 40 dedicated PRBs x 6 CSs = 240. Although capacity can be improved, we are not sure high PSD in this alternative is OK. When a common interlace is used among UEs, which means that any UE using the PSFCH occasion maps PSFCH on the same PRBs, the total PSD becomes quite high in a lot of situations. This may not be preferable from other system perspective.

- Alt 2 with PRB-level CSs

In this alternative, user multiplexing can be done by interlace-domain, PRB-domain, and CS-domain. Each UE uses an interlace from the 5 interlaces. Then at the interlace, a single shared CS is used for the transmission basically, but at only a single PRB from the PRBs belonging to the interlace, UE-specific CS is allocated for each PSFCH transmission instead of the shared CS. This is illustrated below. PSFCH capacity in this method is 5 interlaces x 10 PRBs x 6 CSs = 300. Compared to Alt 1, better capacity is achieved and less PSD issue is expected.

One note is that PRB with UE-specific CS can be multiple instead of single in order to increase power allocated for the PRB with UE-specific CS. There is trade-off between PSFCH capacity and detection performance.

Fig.10: Intra-cell guard band in Example

- Alt 2 with OCC

In this alternative, users are multiplexed by interlace-domain, OCC-domain, and CS-domain. Each UE uses an interlace from the 5 interlaces, as the previous alternative, and then at the interlace, a CS from the 6 CSs is determined. On the CS, multiple UEs are multiplexed by FD-OCC with the PRBs belonging to the interlace. PSFCH capacity is 5 interlaces x 6 CSs x 10 FD-OCC = 300 as maximum. The same capacity as Alt 2 with PRB-level CSs can be obtained.

One note is that the number of FD-OCC can be smaller to avoid performance degradation due to channel’s frequency-selectivity. There is trade-off between PSFCH capacity and detection performance.

According to analysis so far, we believe that at least Alt 2 should be prioritized over Alt 1. For sub-options of Alt 2, we prefer Alt 2 with PRB-level CSs since the best OCC-level from performance perspective is dependent on channel’s frequency-selectivity, which could be dynamically changed. Meanwhile, the best number of PRBs with UE-specific CS is just relative to power-domain. A fixed number can be determined based on the required SNR. This is better for pre-configuration-based system.

Proposal 12:

- For interlaced PSFCH format 0, support Alt 2 with PRB-level CSs.

  - Each PSFCH transmission occupies an interlace from interlaces available for PSFCH.

  - From PRBs belonging to the interlace, UE-specific CS is used at one or more PRBs and common CS is used at the remaining PRBs.

  - FFS: the number of PRBs with UE-specific CS

    - Location of PSFCH resources

One key point in SL-U would be whether different RB-sets (LBT channels) between PSSCH and PSFCH is allowed or not. In Rel-16/17, naturally there is no restriction on the association from perspective of frequency-domain resource. However, in SL-U, it would be valid that the same RB-set is better so that the two TXs can be performed within the same COT. If they are at different RB-sets, basically COT sharing cannot be applied for the two TXs and as the result, PSFCH transmission skipping due to LBT failure occurs more frequently.

Proposal 13:

- The same RB-set is assigned for PSCCH/PSSCH resource and the corresponding PSFCH resource.

    - PSFCH drop due to LBT failure

In SL-U, any transmission is failed more frequently compared to in licensed spectrum or ITS-band due to LBT failure. Frequent PSFCH dropping would be a big issue since corresponding PSSCH retransmissions will occur and thus, performance of resource efficiency and reliability/latency becomes worse. Some enhancement to solve this issue would be necessary. Therefore, we believe that at least Alt 4 should be deprioritized.

On the remaining alternatives, our view is that Alt 3 should be supported. For example, in Alt 3, HARQ-ACK for each HARQ process can be transmitted via new MAC-CE. Alt 1 leads to quite lower resource efficiency. Alt 2/Alt 5 cause too complicated PSFCH conflict avoidance. Meanwhile, Alt 3 is the easiest way. Besides, concern on Alt 3 may be resource inefficiency due to PSSCH transmission increase. For this issue, Alt 3 should be allowed only when there is PSCCH/PSSCH to be transmitted the same UE. That is, resource efficiency degradation can be avoided such that the MAC-CE is transmitted with other data.

Proposal 14:

- When PSFCH TX is dropped due to LBT failure, if the UE will transmit a TB to the same UE, the HARQ-ACK can be transmitted on PSSCH with the TB.

  - S-SSB structure

Agreement To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS: - Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH - Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3) Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement Regarding additional candidate S-SSB occasions: - Their number and time domain locations are (pre-)configured or pre-defined Proposal 5-1-2a: Regarding additional candidate S-SSB occasions (if supported): - In the same S-SSB period, at least the following are to be studied: - Alt 1: UE attempts to transmits on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE always attempts to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details

    - Additional candidate S-SSB occasions

On S-SSB structure, two aspects need to be discussed. The first one is time-domain location. S-SSB TX would be failed at more S-SSB occasions than in ITS-band/licensed spectrum due to LBT failure. More TX occasions would be desirable to solve this issue, and additional candidate S-SSB occasions were agreed in previous meeting.

On the details, more occasions in slot-base are slightly preferred to avoid large workload. To avoid excessive TX, these candidates should be used only when conventional occasion is unavailable. This is a kind of Alt 1, but the current Alt 1 seems unclear; in ‘when it fails to transmit on R16/17 S-SSB occasion(s)’, which R16/17 occasion(s) is used to determine whether the additional occasion(s) can be used should be clarified. In addition, if this condition is met, how many additional occasions are available is also unclear.

Proposal 15:

- Do not introduce 4 symbols S-SSB.

- Support Alt 1 for usage of additional candidate S-SSB occasions with the following update.

  - UE attempts to transmit on an additional candidate S-SSB occasion only when it fails to transmit on the last R16/R17 S-SSB occasion prior to the additional occasion.

    - OCB/PSD requirement

Option 1 and Option 3 were down-selected for S-SSB structure to meet OCB/PSD requirement, and further down-selection is necessary. In our view, basically Option 1 is better since higher energy per RE can bring better S-SSB detection performance.

On interlaced S-SSB structure, the conventional S-SSB is composed of 11 PRBs (PSBCH) or 127 Res (S-PSS/S-SSS). There are cases where interlaced S-SSB structure can be mapped within the available PRBs of a single 20 MHz; for example, when a single 20 MHz is available with 30 kHz SCS, 51 PRBs are available at the channel as defined in 38.101-1, and 11 PRBs/127 REs can be mapped within the 51 PRBs with intervals of 5 PRBs. This is illustrated below. Note that 50 PRBs should be (pre-)configured for a resource pool so that unequal sub-channel size is avoided.

On the other hand, 38.101-1 defines for NR-U that 49/50/51 PRBs are configurable for each RB-set except for intra-cell guard when wideband operation can be performed. For transmission of the interlaced S-SSB structure, it seems that 51 PRBs should be available at the RB-set. However, if 51 PRBs are (pre-)configured for each RB-set, this leads to unequal sub-channel size, which should not be allowed as in R16/17. It would be impossible to avoid this situation by RP (pre-)configuration. Therefore, we believe that a possible way is that 50 PRBs are defined/(pre-)configured for each RB-set, and S-SSB transmission can use one adjacent PRB outside of a RB-set.

On exemption of OCB requirement, the regulation text is ‘During a COT, equipment may operate temporarily with an OCB of less than 80% of…’, which would mean that when the COT includes more than one transmission and (not all but) some of them can ignore the OCB requirement. If the COT includes only S-SSB TX, the case does not satisfy ‘temporarily’. To avoid complex operation, we believe that the exemption should be deprioritized.

Proposal 16:

- For frequency-domain structure of S-SSB, Option 1 is supported.

  - For wideband operation with 30 kHz SCS, each RB-set is composed of 50 PRBs and interlaced S-SSB is mapped within a single RB-set and a PRB adjacent to the RB-set.

- Exemption of OCB requirement is not applied to S-SSB TX.

- Fig.11: Issue – interlaced S-SSB structure over a single RB-set.

- Conclusion

In this contribution, we discussed channel design framework in SL-U. Observations/Proposals are summarized as following:

Proposal 1:

- (Pre-)configurable sub-channel size is kept from R16/17 SL regardless of ICGB configuration, and TBS is calculated according to the (pre-)configured sub-channel size.

- Only for 2nd-SCI/SL-SCH rate-matching and 2nd-SCI/SL-SCH/RS mapping of a transmission, the PRBs within ICGB that are associated with sub-channels where the transmission is performed are used with the sub-channels.

- PSFCH/S-SSB transmission does not use PRBs within ICGB.

Proposal 2:

- Additional S-SSB slots are excluded from SL resource pool.

Observation 1:

- Multiple resource pools within an RB set are possible and feasible when interlaced RB-based structure is applied.

Proposal 3:

- Support a resource pool including sub-set of PRBs belonging to an RB set in interlaced RB-based structure.

  - Not supported for contiguous RB-based structure.

Proposal 4:

- For interlaced RB-based structure, PSCCH/PSSCH is mapped on IRBs as on PRBs in Rel-16/17, and then IRB-to-PRB mapping is performed.

Proposal 5:

- Do not support Option 2 on PSCCH location.

Observation 2:

- A sub-channel composed of PRBs belonging to only a same RB set can achieve lower probability of LBT failure for each transmission and can avoid excessive channel occupation.

Proposal 6:

- For interlaced RB-based structure in a resource pool with multiple RB sets, each sub-channel is composed of PRBs belonging to only a same RB set (i.e., support Alt 1).

Proposal 7:

- 2nd starting symbol is available only when the following conditions are met.

  - The UE attempted the transmission from the 1st starting symbol but the LBT was failed.

  - (Pre-)configured sl-LengthSymbols value is 14.

  - The slot does not include a PSFCH occasion.

Observation 3:

- Due to hidden-node issue, it could occur that some UEs start from 1st starting symbol and other UEs start from 2nd starting symbol within the same slot.

Proposal 8:

- If 2 candidate starting symbols are (pre-)configured, AGC is applied twice within a single slot. The 2nd starting symbol is copied from the next symbol.

Proposal 9:

- Update the following processing time constraints.

  - PSCCH/PSSCH to PSFCH slot offset is sl-MinTimeGapPSFCH + 1.

  - The last monitored PSCCH occasion for a resource selection is the first PSCCH occasion at the last slot within the sensing window.

Observation 4:

- When a TX of a TB is started from the 2nd starting symbol, some CBs of the TB would not be transmitted.

Proposal 10:

- Study how to perform efficient transmission of a TB with multiple CBs when 2 starting positions are available.

Observation 5:

- A mechanism to enable both indication of the same interlace index(s) across RB sets and indication of different interlace indices across RB sets should be introduced.

Proposal 11:

- For frequency-domain resource indication in a RP with multiple RB sets,

  - Option B + Option 2 are supported with the following sub-bullets.

    - Sub-channel indices are defined per RB set.

    - Among RB sets, the same sub-channel index is associated with the same interlace index.

    - Each SCI indicates sub-channel indices per RB set.

Proposal 12:

- For interlaced PSFCH format 0, support Alt 2 with PRB-level CSs.

  - Each PSFCH transmission occupies an interlace from interlaces available for PSFCH.

  - From PRBs belonging to the interlace, UE-specific CS is used at one or more PRBs and common CS is used at the remaining PRBs.

  - FFS: the number of PRBs with UE-specific CS

Proposal 13:

- The same RB-set is assigned for PSCCH/PSSCH resource and the corresponding PSFCH resource.

Proposal 14:

- When PSFCH TX is dropped due to LBT failure, if the UE will transmit a TB to the same UE, the HARQ-ACK can be transmitted on PSSCH with the TB.

Proposal 15:

- Do not introduce 4 symbols S-SSB.

- Support Alt 1 for usage of additional candidate S-SSB occasions with the following update.

  - UE attempts to transmit on an additional candidate S-SSB occasion only when it fails to transmit on the last R16/R17 S-SSB occasion prior to the additional occasion.

Proposal 16:

- For frequency-domain structure of S-SSB, Option 1 is supported.

  - For wideband operation with 30 kHz SCS, each RB-set is composed of 50 PRBs and interlaced S-SSB is mapped within a single RB-set and a PRB adjacent to the RB-set.

- Exemption of OCB requirement is not applied to S-SSB TX.

References

- 3GPP RAN1#110bis-e meeting,	chair’s notes