# 3GPP TSG-RAN WG1 Meeting #111														     R1-2212095

Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on NACK-only mode 2 multicast feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on RAN2 spec, the higher layer moreThanOneNackOnly if configured indicates the NACK-only ‘mode2’. The current specification for the UE behavior The current specification for the UE behavior is not clear when the UE is configured with moreThanOneNackOnlyMode or when the UE is not configured with moreThanOneNackOnlyMode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When moreThanOneNackOnlyMode is configured, the UE behavior should be clarified as - The UE generate the HARQ-ACK information bits for the G-RNTI configured with second reorting mode based on ‘mode2’ as specifie in Table 18-1, including 1~4bits. - The UE does not expect to multiplex more than 4bits in the same PUCCH associated with the second reporting mode. - If the PUCCH with multicast NACK only mode2 feedback collides with other PUCCH/PUSCH, UE does not transform the NACK-only mode 2 to mode 1. It will be too complicated for UE to prepare different types of codebooks dependent on dynamically scheduled PUCCH/PUSCH. When moreThanOneNackOnlyMode is not configured, the UE behavior should be clarified as - If the PUCCH with multicast NACK only feedback collides with other PUCCH/PUSCH, UE applies NACK-only mode 1 and transform the NACK-only into ACK/NACK. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is not clear when the UE is configured with higher layer parameter moreThanOneNackOnlyMode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 18	Multicast Broadcast Services

< Unchanged parts are omitted >

For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2.

For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception.

For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode as ‘mode2’ to provide associated HARQ-ACK information bits in a PUCCHby selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

For a UE that is indicated the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode, all PUCCH resources associated with the second HARQ-ACK reporting mode have same starting symbol and same number of symbols and the UE is not expected to multiplex more than 4 HARQ-ACK information bits in the PUCCH associated with the second HARQ-ACK reporting mode.

Table 18-1: Mapping of values of HARQ-ACK information bits to PUCCH resources for the second HARQ-ACK reporting mode with ‘mode2’

| Value of HARQ-ACK information bits |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| {0} | {0,0} | {0,0,0} | {0,0,0,0} | 1st PUCCH resource from resourceList |
|  | {1,0} | {1,0,0} | {1,0,0,0} | 2nd PUCCH resource from resourceList |
|  | {0,1} | {0,1,0} | {0,1,0,0} | 3rd PUCCH resource from resourceList |
|  |  | {1,1,0} | {1,1,0,0} | 4th PUCCH resource from resourceList |
|  |  | {0,0,1} | {0,0,1,0} | 5th PUCCH resource from resourceList |
|  |  | {1,0,1} | {1,0,1,0} | 6th PUCCH resource from resourceList |
|  |  | {0,1,1} | {0,1,1,0} | 7th PUCCH resource from resourceList |
|  |  |  | {1,1,1,0} | 8th PUCCH resource from resourceList |
|  |  |  | {0,0,0,1} | 9th PUCCH resource from resourceList |
|  |  |  | {1,0,0,1} | 10th PUCCH resource from resourceList |
|  |  |  | {0,1,0,1} | 11th PUCCH resource from resourceList |
|  |  |  | {1,1,0,1} | 12th PUCCH resource from resourceList |
|  |  |  | {0,0,1,1} | 13th PUCCH resource from resourceList |
|  |  |  | {1,0,1,1} | 14th PUCCH resource from resourceList |
|  |  |  | {0,1,1,1} | 15th PUCCH resource from resourceList |

If a UE is provided pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for each G-RNTI or G-CS-RNTI that the UE provides associated HARQ-ACK information according to the first HARQ-ACK reporting mode or the second HARQ-ACK reporting mode, respectively. For HARQ-ACK information associated only with the second HARQ-ACK reporting mode, when the UE is not provided moreThanOneNackOnlyMode and the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode and in response to at least one DCI format detection, the UE determines a PUCCH resource as described in clause 9.2 from pucch-ConfigurationListMulticast1, if provided; otherwise, the UE determines a PUCCH resource as described in clause 9.2 from pucch-Config/pucch-ConfigurationList.

A PDSCH reception providing an initial transmission of a transport block is scheduled only by a multicast DCI format. For the first HARQ-ACK reporting mode, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a multicast DCI format using a same G-RNTI as the G-RNTI of the initial transmission of the transport block, or by a unicast DCI format using a C-RNTI [6, TS 38.214].

An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214].

A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212].

If a UE would multiplex multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the multicast HARQ-ACK information according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or according to the second HARQ-ACK reporting mode when the UE is provided moreThanOneNackOnlyMode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'.

< Unchanged parts are omitted >