3GPP TSG RAN WG1 #111			R1- 2212103

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.1.3.2

Source: 	Qualcomm Incorporated

Title: 	SRS enhancement for TDD CJT and 8 Tx operation

Document for:	Discussion/Decision

# Introduction

In RAN plenary #94e, the work item on MIMO Evolution for Downlink and Uplink was approved ‎[1]. One of the main objectives of the work item is SRS enhancement, which is involved into two sub objectives as listed below

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, the design aspects on the following two topics are discussed.

- SRS enhancement for TDD CJT

- SRS enhancement for 8 Tx UL transmission

# SRS enhancement for TDD CJT

In this contribution, we discuss potential SRS enhancements for TDD CJT separately for enhanced randomization, enhanced capacity/efficiency of SRS, enhanced signaling, and power control enhancements.

## Enhanced interference randomization

The following was agreed in RAN1 #110-bis-e for cyclic shift hopping and comb offset hopping:

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

In the current spec, only sequence hopping or group hopping is supported to randomize SRS interference. In both cases, randomization is in SRS sequence domain / code domain. However, different SRS sequences still create interference when the corresponding SRS resources overlap in RE domain (same symbol, same RBs, and same comb offset). In other words, sequence hopping, group hopping, or even cyclic shift hopping cannot address interference randomization wrt large-scale interference profile, i.e., a strong persistent interferer on the same REs cannot be avoided by code domain randomization techniques.

On the other hand, both comb offset hopping as well as pseudo-random muting of SRS can randomize inter-cell / inter-cluster interference by avoiding persistent interference on the same REs from two interfering UEs. This provides more tools to the network for interference randomization and can be more effective than SRS sequence randomization with group hopping or sequence hopping, or cyclic shift hopping which do not provide interference randomization in RE domain, and hence, cannot avoid a persistent interference from a strong interferer.

It should be noted that for interference randomization by cyclic shift hopping, the SRS sequences of two interfering UEs in different cells / clusters should be different (otherwise, both SRS sequence and cyclic shift may become the same in some instances). As a result, cyclic shift hopping does not result in avoiding interference in the code domain (when SRS sequences are different, different cyclic shifts are not orthogonal). That is, the benefit of cyclic shift hopping is limited to diversifying the interference pattern in the code domain without orthogonality in the code domain.

To realize the benefit of “diversifying the interference pattern in the code domain” (but w/o orthogonality in the code domain as discussed above), another simpler approach can be combinations of group hopping and sequence hopping. In the current specification, these are supported individually, which means that we can either hop across 30 sequences or hop across 2 sequences. However, hopping across all available 60 sequences is not currently possible, which requires minimal spec changes compared to cyclic shift hopping.

The performance of comb offset hopping is evaluated using a similar set-up as the evaluation of pseudo-random muting. In fact, these two schemes can achieve similar impact for inter-cell / inter-cluster interference as discussed above. We consider indoor hot spot layout with 12 TRPs with three clusters each with 4 TRPs, and random drops each with 1 UE per TRP (the user CDF plots are aggregated across 100 random drops). For both cases of w/o and w/ comb offset hopping, it is ensured that UEs in the same cluster do not transmit SRS on the same Comb offset (4 different comb offsets are assigned for the 4 UEs in the same cluster). For each drop, 100 slots are simulated, and the median UL SINR per UE is plotted wrt both the strongest TRP (solid line) as well as the second strongest TRP (dashed line) subject to the condition that PL delta compared to the strongest TRP is not larger than 6dB. We use OLPC for SRS, where the PL in OLPC formula is wrt the strongest TRP for each UE. As it can be seen in Figure 1, the tail UE UL SINR is improved thanks to interference randomization with comb offset hopping.

Figure 1: Comb offset hopping to randomize inter-cell/cluster interference.

Next, we evaluate the performance of cyclic shift hopping through LLS. Note that SLS through UL SINR (similar to the evaluation of comb offset hopping or pseudo-random muting) is not meaningful for cyclic shift hopping as the randomization is in the code domain. Hence, LLS is considered for this evaluation. We consider two UEs each with 2 SRS ports, and with different SRS sequences for the two UEs to simulate the inter-cell/cluster interference. One UE is the intended UE, and another UE is the interferer UE. Without cyclic shift hopping, both UEs use cyclic shift {0,6} across the two ports (but with different SRS sequences). With cyclic shift hopping, the intended UE is still using the same cyclic shift {0,6}, but the interferer UE is using {SC, (SC+6) mod 12}, where SC is randomly selected between {0,1,…,11} in different slots to see the impact of randomizing interference in the cyclic shift domain. Note that collision of SRS sequence is not a factor in this simulation as the two SRS sequences for the two UEs are fixed and are different, i.e., the focus is on interference randomization in cyclic shift domain assuming different SRS sequences are assigned in different cells / clusters. Figure 2 shows the normalized MSE of SRS channel estimation error for both cases of no cyclic shift hopping and with cyclic shift hopping for 10%ile, 50%ile, and 90%ile MSE across 10K slots. As it can be observed, the gain of cyclic shift hopping is negligible (<0.1-0.2 dB in MSE).

Figure 2: Cyclic shift hopping.

To summarize, we observed some gains for comb offset hopping and pseudo-random muting (see the later results in this section), but we did not observe clear gain from cyclic shift hopping. Hence, our view is that at least one of the comb offset hopping and/or pseudo-random muting of SRS should be supported to address the issue of strong persistent interferer in RE domain, and cyclic shift hopping or other code domain randomization enhancements can be further studied.

Proposal 1: For interference randomization, support at least one of a) comb offset hopping, and/or b) pseudo-random muting of SRS.

- FFS: Whether to additionally support cyclic shift hopping and/or combinations of sequence hopping and group hopping

For comb offset hopping, the following needs to be ensured for two UEs with SRS resources transmitted in the same slot and same OFDM symbols (e.g., with the same first symbol for SRS transmission in the slot):

- For inter-cell or inter-cluster interference, randomization should be achieved to avoid persistent interference. Hence, the choice of comb offset should be independent across the two.

- For intra-cell or intra-cluster interference, comb offset hopping consistency should be ensured, i.e., if the two UEs have different comb offsets in a first hopping instance, they should also have different comb offsets in a second hopping instance.

To achieve the above, similar procedure as sequence hopping or group hopping can be used. That is, comb offset is determined based on the pseudo-random sequence c(i) as a function of time (slot number and symbol number) with a certain UE-specific initialization. For this, a comb offset hopping formula can be applied on top of the existing kTC(pi) in 38.211 as shown below:

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcomboffset,hoppingns,fμ,l' mod KTC,

where fcomboffset,hoppingns,fμ,l' can be written based on the pseudo-random sequence c(i) and as a function of time similar to sequence hopping or group hopping. Then, the remaining aspects are

- The unit of applying comb offset hopping: For this, the following choices can be considered:

  - Per SRS symbol: In this case, joint SRS processing across multiple repetitions (for repetition factor R>1) becomes difficult. Hence, it is desired for the comb offset to remain the same on all SRS symbols of a given frequency hop (within the R repetitions). Note that the situation here is different than SRS for positioning in which case a uniform and deterministic comb offset pattern is specified for UE to be able to sound all REs (SRS staggering), and hence, joint processing becomes possible in case of SRS for positioning. This is not the case for non-uniform and pseudo-random comb offset hopping.

    - In this case, l' in the hopping formula above is equal to the SRS symbol number (within the slot).

  - Per R consecutive SRS symbols, where R is the repetition factor: This can address the issue above.

    - In this case, l' in the hopping formula above is equal to the first SRS symbol number (within the slot) from the R consecutive symbols.

  - Per one occasion of SRS resource: This can also address the issue above. On the one hand, this is a simpler method at least for periodic / semi-persistent SRS. On the other hand, randomization can be only done across different periodicities of the SRS resource.

    - In this case, l' in the hopping formula above is equal to the first SRS symbol number (within the slot) from the SRS resource. Alternatively, l' may not be needed in this case.

- The initialization of the pseudo-random sequence c(i), which can be based on the SRS sequence identity nIDSRS configured per SRS resource (similar to existing sequence hopping or group hopping), or can be additionally a function of other SRS parameters, or based on a new configurable identity.

  - As discussed above, hopping consistency needs to be ensured for intra-cell or inter-cluster interference so that two UEs in the same cell or cluster do not select the same comb offset. This can be achieved by same initialization for these two UEs (if they are configured to be orthogonalized by different comb offsets, they always remain on different comb offsets irrespective of hopping). Hence, at least when the SRS sequence and cyclic shift are the same, they need to have the same initialization. Alternatively, a new RRC configuration can be introduced for initialization.

Furthermore, some companies mentioned “problems with coexistence of legacy and Rel-18 UEs” as an issue for comb offset hopping in the previous meeting. However, legacy UEs can be separated in other domains such as time domain (different symbols or slots) or in cyclic shift domain. Hence, we do not see this as an issue for comb offset hopping.

Proposal 2: For SRS comb offset hopping:

- The comb offset is determined based on the pseudo-random sequence c(i) as a function of time with a certain UE-specific initialization.

- The unit of applying comb offset hopping is

  - Option 1: Per one occasion of SRS resource

  - Option 2: Per R consecutive SRS symbols (where R is the repetition factor).

- For initialization of the pseudo-random sequence c(i):

  - Option 1: Based on existing SRS configurations such as SRS sequence identity nIDSRS and/or configured cyclic shift of the first SRS port.

  - Option 2: Based on a new RRC configuration.

Regarding pseudo-random muting of SRS transmission, “hopping” is in the domain of whether a given SRS symbols / SRS resource for a given UE at a given instant in time is transmitted or not. The benefit can be two-fold: First, the overall interference in the system is reduced as some UEs do not transmit SRS on some occasions. Second, for a given SRS resource of a given UE, in different instances of transmission (in different slots / symbols), different sets of UEs create interference to avoid persistent interference.

To evaluate the benefit of pseudo-random muting, we consider indoor hot spot layout with 12 TRPs, and considered random drops each with 2 UEs per TRP (the user CDF plots are aggregated across 50 random drops). In all cases, it is ensured that UEs in the same cluster do not transmit SRS on the same candidate occasion. In the case of “static allocation”, the assigned SRS occasion to each UE does not change throughout simulation, and hence, in each transmitted SRS occasion, the set of interferers are the same for each UE. However, in the case of “pseudo-random muting”, in different instances of SRS transmission for each UE, the set of interferers are different, and the median UL SINR per UE is plotted. The actual scheme of pseudo-random muting is based on Approach 2 explained in detail below. In both cases of static allocation and pseudo-random muting, the number of transmitted SRS occasions is exactly the same. We use OLPC for SRS, where the PL in OLPC formula is wrt the strongest TRP for each UE. In Figure 3, each cluster is one TRP, and each UE transmits SRS in 100 occasions out of 200 candidate occasions. As it can be seen, the tail UE UL SINR is improved thanks to interference randomization. In this case, the UL SINR is wrt the intended TRP, which is the strongest TRP.

Figure 3: Static allocation versus pseudo-random muting with cluster size equal to one TRP.

Next, we consider the case where each cluster consists of 4 TRPs, and hence, there are 8 UEs in each cluster in each drop. In this case, each UE transmits SRS in 100 occasions out of 800 candidate occasions. Similar as before, for both cases of static allocation and pseudo-random muting, it is ensured that 2 UEs in the same cluster do not transmit on the same SRS candidate occasion. In Figure 4, the user CDF of UL SINR is plotted wrt both the strongest TRP (solid line) as well as the second strongest TRP (dashed line) subject to the condition that PL delta compared to the strongest TRP is not larger than 6dB (otherwise, we do not consider the second TRP as an intended receiver for the SRS transmission). As it can be seen, the benefit of pseudo-random muting is even more when cluster size is more than one TRP as there is less SRS “pollution” in the system and interference randomization has larger impact in terms of median UL SINR per UE for the tail UE performance.

Figure 4: Static allocation versus pseudo-random muting with cluster size equal to four TRPs.

An SRS transmission opportunity can be defined as the unit of transmission / muting. For example, SRS transmission opportunity can be per OFDM symbol, per SRS resource, etc. Then, there can be two approaches for pseudo-random muting of SRS as explained below and illustrated in Figure 5:

- Approach 1: UE decides whether to transmit an SRS transmission opportunity according to a formula that is determined based on the pseudo-random sequence c(i) as a function of time (e.g., slot number, symbol number).

  - For a given SRS transmission opportunity, the formula generates 0 or 1 corresponding to muting or transmission, respectively.

- Approach 2: UE selects a binary sequence of length L corresponding to L SRS transmission opportunities according to a formula that is based on the pseudo-random sequence c(i) as a function of time (e.g., slot number or symbol number of the first SRS transmission opportunity).

  - For a given L SRS transmission opportunities, the formula generates an index pointing to a binary sequence of length L out of a set of binary sequences.

Figure 5: Illustration of the two approaches for pseudo-random muting of SRS transmission.

It is important to note that Approach 2 is more flexible and can grantee the performance based on the possible set of binary sequences with respect to the following aspects: i) The min / max number transmissions / muting’s in the L transmission opportunities is based min / max number of 1’s / 0’s of any binary sequence in the set of binary sequences; ii) We can ensure that there are not too many consecutive muting’s or too many consecutive transmissions for a given UE; iii) The number of collisions between UEs that select different binary sequences from the set can be controlled. For example, if L=2 and the possible set of binary sequences is {01, 10}: i) There is exactly one transmission and one muting in the L=2 transmission opportunities; ii) The max number of consecutive muting’s / transmissions is two; iii) There is no collision between UEs that select different binary sequences in the same L=2 transmission opportunities.

Regarding the initialization of the pseudo-random sequence c(i), similar discussions as the case of comb offset hopping as discussed above would be applicable.

Furthermore, in RAN1 #110 and #110-bis-e, there were a couple of misunderstanding about this scheme:

- Misunderstanding 1: “Periodic SRS with larger periodicity can achieve the same goal”. Obviously, periodic SRS cannot achieve interference randomization if in every period the same set of UEs create interference. In the simulation results above, baseline (“static allocation”) is exactly the periodic SRS with larger periodicity, and the benefit of the enhancement can be seen from the Figures above. The whole point of this scheme is the fact that SRS transmission is randomized in time domain, and for fair comparison, the baseline should also have the same average number of transmission occasions.

- Misunderstanding 2: “This scheme decreases the SRS channel estimation accuracy and is unpredictable”. First, pseudo-random muting avoids persistent interference, and hence, it improves the tail UE performance as shown above. Second, the degree of predictability can be controlled by the network as discussed in detail in Approach 2 above. Third, any interference randomization scheme aims to create difference interference patterns, and we do not understand why randomized transmission in time domain in particular is subject to channel estimation inaccuracy compared to frequency-domain or code-domain randomizations.

Also, it was mentioned that similar functionality can be achieved by AP-SRS. However, AP-SRS comes with the cost of DCI overhead as one DCI is required for each transmission occasion. We do not think it is reasonable to use so many DCIs just for the purpose of interference randomization when alternative solutions exist that do not require any DCI overhead.

Proposal 3: For pseudo-random muting of SRS transmission:

- Muting / transmission is per SRS transmission occasion, where a bundle of L transmission occasions is considered together

- An index corresponding to a binary sequence of length L from a set of binary sequences is determined based on the pseudo-random sequence c(i) as a function of time (e.g., slot number, symbol number) with a certain UE-specific initialization.

- FFS: The unit of SRS transmission opportunity, e.g., whether it is per OFDM symbol or per SRS resource.

- FFS: How to define L SRS transmission opportunities corresponding to a binary sequence of length L, and how to define the set of binary sequences to choose from.

- FFS: The details of initialization of the pseudo-random sequence c(i).

## Capacity / efficiency enhancements

With respect to “enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment”, two examples are captured in the agreement: One enhancement is configuration of v (sequence index within a group) per SRS resource, and the other enhancement is configuration of cyclic shift per SRS port per SRS resource.

In existing spec, 60 different base sequences with low cross-correlation are defined when SRS sequence length is equal to or larger than 72 bits by u=0,…,29 and v=0,1. However, when sequence hopping is not configured, currently v is always fixed to 0. This means that out of the 60 base sequences currently defined, only 30 of them can be assigned for the UEs in the system. With interference planning and when network carefully assigns SRS parameters to different UEs, allowing the network to configure any of the 60 base sequences can reduce the inter-cluster interference (or even intra-cluster interference, e.g., in case of large number of UEs and when within a cluster the other dimensions such as different SRS symbols, different comb offsets, different cyclic shifts are already used). Hence, allowing for configuration of v per SRS resource is a very simple and yet effective enhancement that makes all existing 60 different base sequences available to use.

Note that this enhancement is not related to interference randomization. Instead, the benefit is enhancing the reuse factor of SRS sequence, i.e., more SRS sequences can be configured by the network to ensure that 2 UEs with the same SRS sequence are far away and do not clear inter-cell / inter-cluster interference.

Proposal 4: Support to configure v (sequence index within a group) per SRS resource.

With respect to cyclic shift, currently, one cyclic shift can be configured for a given SRS resource corresponding to the first SRS port, and cyclic shifts are evenly distributed among ports. However, if network can configure cyclic shift per SRS port, it can control the cyclic shift spacing among the ports of a given UE versus across ports of multiple UEs. This is illustrated in Figure 6, where Comb spacing 4 (with 12 cyclic shift), and two UEs each with 2 SRS ports (same seq., same comb offset) is assumed. In Case 1, which is a possible configuration based on existing spec, cyclic shifts {0,6} are assigned to UE1 and cyclic shifts {2,8} are assigned to UE2. In Case 2, network can flexibly assign cyclic shift per port per UE, and may choose to maximize the cyclic shift spacing across the UEs, which can be beneficial as two ports of the same UE may be more resilient against small cyclic shift spacing versus two ports across different UEs.

Figure 6: Legacy versus per-port cyclic shift configuration.

To evaluate the performance of this proposal compared to legacy uniform cyclic shift assignments across SRS ports, we have conducted link-level simulations. Both sTRP and mTRP CJT scenarios are considered, where BLER of PDSCH based on precoding obtained from SRS is used as the main metric, and the UEs are co-scheduled for PDSCH by performing zero-forcing precoding at the base station which is calculated based on estimated channel from SRS. In case of mTRP CJT, one SRS transmission from each UE is received on all TRPs from which the channel is estimated for precoding. Case 1 is based on legacy cyclic shift while Case 2 is based on the more flexible cyclic shift assignments across SRS ports. SRS is transmitted on the same symbol by all UEs and the same comb offset is assumed (they are separated in cyclic shift domain in both cases). Also, PDSCH transmitted to each UE occupies the same RBs (16 RBs are assumed) in MU-MIMO manner from all TRPs using CJT. TDL-C channel model is considered for this LLS.

As discussed before, the flexible (per port) cyclic shift assignment provide benefit when UEs have different delay spread (DS) or different Rx timing (different fixed delay). In each of the following scenarios, the performance of Case 1 (legacy) is compared to Case 2 (proposal):

- Scenario 1: 1 TRP, 2 UEs, 2 SRS ports / Rx antennas per UE, UE1/2 DS=100/500ns, UE1/2 fixed delay=0/200ns as illustrated in Figure 7.

  - Cyclic shift in Case 1: UE1  {0,6}, UE2  {1,7}

  - Cyclic shift in Case 2: UE1  {0,1}, UE2  {6,7}

- Scenario 2: Same as Scenario 1 but with 4 SRS ports / Rx antennas per UE.

  - Cyclic shift in Case 1: UE1  {0,3,6,9}, UE2  {1,4,7,10}

  - Cyclic shift in Case 2: UE1  {0,1,3,4}, UE2  {6,7,9,10}

- Scenario 3: Same as Scenario 2 but with 3 UEs (all 12 cyclic shifts are used), DS/fixed delay/delta PL as shown in Figure 7.

  - Cyclic shift in Case 1: UE1  {0,3,6,9}, UE2  {1,4,7,10}, UE3  {2,5,8,11}

  - Cyclic shift in Case 2: UE1  {0,1,2,3}, UE2  {4,5,6,7}, UE3  {8,9,10,11}

- Scenario 4: 2TRP CJT, 2UEs, 4 SRS ports / Rx antennas per UE, DS/fixed delay/delta PL as shown in Figure 7.

  - Cyclic shift same as scenario 2 for Case 1 and Case 2.

- Scenario 5: Same as Scenario 4 except that DS/fixed delay/delta PL are based on the illustration in Figure 7.

Figure 7: Illustrations of Scenarios 1-5.

Figure 8 illustrates the performance comparison between Case 1 (legacy) and Case 2 (proposal) for the five scenarios described above. As it can be seen from the Figure, in all scenarios, performance of at least one UE improves considerably with Case 2 while the performance of other UE(s) either also improves slightly or remains similar to Case 1. This is because by assigning cyclic shifts carefully across ports of different UEs, we can minimize the impact of large DS / fixed delay from one UE to the SRS ports of another UE (which has smaller DS / fixed delay). Furthermore, even when SRS channel estimation for one UE improves, precoding accuracy is enhanced (and hence better interference nulling with zero-forcing precoding), which results in even performance improvement of other co-scheduled UEs.

Figure 8: Link-level simulations on the benefit of per-port cyclic shift configurations for Scenarios 1-5.

In the above, we mostly focused on PDSCH BLER performance as a result of better SRS channel estimation at the gNB, which is the main metric for CJT performance. However, the benefit of the proposal can be also seen directly from the mean square error (MSE) of SRS channel estimation at the gNB. Figure 9 illustrates this for Scenario 1 as an example. It can be seen that MSE of UE 1 is improved significantly by ensuring large cyclic shift spacing from both SRS ports of UE2 (which has high DS and fixed delay). Even though the MSE of UE2 is degraded slightly, this results in BLER improvements of UE1 without impacting the BLER of UE2. As discussed before, better channel estimation even for one UE may help both UEs as they are co-scheduled with zero-forcing precoding.

Figure 9: MSE of channel estimation from SRS in Scenario 1.

In addition, we would like to address the following concerns mentioned by companies in the previous meeting for this scheme:

- “Backward compatibility”: There is no backward compatibility issue as the proposal is to configure cyclic shift per SRS port. When legacy UEs are multiplexed, the occupied cyclic shifts are simply not configured for the Rel-18 UE.

- “DCI overhead”: The proposal is not related to dynamic indication of cyclic shift. Instead, it is related to enhanced RRC configuration. The additional RRC overhead is negligible as only configuration of one cyclic shift per port is needed (e.g., for SRS resource with 4 ports, three cyclic shifts in addition to the existing initial cyclic shift for the first port can be RRC-configured).

- “Increase the channel estimation complexity”: This is not correct as different ports are on different cyclic shifts whether evenly / uniformly distributed or not. For channel estimation at the gNB, the only knowledge needed is cyclic shift per port, and the rest is same as legacy.

- “The scheme cannot enhance SU performance”: This is true only if SU is just referring to SRS transmission since conditioned on a single UE transmitting SRS on a set of REs, the best cyclic shift assignment across ports is obviously uniform, which achieves the highest cyclic shift spacing among the ports. However, if SU refers to DL scheduling, this may not be true since multiple UEs can still use the same comb offset and symbols (and be separated by different cyclic shifts) for the purpose of SRS overhead reduction. The point is that, as long as there are more than one UE transmitting SRS on the same set of REs using different cyclic shifts, the scheme allows for more optimized configuration of cyclic shift per SRS port for each UE.

Proposal 5: Support to configure cyclic shift per SRS port per SRS resource.

With respect to the other potential capacity enhancements listed in the agreement before but not concluded yet, we would like to point out the following:

- SRS TD OCC: This has been discussed in Rel-17 and prior releases, but not agreed. In our understanding, TD-OCC for SRS has significant specification impacts. In addition to considerations related to TD-OCC length and intra-UE (TD-OCC across multiple SRS ports of a given UE / SRS resource) versus inter-UE (TD-OCC across multiple UEs), the unit for applying the TD-OCC requires various considerations such as repetition factor and frequency hopping parameters. Furthermore, the performance is impacted due to loss of orthogonality if one SRS symbol is dropped (as in current spec, SRS dropping is at the symbol level). Additionally, TD-OCC may be more relevant to coverage enhancements as opposed to capacity enhancements. When coverage is not limited, single-symbol SRS (per frequency hop) is enough in which case TD-OCC is not applicable. With antenna switching (e.g., 1T4R), 4 SRS resources are needed. With TD-OCC, the assumption is that each of the 4 SRS resources have at least repetition factor of 2. Even though this is a possible configuration, the point is that TD-OCC can be used only in limited scenarios in practice for the purpose of DL CSI acquisition.

- Precoded SRS for DL CSI acquisition: According to the discussions in RAN1 #110 and the explanations from proponents, the main motivation is to reduce total SRS port number to the PDSCH layer number (instead of # of Rx antennas). We see various issues with this approach:

  - Such precoding means that a max rank smaller than UE capability is RRC-configured (as the number of SRS ports should be fixed and cannot be autonomously decided by the UE).

  - With this, gNB has no longer access to the actual channel, and hence scheduling flexibility is seriously impacted.

  - It is not clear how MU-MIMO operation can be performed anymore as precoding should be a function of channel from all co-scheduled UEs. Zero-forcing precoding (or any precoder with interference nulling toward co-scheduled UEs) can no longer be used with precoded SRS. In order to do interference nulling, the full channel matrix to all UEs are needed, and also the rank can no longer be decided by the UE as it depends on other UEs’ channel.

  - PAPR may be impacted depending on the choice of precoding.

  - The type of precoder that the UE uses for SRS may require standardization as this impacts the DL scheduling decisions (unlike SRS with usage set to non-codebook).

  - It is not clear how such precoding can be applied for antenna switching especially in the case of xTyR with x<y as SRS transmissions will be on different OFDM symbols. As it was discussed in RAN1 #110, there is no benefit (capacity or otherwise) in case of e.g., 1T4R. Some companies argued that power is boosted due to repeating one layer four times. However, this cannot be the case as precoding for a 1-port transmission is meaningless, and obviously, UE cannot transmit from 4 Tx antennas simultaneously to form a virtual port (otherwise, this UE would have been 4T4R).

  - The actual benefit is not clear in terms of capacity enhancement / overhead reduction considering the additional overhead associated with two CSI-RS resources each with potentially large number of CSI-RS ports.

  - Tx/Rx calibrations are needed at the UE for this scheme. For NCB-based UL with associated CSI-RS, we do not think UE is mandated to calibrate Tx/Rx chains. UE may choose to do so, but the point is that it is not required. For example, UE may only do antennas selection or only do SRS port virtualization based on CSI-RS. Calibrating Tx/Rx chains is very difficult for UE. This is not an issue for NCB-based PUSCH as the SRS is measured by gNB based on which one or more SRS resources are indicated to the UE for PUSCH transmission. However, the calibration seems to be necessary for this scheme to work since the estimated channel is directly used for PDSCH precoding later (unlike NCB-based PUSCH, where gNB anyway gets to measure multiple SRS resources and indicate one or more of them for PUSCH transmission).

  - Such precoding based on CSI-RS for the purpose of DL CSI acquisition is not supported even for the single-TRP case (e.g., based on a single CSI-RS resource). It needs to be first justified and supported for single-TRP (with considering the issues mentioned above) before extension to mTRP/CJT can be considered.

Hence, we do not see the need to discuss this enhancement further in this agenda item for TDD CJT.

Given the discussions above and the long list of potential enhancements, we think it would be helpful to focus the study / specification on the more relevant enhancements with clear use case for TDD CJT, and other potential enhancements that either have been discussed extensively in previous releases or would lead to large specification impact without clear use case should have lower priority.

Proposal 6: The following should have low priority for Rel-18 for SRS enhancements for TDD CJT:

- SRS TD OCC

- Precoded SRS for DL CSI acquisition

## Enhanced Signaling

With respect to “enhanced signaling for flexible SRS transmission”, the main motivation is to allow the network to react to a change in scheduling decisions, multi-user grouping (with zero-forcing beamforming assuming CJT across multiple TRPs), or DL traffic availability. Such changes are typically dynamic in nature and result in the need to change SRS parameters or trigger new SRS resources. At the same time, these factors may not change from slot-to-slot, but once they occur, being able to react to them can increase the SRS capacity as less dimensions (e.g., in symbol domain, comb offset domain, cyclic shift domain, etc.) need to be reserved for this purpose. In existing AP-SRS, DCI-based triggering is a one-time request as opposed to activating periodic SRS resources. Also, DCI cannot change SRS parameters. Dynamic update of SRS parameters for AP-SRS was discussed in Rel-17, but not agreed. In existing SP-SRS, MAC-CE is used to activate/deactivate, which has higher latency compared to DCI. Similarly, MAC-CE cannot change SRS parameters, which are configured by RRC.

The following proposal was discussed in RAN1 #110-bis-e:

Proposal 2.8-D: Study enhanced signaling for flexible SRS transmission from the following list:

- Dynamic update of one or more of the following SRS parameters

  - Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

  - Power control parameter

  - RPFS parameters

  - Number of antennas in antenna switching

  - Code-domain parameters (cyclic shift / SRS sequence)

  - Time-domain parameters

- Activation by DCI for SP SRS

- Enhanced cancellation indication in DCI format 2_4, e.g.,

  - Association with SRS resource(s)

  - Periodic/persistent cancellation

For the dynamic update of SRS parameters (first bullet), we do not think discussing SRS parameters one-by-one is helpful. This is because in existing AP-SRS, one or multiple SRS resource sets each with multiple SRS resources can be triggered, and indicating such parameters directly by the DCI for each SRS resource of each triggered SRS resource set may not be feasible.

On the other hand, SP-SRS activated by a DCI enables fast triggering of periodic SRS transmission as well as dynamic indication of some of the SRS parameters. This is very similar to SPS PDSCH, Type 2 CG-PUSCH, or SP-CSI on PUSCH. In all these legacy procedures, the DCI activates periodic receptions / transmissions. This functionality is missing for SRS in the current spec. Similar to legacy SPS/CG/SP-CSI on PUSCH, a different RNTI can be configured for UE to be able to distinguish activation DCI / release DCI from regular scheduling DCI, which means that many of the DCI fields are available to be used to also indicate the parameters of SRS or to use more bits for SRS triggering (hence, achieving similar effect). Furthermore, for an already activated SRS resource set, a reactivation DCI can be used to update the SRS parameters (analogous to SPS/CG reactivation). SP-SRS activated by a DCI is illustrated in Figure 10.

Figure 10: SP-SRS activated by a DCI.

Proposal 7: For enhanced signaling for flexible SRS transmission, support semi-persistent SRS activated by a DCI.

## Power control enhancements

The following was agreed in RAN1 #110-bis-e with respect to power control enhancements:

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Regarding Option 2, the motivation is unclear. The same functionality is already supported by configuring different SRS resource sets. Furthermore, this option results in large SRS overhead as the assumption is that different TRPs cannot receive the same SRS transmission, which is not reasonable in FR1.

Regarding Option 1, the situation is better since each SRS transmission can be received by multiple TRPs while open loop power control (α.PL) can be a function of multiple PL values (calculated through measuring multiple PL-RS) and multiple configured alpha values. However, similar effect may be achieved by closed loop power control. We are open to Option 1 if the benefit compared to legacy power control mechanisms (including CLPC) is shown.

Proposal 8: For SRS power control enhancements, focus on Option 1 and deprioritize Option 2.

# SRS enhancement for 8 Tx UL transmission

Supporting 8 Tx PUSCH transmission is included in the WID of Rel-18 MIMO ‎[2]. To support 8 Tx PUSCH, apparently, SRS enhancement is needed to allow UE to sound 8 SRS ports.

SRS for codebook and non-codebook PUSCH are different. For non-codebook based PUSCH, one SRS resource can only sound one SRS port. For codebook PUSCH, one SRS resource can sound up to 4 SRS ports in Rel-15/16/17. Given this difference, the design of SRS enhancement for codebook and non-codebook based PUSCH are separately addressed in Section ‎3.1 and ‎3.2.

## SRS enhancement for codebook based PUSCH with 8 Tx

For SRS enhancement for codebook based PUSCH, there are two potential approaches.

- Sound 8 SRS ports in one SRS resource.

- Sound 8 SRS ports in multiple SRS resources, each with 2 or 4 SRS ports.

The first approach is introducing SRS resource(s) with up to 8 SRS ports, i.e., sound 8 SRS ports in one SRS resource, in an SRS resource set, as illustrated in Fig 11. In this resource set, SRI can be used to switch among different SRS resources, as in Rel-15/16/17.

Fig 11: An SRS resource set with an SRS resource including 8 SRS ports

The key open issue for the first approach is how to map the 8 SRS ports to resources in time, comb, and cyclic shift domain. Given there are up to 8 SRS ports in a resource, it is beneficial to allow mapping the 8 SRS ports to more than one OFDM symbols, to allow larger SRS sounding power per port for better sounding quality and minimize leakage between SRS ports due to channel delay spread or Doppler. Given 8 SRS ports, it is natural to map them to {1,2,4} OFDM symbols. Mapping to 3 OFDM symbol would create unequal split of SRS ports among symbols, which unnecessarily complicates specification without out much benefit.

Regarding how to map 8 SRS ports to 2 or 4 OFDM symbols, there are two ways. One way is TDM the 8 SRS ports, as shown in Fig 12. The other way is always transmitting 8 ports on each OFDM symbol, then repeating them on 2 or 4 OFDM symbols with TD-OCC codes. Between the two approaches, the TDM approach is preferred. The reason is because TD-OCC is sensitive to high Doppler channel. Furthermore, some SRS OFDM symbol(s) might be punctured by higher priority UL channels such as PUCCH. If that occurs, TD-OCC breaks and all SRS ports sounding are lost. While with TDM, those SRS ports on remaining (unpunctured) OFDM symbols are still useful.

Fig 12: An example to map 8 SRS ports to 2 and 4 OFDM symbols.

One aspect of mapping 8 SRS ports to 2 or 4 OFDM symbols, especially for coherent 8 Tx, is phase and power continuity across the 2 or 4 OFDM symbols. To guarantee phase and power continuity, the 2 or 4 OFDM symbols should be consecutive and not across slot/sub-slot boundary as well.

With the above analysis, the following proposal is made.

Proposal 9: NR Rel-18 support configuring an SRS resource with 8 SRS ports for a codebook based PUSCH transmission with 8 Tx.

- Support mapping the 8 ports in the SRS resource to 1, or 2, or 4 consecutive OFDM symbols in same slot/sub-slot in TDM manner.

- FFS details to mapping the 8 ports to OFDM symbols, frequency combs, and cyclic shifts.

The second approach is sounding the 8 ports via two or more SRS resources. With the second approach, there is no need to specify new type of SRS resources. Two legacy 4-ports SRS resources in an SRS resource set can be linked together to sound the 8 SRS ports for a PUSCH, as illustrated in Fig 13. With approach 2, SRI enhancement is needed to indicate multiple SRS resources in a DCI scheduling the PUSCH transmission. For example, with 2 SRS resources sounding 8 ports, SRI need to be extended to 2 bits to indicate the following 4 codepoints,

- SRI = 01: fallback to only use 4 ports in SRS resource A

- SRI = 10: fallback to only use 4 ports in SRS resource B

- SRI = 00: use both SRS resource A & B with ports in B appending after ports in A

- SRI = 11: use both SRS resource A & B with ports in A appending after ports in B

With more SRS resources, e.g., 4, sounding 8 SRS ports, 3 bits SRI might be needed.

Fig 13: Sound 8 SRS ports via two 4-port SRS resources

If the first approach with mapping 8 ports to multiple OFDM symbols (i.e., proposal 8) is adopted, then there is no fundamental difference between the first and second approach. It is questionable whether the second approach is needed or not, unless additional benefits can be identified.

Proposal 10: Further study if there is additional benefit to support configuring multiple SRS resources to sound 8 SRS ports for a codebook based PUSCH transmission with 8 Tx.

## SRS enhancement for non-codebook based PUSCH with 8 Tx

Regarding SRS enhancement for non-codebook based PUSCH, it is obvious that 8 SRS resources are needed to sound 8 SRS ports for non-codebook based PUSCH, as one SRS resource can only sound one SRS port in this case. The open question is whether the 8 SRS resources should be put into a single or multiple SRS resource sets.

Similar to the discussion in Section ‎3.1, it is preferred to support both, i.e., allowing 8 SRS resources in either a single or multiple SRS resource sets.

If 8 SRS ports are sounded via a single SRS resource set, as illustrated in Fig 14. The SRI enhancement is simply expanding the bit width the SRI field in DCI to indicate the total number of combinations of X ports, which is:X=18X8=255, where X8 denotes the number of combinations of choose X ports out of 8 ports.

Fig 14: 8 SRS ports are sounded via a single SRS resource for NCB based PUSCH

If 8 SRS ports are sounded via multiple SRS resource sets, as illustrated in Fig 15. Multiple SRI fields, e.g., 2, might be needed in the DCI scheduling 8 Tx NCB PUSCH. The details of bit-width of SRI field for each SRS resource set can be further studied.

Fig 15: 8 SRS ports are sounded via two SRS resource sets for NCB based PUSCH

Agreement

For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

In RAN1 110bis-e, regarding this open issue, the following is agreed.

Regarding the FFS, given that SRS power control parameters are configured per SRS resource set, it is beneficial to allow multiple SRS resource sets, one for each UL panel, to allow different power for SRS sounding.

With the above discussion, the following proposal is made for non-codebook based PUSCH with 8 Tx.

Proposal 11: For SRS configuration for non-codebook UL transmission for an 8TX UE, further support configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

# Conclusions

In summary, we have the following proposals for SRS enhancements for Rel-18 MIMO evolution.

Proposal 1: For interference randomization, support at least one of a) comb offset hopping, and/or b) pseudo-random muting of SRS.

- FFS: Whether to additionally support cyclic shift hopping and/or combinations of sequence hopping and group hopping

Proposal 2: For SRS comb offset hopping:

- The comb offset is determined based on the pseudo-random sequence c(i) as a function of time with a certain UE-specific initialization.

- The unit of applying comb offset hopping is

  - Option 1: Per one occasion of SRS resource

  - Option 2: Per R consecutive SRS symbols (where R is the repetition factor).

- For initialization of the pseudo-random sequence c(i):

  - Option 1: Based on existing SRS configurations such as SRS sequence identity nIDSRS and/or configured cyclic shift of the first SRS port.

  - Option 2: Based on a new RRC configuration.

Proposal 3: For pseudo-random muting of SRS transmission:

- Muting / transmission is per SRS transmission occasion, where a bundle of L transmission occasions is considered together

- An index corresponding to a binary sequence of length L from a set of binary sequences is determined based on the pseudo-random sequence c(i) as a function of time (e.g., slot number, symbol number) with a certain UE-specific initialization.

- FFS: The unit of SRS transmission opportunity, e.g., whether it is per OFDM symbol or per SRS resource.

- FFS: How to define L SRS transmission opportunities corresponding to a binary sequence of length L, and how to define the set of binary sequences to choose from.

- FFS: The details of initialization of the pseudo-random sequence c(i).

Proposal 4: Support to configure v (sequence index within a group) per SRS resource.

Proposal 5: Support to configure cyclic shift per SRS port per SRS resource.

Proposal 6: The following should have low priority for Rel-18 for SRS enhancements for TDD CJT:

- SRS TD OCC

- Precoded SRS for DL CSI acquisition

Proposal 7: For enhanced signaling for flexible SRS transmission, support semi-persistent SRS activated by a DCI.

Proposal 8: For SRS power control enhancements, focus on Option 1 and deprioritize Option 2.

Proposal 9: NR Rel-18 support configuring an SRS resource with 8 SRS ports for a codebook based PUSCH transmission with 8 Tx.

- Support mapping the 8 ports in the SRS resource to 1, or 2, or 4 consecutive OFDM symbols in same slot/sub-slot in TDM manner.

- FFS details to mapping the 8 ports to OFDM symbols, frequency combs, and cyclic shifts.

Proposal 10: Further study if there is additional benefit to support configuring multiple SRS resources to sound 8 SRS ports for a codebook based PUSCH transmission with 8 Tx.

Proposal 11: For SRS configuration for non-codebook UL transmission for an 8TX UE, further support configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

# References

- 3GPP TSG RAN Meeting #94e, RP-213598 “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, Electronic Meeting, Dec. 6 - 17, 2021.

- 3GPP TSG RAN WG1 #109-e, R1-2205020, “Enhancements for 8 Tx UL transmissions”, Qualcomm, e-Meeting, May 9th – 20th, 2022