3GPP TSG RAN WG1 #111	R1-2212167

Toulouse, France, November 14th – 18th, 2022

Agenda item:		9.1.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Unified TCI framework extension for multi-TRP

Document for:		Discussion and Decision

# Introduction

New Work Item for Rel18 MIMO evolution was approved in RAN#94e [1]. The unified TCI framework was introduced in Rel-17 which facilitates a streamlined multi-beam operation targeting FR2. As Rel-17 focuses on single-TRP use cases, extension of unified TCI framework that focuses on multi-TRP use cases is seen beneficial and is targeted in Rel-18. Further, with the introduction of features for fast UL panel and beam selection in Rel-17, advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) can benefit from higher UL coverage and average throughput with simultaneous UL multi-panel transmission. In addition, further enhancements to facilitate UL multi-TRP deployments via enhanced UL power control can offer additional UL performance improvement. These aspects are being discussed in this contribution following the related WI description as follows [1]:

… 2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. … 6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. … 7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

# Discussion

## 2.1 General framework for unified TCI framework

### 2.1.1 Combinations of joint/DL/ UL TCI states that can be indicated

Supported combinations of joint/DL/UL TCI states that can be indicated should be selected based on the considered use cases. The related agreements regarding the use cases and combinations have been made in the previous meetings:

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension, at least for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI

The intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 are as follows:

- Single/multi-DCI based M-TRP PDSCH NCJT in Rel.16

- Single-DCI based M-TRP PDSCH repetition with SDM/FDM/TDM in Rel.16

- M-TRP PDCCH/PUSCH/PUCCH repetition in Rel.17

- M-TRP inter cell in Rel.17

- SFN-PDCCH/PDSCH in Rel.17

- M-TRP BFR in Rel.17

Furthermore, it has been discussed whether CJT CoMP with up to four transmitting TRPs would also be supported via unified TCI framework. We think that unified TCI framework shall support all use cases in general and thus we also consider that unified TCI framework should support CJT CoMP for PDSCH.

In the following table, we go through the different use cases and what kind of combinations would be needed.

Table 1 Combinations of joint/DL/UL TCI states needed for different use cases

| Scheme | Combinations |
| --- | --- |
| Multi-DCI M-TRP PDSCH NJCT in Rel16 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP |
| Single-DCI M-TRP PDSCH NJCT in Rel16 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP |
| Single-DCI based M-TRP PDSCH repetition with SDM/FDM/TDM in Rel.16 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP |
| M-TRP inter cell in Rel.17 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (different PCI) - 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (different PCI) - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (different PCI) - 1 pair of DL and UL TCI states indicated for a first TRP and 1 UL TCI state indicated for a second TRP (different PCI) |
| SFN-PDCCH/PDSCH in Rel.17 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP |
| CJT COMP | - Up to 2 joint or DL TCI states |

Observation 1: The following combinations of joint/DL/UL TCI states would be needed:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 UL TCI state indicated for a second TRP (same or different PCI) (different PCI)

- Up to 2 joint or DL TCI states (same PCI)

Proposal 1: Support the following combinations of joint/DL/ UL TCI states that can be indicated:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 UL TCI state indicated for a second TRP (same or different PCI)

- Up to 2 joint or DL TCI states (same PCI)

### 2.1.2	On supporting CJT PDSCH with the unified TCI framework

As discussed above, it has been considered whether the unified TCI framework should support also CJT CoMP with up to four transmitting TRPs in FR1. RAN1#110bis-e made the following agreement:

Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1

The agreement says that while there can be up to four transmitting TRPs for the coherent joint transmission PDSCH the UE may be provided up to 2 (1 or 2 based on UE capability) joint TCI states to provide QCL parameters for the reception of DMRS of PDSCH and PDSCH. The operation is assumed so that a TRP-specific QCL source RS would be provided for each TRP-specific CSI-RS resource set and corresponding CSI-RS resource(s) for the purpose of the CSI acquisition. In the CSI acquisition, the UE is expected to calculate a joint channel state information comprising e.g., desired precoder matrix, rank and channel quality information (CQI) over the TRP specific CSI-RS resources. As described in [3GPP TS 38.214, 17.3.0, 5.1.5], CSI-RS resource for CSI acquisition is required to have a TRS as a QCL-TypeA source:

For a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with an SS/PBCH block, where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - 'typeB' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info when 'typeD' is not applicable.

Thus, for CSI acquisition from N TRPs the UE would be configured N TRSs (CSI-RS resource specifically designed for the time-frequency tracking), each providing channel characteristics like Doppler shift, Doppler spread, average delay, delay spread (QCL-TypeA) of one TRP as illustrated in Figure 1.

Figure 1 CSI acquisition phase in CJT PDSCH.

Then, as described in [3GPP TS 38.214, 17.3.0, 5.1.5], detection of DMRS of PDSCH is required to have a TRS as a QCL-TypeA source:

For the DM-RS of PDSCH, the UE shall expect that an indicated DLorJointTCIState indicates one of the following quasi co-location type(s) if the UE is configured [TCI-State](s) with [tci-StateId_r17]: - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

That means that from the given TRS resource (that is acting as QCL-TypeA source for DMRS of PDSCH) the UE needs to be able to estimate doppler shift, doppler spread, time delay and time spread parameters of the channel in order to set its channel estimate filters for the detection of DMRS of PDSCH properly. PDSCH transmission in coherent joint transmission (CJT) PDSCH is based on the joint precoding across all the antenna ports of the TRPs involved in the transmission. The coherent joint transmission may be having one or multiple layers. The decision of the DL precoder and rank is made dynamically by the scheduler for each CJT PDSCH transmission based on the UE’s CSI report and e.g., based on the HARQ feedback (inner-loop link adaptation). Thus, the precoder may change from one transmission to another transmission instant. Similarly effective channel characteristics at the UE receiver change according to the applied joint precoder (as well as individual channel realizations between each TRP and UE). In addition, for one PDSCH transmission time instant n different TRPs may be transmitting and for the other PDSCH transmission time instant m different TRPs may be involved in the transmission where n ≠ m and {n, m}ϵ{1,2,3,4}, as illustrated by Example A and Example B in Figure 2.

Figure 2 Illustration of two examples of set of transmitting TRPs for CJT PDSCH.

That means that for the UE to estimate the channel characteristics for the CJT PDSCH reception, it is not clear if the UE should use the TRP specific TRSs or should the UE be provided a TRS signal before the CJT PDSCH transmission that is transmitted with the same precoder and by the same TRPs as the CJT PDSCH.

Proposal 2: Study whether the UE should be provided a TRS signal (QCL parameters, like TypeA) for the CJT PDSCH reception that is transmitted with the same precoder (e.g. 1st layer) across the transmitting TRPs as used for the CJT PDSCH.

## 2.2	TCI state update and activation

### 2.2.1	TCI state update for M-DCI based M-TRP

For TCI state update in m-DCI based M-TRP, the following agreements was made in RAN1#110bis-e:

Agreement: On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework

As shown above, the agreements in RAN1#110bis-e cover the PDCCH and PDSCH. In the following, we discuss how unified TCI framework can be extended to other signals and channels in M-DCI m-TRP framework. For instance, AP-CSI-RS can follow Indicated TCI state of the CORESET of the triggering PDCCH. Determining of which of the Indicated TCI states is updated would be based on the coresetPoolIndex of the CORESET of the PDCCH transmitting the DCI carrying the AP-CSI-RS trigger.

Observation 2: In m-DCI, AP CSI-RS can follow Indicated TCI state of the CORESET of the triggering PDCCH.

Observation 3: In m-DCI, determining of which of the Indicated TCI states is updated would be based on the coresetPoolIndex of the CORESET of the PDCCH transmitting the DCI AP-CSI-RS trigger.

Regarding dedicated PUCCH resources, it would be needed to group the resources so that one set of resources would be associated to the first Indicated TCI state (may refer as the indicated TCI state corresponding to the CORESETPoolIndex 0) and other set of the resources would be associated to the second Indicated TCI state (may refer as the indicated TCI state corresponding to the CORESETPoolIndex 1). In general, there may be different alternatives to facilitate the association between PUUCH resources and indicated TCI states, e.g. one alternative could be to define an association between a coresetPoolIndex and a set of PUCCH resources while the other alternative could be to define a separate grouping for the PUCCH resources.

As currently in Rel-17, for SRS resource (set) it’s provided in the configuration whether the SRS resource (set) follows Indicated TCI state or TCI state, it would need to be extended that the configuration should provide whether the SRS resource (set) follows the first Indicated TCI state, the second Indicated TCI state or TCI state.

On the other hand, as in current Rel-17 version of 38.331, the CORESET configuration has a flag whether or not the CORESET follow Indicated TCI state as well as SRS resource set configuration. Thus, the extension would be needed at least for:

- configuration needs to be provided for dedicated PUCCH resource(s) that whether the resource(s) follow Indicated TCI state 1 or Indicated TCI state 2

- SRS resource (set) that whether the resource(s) follow Indicated TCI state 1, Indicated TCI state 2 or TCI state

Observation 4: In m-DCI, configuration needs to be provided for dedicated periodic/semi-persistent PUCCH resource(s) that whether the resource(s) follow Indicated TCI state 0 or Indicated TCI state 1.

Observation 5: In m-DCI, configuration needs to be provided for SRS resource (set) that whether the resource(s) follow Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 3: In M-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether it is the Indicated joint or DL TCI state associated to coresetPoolIndex of the CORESET of the triggering PDCCH or joint or DL TCI state.

Proposal 4: In M-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 5: In M-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Proposal 6: In M-DCI, for the P-/SP-Dedicated PUCCH resource, TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 7: In M-DCI, for the DCI triggered dedicated PUCCH resource, TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 8: In M-DCI, for the SRS resource (set), TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or joint or UL TCI state.

Proposal 9: In M-DCI, for the DCI triggered SRS resource (set), TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 10: In M-DCI, for the CG PUSCH type 1 and 2, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 11: In M-DCI, for the DG PUSCH 0_0, TCI assumption is the Indicated joint or UL TCI state of PUCCH resource ID 0.

Proposal 12: In M-DCI, for the DG PUSCH 0_1/0_2, TCI assumption is the joint or UL TCI state of the Indicated SRS resource.

### 2.2.2	TCI state update for S-DCI based M-TRP

In the case of S-DCI based M-TRP and in the case of separate DL and separate UL TCI states, one TCI codepoint may have up to 4 TCI states: 2 DL TCI and 2 UL TCI states. We consider that for flexibility it should be possible to activate two DL and two UL TCI states per TRP. That would require 16 different codepoints to be supported. Thus, we think that it would make sense to increase the max number of MAC CE activated TCI codepoints e.g., up to 16.

Proposal 13: A single DCI codepoint can be configured to indicate multiple (e.g. two) joint TCI states or multiple separate DL and/or UL TCI States.

- For Rel-16/17 s-DCI mTRP schemes, only two joint or two separate DL and/or two UL TCI states may be configured in a single codepoint and it may depend on the configured/applicable Rel-16/17 mTRP scheme

Proposal 14: Support the max number of MAC CE activated TCI codepoints being 16.

Proposal 15: Support up to four-bit TCI field in DCI.

### 2.2.3	Potential impact and enhancement to common TCI state ID update/activation for CA when S-DCI/M-DCI MTRP is configured

In Rel-17 multi-CC scenario, RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC as depicted in Figure 3:

Figure 3 Illustration of the reference BWP/CC.

Then, the next question is that how common TCI state update operation should be defined in case for multi-TRP in general and for a mix of CCs/cells configured with single-TRP and multi-TRP modes.

S-DCI scenario

In general, in S-DCI MTRP scenario the CC/BWPs may be configured with either multi-TRP and single-TRP configurations. Then, each CC/BWP should be provided that which indicated TCI state(s) of the reference CC/BWP the CC/BWP would follow.

Proposal 16: Study in S-DCI M-TRP scenario that how UE can determine which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured either with single- or multi-TRP configuration.

M-DCI scenario

In m-DCI MTRP scenario the CC/BWPs may be configured with coreset pool indices and also one CC/BWP may have either one of two coreset pool indices configured. In that case in general, the assumption of which indicated TCI state to follow could be based on the coreset pool indices.

Proposal 17: Study in M-DCI M-TRP scenario that how UE can determine which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured with one or two coreset pool indices.

## How to associate the indicated TCI state(s) with each target channel/signal in S-DCI based MTRP

For TCI state indication in s-DCI based MTRP, the following agreements was made in RAN1#110bis-e:

Agreement: On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for S-DCI based MTRP , down-select one alternative from the followings in RAN1#111: - Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

In s-DCI PDSCH schemes, some signals and channels, like PUCCHs, may be associated to a single Indicated TCI state (one of the two Indicated TCI states, if two indicated TCI states) while PDSCH transmission occasions and AP CSI-RSs would need two Indicated TCI states. This is to facilitate CSI acquisition from the intended PDSCH beams.

Observation 6: In s-DCI PDSCH schemes, AP CSI-RS resource (set) should be able to be associated to one of the (two) Indicated TCI state where one the of Indicated TCI states is the same as for the CORESET used for the PDCCH triggering the AP CSI-RS.

Observation 7: In s-DCI PDSCH schemes, when scheduled two PDSCHs (two PDSCH transmission occasions), one PDSCH would follow Indicated TCI state 0 and the other PDSCH would follow Indicated TCI state 1.

Observation 8: In s-DCI PUSCH schemes, configuration needs to be provided for SRS resource (set) that whether the resource(s) follow Indicated TCI state 0, Indicated TCI state 1 or TCI state.

In mTRP PUCCH schemes (mainly s-DCI), some PUCCH resources shall be associated to a single Indicated TCI state and other PUCCH resources shall be associated to two Indicated TCI states (PUCCH repetition), and the consideration for other channels are not fully relevant for the operations defined for PUCCH repetition.

Observation 9: For PUCCH repetition and to facilitate dynamic switching between sTRP and mTRP schemes, some PUCCH resource should be able to be associated to a single Indicated TCI state while other PUCCH resources should be able to be associated to two Indicated TCI states.

Proposal 18: In S-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 19: In S-DCI, for the first PDSCH, TCI state assumption is the Indicated TCI state 0.

Proposal 20: In S-DCI, for the second PDSCH, TCI state assumption is the Indicated TCI state 1.

Proposal 21: In S-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 22: In S-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Associations between TCI state(s) and UL signals or channels could follow the same principles as above in general. What comes to PUSCH transmission triggered by the DCI format 0_1/0_2 we think that legacy functionality should be remained, i.e. that SRI is provided in the DCI while Indicated TCI state can be associated to the corresponding SRS resource set. This is to enable port indication via SRI as in legacy.

In addition, one further aspect should be taken into consideration. It’s that a UE may have more than two antenna panels equipped typically while two SRS resource sets are assumed to be configured to the UE currently. In order to enable dynamic panel switching it should be possible to configure higher number of SRS resource sets, dynamically activate SRS resource set(s) and associate the certain active set(s) to the certain Indicated TCI state(s). It’s also to be noted that each panel (more than two panels) may have different number of antenna ports and thus each panel would need own SRS resource set configuration.

Observation 10: In order to enable dynamic panel switching in case of UE is equipped with more than two panels it should be possible to configure higher number of SRS resource sets than each set representing characteristics of a panel (number of SRS ports), dynamically activate SRS resource set(s) and dynamically associate the certain active set(s) to the certain Indicated TCI state(s).

Proposal 23: In S-DCI, for the SRS resource set, support dynamic association between the SRS resource set and Indicated TCI state.

Proposal 24: In S-DCI, for the dedicated PUCCH resource or dedicated PUCCH resource for repetition for S-TRP, TCI state assumption is Indicated joint or UL TCI state 0.

Proposal 25: In S-DCI, for the dedicated PUCCH resource for repetition for M-TRP, TCI state assumption is the Indicated joint or UL TCI state 0 and the Indicated joint or UL TCI state 1.

Proposal 26: In S-DCI, for the SRS resource (set), TCI state assumption is configured in RRC whether it is Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or joint or UL TCI state.

Proposal 27: In S-DCI, for the CG PUSCH type 1 and 2, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 28: In S-DCI, for the DG PUSCH 0_0, TCI assumption is the Indicated joint or UL TCI state of PUCCH resource ID 0.

Proposal 29: In S-DCI, for the DG PUSCH 0_1/0_2, TCI assumption is the joint or UL TCI state of the indicated SRS resource.

Regarding the following agreement

Agreement

On unified TCI framework extension for S-DCI based MTRP , down-select one alternative from the followings in RAN1#111:

- Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

- Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

  - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded

we consider that Alt1 would be the preferred choice because of better flexibility. In other words, it should be possible in S-DCI based MTRP to indicate joint/DL/UL TCI state(s) for one of the two TRPs or both TRPs in a CC BWP or a set of CCs/BWPs in a CC list. That can be performed e.g. by indicating a TCI codepoint that comprises one joint TCI state for one TRP being the same as the current one for that TRP and the other joint state in the codepoint is not the same as the previous one for the other TRP. In this case the UE would update the latter Indicated TCI state while the first one would remain the earlier one.

Proposal 30: On unified TCI framework extension for S-DCI based MTRP, support Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list.

### 2.3.1	Dynamic switching between single-TRP and multi-TRP DL/UL operations, TRP selection and ordering

In the following, we will discuss the need and potential ways for dynamic switching between S-TRP and M-TRP DL/UL operations as well as for TRP selection and ordering. But before, we would like to emphasize that such ways and the overall ways (that remain to be agreed) to associate TCI state(s) to UL transmission(s) under S-DCI based MTRP shall be at least as flexible as legacy Rel-17 and Rel-16 adopted ways. This is important in order to make sure that Rel-18 unified TCI framework doesn’t ‘downgrade’ existing features and operations.

Proposal 31: For dynamic switching between S-TRP and M-TRP DL/UL operations as well as for TRP selection and ordering, and for the overall association of TCI state(s) to UL transmission(s), under Rel-18 S-DCI based MTRP, RAN1 shall not aim at ways that are less flexible (or different) than related legacy procedures and operations.

For PDSCH:

One might argue that the switching between S-TRP and M-TRP PDSCH operations, and dynamic TRP selection/ordering, could be achieved using TCI state codepoints in MAC CE (in a similar way as Rel-16 M-TRP PDSCH operation), however this would reduce the flexibility as, for example, 4 TCI codepoints would be needed to indicate first TCI state, second TCI state, first TCI state + second TCI state, or second TCI state + first TCI state; and this is even if the maximum number of codepoints is extended to 16. Also, relying on such an approach would incur delays in the switching mechanism as each PDCCH carrying TCI state indication should be first acknowledged by the UE as defined in Rel-17. Thus, for the case with two (DL or joint) indicated TCI states, the switching between S-TRP and M-TRP PDSCH operations may require an indication e.g., in the DCI scheduling the PDSCH. Such an indication could also be extended to be used for selecting which TCI state to use or which TCI state is used first for the S-TRP and M-TRP PDSCH operations, respectively.

Proposal 32: For s-DCI mTRP PDSCH schemes, enable dynamic switching between S-TRP and M-TRP, as well as flexibility of TRP selection/ordering (as supported in Rel-16) using the same DCI that is scheduling the PDSCH transmission,

- FFS the related details once the association of joint/UL TCI state(s) to PDSCH transmission(s) with S-DCI based MTRP is designed and agreed.

For PUSCH:

An indication, similar to the one introduced in Rel-17 for M-TRP PUSCH repetition operation, would be needed for Rel-18. Specifically, when there are two indicated (UL or joint) TCI states, this indication would allow to dynamically select whether

- both TCI states should be used for the PUSCH, i.e., M-TRP PUSCH operation is applicable, or

- one TCI state, and if so, which TCI state should be used for the PUSCH, i.e., S-TRP PUSCH operation is applicable.

This indication would then in a way be similar to the one suggested above for PDSCH.

Proposal 33: Enable dynamic switching (as supported in Rel-17) between S-TRP and M-TRP PUSCH operations, as well as TRP or TCI state ordering and selection for these operations by reusing a similar method as for Rel-17 M-TRP/S-TRP PUSCH operations.

- FFS the related details, if any, once the association of joint/UL TCI state(s) to PUSCH transmission(s) with S-DCI based MTRP is designed and agreed.

For PUCCH:

Based on the potential ways for TCI state determination for PUCCH explained above, the UE would know whether/which one or two TCI states are associated with a PUCCH resource. Thus, the UE would know whether to apply S-TRP or M-TRP PUCCH repetition operation. Specifically, if the PUCCH resource is associated/indicated with two TCI states, then M-TRP PUCCH operation is applicable, whereas S-TRP PUCCH operation is applicable if the PUCCH resource is associated/indicated with one TCI state. On the TRP or TCI state ordering for the M-TRP PUCCH operation, the order of the TCI states associated/indicated for the PUCCH resource can be used.

Proposal 34: Enable dynamic switching (as supported in Rel-17) between S-TRP and M-TRP PUCCH operations by reusing a similar method as for the Rel-17 M-TRP/S-TRP PUCCH operations.

- FFS the related details, if any, once the association of joint/UL TCI state(s) to PUCCH transmission(s) with S-DCI based MTRP is designed and agreed.

For PUCCH resource groups:

Regarding PUCCH resource groups in single-DCI scenario, the following related agreement was made in RAN1#110bis-e:

Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group

Based on the Rel-16 specifications, up to four PUCCH resource groups (per BWP) can be configured via RRC. The main intention of this feature was to enable different PUCCH resource groups to have different spatial relation info, i.e., simultaneous update/indication of a single spatial relation per group of PUCCH resources by using one MAC CE. The Rel-17 multi-TRP PUCCH enhancement then introduced the possibility to indicate a PUCCH resource with two spatial-relation info via MAC CE, where these spatial-relation infos are then applicable to the PUCCH resource group to which the PUCCH resource belongs.

Considering now the Rel-18 objective on extending the unified TCI framework for multi-TRP, when there are PUCCH resource groups configured, there would be a need for the UE to determine the association between indicated (UL or joint) one or two TCI states and one or more of the configured PUCCH resource groups. In our view, the following options to provide such an association to the UE could be used (where these options could be complementing each other):

- Option 1: for the PUCCH that has a corresponding PDCCH, after the UE derives the PUCCH resource from the DCI field that indicates the PUCCH resource indicator (PRI), the UE can determine the PUCCH resource group that contains the derived PUCCH resource as the PUCCH resource group that should be associated to the indicated or applicable one/two (UL or joint) TCI states.

  - Note that this approach somewhat follows similar logic as that for the Rel-17 M-TRP PUCCH operation mentioned above.

- Option 2: Explicit indication providing the UE with the association between indicated (UL or joint) one or two TCI states and one or more of the configured PUCCH resource groups. This option is more in line with Alt.3 in the above related agreement.

In our view, and as highlighted previously, the approach to adopt here should be similar and provide same flexibility as the one agreed under Rel-17 M-TRP PUCCH enhancement. Hence, we think that Alt.3 should be adopted. However, if Alt.3 is not agreeable by a majority of companies, then we would be fine with Alt.1.

Proposal 35: On unified TCI framework extension for S-DCI based MTRP, select Alt.3 (1st preference) or Alt.1 (2nd preference) for PUCCH transmission:

- Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group.

- Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group.

### 2.3.2	On PUSCH transmission

Regarding PUSCH transmission under unified TCI framework the following agreement was made in the previous meeting:

Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS : PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH

Figure 4 illustrates the Alt1 and Alt2. Basically, both approaches have similar functionality but Alt2 (legacy) would allow higher number of candidate UL beams in the UL sounding phase while Alt1 would have up to two UL candidate beams. Regarding power control in Alt2, since SRS resource indicated in DCI by SRI would be having a TCI state to provide spatial source as well as associated power control parameters, PC control setting should be similar as in Alt1:

TCI-UL-State-r17 ::= SEQUENCE { tci-UL-State-Id-r17 TCI-UL-State-Id-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PUSCH-PathlossReferenceRS-Id-r17 OPTIONAL, -- Need R ... } Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R }

It’s also to be noted that Alt2 can provide the functionality of Alt1 by configuring SRS resource set(s) to follow Indicated TCI state(s). Thus, we consider that it would be preferable to support Alt2.

Figure 4 Illustration of Alt1 and Alt2.

Proposal 36: Support Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2.

## UL PC for UL MTRP

In this section, we discuss potential UL power control enhancements for UL single-DCI for multi-TRP operation considering unified TCI framework, as captured in the Rel-18 objective below:

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

### 2.4.1	General power control enhancements

In RAN1#109-e, the support of the extension of the Rel-17 unified TCI framework in terms of power control for PUCCH/PUSCH was agreed as follows:

Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

On the first FFS point in the above agreement, we think that the agreed operation would also apply to Rel-18 M-TRP with STxMP. Whether additional enhancements are needed can be further discussed.

On the other hand, it’s worth recalling that Rel-17 introduced some power control enhancements for multi-TRP PUCCH/PUSCH operations, such as:

- the possibility for the gNB to configure a second TPC command field in DCI (formats 1_1/1_2 and 0_1/0_2) for both multi-TRP PUCCH and PUSCH operations,

- the possibility to indicate two open-loop power control parameter P0 values for M-TRP PUSCH operation, in case there is no SRI field in DCI (format 0_1 and 0_2).

Considering the extension of the Rel-17 unified TCI framework to multi-TRP use cases in Rel-18, the following can be observed on the Rel-17 M-TRP power control enhancements (for single-DCI mode):

- The Rel-17 enhancements on introducing a second TPC command field in DCI for both multi-TRP PUCCH and PUSCH operations could be used in Rel-18. Specifically, a second TPC command field can be configured in DCI, and for a multi-TRP PUSCH/PUCCH operation considering two indicated TCI states (each associated with a different closed-loop index), first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to a first and second TCI states, respectively; this assumes that these two TCI states are associated to different closed-loop indexes.

  - If the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second TCI states.

- Note that the Rel-17 enhancement to indicate two open-loop power control parameter P0 values for M-TRP PUSCH operation is inherited in the Rel-17 unified TCI framework design, since P0 is associated with an UL or joint TCI state. And the extension of this feature to M-TRP has been agreed in RAN1#109-e, as shown earlier.

Proposal 37: For Rel-18 multi-TRP PUSCH/PUCCH operations under unified TCI framework, reuse the Rel-17 multi-TRP power control enhancement as follows:

- a second TPC field can be configured in DCI formats 1_1/1_2 and 0_1/0_2 to support per-TRP closed-loop power control. Specifically, first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to first and second indicated TCI states, respectively;

- if the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second indicated TCI states.

Further, for the cases where at least one of the indicated joint/UL TCI states would not be associated to UL PC parameter setting, the following proposal has been proposed in RAN1#110bis-e (based on the related RAN1#110 proposal listing three alternatives):

Proposal 4.A: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, down-selection one alternative from the followings: - Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting - Alt2: No change from Rel-17 unified TCI framework

Overall, we support this proposal and are in principle fine with Alt.1 and Alt.2, with a slight preference towards Alt.1. It’s worth noting that, under Rel-17 M-TRP UL enhancements, two default power control parameter sets are considered. And this is more in line with Alt.1.

Proposal 38: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, select any of Alt1 and Alt2:

- Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP

  - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting

- Alt2: No change from Rel-17 unified TCI framework, i.e., the UE should apply the one single default UL PC parameter setting configured in the corresponding UL BWP regardless the UL PC parameter setting is absent from one or both of indicated joint/UL TCI states.

### 2.4.2	Power control related enhancements considering multi-panel UE and simultaneous UL transmissions

In legacy operations, PCMAX,f,c, which basically represents the maximum allowed power for a UE for its UL transmissions, is defined per carrier for a serving cell. For simultaneous UL transmissions, where two PUSCHs or two PUCCHs that overlap in time would be (simultaneously) transmitted on the same carrier, and more generally for multi-panel UE, one important point to discuss and decide is whether to keep PCMAX per carrier or whether to define panel specific PCMAX. Panel specific PCMAX may be needed, when, for example, panel-specific UE power class (e.g., due to different PA characteristics for each panel) is introduced, or panel specific MPR related considerations are assumed, etc.

A related discussion happened in RAN1#109-e, where it was agreed to send an LS to RAN4 in order to check some related aspects, such as whether it’s feasible to assume power limitation per panel or over all UE panels for STxMP, as captured in the below agreement:

Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

In FR2, typically UE devices are having a distributed PA architecture where the PA is part of the mmW module instead of a PA that can be switched to different antennas which is the typical architecture assumption in FR1. The distributed PA architecture means that there is a low power PA per antenna element or per small set of antenna elements within a panel. That means that antenna panels are having their own PAs, in contrast to FR1 where multiple antennas may be switched to PA(s).

Observation 11: In FR2, typically devices are equipped with a distributed PA architecture where each antenna panel is having its own PA(s).

That makes the Assumption1 (Power limitation per panel for STxMP) more suitable for UEs to be adopted in FR2, especially for the high-power class UE devices as targeted in Rel18: FWA, CPE, Vehicular, etc. In addition, as uplink coverage is typically the bottleneck, the higher total EIRP would be beneficial to alleviate uplink coverage issues.

Proposal 39: Support at least Assumption 1: Power limitation per panel for STxMP.

Once the above aspects are clarified, PH (power headroom) aspect would need to be discussed. It’s worth recalling that Rel-17 introduced the reporting of two PHs for multi-TRP PUSCH operation. This could be used as a starting point for the Rel-18 discussions on PH aspects for simultaneous UL transmissions, and more generally for multi-panel transmissions. However, given that the network will be switching between single-TRP and multi-TRP UL schemes (such as SDM and TDM), e.g., based on some UE reporting such as capability value set index reporting, having two PHRs calculated and reported for a cell may not be needed when M-TRP UL schemes are not feasible/needed/applicable, i.e., when only single-TRP scheme is feasible/needed, be it from the UE and/or network perspectives. For instance, if there is a single (UL/joint) TCI state indicated/applicable in the cell, then calculating and reporting two PHRs would not be necessary and would thus unnecessarily increase the UL control overhead.

Another aspect that would need to be discussed is when two-PHR mode is not configured for a cell, i.e., the UE would potentially be required to calculate and report one PH. In this case, for two simultaneous PUSCH transmissions, it should be discussed which PUSCH transmission to consider for the PH calculation and reporting.

Observation 12: Considering the feasibility/need of single-TRP UL scheme versus multi-TRP UL scheme (including simultaneous UL transmissions, such as SDM) from UE and/or network perspectives, two-PHR reporting may not be needed when M-TRP UL schemes are not feasible/applicable. In addition, the UE would be required to potentially calculate and report one PH in case the two-PHR mode is not configured.

Proposal 40: RAN1 to discuss PH (power headroom) aspects considering the Rel-18 schemes, including simultaneous UL transmissions, such as (i) number of PHRs to report at a given time and which UL scheme to use for the calculation of the PHR(s), and (ii) the cases where the UE is not configured with two-PHR mode.

Further, given that Rel-18 MIMO objective 6 is targeting “CPE/FWA/vehicle/industrial devices”, it should be discussed whether MPE issues would need to be considered in the study. In other words, it should be discussed whether, for instance, some industrial devices can be handheld devices or not. Based on this, RAN1 could decide whether, considering Rel-17 MPE reporting as a basis, some MPE enhancements for simultaneous UL transmission operation are needed.

Proposal 41: Decide whether MPE is an issue considering the targeted devices in the Rel-18 objective on simultaneous UL transmissions.

## Beam reporting and indication for simultaneous multi-panel UL

In this section we discuss about beam indication for simultaneous multi-panel transmission. First, we discuss about capability value index reporting developed in Rel-17 and whether an extension is needed to facilitate simultaneous multi-panel UL transmissions. Then, we discuss about beam indication aspects.

### 2.5.1 Enhanced capability reporting

Rel-17 has introduced a panel capability aware beam reporting to support fast UL beam and panel selection for single panel UL transmission as described in the following RAN1#108-e agreements and conclusion:

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - Each UE capability value [set] comprises the max supported number of SRS ports - Any two capability values [sets] are different - Whether the UE capability value [set] can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, UE can report one index of UE capability value [set] for each reported CRI/SSBRI in one beam reporting. Conclusion On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, there is no consensus in supporting indication of DL-only panel using one value [set] of the max supported number of SRS ports Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, all types of time-domain behavior, i.e., periodic, semi-persistent, and aperiodic reporting, are supported for the enhanced beam report with index(es) of UE capability value [set]. - Support of aperiodic and semi-persistent reporting is optional

Rel-17 new reporting was designed with a single-panel PUSCH (codebook based PUSCH) in mind. Thus, for instance two panels at the UE side having the same number of SRS antenna ports are seen at the gNB side as one panel due to the following agreement:

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - … - Any two capability values [sets] are different - …

In other words, the UE may report two different SSBRIs/CRIs with the same capability value index but the gNB cannot determine whether the UE has measured the reported SSBRIs/CRIs with the same or different panel. Thus, the gNB cannot get aware based on Rel-17 reporting whether the UE would be able to transmit to different TRPs with one panel (e.g., in TDM manner) or with two different panels (e.g., in FDM or SDM manner) at a time. Thus, we think that it would be beneficial to extend the Rel-17 capability value index reporting to support also simultaneous multi-panel uplink transmission. In the enhanced reporting, the report should carry information about UE’s simultaneous transmission possibility of two panels.

Observation 13: The UE may report two different SSBRIs/CRIs with the same capability value index but the gNB cannot determine whether the UE has measured the reported SSBRIs/CRIs with the same or different panel. Thus, the gNB cannot get aware based on Rel-17 reporting whether the UE would be able to transmit to different TRPs with one panel (e.g., in TDM manner) or with two different panels (e.g., in FDM or SDM manner) at a time.

Proposal 42: Enhance capability value index reporting to carry information to gNB on whether or not the UE is able to transmit simultaneously from two panels, where the panels may have same or different maximum number of SRS ports.

### 2.5.2 Enhanced reporting to alleviate inter-beam interference in simultaneous multi-panel uplink transmissions

In multi-TRP scenario, the distance and radio conditions between the transmitting panels and receiving TRPs may differ quite much. For the uplink transmission, an open loop power control is applied that aims at compensating the higher path loss by the higher transmission power. Thus, the panels transmitting simultaneously may be having quite different EIRP per resource element/subcarrier. Even though the transmit beams may be spatially separated (main lobes), the higher EIRP panel may have sidelobes in relatively high level compared to the main lobe of the transmit beam of the other panel. Depending on the transmit beams the higher EIRP panel may cause high interference with its side lobes to the receiving TRP of the other panel with lower EIRP. In that case, FDM transmission scheme may be preferred over the SDM scheme while in other cases SDM may be preferred.

Then the question is that how the gNB could get aware of the situation so that it could make a proper transmission scheme selection for the simultaneous multi-panel uplink transmission. One way to determine feasible transmission scheme is when the SRS transmissions are received and potential precoder matrix/matrices are estimated at the gNB side. Another way could be to consider how the UE would choose RSs for reporting as potential source RSs to determine uplink transmit spatial filters. Let’s denote RS1 as the candidate spatial source RS for the first panel towards the first TRP and RS2 as the candidate spatial source RS for the second panel towards the second TRP. Both panels measure using their intended TX beam (TX beam of first panel according to RS1 and TX beam of second panel according to RS2) as RX beam RSRP of both spatial source RSs. Then if the RSRP of RS1 measured by the second panel is in the same level as the RSRP of RS1 measured by the first panel, or if the RSRP of RS2 measured by the first panel is in the level as the RSRP of RS2 measured by the second panel, there may be an inter-beam interference issue and e.g. SDM may not be feasible but rather FDM would be preferable in this case. In the beam reporting the UE could inform gNB about feasible pairs of DL RSs to determine uplink spatial transmit filters and feasible multiplexing method(s) (SDM, FDM, SFN or TDM). DL RSs could also reflect the QCL-Type D RSs of the indicated TCI states.

Proposal 43: Study schemes to alleviate the inter-beam interference issue for simultaneous multi-panel uplink transmissions.

Proposal 44: Consider UE to report information regarding feasible multiplexing schemes of simultaneous two-panel UL PUSCH based on DL RS measurements where DL RSs represent the candidate UL TX beams.

### 2.5.3 Beam indication for single-DCI and multi-DCI modes

Once the reporting capability is defined/supported to enable the simultaneous UL transmission operation, both UE and gNB would have same understanding on which panels and UL/joint TCI states could be used for simultaneous UL transmissions. Then, for two UL/joint TCI states, both the UE and gNB would know whether the UE can perform simultaneous UL transmissions or not using these two TCI states, where each TCI state would be mapped to a different UL transmission(s) of the simultaneous UL transmission operation.

Hence, if the gNB decides to schedule the UE with simultaneous UL transmissions, the gNB would need to have two indicated UL/joint TCI states corresponding to capability value set indices / panels for which the UE has already indicated the capability of simultaneous transmissions using these panels / TCI states – through for example the enhanced reporting suggested in the previous subsections.

Note that the above reasoning could be applicable for simultaneous UL transmissions scheduled via single-DCI mode or multi-DCI mode.

Observation 14: Enhanced capability value set reporting to enable beam indication for simultaneous UL transmissions could be used for both single-DCI and multi-DCI modes.

In general, for single-DCI and multi-DCI modes, it’s expected that the beam indication for simultaneous UL transmissions would follow the respective operations that will be defined to enable Rel-18 multi-TRP UL operations considering unified TCI framework; we discuss such operations in an earlier section.

Observation 15: For single-DCI and multi-DCI modes, it’s expected that the beam indication for simultaneous UL transmissions would follow the respective operations that will be defined to generally enable Rel-18 multi-TRP UL operations considering unified TCI framework.

## 2.6 BFR Aspects

RAN1#110bis-e made the following agreements to study enhancements for TRP-specific BFR.

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

### 2.6.1 S-DCI BFR extension

As the S-DCI mTRP does not use CORESETpoolIndex, the determination of BFD-RS in BFD-RS sets would need to be based on the indicated TCI states (first and/or second). The S-DCI framework allows one or two indicated TCI states for PDCCH (e.g. for repetition) based on the configuration associated with one or more CORESETs.

If the CORESET(s) of a BWP are associated with a configuration for first indicated TCI state only UE determines that it monitors the beam failure on using one BFD-RS set. If UE receives indication for second TCI State it is applied for at least PDSCH reception but not for PDCCH monitoring and thus it does not have effect on implicit BFD-RS configuration

Proposal 45: If CORESET(s) of a BWP are associated with configuration of first indicated TCI state, UE monitors beam failure on one set of q0, and the set includes RS included in the TCI state.

If a CORESET (or set) are associated with a configuration for first indicated TCI state and other CORESET (or sets) with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets. The RS indicated by the first TCI state is included in the first BFD-RS set and the RS indicated by the second TCI state in the second BFD-RS set.

Proposal 46: If a CORESET (or set) are associated with a configuration for first indicated TCI state and other CORESET (or sets) with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets (set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state).

If a CORESET on BWP is associated with configuration of both for first indicated TCI state and second indicated TCI state i.e. the CORESET is assumed as SFN CORESET, the RSs of both TCI States could be monitored on one BFD-RS set. However, if on a BWP there is one CORESET with SFN configuration and e.g additional CORESETs set without SFN configuration (associated with one of the indicated TCI state configurations) the UE could monitor the beam failure on two sets: set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state.

Proposal 47: If BWP has configuration of only an SFN CORESET, the beam failure is monitored on one BFD-RS including RS indicated by the first and second sets.

Proposal 48: If BWP has configuration of an SFN CORESET and a non-SFN CORESET, the beam failure is monitored on two BFD-RS sets: set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state.

### 2.6.2 M-DCI BFR extension

For M-DCI, the extension should be rather straightforward if the CORESETpoolIndex approach is used. Each CORESETPoolIndex is associated with should be rather straightforward to extend the R17 mTRP BFR to unified TCI for M-DCI. The BFD-RS sets are associated with the CORESETpoolIndex, and the RS in respective sets are based on the indicated joint or DL unified TCI State for the CORESETPoolIndex.

Proposal 49: On unified TCI framework extension for M-DCI, the CORESETpoolindices are associated with respective BFD-RS sets.

Proposal 50: For a BFD-RS set, the RS to be included in the set is determined based on the indicated joint or (separate) DL unified TCI state for the CORESETpoolindex.

Proposal 51: The beam update after BFR is applied for associated with the CORESETpoolIndex of the respective failed BFD-RS set.

# 3.	Conclusions

Based on the discussion above we made the following observations and proposals:

General framework for unified TCI framework:

Proposal 1: Support the following combinations of joint/DL/ UL TCI states that can be indicated:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 UL TCI state indicated for a second TRP (same or different PCI)

- Up to 2 joint or DL TCI states (same PCI)

Proposal 2: Study whether the UE should be provided a TRS signal (QCL parameters, like TypeA) for the CJT PDSCH reception that is transmitted with the same precoder (e.g. 1st layer) across the transmitting TRPs as used for the CJT PDSCH.

TCI state update and activation:

Observation 2: In m-DCI, AP CSI-RS can follow Indicated TCI state of the CORESET of the triggering PDCCH.

Observation 3: In m-DCI, determining of which of the Indicated TCI states is updated would be based on the coresetPoolIndex of the CORESET of the PDCCH transmitting the DCI AP-CSI-RS trigger.

Observation 4: In m-DCI, configuration needs to be provided for dedicated periodic/semi-persistent PUCCH resource(s) that whether the resource(s) follow Indicated TCI state 0 or Indicated TCI state 1.

Observation 5: In m-DCI, configuration needs to be provided for SRS resource (set) that whether the resource(s) follow Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 3: In M-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether it is the Indicated joint or DL TCI state associated to coresetPoolIndex of the CORESET of the triggering PDCCH or joint or DL TCI state.

Proposal 4: In M-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 5: In M-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Proposal 6: In M-DCI, for the P-/SP-Dedicated PUCCH resource, TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 7: In M-DCI, for the DCI triggered dedicated PUCCH resource, TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 8: In M-DCI, for the SRS resource (set), TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or joint or UL TCI state.

Proposal 9: In M-DCI, for the DCI triggered SRS resource (set), TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 10: In M-DCI, for the CG PUSCH type 1 and 2, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 11: In M-DCI, for the DG PUSCH 0_0, TCI assumption is the Indicated joint or UL TCI state of PUCCH resource ID 0.Proposal 12: In M-DCI, for the DG PUSCH 0_1/0_2, TCI assumption is the joint or UL TCI state of the Indicated SRS resource.

Proposal 13: A single DCI codepoint can be configured to indicate multiple (e.g. two) joint TCI states or multiple separate DL and/or UL TCI States.

- For Rel-16/17 s-DCI mTRP schemes, only two joint or two separate DL and/or two UL TCI states may be configured in a single codepoint and it may depend on the configured/applicable Rel-16/17 mTRP scheme

Proposal 14: Support the max number of MAC CE activated TCI codepoints being 16.

Proposal 15: Support up to four-bit TCI field in DCI.

Proposal 16: Study in S-DCI M-TRP scenario that how UE can determine which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured either with single- or multi-TRP configuration.

Proposal 17: Study in M-DCI M-TRP scenario that how UE can determine which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured with one or two coreset pool indices.

How to associate the indicated TCI state(s) with each target channel/signal in S-DCI based MTRP:

Observation 6: In s-DCI PDSCH schemes, AP CSI-RS resource (set) should be able to be associated to one of the (two) Indicated TCI state where one the of Indicated TCI states is the same as for the CORESET used for the PDCCH triggering the AP CSI-RS.

Observation 7: In s-DCI PDSCH schemes, when scheduled two PDSCHs (two PDSCH transmission occasions), one PDSCH would follow Indicated TCI state 0 and the other PDSCH would follow Indicated TCI state 1.

Observation 8: In s-DCI PUSCH schemes, configuration needs to be provided for SRS resource (set) that whether the resource(s) follow Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Observation 9: For PUCCH repetition and to facilitate dynamic switching between sTRP and mTRP schemes, some PUCCH resource should be able to be associated to a single Indicated TCI state while other PUCCH resources should be able to be associated to two Indicated TCI states.

Proposal 18: In S-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 19: In S-DCI, for the first PDSCH, TCI state assumption is the Indicated TCI state 0.

Proposal 20: In S-DCI, for the second PDSCH, TCI state assumption is the Indicated TCI state 1.

Proposal 21: In S-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 22: In S-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Observation 10: In order to enable dynamic panel switching in case of UE is equipped with more than two panels it should be possible to configure higher number of SRS resource sets than each set representing characteristics of a panel (number of SRS ports), dynamically activate SRS resource set(s) and dynamically associate the certain active set(s) to the certain Indicated TCI state(s).

Proposal 23: In S-DCI, for the SRS resource set, support dynamic association between the SRS resource set and Indicated TCI state.

Proposal 24: In S-DCI, for the dedicated PUCCH resource or dedicated PUCCH resource for repetition for S-TRP, TCI state assumption is Indicated joint or UL TCI state 0.

Proposal 25: In S-DCI, for the dedicated PUCCH resource for repetition for M-TRP, TCI state assumption is the Indicated joint or UL TCI state 0 and the Indicated joint or UL TCI state 1.

Proposal 26: In S-DCI, for the SRS resource (set), TCI state assumption is configured in RRC whether it is Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or joint or UL TCI state.

Proposal 27: In S-DCI, for the CG PUSCH type 1 and 2, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 28: In S-DCI, for the DG PUSCH 0_0, TCI assumption is the Indicated joint or UL TCI state of PUCCH resource ID 0.

Proposal 29: In S-DCI, for the DG PUSCH 0_1/0_2, TCI assumption is the joint or UL TCI state of the indicated SRS resource.

Proposal 30: On unified TCI framework extension for S-DCI based MTRP, support Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list.

Proposal 31: For dynamic switching between S-TRP and M-TRP DL/UL operations as well as for TRP selection and ordering, and for the overall association of TCI state(s) to UL transmission(s), under Rel-18 S-DCI based MTRP, RAN1 shall not aim at ways that are less flexible (or different) than related legacy procedures and operations.

Proposal 32: For s-DCI mTRP PDSCH schemes, enable dynamic switching between S-TRP and M-TRP, as well as flexibility of TRP selection/ordering (as supported in Rel-16) using the same DCI that is scheduling the PDSCH transmission,

- FFS the related details once the association of joint/UL TCI state(s) to PDSCH transmission(s) with S-DCI based MTRP is designed and agreed.

Proposal 33: Enable dynamic switching (as supported in Rel-17) between S-TRP and M-TRP PUSCH operations, as well as TRP or TCI state ordering and selection for these operations by reusing a similar method as for Rel-17 M-TRP/S-TRP PUSCH operations.

- FFS the related details, if any, once the association of joint/UL TCI state(s) to PUSCH transmission(s) with S-DCI based MTRP is designed and agreed.

Proposal 34: Enable dynamic switching (as supported in Rel-17) between S-TRP and M-TRP PUCCH operations by reusing a similar method as for the Rel-17 M-TRP/S-TRP PUCCH operations.

- FFS the related details, if any, once the association of joint/UL TCI state(s) to PUCCH transmission(s) with S-DCI based MTRP is designed and agreed.

Proposal 35: On unified TCI framework extension for S-DCI based MTRP, select Alt.3 (1st preference) or Alt.1 (2nd preference) for PUCCH transmission:

- Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group.

- Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group.

Proposal 36: Support Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2.

UL PC for UL MTRP:

Proposal 37: For Rel-18 multi-TRP PUSCH/PUCCH operations under unified TCI framework, reuse the Rel-17 multi-TRP power control enhancement as follows:

- a second TPC field can be configured in DCI formats 1_1/1_2 and 0_1/0_2 to support per-TRP closed-loop power control. Specifically, first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to first and second indicated TCI states, respectively;

- if the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second indicated TCI states.

Proposal 38: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, select any of Alt1 and Alt2:

- Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP

  - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting

- Alt2: No change from Rel-17 unified TCI framework, i.e., the UE should apply the one single default UL PC parameter setting configured in the corresponding UL BWP regardless the UL PC parameter setting is absent from one or both of indicated joint/UL TCI states.

Observation 11: In FR2, typically devices are equipped with a distributed PA architecture where each antenna panel is having its own PA(s).

Proposal 39: Support at least Assumption 1: Power limitation per panel for STxMP.

Observation 12: Considering the feasibility/need of single-TRP UL scheme versus multi-TRP UL scheme (including simultaneous UL transmissions, such as SDM) from UE and/or network perspectives, two-PHR reporting may not be needed when M-TRP UL schemes are not feasible/applicable. In addition, the UE would be required to potentially calculate and report one PH in case the two-PHR mode is not configured.

Proposal 40: RAN1 to discuss PH (power headroom) aspects considering the Rel-18 schemes, including simultaneous UL transmissions, such as (i) number of PHRs to report at a given time and which UL scheme to use for the calculation of the PHR(s), and (ii) the cases where the UE is not configured with two-PHR mode.

Proposal 41: Decide whether MPE is an issue considering the targeted devices in the Rel-18 objective on simultaneous UL transmissions.

Beam reporting and indication for simultaneous multi-panel UL:

Observation 13: The UE may report two different SSBRIs/CRIs with the same capability value index but the gNB cannot determine whether the UE has measured the reported SSBRIs/CRIs with the same or different panel. Thus, the gNB cannot get aware based on Rel-17 reporting whether the UE would be able to transmit to different TRPs with one panel (e.g., in TDM manner) or with two different panels (e.g., in FDM or SDM manner) at a time.

Proposal 42: Enhance capability value index reporting to carry information to gNB on whether or not the UE is able to transmit simultaneously from two panels, where the panels may have same or different maximum number of SRS ports.

Proposal 43: Study schemes to alleviate the inter-beam interference issue for simultaneous multi-panel uplink transmissions.

Proposal 44: Consider UE to report information regarding feasible multiplexing schemes of simultaneous two-panel UL PUSCH based on DL RS measurements where DL RSs represent the candidate UL TX beams.

Observation 14: Enhanced capability value set reporting to enable beam indication for simultaneous UL transmissions could be used for both single-DCI and multi-DCI modes.

Observation 15: For single-DCI and multi-DCI modes, it’s expected that the beam indication for simultaneous UL transmissions would follow the respective operations that will be defined to generally enable Rel-18 multi-TRP UL operations considering unified TCI framework.

BFR Aspects:

Proposal 45: If CORESET(s) of a BWP are associated with configuration of first indicated TCI state, UE monitors beam failure on one set of q0, and the set includes RS included in the TCI state.

Proposal 46: If a CORESET (or set) are associated with a configuration for first indicated TCI state and other CORESET (or sets) with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets (set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state).

Proposal 47: If BWP has configuration of only an SFN CORESET, the beam failure is monitored on one BFD-RS including RS indicated by the first and second sets.

Proposal 48: If BWP has configuration of an SFN CORESET and a non-SFN CORESET, the beam failure is monitored on two BFD-RS sets: set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state.

Proposal 49: On unified TCI framework extension for M-DCI, the CORESETpoolindices are associated with respective BFD-RS sets.

Proposal 50: For a BFD-RS set, the RS to be included in the set is determined based on the indicated joint or (separate) DL unified TCI state for the CORESETpoolindex.

Proposal 51: The beam update after BFR is applied for associated with the CORESETpoolIndex of the respective failed BFD-RS set.

# References

- RP-213598, ”New WID: MIMO Evolution for Downlink and Uplink”, Samsung (Moderator)